3GPP TSG-CT WG1 Meeting #129-e	C1-212xyz

- Electronic meeting; 19-23 April 2021	(was C1-212320)

Source:	Huawei, HiSilicon, CATT, China Telecom, China Unicom

Title:	Handling of unknown, unforeseen, and erroneous service data

Spec:	3GPP TS 24.558

Agenda item:	17.2.16

Document for:	Agreement

1. Introduction

3GPP TS 24.558 contains under the Annex B a clause on Handling of unknown, unforeseen, and erroneous service data.

2. Reason for Change

The handling of unknown, unforeseen, and erroneous service data needs to be defined.

As per other CT1 specifications, handling of unknown, unforseen, and erroneous service date should defined, e.g., 3GPP TS 24.519, which follow rules defined by 3GPP TS 24.007.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.558 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[r24007]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

API	Application Programming Interface

EAS	Edge Application Server

ECS	Edge Configuration Server

EEC	Edge Enabler Client

EES	Edge Enabler Server

NAS	Non Access Stratum

SPS	Service provisioning service

UE	User Equipment

* * * Next Change * * * *

### B.2.3	Handling of unknown, unforeseen, and erroneous service data

#### B.2.3.1	General

The procedures specified in clause B.2.2.2 apply to those messages which pass the checks described in clause B.2.3.

Clause B.2.3 also specifies procedures for the handling of unknown, unforeseen, and erroneous service provisioning service (SPS) data by the receiving entity. These procedures are called "error handling procedures", but in addition to providing recovery mechanisms for error situations they define a compatibility mechanism for future extensions of the SPS.

Clauses B.2.3.1 to B.2.3.7 shall be applied in order of precedence.

Detailed error handling procedures in the ECS are implementation dependent and may vary from network to network. However, when extensions of SPS are developed, ECS s are assumed to have the error handling which is indicated in this clause as mandatory ("shall") and that is indicated as strongly recommended ("should").

Also, the error handling of the ECS is only considered as mandatory or strongly recommended when certain thresholds for errors are not reached during a dedicated connection.

For definition of semantical and syntactical errors see 3GPP TS 24.007 [r24007], clause 11.4.2.

* * * Next Change * * * *

#### B.2.3.2	Message too short or too long

B.2.3.2.1	Message too short

When a message is received that is too short to contain a complete message type information element, that message shall be ignored, cf. 3GPP TS 24.007 [r24007].

##### B.2.3.2.2	Message too long

The maximum size of a SPS message sent by the EEC to the ECS or sent by the ECS to the ECS is 65535 octets.

* * * Next Change * * * *

#### B.2.3.3	Unknown or unforeseen message type

If the UE or the ECS receives an SPS message with message type not defined for the SPS or not implemented by the receiver, it shall ignore the SPS message.

NOTE:	A message type not defined for the SPS in the given direction is regarded by the receiver as a message type not defined for the SPS, see 3GPP TS 24.007 [r24007].

* * * Next Change * * * *

#### B.2.3.4	Non-semantical mandatory information element

When on receipt of a message,

a)	an "imperative message part" error; or

b)	a "missing mandatory IE" error

is diagnosed or when a message containing:

a)	a syntactically incorrect mandatory IE;

b)	an IE unknown in the message, but encoded as "comprehension required" (see 3GPP TS 24.007 [4]); or

c)	an out of sequence IE encoded as "comprehension required" (see 3GPP TS 24.007 [4]) is received,

If the message is an SPS message, the EEC and the ECS shall proceed as follows:

1)	try to treat the message (the exact further actions are implementation dependent); or

2)	ignore the message.

* * * Next Change * * * *

#### B.2.3.5	Unknown and unforeseen IEs in the non-imperative message part

B.2.3.5.1	IEIs unknown in the message

The EEC and ECS shall ignore all IEs unknown in a message which are not encoded as "comprehension required" (see 3GPP TS 24.007 [r24007]).

##### B.2.3.5.2	Out of sequence IEs

The EEC and ECS shall ignore all out of sequence IEs in a message which are not encoded as "comprehension required" (see 3GPP TS 24.007 [4]).

##### B.2.3.5.3	Repeated IEs

If an information element with format T, TV, TLV, or TLV-E is repeated in a message in which repetition of the information element is not specified in clause 8, the EEC shall handle only the contents of the information element appearing first and shall ignore all subsequent repetitions of the information element. When repetition of information elements is specified, the EEC shall handle only the contents of specified repeated information elements. If the limit on repetition of information elements is exceeded, the EEC shall handle the contents of information elements appearing first up to the limit of repetitions and shall ignore all subsequent repetitions of the information element.

The ECS should follow the same procedures.

* * * Next Change * * * *

#### B.2.3.6	Non-imperative message part errors

B.2.3.6.1	IEIs unknown in the message

This category includes:

a)	syntactically incorrect optional IEs; and

b)	conditional IE errors.

##### B.2.3.6.2	Syntactically incorrect optional IEs

The EEC shall treat all optional IEs that are syntactically incorrect in a message as not present in the message.

The ECS shall take the same approach.

##### B.2.3.6.2	Conditional IE errors

When upon receipt of an SPS message the EEC diagnoses a "missing conditional IE" error or an "unexpected conditional IE" error, or when it receives an SPS message containing at least one syntactically incorrect conditional IE, the EEC shall ignore the message.

When the ECS receives a message and diagnoses a "missing conditional IE" error or an "unexpected conditional IE" error or when it receives a message containing at least one syntactically incorrect conditional IE, the ECS shall either:

a)	try to treat the message (the exact further actions are implementation dependent); or

b)	ignore the message.

* * * Next Change * * * *

#### B.2.3.7	Messages with semantically incorrect contents

When a message with semantically incorrect contents is received, the EEC shall perform the foreseen reactions of the procedural part of clause B.2.2. If, however no such reactions are specified, the EEC shall ignore the message.

The ECS should follow the same procedure.