3GPP TSG-CT WG1 Meeting #131-e	C1-214253-r2

- E-meeting, 19-27 August 2021

Title:	Indication of country of UE location and its use in PLMN selection

Response to:	- -

Release:	Release 17

Work Item:	5GSAT_ARCH-CT

Source:	CT1

To:	SA1

Cc:	SA2, SA3-LI

Contact person:	Chen-Ho CHIN

chin.chenho@oppo.com

+32 477 413371

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S3i200056, 23.502CR2482r1 (S2-2101667)

# 1	Overall description

SA1 would be aware that SA3-LI has for FS_5GSAT_ARCH, indicated the requirement that "Any solution shall support the ability to enforce the use of a Core Network of PLMN in the country where the UE is physically located " – see attached S3i200056

To this requirement, SA2 has introduced a solution wherein should the Core Network having determined where the UE is located is a location in which the PLMN is not allowed to provide service to the UE, will reject the UE with an appropriate reject cause and optionally indicate "the country of the UE location using the MCC(s) assigned to that country" (formulation used in one place) and "the country of the UE location" (formulation used in other places) – see attached 23.502CR2482r1 (S2-2101667).

CT1 is working on the stage 3 changes needed to realize the SA2 agreed solution and has for the last few CT1 eMeetings been unable to reach a consensus on how to interpret and implement the SA2 solution.

To assist CT1 to make progress, CT1 seeks guidance from SA1 for the following:-

1)	Should the indication of UE location represent a "country" in the manner a "country" is defined in TS 23.122, or should the "country" be the TR 22.926 definition of "a country is defined as the area embedded within a set of borders and for which a unique set of regulations applies for the provision of communication services through mobile networks"?

Further CT1 would like to ask SA1's guidance on the usage/applicability, if at all, of the indicated country of UE location, in the subsequent PLMN selection, i.e. shall it be considered by the UE:

2)	that the UE is not allowed in the current geographical position to select any PLMN which does not correspond to the indicated country of UE location; or

3)	the UE might select available and allowable PLMNs of that and other countries.

# 2	Actions

To 3GPP TSG SA WG1

ACTION:	CT1 kindly request SA1 to provide guidance and answers to questions above..

# 3	Dates of next TSG CT WG1 meetings

CT1#132e	11th-15th October 2021	electronic meeting

CT1#133e	11th-19th November 2021	electronic meeting