3GPP TSG-CT WG1 Meeting #131-e	C1-21xxxx

E-meeting, 19-27 August 2021 	was C1-214273

Source:	ZTE

Title:	Layer-3 UE-to-Network relay with N3IWF support

Spec:	3GPP TS 24.554 v0.3.0

Agenda item:	17.2.18

Document for:	Agreement

1. Introduction

Excerpted from TS 23.304, clause 5.4.1.2:

"…

To support Remote UE services with end-to-end confidentiality and IP address preservation requirements, the 5G ProSe Layer-3 UE-to-Network Relay with N3IWF shall provide access to the 5GC for the Remote UE via N3IWF using the features defined in clause 4.2.8 and clause 5.30.2.7 of TS 23.501 [4].

5G ProSe Layer-3 UE-to-Network Relay is provisioned with Relay Service Code(s) and the corresponding PDU session parameters (e.g. S-NSSAI) to support N3IWF access as part of 5G ProSe Layer-3 UE-to-Network Relay Policy/parameters. When a Remote UE connects with the corresponding Relay Service Code, the 5G ProSe Layer-3 UE-to-Network Relay determines the corresponding PDU session parameters based on the requested Relay Service Code.

NOTE:	The 5G ProSe Layer-3 UE-to-Network Relay only includes a Relay Service Code in discovery message when the corresponding PDU session parameters (e.g. S-NSSAI) are authorized to be used in the accessed network.

When selecting the N3IWF as specified in TS 23.501 [4] clause 6.3.6, the UE uses N3IWF configuration. Selection of the N3IWF follows the regulatory rules of the country where it is located, and when required by the regulations the Remote UE only selects a N3IWF within the local country. QoS differentiation can be provided on per-IPsec Child Security Association basis and the details are provided in clause 5.6.2.2.

The 5GC to which the 5G ProSe Layer-3 UE-to-Network Relay registers and the 5GC to which the 5G ProSe Remote UE registers may be in the same PLMN or different PLMN.

…"

2. Reason for Change

5G ProSe layer 3 UE-to-network relay with N3IWF support needs to be specified in stage 3.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554 v0.3.0.

* * * First Change * * * *

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS); Stage 2".

[3]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[4]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[5]	3GPP TS 24.526: "UE policies for 5G System (5GS); Stage 3".

[6]	OMA-WAP-TS-PushOTA-V2_1-20110405-A: "Push Over the Air".

[7]	OMA-AD-Push-V2_2-20110809-A: "Push Architecture".

[8]	WAP-168-ServiceLoad-20010731-a: "Service Loading".

[9]	3GPP TS 29.555: "Inter-5G Direct Discovery Name Management Function (DDNMF) signalling aspects; Stage 3".

[10]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[11]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[12]	3GPP TS 23.003: "Numbering, addressing and identification".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[14]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[15]	3GPP TS 38.304: "User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[16]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[17]	3GPP TS 24.555: "Proximity-services (ProSe) in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[18]	3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS); Protocol aspects; Stage 3".

[19]	3GPP TS 29.343: "Proximity-services (ProSe) function to ProSe application server aspects (PC2); Stage 3".

[20]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[21]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[xx]	3GPP TS 24.502: "Access to the 3GPP 5G Core Network (5GCN) via Non-3GPP Access Networks (N3AN); Stage 3".

* * * Next Change * * * *

### 8.2.X	5G ProSe layer 3 UE-to-network relay with N3IWF support

#### 8.2.X.1	General

As specified in clause 5.4.1.2 of 3GPP TS 23.304 [2], the 5G ProSe layer 3 UE-to-Network relay with N3IWF support shall provide the 5G ProSe layer 3 remote UE with the connection which can access to the N3IWF. In this way, the 5G ProSe layer 3 remote UE is able to select the N3IWF and access to the 5GC via the N3IWF.

The layer 3 ProSe UE-to-network relay is provisioned with the UE policies for 5G ProSe layer 3 UE-to-network relay including the Relay Service Code which corresponds to 5G ProSe layer 3 UE-to-network relay with N3IWF support as defined in 3GPP TS 24.555 [17] and provides the Relay Service Code in the PROSE PC5 DISCOVERY message for UE-to-Network Rrelay Discovery Announcement as specified in clause 8.2.1. If the 5G ProSe layer 3 remote UE intends to access 5GC via N3IWF, the 5G ProSe layer 3 remote UE:

a)	selects the Relay Service Code which corresponds to 5G ProSe layer 3 UE-to-network relay with N3IWF support in the received PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Announcement; and

b)	sends the PROSE DIRECT LINK ESTABLISHMENT REQUEST message including the Relay Service Code which corresponds to 5G ProSe layer 3 UE-to-network relay with N3IWF support to the 5G ProSe layer 3 UE-to-network relay.

The 5G ProSe layer 3 UE-to-network relay establishes the PDU session with corresponding parameters for the requested Relay Service Code as specified in clause 8.2.X.2.

The 5G ProSe layer 3 remote UE performs the N3IWF selection as specified in 8.2.X.3 once the IP address/prefix allocation is completed.

#### 8.2.X.2	5G ProSe layer 3 UE-to-network relay establishing PDU session to access N3IWF

The 5G ProSe layer 3 UE-to-network relay establishes the PDU session  based on the UE policies for 5G ProSe layer 3 UE-to-network relay Relay Service Codeas defined in 3GPP TS 24.555 [17]. The PDU session establishment is triggerd upon receipt of the PROSE DIRECT LINK ESTABLISHMENT REQUEST message including the Relay Service Code or at any time based on the 5G ProSe layer 3 UE-to-network relay local policy.

Editor's Note:	Whether explicit indication for N3IWF support to the Relay Service Code configured in the 5G ProSe Layer 3 UE-to-Network Relay will be clarified once stage-2 requirement is made.

#### 8.2.X.3	N3IWF selection by 5G ProSe layer 3 remote UE

As specified in clause 6.5.1.2.2 of 3GPP TS 23.304 [2], the 5G ProSe layer 3 remote UE selects the N3IWF using following information included in UE policies for 5G ProSe UE-to-network remote UE:

a)	N3IWF identifier configuration in the HPLMN; or

b)	5G ProSe layer 3 UE-to-network relay access node selection information.

Editor's note:	The definition of the N3IWF identifier configuration in the HPLMN and 5G ProSe layer 3 UE-to-network relay access node selection information is FFS.

The 5G ProSe layer 3 remote UE shall perform the N3IWF selection procedure as specified in clause 7.2.4.3 of 3GPP TS 24.502 [xx].

Editor's note:	It is FFS what additional procedure is needed. It depends on the definition of the N3IWF identifier configuration in the HPLMN and the access node selection information in UE policies for 5G ProSe layer 3 UE-to-network relay.

* * * End of Change * * * *