3GPP TSG-CT WG1 Meeting #133-bis-e	C1-220121

E-meeting, 17-21 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 3845 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NID IE inclusion condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2022-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Condition for NID IE inclusion in subclause 8.2.6.38 is: ----------- The UE may include this IE if the UE accesses to an SNPN using credentials from any other SNPN. ----------- However, when the UE accesses an SNPN using credentials from any other SNPN and: - the UE provides SUCI in 5GS mobile identity, there is no need to provide NID. - the UE provides 5G-GUTI in 5GS mobile identity and the 5G-GUTI was assigned by the SNPN with which the UE is registering, there is no need to provide NID. - the UE provides 5G-GUTI in 5GS mobile identity and the 5G-GUTI was assigned by the another SNPN, then the UE needs to provide NID according to 5.5.1.2.2 and 5.5.1.3.2 (see quotes below). Additionally, when the UE moves from a non-subscribed SNPN to the subscribed SNPN, the UE provides 5G-GUTI in 5GS mobile identity and the 5G-GUTI was assigned by non-subscribed SNPN, the UE also needs to provides the NID according to same text of 5.5.1.2.2 and 5.5.1.3.2 (same quotes below apply). Finally, when the UE moves from a non-subscribed SNPN to another non-subscribed SNPN using credentials from a PLMN, and the UE provides 5G-GUTI in 5GS mobile identity and the 5G-GUTI was assigned by the previously used SNPN, the UE also needs to provides the NID according to same text of 5.5.1.2.2 and 5.5.1.3.2 (same quotes below apply) Thus, the UE needs to provide the NID only when the UE provides 5G-GUTI in 5GS mobile identity and the 5G-GUTI was assigned by an SNPN other than the SNPN with which the UE is registering, and in that case providing NID is mandatory according to 5.5.1.2.2 and 5.5.1.3.2 (same quotes below apply). 5.5.1.2.2: ----------- During initial registration the UE handles the 5GS mobile identity IE in the following order: ... d) if: ... 2) the UE is registering with an SNPN, the UE holds a valid 5G-GUTI that was previously assigned, over 3GPP access or non-3GPP access, by any other SNPN, and the UE is not initiating the initial registration for onboarding services in SNPN, the UE shall indicate the 5G-GUTI in the 5GS mobile identity IE and shall additionally include the NID of the other SNPN in the NID IE; ----------- 5.5.1.3.2: ----------- The UE shall handle the 5GS mobile identity IE in the REGISTRATION REQUEST message as follows: ... b) for all other cases, if the UE holds a valid 5G-GUTI, the UE shall indicate the 5G-GUTI in the 5GS mobile identity IE. If the UE is registering with an SNPN and the valid 5G-GUTI was previously assigned by another SNPN, the UE shall additionally include the NID of the other SNPN in the NID IE. ... ----------- |  |  |  |  |  |  |  |  |
| Summary of change: |  | Condition for NID IE inclusion corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to indicate NID when moving from a non-subscribed SNPN to the subscribed SNPN. As result, the subscribed SNPN would not be able to identify the non-subscribed SNPN and would not be able to fetch the UE's context. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6.36 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** change *****

#### 8.2.6.36	NID

The UE shall include this IE if the 5G-GUTI in the 5GS mobile identity IE was assigned by an SNPN other than the SNPN with which the UE is registering.