3GPP TSG-CT WG1 Meeting #133-e	C1-216945 Rev2

- E-meeting, 11-19 November 2021

Source:	China Mobile

Title:	pCR on MSGin5G Client sending an MSGin5G message delivery status report

Spec:	3GPP TS 24.538 v0.1.0

Agenda item:	17.2.30

Document for:	Agreement

1. Introduction

This pCR proposes the the procedure of MSGin5G Client sending an MSGin5G message delivery status report.

2. Reason for Change

Contribute new text.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.538 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2;".

[3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals".

[y]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

##### 6.4.1.1.4	Sending of an MSGin5G message delivery status report

In order to send a MSGin5G message delivery status report, the MSGin5G Client shall send an CoAP POST request according to procedures specified in IETF RFC 7252 [y]. In the CoAP POST request, the MSGin5G Client:

a)	shall sets the "T" field in the CoAP header to 0, i.e. indicates that this message is the type of Confirmable, to ensure the MSGin5G message delivery status report can be received by the originator of the receiving MSGin5G message;

b)	shall include the MSGin5G Server address in an CoAP Option, e.g. if the MSGin5G Server address is a URI, include a Uri-Path Option with the value of the URI;

c)	shall set the CoAP Content-Format to "50", i.e. application/json;

d)	shall include the information elements specified in 3GPP TS 23.554[2] in the CoAP payload encoded in JSON format:

Editor’s note:	the JSON CoAP payload is to be specified in clause 7.

1)	shall include an "MSGin5G service identifier" element to indicate that this CoAP POST request message is used for MSGin5G service;

2)	shall include an "Originating UE Service ID" element set to the UE which requests the sending of the MSGin5G message delivery status report;

3)	shall include a "Recipient UE Service ID/AS Service ID" element if the recipient(s) is(are) MSGin5G UE/Non-MSGin5G UE or Application Server, this is the sender of the message that this message delivery status report is for;

4)	shall include the "Message ID" element copied from the MSGin5G message that is being acknowledged;

5)	may include the "Security credentials" which is required by the MSGin5G Server;

Editor's note:	the Security credentials element is a placeholder for SA3 security information.

6)	shall include a "Delivery Status" element to carry the delivery status description, the delivery status can be success or failure in delivery;

7)	may include a "Failure Cause" element indicates the failure reason if the delivery status is failure;

* * * Next Change * * * *

##### 6.4.1.1.5	Sending of an aggregated MSGin5G message delivery status report

The MSGin5G Client can aggregate multiple MSGin5G message delivery status reports into one single message. The MSGin5G Client shall check whether the MSGin5G message delivery status reports can be aggregated as specified in clause 6.4.1.1.3.

If the MSGin5G message delivery status reports can be aggregated, the MSGin5G Client aggregates MSGin5G message delivery status reports intended for a selected target and sends the aggregated MSGin5G message delivery status reports in a single CoAP POST request message. The sending of the CoAP POST request message shall follow the procedures specified in clause 6.4.1.1.4 with the clarifications listed below:

1)	In step d) of clause 6.4.1.1.4, the "Delivery Status" element and the "Failure Cause" element should not be included;

2)	In addition to the step d) of clause 6.4.1.1.4, the MSGin5G Client should include a "Number of individual messages" element in this message to indicate the total number of MSGin5G message delivery status reports which are aggregated into this single message.

3)	In addition to the step d) of clause 6.4.1.1.4, the MSGin5G Client should include a “"List of individual messages" element in this message. Each child element in this "List of individual messages" element contains information elements listed below:

i)	"Message ID" of the individual MSGin5G message delivery status reports which is copied from the MSGin5G message that is being acknowledged;

ii)	"Delivery Status" element; and

iii)	an optional "Failure Cause" element.