3GPP TSG-CT WG1 Meeting #133-e	C1-216946 rev1

- E-meeting, 11-19 November 2021

Source:	China Mobile, Samsung

Title:	pCR on MSGin5G Client receiving an MSGin5G message

Spec:	3GPP TS 24.538 v0.1.0

Agenda item:	17.2.30

Document for:	Agreement

1. Introduction

This pCR proposes the procedure of MSGin5G Client receiving an MSGin5G message.

2. Reason for Change

Contribute new text.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.538 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2;".

[3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals".

[y]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

##### 6.4.1.1.6	Reception of an MSGin5G message

Upon receiving an CoAP POST request containing the MSGin5G Service identifier, if the "Number of individual messages" element and "List of individual messages" element are not be included, the MSGin5G Client shall handle the CoAP POST request according to procedures specified in IETF RFC 7252 [y] with the clarifications listed below:

a)	The MSGin5G Client shall check whether a "Message is segmented" element is included in the CoAP POST request. If this element is included, the MSGin5G Client shall wait until all the segmented messages have been received by checking the "Segmentation set identifier", "Total number of message segments", "Message segment number" and "Last segment flag" elements. the MSGin5G Client shall reassembles all the segmented messages into a single MSGin5G message.

b)	The MSGin5G Client shall provide the received information in the “payload” element to the application client(s) if one or more "Application ID" element(s) is(are) included. The application client(s) is(are) indicated by the "Application ID" element(s):

1)	If the Application Client is on the other MSGin5G UE-2 for which this MSGin5G Client is acting as Gateway UE, the MSGin5G Client shall send the received information to the corresponding MSGin5G UE via MSGin5G-6 (if MSGin5G Client is supported by MSGin5G UE-2) or MSGin5G-5 reference point (if MSGin5G Client is not supported by MSGin5G UE-2) as specified in clause 6.4.2.1.1.

2)	If the Application Client is on the same MSGin5G UE with the MSGin5G Client, the MSGin5G Client shall deliver the received information to the Application Client via MSGin5G-5 reference point;

NOTE:	when the Application Client and MSGin5G Client are resided on the same MSGin5G UE, the interaction in MSGin5G-5 reference point may implementation specific and is out of scope of the present document.

c)	If a "Delivery status required" element is included in the CoAP POST request, the MSGin5G Client shall send an MSGin5G message delivery status report as specified in clause 6.4.1.1.4 or 6.4.1.1.5 with the clarifications listed below:

1)	if the message delivery status is not supported by the Application Client(s), the MSGin5G message delivery status report shall be sent after the delivery status information is received from the Application Client(s), and shall be generated based on this delivery status information;

2)	if the message delivery status is not supported by the Application Client, the MSGin5G message delivery status report shall be sent immediately by the MSGin5G Client on behalf of the Application Client(s).

* * * Next Change * * * *

##### 6.4.1.1.7	Reception of an aggregated MSGin5G message

Upon receiving an CoAP POST request containing the MSGin5G Service identifier, if a "Number of individual messages" and a "List of individual messages" are included, the MSGin5G Client determines that this message is an aggregated MSGin5G message. The MSGin5G Client shall handle the CoAP POST request according to procedures specified in IETF RFC 7252 [y] with the clarifications listed below:

a)	The MSGin5G Client shall split the received aggregated message request into multiple individual MSGin5G message, each individual MSGin5G message contains the information elements included in the child element in this "List of individual messages" element.

Editor’s note: How the MSGin5G client splits the aggregated message is FFS.

b)	The MSGin5G Client shall handle each individual MSGin5G messages according to step b) and c) specified in clause 6.4.1.1.6.