3GPP TSG-CT WG1 Meeting #133-e	C1-216895

- E-meeting, 11-19 November 2021

Source:	vivo

Title:	Use ProSe ID to represent a ProSe service

Spec:	3GPP TS 24.554 v0.5.0

Agenda item:	17.2.18

Document for:	Agreement

1. Introduction

A) Use ProSe identity to represent a ProSe service

According to stage-2 definition (see S2-2107977), ProSe identifier is used to identify the ProSe application associated with the ProSe operation in ProSe direct discovery and ProSe direct communication.

ProSe identifier: A globally unique identifier used to identify the ProSe Application associated with the ProSe operation in 5G ProSe Direct Discovery and 5G ProSe Direct Communication. In this release, the "Application ID" defined in 23.303 [3] can be used as the ProSe identifier in 5G ProSe Direct Discovery and in a consequent 5G ProSe Direct Communication.

Though it is not clear that what the ProSe identifier is if the 5G ProSe Direct Communication is not as a consequent of a 5G ProSe Direct Discovery in this release of specification, we can confirm that a ProSe service can be clearly identified by a ProSe identifier.

The following EN in TS 24.554 can be removed directly:

Editor's Note: How to identify ProSe Service(s) is up to stage-2 decision.

The following EN:

Editor’s note:	 The definition of ProSe identifier for ProSe direct communication is FFS.

can be removed with adding the following NOTE:

NOTE:	In this release of specification, the Application ID can be used as the ProSe identifier in 5G ProSe Direct Discovery and in a consequent 5G ProSe Direct Communication.

B) The coding of ProSe identity

As mentioned, in this release, the "Application ID" defined in 23.303 can be used as the ProSe identifier in 5G ProSe Direct Discovery and in a consequent 5G ProSe Direct Communication.

In TS 24.554, the Application ID is used in PC3a and the coding of Application ID reads:

#### 11.4.2.5	Application identity

This parameter is used to identify the particular application that triggers the DISCOVERY_REQUEST message. The format of the Application Identity consists of two parts:

a)	OS ID: operating system identifier. The format of the OS ID is a Universally Unique IDentifier (UUID) as specified in IETF RFC 4122 [30]; and

b)	OS App ID: a string containing the OS specific application identifier.

NOTE:	Further definition of the format of OS App ID is beyond the scope of this specification.

However, the coding of ProSe identifier is serval binary bits as quoted below:

### 11.3.3	ProSe identifier

The purpose of the ProSe identifier parameter is to carry the identifier of a ProSe application.

The ProSe identifier information element is coded as shown in figure 11.3.3.1 and table 11.3.3.1.

The ProSe identifier is a type 4 information element with a minimum length of 6 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ProSe identifier IEI |  |  |  |  |  |  |  |  | octet 1 |
| Length of ProSe identifier contents |  |  |  |  |  |  |  |  | octet 2 |
| ProSe identifier 1 |  |  |  |  |  |  |  |  | octet 3 octet 6 |
| ProSe identifier 2 |  |  |  |  |  |  |  |  | octet 7* octet 10* |
| … |  |  |  |  |  |  |  |  | octet 11* octet 4n-2* |
| ProSe identifier n |  |  |  |  |  |  |  |  | octet 4n-1* |
|  |  |  |  |  |  |  |  |  | octet 4n+2* |

Figure 11.3.3.1: ProSe identifier information element

Table 11.3.3.1: ProSe identifier information element

ProSe identifier: The ProSe identifier field contains a binary coded ProSe identifier. The format of ProSe identifier is out of scope of this specification.

Hence whether the coding of ProSe identifier should be enhanced to support the transmission of application identity over PC5.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554 v0.5.0.

* * * First Change * * * *

### 5.2.2	Precedence of 5G ProSe configuration information

The 5G ProSe configuration information for 5G ProSe direct discovery, 5G ProSe direct communication, 5G ProSe UE-to-network relay and 5G ProSe usage reporting configuration and rules can be:

a)	pre-configured in the ME;

b)	configured in the UICC;

c)	provided as a ProSeP by PCF;

d)	provided by a ProSe application server via PC1 reference point except:

1)	the authorization policy for 5G ProSe direct discovery and restricted ProSe discovery UE ID for restricted direct discovery in parameters for 5G ProSe direct discovery;

2)	the authorization policy in parameters for 5G ProSe direct communication;

3)	the following parameters for the role of a 5G ProSe UE-to-network relay UE:

i)	authorization policy for acting as a 5G ProSe layer-3 and/or layer-2 UE-to-network relay when "served by NG-RAN";

ii)	QoS mapping rules for 5G ProSe layer-3 ProSe UE-to-network relay; and

iii)	 a mapping of ProSe identifier(s) to ProSe application server address information for 5G ProSe layer-3 UE-to-network relay to relay Ethernet or Unstructured traffic from Remote UE by using IP type PDU session; or

4)	the following parameters for the role of a remote UE:

i)	authorization policy for using a 5G ProSe layer-3 and/or layer-2 UE-to-network relay; and

5)	the following parameters for the role of a 5G ProSe UE-to-network relay as well as for the role of a 5G ProSe remote UE:

i)	radio parameters for 5G ProSe relay discovery when the UE is not "served by NG-RAN "; and

ii)	radio parameters for 5G ProSe relay communication when the UE is not "served by NG-RAN"; or

e)	a combination of case a), b), c) or d) above.

The UE should not request or accept any 5G ProSe configuration information from PCF when the UE is working as a remote UE using a 5G ProSe layer-3 ProSe UE-to-network relay without involving N3IWF.

The UE shall use the 5G ProSe configuration information in the following order of decreasing precedence:

a)	the 5G ProSe configuration information provided as a ProSeP by PCF;

b)	the 5G ProSe configuration information provided by a ProSe application server via PC1 reference point;

c)	the 5G ProSe configuration information configured in the UICC; and

d)	the 5G ProSe configuration information pre-configured in the ME.

* * * Next Change * * * *

### 5.2.4	Configuration parameters for 5G ProSe direct communication over PC5 interface

The configuration parameters for 5G ProSe direct communication over PC5 interface consist of:

a)	a validity timer for the validity of the configuration parameters for 5G ProSe direct communication over PC5 interface;

b)	a list of PLMNs in which the UE is authorized to use 5G ProSe direct communication over PC5 interface when the UE is served by NG-RAN. Each entry of the list contains a PLMN ID in which the UE is authorized to use 5G ProSe direct communication over PC5 interface;

c)	an indication of whether the UE is authorized to use 5G ProSe direct communication over PC5 interface when the UE is not served by NG-RAN;

d)	the radio parameters of the 5G ProSe direct communication over PC5 interface applicable per geographical area with an indication of whether these radio parameters are "operator managed" or "non-operator managed" when the UE is not served by NG-RAN;

e)	configuration parameters for groupcast mode 5G ProSe direct communication for each application layer group, consisting of:

1)	application layer group ID;

2)	ProSe layer-2 group identifier;

3)	ProSe group IP multicast address;

4)	an indication of whether the UE is authorized to use IPv4 or IPv6;

5)	optionally, an IPv4 address to be used by the UE as a source address for a specific Group if the UE is authorized to use IPv4; and

6)	group security related content;

Editor’s note:	 Details of group security related content are FFS and will be determinated by SA3 WG.

f)	configuration parameters for privacy support, consisting of:

1)	a list of ProSe applications requiring privacy. Each entry of the list contains one or more ProSe identifiers and one or more geographical areas where the privacy is required; and

2)	a privacy timer value as specified in 3GPP TS 24.555 [17];

g)	optionally, a list of ProSe identifier to ProSe NR frequency mapping rules. Each mapping rule contains one or more ProSe identifiers and the ProSe NR frequencies with associated geographical areas;

h)	a list of ProSe identifier to destination layer-2 ID for broadcast mapping rules. Each mapping rule contains one or more ProSe identifiers and the destination layer-2 ID for broadcast;

i)	optionally, a default destination layer-2 ID for broadcast;

j)	a list of ProSe identifier to default destination layer-2 ID for unicast initial signaling mapping rules. Each mapping rule contains one or more ProSe identifiers and the default destination layer-2 ID for initial signalling to establish unicast connection;

k)	a list of ProSe identifier to PC5 QoS parameters mapping rules. The PC5 QoS parameters are specified in clause 5.7 of 3GPP TS 23.304 [2];

l)	an AS configuration, including a list of SLRB mapping rules applicable when the UE is not served by NG-RAN. Each SLRB mapping rule contains a PC5 QoS profile and an SLRB. The PC5 QoS profile contains the following parameters:

1)	the PC5 QoS profile containing a PQI;

2)	if the PQI of the PC5 QoS profile identifies a GBR QoS, the PC5 QoS profile containing a PC5 flow bit rates consisting of a guaranteed flow bit rate (GFBR) and a maximum flow bit rate (MFBR);

3)	if the PQI of the PC5 QoS profile identifies a non-GBR QoS, the PC5 QoS profile containing the PC5 link aggregated bit rate consisting of a per link aggregate maximum bit rate (PC5 LINK-AMBR);

NOTE:	PC5 link aggregated bit rate is only used for unicast mode communications over PC5 interface.

4)	the PC5 QoS profile containing a range, which is only used for groupcast mode communications over PC5 interface; and

5)	the PC5 QoS profile optionally containing the priority level, the averaging window, and the maximum data burst volume. If one or more of the priority level, the averaging window or the maximum data burst volume are not contained in the PC5 QoS profile, their default values apply;

m)	a list of 5G ProSe direct security policies. Each entry in the list contains a 5G ProSe direct security policy composed of:

1)	one or more ProSe identifiers;

2)	the signalling integrity protection policy for the ProSe identifier(s);

3)	the signalling ciphering policy for the ProSe identifier(s);

4)	 the user plane integrity protection policy for the ProSe identifier(s);

5)	the user plane ciphering policy for the ProSe identifier(s);

6)	one or more geographical areas where the 5G ProSe direct security policy applies;

Editor’s note:	 Details of 5G ProSe direct security policies related content are FFS and will be determinated by SA3 WG.

n)	a list of ProSe identifiers to default mode of communication mapping rules. Each mapping rule contains one or more ProSe identifiers and the default mode of communication (one of unicast, groupcast or broadcast); and

o)	a list of ProSe application to path preference mapping rules (i.e. PC5 preferred, Uu preferred, or no preference) as defined in clause 5.4 in 3GPP TS 24.555 [17].

NOTE:	In this release of specification, the application ID defined in 3GPP TS 23.303 [r23303] can be used as the ProSe identifier in 5G ProSe direct discovery and in a consequent 5G ProSe direct communication.

* * * Next Change * * * *

### 5.2.5	Configuration parameters for 5G ProSe UE-to-network relay

The configuration parameters for the role of a ProSe UE-to-network relay UE over PC5 reference point consist of:

a)	a validity timer for the validity of the configuration parameter for 5G ProSe UE-to-network relay over PC5 interface;

b)	a list of PLMNs in which the UE is authorised to relay traffic for 5G ProSe layer-3 remote UEs when the UE is served by NG-RAN, and in each PLMN;

c)	a list of PLMNs in which the UE is authorised to relay traffic for 5G ProSe layer-2 remote UEs when the UE is served by NG-RAN, and in each PLMN;

d)	the default destination layer-2 ID(s) for sending and receiving the initial UE-to-network relay discovery signaling;

NOTE 1:	Which default destination layer-2 ID is selected is up to UE implementation when there are more than one default destination layer-2 ID.

e)	a User info ID for the UE-to-network relay discovery;

f)	one or more relay service code(s) for the UE-to-network relay discovery, and for each relay service code:

1)	security related content for 5G ProSe relay discovery;

2)	an indication of whether the relay service code is offering 5G ProSe layer-2 or layer-3 UE-to-network relay service; and

3)	for 5G ProSe layer-3 UE-to-network relay UE, a set of PDU session parameters:

i)	PDU Session type;

ii)	optionally, DNN;

iii)	optionally, SSC Mode;

iv)	optionally, S-NSSAI; and

v)	optionally, access type preference;

Editor's note:	Details of security related content are FFS and will be determinated by SA WG3.

g)	for 5G ProSe layer-3 UE-to-network relay UE, QoS mapping rules including:

1)	a mapping between a 5QI value and a 5G ProSe PQI value over PC5 for traffic relayed over the PC5 interface;

2)	a PDB adjustment factor of the standardized PDB identified by the PQI; and

3)	optionally, the relay service code(s) associated with the QoS mapping rule;

h)	the radio parameters of the 5G ProSe UE-to-network relay discovery applicable per geographical area with an indication of whether these radio parameters are "operator managed" or "non-operator managed" when the UE is not served by NG-RAN;

i)	for 5G ProSe layer-3 UE-to-network relay UE, for Ethernet and Unstructured traffic using IP type PDU session, a list of ProSe identifier(s) to ProSe application server address mapping rule. Each mapping rule contains one or more ProSe identifier(s) and IP address/FQDN and transport layer port number; and

j)	the radio parameters of the 5G ProSe direct communication applicable per geographical area with an indication of whether these radio parameters are "operator managed" or "non-operator managed" when the UE is not served by NG-RAN.

The configuration parameters for the role of a 5G ProSe remote UE consist of:

a)	a validity timer for the validity of the configuration parameters for 5G ProSe remote UE;

b)	a list of PLMNs in which the UE is authorized to use a 5G ProSe layer-3 UE-to-network relay UE;

c)	a list of PLMNs in which the UE is authorized to use a 5G ProSe layer-2 UE-to-network relay UE;

d)	default destination layer-2 ID for sending and receiving the initial UE-to-network relay discovery signalling;

e)	a User info ID for the UE-to-network relay discovery;

f)	one or more relay service code(s) for the UE-to-network relay discovery, and for each relay service code:

1)	security related content for 5G ProSe relay discovery;

2)	an indication of whether the relay service code is offering 5G ProSe layer-2 or layer-3 UE-to-network relay service; and

3)	for 5G ProSe remote UE using 5G ProSe layer-3 UE-to-network relays, one of the following:

i)	a set of PDU session parameters for the relayed traffic without using N3IWF access:

A)	PDU Session type;

B)	optionally, DNN;

C)	optionally, SSC Mode;

D)	optionally, S-NSSAI; and

E)	optionally, access type preference; or

ii)	an indication of using N3IWF access for the relayed traffic;

Editor's note:	Details of security related content are FFS and will be determinated by SA WG3.

g)	the radio parameters of the 5G ProSe Relay Discovery applicable per geographical area with an indication of whether these radio parameters are "operator managed" or "non-operator managed" when the UE is not served by NG-RAN;

h)	the radio parameters of the 5G ProSe direct communication applicable per geographical area with an indication of whether these radio parameters are "operator managed" or "non-operator managed" when the UE is not served by NG-RAN; and

NOTE 2:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

i)	the N3IWF selection information for 5G ProSe layer-3 remote UE:

1)	N3IWF identifier configuration (either FQDN or IP address); and

2)	5G ProSe layer-3 UE-to-network relays, access node selection information consists of a prioritized list of PLMNs for N3IWF selection and an indication that the selection of an N3IWF in a PLMN should be based on Tracking Area Identity FQDN or on Operator Identifier FQDN.

* * * Next Change * * * *

### 11.3.3	ProSe identifier

The purpose of the ProSe identifier parameter is to carry the identifier of a ProSe application.

The ProSe identifier information element is coded as shown in figure 11.3.3.1 and table 11.3.3.1.

The ProSe identifier is a type 4 information element with a minimum length of 6 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ProSe identifier IEI |  |  |  |  |  |  |  |  | octet 1 |
| Length of ProSe identifier contents |  |  |  |  |  |  |  |  | octet 2 |
| ProSe identifier 1 |  |  |  |  |  |  |  |  | octet 3 octet 6 |
| ProSe identifier 2 |  |  |  |  |  |  |  |  | octet 7* octet 10* |
| … |  |  |  |  |  |  |  |  | octet 11* octet 4n-2* |
| ProSe identifier n |  |  |  |  |  |  |  |  | octet 4n-1* |
|  |  |  |  |  |  |  |  |  | octet 4n+2* |

Figure 11.3.3.1: ProSe identifier information element

Table 11.3.3.1: ProSe identifier information element

ProSe identifier: The ProSe identifier field contains two parts: - OS ID: operating system identifier. The format of the OS ID is a Universally Unique IDentifier (UUID) as specified in IETF RFC 4122 [30]; and - OS App ID: a string containing the OS specific application identifier.

NOTE: Further definition of the format of OS App ID is beyond the scope of this specification.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS); Stage 2".

[3]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[4]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[5]	3GPP TS 24.526: "UE policies for 5G System (5GS); Stage 3".

[6]	OMA-WAP-TS-PushOTA-V2_1-20110405-A: "Push Over the Air".

[7]	OMA-AD-Push-V2_2-20110809-A: "Push Architecture".

[8]	WAP-168-ServiceLoad-20010731-a: "Service Loading".

[9]	3GPP TS 29.555: "Inter-5G Direct Discovery Name Management Function (DDNMF) signalling aspects; Stage 3".

[10]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[11]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[12]	3GPP TS 23.003: "Numbering, addressing and identification".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[14]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[15]	3GPP TS 38.304: "User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[16]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[17]	3GPP TS 24.555: "Proximity-services (ProSe) in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[18]	3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS); Protocol aspects; Stage 3".

[19]	3GPP TS 29.557: "5G System; Application Function ProSe Service; Stage 3".

[20]	3GPP TS 24.007: "Mobile radio interface signalling layer-3; General aspects".

[21]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[22]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[23]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[24]	IETF RFC 4039: "Rapid Commit Option for the Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[25]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[26]	3GPP TS 24.502: "Access to the 5G System (5GS) via non-3GPP access networks; Stage 3".

[27]	ITU-T Recommendation E.212: "The international identification plan for mobile terminals and mobile users".

[28]	ISO/IEC 10118-3:2018: "IT Security techniques – Hash-functions – Part 3: Dedicated hash-functions".

[29]	W3C REC-xmlschema-2-20041028: "XML Schema Part 2: Datatypes".

[30]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[31]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[r23303]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".