- 3GPP TSG-CT WG1 Meeting #133-e	rev2 of C1-216843

- E-meeting, 11-19 November 2021

Title:	Reply LS on LCS MO-LR Procedure in 5G

Response to:	LS C1-216522/C4-214537 on LCS MO-LR Procedure in 5G

Release:	Rel-16

Work Item:	5G_eLCS

Source:	CT1

To:	CT4

Cc:	SA2

Contact person:	Xingyue (Joy) Zhou

zhou.xingyue@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TBD

# 1	Overall description

CT1 thanks CT4 for the LS on LCS MO-LR procedure in 5G and would like to provide following feedback on the three questions asked in the LS.

Q1: In 5GC-MO-LR procedure, which information (GMLC Number or H-GMLC Address) is actually provided by the UE and used by the AMF to select the (H-)GMLC?

Answer 1: It is "mlc-number" provided by the UE in MO-LR procedure. Please check the attached the agreed C1-21xxxx in CT1#133e for details. This CR removes "h-gmlc-address" from LCS-MOLR invoke in MO-LR procedure.

Q2: In 5GC-MO-LR procedure, which information (LCS Service Type or Service Identity) is actually provided by the UE and used by the GMLC to perform service privacy check?

Answer 2: In TS 24.571, it is "LCSServiceTypeID" as defined in TS 29.002 provided by the UE in MO-LR procedure. The "LCSServiceTypeID" corresponds to the term "Service Identity" in TS 23.271.

Q3: What’s the consideration of backward compatibility with early release of LCS, if SA2/CT1 decides to select new mechanism different from 4G LCS?

Answer 3: The agreed CR to include only "mlc-number" in LCS-MOLR invoke in MO-LR procedure is backward compatible with TS 24.571 v16.2.0 and earlier versions. No interoperability issue between 5G LCS and 4G LCS is expected.

# 2	Actions

To CT4 group

ACTION: 	CT1 kindly asks CT4 to take above into account.

# 3	Dates of next TSG CT WG1 meetings

CT1#133-bis-e	17th-21st January 2022	electronic meeting

CT1#134e	17th-25th February 2022	electronic meeting