3GPP TSG-CT WG1 Meeting #133-e	C1-21abcd

- E-meeting, 11-19 November 2021

Title:	LS on RSC determination in the remote UE for 5G ProSe Layer-3 UE-to-network relay scenario

Release:	Rel-17

Work Item:	5G_ProSe

Source:	3GPP CT WG1

To:	3GPP SA WG2

Cc:	-

Contact person:	Christian Herrero-Veron

Christian.Herrero at Huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: 	none

# 1	Overall description

CT1, while working in the stage 3 details of their 5G_ProSe work, has found an issue on relay service code (RSC) determination in the remote UE under the following scenario:

- The remote UE registers towards the 5GCN and receives both URSP rules and ProSe policies from the network.

- When an application starts, the remote UE performs URSP matching and may determine to transmit the application via 5G ProSe Layer-3 UE-to-network relay, based on the “5G ProSe Layer 3 UE-to-Network Relay Offload indication” within the matched rule selection descriptor (RSD) as defined in clause 6.5.4 of TS 23.304.

- The remote UE then triggers the discovery of 5G ProSe relay with the identifiers defined in clause 5.8.3 of TS 23.304, including the RSC.

In this process, CT1 wonders how the remote UE can determine the RSC for relay discovery as it seems to be absent (as no other component is allowed to be transferred together with the “5G ProSe Layer 3 UE-to-Network Relay Offload indication” in the same RSD, and thus the remote UE has no idea which RSC should be used for this application).

Additionally, another problem is found as when the relayed applications have different requirements on PDU session parameters, e.g., DNN or S-NSSAI, the relay UE may further establish relay PDU session using wrong PDU session parameters as each RSC is associated with a specific set of PDU session parameters.

So CT1 believes that, based on the current stage 2 specification, the remote UE cannot determine the right RSC for relayed traffic, and hence CT1 would like to ask the following question to SA2.

Question:	As in the above mentioned scenario, the remote UE cannot determine the right RSC for related traffic, so how can the remote UE determine the correct RSC to use?

# 2	Actions

To 3GPP SA WG2

ACTION: 	CT1 kindly asks SA2 to answer the questions above and amend SA2 specifications accordingly, if appropriate.

# 3	Dates of next TSG CT WG1 meetings

CT1#133-bis-e	17th-21st January 2022	electronic meeting

CT1#134e	17th-25th February 2022	electronic meeting