3GPP TSG-CT Meeting #97-e	C1-225809

# E-Meeting, 14th – 18th October 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 4675 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NSAG-aware random access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NRslice |  |  |  |  |  | Date: |  |  | 2022-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 4.6.2.6 describes the NAS behavior with respect to NSAG-aware cell reselection. Stage 2 has agreed on the support for NSAG-aware random access. Clause 4.6.2.6 needs to be updated to include the guidance for the NAS behavior with respect to NSAG-aware random access. We observe the following two facts: - The UE already provides NSSAI to the lower layers for every access attempt, as per clause 4.6.2.3. For inclusion mode B, the UE initiating a service request provides to the lower layers “the S-NSSAIs of the PDU sessions that have the user-plane resources requested to be re-established by the service request procedure or the S-NSSAIs of a control plane interaction triggering the service request is related”. - S-NSSAIs of the PDU sessions that have the user-plane resources requested to be re-established by the service request procedure or the S-NSSAIs of a control plane interaction triggering the service request is related Based on the above, we can define a unified approach for providing NSAG info to the lower layers that is applicable to both cell reselection and random access and re-uses the existing rules for providing NSSAI to the lower layers for access attempts. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The provides to lower layers with: - NSSAI according to the clause 4.6.2.3 for inclusion mode B; and - NSAG info Backward compatibility analysis No backward compatibility issues since the feature is new in Rel-17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How the UE selects an NSAG for random access will remain unspecified in stage 3, leading to differing UE implementations and interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 4.6.2.6	Provision of NSAG information to lower layers

NSAG information includes a list of NSAG IDs each of which is associated with:

a)	a list of S-NSSAIs, which shall be the ones included in the configured NSSAI;

b)	an NSAG area containing a list of TAIs which identify an area where the mapping between the S-NSSAI(s) in bullet a) and the NSAG ID is valid; and

c)	a priority value that is associated with each NSAG ID in the NSAG information.

If NSAG information is available, the UE NAS layer shall provide the lower layers with:

-	the most recent NSAG information;

-	NSSAI as specified in clause 4.6.2.3 for inclusion mode B; and

-	if the service request triggering the access attempt is related to more than one S-NSSAI, and these S-NSSAIs are associated with more than one NSAG, the NSAG with the highest priority.

* * * End of Changes * * * *