3GPP TSG-CT WG1 Meeting #138-e	C1-226053

# E-Meeting, 10th – 14th October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 6565 | rev | 2 | Current version: | 17.8.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex A update: support of Reason header with "STIR" protocol value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SPECTRE_Ph3 |  |  |  |  |  | Date: |  |  | 2022-09-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support of the Reason header field with protocol value set to "STIR" in provisional or final responses of the INVITE or MESSAGE request was added in the procedure part, but the corresponding profile tables in Annex A were not updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The annex A profile tables updated as follows: - in tables A.4 and A.162 added new "Identity Header Error Handling (carrying STIR codes in Reason header fields in SIP responses)" capability; - in tables A.50, A.50B, A.51A, A.62DA, A.208, A.208B, A.209A and A.218DA specified the use of the Reason header field in SIP responses carrying STIR codes. Backward compatibility analysis: The CR is backward compatible, as it updates Annex A profile tables in accordance with the requirements in the procedure part of specification for support of the Reason header field with protocol value set to "STIR" in provisional or final responses of the INVITE or MESSAGE request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support of the Reason header field with protocol value set to "STIR" in provisional or final responses is not specified in profile tables and may lead to incorrect feature implementation and possible interoperability problems since the incomplete specification makes different implementations possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.1.2, A.2.1.4.7, A.2.1.4.7A, A.2.2.2, A.2.2.4.7, A.2.2.4.7A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

### A.2.1.2	Major capabilities

Table A.4: Major capabilities

| Item | Does the implementation support |  | Reference | RFC status | Profile status |
| --- | --- | --- | --- | --- | --- |
|  | Capabilities within main protocol |  |  |  |  |
| 1 | client behaviour for registration? |  | [26] subclause 10.2 | o | c3 |
| 2 | registrar? |  | [26] subclause 10.3 | o | c4 |
| 2A | registration of multiple contacts for a single address of record |  | [26] 10.2.1.2, 16.6 | o | o |
| 2B | initiating a session? |  | [26] subclause 13 | o | o |
| 2C | initiating a session which require local and/or remote resource reservation? |  | [30] | o | c43 |
| 3 | client behaviour for INVITE requests? |  | [26] subclause 13.2 | c18 | c18 |
| 4 | server behaviour for INVITE requests? |  | [26] subclause 13.3 | c18 | c18 |
| 5 | session release? |  | [26] subclause 15.1 | c18 | c18 |
| 6 | timestamping of requests? |  | [26] subclause 8.2.6.1 | o | o |
| 7 | authentication between UA and UA? |  | [26] subclause 22.2, [287] | c34 | c34 |
| 8 | authentication between UA and registrar? |  | [26] subclause 22.2, [287] | o | c74 |
| 8A | authentication between UA and proxy? |  | [26] 20.28, 22.3, [287] | o | c75 |
| 9 | server handling of merged requests due to forking? |  | [26] 8.2.2.2 | m | m |
| 10 | client handling of multiple responses due to forking? |  | [26] 13.2.2.4 | m | m |
| 11 | insertion of date in requests and responses? |  | [26] subclause 20.17 | o | o |
| 12 | downloading of alerting information? |  | [26] subclause 20.4 | o | o |
|  | Extensions |  |  |  |  |
| 13 | SIP INFO method and package framework? |  | [25] | o | c100 |
| 13A | legacy INFO usage? |  | [25] 2, 3 | o | c90 |
| 14 | reliability of provisional responses in SIP? |  | [27] | c19 | c44 |
| 15 | the REFER method? |  | [36] | o | c33 |
| 15A | clarifications for the use of REFER with RFC6665? |  | [231] | c121 | c121 |
| 15B | explicit subscriptions for the REFER method? |  | [232] | o | o |
| 16 | integration of resource management and SIP? |  | [30] [64] | c19 | c44 |
| 17 | the SIP UPDATE method? |  | [29] | c5 | c44 |
| 19 | SIP extensions for media authorization? |  | [31] | o | c14 |
| 20 | SIP specific event notification? |  | [28] | o | c13 |
| 22 | acting as the notifier of event information? |  | [28] | c2 | c15 |
| 22A | a clarification on the use of GRUUs in the SIP event notification framework? |  | [233] | c122 | c122 |
| 23 | acting as the subscriber to event information? |  | [28] | c2 | c16 |
| 24 | session initiation protocol extension header field for registering non-adjacent contacts? |  | [35] | o | c6 |
| 25 | private extensions to the Session Initiation Protocol (SIP) for network asserted identity within trusted networks? |  | [34] | o | m |
| 26 | a privacy mechanism for the Session Initiation Protocol (SIP)? |  | [33] | o | m |
| 26A | request of privacy by the inclusion of a Privacy header indicating any privacy option? |  | [33] | c9 | c11 |
| 26B | application of privacy based on the received Privacy header? |  | [33] | c9 | n/a |
| 26C | passing on of the Privacy header transparently? |  | [33] | c9 | c12 |
| 26D | application of the privacy option "header" such that those headers which cannot be completely expunged of identifying information without the assistance of intermediaries are obscured? |  | [33] 5.1 | c10 | c27 |
| 26E | application of the privacy option "session" such that anonymization for the session(s) initiated by this message occurs? |  | [33] 5.2 | c10 | c27 |
| 26F | application of the privacy option "user" such that user level privacy functions are provided by the network? |  | [33] 5.3 | c10 | c27 |
| 26G | application of the privacy option "id" such that privacy of the network asserted identity is provided by the network? |  | [34] 7 | c10 | n/a |
| 26H | application of the privacy option "history" such that privacy of the History-Info header is provided by the network? |  | [66] 7.2 | c37 | c37 |
| 27 | a messaging mechanism for the Session Initiation Protocol (SIP)? |  | [50] | o | c7 |
| 28 | session initiation protocol extension header field for service route discovery during registration? |  | [38] | o | c17 |
| 29 | compressing the session initiation protocol? |  | [55] | o | c8 |
| 30 | private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP)? |  | [52] | o | m |
| 30A | act as first entity within the trust domain for asserted identity? |  | [34] | c96 | c97 |
| 30B | act as entity within trust network that can route outside the trust network? |  | [34] | c96 | c97 |
| 30C | act as entity passing on identity transparently independent of trust domain? |  | [34] | c96 | c98 |
| 31 | the P-Associated-URI header extension? |  | [52] 4.1, [52A] 4 | c21 | c22 |
| 32 | the P-Called-Party-ID header extension? |  | [52] 4.2, [52A] 4 | c21 | c23 |
| 33 | the P-Visited-Network-ID header extension? |  | [52] 4.3, [52A] 4 | c21 | c24 |
| 34 | the P-Access-Network-Info header extension? |  | [52] 4.4, [52A] 4, [234] 2 | c21 | c25 |
| 35 | the P-Charging-Function-Addresses header extension? |  | [52] 4.5, [52A] 4 | c21 | c26 |
| 36 | the P-Charging-Vector header extension? |  | [52] 4.6, [52A] 4 | c21 | c26 |
| 37 | security mechanism agreement for the session initiation protocol? |  | [48] | o | c20 |
| 37A | mediasec header field parameter for marking security mechanisms related to media? |  | Subclause 7.2A.7 | n/a | c101 |
| 38 | the Reason header field for the session initiation protocol? |  | [34A] | o | c68 |
| 38A | carrying Q.850 codes in Reason header fields in SIP (Session Initiation Protocol) responses? |  | [130] | o | c82 |
| 38B | the location parameter for the SIP Reason header field? |  | [255] | o | c131 |
| 38C | Identity Header Error Handling (carrying STIR codes in Reason header fields in SIP responses)? |  | [294] | o | c137 |
| 39 | an extension to the session initiation protocol for symmetric response routeing? |  | [56A] | o | c62 |
| 40 | caller preferences for the session initiation protocol? |  | [56B] | C29 | c29 |
| 40A | the proxy-directive within caller-preferences? |  | [56B] 9.1 | o.5 | o.5 |
| 40B | the cancel-directive within caller-preferences? |  | [56B] 9.1 | o.5 | o.5 |
| 40C | the fork-directive within caller-preferences? |  | [56B] 9.1 | o.5 | o.5 |
| 40D | the recurse-directive within caller-preferences? |  | [56B] 9.1 | o.5 | o.5 |
| 40E | the parallel-directive within caller-preferences? |  | [56B] 9.1 | o.5 | o.5 |
| 40F | the queue-directive within caller-preferences? |  | [56B] 9.1 | o.5 | o.5 |
| 41 | an event state publication extension to the session initiation protocol? |  | [70] | o | c30 |
| 42 | SIP session timer? |  | [58] | c19 | c19 |
| 43 | the SIP Referred-By mechanism? |  | [59] | o | c33 |
| 44 | the Session Inititation Protocol (SIP) "Replaces" header? |  | [60] | c19 | c38 (note 1) |
| 45 | the Session Inititation Protocol (SIP) "Join" header? |  | [61] | c19 | c19 (note 1) |
| 46 | the callee capabilities? |  | [62] | o | c35 |
| 47 | an extension to the session initiation protocol for request history information? |  | [66] | o | o |
| 47A | application of the "mp" optional header field parameter? |  | [66] | o | o |
| 47B | application of the "rc" optional header field parameter? |  | [66] | o | o |
| 47C | application of the "np" optional header field parameter? |  | [66] | o | o |
| 48 | Rejecting anonymous requests in the session initiation protocol? |  | [67] | o | o |
| 49 | session initiation protocol URIs for applications such as voicemail and interactive voice response? |  | [68] | o | o |
| 49A | Session Initiation Protocol (SIP) cause URI parameter for service number translation? |  | [230] | c118 | c118 |
| 50 | Session Initiation Protocol's (SIP) non-INVITE transactions? |  | [84] | m | m |
| 51 | the P-User-Database private header extension? |  | [82] 4 | o | c94 |
| 52 | a uniform resource name for services? |  | [69] | n/a | c39 |
| 53 | obtaining and using GRUUs in the Session Initiation Protocol (SIP)? |  | [93] | o | c40 (note 2) |
| 55 | the Stream Control Transmission Protocol (SCTP) as a Transport for the Session Initiation Protocol (SIP)? |  | [96] | o | c42 |
| 56 | the SIP P-Profile-Key private header extension? |  | [97] | n/a | n/a |
| 57 | managing client initiated connections in SIP? |  | [92] | o | c45 |
| 58 | indicating support for interactive connectivity establishment in SIP? |  | [102] | o | c46 |
| 59 | multiple-recipient MESSAGE requests in the session initiation protocol? |  | [104] | c47 | c48 |
| 60 | SIP location conveyance? |  | [89] | o | c49 |
| 60A | the Location Source parameter for the SIP Geolocation header field? |  | [xxx] | o | c134 |
| 61 | referring to multiple resources in the session initiation protocol? |  | [105] | c50 | c50 |
| 62 | conference establishment using request-contained lists in the session initiation protocol? |  | [106] | c51 | c52 |
| 63 | subscriptions to request-contained resource lists in the session initiation protocol? |  | [107] | c53 | c53 |
| 64 | dialstring parameter for the session initiation protocol uniform resource identifier? |  | [103] | o | c19 |
| 65 | the P-Answer-State header extension to the session initiation protocol for the open mobile alliance push to talk over cellular? |  | [111] | o | c60 |
| 66 | the SIP P-Early-Media private header extension for authorization of early media? |  | [109] 8 | o | c58 |
| 67 | number portability parameters for the 'tel' URI? |  | [112] | o | c54 |
| 67A | assert or process carrier indication? |  | [112] | o | c55 |
| 67B | local number portability? |  | [112] | o | c57 |
| 69 | extending the session initiation protocol Reason header for preemption events |  | [115] | c69 | c69 |
| 70 | communications resource priority for the session initiation protocol? |  | [116] | o | c70 |
| 70A | inclusion of MESSAGE, SUBSCRIBE, NOTIFY in communications resource priority for the session initiation protocol? |  | [116] 4.2 | c72 | c72 |
| 70B | inclusion of CANCEL, BYE, REGISTER and PUBLISH in communications resource priority for the session initiation protocol? |  | [116] 4.2 | c72 | c72 |
| 71 | addressing an amplification vulnerability in session initiation protocol forking proxies? |  | [117] | o | c87 |
| 72 | the remote application identification of applying signalling compression to SIP |  | [79] 9.1 | o | c8 |
| 73 | a session initiation protocol media feature tag for MIME application subtypes? |  | [120] | o | c59 |
| 74 | SIP extension for the identification of services? |  | [121] | o | c61 |
| 75 | a framework for consent-based communications in SIP? |  | [125] | c76 | c76 |
| 75A | a relay within the framework for consent-based communications in SIP? |  | [125] | c77 | c78 |
| 75B | a recipient within the framework for consent-based communications in SIP? |  | [125] | c80 | c79 |
| 76 | a mechanism for transporting user-to user-call control information in SIP? |  | [126] | o | c81 |
| 76A | interworking ISDN call control user information with SIP? |  | [126A] | c109 | c109 |
| 77 | The SIP P-Private-Network-Indication private-header (P-Header)? |  | [134] | o | o |
| 78 | the SIP P-Served-User private header for the 3GPP IM CN subsystem? |  | [133] 6 | o | c93 |
| 79 | the SIP P-Served-User header extension for Originating CDIV session case? |  | [239] 4 | c126 | c127 |
| 80 | marking SIP messages to be logged? |  | [140] | o | c85 |
| 81 | the 199 (Early Dialog Terminated) response code) |  | [142] | o | c86 |
| 82 | message body handling in SIP? |  | [150] | m | m |
| 83 | indication of support for keep-alive |  | [143] | o | c88 |
| 84 | SIP Interface to VoiceXML Media Services? |  | [145] | o | c89 |
| 85 | common presence and instant messaging (CPIM): message format? |  | [151] | o | c91 |
| 86 | instant message disposition notification? |  | [157] | o | c91 |
| 87 | requesting answering modes for SIP? |  | [158] | o | c60 |
| 89 | the early session disposition type for SIP? |  | [74B] | o | o |
| 91 | The Session-ID header? |  | [162] | o | c102 |
| 92 | correct transaction handling for 2xx responses to Session Initiation Protocol INVITE requests? |  | [163] | c18 | c18 |
| 93 | addressing Record-Route issues in the Session Initiation Protocol (SIP)? |  | [164] | n/a | n/a |
| 94 | essential correction for IPv6 ABNF and URI comparison in RFC3261? |  | [165] | m | m |
| 95 | suppression of session initiation protocol REFER method implicit subscription? |  | [173] | o | c99 |
| 96 | Alert-Info URNs for the Session Initiation Protocol? |  | [175] | o | o |
| 97 | multiple registrations? |  | Subclause 3.1 | n/a | c103 |
| 98 | the SIP P-Refused-URI-List private-header? |  | [183] | o | c104 |
| 99 | request authorization through dialog Identification in the session initiation protocol? |  | [184] | o | c105 |
| 100 | indication of features supported by proxy? |  | [190] | o | c106 |
| 101 | registration of bulk number contacts? |  | [191] | o | c107 |
| 102 | media control channel framework? |  | [146] | o | c108 |
| 103 | S-CSCF restoration procedures? |  | Subclause 4.14 | n/a | c110 |
| 104 | SIP overload control? |  | [198] | o | c112 |
| 104A | feedback control? |  | [199] | c113 | c113 |
| 104B | distribution of load filters? |  | [201] | c113 | c114 |
| 105 | handling of a 380 (Alternative service) response? |  | Subclauses 5.1.2A.1.1, 5.1.3.1, 5.1.6.8, and 5.2.10 | n/a | c111 |
| 106 | indication of adjacent network in the Via "received-realm" header field parameter? |  | [208] | o | c115 |
| 107 | PSAP callback indicator? |  | [209] | o | c116 |
| 108 | SIP URI parameter to indicate traffic leg? |  | [225] | o | c117 |
| 109 | PCF or PCRF based P-CSCF restoration? |  | Subclause 4.14.2 | n/a | c119 |
| 110 | UDM/HSS or HSS based P-CSCF restoration? |  | Subclause 4.14.2 | n/a | c120 |
| 111 | the Relayed-Charge header field extension? |  | Subclause 7.2.12 | n/a | c123 |
| 112 | resource sharing? |  | Subclause 4.15 | n/a | c124 |
| 113 | the Cellular-Network-Info header extension? |  | Subclause 7.2.15 | n/a | c125 |
| 114 | the Priority-Share header field extension? |  | Subclause 7.2.16 | n/a | c128 |
| 115 | the Response-Source header field extension? |  | Subclause 7.2.17 | n/a | o |
| 116 | authenticated identity management in the Session Initiation Protocol? |  | [252] | o | c129 |
| 117 | a SIP response code for unwanted calls extension? |  | [254] | o | o |
| 118 | the 3GPP PS data off extension |  | Subclause 4.17 | n/a | c130 |
| 119 | Content-ID header field in Session Initiation Protocol (SIP)? |  | [256] | o | o |
| 120 | Next-Generation Pan-European eCall emergency service? |  | [244] | o | c62 |
| 121 | the Attestation-Info header field extension? | Subclause 7.2.18 |  | n/a | c132 |
| 122 | the Origination-Id header field extension? | Subclause 7.2.19 |  | n/a | c132 |
| 123 | Dynamic services interactions? | Subclause 4.18 |  | n/a | c133 |
| 124 | the Additional-Identity header field extension? | Subclause 7.2.20 |  | n/a | c135 |
| 125 | RLOS? | Subclause 4.19 |  | n/a | c136 |
| 126 | the Priority-Verstat header field extension? | Subclause 7.2.21 |  | n/a | c72 |
| c2: IF A.4/20 THEN o.1 ELSE n/a - - SIP specific event notification extension. c3: IF A.3/1 OR A.3/4 OR A.3A/81 THEN m ELSE n/a - - UE or S-CSCF functional entity or MSC Server enhanced for ICS. c4: IF A.3/4 THEN m ELSE IF A.3/7 THEN o ELSE n/a - - S-CSCF or AS functional entity. c5: IF A.4/16 THEN m ELSE o - - integration of resource management and SIP extension. c6: IF A.3/4 OR A.3/1 OR A.3A/81 THEN m ELSE n/a. - - S-CSCF or UE or MSC Server enhanced for ICS. c7: IF A.3/1 OR A.3/4 OR A.3/7A OR A.3/7B OR A.3/7D OR A.3/9B OR A.3/13B OR A.3A/83 OR A.3A/89 THEN m ELSE n/a - - UA or S-CSCF or AS acting as terminating UA or AS acting as originating UA or AS performing 3rd party call control or IBCF (IMS-ALG), ISC gateway function (IMS-ALG), SCC application server, ATCF (UA). c8: IF A.3/1 THEN (IF (A.3B/1 OR A.3B/2 OR A.3B/3 OR A.3B/4 OR A.3B/5 OR A.3B/6 OR A.3B/7 OR A.3B/8 OR A.3B/11 OR A.3B/12 OR A.3B/13 OR A.3B/14 OR A.3B/15) THEN m ELSE o) ELSE n/a - - UE behaviour (based on P-Access-Network-Info usage). c9: IF A.4/26 THEN o.2 ELSE n/a - - a privacy mechanism for the Session Initiation Protocol (SIP). c10: IF A.4/26B THEN o.3 ELSE n/a - - application of privacy based on the received Privacy header. c11: IF A.3/1 OR A.3/6 OR A.3A/81 OR A.3A/81A OR A.3A/81B THEN o ELSE IF A.3/9B OR A.3/13B THEN m ELSE n/a - - UE or MGCF, IBCF (IMS-ALG), ISC gateway function (IMS-ALG), MSC Server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c12: IF A.3/7D OR A3A/84 OR A.3A/89 THEN m ELSE n/a - - AS performing 3rd-party call control, EATF, ATCF (UA). c13: IF A.3/1 OR A.3/2 OR A.3/4 OR A.3/9B OR A.3/11 OR A.3/12 OR A.3/13B OR A.3A/81 THEN m ELSE o - - UE or S-CSCF or IBCF (IMS-ALG) or E-CSCF or LRF or ISC gateway function (IMS-ALG) or MSC Server enhanced for ICS. c14: IF A.3/1 AND A4/2B AND (A.3B/1 OR A.3B/2 OR A.3B/3) THEN m ELSE IF A.3/2 THEN o ELSE n/a – UE and initiating sessions and GPRS IP-CAN or P-CSCF. c15: IF A.4/20 AND (A.3/4 OR A.3/9B OR A.3/11 OR A.3/13B) THEN m ELSE o – SIP specific event notification extensions and S-CSCF or IBCF (IMS-ALG) or E-CSCF or ISC gateway function (IMS-ALG). c16: IF A.4/20 AND (A.3/1 OR A.3/2 OR A.3/9B OR A.3/12 OR A.3/13B OR A.3A/81) THEN m ELSE o - - SIP specific event notification extension and UE or P-CSCF or IBCF (IMS-ALG) or MSC Server enhanced for ICS or LRF or ISC gateway function (IMS-ALG). c17: IF A.3/1 OR A.3/4 OR A.3A/81 THEN m ELSE n/a - - UE or S-CSCF or MSC Server enhanced for ICS. c18: IF A.4/2B THEN m ELSE n/a - - initiating sessions. c19: IF A.4/2B THEN o ELSE n/a - - initiating sessions. c20: IF A.3/1 AND (A.3D/1 OR A.3D/4) THEN m ELSE n/a - - UE and (IMS AKA plus IPsec ESP or SIP digest with TLS). c21: IF A.4/30 THEN o.4 ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP). c22: IF A.4/30 AND (A.3/1 OR A.3/4 OR A.3A/81) THEN m ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP) and S-CSCF or UE or MSC Server enhanced for ICS. c23: IF A.4/30 AND (A.3/1 OR A.3A/81) THEN o ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP) and UE or MSC Server enhanced for ICS. c24: IF A.4/30 AND (A.3/4 OR A.3A/81 OR A.3A/81A) THEN m ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP) and S-CSCF or MSC Server enhanced for ICS or MSC server enhanced for SRVCC using SIP interface. c25: IF A.4/30 AND (A.3A/81 OR A.3/4 OR A.3/6 OR A.3/7A OR A.3/7D OR A.3/9B OR A.3/13B OR A3A/84 OR A.3A/81A OR A.3A/81B) THEN m ELSE IF A.4/30 AND A.3/1 AND (A.3B/1OR A.3B/2 OR A.3B/3 OR A.3B/4 OR A.3B/5 OR A.3B/6 OR A.3A/7 OR A.3A/8 OR A.3B/11OR A.3B/12 OR A.3B/13 OR A.3B/14 OR A.3A/15 OR A.3B/41) THEN m ELSE IF A4/30 AND A.3/1 AND (A.3B/21 OR A.3B/22 OR A.3B/23 OR A.3B/24 OR A.3B/25 OR A.3B/26 OR A.3A/27 OR A.3A/28 OR A.3B/29 OR A.3B/30) THEN o ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP), MSC Server enhanced for ICS, S-CSCF, MGCF or AS acting as terminating UA or AS acting as third-party call controller or IBCF (IMS-ALG), ISC gateway function (IMS-ALG), UE, EATF, P-Access-Network-Info values or MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c26: IF A.4/30 AND (A.3A/81 OR (A.3/4 AND A.4/2) OR A.3/6 OR A.3/7A OR A.3/7B or A.3/7D OR A.3/9B OR A.3/13B OR A3A/84 OR A.3A/89 OR A.3A/81A OR A.3A/81B) THEN m ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP) MSC Server enhanced for ICS, S-CSCF, registrar, MGCF, AS acting as a terminating UA, or AS acting as an originating UA, or AS acting as third-party call controller, IBCF (IMS-ALG), ISC gateway function (IMS-ALG), EATF, ATCF (UA), MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c27: IF A.3/7D OR A.3/9D THEN o ELSE x - - AS performing 3rd party call control, IBCF (Privacy). c29: IF A.4/40A OR A.4/40B OR A.4/40C OR A.4/40D OR A.4/40E OR A.4/40F THEN m ELSE n/a - - support of any directives within caller preferences for the session initiation protocol. c30: IF A.3A/1 OR A.3A/2 THEN m ELSE IF A.3/1 OR A.3/11A OR A.3/2A THEN o ELSE n/a - - presence server, presence user agent, UE, AS, E-CSCF acting as UA, P-CSCF (IMS-ALG). |  |  |  |  |  |
| c33: IF A.3/9B OR A.3/12 OR A.3/13B OR A.3A/81 OR A.3A/11 OR A.3A/12 OR A.4/44 OR A.3A/81A OR A.3A/81B THEN m ELSE o - - IBCF (IMS-ALG) or LRF or ISC gateway function (IMS-ALG) or MSC Server enhanced for ICS or conference focus or conference participant or the Session Inititation Protocol (SIP) "Replaces" header, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c34: IF A.4/44 OR A.4/45 OR A.3/9B OR A.3/13 THEN m ELSE n/a - - the Session Inititation Protocol (SIP) "Replaces" header or the Session Inititation Protocol (SIP) "Join" header or IBCF (IMS-ALG) or ISC gateway function (IMS-ALG). c35: IF A.3/4 OR A.3/9B OR A.3/13B OR A.3A/82 OR A.3A/83 OR A.3A/21 OR A.3A/22 OR A3A/84 THEN m ELSE IF (A.3/1 OR A.3/6 OR A.3/7 OR A.3/8 OR A.3A/81 OR A.3A/81A OR A.3A/81B) THEN o ELSE n/a - - S-CSCF or IBCF (IMS-ALG) or ISC gateway function (IMS-ALG) functional entities or ICS user agent or SCC application server or CSI user agent or CSI application server, UE or MGCF or AS or MRFC functional entity or MSC Server enhanced for ICS or EATF or MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c37 IF A.4/47 THEN o.3 ELSE n/a - - an extension to the session initiation protocol for request history information. c38: IF A.4/2B AND (A.3A/11 OR A.3A/12 OR A.3/7D) THEN m ELSE IF A.4/2B THEN o ELSE n/a - - initiating sessions, conference focus, conference participant, AS performing 3rd party call control. c39: IF A.3/1 THEN m ELSE IF A.3/7B OR A.3/7D OR A.3/9 THEN o ELSE n/a - - UE, AS acting as an originating UA, or AS acting as third-party call controller, IBCF. c40 IF A.3/4 OR (A.3/1 AND NOT A.3C/1) OR A.3A/81 OR A.4/22 THEN m ELSE IF (A.3/7A OR A.3/7B OR A.3/7D) THEN o ELSE n/a - - S-CSCF, UE, UE performing the functions of an external attached network, MSC Server enhanced for ICS, notifier of event information, AS, AS acting as terminating UA, or redirect server, AS acting as originating UA, AS performing 3rd party call control. c42: IF A.3/1 THEN n/a ELSE o - - UE. c43: IF A.4/2B THEN o ELSE n/a - - initiating sessions. c44: IF A.4/2C THEN m ELSE o - - initiating a session which require local and/or remote resource reservation. c45: IF A.4/97 THEN m ELSE n/a - - multiple registrations. c46 IF A.3/1 OR A.3/4 THEN o ELSE n/a - - UE, S-CSCF. c47: IF A.4/27 THEN o ELSE n/a - - a messaging mechanism for the Session Initiation Protocol (SIP). c48: IF A.3A/32 AND A.4/27 THEN m ELSE IF A.4/27 THEN o ELSE n/a - - messaging list server, a messaging mechanism for the Session Initiation Protocol (SIP). c49: IF A.3/1 OR A.3/9B OR A.3/13B OR A.3A/81 OR A.3/11 OR A.3/12 OR A3A/84 THEN m ELSE o - - UE, IBCF (IMS-ALG), ISC gateway function (IMS-ALG), MSC Server enhanced for ICS, E-CSCF, LRF, EATF. c50: IF A.3A/81 OR A.3A/81A OR A.3A/81B THEN n/a ELSE IF A.4/15 THEN o ELSE n/a - - MSC Server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface, the REFER method. c51: IF A.4/2B THEN o ELSE n/a - - initiating a session. c52: IF A.3A/11 AND A.4/2B THEN m ELSE IF A.4/2B THEN o ELSE n/a - - conference focus, initiating a session. c53: IF A.3A/81 THEN n/a ELSE IF A.4/20 THEN o ELSE n/a - - MSC Server enhanced for ICS, SIP specific event notification. c54: IF A.3/1 OR A.3/6 OR A.3/7A OR A.3/7D OR A.3/9 THEN o, ELSE n/a - - UE, MGCF, AS acting as originating UA, AS performing 3rd party call control, IBCF. c55: IF A.4/67 THEN m ELSE n/a - - number portability parameters for the 'tel' URI. c57: IF A.4/67 THEN m ELSE n/a - - number portability parameters for the 'tel' URI. c58: IF A.3/9B OR A.3/13B OR A.3/6 OR A.3A/81 OR A.3A/81A OR A.3A/81B THEN m ELSE o - - IBCF (IMS-ALG), ISC gateway function (IMS-ALG), MGCF, MSC Server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c59: IF A.3/4 THEN m ELSE IF (A.3/1 OR A.3/6 OR A.3/7A OR A.3/7B OR A.3/7D OR A.3/8) THEN o ELSE n/a - - S-CSCF, UE, MGCF, AS, AS acting as terminating UA, or redirect server, AS acting as originating UA, AS performing 3rd party call control, or MRFC. c60: IF A.3/9B OR A.3/13B THEN m ELSE IF A.3/1 OR A.3/7A OR A.3/7B OR A.3/7D THEN o ELSE n/a - - IBCF (IMS-ALG), ISC gateway function (IMS-ALG), UE, AS acting as terminating UA, AS acting as originating UA, AS performing 3rd party call control. c61: IF (A.3/1 OR A.3A/81 OR A.3/6 OR A.3/7A OR A.3/7B OR A.3/7D OR A.3/8 OR A.3/9B OR A.3/13 OR A3A/84 OR A.3A/81A OR A.3A/81B) THEN o ELSE n/a - - UE, MSC Server enhanced for ICS, MGCF, AS, AS acting as terminating UA, or redirect server, AS acting as originating UA, AS performing 3rd party call control, or MRFC or IBCF (IMS-ALG), ISC gateway function (IMS-ALG), EATF, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c62: IF A.3/1 THEN o ELSE n/a - - UE. c68: IF A.3/2A OR A.3/9 OR A.4/69 OR A.3A/83 THEN m ELSE o - - P-CSCF (IMS-ALG), IBCF, extending the session initiation protocol Reason header for preemption events and Q.850 causes, SCC application server. c69: IF A.4/70 THEN o ELSE n/a - - communications resource priority for the session initiation protocol. c70: IF A.3/9B OR A.3/13B OR A.3A/102 OR A.3A/103 THEN m ELSE IF A.3/1 OR A.3/6 OR A.3/7 OR A.3/7A OR A.3/7B OR A.3/7D OR A.3A/81 OR A.3A/81A OR A.3A/81B THEN o ELSE n/a - - IBCF (IMS-ALG), ISC gateway function (IMS-ALG), MCPTT client, MCPTT server, UE, MGCF, AS, AS acting as terminating UA, or redirect server, AS acting as originating UA, AS performing 3rd party call control, MSC Server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c72: IF A.4/70 THEN o ELSE n/a - - communications resource priority for the session initiation protocol |  |  |  |  |  |
| c74: IF A.3/4 OR A.3/1 THEN o ELSE n/a. - - S-CSCF or UE. c75: IF A.3/1 THEN o ELSE n/a. - - UE. c76: IF A.4/75A OR A.4/75B THEN m ELSE n/a - - a relay within the framework for consent-based communications in SIP, a recipient within the framework for consent-based communications in SIP. c77: IF A.4/59 OR A.4/61 OR A.4/62 OR A.4/63 THEN m ELSE o - - multiple-recipient MESSAGE requests in the session initiation protocol, referring to multiple resources in the session initiation protocol, conference establishment using request-contained lists in the session initiation protocol, subscriptions to request-contained resource lists in the session initiation protocol. c78: IF (A.4/59 OR A.4/61 OR A.4/62 OR A.4/63) AND (A.3A/11 OR A.3A/31) THEN m ELSE o - - multiple-recipient MESSAGE requests in the session initiation protocol, referring to multiple resources in the session initiation protocol, conference establishment using request-contained lists in the session initiation protocol, subscriptions to request-contained resource lists in the session initiation protocol, conference focus, messaging application server. c79: IF A.3/9B OR A.3/13B OR (A.3/1 AND (A.4/2B OR A.4/15 OR A.4/20 OR A.4/27)) THEN m ELSE IF A.3/6 OR A.3/7A OR A.3/7D THEN o ELSE n/a - - IBCF (IMS-ALG), ISC gateway function (IMS-ALG), UE, initiating a session, the REFER method, SIP specific event notification, a messaging mechanism for the Session Initiation Protocol (SIP), AS acting as terminating UA, or redirect server, AS performing 3rd party call control. c80: IF A.4/2B OR A.4/15 OR A.4/20 OR A.4/27 THEN m ELSE n/a - - initiating a session, the REFER method, SIP specific event notification, a messaging mechanism for the Session Initiation Protocol (SIP). c81: IF A.3/1 OR A.3/6 OR A.3/7A OR A.3/7B OR A.3/7D THEN o ELSE IF A.3/9B OR A.3/13B THEN m ELSE n/a - - UE, MGCF, AS acting as terminating UA, or redirect server, AS acting as originating UA, AS performing 3rd party call control, IBCF (IMS-ALG), ISC gateway function (IMS-ALG). c82: IF A.3/6 OR A.3A/81 OR A.3A/81A OR A.3A/81B THEN m ELSE n/a - - MGCF, MSC server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c85: IF A.3/1 OR A.3/6 OR A.3A/81 OR A.3A/81A OR A.3A/81B OR A.3/2 OR A.3/7B THEN m ELSE n/a - - UE, MGCF, MSC Server enhanced for ICS, MSC Server enhanced for SRVCC using SIP interface, MSC Server enhanced for DRVCC using SIP interface, P-CSCF, AS acting as originating UA. c86: IF A.4/3 OR A.4/4 THEN m ELSE n/a - - client behaviour for INVITE requests, server behaviour for INVITE requests. c87: IF A.3/9B OR A.3/9C OR A.3/13B OR A.3/13C THEN m ELSE o - - IBCF (IMS-ALG), IBCF (Screening of SIP signalling), ISC gateway function (IMS-ALG), ISC gateway function (Screening of SIP signalling). c88: IF A.3/1 OR A.3/2 THEN m ELSE o - - UE, P-CSCF. c89: IF A.3/7A OR A.3/8 THEN o ELSE n/a - - AS performing 3rd party call control, MRFC. c90: IF A.4/13 OR A.3A/53 OR A.3A/54 OR A.3A/91 OR A.3A/85 OR A.3A/86 THEN m ELSE o - - SIP INFO method and package framework, advice of charge application server, advice of charge UA client, malicious communication identification application server, in-dialog overlap signalling application server, in-dialog overlap signalling UA client. c91: IF A.3A/61 OR A.3A/62 OR A.3A/63 OR A.3A/71 THEN m ELSE o - - SM-over-IP sender, SM-over-IP receiver, IP-SM-GW, IP-SM-GW. c93: IF A.3/7B OR A.3/7D OR A3A/84 THEN o ELSE n/a - - AS acting as originating UA, AS performing 3rd party call control, EATF. c94: IF A.3/4 OR A.3/7A OR A.3/7D THEN o ELSE n/a - - S-CSCF and AS acting as terminating UA or redirect server or AS performing 3rd party call control. c96: IF A.4/30 THEN o ELSE n/a - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c97: IF (A.3/9B OR A.3/9C OR A.3/13B OR A.3/13C) AND A.4/30 THEN m ELSE IF (A.3/7D OR A.3/11 OR A.3C/1) AND A.4/30 THEN o ELSE n/a - - IBCF (IMS-ALG), IBCF (Screening of SIP signalling), ISC gateway function (IMS-ALG), ISC gateway function (Screening of SIP signalling), AS performing 3rd party call control, E-CSCF, UE performing the functions of an external attached network and extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c98: IF A.3/7D OR A.3/9B OR A.3/9C OR A.3/13B OR A.3/13C OR A.3C/1 OR A3A/84 OR A.3A/89 THEN m ELSE n/a - - AS performing 3rd party call control, IBCF (IMS-ALG), IBCF (Screening of SIP signalling), ISC gateway function (IMS-ALG), ISC gateway function (Screening of SIP signalling), UE performing the functions of an external attached network, EATF, ATCF (UA). c99: IF A.4/15 AND (A.3/9B OR A.3/9C OR A.13/B OR A.13/C) THEN m ELSE IF A.4/15 THEN o ELSE n/a - - the REFER method, IBCF (IMS-ALG), IBCF (Screening of SIP signalling), ISC gateway function (IMS-ALG), ISC gateway function (Screening of SIP signalling). c100: IF A.3/6 OR A.3A/57 OR A.3A/58 OR A.3A/59 OR A.3A/60 OR A.3A/81 OR A.3A/81A OR A.3A/81B THEN m ELSE o - - MGCF, customized alerting tones application server, customized alerting tones UA client, customized ringing signal application server, customized ringing signal UA client, MSC server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c101: IF A.3D/30 OR A.3D/20A OR A.3D/20B OR A.3D/20C OR A.3D/31 THEN m ELSE n/a - - end-to-access-edge media security using SDES, end-to-access-edge media security for MSRP using TLS and certificate fingerprints, end-to-access-edge media security for BFCP using TLS and certificate fingerprints, end-to-access-edge media security for UDPTL using DTLS and certificate fingerprints, end-to-access-edge media security for RTP media using DTLS-SRTP and certificate fingerprints. c102: IF A.3A/11 OR A.3A/12 OR A.3/9 THEN m ELSE n/a - - conference focus, conference participant, IBCF. c103: IF A.3/1 THEN o ELSE IF A.3/2 OR A.3/4 THEN m ELSE n/a - - UE, P-CSCF, S-CSCF. c104: IF A.3/9B OR A.3/13B THEN m ELSE IF A.3/7A OR A.3/7B OR A.3/7D THEN o ELSE n/a - - IBCF (IMS-ALG), ISC gateway function (IMS-ALG), AS acting as terminating UA, AS acting as originating UA, AS performing 3rd party call control. |  |  |  |  |  |
| c105: IF A.3/9B OR A.3/13B OR A.3A/82 OR A.3A/83 OR A.3A/87 OR A.3A/89 THEN m ELSE o - - IBCF (IMS-ALG), ISC gateway function (IMS-ALG), ICS user agent, SCC application server, Session continuity controller UE, ATCF (UA). c106: IF A.3A/50A OR A.3A/83 OR A.3A/89 THEN m ELSE o - - Multimedia telephony application server, SCC application server, ATCF (UA). c107: IF A.3C/1 OR A.4/2 THEN o ELSE n/a - - UE performing the functions of an external attached network, registrar. c108: IF A.3/7 OR A.3/8 OR A.3/8A THEN o ELSE n/a - - AS, MRFC, MRB. c109: IF A.4/76 THEN o ELSE n/a - - a mechanism for transporting user to user call control information in SIP. c110: IF A.3/1 THEN m ELSE IF A.3/2 OR A.3/3 OR A.3/4 THEN o ELSE n/a - - UE, P-CSCF, I-CSCF, S-CSCF. c111: IF A.3/1 OR A.3/2 THEN m ELSE n/a - - UE, P-CSCF. c112: IF NOT (A.3/1 AND NOT A.3C/1) THEN o ELSE n/a - - not UE, UE performing the functions of an external attached network. c113: IF A.4/104 THEN o.7 ELSE n/a - - SIP overload control. c114: IF A.4/104 THEN IF A.3/4 OR A.3/7 OR A.3/10 THEN o.7 ELSE n/a - - SIP overload control, S-CSCF, AS, additional routeing functionality. c115: IF A.3/6 OR A.3/9 OR A.3/7 THEN o ELSE n/a - - MGCF, IBCF, AS c116: IF A.3/2A OR A.3/6 OR A.3/7 OR A.3/9 THEN o ELSE IF A.3/1 THEN x ELSE n/a - - P-CSCF (IMS-ALG), MGCF, AS, IBCF, UE. c117 IF A.3/2 OR A.3/4 OR OR A.3/9 OR A.3A/81 OR A.3A/83 OR A.3A/89 OR A.3A/81A THEN o ELSE n/a - - P-CSCF, S-CSCF, IBCF, MSC server enhanced for ICS, SCC application server, ATCF (UA), MSC server enhanced for SRVCC using SIP interface. c118: IF A.4/49 THEN o ELSE n/a - - session initiation protocol URIs for applications such as voicemail and interactive voice response (NOTE 3). c119: IF A.3/2A OR A.3/9 THEN o ELSE n/a - - P-CSCF (IMS-ALG), IBCF. c120: IF A.3/2A OR A.3/9 THEN o ELSE n/a - - P-CSCF (IMS-ALG), IBCF. c121: IF A.4/15 THEN m ELSE n/a - - the REFER method. c122: IF A.4/22 THEN m ELSE n/a - - act as a notifier. c123: IF A.4/111 AND (A.3/7A OR A.3/7B OR A.3/9A OR A.3/9B OR A.3/13A OR A.3/13B) THEN m ELSE IF A.3/4 OR A.3/7 OR A.3A/102 THEN o ELSE n/a.-.-.the Relayed-Charge header field extension, AS acting as terminating UA, or redirect server, AS acting as originating UA, IBCF (THIG), IBCF (IMS-ALG), ISC gateway function (THIG), ISC gateway function (THIG), S-CSCF, AS, transit function. c124: IF A.3/2A OR A.3/9B OR A.3/7 THEN o ELSE n/a - - P-CSCF (IMS-ALG), I-BCF (IMS-ALG), AS. c125: IF (A.3/4 OR A.3/6 OR A.3/7A OR A.3/7D OR A.3/9B OR A.3/13B OR A.3A/84 OR A.3A/89 OR A.3/2A OR A.3/8 OR A.3/11A) THEN m ELSE IF A.3/1 AND (A.3B/11 OR A.3B/12 OR A.3B/13 OR A.3B/14 OR A.3B/15) AND (A.3B/1 OR A.3B/2 OR A.3B/3 OR A.3B/4 OR A.3B/5 OR A.3B/6 OR A.3B/7 OR A.3B/8 OR A.3B/9) THEN m ELSE n/a. - - S-CSCF, MGCF, AS acting as terminating UA, AS acting as third-party call controller, IBCF (IMS-ALG), ISC gateway function (IMS-ALG), EATF, ATCF acting as UA, P-CSCF (IMS-ALG), MRFC, E-CSCF acting as UA, UE. c126: IF A.4/78 THEN o ELSE n/a - - the SIP P-Served-User private header for the 3GPP IM CN subsystem. c127: IF A.4/78 THEN m ELSE n/a - - the SIP P-Served-User private header for the 3GPP IM CN subsystem. c128: IF A.3/2A OR A.3/9B OR A.3/7 OR A.3A/103 THEN o ELSE n/a - - P-CSCF (IMS-ALG), IBCF (IMS-ALG), AS, MCPTT server. c129 IF A.3/6 OR A.3/7 OR A.3/9 OR A.3A/81 OR A.3A/81A OR A.3A/81B THEN o ELSE n/a - - MGCF, AS, IBCF, MSC Server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c130: IF A.3/1 OR A.3/7 THEN o ELSE n/a - - UE, AS, c131: IF A.3/6 OR A.3A/81 OR A.3A/81A OR A.3A/81B THEN o ELSE n/a - - MGCF, MSC server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c132: IF A.3/6 OR A.3/7 OR A.3/9 THEN o ELSE n/a - - MGCF, AS, IBCF. c133: IF A.3/2 OR A.3/7 OR A.3/9 THEN o ELSE n/a - - P-CSCF, AS, IBCF. c134 IF A.4/60 THEN o ELSE n/a - - the Geolocation header field c135: IF A.3/1 OR A.3/2 OR A.3/7 OR A.3/9 THEN o ELSE n/a - - UE, P-CSCF, AS, IBCF. c136: IF A.3/1 THEN o ELSE n/a - - UE. c137: IF A.3/7 OR A.3/9 THEN o ELSE n/a - - AS, IBCF. o.1: At least one of these capabilities is supported. o.2: At least one of these capabilities is supported. o.3: At least one of these capabilities is supported. o.4: At least one of these capabilities is supported. o.5: At least one of these capabilities is supported. o.6: It is mandatory to support at least one of these items. o.7: At least one of these capabilities is supported. |  |  |  |  |  |
| NOTE 1: An AS acting as a proxy may be outside the trust domain, and therefore not able to support the capability for that reason; in this case it is perfectly reasonable for the header to be passed on transparently, as specified in the PDU parts of the profile. NOTE 2: If a UE is unable to become engaged in a service that potentially requires the ability to identify and interact with a specific UE even when multiple UEs share the same single Public User Identity then the UE support can be "o" instead of "m". Examples include telemetry applications, where point-to-point communication is desired between two users. NOTE 3: AS performing a service number translation (eg. Freephone) |  |  |  |  |  |

Prerequisite A.4/20 - - SIP specific event notification

Table A.4A: Supported event packages

| Item | Does the implementation support | Subscriber |  |  | Notifier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | reg event package? | [43] | c1 | c3 | [43] | c2 | c4 |
| 1A | reg event package extension for GRUUs? | [94] | c1 | c25 | [94] | c2 | c4 |
| 2 | refer package? | [36] 3 | c13 | c13 | [36] 3 | c13 | c13 |
| 3 | presence package? | [74] 6 | c1 | c5 | [74] 6 | c2 | c6 |
| 4 | eventlist with underlying presence package? | [75], [74] 6 | c1 | c7 | [75], [74] 6 | c2 | c8 |
| 5 | presence.winfo template-package? | [72] 4 | c1 | c9 | [72] 4 | c2 | c10 |
| 6 | xcap-diff package? | [77] 4 | c1 | c11 | [77] 4 | c2 | c12 |
| 7 | conference package? | [78] 3 | c1 | c21 | [78] 3 | c1 | c22 |
| 8 | message-summary package? | [65] | c1 | c23 | [65] 3 | c2 | c24 |
| 9 | poc-settings package? | [110] | c1 | c26 | [110] | c2 | c27 |
| 11 | dialog event package? | [171] | c1 | c14 | [171] | c2 | c15 |
| 12 | load-control package? | [201] | c29 | c30 | [201] | c29 | c31 |
| c1: IF A.4/23 THEN o ELSE n/a - - acting as the subscriber to event information. c2: IF A.4/22 THEN o ELSE n/a - - acting as the notifier of event information. c3: IF A.3/1 OR A.3A/81 OR A.3/2 THEN m ELSE IF A.3/7 THEN o ELSE n/a - - UE, MSC Server enhanced for ICS, P-CSCF, AS. c4: IF A.3/4 THEN m ELSE IF A.3C/1 THEN o ELSE n/a - - S-CSCF, UE performing the functions of an external attached network. c5: IF A.3A/3 OR A.3A/4 THEN m ELSE IF A.4/23 OR A.3/12 THEN o ELSE n/a - - resource list server or watcher, acting as the subscriber to event information, LRF. c6: IF A.3A/1 THEN m ELSE IF A.4/22 OR A.3/11A THEN o ELSE n/a - - presence server, acting as the notifier of event information, E-CSCF acting as UA. c7: IF A.3A/4 THEN m ELSE IF A.4/23 THEN o ELSE n/a - - watcher, acting as the subscriber to event information. c8: IF A.3A/3 THEN m ELSE IF A.4/22 THEN o ELSE n/a - - resource list server, acting as the notifier of event information. c9: IF A.3A/2 THEN m ELSE IF A.4/23 THEN o ELSE n/a - - presence user agent, acting as the subscriber to event information. c10: IF A.3A/1 THEN m ELSE IF A.4/22 THEN o ELSE n/a - - presence server, acting as the notifier of event information. c11: IF A.3A/2 OR A.3A/4 OR A.3A/56 THEN o ELSE IF A.4/23 THEN o ELSE n/a - - presence user agent or watcher or Ut reference point XCAP client for supplementary services, acting as the subscriber to event information. c12: IF A.3A/1 OR A.3A/3 OR A.3A/55 THEN m ELSE IF A.4/22 THEN o ELSE n/a - - presence server or resource list server or Ut reference point XCAP server for supplementary services, acting as the notifier of event information. c13: IF A.4/15 THEN m ELSE n/a - - the REFER method. c14: IF A.3/12 OR A.3A/87 THEN m ELSE IF A.3/1 OR A.3/7B OR A.3/7D THEN o ELSE n/a - - LRF, session continuity controller UE, UE, AS acting as originating UA, AS performing 3rd party call control. c15: IF A.3/11 OR A.3A/83 THEN m ELSE IF A.3/1 OR A.3/7A OR A.3/7D THEN o ELSE n/a - - E-CSCF, SCC application server, UE, AS acting as terminating UA, or redirect server, AS performing 3rd party call control. c21: IF A.3A/12 THEN m ELSE IF A.4/23 THEN o ELSE n/a - - conference participant or acting as the subscriber to event information. c22: IF A.3A/11 THEN m ELSE IF A.4/22 THEN o ELSE n/a - - conference focus or acting as the notifier of event information. c23: IF A.3A/52 THEN m ELSE (A.3/1 OR A.3/7A OR A.3/7B) AND A.4/23 THEN o ELSE n/a - - message waiting indication subscriber UA, UE, AS acting as terminating UA, or redirect server, AS acting as originating UA all as subscriber of event information. c24: IF A.3A/52 THEN m ELSE (A.3/1 OR A.3/7A OR A.3/7B) AND A.4/22 THEN o ELSE n/a - - message waiting indication notifier UA, UE, AS acting as terminating UA, or redirect server, AS acting as originating UA all as notifier of event information. c25: IF A.4A/1 THEN (IF A.3/1 AND A.4/53 THEN m ELSE o) ELSE n/a - - reg event package, UE, reg event package extension for GRUUs. c26: IF (A.3/7B OR A.3/1) AND (A.4/23 OR A.4/41) THEN o ELSE n/a - - AS acting as originating UA, UE, acting as the subscriber to event information, an event state publication extension to the session initiation protocol. c27: IF (A.4/22 OR A.4/41) AND A.3/1 THEN o ELSE n/a - - UE, acting as the notifier of event information, an event state publication extension to the session initiation protocol. c28: IF A.3/1 OR A.3A/81 OR A.3/2 OR A.3/7B THEN m ELSE n/a - - UE, MSC Server enhanced for ICS, P-CSCF, AS acting as originating UA. c29: IF A.4/104B THEN m ELSE n/a - - distribution of load filters. c30: IF A.4/104B THEN IF A.3/4 OR A.3/7 OR A.3/9 THEN m ELSE n/a - - distribution of load filters. S-CSCF, IBCF, AS. c31: IF A.4/104B THEN If A.3/7 THEN m ELSE n/a - - distribution of load filters, AS. |  |  |  |  |  |  |  |

Prerequisite A.4/13 - - SIP INFO method and package framework.

Table A.4B: Supported info packages

| Item | Does the implementation support | Sender |  |  | Receiver |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | DTMF info package? | 7.12.1 | n/a | c1 | 7.12.1 | n/a | c1 |
| 2 | g.3gpp.mid-call? | [8M] | n/a | c2 | [8M] | n/a | c3 |
| 3 | g.3gpp.ussd? | [8W] | n/a | c4 | [8W] | n/a | c4 |
| 4 | g.3gpp.current-location-discovery info package ? | subclause 7.12.2.1 | n/a | c5 | subclause 7.12.2.1 | n/a | c6 |
| 5 | EmergencyCallData.eCall.MSD Info-Package | [244] 14.9 | m | c7 | [244] 14.9 | m | c7 |
| c1: IF A.3/6 OR A.3A/57 OR A.3A/58 OR A.3A/59 OR A.3A/60 THEN m ELSE o - - MGCF, customized alerting tones application server, customized alerting tones UA client, customized ringing signal application server, customized ringing signal UA client. c2: IF A.3A/83 THEN o ELSE n/a - - SCC application server. c3: IF A.3A/81 OR A.3A/81B THEN o ELSE n/a - - MSC server enhanced for ICS, MSC server enhanced for DRVCC using SIP interface. c4: IF A.3A/92 OR A.3A/93 THEN m ELSE n/a - - USSI UE, USSI AS. c5: IF A.3/11A OR A.3/2A THEN o ELSE n/a - - E-CSCF acting as UA, P-CSCF (IMS-ALG). c6: IF (A.3/1 AND (A.3B/11 OR A.3B/12 OR A.3B/13 OR A.3B/14 OR A.3B/15)) OR A.3/2A THEN o ELSE n/a - - UE, IEEE-802.11, IEEE-802.11a, IEEE-802.11b, IEEE-802.11g, IEEE-802.11n, IEEE-802.11ac, P-CSCF (IMS-ALG). c7: IF (A.3/1 AND A.4/120) THEN m ELSE n/a - - UE, Next-Generation Pan-European eCall emergency service. |  |  |  |  |  |  |  |

Table A.4C: Supported media control packages

| Item | Does the implementation support | Sender |  |  | Receiver |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | msc-ivr/1.0 | [147] |  | c1 | [147] |  | c2 |
| 2 | msc-mixer/1.0 | [148] |  | c1 | [148] |  | c2 |
| 3 | mrb-publish/1.0 | [192] |  | c3 | [192] |  | c4 |
| c1: IF A.3/7D THEN o ELSE n/a - - AS performing 3rd party call control. c2: IF A.3/8 THEN o ELSE n/a - - MRFC. c3: IF A.3/8 THEN o ELSE n/a - - MRFC. c4: IF A.3/8A THEN o ELSE n/a - - MRB. |  |  |  |  |  |  |  |

# *** Next Change ***

#### A.2.1.4.7	INVITE method

Prerequisite A.5/8 - - INVITE request

Table A.46: Supported header fields within the INVITE request

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept | [26] 20.1 | o | c47 | [26] 20.1 | m | m |
| 1A | Accept-Contact | [56B] 9.2 | c24 | c24 | [56B] 9.2 | c32 | c32 |
| 2 | Accept-Encoding | [26] 20.2 | o | o | [26] 20.2 | m | m |
| 3 | Accept-Language | [26] 20.3 | o | o | [26] 20.3 | m | m |
| 3A | Additional-Identity | 7.2.20 | n/a | c75 | 7.2.20 | n/a | c76 |
| 4 | Alert-Info | [26] 20.4 | o | o | [26] 20.4 | c1 | c1 |
| 5 | Allow | [26] 20.5, [26] 5.1 | o (note 1) | o | [26] 20.5, [26] 5.1 | m | m |
| 6 | Allow-Events | [28] 8.2.2 | c2 | c2 | [28] 8.2.2 | c53 | c53 |
| 6A | Attestation-Info | 7.2.18 | n/a | c71 | 7.2.18 | n/a | c71 |
| 7 | Answer-Mode | [158] | c49 | c49 | [158] | c50 | c50 |
| 8 | Authorization | [26] 20.7 | c3 | c3 | [26] 20.7 | c3 | c3 |
| 9 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 10 | Call-Info | [26] 20.9 | o | o | [26] 20.9 | o | o |
| 10A | Cellular-Network-Info | 7.2.15 | n/a | c63 | 7.2.15 | n/a | c64 |
| 11 | Contact | [26] 20.10 | m | m | [26] 20.10 | m | m |
| 12 | Content-Disposition | [26] 20.11 | o | o | [26] 20.11 | m | m |
| 13 | Content-Encoding | [26] 20.12 | o | o | [26] 20.12 | m | m |
| 13A | Content-ID | [256] 3.2 | o | c69 | [256] 3.2 | m | c70 |
| 14 | Content-Language | [26] 20.13 | o | o | [26] 20.13 | m | m |
| 15 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 16 | Content-Type | [26] 20.15 | m | m | [26] 20.15 | m | m |
| 17 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 18 | Date | [26] 20.17 | c4 | c4 | [26] 20.17 | m | m |
| 19 | Expires | [26] 20.19 | o | o | [26] 20.19 | o | o |
| 19A | Feature-Caps | [190] | c59 | c59 | [190] | c58 | c58 |
| 20 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 20A | Geolocation | [89] 4.1 | c33 | c33 | [89] 4.1 | c33 | c33 |
| 20B | Geolocation-Routing | [89] 4.2 | c33 | c33 | [89] 4.2 | c33 | c33 |
| 20C | History-Info | [66] 4.1 | c31 | c31 | [66] 4.1 | c31 | c31 |
| 20D | Identity | [252] 4 | c68 | c68 | [252] 4 | c68 | c68 |
| 21 | In-Reply-To | [26] 20.21 | o | o | [26] 20.21 | o | o |
| 21A | Join | [61] 7.1 | c30 | c30 | [61] 7.1 | c30 | c30 |
| 21B | Max-Breadth | [117] 5.8 | n/a | c45 | [117] 5.8 | c46 | c46 |
| 22 | Max-Forwards | [26] 20.22 | m | m | [26] 20.22 | n/a | c52 |
| 23 | MIME-Version | [26] 20.24 | o | o | [26] 20.24 | m | m |
| 23A | Min-SE | [58] 5 | c26 | c26 | [58] 5 | c25 | c25 |
| 24 | Organization | [26] 20.25 | o | o | [26] 20.25 | o | o |
| 24AA | Origination-Id | 7.2.19 | n/a | c72 | 7.2.19 | n/a | c72 |
| 24A | P-Access-Network-Info | [52] 4.4, [234] 2 | c15 | c16 | [52] 4.4, [234] 2 | c15 | c17 |
| 24B | P-Asserted-Identity | [34] 9.1 | n/a | c65 | [34] 9.1 | c7 | c7 |
| 24C | P-Asserted-Service | [121] 4.1 | n/a | c67 | [121] 4.1 | c38 | c38 |
| 24D | P-Called-Party-ID | [52] 4.2 | x | x | [52] 4.2 | c13 | c13 |
| 24E | P-Charging-Function-Addresses | [52] 4.5 | c20 | c21 | [52] 4.5 | c20 | c21 |
| 24F | P-Charging-Vector | [52] 4.6 | c18 | c19 | [52] 4.6 | c18 | c19 |
| 24H | P-Early-Media | [109] 8 | c34 | c34 | [109] 8 | c34 | c34 |
| 25 | P-Media-Authorization | [31] 5.1 | n/a | n/a | [31] 5.1 | c11 | c12 |
| 25A | P-Preferred-Identity | [34] 9.2 | c7 | c5 | [34] 9.2 | n/a | n/a |
| 25B | P-Preferred-Service | [121] 4.2 | c37 | c36 | [121] 4.2 | n/a | n/a |
| 25C | P-Private-Network-Indication | [134] | c42 | c42 | [134] | c42 | c42 |
| 25D | P-Profile-Key | [97] 5 | n/a | n/a | [97] 5 | n/a | n/a |
| 25E | P-Served-User | [133] 6 | c51 | c51 | [133] 6 | c51 | c51 |
| 25F | P-User-Database | [82] 4 | n/a | n/a | [82] 4 | n/a | n/a |
| 25G | P-Visited-Network-ID | [52] 4.3 | x (note 3) | x | [52] 4.3 | c14 | n/a |
| 26 | Priority | [26] 20.26 | o | o | [26] 20.26 | o | o |
| 26AA | Priority-Share | Subclause 7.2.16 | n/a | c66 | Subclause 7.2.16 | n/a | c66 |
| 26AB | Priority-Verstat | Subclause 7.2.21 | n/a | c77 | Subclause 7.2.21 | n/a | c78 |
| 26A | Privacy | [33] 4.2 | c9 | c9 | [33] 4.2 | c9 | c9 |
| 26B | Priv-Answer-Mode | [158] | c49 | c49 | [158] | c50 | c50 |
| 27 | Proxy-Authorization | [26] 20.28 | c6 | c6 | [26] 20.28 | n/a | n/a |
| 28 | Proxy-Require | [26] 20.29 | o (note 2) | o (note 2) | [26] 20.29 | n/a | n/a |
| 28A | Reason | [34A] 2 | c8 | c8 | [34A] 2 | c8 | c55 |
| 29 | Record-Route | [26] 20.30 | n/a | c52 | [26] 20.30 | m | m |
| 29A | Recv-Info | [25] 5.2.3 | c48 | c48 | [25] 5.2.3 | c48 | c48 |
| 30 | Referred-By | [59] 3 | c27 | c27 | [59] 3 | c28 | c28 |
| 31 | Reject-Contact | [56B] 9.2 | c24 | c24 | [56B] 9.2 | c32 | c32 |
| 31A | Relayed-Charge | 7.2.12 | n/a | c61 | 7.2.12 | n/a | c61 |
| 31B | Replaces | [60] 6.1 | c29 | c29 | [60] 6.1 | c29 | c29 |
| 31C | Reply-To | [26] 20.31 | o | o | [26] 20.31 | o | o |
| 31D | Request-Disposition | [56B] 9.1 | c24 | c24 | [56B] 9.1 | c32 | c32 |
| 32 | Require | [26] 20.32 | m | m | [26] 20.32 | m | m |
| 32A | Resource-Priority | [116] 3.1 | c35 | c35 | [116] 3.1 | c35 | c35 |
| 32B | Restoration-Info | Subclause 7.2.11 | n/a | n/a | Subclause 7.2.11 | n/a | c60 |
| 32C | Resource-Share | Subclause 7.2.13 | n/a | c62 | Subclause 7.2.13 | n/a | c62 |
| 33 | Route | [26] 20.34 | m | m | [26] 20.34 | n/a | c52 |
| 33A | Security-Client | [48] 2.3.1 | c22 | c22 | [48] 2.3.1 | n/a | n/a |
| 33B | Security-Verify | [48] 2.3.1 | c23 | c23 | [48] 2.3.1 | n/a | n/a |
| 33DA | Service-Interact-Info | Subclause 7.2.14 | n/a | c73 | Subclause 7.2.14 | n/a | c74 |
| 33D | Session-Expires | [58] 4 | c25 | c25 | [58] 4 | c25 | c25 |
| 33E | Session-ID | [162] | o | c54 | [162] | o | c54 |
| 34 | Subject | [26] 20.36 | o | o | [26] 20.36 | o | o |
| 35 | Supported | [26] 20.37 | m | m | [26] 20.37 | m | m |
| 35A | Target-Dialog | [184] 7 | c56 | c56 | [184] 7 | c57 | c57 |
| 36 | Timestamp | [26] 20.38 | c10 | c10 | [26] 20.38 | m | m |
| 37 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 37A | Trigger-Consent | [125] 5.11.2 | c39 | c39 | [125] 5.11.2 | c40 | c40 |
| 38 | User-Agent | [26] 20.41 | o | o | [26] 20.41 | o | o |
| 38A | User-to-User | [126] 7 | c41 | c41 | [126] 7 | c41 | c41 |
| 39 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| c1: IF A.4/12 THEN m ELSE n/a - - downloading of alerting information. c2: IF A.4/22 THEN m ELSE n/a - - acting as the notifier of event information. c3: IF A.4/7 THEN m ELSE n/a - - authentication between UA and UA. c4: IF A.4/11 THEN o ELSE n/a - - insertion of date in requests and responses. c5: IF A.3/1 AND A.4/25 THEN o ELSE n/a - - UE and private extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c6: IF A.4/8A THEN m ELSE n/a - - authentication between UA and proxy. c7: IF A.4/25 THEN o ELSE n/a - - private extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c8: IF A.4/38 THEN o ELSE n/a - - the Reason header field for the session initiation protocol. c9: IF A.4/26 THEN o ELSE n/a - - a privacy mechanism for the Session Initiation Protocol (SIP). c10: IF A.4/6 THEN o ELSE n/a - - timestamping of requests. c11: IF A.4/19 THEN m ELSE n/a - - SIP extensions for media authorization. c12: IF A.3/1 AND A.4/19 THEN m ELSE n/a - - UE, SIP extensions for media authorization. c13: IF A.4/32 THEN o ELSE n/a - - the P-Called-Party-ID extension. c14: IF A.4/33 THEN o ELSE n/a - - the P-Visited-Network-ID extension. c15: IF A.4/34 THEN o ELSE n/a - - the P-Access-Network-Info header extension. c16: IF A.4/34 AND (A.3/1 OR A.3/2A OR A.3/7 OR A.3A/81 OR A.3A/81A OR A.3A/81B OR A.3/6) THEN m ELSE n/a - - the P-Access-Network-Info header extension and UE, P-CSCF (IMS-ALG), the AS, the MSC server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface or MGCF. c17: IF A.4/34 AND (A.3/2A OR A.3A/81 OR A.3/7A OR A.3/7D OR A3A/84 OR A.3/6) THEN m ELSE n/a - - the P-Access-Network-Info header extension and P-CSCF (IMS-ALG), the MSC server enhanced for ICS, AS acting as terminating UA, AS acting as third-party call controller, EATF or MGCF. c18: IF A.4/36 THEN o ELSE n/a - - the P-Charging-Vector header extension. c19: IF A.4/36 THEN m ELSE n/a - - the P-Charging-Vector header extension. c20: IF A.4/35 THEN o ELSE n/a - - the P-Charging-Function-Addresses header extension. c21: IF A.4/35 THEN m ELSE n/a - - the P-Charging-Function-Addresses header extension. c22: IF A.4/37 OR A.4/37A THEN o ELSE n/a - - security mechanism agreement for the session initiation protocol or mediasec header field parameter for marking security mechanisms related to media (note 4). c23: IF A.4/37 OR A.4/37A THEN m ELSE n/a - - security mechanism agreement for the session initiation protocol or mediasec header field parameter for marking security mechanisms related to media. c24: IF A.4/40 THEN o ELSE n/a - - caller preferences for the session initiation protocol. c25: IF A.4/42 THEN m ELSE n/a - - the SIP session timer. c26: IF A.4/42 THEN o ELSE n/a - - the SIP session timer. c27: IF A.4/43 THEN m ELSE n/a - - the SIP Referred-By mechanism. c28: IF A.4/43 THEN o ELSE n/a - - the SIP Referred-By mechanism. c29: IF A.4/44 THEN m ELSE n/a - - the Session Inititation Protocol (SIP) "Replaces" header. c30: IF A.4/45 THEN m ELSE n/a - - the Session Inititation Protocol (SIP) "Join" header. c31: IF A.4/47 THEN m ELSE n/a - - an extension to the session initiation protocol for request history information. c32: IF A.4/40 THEN m ELSE n/a - - caller preferences for the session initiation protocol. c33: IF A.4/60 THEN m ELSE n/a - - SIP location conveyance. c34: IF A.4/66 THEN m ELSE n/a - - The SIP P-Early-Media private header extension for authorization of early media. c35: IF A.4/70 THEN m ELSE n/a - - communications resource priority for the session initiation protocol. c36: IF (A.3/1 OR A.3A/81 OR A.3A/81A OR A.3A/81B) AND A.4/74 THEN o ELSE n/a - - UE, MSC Server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface and SIP extension for the identification of services. c37: IF A.4/74 THEN o ELSE n/a - - SIP extension for the identification of services. c38: IF A.4/74 THEN m ELSE n/a - - SIP extension for the identification of services. c39: IF A.4/75A THEN m ELSE n/a - - a relay within the framework for consent-based communications in SIP. c40: IF A.4/75B THEN m ELSE n/a - - a recipient within the framework for consent-based communications in SIP. c41: IF A.4/76 THEN o ELSE n/a - - transporting user to user information for call centers using SIP. c42: IF A.4/77 THEN m ELSE n/a - - the SIP P-Private-Network-Indication private-header (P-Header). c45: IF A.4/71 AND (A.3/9B OR A.3/9C OR A.3/13B OR A.3/13C) THEN m ELSE IF A.3/1 AND NOT A.3C/1 THEN n/a ELSE o - - addressing an amplification vulnerability in session initiation protocol forking proxies, IBCF (IMS-ALG), IBCF (Screening of SIP signalling), ISC gateway function (IMS-ALG), ISC gateway function (Screening of SIP signalling), UE, UE performing the functions of an external attached network. c46: IF A.4/71 THEN m ELSE n/a - - addressing an amplification vulnerability in session initiation protocol forking proxies. c47: IF A.3/1 AND A.4/2B THEN m ELSE o - - UE and initiating a session. c48: IF A.4/13 THEN m ELSE IF A.4/13A THEN m ELSE n/a - - SIP INFO method and package framework, legacy INFO usage. |  |  |  |  |  |  |  |
| c49: IF A.4/87 THEN o ELSE n/a - - requesting answering modes for SIP. c50: IF A.4/87 THEN m ELSE n/a - - requesting answering modes for SIP. c51: IF A.4/78 THEN m ELSE n/a - - the SIP P-Served-User private header. c52: IF A.3/1 AND NOT A.3C/1 THEN n/a ELSE o - - UE, UE performing the functions of an external attached network. c53: IF A.4/23 THEN m ELSE n/a - - acting as the subscriber to event information. c54: IF A.4/91 THEN m ELSE n/a - - the Session-ID header. c55: IF A.4/38 THEN IF A.3A/83 THEN m ELSE o ELSE n/a - - the Reason header field for the session initiation protocol, SCC application server. c56: IF A.4/99 THEN o ELSE n/a - - request authorization through dialog Identification in the session initiation protocol. c57: IF A.4/99 THEN m ELSE n/a - - request authorization through dialog Identification in the session initiation protocol. c58: IF A.4/100 THEN m ELSE n/a - - indication of features supported by proxy. c59: IF A.4/100 AND A.3/1 AND NOT A.3C/1 THEN n/a ELSE IF A.4/100 THEN m ELSE n/a - - indication of features supported by proxy, UE, UE performing the functions of an external attached network. c60: IF A. 4/109 THEN o ELSE n/a - - PCRF based P-CSCF restoration. c61: IF A.4/111 THEN m ELSE n/a - - the Relayed-Charge header field extension. c62: IF A.4/112 THEN o ELSE n/a - - resource sharing. c63: IF A.4/113 AND (A.3/1 OR A.3/2A OR A.3/7) THEN m ELSE n/a - - the Cellular-Network-Info header extension and UE, P-CSCF (IMS-ALG) or the AS. c64: IF A.4/113 AND (A.3/2A OR A.3/7A OR A.3/7D OR A3A/84) THEN m ELSE IF A.4/113 AND A.3/6 THEN o ELSE n/a - - the Cellular-Network-Info header extension and P-CSCF (IMS-ALG), AS acting as terminating UA or AS acting as third-party call controller, EATF or MGCF. c65: IF A.4/25 AND (A.3/6 OR A.3/7B OR A.3/8 OR A.3A/81 OR A.3A/81A OR A.3A/81B OR A.3A/83 OR A.3A/89) THEN o ELSE n/a - - private extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks and MGCF, AS acting as originating UA, MRFC, MSC Server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface, SCC application server, ATCF (UA). c66: IF A.4/114 THEN o ELSE n/a - - priority sharing. c67: IF A.4/74 AND A.3/7B THEN o ELSE n/a - - SIP extension for the identification of services and AS acting as originating UA. c68: IF A.4/116 AND (A.3/7 OR A.3/9) THEN m ELSE n/a - - authenticated identity management in the Session Initiation Protocol, AS, IBCF. c69: IF A.4/119 THEN o ELSE n/a - - Content-ID header field in Session Initiation Protocol (SIP). c70: IF A.4/119 THEN m ELSE n/a - - Content-ID header field in Session Initiation Protocol (SIP). c71: IF A.4/121 AND (A.3/6 OR A.3/7 OR A.3/9) THEN m ELSE n/a - - the Attestation-Info header field extension, MGCF, AS, IBCF. c72: IF A.4/122 AND (A.3/6 OR A.3/7 OR A.3/9) THEN m ELSE n/a - - the Origination-Id header field extension, MGCF, AS, IBCF. c73: IF A.4/123 AND (A.3/7 OR A.3/9) THEN m ELSE n/a - - Dynamic services interactions, AS, IBCF. c74: IF A.4/123 AND (A.3/2 OR A.3/7 OR A.3/9) THEN m ELSE n/a - - Dynamic services interactions, P-CSCF, AS, IBCF. c75: IF A.4/124 THEN o ELSE n/a - - the Additional-Identity header field extension. c76: IF A.4/124 THEN m ELSE n/a - - the Additional-Identity header field extension. c77: IF A.4/70 AND A.4/126 AND A.3/9 THEN m ELSE n/a - - communications resource priority for the session initiation protocol, the Priority-Verstat header field extension, IBCF. c78: IF A.4/70 AND A.4/126 THEN o ELSE n/a - - communications resource priority for the session initiation protocol, the Priority-Verstat header field extension. o.1: At least one of these shall be supported. |  |  |  |  |  |  |  |
| NOTE 1: RFC 3261 [26] gives the status of this header as SHOULD rather than OPTIONAL. NOTE 2: No distinction has been made in these tables between first use of a request on a From/To/Call-ID combination, and the usage in a subsequent one. Therefore the use of "o" etc. above has been included from a viewpoint of first usage. NOTE 3: The strength of this requirement in RFC 7315 [52] is SHOULD NOT, rather than MUST NOT. NOTE 4: Support of this header in this method is dependent on the security mechanism and the security architecture which is implemented. Use of this header in this method is not appropriate to the security mechanism defined by 3GPP TS 33.203 [19]. |  |  |  |  |  |  |  |

Prerequisite A.5/8 - - INVITE request

Table A.47: Supported message bodies within the INVITE request

| Item | Header | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | XML Schema for PSTN | [11B] |  | c1 | [11B] |  | c1 |
| 2 | application/vnd.3gpp.ussd | [8W] |  | c2 | [8W] |  | c3 |
| 3 | application/vnd.3gpp.mcptt-info+xml | [8ZE] | n/a | c4 | [8ZE] | n/a | c4 |
| 4 | application/vnd.etsi.aoc+xml | [8N] 4.7.2 | n/a | c5 | [8N] 4.7.2 | n/a | c6 |
| 5 | application/EmergencyCallData.eCall.MSD | [244] 14.3 | m | c7 | [244] 14.3 | m | c8 |
| c1: IF A.3/6 OR A.3/7A OR A.3/7B OR A.3/7D OR A.3/9B OR A.3/13B THEN o ELSE n/a - - MGCF, AS acting as terminating UA, or redirect server, AS acting as originating UA, AS performing 3rd party call control, IBCF (IMS-ALG), ISC gateway function (IMS-ALG). c2: IF A.3A/92A OR A.3A/93B OR A.3/9 OR A.3/2 OR A.3A/89 THEN m ELSE n/a - - USSI UE supporting user-initiated USSD operations, USSI AS supporting network-initiated USSD operations, IBCF, P-CSCF, ATCF (UA). c3: IF A.3A/93A OR A.3A/92B OR A.3/9 OR A.3/2 OR A.3A/89 THEN m ELSE n/a - - USSI AS supporting user-initiated USSD operations, USSI UE supporting network-initiated USSD operations, IBCF, P-CSCF, ATCF (UA). c4: IF A.3A/102 OR A.3A/103 THEN m ELSE n/a - - MCPTT client, MCPTT server. c5 IF A.3A/53 THEN m ELSE n/a - - Advice of charge application server. c6 IF A.3A/54 THEN m ELSE n/a - - Advice of charge UA client. c7: IF (A.3/1 AND A.4/120) THEN m ELSE IF ((A.3/2A OR A.3/11A OR A.3A/84) AND A.4/120) THEN i ELSE n/a - - UE, Next-Generation Pan-European eCall emergency service, P-CSCF (IMS-ALG), E-CSCF acting as UA, EATF. c8: IF ((A.3/2A OR A.3/11A OR A.3A/84 OR A.3/12) AND A.4/120) THEN i ELSE n/a - - P-CSCF (IMS-ALG), E-CSCF acting as UA, EATF, LRF, Next-Generation Pan-European eCall emergency service. |  |  |  |  |  |  |  |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/1 - - Additional for 100 (Trying) response

Table A.48: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 2 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 3 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 4 | Date | [26] 20.17 | c1 | c1 | [26] 20.17 | m | m |
| 5 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 6 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 7 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| c1: IF A.4/11 THEN o ELSE n/a - - insertion of date in requests and responses. |  |  |  |  |  |  |  |

Prerequisite A.5/9 - - INVITE response for all remaining status-codes

Table A.49: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 0A | Allow | [26] 20.5 | c12 | c12 | [26] 20.5 | m | m |
| 1 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 1A | Call-Info | [26] 20.9 | o | o | [26] 20.9 | o | o |
| 1B | Cellular-Network-Info | 7.2.15 | n/a | c20 | 7.2.15 | n/a | c21 |
| 2 | Content-Disposition | [26] 20.11 | o | o | [26] 20.11 | m | m |
| 3 | Content-Encoding | [26] 20.12 | o | o | [26] 20.12 | m | m |
| 3A | Content-ID | [256] 3.2 | o | c23 | [256] 3.2 | m | c24 |
| 4 | Content-Language | [26] 20.13 | o | o | [26] 20.13 | m | m |
| 5 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 6 | Content-Type | [26] 20.15 | m | m | [26] 20.15 | m | m |
| 7 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 8 | Date | [26] 20.17 | c1 | c1 | [26] 20.17 | m | m |
| 8ª | Expires | [26] 20.19 | o | o | [26] 20.19 | o | o |
| 9 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 9A | Geolocation-Error | [89] 4.3 | c14 | c14 | [89] 4.3 | c14 | c14 |
| 9B | History-Info | [66] 4.1 | c13 | c13 | [66] 4.1 | c13 | c13 |
| 10 | MIME-Version | [26] 20.24 | o | o | [26] 20.24 | m | m |
| 11 | Organization | [26] 20.25 | o | o | [26] 20.25 | o | o |
| 11A | P-Access-Network-Info | [52] 4.4, [52A] 4, [234] 2 | c5 | c6 | [52] 4.4, [52A] 4, [234] 2 | c5 | c7 |
| 11B | P-Asserted-Identity | [34] 9.1 | n/a | c22 | [34] 9.1 | c3 | c3 |
| 11C | P-Charging-Function-Addresses | [52] 4.5, [52A] 4 | c10 | c11 | [52] 4.5, [52A] 4 | c11 | c11 |
| 11D | P-Charging-Vector | [52] 4.6, [52A] 4 | c8 | c9 | [52] 4.6, [52A] 4 | c8 | c9 |
| 11F | P-Preferred-Identity | [34] 9.2 | c3 | x | [34] 9.2 | n/a | n/a |
| 11G | Privacy | [33] 4.2 | c4 | c4 | [33] 4.2 | c4 | c4 |
| 11H | Relayed-Charge | 7.2.12 | n/a | c19 | 7.2.12 | n/a | c19 |
| 11I | Reply-To | [26] 20.31 | o | o | [26] 20.31 | o | o |
| 11J | Require | [26] 20.32 | m | m | [26] 20.32 | m | m |
| 11K | Server | [26] 20.35 | o | o | [26] 20.35 | o | o |
| 11LA | Service-Interact-Info | Subclause 7.2.14 | n/a | c25 | Subclause 7.2.14 | n/a | c26 |
| 11L | Session-ID | [162] | o | c18 | [162] | o | c18 |
| 12 | Timestamp | [26] 20.38 | m | m | [26] 20.38 | c2 | c2 |
| 13 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 13A | User-Agent | [26] 20.41 | o | o | [26] 20.41 | o | o |
| 13B | User-to-User | [126] 7 | c15 | c15 | [126] 7 | c15 | c15 |
| 14 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| 15 | Warning | [26] 20.43 | o (note) | o | [26] 20.43 | o | o |
| c1: IF A.4/11 THEN o ELSE n/a - - insertion of date in requests and responses. c2: IF A.4/6 THEN m ELSE n/a - - timestamping of requests. c3: IF A.4/25 THEN o ELSE n/a - - private extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c4: IF A.4/26 THEN o ELSE n/a - - a privacy mechanism for the Session Initiation Protocol (SIP). c5: IF A.4/34 THEN o ELSE n/a - - the P-Access-Network-Info header extension. c6: IF A.4/34 AND (A.3/1 OR A.3/2A OR A.3/7 OR A.3A/81 OR A.3/81A OR A.3A/81B) THEN m ELSE n/a - - the P-Access-Network-Info header extension and UE, P-CSCF (IMS-ALG), AS or MSC server enhanced for ICS or MSC Server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c7: IF A.4/34 AND (A.3/2A OR A.3A/81 OR A.3/7A OR A.3/7D OR A3A/84) THEN m ELSE n/a - - the P-Access-Network-Info header extension and P-CSCF (IMS-ALG), MSC server enhanced for ICS, AS acting as terminating UA, AS acting as third-party call controller or EATF. c8: IF A.4/36 THEN o ELSE n/a - - the P-Charging-Vector header extension. c9: IF A.4/36 THEN m ELSE n/a - - the P-Charging-Vector header extension. c10: IF A.4/35 THEN o ELSE n/a - - the P-Charging-Function-Addresses header extension. c11: IF A.4/35 THEN m ELSE n/a - - the P-Charging-Function-Addresses header extension. c12: IF A.6/102 OR A.6/18 THEN m ELSE o - - 2xx response, 405 (Method Not Allowed). c13: IF A.4/47 THEN m ELSE n/a - - an extension to the session initiation protocol for request history information. c14: IF A.4/60 THEN m ELSE n/a - - SIP location conveyance. c15: IF A.4/76 THEN o ELSE n/a - - transporting user to user information for call centers using SIP. c18: IF A.4/91 THEN m ELSE n/a - - the Session-ID header. c19: IF A.4/111 THEN m ELSE n/a - - the Relayed-Charge header field extension. c20: IF A.4/113 AND (A.3/1 OR A.3/2A OR A.3/7) THEN m ELSE n/a - - the Cellular-Network-Info header extension and UE, P-CSCF (IMS-ALG) or AS. c21: IF A.4/113 AND (A.3/2A OR A.3/7A OR A.3/7D OR A3A/84) THEN m ELSE n/a - - the Cellular-Network-Info header extension and P-CSCF (IMS-ALG), AS acting as terminating UA, AS acting as third-party call controller or EATF. c22: IF A.4/25 AND (A.3/6 OR A.3/7B OR A.3/8 OR A.3A/81 OR A.3A/81A OR A.3A/81B OR A.3A/83 OR A.3A/89) THEN o ELSE n/a - - private extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks and MGCF, AS acting as originating UA, MRFC, MSC Server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface, SCC application server, ATCF (UA). c23: IF A.4/119 THEN o ELSE n/a - - Content-ID header field in Session Initiation Protocol (SIP). c24: IF A.4/119 THEN m ELSE n/a - - Content-ID header field in Session Initiation Protocol (SIP). c25: IF A.4/123 AND (A.3/7 OR A.3/9) THEN m ELSE n/a - - Dynamic services interactions, AS, IBCF. c26: IF A.4/123 AND (A.3/2 OR A.3/7 OR A.3/9) THEN m ELSE n/a - - Dynamic services interactions, P-CSCF, AS, IBCF. |  |  |  |  |  |  |  |
| NOTE: For a 488 (Not Acceptable Here) response, RFC 3261 [26] gives the status of this header as SHOULD rather than OPTIONAL. |  |  |  |  |  |  |  |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/101A - - Additional for 18x response

Table A.50: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 4 | Contact | [26] 20.10 | o | m | [26] 20.10 | m | m |
| 4A | Feature-Caps | [190] | c17 | c17 | [190] | c16 | c16 |
| 5 | P-Answer-State | [111] | c13 | c13 | [111] | c13 | c13 |
| 5A | P-Early-Media | [109] 8 | c14 | c14 | [109] 8 | c14 | c14 |
| 6 | P-Media-Authorization | [31] 5.1 | n/a | n/a | [31] 5.1 | c11 | c12 |
| 6AA | Priority-Share | Subclause 7.2.16 | n/a | c19 | Subclause 7.2.16 | n/a | c19 |
| 6A | Reason | [130], [294] | o | c15 | [130], [294] | o | c15 |
| 7 | Record-Route | [26] 20.30 | o | m | [26] 20.30 | m | m |
| 8 | Recv-Info | [25] 5.2.3 | c4 | c4 | [25] 5.2.3 | c4 | c4 |
| 8A | Resource-Share | Subclause 7.2.13 | n/a | c18 | Subclause 7.2.13 | n/a | c18 |
| 9 | RSeq | [27] 7.1 | c2 | m | [27] 7.1 | c3 | m |
| c2: IF A.4/14 THEN o ELSE n/a - - reliability of provisional responses in SIP. c3: IF A.4/14 THEN m ELSE n/a - - reliability of provisional responses in SIP. c4: IF A.4/13 THEN m ELSE IF A.4/13A THEN m ELSE n/a - - SIP INFO method and package framework, legacy INFO usage. c11: IF A.4/19 THEN m ELSE n/a - - SIP extensions for media authorization. c12: IF A.3/1 AND A.4/19 THEN m ELSE n/a - - UE, SIP extensions for media authorization. c13: IF A.4/65 THEN m ELSE n/a - - the P-Answer-State header extension to the session initiation protocol for the open mobile alliance push to talk over cellular. c14: IF A.4/66 THEN m ELSE n/a - - the SIP P-Early-Media private header extension for authorization of early media. c15: IF A.4/38A OR A.4/38C THEN o ELSE n/a - - use of the Reason header field in Session Initiation Protocol (SIP) responses (carrying Q.850 or STIR codes). c16: IF A.4/100 THEN m ELSE n/a - - indication of features supported by proxy. c17: IF A.4/100 AND A.3/1 AND NOT A.3C/1 THEN n/a ELSE IF A.4/100 THEN m ELSE n/a - - indication of features supported by proxy, UE, UE performing the functions of an external attached network. c18: IF A.4/112 THEN o ELSE n/a - - resource sharing. c19: IF A.4/114THEN o ELSE n/a - - priority sharing. |  |  |  |  |  |  |  |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/2 - - Additional for 180 (Ringing) response

Table A.50A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Alert-Info | [26] 20.4 | o | c1 | [26] 20.4 | o | c1 |
| c1: IF A.4/96 THEN m ELSE o - - Alert-Info URNs for the Session Initiation Protocol. |  |  |  |  |  |  |  |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/5A - - Additional for 199 (Early Dialog Terminated) response

Table A.50B: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 4 | Contact | [26] 20.10 | o | m | [26] 20.10 | m | m |
| 5 | Reason | [130], [294] | o | c5 | [130], [294] | o | c5 |
| 7 | Record-Route | [26] 20.30 | o | m | [26] 20.30 | m | m |
| 8 | Recv-Info | [25] 5.2.3 | c4 | c4 | [25] 5.2.3 | c4 | c4 |
| 9 | RSeq | [27] 7.1 | c2 | m | [27] 7.1 | c3 | m |
| c2: IF A.4/14 THEN o ELSE n/a - - reliability of provisional responses in SIP. c3: IF A.4/14 THEN m ELSE n/a - - reliability of provisional responses in SIP. c4: IF A.4/13 THEN m ELSE IF A.4/13A THEN m ELSE n/a - - SIP INFO method and package framework, legacy INFO usage. c5: IF A.4/38A OR A.4/38C THEN o ELSE n/a - - use of the Reason header field in Session Initiation Protocol (SIP) responses (carrying Q.850 or STIR codes). |  |  |  |  |  |  |  |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/102 - - Additional for 2xx response

Table A.51: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept | [26] 20.1 | o | o | [26] 20.1 | m | m |
| 1A | Accept-Encoding | [26] 20.2 | o | o | [26] 20.2 | m | m |
| 1B | Accept-Language | [26] 20.3 | o | o | [26] 20.3 | m | m |
| 1C | Accept-Resource-Priority | [116] 3.2 | c15 | c15 | [116] 3.2 | c15 | c15 |
| 2 | Allow-Events | [28] 8.2.2 | c3 | c3 | [28] 8.2.2 | c4 | c4 |
| 3 | Answer-Mode | [158] | c6 | c6 | [158] | c7 | c7 |
| 4 | Authentication-Info | [26] 20.6 | c1 | c1 | [26] 20.6 | c2 | c2 |
| 6 | Contact | [26] 20.10 | m | m | [26] 20.10 | m | m |
| 6A | Feature-Caps | [190] | c18 | c18 | [190] | c17 | c17 |
| 7 | P-Answer-State | [111] | c14 | c14 | [111] | c14 | c14 |
| 8 | P-Media-Authorization | [31] 5.1 | n/a | n/a | [31] 5.1 | c11 | c12 |
| 8AA | Priority-Share | Subclause 7.2.16 | n/a | c20 | Subclause 7.2.16 | n/a | c20 |
| 8A | Priv-Answer-Mode | [158] | c6 | c6 | [158] | c7 | c7 |
| 9 | Record-Route | [26] 20.30 | m | m | [26] 20.30 | m | m |
| 9A | Recv-Info | [25] 5.2.3 | c5 | c5 | [25] 5.2.3 | c5 | c5 |
| 9B | Resource-Share | Subclause 7.2.13 | n/a | c19 | Subclause 7.2.13 | n/a | c19 |
| 10 | Session-Expires | [58] 4 | c13 | c13 | [58] 4 | c13 | c13 |
| 13 | Supported | [26] 20.37 | m | m | [26] 20.37 | m | m |
| c1: IF A.4/7 THEN o ELSE n/a - - authentication between UA and UA. c2: IF A.4/7 THEN m ELSE n/a - - authentication between UA and UA. c3: IF A.4/22 THEN o ELSE n/a - - acting as the notifier of event information. c4: IF A.4/23 THEN m ELSE n/a - - acting as the subscriber to event information. c5: IF A.4/13 THEN m ELSE IF A.4/13A THEN m ELSE n/a - - SIP INFO method and package framework, legacy INFO usage. c6 IF A.4/87 THEN o ELSE n/a - - requesting answering modes for SIP. c7 IF A.4/87 THEN m ELSE n/a - - requesting answering modes for SIP. c11: IF A.4/19 THEN m ELSE n/a - - SIP extensions for media authorization. c12: IF A.3/1 AND A.4/19 THEN m ELSE n/a - - UE, SIP extensions for media authorization. c13: IF A.4/42 THEN m ELSE n/a - - the SIP session timer. c14: IF A.4/65 THEN m ELSE n/a - - the P-Answer-State header extension to the session initiation protocol for the open mobile alliance push to talk over cellular. c15: IF A.4/70 THEN m ELSE n/a - - communications resource priority for the session initiation protocol. c17: IF A.4/100 THEN m ELSE n/a - - indication of features supported by proxy. c18: IF A.4/100 AND A.3/1 AND NOT A.3C/1 THEN n/a ELSE IF A.4/100 THEN m ELSE n/a - - indication of features supported by proxy, UE, UE performing the functions of an external attached network. c19: IF A.4/112 THEN m ELSE n/a - - resource sharing. c20: IF A.4/114THEN o ELSE n/a - - priority sharing. |  |  |  |  |  |  |  |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/103 OR A.6/104 OR A.6/105 OR A.6/106 - - Additional for 3xx – 6xx response

Table A.51A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Error-Info | [26] 20.18 | o | o | [26] 20.18 | o | o |
| 2 | Reason | [130], [294] | o | c1 | [130], [294] | o | c1 |
| 3 | Response-Source | 7.2.17 | n/a | c2 | 7.2.17 | n/a | c2 |
| c1: IF A.4/38A OR A.4/38C THEN o ELSE n/a - - use of the Reason header field in Session Initiation Protocol (SIP) responses (carrying Q.850 or STIR codes). c2: IF A.4/115 THEN o ELSE n/a - - use of the Response-Source header field in SIP error responses. |  |  |  |  |  |  |  |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/103 OR A.6/35 - - Additional for 3xx or 485 (Ambiguous) response

Table A.52: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 4 | Contact | [26] 20.10 | o (note 1) | o | [26] 20.10 | m | m |
| NOTE: The strength of this requirement is RECOMMENDED rather than OPTIONAL. |  |  |  |  |  |  |  |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/14 - - Additional for 401 (Unauthorized) response

Table A.53: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 6 | Proxy-Authenticate | [26] 20.27 | c3 | c3 | [26] 20.27 | c3 | c3 |
| 13 | WWW-Authenticate | [26] 20.44 | m | m | [26] 20.44 | m | m |
| c1: IF A.4/11 THEN o ELSE n/a - - insertion of date in requests and responses. c2: IF A.4/6 THEN m ELSE n/a - - timestamping of requests. c3: IF A.4/7 THEN m ELSE n/a - - support of authentication between UA and UA. |  |  |  |  |  |  |  |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/16 - - Additional for 403 (Forbidden) response

Table A.53A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | P-Refused-URI-List | [183] | c1 | c1 | [183] | c1 | c1 |
| c1: IF A.4/98 THEN m ELSE n/a -- The SIP P-Refused-URI-List private-header. |  |  |  |  |  |  |  |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/17 OR A.6/23 OR A.6/30 OR A.6/36 OR A.6/50 OR A.6/51 - - Additional for 404 (Not Found), 413 (Request Entity Too Large), 480(Temporarily not available), 486 (Busy Here), 500 (Internal Server Error), 600 (Busy Everywhere), 603 (Decline) response

Table A.54: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 8 | Retry-After | [26] 20.33 | o | o | [26] 20.33 | o | o |

Table A.55: Void

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/20 - - Additional for 407 (Proxy Authentication Required) response

Table A.56: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 6 | Proxy-Authenticate | [26] 20.27 | c1 | c1 | [26] 20.27 | c1 | c1 |
| 11 | WWW-Authenticate | [26] 20.44 | o | o | [26] 20.44 | o | o |
| c1: IF A.4/7 THEN m ELSE n/a - - support of authentication between UA and UA. |  |  |  |  |  |  |  |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/21 - - Additional for 408 (Request timeout) response

Table A.56A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Restoration-Info | subclause 7.2.11 | n/a | c1 | subclause 7.2.11 | n/a | n/a |
| c1: IF A.4/110 THEN o ELSE n/a - - HSS based P-CSCF restoration. |  |  |  |  |  |  |  |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/25 - - Additional for 415 (Unsupported Media Type) response

Table A.57: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept | [26] 20.1 | o.1 | o.1 | [26] 20.1 | m | m |
| 2 | Accept-Encoding | [26] 20.2 | o.1 | o.1 | [26] 20.2 | m | m |
| 3 | Accept-Language | [26] 20.3 | o.1 | o.1 | [26] 20.3 | m | m |
| o.1 At least one of these capabilities is supported. |  |  |  |  |  |  |  |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/26A - - Additional for 417 (Unknown Resource-Priority) response

Table A.57A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept-Resource-Priority | [116] 3.2 | c1 | c1 | [116] 3.2 | c1 | c1 |
| c1: IF A.4/70 THEN m ELSE n/a - - communications resource priority for the session initiation protocol. |  |  |  |  |  |  |  |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/27 - - Additional for 420 (Bad Extension) response

Table A.58: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 10 | Unsupported | [26] 20.40 | m | m | [26] 20.40 | m | m |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/28 OR A.6/41A - - Additional for 421 (Extension Required), 494 (Security Agreement Required) response

Table A.58A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 3 | Security-Server | [48] 2 | x | x | [48] 2 | c1 | c1 |
| c1: IF A.4/37 THEN m ELSE n/a - - security mechanism agreement for the session initiation protocol. |  |  |  |  |  |  |  |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/28A - - Additional for 422 (Session Interval Too Small) response

Table A.58B: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Min-SE | [58] 5 | c1 | c1 | [58] 5 | c1 | c1 |
| c1: IF A.4/42 THEN o ELSE n/a - - the SIP session timer. |  |  |  |  |  |  |  |

Table A.59: Void

Table A.60: Void

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/29H - - Additional for 470 (Consent Needed) response

Table A.60A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Permission-Missing | [125] 5.9.3 | m | m | [125] 5.9.3 | m | m |

Prerequisite A.5/9 - - INVITE response

Prerequisite: A.6/45 - - 503 (Service Unavailable)

Table A.61: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 8 | Retry-After | [26] 20.33 | o | o | [26] 20.33 | o | m |

Table A.61A: Void

Prerequisite A.5/9 - - INVITE response

Table A.62: Supported message bodies within the INVITE response

| Item | Header | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | XML Schema for PSTN | [11B] |  | c1 | [11B] |  | c1 |
| 2 | Recipient list | [183] | c2 | c2 | [183] | c2 | c2 |
| 3 | 3GPP IM CN subsystem XML body | subclause 7.6 | n/a | c3 | subclause 7.6 | n/a | c4 (note) |
| 4 | application/vnd.3gpp.mcptt-info+xml | [8ZE] | n/a | c5 | [8ZE] | n/a | c5 |
| 5 | application/vnd.etsi.aoc+xml | [8N] 4.7.2 | n/a | c6 | [8N] 4.7.2 | n/a | c7 |
| 6 | application/EmergencyCallData.Control+xml | [244] 14.4 | m | c8 | [244] 14.4 | m | c9 |
| c1: IF A.3/6 OR A.3/7A OR A.3/7B OR A.3/7D OR A.3/9B OR A.3/13B THEN o ELSE n/a - - MGCF, AS acting as terminating UA, or redirect server, AS acting as originating UA, AS performing 3rd party call control, IBCF (IMS-ALG), ISC gateway function (IMS-ALG). c2: IF A.3/9B OR A.3/13B THEN m ELSE IF A.3/7A OR A.3/7B OR A.3/7D THEN o ELSE n/a - - IBCF (IMS-ALG), ISC gateway function (IMS-ALG), AS acting as terminating UA, AS acting as originating UA, AS performing 3rd party call control. c3: IF A.3/9B OR A.3/9C OR A.3/13B OR A.3/13C OR (A.4/103 AND A.3/2) OR (A.4/103 AND A.3/4) THEN m ELSE n/a - - IBCF (IMS-ALG), IBCF (Screening of SIP signalling), ISC gateway function (IMS-ALG), ISC gateway function (Screening of SIP signalling), S-CSCF restoration procedures, P-CSCF, S-CSCF. c4: IF A.3/1 OR A.3/2 OR A.3/9B OR A.3/9C OR A.3/13B OR A.3/13C THEN m ELSE IF A.3/4 THEN o ELSE n/a - - UE, P-CSCF, IBCF (IMS-ALG), IBCF (Screening of SIP signalling), ISC gateway function (IMS-ALG), S-CSCF. c5: IF A.3A/102 OR A.3A/103 THEN M ELSE n/a - - MCPTT client, MCPTT server. c6 IF A.3A/53 THEN m ELSE n/a - - Advice of charge application server. c7 IF A.3A/54 THEN m ELSE n/a - - Advice of charge UA client. c8: IF ((A.3/2A OR A.3/11A OR A.3A/84) AND A.4/120) THEN i ELSE n/a - - P-CSCF (IMS-ALG), E-CSCF acting as UA, EATF, Next-Generation Pan-European eCall emergency service. c9: IF (A.3/1 AND A.4/120) THEN m ELSE IF ((A.3/2A OR A.3/11A OR A.3A/84) AND A.4/120) THEN i ELSE n/a - - UE, Next-Generation Pan-European eCall emergency service, P-CSCF (IMS-ALG), E-CSCF acting as UA, EATF. |  |  |  |  |  |  |  |
| NOTE: If a IBCF (IMS-ALG) or a IBCF (Screening of SIP signalling) is unable to receive a 3GPP IM CN subsystem XML body from a S-CSCF in a serving network then the IBCF (IMS-ALG) or the IBCF (Screening of SIP signalling) support can be "o" instead of "m". Examples include an S-CSCF supporting S-CSCF restoration procedures. |  |  |  |  |  |  |  |

# *** Next Change ***

#### A.2.1.4.7A	MESSAGE method

Prerequisite A.5/9A - - MESSAGE request

Table A.62A: Supported header fields within the MESSAGE request

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept-Contact | [56B] 9.2 | c24 | c24 | [56B] 9.2 | c28 | c28 |
| 1AA | Additional-Identity | 7.2.20 | n/a | c58 | 7.2.20 | n/a | c59 |
| 1A | Allow | [26] 20.5 | o | o | [26] 20.5 | m | m |
| 2 | Allow-Events | [28] 8.2.2 | c1 | c1 | [28] 8.2.2 | c2 | c2 |
| 2A | Attestation-Info | 7.2.18 | n/a | c54 | 7.2.18 | n/a | c54 |
| 3 | Authorization | [26] 20.7 | c3 | c3 | [26] 20.7 | c3 | c3 |
| 4 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 5 | Call-Info | [26] 20.9 | o | o | [26] 20.9 | o | o |
| 5A | Cellular-Network-Info | 7.2.15 | n/a | c47 | 7.2.15 | n/a | c48 |
| 6 | Content-Disposition | [26] 20.11 | o | o | [26] 20.11 | m | m |
| 7 | Content-Encoding | [26] 20.12 | o | o | [26] 20.12 | m | m |
| 7A | Content-ID | [256] 3.2 | o | c52 | [256] 3.2 | m | c53 |
| 8 | Content-Language | [26] 20.13 | o | o | [26] 20.13 | m | m |
| 9 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 10 | Content-Type | [26] 20.15 | m | m | [26] 29.15 | m | m |
| 11 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 12 | Date | [26] 20.17 | c4 | c4 | [26] 20.17 | m | m |
| 13 | Expires | [26] 20.19 | o | o | [26] 20.19 | o | o |
| 13A | Feature-Caps | [190] | c45 | c45 | [190] | c44 | c44 |
| 14 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 14A | Geolocation | [89] 4.1 | c29 | c29 | [89] 4.1 | c29 | c29 |
| 14B | Geolocation-Routing | [89] 4.2 | c29 | c29 | [89] 4.2 | c29 | c29 |
| 14C | History-Info | [66] 4.1 | c27 | c27 | [66] 4.1 | c27 | c27 |
| 14D | Identity | [252] 4 | c51 | c51 | [252] 4 | c51 | c51 |
| 15 | In-Reply-To | [26] 20.21 | o | o | [26] 20.21 | o | o |
| 15A | Max-Breadth | [117] 5.8 | n/a | c39 | [117] 5.8 | c40 | c40 |
| 16 | Max-Forwards | [26] 20.22 | m | m | [26] 20.22 | n/a | c42 |
| 17 | MIME-Version | [26] 20.24 | o | o | [26] 20.24 | m | m |
| 18 | Organization | [26] 20.25 | o | o | [26] 20.25 | o | o |
| 18AA | Origination-Id | 7.2.19 | n/a | c55 | 7.2.19 | n/a | c55 |
| 18A | P-Access-Network-Info | [52] 4.4, [234] 2 | c15 | c16 | [52] 4.4, [234] 2 | c15 | c16 |
| 18B | P-Asserted-Identity | [34] 9.1 | n/a | c49 | [34] 9.1 | c11 | c11 |
| 18C | P-Asserted-Service | [121] 4.1 | n/a | c50 | [121] 4.1 | c33 | c33 |
| 18D | P-Called-Party-ID | [52] 4.2 | x | x | [52] 4.2 | c13 | c13 |
| 18E | P-Charging-Function-Addresses | [52] 4.5 | c20 | c21 | [52] 4.5 | c20 | c21 |
| 18F | P-Charging-Vector | [52] 4.6 | c18 | c19 | [52] 4.6 | c18 | c19 |
| 18H | P-Preferred-Identity | [34] 9.2 | c11 | c7 | [34] 9.2 | n/a | n/a |
| 18I | P-Preferred-Service | [121] 4.2 | c32 | c31 | [121] 4.2 | n/a | n/a |
| 18J | P-Private-Network-Indication | [134] | c36 | c36 | [134] | c36 | c36 |
| 18K | P-Profile-Key | [97] 5 | n/a | n/a | [97] 5 | n/a | n/a |
| 18L | P-Served-User | [133] 6 | c41 | c41 | [133] 6 | c41 | c41 |
| 18M | P-User-Database | [82] 4 | n/a | n/a | [82] 4 | n/a | n/a |
| 18N | P-Visited-Network-ID | [52] 4.3 | x (note 1) | x | [52] 4.3 | c14 | n/a |
| 19 | Priority | [26] 20.26 | o | o | [26] 20.26 | o | o |
| 19A | Privacy | [33] 4.2 | c12 | c12 | [33] 4.2 | c12 | c12 |
| 20 | Proxy-Authorization | [26] 20.28 | c5 | c5 | [26] 20.28 | n/a | n/a |
| 21 | Proxy-Require | [26] 20.29 | o | n/a | [26] 20.29 | n/a | n/a |
| 21A | Reason | [34A] 2 | c6 | c6 | [34A] 2 | c6 | c6 |
| 22A | Referred-By | [59] 3 | c25 | c25 | [59] 3 | c26 | c26 |
| 23 | Reject-Contact | [56B] 9.2 | c24 | c24 | [56B] 9.2 | c28 | c28 |
| 23A | Relayed-Charge | 7.2.12 | n/a | c46 | 7.2.12 | n/a | c46 |
| 23B | Reply-To | [26] 20.31 | o | o | [26] 20.31 | o | o |
| 23C | Request-Disposition | [56B] 9.1 | c24 | c24 | [56B] 9.1 | c28 | c28 |
| 24 | Require | [26] 20.32 | m | m | [26] 20.32 | m | m |
| 24A | Resource-Priority | [116] 3.1 | c30 | c30 | [116] 3.1 | c30 | c30 |
| 25 | Route | [26] 20.34 | m | m | [26] 20.34 | n/a | n/a |
| 25A | Security-Client | [48] 2.3.1 | c22 | c22 | [48] 2.3.1 | n/a | n/a |
| 25B | Security-Verify | [48] 2.3.1 | c23 | c23 | [48] 2.3.1 | n/a | n/a |
| 25CA | Service-Interact-Info | Subclause 7.2.14 | n/a | c56 | Subclause 7.2.14 | n/a | c57 |
| 25C | Session-ID | [162] | o | c43 | [162] | o | c43 |
| 26 | Subject | [26] 20.35 | o | o | [26] 20.36 | o | o |
| 27 | Supported | [26] 20.37 | c9 | m | [26] 20.37 | m | m |
| 28 | Timestamp | [26] 20.38 | c10 | c10 | [26] 20.38 | m | m |
| 29 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 29A | Trigger-Consent | [125] 5.11.2 | c34 | c34 | [125] 5.11.2 | c35 | c35 |
| 30 | User-Agent | [26] 20.41 | o | o | [26] 20.41 | o | o |
| 31 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| c1: IF A.4/22 THEN o ELSE n/a - - acting as the notifier of event information. c2: IF A.4/23 THEN m ELSE n/a - - acting as the subscriber to event information. c3: IF A.4/7 THEN m ELSE n/a - - authentication between UA and UA. c4: IF A.4/11 THEN o ELSE n/a - - insertion of date in requests and responses. c5: IF A.4/8A THEN m ELSE n/a - - authentication between UA and proxy. c6: IF A.4/38 THEN o ELSE n/a - - the Reason header field for the session initiation protocol. c7: IF A.3/1 AND A.4/25 THEN o ELSE n/a - - UE and private extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c9: IF A.4/14 THEN m ELSE o - - support of reliable transport. c10: IF A.4/6 THEN o ELSE n/a - - timestamping of requests. c11: IF A.4/25 THEN o ELSE n/a - - private extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c12: IF A.4/26 THEN o ELSE n/a - - a privacy mechanism for the Session Initiation Protocol (SIP). c13: IF A.4/32 THEN o ELSE n/a - - the P-Called-Party-ID extension. c14: IF A.4/33 THEN o ELSE n/a - - the P-Visited-Network-ID extension. c15: IF A.4/34 THEN o ELSE n/a - - the P-Access-Network-Info header extension. c16: IF A.4/34 AND A.3/1 THEN m ELSE n/a - - the P-Access-Network-Info header extension and UE. c17: IF A.4/34 AND (A.3/7A OR A.3/7D) THEN m ELSE n/a - - the P-Access-Network-Info header extension and AS acting as terminating UA or AS acting as third-party call controller. c18: IF A.4/36 THEN o ELSE n/a - - the P-Charging-Vector header extension. c19: IF A.4/36 THEN m ELSE n/a - - the P-Charging-Vector header extension. c20: IF A.4/35 THEN o ELSE n/a - - the P-Charging-Function-Addresses header extension. c21: IF A.4/35 THEN m ELSE n/a - - the P-Charging-Function-Addresses header extension. c22: IF A.4/37 OR A.4/37A THEN o ELSE n/a - - security mechanism agreement for the session initiation protocol or mediasec header field parameter for marking security mechanisms related to media (note 2). c23: IF A.4/37 OR A.4/37A THEN m ELSE n/a - - security mechanism agreement for the session initiation protocol or mediasec header field parameter for marking security mechanisms related to media. c24: IF A.4/40 THEN o ELSE n/a - - caller preferences for the session initiation protocol. c25: IF A.4/43 THEN m ELSE n/a - - the SIP Referred-By mechanism. c26: IF A.4/43 THEN o ELSE n/a - - the SIP Referred-By mechanism. c27: IF A.4/47 THEN m ELSE n/a - - an extension to the session initiation protocol for request history information. c28: IF A.4/40 THEN m ELSE n/a - - caller preferences for the session initiation protocol. c29: IF A.4/60 THEN m ELSE n/a - - SIP location conveyance. c30: IF A.4/70A THEN m ELSE n/a - - inclusion of MESSAGE, SUBSCRIBE, NOTIFY in communications resource priority for the session initiation protocol. c31: IF A.3/1 AND A.4/74 THEN o ELSE n/a - - UE and SIP extension for the identification of services. c32: IF A.4/74 THEN o ELSE n/a - - SIP extension for the identification of services. c33: IF A.4/74 THEN m ELSE n/a - - SIP extension for the identification of services. c34: IF A.4/75A THEN m ELSE n/a - - a relay within the framework for consent-based communications in SIP. c35: IF A.4/75B THEN m ELSE n/a - - a recipient within the framework for consent-based communications in SIP. c36: IF A.4/77 THEN m ELSE n/a - - the SIP P-Private-Network-Indication private-header (P-Header). c39: IF A.4/71 AND (A.3/9B OR A.3/9C OR A.3/13B OR A.3/13C) THEN m ELSE IF A.3/1 AND NOT A.3C/1 THEN n/a ELSE o - - addressing an amplification vulnerability in session initiation protocol forking proxies, IBCF (IMS-ALG), IBCF (Screening of SIP signalling), ISC gateway function (IMS-ALG), ISC gateway function (Screening of SIP signalling), UE, UE performing the functions of an external attached network. c40: IF A.4/71 THEN m ELSE n/a - - addressing an amplification vulnerability in session initiation protocol forking proxies. c41: IF A.4/78 THEN m ELSE n/a - - the SIP P-Served-User private header. c42: IF A.3/1 AND NOT A.3C/1 THEN n/a ELSE o - - UE, UE performing the functions of an external attached network. c43: IF A.4/91 THEN m ELSE n/a - - the Session-ID header. c44: IF A.4/100 THEN m ELSE n/a - - indication of features supported by proxy. c45: IF A.4/100 AND A.3/1 AND NOT A.3C/1 THEN n/a ELSE IF A.4/100 THEN m ELSE n/a - - indication of features supported by proxy, UE, UE performing the functions of an external attached network. c46: IF A.4/111 THEN m ELSE n/a - - the Relayed-Charge header field extension. c47: IF A.4/113 AND A.3/1 THEN m ELSE n/a - - the Cellular-Network-Info header extension and UE. c48: IF A.4/113 AND (A.3/7A OR A.3/7D) THEN m ELSE n/a - - the Cellular-Network-Info header extension and AS acting as terminating UA or AS acting as third-party call controller. c49: IF A.4/25 AND (A.3/7B OR A.3/8) THEN o ELSE n/a - - private extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks and AS acting as originating UA, MRFC. c50: IF A.4/74 AND A.3/7B THEN o ELSE n/a - - SIP extension for the identification of services and AS acting as originating UA. c51: IF A.4/116 AND (A.3/7 OR A.3/9) THEN m ELSE n/a - - authenticated identity management in the Session Initiation Protocol, AS, IBCF. c52: IF A.4/119 THEN o ELSE n/a - - Content-ID header field in Session Initiation Protocol (SIP). c53: IF A.4/119 THEN m ELSE n/a - - Content-ID header field in Session Initiation Protocol (SIP). c54: IF A.4/121 AND (A.3/6 OR A.3/7 OR A.3/9) THEN m ELSE n/a - - the Attestation-Info header field extension, MGCF, AS, IBCF. c55: IF A.4/122 AND (A.3/6 OR A.3/7 OR A.3/9) THEN m ELSE n/a - - the Origination-Id header field extension, MGCF, AS, IBCF. c56: IF A.4/123 AND (A.3/7 OR A.3/9) THEN m ELSE n/a - - Dynamic services interactions, AS, IBCF. c57: IF A.4/123 AND (A.3/2 OR A.3/7 OR A.3/9) THEN m ELSE n/a - - Dynamic services interactions, P-CSCF, AS, IBCF. c58: IF A.4/124 THEN o ELSE n/a - - the Additional-Identity header field extension. c59: IF A.4/124 THEN m ELSE n/a - - the Additional-Identity header field extension. |  |  |  |  |  |  |  |
| NOTE 1: The strength of this requirement in RFC 7315 [52] is SHOULD NOT, rather than MUST NOT. NOTE 2: Support of this header in this method is dependent on the security mechanism and the security architecture which is implemented. Use of this header in this method is not appropriate to the security mechanism defined by 3GPP TS 33.203 [19]. |  |  |  |  |  |  |  |

Prerequisite A.5/9A - - MESSAGE request

Table A.62B: Supported message bodies within the MESSAGE request

| Item | Header | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | permission document | [125] 5.4 | c1 | c1 | [125] 5.4 | c2 | c2 |
| 2 | application/vnd.3gpp.sms | [4D] | c3 | c3 | [4D] | c3 | c3 |
| 3 | message/cpim | [151] | c4 | c4 | [151] | c4 | c4 |
| 4 | message/imdn+xml | [157] | c5 | c5 | [157] | c5 | c5 |
| 5 | application/vnd.3gpp.mcptt-info+xml | [8ZE] | n/a | c6 | [8ZE] | n/a | c6 |
| 6 | application/vnd.3gpp.mcptt-mbms-usage-info+xml | [8ZE] | n/a | c6 | [8ZE] | n/a | c6 |
| 7 | application/vnd.3gpp.mcptt-location-info+xml | [8ZE] | n/a | c6 | [8ZE] | n/a | c6 |
| 8 | application/vnd.3gpp.mcptt-floor-request+xml | [8ZE] | n/a | c7 | [8ZE] | n/a | c7 |
| 9 | application/vnd.3gpp.mcptt-affiliation-command+xml | [8ZE] | n/a | c6 | [8ZE] | n/a | c6 |
| c1: IF A.4/75A THEN m ELSE n/a - - a relay within the framework for consent-based communications in SIP. c2: IF A.4/75B THEN m ELSE n/a - - a recipient within the framework for consent-based communications in SIP. c3: IF A.3A/61 OR A.3A/62 OR A.3A/63 THEN m ELSE o - - an SM-over-IP sender or an SM-over-IP receiver or an IP-SM-GW for SMS over IP. c4: IF A.3A/71 AND A.4/85 THEN m ELSE n/a - - common presence and instant messaging (CPIM): message format. c5: IF A.3A/71 AND A.4/86 THEN m ELSE n/a - - instant message disposition notification. c6: IF A.3A/102 OR A.3A/103 THEN m ELSE n/a - - MCPTT client, MCPTT server. c7: IF A.3A/103 THEN m ELSE n/a - - MCPTT server. |  |  |  |  |  |  |  |

Prerequisite A.5/9B - - MESSAGE response

Prerequisite: A.6/1 - - Additional for 100 (Trying) response

Table A.62BA: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 2 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 3 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 4 | Date | [26] 20.17 | c1 | c1 | [26] 20.17 | m | m |
| 5 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 6 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 7 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| c1: IF A.4/11 THEN o ELSE n/a - - insertion of date in requests and responses. |  |  |  |  |  |  |  |

Prerequisite A.5/9B - - MESSAGE response for all remaining status-codes

Table A.62C: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 0A | Allow | [26] 20.5 | c12 | c12 | [26] 20.5 | m | m |
| 1 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 2 | Call-Info | [26] 20.9 | o | o | [26] 20.9 | o | o |
| 2A | Cellular-Network-Info | 7.2.15 | n/a | c19 | 7.2.15 | n/a | c20 |
| 3 | Content-Disposition | [26] 20.11 | o (note 1) | o (note 1) | [26] 20.11 | m (note 1) | m (note 1) |
| 4 | Content-Encoding | [26] 20.12 | o (note 1) | o (note 1) | [26] 20.12 | m (note 1) | m (note 1) |
| 4A | Content-ID | [256] 3.2 | o | c22 | [256] 3.2 | m | c23 |
| 5 | Content-Language | [26] 20.13 | o (note 1) | o (note 1) | [26] 20.13 | m (note 1) | m (note 1) |
| 6 | Content-Length | [26] 20.14 | m (note 1) | m (note 1) | [26] 20.14 | m (note 1) | m (note 1) |
| 7 | Content-Type | [26] 20.15 | m (note 1) | m (note 1) | [26] 20.15 | m (note 1) | m (note 1) |
| 8 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 9 | Date | [26] 20.17 | c1 | c1 | [26] 20.17 | m | m |
| 9A | Expires | [26] 20.19 | o | o | [26] 20.19 | o | o |
| 10 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 10A | Geolocation-Error | [89] 4.3 | c14 | c14 | [89] 4.3 | c14 | c14 |
| 10B | History-Info | [66] 4.1 | c13 | c13 | [66] 4.1 | c13 | c13 |
| 11 | MIME-Version | [26] 20.24 | o (note 1) | o (note 1) | [26] 20.24 | m (note 1) | m (note 1) |
| 12 | Organization | [26] 20.25 | o | o | [26] 20.25 | o | o |
| 12A | P-Access-Network-Info | [52] 4.4, [52A] 4, [234] 2 | c5 | c6 | [52] 4.4, [52A] 4, [234] 2 | c5 | c7 |
| 12B | P-Asserted-Identity | [34] 9.1 | n/a | c21 | [34] 9.1 | c3 | c3 |
| 12C | P-Charging-Function-Addresses | [52] 4.5, [52A] 4 | c10 | c11 | [52] 4.5, [52A] 4 | c10 | c11 |
| 12D | P-Charging-Vector | [52] 4.6, [52A] 4 | c8 | c9 | [52] 4.6, [52A] 4 | c8 | c9 |
| 12F | P-Preferred-Identity | [34] 9.2 | c3 | x | [34] 9.2 | n/a | n/a |
| 12G | Privacy | [33] 4.2 | c4 | c4 | [33] 4.2 | c4 | c4 |
| 12H | Relayed-Charge | 7.2.12 | n/a | c18 | 7.2.12 | n/a | c18 |
| 12I | Reply-To | [26] 20.31 | o | o | [26] 20.31 | o | o |
| 12J | Require | [26] 20.32 | m | m | [26] 20.32 | m | m |
| 13 | Server | [26] 20.35 | o | o | [26] 20.35 | o | o |
| 13AA | Service-Interact-Info | Subclause 7.2.14 | n/a | c24 | Subclause 7.2.14 | n/a | c25 |
| 13A | Session-ID | [162] | o | c17 | [162] | o | c17 |
| 14 | Timestamp | [26] 20.38 | m | m | [26] 20.38 | c2 | c2 |
| 15 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 16 | User-Agent | [26] 20.41 | o | o | [26] 20.41 | o | o |
| 17 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| 18 | Warning | [26] 20.43 | o | o | [26] 20.43 | o | o |
| c1: IF A.4/11 THEN o ELSE n/a - - insertion of date in requests and responses. c2: IF A.4/6 THEN m ELSE n/a - - timestamping of requests. c3: IF A.4/25 THEN o ELSE n/a - - private extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c4: IF A.4/26 THEN o ELSE n/a - - a privacy mechanism for the Session Initiation Protocol (SIP). c5: IF A.4/34 THEN o ELSE n/a - - the P-Access-Network-Info header extension. c6: IF A.4/34 AND A.3/1 THEN m ELSE n/a - - the P-Access-Network-Info header extension and UE. c7: IF A.4/34 AND (A.3/7A OR A.3/7D) THEN m ELSE n/a - - the P-Access-Network-Info header extension and AS acting as terminating UA or AS acting as third-party call controller. c8: IF A.4/36 THEN o ELSE n/a - - the P-Charging-Vector header extension. c9: IF A.4/36 THEN m ELSE n/a - - the P-Charging-Vector header extension. c10: IF A.4/35 THEN o ELSE n/a - - the P-Charging-Function-Addresses header extension. c11: IF A.4/35 THEN m ELSE n/a - - the P-Charging-Function-Addresses header extension. c12: IF A.6/18 THEN m ELSE o - - 405 (Method Not Allowed). c13: IF A.4/47 THEN m ELSE n/a - - an extension to the session initiation protocol for request history information. c14: IF A.4/60 THEN m ELSE n/a - - SIP location conveyance. c17: IF A.4/91 THEN m ELSE n/a - - the Session-ID header. c18: IF A.4/111 THEN m ELSE n/a - - the Relayed-Charge header field extension. c19: IF A.4/113 AND A.3/1 THEN m ELSE n/a - - the Cellular-Network-Info header extension and UE. c20: IF A.4/113 AND (A.3/7A OR A.3/7D) THEN m ELSE n/a - - the Cellular-Network-Info header extension and AS acting as terminating UA or AS acting as third-party call controller. c21: IF A.4/25 AND (A.3/7B OR A.3/8 OR A.3A/83 OR A.3A/89) THEN o ELSE n/a - - private extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks and AS acting as originating UA, MRFC, SCC application server, ATCF (UA). c22: IF A.4/119 THEN o ELSE n/a - - Content-ID header field in Session Initiation Protocol (SIP). c23: IF A.4/119 THEN m ELSE n/a - - Content-ID header field in Session Initiation Protocol (SIP). c24: IF A.4/123 AND (A.3/7 OR A.3/9) THEN m ELSE n/a - - Dynamic services interactions, AS, IBCF. c25: IF A.4/123 AND (A.3/2 OR A.3/7 OR A.3/9) THEN m ELSE n/a - - Dynamic services interactions, P-CSCF, AS, IBCF. |  |  |  |  |  |  |  |
| NOTE 1: RFC 3428 [50] clause 7 states that all 2xx class responses to a MESSAGE request must not include any body, therefore for 2xx responses to the MESSAGE request the values on Sending side for "RFC status" and "Profile status" are "x", the values for Receiving side for "RFC status" and "Profile Status" are "n/a". RFC 3261 [26] subclause 7.4 states that all responses may contain bodies, therefore for all responses to the MESSAGE request other than 2xx responses, the values on Sending side for "RFC status" and "Profile status" are "o", the values for Receiving side for "RFC status" and "Profile Status" are "m". |  |  |  |  |  |  |  |

Prerequisite A.5/9B - - MESSAGE response

Prerequisite: A.6/102 - - Additional for 2xx response

Table A.62D: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 0A | Accept-Resource-Priority | [116] 3.2 | c5 | c5 | [116] 3.2 | c5 | c5 |
| 1 | Allow-Events | [28] 8.2.2 | c3 | c3 | [28] 8.2.2 | c4 | c4 |
| 2 | Authentication-Info | [26] 20.6 | c1 | c1 | [26] 20.6 | c2 | c2 |
| 3 | Feature-Caps | [190] | c8 | c8 | [190] | c7 | c7 |
| 6 | Supported | [26] 20.37 | o | o | [26] 20.37 | m | m |
| c1: IF A.4/7 THEN o ELSE n/a - - authentication between UA and UA. c2: IF A.4/7 THEN m ELSE n/a - - authentication between UA and UA. c3: IF A.4/22 THEN o ELSE n/a - - acting as the notifier of event information. c4: IF A.4/23 THEN m ELSE n/a - - acting as the subscriber to event information. c5: IF A.4/70A THEN m ELSE n/a - - inclusion of MESSAGE, SUBSCRIBE, NOTIFY in communications resource priority for the session initiation protocol. c7: IF A.4/100 THEN m ELSE n/a - - indication of features supported by proxy. c8: IF A.4/100 AND A.3/1 AND NOT A.3C/1 THEN n/a ELSE IF A.4/100 THEN m ELSE n/a - - indication of features supported by proxy, UE, UE performing the functions of an external attached network. |  |  |  |  |  |  |  |

Prerequisite A.5/9B - - MESSAGE response

Prerequisite: A.6/103 OR A.6/104 OR A.6/105 OR A.6/106 - - Additional for 3xx – 6xx response

Table A.62DA: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Error-Info | [26] 20.18 | o | o | [26] 20.18 | o | o |
| 1A | Reason | [294] | o | c2 | [294] | o | c2 |
| 2 | Response-Source | 7.2.17 | n/a | c1 | 7.2.17 | n/a | c1 |
| c1: IF A.4/115 THEN o ELSE n/a - - use of the Response-Source header field in SIP error responses. c2: IF A.4/38C THEN o ELSE n/a - - use of the Reason header field in Session Initiation Protocol (SIP) responses (carrying STIR codes). |  |  |  |  |  |  |  |

Prerequisite A.5/9B - - MESSAGE response

Prerequisite: A.6/103 - - Additional for 3xx or 485 (Ambiguous) response

Table A.62E: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 2 | Contact | [26] 20.10 | o (note) | o | [26] 20.10 | m | m |
| NOTE: The strength of this requirement is RECOMMENDED rather than OPTIONAL. |  |  |  |  |  |  |  |

Prerequisite A.5/9B - - MESSAGE response

Prerequisite: A.6/14 - - Additional for 401 (Unauthorized) response

Table A.62F: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 3 | Proxy-Authenticate | [26] 20.27 | c1 | c1 | [26] 20.27 | c1 | c1 |
| 6 | WWW-Authenticate | [26] 20.44 | m | m | [26] 20.44 | m | m |
| c1: IF A.4/7 THEN m ELSE n/a - - support of authentication between UA and UA. |  |  |  |  |  |  |  |

Prerequisite A.5/9B - - MESSAGE response

Prerequisite: A.6/17 OR A.6/23 OR A.6/30 OR A.6/36 OR A.6/42 OR A.6/45 OR A.6/50 OR A.6/51 - - Additional for 404 (Not Found), 413 (Request Entity Too Large), 480(Temporarily not available), 486 (Busy Here), 500 (Internal Server Error), 503 (Service Unavailable), 600 (Busy Everywhere), 603 (Decline) response

Table A.62G: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 4 | Retry-After | [26] 20.33 | o | o | [26] 20.33 | o | o |

Table A.62H: Void

Prerequisite A.5/9B - - MESSAGE response

Prerequisite: A.6/20 - - Additional for 407 (Proxy Authentication Required) response

Table A.62I: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 3 | Proxy-Authenticate | [26] 20.27 | c1 | c1 | [26] 20.27 | c1 | c1 |
| 6 | WWW-Authenticate | [26] 20.44 | o | o | [26] 20.44 | o | o |
| c1: IF A.4/7 THEN m ELSE n/a - - support of authentication between UA and UA. |  |  |  |  |  |  |  |

Table A.62IA: Void

Prerequisite A.5/9B - - MESSAGE response

Prerequisite: A.6/25 - - Additional for 415 (Unsupported Media Type) response

Table A.62J: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept | [26] 20.1 | o.1 | o.1 | [26] 20.1 | m | m |
| 2 | Accept-Encoding | [26] 20.2 | o.1 | o.1 | [26] 20.2 | m | m |
| 3 | Accept-Language | [26] 20.3 | o.1 | o.1 | [26] 20.3 | m | m |
| o.1 At least one of these capabilities is supported. |  |  |  |  |  |  |  |

Prerequisite A.5/9B - - MESSAGE response

Prerequisite: A.6/26A - - Additional for 417 (Unknown Resource-Priority) response

Table A.62JA: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept-Resource-Priority | [116] 3.2 | c1 | c1 | [116] 3.2 | c1 | c1 |
| c1: IF A.4/70A THEN m ELSE n/a - - inclusion of MESSAGE, SUBSCRIBE, NOTIFY in communications resource priority for the session initiation protocol. |  |  |  |  |  |  |  |

Prerequisite A.5/9B - - MESSAGE response

Prerequisite: A.6/27 - - Additional for 420 (Bad Extension) response

Table A.62K: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 5 | Unsupported | [26] 20.40 | m | m | [26] 20.40 | m | m |

Prerequisite A.5/9B - - MESSAGE response

Prerequisite: A.6/28 OR A.6/41A - - Additional for 421 (Extension Required), 494 (Security Agreement Required) response

Table A.62L: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 3 | Security-Server | [48] 2 | x | x | [48] 2 | c1 | c1 |
| c1: IF A.4/37 THEN m ELSE n/a - - security mechanism agreement for the session initiation protocol. |  |  |  |  |  |  |  |

Table A.62M: Void

Prerequisite A.5/9B - - MESSAGE response

Prerequisite: A.6/29H - - Additional for 470 (Consent Needed) response

Table A.62MA: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Permission-Missing | [125] 5.9.3 | m | m | [125] 5.9.3 | m | m |

Prerequisite A.5/9B - - MESSAGE response

Prerequisite: A.6/46 - - Additional for 504 (Server Time-out) response

Table A.62MB: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Restoration-Info | subclause 7.2.11 | n/a | c1 | subclause 7.2.11 | n/a | n/a |
| c1: IF A.4/110 THEN o ELSE n/a - - HSS based P-CSCF restoration. |  |  |  |  |  |  |  |

Prerequisite A.5/9B - - MESSAGE response

Table A.62N: Supported message bodies within the MESSAGE response

| Item | Header | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 |  |  |  |  |  |  |  |

# *** Next Change ***

### A.2.2.2	Major capabilities

Table A.162: Major capabilities

| Item | Does the implementation support | Reference | RFC status | Profile status |
| --- | --- | --- | --- | --- |
|  | Capabilities within main protocol |  |  |  |
| 3 | initiate session release? | [26] 16 | x | c27 |
| 4 | stateless proxy behaviour? | [26] 16.11 | o.1 | c29 |
| 5 | stateful proxy behaviour? | [26] 16.2 | o.1 | c28 |
| 6 | forking of initial requests? | [26] 16.1 | c1 | c31 |
| 7 | support of indication of TLS connections in the Record-Route header on the upstream side? | [26] 16.7 | o | n/a |
| 8 | support of indication TLS connections in the Record-Route header on the downstream side? | [26] 16.7 | o | n/a |
| 8A | authentication between UA and proxy? | [26] 20.28, 22.3, [287] | o | c85 |
| 9 | insertion of date in requests and responses? | [26] 20.17 | o | o |
| 10 | suppression or modification of alerting information data? | [26] 20.4 | o | o |
| 11 | reading the contents of the Require header before proxying the request or response? | [26] 20.32 | o | o |
| 12 | adding or modifying the contents of the Require header before proxying the REGISTER request or response | [26] 20.32 | o | m |
| 13 | adding or modifying the contents of the Require header before proxying the request or response for methods other than REGISTER? | [26] 20.32 | o | o |
| 14 | being able to insert itself in the subsequent transactions in a dialog (record-routing)? | [26] 16.6 | o | c2 |
| 15 | the requirement to be able to use separate URIs in the upstream direction and downstream direction when record routeing? | [26] 16.7 | c3 | c3 |
| 16 | reading the contents of the Supported header before proxying the response? | [26] 20.37 | o | o |
| 17 | reading the contents of the Unsupported header before proxying the 420 response to a REGISTER? | [26] 20.40 | o | m |
| 18 | reading the contents of the Unsupported header before proxying the 420 response to a method other than REGISTER? | [26] 20.40 | o | o |
| 19 | the inclusion of the Error-Info header in 3xx - 6xx responses? | [26] 20.18 | o | o |
| 19A | reading the contents of the Organization header before proxying the request or response? | [26] 20.25 | o | o |
| 19B | adding or concatenating the Organization header before proxying the request or response? | [26] 20.25 | o | o |
| 19C | reading the contents of the Call-Info header before proxying the request or response? | [26] 20.9 | o | o |
| 19D | adding or concatenating the Call-Info header before proxying the request or response? | [26] 20.9 | o | o |
| 19E | delete Contact headers from 3xx responses prior to relaying the response? | [26] 20 | o | o |
| 19F | proxy reading the contents of a body or including a body in a request or response? | [26] | o | c88 |
| 19G | proxy modifying the content of a body | 3GPP TS 24.237 [8M] | n/a | c103 |
|  | Extensions |  |  |  |
| 20 | SIP INFO method and package framework? | [25] | o | o |
| 20A | legacy INFO usage? | [25] 2, 3 | o | o |
| 21 | reliability of provisional responses in SIP? | [27] | o | i |
| 22 | the REFER method? | [36] | o | o |
| 22A | clarifications for the use of REFER with RFC6665? | [231] | c113 | c113 |
| 22B | explicit subscriptions for the REFER method? | [232] | o | o |
| 23 | integration of resource management and SIP? | [30] [64] | o | i |
| 24 | the SIP UPDATE method? | [29] | c4 | i |
| 26 | SIP extensions for media authorization? | [31] | o | c7 |
| 27 | SIP specific event notification | [28] | o | i |
| 28 | a clarification on the use of GRUUs in the SIP event notification framework? | [232] | n/a | n/a |
| 29 | Session Initiation Protocol Extension Header Field for Registering Non-Adjacent Contacts | [35] | o | c6 |
| 30 | private extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks | [34] | o | m |
| 30A | act as first entity within the trust domain for asserted identity? | [34] | c5 | c9 |
| 30B | act as entity within trust network that can route outside the trust network? | [34] | c5 | c9 |
| 30C | act as entity passing on identity transparently independent of trust domain? | [34] | c5 | c96 |
| 31 | a privacy mechanism for the Session Initiation Protocol (SIP) | [33] | o | m |
| 31A | request of privacy by the inclusion of a Privacy header | [33] | n/a | n/a |
| 31B | application of privacy based on the received Privacy header | [33] | c10 | c12 |
| 31C | passing on of the Privacy header transparently | [33] | c10 | c13 |
| 31D | application of the privacy option "header" such that those headers which cannot be completely expunged of identifying information without the assistance of intermediaries are obscured? | [33] 5.1 | x | x |
| 31E | application of the privacy option "session" such that anonymization for the session(s) initiated by this message occurs? | [33] 5.2 | n/a | n/a |
| 31F | application of the privacy option "user" such that user level privacy functions are provided by the network? | [33] 5.3 | n/a | n/a |
| 31G | application of the privacy option "id" such that privacy of the network asserted identity is provided by the network? | [34] 7 | c11 | c12 |
| 31H | application of the privacy option "history" such that privacy of the History-Info header is provided by the network? | [66] 7.2 | c34 | c34 |
| 32 | Session Initiation Protocol Extension Header Field for Service Route Discovery During Registration | [38] | o | c30 |
| 33 | a messaging mechanism for the Session Initiation Protocol (SIP) | [50] | o | m |
| 34 | Compressing the Session Initiation Protocol | [55] | o | c7 |
| 35 | private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP)? | [52] | o | m |
| 36 | the P-Associated-URI header extension? | [52] 4.1, [52A] 4 | c14 | c15 |
| 37 | the P-Called-Party-ID header extension? | [52] 4.2, [52A] 4` | c14 | c16 |
| 38 | the P-Visited-Network-ID header extension? | [52] 4.3, [52A] 4, [52B] 3 | c14 | c17 |
| 39 | reading, or deleting the P-Visited-Network-ID header before proxying the request or response? | [52] 4.3 | c18 | n/a |
| 41 | the P-Access-Network-Info header extension? | [52] 4.4, [52A] 4, [234] 2 | c14 | c19 |
| 42 | act as first entity within the trust domain for access network information? | [52] 4.4 | c20 | c21 |
| 43 | act as subsequent entity within trust network for access network information that can route outside the trust network? | [52] 4.4 | c20 | c22 |
| 44 | the P-Charging-Function-Addresses header extension? | [52] 4.5, [52A] 4 | c14 | m |
| 44A | adding, deleting or reading the P-Charging-Function-Addresses header before proxying the request or response? | [52] 4.6 | c25 | c26 |
| 45 | the P-Charging-Vector header extension? | [52] 4.6, [52A] 4 | c14 | m |
| 46 | adding, deleting, reading or modifying the P-Charging-Vector header before proxying the request or response? | [52] 4.6 | c23 | c24 |
| 47 | security mechanism agreement for the session initiation protocol? | [48] | o | c7 |
| 47A | mediasec header field parameter for marking security mechanisms related to media? | Subclause 7.2A.7 | n/a | c99 |
| 48 | the Reason header field for the session initiation protocol | [34A] | o | c78 |
| 48A | carrying Q.850 codes in Reason header fields in SIP (Session Initiation Protocol) responses? | [130] | o | o |
| 48B | the location parameter for the SIP Reason header field? | [255] | o | o |
| 48C | Identity Header Error Handling (carrying STIR codes in Reason header fields in SIP responses)? | [294] | o | c130 |
| 49 | an extension to the session initiation protocol for symmetric response routeing | [56A] | o | m |
| 50 | caller preferences for the session initiation protocol? | [56B] | c33 | c33 |
| 50A | the proxy-directive within caller-preferences? | [56B] 9.1 | o.4 | o.4 |
| 50B | the cancel-directive within caller-preferences? | [56B] 9.1 | o.4 | o.4 |
| 50C | the fork-directive within caller-preferences? | [56B] 9.1 | o.4 | c32 |
| 50D | the recurse-directive within caller-preferences? | [56B] 9.1 | o.4 | o.4 |
| 50E | the parallel-directive within caller-preferences? | [56B] 9.1 | o.4 | c32 |
| 50F | the queue-directive within caller-preferences? | [56B] 9.1 | o.4 | o.4 |
| 51 | an event state publication extension to the session initiation protocol? | [70] | o | m |
| 52 | SIP session timer? | [58] | o | o |
| 53 | the SIP Referred-By mechanism? | [59] | o | o |
| 54 | the Session Inititation Protocol (SIP) "Replaces" header? | [60] | o | o |
| 55 | the Session Inititation Protocol (SIP) "Join" header? | [61] | o | o |
| 56 | the callee capabilities? | [62] | o | o |
| 57 | an extension to the session initiation protocol for request history information? | [66] | o | o |
| 57A | application of the "mp" optional header field parameter? | [66] | o | o |
| 57B | application of the "rc" optional header field parameter? | [66] | o | o |
| 57C | application of the "np" optional header field parameter? | [66] | o | o |
| 58 | Rejecting anonymous requests in the session initiation protocol? | [67] | o | o |
| 59 | session initiation protocol URIs for applications such as voicemail and interactive voice response | [68] | o | o |
| 59A | Session Initiation Protocol (SIP) cause URI parameter for service number translation? | [230] | c111 | c111 |
| 60 | the P-User-Database private header extension? | [82] | o | c95 |
| 61 | Session initiation protocol's non-INVITE transactions? | [84] | m | m |
| 62 | a uniform resource name for services | [69] | n/a | c35 |
| 63 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) | [93] | o | c36 |
| 65 | the Stream Control Transmission Protocol (SCTP) as a Transport for the Session Initiation Protocol (SIP)? | [96] | o | o (note2) |
| 66 | the SIP P-Profile-Key private header extension? | [97] | o | c41 |
| 66A | making the first query to the database in order to populate the P-Profile-Key header? | [97] | c38 | c39 |
| 66B | using the information in the P-Profile-Key header? | [97] | c38 | c40 |
| 67 | managing client initiated connections in SIP? | [92] 11 | o | c42 |
| 68 | indicating support for interactive connectivity establishment in SIP? | [102] | o | o |
| 69 | multiple-recipient MESSAGE requests in the session initiation protocol | [104] | n/a | n/a |
| 70 | SIP location conveyance? | [89] | o | c94 |
| 70A | addition or modification of location in a SIP method? | [89] | c44 | c45 |
| 70B | passes on locations in SIP method without modification? | [89] | c44 | c46 |
| 71 | referring to multiple resources in the session initiation protocol? | [105] | n/a | n/a |
| 72 | conference establishment using request-contained lists in the session initiation protocol? | [106] | n/a | n/a |
| 73 | subscriptions to request-contained resource lists in the session initiation protocol? | [107] | n/a | n/a |
| 74 | dialstring parameter for the session initiation protocol uniform resource identifier? | [103] | o | n/a |
| 75 | the P-Answer-State header extension to the session initiation protocol for the open mobile alliance push to talk over cellular? | [111] | o | c60 |
| 76 | the SIP P-Early-Media private header extension for authorization of early media? | [109] 8 | o | c51 |
| 77 | number portability parameters for the 'tel' URI? | [112] | o | c47 |
| 77A | assert or process carrier indication? | [112] | o | c48 |
| 77B | local number portability? | [112] | o | c50 |
| 79 | extending the session initiation protocol Reason header for preemption events | [115] | c79 | c79 |
| 80 | communications resource priority for the session initiation protocol? | [116] | o | c80 |
| 80A | inclusion of MESSAGE, SUBSCRIBE, NOTIFY in communications resource priority for the session initiation protocol? | [116] 4.2 | c82 | c82 |
| 80B | inclusion of CANCEL, BYE, REGISTER and PUBLISH in communications resource priority for the session initiation protocol? | [116] 4.2 | c82 | c82 |
| 81 | addressing an amplification vulnerability in session initiation protocol forking proxies? | [117] | c52 | c52 |
| 82 | the remote application identification of applying signalling compression to SIP | [79] 9.1 | o | c7 |
| 83 | a session initiation protocol media feature tag for MIME application subtypes? | [120] | o | c53 |
| 84 | SIP extension for the identification of services? | [121] | o | c54 |
| 84A | act as authentication entity within the trust domain for asserted service? | [121] | c55 | c56 |
| 85 | a framework for consent-based communications in SIP? | [125] | o | m |
| 86 | a mechanism for transporting user-to-user call control information in SIP? | [126] | o | c84 |
| 87 | the SIP P-Private-Network-Indication private-header (P-Header)? | [134] | o | o |
| 88 | the SIP P-Served-User private header in the 3GG IM CN subsystem? | [133] 6 | o | o |
| 89 | the SIP P-Served-User header extension for Originating CDIV session case? | [239] 4 | c126 | c126 |
| 90 | marking SIP messages to be logged? | [140] | o | m |
| 91 | the 199 (Early Dialog Terminated) response code | [142] | o | c90 |
| 92 | message body handling in SIP? | [150] | o | c89 |
| 93 | indication of support for keep-alive? | [143] | o | c51 |  |
| 94 | SIP Interface to VoiceXML Media Services? | [145] | o | c91 |
| 95 | common presence and instant messaging (CPIM): message format? | [151] | o | o |
| 96 | instant message disposition notification? | [157] | o | o |
| 97 | requesting answering modes for SIP? | [158] | o | o |
| 97A | adding, deleting or reading the Answer-Mode header or Priv-Answer-Mode before proxying the request or response? | [158] | o | c92 |
| 99 | the early session disposition type for SIP? | [74B] | i | i |
| 101 | The Session-ID header? | [162] | o | o |
| 102 | correct transaction handling for 2xx responses to Session Initiation Protocol INVITE requests? | [163] | m | m |
| 103 | addressing Record-Route issues in the Session Initiation Protocol (SIP)? | [164] | o | o |
| 104 | essential correction for IPv6 ABNF and URI comparison in RFC3261? | [165] | m | m |
| 105 | suppression of session initiation protocol REFER method implicit subscription? | [173] | o | c100 |
| 106 | Alert-Info URNs for the Session Initiation Protocol? | [175] | o | o |
| 107 | multiple registrations? | Subclause 3.1 | n/a | c101 |
| 108 | the SIP P-Refused-URI-List private-header? | [183] | o | c102 |
| 109 | request authorization through dialog Identification in the session initiation protocol? | [184] | o | o |
| 110 | indication of features supported by proxy? | [190] | o | c104 |
| 111 | registration of bulk number contacts? | [191] | o | c105 |
| 112 | media control channel framework? | [146] | n/a | n/a |
| 113 | S-CSCF restoration procedures? | Subclause 4.14 | n/a | n/a |
| 114 | SIP overload control? | [198] | o | o |
| 114A | feedback control? | [199] | c106 | c106 |
| 114B | distribution of load filters? | [201] | n/a | n/a |
| 115 | handling of a 380 (Alternative service) response? | Subclause 5.2.10 | n/a | n/a |
| 116 | indication of adjacent network in the Via "received-realm" header field parameter? | [208] | o | c107 |
| 117 | PSAP callback indicator? | [209] | o | c108 |
| 118 | SIP URI parameter to indicate traffic leg? | [225] | o | c109 |
| 119 | PCF or PCRF based P-CSCF restoration? | Subclause 4.14.2 | n/a | c110 |
| 120 | UDM/HSS or HSS based P-CSCF restoration? | Subclause 4.14.2 | n/a | c112 |
| 121 | the Relayed-Charge header field extension? | Subclause 7.2.12 | n/a | c114 |
| 122 | resource sharing? | Subclause 4.15 | n/a | c115 |
| 123 | the Cellular-Network-Info header extension? | Subclause 7.2.15 | n/a | c116 |
| 124 | the Priority-Share header field extension? | Subclause 7.2.16 | n/a | c127 |
| 125 | the Response-Source header field extension? | Subclause 7.2.17 | n/a | o |
| 126 | authenticated identity management in the Session Initiation Protocol? | [252] | o | c128 |
| 127 | a SIP response code for unwanted calls extension? | [254] | o | o |
| 128 | the Attestation-Info header field extension? | Subclause 7.2.18 | n/a | o |
| 129 | the Origination-Id header field extension? | Subclause 7.2.19 | n/a | o |
| 130 | Dynamic services interactions? | Subclause 4.18 | n/a | c128 |
| 131 | the Additional-Identity header field extension? | Subclause 7.2.20 | n/a | o |
| 132 | RLOS? | Subclause 4.19 | n/a | c129 |
| 133 | the Priority-Verstat header field extension? | Subclause 7.2.21 | n/a | c82 |
| c1: IF A.162/5 THEN o ELSE n/a - - stateful proxy behaviour. c2: IF A.3/2 OR A.3/9A OR A.3/4 OR A.3/13A OR A.3A/88 THEN m ELSE o - - P-CSCF, IBCF (THIG), S-CSCF, ISC gateway function (THIG), ATCF (proxy). c3: IF (A.162/7 AND NOT A.162/8) OR (NOT A.162/7 AND A.162/8) THEN m ELSE IF A.162/14 THEN o ELSE n/a - - TLS interworking with non-TLS else proxy insertion. c4: IF A.162/23 THEN m ELSE o - - integration of resource management and SIP. c5: IF A.162/30 THEN o ELSE n/a - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c6: IF A.3/2 OR A.3/9A OR A.3A/88 THEN m ELSE n/a - - P-CSCF, IBCF (THIG), ATFC (proxy). c7: IF A.3/2 AND (A.3D/1 OR A.3D/4) THEN m ELSE n/a - - P-CSCF and (IMS AKA plus IPsec ESP or SIP digest with TLS). c9: IF (A.3/2 OR A.3/4 OR A.3/9A OR A.3/13A) AND A.162/30 THEN m ELSE IF A.3/7C AND A.162/30 THEN o ELSE n/a - - P-CSCF or S-CSCF or IBCF (THIG) or ISC gateway function (THIG) or AS acting as proxy and extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks (NOTE 1). c10: IF A.162/31 THEN o.2 ELSE n/a - - a privacy mechanism for the Session Initiation Protocol (SIP). c11: IF A.162/31B THEN o ELSE x - - application of privacy based on the received Privacy header. c12: IF A.162/31 AND A.3/4 THEN m ELSE IF A.3/11 THEN o ELSE n/a - - S-CSCF, E-CSCF. c13: IF A.162/31 AND (A.3/2 OR A.3/3 OR A.3/7C OR A.3/9A OR A.3/13A OR A.3A/88) THEN m ELSE n/a - - P-CSCF, I-CSCF, AS acting as a SIP proxy or IBCF (THIG), ISC gateway function (THIG), ATCF (proxy). c14: IF A.162/35 THEN o.3 ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP). c15: IF A.162/35 AND (A.3/2 OR A.3/3 OR A.3/9A OR A.3/13A) THEN m ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP) and P-CSCF or I-CSCF or IBCF (THIG) or ISC gateway function (THIG). c16: IF A.162/35 AND (A.3/2 OR A.3/3 OR A.3/4 OR A.3/9A OR A.3/13A) THEN m ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP) and P-CSCF or I-CSCF or S-CSCF or IBCF (THIG) or ISC gateway function (THIG). c17: IF A.162/35 AND (A.3/2 OR A.3/3 OR A.3/9A OR A.3/13A) THEN m ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP) and P-CSCF or I-CSCF or IBCF (THIG) or ISC gateway function (THIG). c18: IF A.162/38 THEN o ELSE n/a - - the P-Visited-Network-ID header extension. c19: IF A.162/35 AND (A.3/2 OR A.3.3 OR A.3/4 OR A.3/7 THEN m ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP) and P-CSCF, I-CSCF, S-CSCF, AS acting as a proxy. c20: IF A.162/41 THEN o ELSE n/a - - the P-Access-Network-Info header extension. c21: IF A.162/41 AND A.3/2 THEN m ELSE n/a - - the P-Access-Network-Info header extension and P-CSCF. c22: IF A.162/41 AND A.3/4 THEN m ELSE n/a - - the P-Access-Network-Info header extension and S-CSCF. c23: IF A.162/45 THEN o ELSE n/a - - the P-Charging-Vector header extension. c24: IF A.162/45 THEN m ELSE n/a - - the P-Charging-Vector header extension. c25: IF A.162/44 THEN o ELSE n/a - - the P-Charging-Function-Addresses header extension. c26: IF A.162/44 THEN m ELSE n/a - - the P-Charging-Function Addresses header extension. c27: IF A.3/2 OR A.3/4 THEN m ELSE x - - P-CSCF or S-CSCF. c28: IF A.3/2 OR A.3/3 OR A.3/4 THEN m ELSE o.8 - - P-CSCF or I-CSCF or S-CSCF. c29: IF A.3/2 OR A.3/4 THEN n/a ELSE IF A.3/3 THEN o ELSE o.8 - - P-CSCF or S-CSCF or I-CSCF. c30: IF A.3/2 o ELSE i - - P-CSCF. c31: IF A.3/4 THEN m ELSE x - - S-CSCF. c32: IF A.3/4 THEN m ELSE o.4 - - S-CSCF. c33: IF A.162/50A OR A.162/50B OR A.162/50C OR A.162/50D OR A.162/50E OR A.162/50F THEN m ELSE n/a - - support of any directives within caller preferences for the session initiation protocol. c34: IF A.162/57 THEN m ELSE n/a - - an extension to the session initiation protocol for request history information. c35: IF A.3/2 OR A.3/11 THEN m ELSE IF A.3/7C OR A.3/9 OR A.3/13A THEN o ELSE n/a - - P-CSCF, E-CSCF, AS acting as proxy, IBCF, ISC gateway function (THIG). |  |  |  |  |
| c36: IF A.3/4 THEN m ELSE n/a - - S-CSCF. c38: IF A.162/66 THEN o ELSE n/a - - the SIP P-Profile-Key private header. c39: IF A.162/66 AND (A.3/3 OR A.3/9A) THEN m ELSE n/a - - the SIP P-Profile-Key private header, I-CSCF or IBCF (THIG). c40: IF A.162/66 AND A.3/4 THEN m ELSE n/a - - the SIP P-Profile-Key private header, S-CSCF. c41: IF A.3/3 OR A.3/4 OR A.3/9A THEN o ELSE n/a - - I-CSCF or S-CSCF or IBCF (THIG). c42: IF A.162/107 THEN m ELSE n/a - - multiple registrations. c44: IF A.162/70 THEN o.5 ELSE n/a - - SIP location conveyance. c45: IF A.3/11 THEN m ELSE IF A.162/70 AND A.3/7C THEN o.6 ELSE n/a - - E-CSCF, SIP location conveyance, AS acting as a SIP proxy. c46: IF A.162/70 AND A.3/2 OR A.3/3 OR A.3/5 OR A.3/10 OR A.3A/88 THEN m ELSE IF A.162/70 AND A.3/7C THEN o.6 ELSE n/a - - SIP location conveyance, P-CSCF, I-CSCF, S-CSCF, BGCF, additional routeing functionality, ATCF (proxy). c47: IF A.3/3 OR A.3/4 OR A.3/5 OR A.3/7C THEN o ELSE n/a - - I-CSCF, S-CSCF, BGCF, AS acting as a SIP proxy. c48: IF A.162/77 THEN m ELSE n/a - - number portability parameters for the 'tel' URI. c50: IF A.162/77 THEN m ELSE n/a - - number portability parameters for the 'tel' URI. c51: IF A.3/2 THEN m ELSE o - - P-CSCF. c52: IF A.162/6 THEN m ELSE o - - forking of initial requests. c53: IF A.3/4 THEN m ELSE n/a - - S-CSCF. c54: IF A.3/3 OR A.3/4 OR A.3/7 OR A.3/2 OR A.3/9A OR A.3/13A THEN m ELSE n/a - - I-CSCF, S-CSCF, BGCF, P-CSCF. IBCF (THIG), ISC gateway function (THIG). c55: IF A.162/84 THEN o ELSE n/a - - SIP extension for the identification of services. c56: IF A.3/4 AND A.162/84 THEN m ELSE n/a - - S-CSCF and SIP extension for the identification of services. c60: IF A.3/2 OR A.3/3 OR A.3/4 THEN o ELSE n/a - - P-CSCF, I-CSCF, S-CSCF. c78: IF A.3/2 OR A.3/4 OR A.3/9 OR A.162/79 OR A.162/3 THEN m ELSE o - - P-CSCF, S-CSCF, IBCF, extending the session initiation protocol Reason header for preemption events, initiate session release. c79: IF A.162/80 THEN o ELSE n/a - - communications resource priority for the session initiation protocol. c80: IF A.3/2 OR A.3/3 OR A.3/4 OR A.3/5 OR A.3/7C OR A.3/9A OR A.3/10 OR A.3/13A THEN o ELSE n/a - - P-CSCF, I-CSCF, S-CSCF, BGCF, AS acting as proxy, IBCF (THIG), additional routeing functionality, ISC gateway function (THIG). c82: IF A.162/80 THEN o ELSE n/a - - communications resource priority for the session initiation protocol. c84: A.3/2 OR A.3/3 OR A.3/4 OR A.3/5 OR A.3/7C OR A.3/9A OR A.3/10 OR A.3/11 OR A.3/13A THEN o ELSE n/a - - P-CSCF, I-CSCF, S-CSCF, BGCF, AS acting as proxy, IBCF (THIG), additional routeing functionality, E-CSCF, ISC gateway function (THIG). c85: IF A.3/2 OR A.3/3 OR A.3/4 THEN o ELSE x - - P-CSCF, I-CSCF, S-CSCF. c88: IF A.3/2 OR A.3/4 OR A.3/7 OR A.3/7C OR A.3/9C OR A.3/11 OR A.3/13C OR A.3A/88 THEN m ELSE o - - P-CSCF, S-CSCF, AS, AS acting as a SIP proxy, IBCF (Screening of SIP signalling), E-CSCF, ISC gateway function (Screening of SIP signalling), ATCF (proxy). c89: IF A.162/19F THEN m ELSE n/a - - proxy reading the contents of a body or including a body in a request or response. c90: IF A.3/4 THEN m ELSE i - - S-CSCF. c91: IF A.3/4 THEN o ELSE n/a - - S-CSCF. c92: IF A.162/92 THEN o ELSE n/a - - requesting answering modes for SIP. c94: IF A.3/11 THEN m ELSE o - - E-CSCF. c95: IF A.3/3 OR A.3/4 OR A.3/7C THEN o ELSE n/a - - I-CSCF, S-CSCF, AS acting as a SIP proxy. c96: IF A.3/2 OR A.3/11 OR A.3A/88 THEN m ELSE n/a - - P-CSCF, E-CSCF, ATCF (proxy). c99: IF A.3/2A AND (A.3D/30 OR A.3D/20A OR A.3D/20B OR A.3D/20C OR A.3D/31) THEN m ELSE n/a - - P-CSCF (IMS-ALG) and end-to-access-edge media security using SDES, end-to-access-edge media security for MSRP using TLS and certificate fingerprints, end-to-access-edge media security for BFCP using TLS and certificate fingerprints, end-to-access-edge media security for UDPTL using DTLS and certificate fingerprints, end-to-access-edge media security for RTP media using DTLS-SRTP and certificate fingerprints. c100: IF A.4/22 THEN o ELSE n/a - - the REFER method. c101: IF A.3/2 OR A.3/4 THEN m ELSE n/a - - P-CSCF, S-CSCF. |  |  |  |  |
| c102: IF A.3/9B THEN m ELSE IF A.3/7A OR A.3/7B OR A.3/7D THEN o ELSE n/a - - IBCF (IMS-ALG), AS acting as terminating UA, AS acting as originating UA, AS performing 3rd party call control. c103: IF A.3A/88 THEN m ELSE n/a - - ATCF (proxy). c104: IF A.3/2 OR A.3A/50A OR A.3A/83 OR A.3A/88 THEN m ELSE o - - P-CSCF, Multimedia telephony application server, SCC application server, ATCF (proxy). c105: IF A.3/2 OR A.3/3 OR A.3/9 THEN o ELSE n/a - - P-CSCF, I-CSCF, IBCF. c106: IF A.162/114 THEN o.9 ELSE n/a - - SIP overload control. c107: IF A.162/115 THEN o.9 ELSE n/a - - indication of adjacent network in the Via "received-realm" header field parameter. c108: IF A.3/2 OR A.3/3 OR A.3/4 OR A.3/7 OR A.3/9 THEN o ELSE n/a - - P-CSCF, I-CSCF, S-CSCF, AS, IBCF. c109: IF A.3/2 OR A.3/4 OR A.3/5 OR A.3/9 OR A.3/10 OR A.3A/83 OR A.3A/88 OR A.3/3 THEN o ELSE n/a - - P-CSCF, S-CSCF, BGCF, IBCF, Additional routeing functionality, SCC application server, ATCF (proxy), I-CSCF. c110: IF A.3/2 OR A.3/4 OR A.3/9 THEN o ELSE n/a - - P-CSCF, S-CSCF, IBCF. c111: IF A.162/59 THEN o ELSE n/a - - session initiation protocol URIs for applications such as voicemail and interactive voice response (NOTE 3). c112: IF A.3/2 OR A.3/4 OR A.3/9 THEN o ELSE n/a - - P-CSCF, S-CSCF, IBCF. c113: IF A.162/22 THEN m ELSE n/a - - the REFER method. c114: IF A.3/4 OR A.3/7 OR A.3A/102 THEN o ELSE n/a.-.-.S-CSCF, AS, transit function. c115: IF A.3/2 OR A.3/7C OR A.3/9 THEN o ELSE n/a - - P-CSCF, AS acting as a SIP proxy, IBCF. c116: IF A.3/2 OR A.3.3 OR A.3/4 OR A.3/7C OR A.3/9 OR A.3/11B OR A.3A/88 OR A.3/5 THEN m ELSE n/a - - P-CSCF, I-CSCF, S-CSCF, AS acting as a proxy, IBCF, E-CSCF acting as a SIP Proxy, ATCF (proxy), BGCF. c126: IF A.162/88 THEN o ELSE n/a - - the SIP P-Served-User private header for the 3GPP IM CN subsystem. c127: IF A.3/2 OR A.3/7C OR A.3/9 THEN o ELSE n/a - - P-CSCF, AS acting as a SIP proxy, IBCF. c128: IF A.3/7 OR A.3/9 THEN o ELSE n/a - - AS, IBCF. c129: IF A.3/2 OR A.3/4 THEN o ELSE n/a - - P-CSCF, S-CSCF. c130: IF A.3/4 OR A.3/7C OR A.3/9 THEN o ELSE n/a - - S-CSCF, AS acting as a SIP proxy, IBCF. o.1: It is mandatory to support at least one of these items. o.2: It is mandatory to support at least one of these items. o.3: It is mandatory to support at least one of these items. o.4 At least one of these capabilities is supported. o.5: It is mandatory to support exactly one of these items. o.6: It is mandatory to support exactly one of these items. o.7: It is mandatory to support at least one of these items. o.8 It is mandatory to support at least one of these items. o.9: At least one of these capabilities is supported. |  |  |  |  |
| NOTE 1: An AS acting as a proxy may be outside the trust domain, and therefore not able to support the capability for that reason; in this case it is perfectly reasonable for the header to be passed on transparently, as specified in the PDU parts of the profile. NOTE 2: Not applicable over Gm reference point (UE – P-CSCF). NOTE 3: AS performing a service number translation (e.g. Freephone) |  |  |  |  |

# *** Next Change ***

#### A.2.2.4.7	INVITE method

Prerequisite A.163/8 - - INVITE request

Table A.204: Supported header fields within the INVITE request

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept | [26] 20.1 | m | m | [26] 20.1 | i | i |
| 1A | Accept-Contact | [56B] 9.2 | c34 | c34 | [56B] 9.2 | c34 | c35 |
| 2 | Accept-Encoding | [26] 20.2 | m | m | [26] 20.2 | i | i |
| 3 | Accept-Language | [26] 20.3 | m | m | [26] 20.3 | i | i |
| 3A | Additional-Identity | 7.2.20 | n/a | c85 | 7.2.20 | n/a | c85 |
| 4 | Alert-Info | [26] 20.4 | c2 | c2 | [26] 20.4 | c3 | c3 |
| 5 | Allow | [26] 20.5 | m | m | [26] 20.5 | i | i |
| 6 | Allow-Events | [28] 8.2.2 | m | m | [28] 8.2.2 | c1 | c1 |
| 7 | Answer-Mode | [158] | c67 | c67 | [158] | c68 | c68 |
| 7A | Attestation-Info | 7.2.18 | n/a | c82 | 7.2.18 | n/a | c82 |
| 8 | Authorization | [26] 20.7 | m | m | [26] 20.7 | i | i |
| 9 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 10 | Call-Info | [26] 20.9 | m | m | [26] 20.9 | c12 | c12 |
| 10A | Cellular-Network-Info | 7.2.15 | n/a | c78 | 7.2.15 | n/a | c79 |
| 11 | Contact | [26] 20.10 | m | m | [26] 20.10 | i | i |
| 12 | Content-Disposition | [26] 20.11 | m | m | [26] 20.11 | i | c6 |
| 13 | Content-Encoding | [26] 20.12 | m | m | [26] 20.12 | i | c6 |
| 14 | Content-Language | [26] 20.13 | m | m | [26] 20.13 | i | c6 |
| 15 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 16 | Content-Type | [26] 20.15 | m | m | [26] 20.15 | i | c6 |
| 17 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 18 | Date | [26] 20.17 | m | m | [26] 20.17 | c4 | c4 |
| 19 | Expires | [26] 20.19 | m | m | [26] 20.19 | i | i |
| 19A | Feature-Caps | [190] | c73 | c73 | [190] | c73 | c73 |
| 20 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 20A | Geolocation | [89] 4.1 | c47 | c47 | [89] 4.1 | c48 | c48 |
| 20B | Geolocation-Routing | [89] 4.1 | c47 | c47 | [89] 4.1 | c48 | c48 |
| 20C | History-Info | [66] 4.1 | c43 | c43 | [66] 4.1 | c43 | c43 |
| 20D | Identity | [252] 4 | c81 | c81 | [252] 4 | c81 | c81 |
| 21 | In-Reply-To | [26] 20.21 | m | m | [26] 20.21 | i | i |
| 21A | Join | [61] 7.1 | c41 | c41 | [61] 7.1 | c42 | c42 |
| 21B | Max-Breadth | [117] 5.8 | c63 | c63 | [117] 5.8 | c64 | c64 |
| 22 | Max-Forwards | [26] 20.22 | m | m | [26] 20.22 | m | m |
| 23 | MIME-Version | [26] 20.24 | m | m | [26] 20.24 | i | c6 |
| 23A | Min-SE | [58] 5 | o | o | [58] 5 | o | o |
| 24 | Organization | [26] 20.25 | m | m | [26] 20.25 | c5 | c5 |
| 24AA | Origination-Id | 7.2.19 | n/a | c83 | 7.2.19 | n/a | c83 |
| 24A | P-Access-Network-Info | [52] 4.4, [234] 2 | c28 | c28 | [52] 4.4, [234] 2 | c29 | c30 |
| 24B | P-Asserted-Identity | [34] 9.1 | c15 | c15 | [34] 9.1 | c16 | c16 |
| 24C | P-Asserted-Service | [121] 4.1 | c53 | c53 | [121] 4.1 | c54 | c54 |
| 24D | P-Called-Party-ID | [52] 4.2 | c19 | c19 | [52] 4.2 | c20 | c21 |
| 24E | P-Charging-Function-Addresses | [52] 4.5 | c26 | c27 | [52] 4.5 | c26 | c27 |
| 24F | P-Charging-Vector | [52] 4.6 | c24 | c24 | [52] 4.6 | c25 | c25 |
| 24H | P-Early-Media | [109] 8 | o | c50 | [109] 8 | o | c50 |
| 25 | P-Media-Authorization | [31] 5.1 | c9 | x | [31] 5.1 | n/a | n/a |
| 25A | P-Preferred-Identity | [34] 9.2 | x | c69 | [34] 9.2 | c14 | c14 |
| 25B | P-Preferred-Service | [121] 4.2 | x | x | [121] 4.2 | c52 | c52 |
| 25C | P-Private-Network-Indication | [134] | c59 | c59 | [134] | c59 | c59 |
| 25D | P-Profile-Key | [97] 5 | c45 | c45 | [97] 5 | c46 | c46 |
| 25E | P-Served-User | [133] 6 | c60 | c60 | [133] 6 | c60 | c60 |
| 25F | P-User-Database | [82] 4 | c44 | c44 | [82] 4 | c44 | c44 |
| 25G | P-Visited-Network-ID | [52] 4.3 | c22 | o | [52] 4.3 | c23 | o |
| 26 | Priority | [26] 20.26 | m | m | [26] 20.26 | i | c74 |
| 26AA | Priority-Share | Subclause 7.2.16 | n/a | c80 | Subclause 7.2.16 | n/a | c80 |
| 26A | Privacy | [33] 4.2 | c17 | c17 | [33] 4.2 | c18 | c18 |
| 26AB | Priority-Verstat | Subclause 7.2.21 | n/a | c86 | Subclause 7.2.21 | n/a | c86 |
| 26B | Priv-Answer-Mode | [158] | c67 | c67 | [158] | c68 | c68 |
| 27 | Proxy-Authorization | [26] 20.28 | m | m | [26] 20.28 | c13 | c13 |
| 28 | Proxy-Require | [26] 20.29, [34] 4 | m | m | [26] 20.29, [34] 4 | m | m |
| 28A | Reason | [34A] 2 | c32 | c32 | [34A] 2 | c33 | c33 |
| 29 | Record-Route | [26] 20.30 | m | m | [26] 20.30 | c11 | c11 |
| 29A | Recv-Info | [25] 5.2.3 | c65 | c65 | [25] 5.2.3 | c66 | c66 |
| 30 | Referred-By | [59] 3 | c37 | c37 | [59] 3 | c38 | c38 |
| 31 | Reject-Contact | [56B] 9.2 | c34 | c34 | [56B] 9.2 | c34 | c35 |
| 31A | Relayed-Charge | 7.2.12 | n/a | c76 | 7.2.12 | n/a | c76 |
| 31B | Replaces | [60] 6.1 | c39 | c39 | [60] 6.1 | c40 | c40 |
| 31C | Reply-To | [26] 20.31 | m | m | [26] 20.31 | i | i |
| 31D | Request-Disposition | [56B] 9.1 | c34 | c34 | [56B] 9.1 | c34 | c34 |
| 32 | Require | [26] 20.32 | m | m | [26] 20.32 | c7 | c7 |
| 32A | Resource-Priority | [116] 3.1 | c49 | c49 | [116] 3.1 | c49 | c49 |
| 32B | Restoration-Info | Subclause 7.2.11 | n/a | c75 | Subclause 7.2.11 | n/a | c75 |
| 32C | Resource-Share | Subclause 4.15 | n/a | c77 | Subclause 4.15 | n/a | c77 |
| 33 | Route | [26] 20.34 | m | m | [26] 20.34 | m | m |
| 33A | Security-Client | [48] 2.3.1 | x | x | [48] 2.3.1 | c31 | c31 |
| 33B | Security-Verify | [48] 2.3.1 | x | x | [48] 2.3.1 | c31 | c31 |
| 33DA | Service-Interact-Info | Subclause 7.2.14 | n/a | c84 | Subclause 7.2.14 | n/a | c84 |
| 33D | Session-Expires | [58] 4 | c36 | c36 | [58] 4 | c36 | c36 |
| 33E | Session-ID | [162] | c70 | c70 | [162] | c70 | c70 |
| 34 | Subject | [26] 20.36 | m | m | [26] 20.36 | i | i |
| 35 | Supported | [26] 20.37 | m | m | [26] 20.37 | c8 | c8 |
| 35A | Target-Dialog | [184] 7 | c71 | c71 | [184] 7 | c72 | c72 |
| 36 | Timestamp | [26] 20.38 | m | m | [26] 20.38 | i | i |
| 37 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 37A | Trigger-Consent | [125] 5.11.2 | c55 | c55 | [125] 5.11.2 | c56 | c56 |
| 38 | User-Agent | [26] 20.41 | m | m | [26] 20.41 | i | i |
| 38A | User-to-User | [126] 7 | c57 | c57 | [126] 7 | c58 | c58 |
| 39 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| c1: IF A.4/20 THEN m ELSE i - - SIP specific event notification extension. c2: IF A.162/10 THEN n/a ELSE m - - suppression or modification of alerting information data. c3: IF A.162/10 THEN m ELSE i - - suppression or modification of alerting information data. c4: IF A.162/9 THEN m ELSE i - - insertion of date in requests and responses. c5: IF A.162/19A OR A.162/19B THEN m ELSE i - - reading, adding or concatenating the Organization header. c6: IF A.3/2 OR A.3/4 THEN m ELSE i - - P-CSCF or S-CSCF. c7: IF A.162/11 OR A.162/13 THEN m ELSE i - - reading the contents of the Require header before proxying the request or response or adding or modifying the contents of the Require header before proxying the request or response for methods other than REGISTER. c8: IF A.162/16 THEN m ELSE i - - reading the contents of the Supported header before proxying the response. c9: IF A.162/26 THEN m ELSE n/a - - SIP extensions for media authorization. c11: IF A.162/14 THEN m ELSE i - - the requirement to be able to insert itself in the subsequent transactions in a dialog. c12: IF A.162/19C OR A.162/19D THEN m ELSE i - - reading, adding or concatenating the Call-Info header. c13: IF A.162/8A THEN m ELSE i - - authentication between UA and proxy. c14: IF A.162/30A OR A.162/30C THEN m ELSE n/a - - act as first entity within the trust domain for asserted identity, act as entity passing on identity transparently independent of trust domain. c15: IF A.162/30 THEN m ELSE n/a - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c16: IF A.162/30A or A.162/30B THEN m ELSE i - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks or subsequent entity within trust network that can route outside the trust network. c17: IF A.162/31 THEN m ELSE n/a - - a privacy mechanism for the Session Initiation Protocol (SIP). c18: IF A.162/31D OR A.162/31G THEN m ELSE IF A.162/31C THEN i ELSE n/a - - application of the privacy option "header" or application of the privacy option "id" or passing on of the Privacy header transparently. c19: IF A.162/37 THEN m ELSE n/a - - the P-Called-Party-ID header extension. c20: IF A.162/37 THEN i ELSE n/a - - the P-Called-Party-ID header extension. c21: IF A.162/37 AND A.3/2 THEN m ELSE IF A.162/37 AND (A.3/3 OR A.3/9A) THEN i ELSE n/a - - the P-Called-Party-ID header extension and P-CSCF or (I-CSCF or IBCF (THIG)). c22: IF A.162/38 THEN m ELSE n/a - - the P-Visited-Network-ID header extension. c23: IF A.162/39 THEN m ELSE i - - reading, or deleting the P-Visited-Network-ID header before proxying the request or response. c24: IF A.162/45 THEN m ELSE n/a - - the P-Charging-Vector header extension. c25: IF A.162/46 THEN m ELSE IF A.162/45 THEN i ELSE n/a - - adding, deleting, reading or modifying the P-Charging-Vector header before proxying the request or response or the P-Charging-Vector header extension. c26: IF A.162/44 THEN m ELSE n/a - - the P-Charging-Function-Addresses header extension. c27: IF A.162/44A THEN m ELSE IF A.162/44 THEN i ELSE n/a - - adding, deleting or reading the P-Charging-Function-Addresses header before proxying the request or response, or the P-Charging-Function-Addresses header extension. c28: IF A.162/43 THEN x ELSE IF A.162/41 THEN m ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension. c29: IF A.162/43 THEN m ELSE IF A.162/41 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension. c30: IF A.162/43 OR (A.162/41 AND A.3/2) THEN m ELSE IF A.162/41 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension (with or without P-CSCF). c31: IF A.162/47 OR A.162/47A THEN m ELSE n/a - - security mechanism agreement for the session initiation protocol or mediasec header field parameter for marking security mechanisms related to media. c32: IF A.162/48 THEN m ELSE n/a - - the Reason header field for the session initiation protocol. c33: IF A.162/48 THEN i ELSE n/a - - the Reason header field for the session initiation protocol. c34: IF A.162/50 THEN m ELSE n/a - - caller preferences for the session initiation protocol. c35: IF A.162/50 AND A.4/3 THEN m ELSE IF A.162/50 AND NOT A.4/3 THEN i ELSE n/a - - caller preferences for the session initiation protocol, and S-CSCF. c36: IF A.162/52 THEN m ELSE n/a - - the SIP session timer. c37: IF A.162/53 THEN i ELSE n/a - - the SIP Referred-By mechanism. c38: IF A.162/53 THEN m ELSE n/a - - the SIP Referred-By mechanism. c39: IF A.162/54 THEN m ELSE n/a - - the Session Inititation Protocol (SIP) "Replaces" header. c40: IF A.162/54 THEN i ELSE n/a - - the Session Inititation Protocol (SIP) "Replaces" header. |  |  |  |  |  |  |  |
| c41: IF A.162/55 THEN m ELSE n/a - - the Session Inititation Protocol (SIP) "Join" header. c42: IF A.162/55 THEN i ELSE n/a - - the Session Inititation Protocol (SIP) "Join" header. c43: IF A.162/57 THEN m ELSE n/a - - an extension to the session initiation protocol for request history information. c44: IF A.162/60 THEN m ELSE n/a - - the P-User-Database private header extension. c45: IF A.162/66A THEN m ELSE n/a - - making the first query to the database in order to populate the P-Profile-Key header. c46: IF A.162/66B THEN m ELSE n/a - - using the information in the P-Profile-Key header. c47: IF A.162/70 THEN m ELSE n/a - - SIP location conveyance. c48: IF A.162/70A THEN m ELSE IF A.162/70B THEN i ELSE n/a - - addition or modification of location in a SIP method, passes on locations in SIP method without modification. c49: IF A.162/80 THEN m ELSE n/a - - communications resource priority for the session initiation protocol. c50: IF A.162/76 THEN m ELSE n/a - - the SIP P-Early-Media private header extension for authorization of early media. c52: IF A.162/84A THEN m ELSE n/a - - act as authentication entity within the trust domain for asserted service. c53: IF A.162/84 THEN m ELSE n/a - - SIP extension for the identification of services. c54: IF A.162/84 OR A.162/30B THEN m ELSE i - - SIP extension for the identification of services or subsequent entity within trust network that can route outside the trust network. c55: IF A.162/85 THEN m ELSE n/a - - a framework for consent-based communications in SIP. c56: IF A.162/85 THEN i ELSE n/a - - a framework for consent-based communications in SIP. c57: IF A.162/86 THEN m - - transporting user to user information for call centers using SIP. c58: IF A.162/86 THEN i - - transporting user to user information for call centers using SIP. c59: IF A.162/87 THEN m ELSE n/a - - the SIP P-Private-Network-Indication private-header (P-Header). c60: IF A.162/88 THEN m - - the SIP P-Served-User private header. c63: IF A.162/81 THEN m ELSE n/a - - addressing an amplification vulnerability in session initiation protocol forking proxies. c64: IF A.162/81 AND A.162/6 THEN m ELSE IF A.162/81 AND NOT A.162/6 THEN i ELSE n/a - - addressing an amplification vulnerability in session initiation protocol forking proxies, forking of initial requests. c65: IF A.162/20 THEN m ELSE n/a - - SIP INFO method and package framework. c66: IF A.162/20 THEN i ELSE n/a - - SIP INFO method and package framework. c67: IF A.162/97 THEN m ELSE n/a - - requesting answering modes for SIP. c68: IF NOT A.162/97 THEN n/a ELSE IF A.162/97A THEN m ELSE i - - requesting answering modes for SIP, adding, deleting or reading the Answer-Mode header or Priv-Answer-Mode header before proxying the request or response. c69: IF A.162/30C THEN m ELSE x - - act as entity passing on identity transparently independent of trust domain. c70: IF A.162/101 THEN m ELSE n/a - - the Session-ID header. c71: IF A.162/109 THEN m ELSE n/a - - request authorization through dialog Identification in the session initiation protocol. c72: IF A.162/109 THEN i ELSE n/a - - request authorization through dialog Identification in the session initiation protocol. c73: IF A.162/110 THEN m ELSE n/a - - indication of features supported by proxy. c74: IF A.162/115 THEN m ELSE i - - PSAP callback indicator. c75: IF A.162/119 THEN o ELSE n/a - - PCRF based P-CSCF restoration. c76: IF A.162/121 THEN m ELSE n/a - - the Relayed-Charge header field extension. c77: IF A.162/122 THEN o ELSE n/a - - resource sharing. c78: IF A.162/43 THEN x ELSE IF A.162/123 THEN m ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the Cellular-Network-Info header extension. c79: IF A.162/43 OR (A.162/41A AND A.3/2) THEN m ELSE IF A.162/123 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the Cellular-Network-Info header extension (with or without P-CSCF). c80: IF A.162/124 THEN o ELSE n/a - - priority sharing. c81: IF A.162/126 THEN o ELSE n/a - - authenticated identity management in the Session Initiation Protocol. c82: IF A.162/128 THEN o ELSE n/a - - the Attestation-Info header field extension. c83: IF A.162/129 THEN o ELSE n/a - - the Origination-Id header field extension. c84: IF A.162/130 THEN m ELSE n/a - - Dynamic services interactions. c85: IF A.162/131 THEN o ELSE n/a - - the Additional-Identity header field extension. c86: IF A.162/80 AND A.162/133 THEN o ELSE n/a - - communications resource priority for the session initiation protocol, the Priority-Verstat header field extension. |  |  |  |  |  |  |  |
| NOTE: c1 refers to the UA role major capability as this is the case of a proxy that also acts as a UA specifically for SUBSCRIBE and NOTIFY. |  |  |  |  |  |  |  |

Prerequisite A.163/8 - - INVITE request

Table A.205: Supported message bodies within the INVITE request

| Item | Header | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | XML Schema for PSTN | [11B] | n/a | c1 | [11B] | n/a | i |
| 2 | application/vnd.3gpp.ussd | [8W] | n/a | m | [8W] | n/a | i |
| 3 | application/vnd.3gpp.mcptt-info+xml | [8ZE] | n/a | c1 | [8ZE] | n/a | i |
| c1: A.3/3 OR A.3/4 OR A.3/5 OR A.3/7C OR A.3/9A OR A.3/10 OR A.3/11 OR A.3/13A THEN m ELSE n/a - - I-CSCF, S-CSCF, BGCF, AS acting as proxy, IBCF (THIG), additional routeing functionality, E-CSCF, ISC gateway function (THIG). |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/1 - - Additional for 100 (Trying) response

Table A.206: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 2 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 3 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 4 | Date | [26] 20.17 | c1 | c1 | [26] 20.17 | c2 | c2 |
| 5 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 6 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 7 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| c1: IF (A.162/9 AND A.162/5) OR A.162/4 THEN m ELSE n/a - - stateful proxy behaviour that inserts date, or stateless proxies. c2: IF A.162/4 THEN i ELSE m - - Stateless proxy passes on. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response for all remaining status-codes

Table A.207: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 0A | Allow | [26] 20.5 | m | m | [26] 20.5 | i | i |
| 1 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 1A | Call-Info | [26] 20.9 | m | m | [26] 20.9 | c4 | c4 |
| 1B | Cellular-Network-Info | 7.2.15 | n/a | c27 | 7.2.15 | n/a | c28 |
| 2 | Content-Disposition | [26] 20.11 | m | m | [26] 20.11 | i | c3 |
| 3 | Content-Encoding | [26] 20.12 | m | m | [26] 20.12 | i | c3 |
| 4 | Content-Language | [26] 20.13 | m | m | [26] 20.13 | i | c3 |
| 5 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 6 | Content-Type | [26] 20.15 | m | m | [26] 20.15 | i | c3 |
| 7 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 8 | Date | [26] 20.17 | m | m | [26] 20.17 | c1 | c1 |
| 8A | Expires | [26] 20.19 | m | m | [26] 20.19 | i | i |
| 9 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 9A | Geolocation-Error | [89] 4.3 | c24 | c24 | [89] 4.3 | c24 | c24 |
| 9B | History-Info | [66] 4.1 | c17 | c17 | [66] 4.1 | c17 | c17 |
| 10 | MIME-Version | [26] 20.24 | m | m | [26] 20.24 | i | c3 |
| 11 | Organization | [26] 20.25 | m | m | [26] 20.25 | c2 | c2 |
| 11A | P-Access-Network-Info | [52] 4.4, [52A] 4, [234] 2 | c14 | c14 | [52] 4.4, [52A] 4, [234] 2 | c15 | c15 |
| 11B | P-Asserted-Identity | [34] 9.1 | c6 | c6 | [34] 9.1 | c7 | c7 |
| 11C | P-Charging-Function-Addresses | [52] 4.5, [52A] 4 | c12 | c12 | [52] 4.5, [52A] 4 | c13 | c13 |
| 11D | P-Charging-Vector | [52] 4.6, [52A] 4 | c10 | c10 | [52] 4.6, [52A] 4 | c11 | c11 |
| 11F | P-Preferred-Identity | [34] 9.2 | x | x | [34] 9.2 | c5 | n/a |
| 11G | Privacy | [33] 4.2 | c8 | c8 | [33] 4.2 | c9 | c9 |
| 11H | Relayed-Charge | 7.2.12 | n/a | c26 | 7.2.12 | n/a | c26 |
| 11I | Reply-To | [26] 20.31 | m | m | [26] 20.31 | i | i |
| 11J | Require | [26] 20.32 | m | m | [26] 20.32 | c16 | c16 |
| 11K | Server | [26] 20.35 | m | m | [26] 20.35 | i | i |
| 11LA | Service-Interact-Info | Subclause 7.2.14 | n/a | c29 | Subclause 7.2.14 | n/a | c29 |
| 11L | Session-ID | [162] | c25 | c25 | [162] | c25 | c25 |
| 12 | Timestamp | [26] 20.38 | m | m | [26] 20.38 | i | i |
| 13 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 13A | User-Agent | [26] 20.41 | m | m | [26] 20.41 | i | i |
| 13B | User-to-User | [126] 7 | c20 | c20 | [126] 7 | c21 | c21 |
| 14 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| 15 | Warning | [26] 20.43 | m | m | [26] 20.43 | i | i |
| c1: IF A.162/9 THEN m ELSE i - - insertion of date in requests and responses. c2: IF A.162/19A OR A.162/19B THEN m ELSE i - - reading, adding or concatenating the Organization header. c3: IF A.3/2 OR A.3/4 THEN m ELSE i - - P-CSCF or S-CSCF. c4: IF A.162/19C OR A.162/19D THEN m ELSE i - - reading, adding or concatenating the Call-Info header. c5: IF A.162/30A THEN m ELSE n/a - - act as first entity within the trust domain for asserted identity. c6: IF A.162/30 THEN m ELSE n/a - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c7: IF A.162/30A or A.162/30B THEN m ELSE i - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks or subsequent entity within trust network that can route outside the trust network. c8: IF A.162/31 THEN m ELSE n/a - - a privacy mechanism for the Session Initiation Protocol (SIP). c9: IF A.162/31D OR A.162/31G THEN m ELSE IF A.162/31C THEN i ELSE n/a - - application of the privacy option "header" or application of the privacy option "id" or passing on of the Privacy header transparently. c10: IF A.162/45 THEN m ELSE n/a - - the P-Charging-Vector header extension. c11: IF A.162/46 THEN m ELSE IF A.162/45 THEN i ELSE n/a - - adding, deleting, reading or modifying the P-Charging-Vector header before proxying the request or response or the P-Charging-Vector header extension. c12: IF A.162/44 THEN m ELSE n/a - - the P-Charging-Function-Addresses header extension. c13: IF A.162/44A THEN m ELSE IF A.162/44 THEN i ELSE n/a - - adding, deleting or reading the P-Charging-Function-Addresses header before proxying the request or response, or the P-Charging-Function-Addresses header extension. c14: IF A.162/43 THEN x ELSE IF A.162/41 THEN m ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension. c15: IF A.162/43 THEN m ELSE IF A.162/41 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension. c16: IF A.162/11 OR A.162/13 THEN m ELSE i - - reading the contents of the Require header before proxying the request or response or adding or modifying the contents of the Require header before proxying the request or response for methods other than REGISTER. c17: IF A.162/57 THEN m ELSE n/a - - an extension to the session initiation protocol for request history information. c18: IF A.162/70 THEN m ELSE n/a - - SIP location conveyance. c19: IF A.162/70A THEN m ELSE IF A.162/70B THEN i ELSE n/a - - addition or modification of location in a SIP method, passes on locations in SIP method without modification. c20: IF A.162/86 THEN m - - transporting user to user information for call centers using SIP. c21: IF A.162/86 THEN i - - transporting user to user information for call centers using SIP. c24: IF A.4/60 THEN m ELSE n/a - - SIP location conveyance. c25: IF A.162/101 THEN m ELSE n/a - - the Session-ID header. c26: IF A.162/121 THEN m ELSE n/a - - the Relayed-Charge header field extension. c27: IF A.162/43 THEN x ELSE IF A.162/123 THEN m ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the Cellular-Network-Info header extension. c28: IF A.162/43 THEN m ELSE IF A.162/123 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the Cellular-Network-Info header extension. c29: IF A.162/130 THEN m ELSE n/a - - Dynamic services interactions. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/101A - - Additional for 18x response

Table A.208: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 4 | Contact | [26] 20.10 | m | m | [26] 20.10 | i | i |
| 4A | Feature-Caps | [190] | c19 | c19 | [190] | c19 | c19 |
| 5 | P-Answer-State | [111] | c13 | c13 | [111] | c14 | c14 |
| 5A | P-Early-Media | [109] 8 | o | c11 | [109] 8 | o | c11 |
| 6 | P-Media-Authorization | [31] 5.1 | c9 | x | [31] 5.1 | n/a | n/a |
| 6AA | Priority-Share | Subclause 7.2.16 | n/a | c21 | Subclause 7.2.16 | n/a | c21 |
| 6A | Reason | [130], [294] | o | c18 | [130], [294] | o | c18 |
| 7 | Record-Route | [26] 20.30 | m | m | [26] 20.30 | c15 | c15 |
| 8 | Recv-Info | [25] 5.2.3 | c16 | c16 | [25] 5.2.3 | c17 | c17 |
| 8A | Resource-Share | Subclause 4.15 | n/a | c20 | Subclause 4.15 | n/a | c20 |
| 9 | RSeq | [27] 7.1 | m | m | [27] 7.1 | i | i |
| c9: IF A.162/26 THEN m ELSE n/a - - SIP extensions for media authorization. c11: IF A.162/76 THEN m ELSE n/a - - the SIP P-Early-Media private header extension for authorization of early media. c13: IF A.162/75 THEN m ELSE n/a - - the P-Answer-State header extension to the session initiation protocol for the open mobile alliance push to talk over cellular. c14: IF A.162/75 THEN i ELSE n/a - - the P-Answer-State header extension to the session initiation protocol for the open mobile alliance push to talk over cellular. c15: IF A.162/14 THEN m ELSE i - - the requirement to be able to insert itself in the subsequent transactions in a dialog. c16: IF A.162/20 THEN m ELSE n/a - - SIP INFO method and package framework. c17: IF A.162/20 THEN i ELSE n/a - - SIP INFO method and package framework. c18: IF A.162/48A OR A.162/48C THEN o ELSE n/a - - use of the Reason header field in Session Initiation Protocol (SIP) responses (carrying Q.850 or STIR codes). c19: IF A.162/110 THEN m ELSE n/a - - indication of features supported by proxy. c20: IF A.162/122 THEN o ELSE n/a - - resource sharing. c21: IF A.162/124 THEN o ELSE n/a - - priority sharing. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/2 - - Additional for 180 (Ringing) response

Table A.208A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Alert-Info | [26] 20.4 | m | m | [26] 20.4 | i | i |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/5A - - Additional for 199 (Early Dialog Terminated) response

Table A.208B: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 4 | Contact | [26] 20.10 | m | m | [26] 20.10 | i | i |
| 5 | Reason | [130], [294] | o | c18 | [130], [294] | o | c18 |
| 7 | Record-Route | [26] 20.30 | m | m | [26] 20.30 | c15 | c15 |
| 8 | Recv-Info | [25] 5.2.3 | c16 | c16 | [25] 5.2.3 | c17 | c17 |
| 9 | RSeq | [27] 7.1 | m | m | [27] 7.1 | i | i |
| c15: IF A.162/14 THEN m ELSE i - - the requirement to be able to insert itself in the subsequent transactions in a dialog. c16: IF A.162/20 THEN m ELSE n/a - - SIP INFO method and package framework. c17: IF A.162/20 THEN i ELSE n/a - - SIP INFO method and package framework. c18: IF A.162/48A OR A.162/48C THEN o ELSE n/a - - use of the Reason header field in Session Initiation Protocol (SIP) responses (carrying Q.850 or STIR codes in SIP responses). |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/102 - - Additional for 2xx response

Table A.209: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept | [26] 20.1 | m | m | [26] 20.1 | i | i |
| 1A | Accept-Encoding | [26] 20.2 | m | m | [26] 20.2 | i | i |
| 1B | Accept-Language | [26] 20.3 | m | m | [26] 20.3 | i | i |
| 1C | Accept-Resource-Priority | [116] 3.2 | c12 | c12 | [116] 3.2 | c12 | c12 |
| 2 | Allow-Events | [28] 8.2.2 | m | m | [28] 8.2.2 | c1 | c1 |
| 3 | Answer-Mode | [158] | c19 | c19 | [158] | c20 | c20 |
| 4 | Authentication-Info | [26] 20.6 | m | m | [26] 20.6 | i | i |
| 6 | Contact | [26] 20.10 | m | m | [26] 20.10 | i | i |
| 6A | Feature-Caps | [190] | c22 | c22 | [190] | c22 | c22 |
| 7 | P-Answer-State | [111] | c13 | c13 | [111] | c14 | c14 |
| 7A | P-Visited-Network-ID | [52B] 3 | o | o | [52B] 3 | o | o |
| 8 | P-Media-Authorization | [31] 5.1 | c9 | x | [31] 5.1 | n/a | n/a |
| 8AA | Priority-Share | Subclause 7.2.16 | n/a | c24 | Subclause 7.2.16 | n/a | c24 |
| 8A | Priv-Answer-Mode | [158] | c19 | c19 | [158] | c20 | c20 |
| 9 | Record-Route | [26] 20.30 | m | m | [26] 20.30 | c3 | c3 |
| 9A | Recv-Info | [25] 5.2.3 | c17 | c17 | [25] 5.2.3 | c18 | c18 |
| 9B | Resource-Share | Subclause 4.15 | n/a | c23 | Subclause 4.15 | n/a | c23 |
| 10 | Session-Expires | [58] 4 | c11 | c11 | [58] 4 | c11 | c11 |
| 13 | Supported | [26] 20.37 | m | m | [26] 20.37 | i | i |
| c1: IF A.4/20 THEN m ELSE i - - SIP specific event notification extension. c3: IF A.162/14 THEN m ELSE i - - the requirement to be able to insert itself in the subsequent transactions in a dialog. c9: IF A.162/26 THEN m ELSE n/a - - SIP extensions for media authorization. c11: IF A.162/52 THEN m ELSE n/a - - the SIP session timer. c12: IF A.162/80 THEN m ELSE n/a - - communications resource priority for the session initiation protocol. c13: IF A.162/75 THEN m ELSE n/a - - the P-Answer-State header extension to the session initiation protocol for the open mobile alliance push to talk over cellular. c14: IF A.162/75 THEN i ELSE n/a - - the P-Answer-State header extension to the session initiation protocol for the open mobile alliance push to talk over cellular. c17: IF A.162/20 THEN m ELSE n/a - - SIP INFO method and package framework. c18: IF A.162/20 THEN i ELSE n/a - - SIP INFO method and package framework. c19: IF A.162/97 THEN m ELSE n/a - - requesting answering modes for SIP. c20: IF NOT A.162/97 THEN n/a ELSE IF A.162/97A THEN m ELSE i - - requesting answering modes for SIP, adding, deleting or reading the Answer-Mode header or Priv-Answer-Mode header before proxying the request or response. c22: IF A.162/110 THEN m ELSE n/a - - indication of features supported by proxy. c23: IF A.162/122 THEN o ELSE n/a - - resource sharing. c24: IF A.162/124 THEN o ELSE n/a - - priority sharing. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/103 OR A.164/104 OR A.164/105 OR A.164/106 - - Additional for 3xx – 6xx response

Table A.209A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Error-Info | [26] 20.18 | m | m | [26] 20.18 | i | i |
| 2 | Reason | [130], [294] | o | c1 | [130], [294] | o | c1 |
| 3 | Response-Source | 7.2.17 | n/a | c2 | 7.2.17 | n/a | c2 |
| c1: IF A.162/48A OR A.162/48C THEN o ELSE n/a - - use of the Reason header field in Session Initiation Protocol (SIP) responses (carrying Q.850 or STIR codes). c2: IF A.162/125 THEN o ELSE n/a - - use of the Response-Source header field in SIP error responses. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/103 OR A.164/35 - - Additional for 3xx or 485 (Ambiguous) response

Table A.210: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 4 | Contact | [26] 20.10 | m | m | [26] 20.10 | c1 | c1 |
| c1: IF A.162/19E THEN m ELSE i - - deleting Contact headers. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/14 - - Additional for 401 (Unauthorized) response

Table A.211: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 6 | Proxy-Authenticate | [26] 20.27 | m | m | [26] 20.27 | m | m |
| 15 | WWW-Authenticate | [26] 20.44 | m | m | [26] 20.44 | i | i |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.6/16 - - Additional for 403 (Forbidden) response

Table A.211A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | P-Refused-URI-List | [183] | c1 | c1 | [183] | c1 | c1 |
| c1: IF A.162/108 THEN m ELSE n/a -- The SIP P-Refused-URI-List private-header. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/17 OR A.164/23 OR A.164/30 OR A.164/36 OR A.164/50 OR A.164/51 - - Additional for 404 (Not Found), 413 (Request Entity Too Large), 480(Temporarily not available), 486 (Busy Here), 500 (Internal Server Error), 600 (Busy Everywhere), 603 (Decline) response

Table A.212: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 8 | Retry-After | [26] 20.33 | m | m | [26] 20.33 | i | i |

Table A.213: Void

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/20 - - Additional for 407 (Proxy Authentication Required) response

Table A.214: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 6 | Proxy-Authenticate | [26] 20.27 | m | m | [26] 20.27 | m | m |
| 11 | WWW-Authenticate | [26] 20.44 | m | m | [26] 20.44 | i | i |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/21 - - Additional for 408 (Request timeout) response

Table A.214A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Restoration-Info | subclause 7.2.11 | n/a | c1 | subclause 7.2.11 | n/a | c2 |
| c1: IF A.162/120 THEN o ELSE n/a - - HSS based P-CSCF restoration. c2: IF A.162/120 THEN m ELSE n/a - - HSS based P-CSCF restoration. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/25 - - Additional for 415 (Unsupported Media Type) response

Table A.215: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept | [26] 20.1 | m | m | [26] 20.1 | i | i |
| 2 | Accept-Encoding | [26] 20.2 | m | m | [26] 20.2 | i | i |
| 3 | Accept-Language | [26] 20.3 | m | m | [26] 20.3 | i | i |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/26A - - Additional for 417 (Unknown Resource-Priority) response

Table A.215A: Supported header field	s within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept-Resource-Priority | [116] 3.2 | c1 | c1 | [116] 3.2 | c1 | c1 |
| c1: IF A.162/80 THEN m ELSE n/a - - communications resource priority for the session initiation protocol. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/27 - - Additional for 420 (Bad Extension) response

Table A.216: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 10 | Unsupported | [26] 20.40 | m | m | [26] 20.40 | c3 | c3 |
| c3: IF A.162/18 THEN m ELSE i - - reading the contents of the Unsupported header before proxying the 420 response to a method other than REGISTER. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/28 OR A.164/41A - - Additional for 421 (Extension Required), 494 (Security Agreement Required) response

Table A.216A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 3 | Security-Server | [48] 2 | c1 | c1 | [48] 2 | n/a | n/a |
| c1: IF A.162/47 THEN m ELSE n/a - - security mechanism agreement for the session initiation protocol. |  |  |  |  |  |  |  |

Prerequisite A.16/9 - - INVITE response

Prerequisite: A.164/28A - - Additional for 422 (Session Interval Too Small) response

Table A.216B: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Min-SE | [58] 5 | c1 | c1 | [58] 5 | c1 | c1 |
| c1: IF A.162/52 THEN m ELSE n/a - - the SIP session timer. |  |  |  |  |  |  |  |

Table A.217: Void

Table A.217A: Void

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/29H - - Additional for 470 (Consent Needed) response

Table A.217AA: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Permission-Missing | [125] 5.9.3 | m | m | [125] 5.9.3 | m | m |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/45 - - 503 (Service Unavailable)

Table A.217B: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 8 | Retry-After | [26] 20.33 | m | m | [26] 20.33 | i | i |

Table A.217C: void

Prerequisite A.163/9 - - INVITE response

Table A.218: Supported message bodies within the INVITE response

| Item | Header | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | XML Schema for PSTN | [11B] |  | c1 | [11B] |  | i |
| 2 | Recipient List | [183] | c2 | c2 | [183] | c3 | c3 |
| 3 | 3GPP IM CN subsystem XML body | subclause 7.6 | n/a | c4 | subclause 7.6 | n/a | c5 |
| 4 | application/vnd.3gpp.mcptt-info+xml | [8ZE] | n/a | c1 | [8ZE] | n/a | i |
| c1: A.3/3 OR A.3/4 OR A.3/5 OR A.3/7C OR A.3/9A OR A.3/10 OR A.3/11 OR A.3/13A THEN m ELSE n/a - - I-CSCF, S-CSCF, BGCF, AS acting as proxy, IBCF (THIG), additional routeing functionality, E-CSCF, ISC gateway function (THIG). c2 IF A.3/9B THEN m ELSE IF A.3/7A OR A.3/7B OR A.3/7D THEN m ELSE n/a - - IBCF (IMS-ALG), AS acting as terminating UA, AS acting as originating UA, AS performing 3rd party call control. c3 IF A.3/9B THEN m ELSE IF A.3/7A OR A.3/7B OR A.3/7D THEN i ELSE n/a - - IBCF (IMS-ALG), AS acting as terminating UA, AS acting as originating UA, AS performing 3rd party call control. c4: IF A.3/2 OR (A.3/9 AND NOT A.3/9B) OR A.3A/88 THEN m ELSE n/a - - P-CSCF, IBCF, IBCF (IMS-ALG), ATCF (proxy). c5: IF A.3/2 OR (A.3/9 AND NOT A.3/9B) OR A.3A/88 THEN i ELSE n/a - - P-CSCF, IBCF, IBCF (IMS-ALG), ATCF (proxy). |  |  |  |  |  |  |  |

# *** Next Change ***

#### A.2.2.4.7A	MESSAGE method

Prerequisite A.163/9A - - MESSAGE request

Table A.218A: Supported header fields within the MESSAGE request

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept-Contact | [56B] 9.2 | c28 | c28 | [56B] 9.2 | c28 | c29 |
| 1AA | Additional-Identity | 7.2.20 | n/a | c79 | 7.2.20 | n/a | c79 |
| 1A | Allow | [26] 20.5 | m | m | [50] 10 | i | i |
| 2 | Allow-Events | [28] 8.2.2 | m | m | [28] 8.2.2 | c1 | c1 |
| 2A | Attestation-Info | 7.2.18 | n/a | c76 | 7.2.18 | n/a | c76 |
| 3 | Authorization | [26] 20.7 | m | m | [26] 20.7 | i | i |
| 4 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 5 | Call-Info | [26] 20.9 | m | m | [26] 20.9 | c4 | c4 |
| 5A | Cellular-Network-Info | 7.2.15 | n/a | c73 | 7.2.15 | n/a | c74 |
| 6 | Content-Disposition | [26] 20.11 | m | m | [26] 20.11 | i | i |
| 7 | Content-Encoding | [26] 20.12 | m | m | [26] 20.12 | i | i |
| 8 | Content-Language | [26] 20.13 | m | m | [26] 20.13 | i | i |
| 9 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 10 | Content-Type | [26] 20.15 | m | m | [26] 20.15 | i | i |
| 11 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 12 | Date | [26] 20.17 | m | m | [26] 20.17 | c2 | c2 |
| 13 | Expires | [26] 20.19 | m | m | [26] 20.19 | I | i |
| 13A | Feature-Caps | [190] | c71 | c71 | [190] | c71 | c71 |
| 14 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 14A | Geolocation | [89] 4.1 | c36 | c36 | [89] 4.1 | c37 | c37 |
| 14B | History-Info | [66] 4.1 | c32 | c32 | [66] 4.1 | c32 | c32 |
| 14C | Geolocation-Routing | [89] 4.1 | c36 | c36 | [89] 4.1 | c37 | c37 |
| 14D | Identity | [252] 4 | c75 | c75 | [252] 4 | c75 | c75 |
| 15 | In-Reply-To | [26] 20.21 | m | m | [50] 10 | i | i |
| 15A | Max-Breadth | [117] 5.8 | c48 | c48 | [117] 5.8 | c49 | c49 |
| 16 | Max-Forwards | [26] 20.22 | m | m | [26] 20.22 | m | m |
| 17 | MIME-Version | [26] 20.24 | m | m | [26] 20.24 | i | i |
| 18 | Organization | [26] 20.25 | m | m | [26] 20.25 | c3 | c3 |
| 18AA | Origination-Id | 7.2.19 | n/a | c77 | 7.2.19 | n/a | c77 |
| 18A | P-Access-Network-Info | [52] 4.4, [234] 2 | c23 | c23 | [52] 4.4, [234] 2 | c24 | c24 |
| 18B | P-Asserted-Identity | [34] 9.1 | c10 | c10 | [34] 9.1 | c11 | c11 |
| 18C | P-Asserted-Service | [121] 4.1 | c40 | c40 | [121] 4.1 | c41 | c41 |
| 18D | P-Called-Party-ID | [52] 4.2 | c14 | c14 | [52] 4.2 | c15 | c16 |
| 18E | P-Charging-Function-Addresses | [52] 4.5 | c21 | c21 | [52] 4.5 | c22 | c22 |
| 18F | P-Charging-Vector | [52] 4.6 | c19 | c19 | [52] 4.6 | c20 | c20 |
| 18H | P-Preferred-Identity | [34] 9.2 | x | c69 | [34] 9.2 | c9 | c9 |
| 18I | P-Preferred-Service | [121] 4.2 | x | x | [121] 4.2 | c39 | c39 |
| 18J | P-Private-Network-Indication | [134] | c44 | c44 | [134] | c44 | c44 |
| 18K | P-Profile-Key | [97] 5 | c34 | c34 | [97] 5 | c35 | c35 |
| 18L | P-Served-User | [133] 6 | c45 | c45 | [133] 6 | c45 | c45 |
| 18M | P-User-Database | [82] 4 | c33 | c33 | [82] 4 | c33 | c33 |
| 18N | P-Visited-Network-ID | [52] 4.3 | c17 | o | [52] 4.3 | c18 | o |
| 19 | Priority | [26] 20.26 | m | m | [26] 20.26 | i | c50 |
| 19A | Privacy | [33] 4.2 | c12 | c12 | [33] 4.2 | c13 | c13 |
| 20 | Proxy-Authorization | [26] 20.28 | m | m | [26] 20.28 | c8 | c8 |
| 21 | Proxy-Require | [26] 20.29 | m | m | [26] 20.29 | m | m |
| 21A | Reason | [34A] 2 | c26 | c26 | [34A] 2 | c27 | c27 |
| 22A | Referred-By | [59] 3 | c30 | c30 | [59] 3 | c31 | c31 |
| 23 | Reject-Contact | [56B] 9.2 | c28 | c28 | [56B] 9.2 | c28 | c29 |
| 23A | Relayed-Charge | 7.2.12 | n/a | c72 | 7.2.12 | n/a | c72 |
| 23B | Reply-To | [26] 20.31 | m | m | [26] 20.31 | i | i |
| 23C | Request-Disposition | [56B] 9.1 | c28 | c28 | [56B] 9.1 | c28 | c28 |
| 24 | Require | [26] 20.32 | m | m | [26] 20.32 | c5 | c5 |
| 24A | Resource-Priority | [116] 3.1 | c38 | c38 | [116] 3.1 | c38 | c38 |
| 25 | Route | [26] 20.34 | m | m | [26] 20.34 | m | m |
| 25A | Security-Client | [48] 2.3.1 | x | x | [48] 2.3.1 | c25 | c25 |
| 25B | Security-Verify | [48] 2.3.1 | x | x | [48] 2.3.1 | c25 | c25 |
| 25D | Service-Interact-Info | Subclause 7.2.14 | n/a | c78 | Subclause 7.2.14 | n/a | c78 |
| 26 | Subject | [26] 20.36 | m | m | [26] 20.36 | i | i |
| 25C | Session-ID | [162] | c70 | c70 | [162] | c70 | c70 |
| 27 | Supported | [26] 20.37 | m | m | [26] 20.37 | c6 | c6 |
| 28 | Timestamp | [26] 20.38 | m | m | [26] 20.38 | i | i |
| 29 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 29A | Trigger-Consent | [125] 5.11.2 | c42 | c42 | [125] 5.11.2 | c43 | c43 |
| 30 | User-Agent | [26] 20.41 | m | m | [26] 20.41 | i | i |
| 31 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| c1: IF A.4/20 THEN m ELSE i - - SIP specific event notification extension. c2: IF A.162/9 THEN m ELSE i - - insertion of date in requests and responses. c3: IF A.162/19A OR A.162/19B THEN m ELSE i - - reading, adding or concatenating the Organization header. c4: IF A.162/19C OR A.162/19D THEN m ELSE i - - reading, adding or concatenating the Call-Info header. c5: IF A.162/11 OR A.162/13 THEN m ELSE i - - reading the contents of the Require header before proxying the request or response or adding or modifying the contents of the Require header before proxying the request or response for methods other than REGISTER. c6: IF A.162/16 THEN m ELSE i - - reading the contents of the Supported header before proxying the response. c8: IF A.162/8A THEN m ELSE i - - authentication between UA and proxy. c9: IF A.162/30A OR A.162/30C THEN m ELSE n/a - - act as first entity within the trust domain for asserted identity, act as entity passing on identity transparently independent of trust domain. c10: IF A.162/30 THEN m ELSE n/a - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c11: IF A.162/30A or A.162/30B THEN m ELSE i - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks or subsequent entity within trust network that can route outside the trust network. c12: IF A.162/31 THEN m ELSE n/a - - a privacy mechanism for the Session Initiation Protocol (SIP). c13: IF A.162/31D OR A.162/31G THEN m ELSE IF A.162/31C THEN i ELSE n/a - - application of the privacy option "header" or application of the privacy option "id" or passing on of the Privacy header transparently. c14: IF A.162/37 THEN m ELSE n/a - - the P-Called-Party-ID header extension. c15: IF A.162/37 THEN i ELSE n/a - - the P-Called-Party-ID header extension. c16: IF A.162/37 AND A.3/2 THEN m ELSE IF A.162/37 AND (A.3/3 OR A.3/9A) THEN i ELSE n/a - - the P-Called-Party-ID header extension and P-CSCF or (I-CSCF or IBCF (THIG)). c17: IF A.162/38 THEN m ELSE n/a - - the P-Visited-Network-ID header extension. c18: IF A.162/39 THEN m ELSE i - - reading, or deleting the P-Visited-Network-ID header before proxying the request or response. c19: IF A.162/45 THEN m ELSE n/a - - the P-Charging-Vector header extension. c20: IF A.162/46 THEN m ELSE IF A.162/45 THEN i ELSE n/a - - adding, deleting, reading or modifying the P-Charging-Vector header before proxying the request or response or the P-Charging-Vector header extension. c21: IF A.162/44 THEN m ELSE n/a - - the P-Charging-Function-Addresses header extension. c22: IF A.162/44A THEN m ELSE IF A.162/44 THEN i ELSE n/a - - adding, deleting or reading the P-Charging-Function-Addresses header before proxying the request or response, or the P-Charging-Function-Addresses header extension. c23: IF A.162/43 THEN x ELSE IF A.162/41 THEN m ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension. c24: IF A.162/43 THEN m ELSE IF A.162/41 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension. c25: IF A.162/47 OR A.162/47A THEN m ELSE n/a - - security mechanism agreement for the session initiation protocol or mediasec header field parameter for marking security mechanisms related to media. c26: IF A.162/48 THEN m ELSE n/a - - the Reason header field for the session initiation protocol. c27: IF A.162/48 THEN i ELSE n/a - - the Reason header field for the session initiation protocol. c28: IF A.162/50 THEN m ELSE n/a - - caller preferences for the session initiation protocol. c29: IF A.162/50 AND A.4/3 THEN m ELSE IF A.162/50 AND NOT A.4/3 THEN i ELSE n/a - - caller preferences for the session initiation protocol, and S-CSCF. c30: IF A.162/53 THEN i ELSE n/a - - the SIP Referred-By mechanism. c31: IF A.162/53 THEN m ELSE n/a - - the SIP Referred-By mechanism. c32: IF A.162/57 THEN m ELSE n/a - - an extension to the session initiation protocol for request history information. c33: IF A.162/60 THEN m ELSE n/a - - the P-User-Database private header extension. c34: IF A.162/66A THEN m ELSE n/a - - making the first query to the database in order to populate the P-Profile-Key header. c35: IF A.162/66B THEN m ELSE n/a - - using the information in the P-Profile-Key header. c36: IF A.162/70 THEN m ELSE n/a - - SIP location conveyance. c37: IF A.162/70A THEN m ELSE IF A.162/70B THEN i ELSE n/a - - addition or modification of location in a SIP method, passes on locations in SIP method without modification. c38: IF A.162/80A THEN m ELSE n/a - - inclusion of MESSAGE, SUBSCRIBE, NOTIFY in communications resource priority for the session initiation protocol. c39: IF A.162/84A THEN m ELSE n/a - - act as authentication entity within the trust domain for asserted service. |  |  |  |  |  |  |  |
| c40: IF A.162/84 THEN m ELSE n/a - - SIP extension for the identification of services. c41: IF A.162/84 OR A.162/30B THEN m ELSE i - - SIP extension for the identification of services or subsequent entity within trust network that can route outside the trust network. c42: IF A.162/85 THEN m ELSE n/a - - a framework for consent-based communications in SIP. c43: IF A.162/85 THEN i ELSE n/a - - a framework for consent-based communications in SIP. c44: IF A.162/87 THEN m ELSE n/a - - the SIP P-Private-Network-Indication private-header (P-Header). c45: IF A.162/88 THEN m ELSE n/a - - the SIP P-Served-User private header. c48: IF A.162/81 THEN m ELSE n/a - - addressing an amplification vulnerability in session initiation protocol forking proxies. c49: IF A.162/81 AND A.162/6 THEN m ELSE IF A.162/81 AND NOT A.162/6 THEN i ELSE n/a - - addressing an amplification vulnerability in session initiation protocol forking proxies, forking of initial requests. c50: IF A.162/115 THEN m ELSE i - - PSAP callback indicator. c69: IF A.162/30C THEN m ELSE x - - act as entity passing on identity transparently independent of trust domain. c70: IF A.162/101 THEN m ELSE n/a - - the Session-ID header. c71: IF A.162/110 THEN m ELSE n/a - - indication of features supported by proxy. c72: IF A.162/121 THEN m ELSE n/a - - the Relayed-Charge header field extension. c73: IF A.162/43 THEN x ELSE IF A.162/123 THEN m ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the Cellular-Network-Info header extension. c74: IF A.162/43 THEN m ELSE IF A.162/123 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the Cellular-Network-Info header extension. c75: IF A.162/126 THEN o ELSE n/a - - authenticated identity management in the Session Initiation Protocol. c76: IF A.162/128 THEN o ELSE n/a - - the Attestation-Info header field extension. c77: IF A.162/129 THEN o ELSE n/a - - the Origination-Id header field extension. c78: IF A.162/130 THEN m ELSE n/a - - Dynamic services interactions. c79: IF A.162/131 THEN o ELSE n/a - - the Additional-Identity header field extension. |  |  |  |  |  |  |  |
| NOTE: c1 refers to the UA role major capability as this is the case of a proxy that also acts as a UA specifically for SUBSCRIBE and NOTIFY. |  |  |  |  |  |  |  |

Prerequisite A.163/9A - - MESSAGE request

Table A.218B: Supported message bodies within the MESSAGE request

| Item | Header | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | permission document | [125] 5.4 | c1 | c1 | [125] 5.4 | c2 | c2 |
| 2 | application/vnd.3gpp.sms | [4D] | m | m | [4D] | i | i |
| 2 | message/cpim | [151] | c3 | c3 | [151] | c4 | c4 |
| 3 | message/imdn+xml | [157] | c5 | c5 | [157] | c6 | c6 |
| 4 | application/vnd.3gpp.mcptt-info+xml | [8ZE] | n/a | c7 | [8ZE] | n/a | i |
| 5 | application/vnd.3gpp.mcptt-mbms-usage-info+xml | [8ZE] | n/a | c7 | [8ZE] | n/a | i |
| 6 | application/vnd.3gpp.mcptt-location-info+xml | [8ZE] | n/a | c7 | [8ZE] | n/a | i |
| 7 | application/vnd.3gpp.mcptt-floor-request+xml | [8ZE] | n/a | c7 | [8ZE] | n/a | i |
| 8 | application/vnd.3gpp.mcptt-affiliation-command+xml | [8ZE] | n/a | c7 | [8ZE] | n/a | i |
| c1: IF A.162/85 THEN m ELSE n/a - - a framework for consent-based communications in SIP. c2: IF A.162/85 THEN i ELSE n/a - - a framework for consent-based communications in SIP. c3: IF A.162/95 THEN m ELSE n/a - - common presence and instant messaging (CPIM): message format. c4: IF A.162/95 THEN i ELSE n/a - - common presence and instant messaging (CPIM): message format. c5: IF A.162/96 THEN m ELSE n/a - - instant message disposition notification. c6: IF A.162/96 THEN i ELSE n/a - - instant message disposition notification. c7: A.3/3 OR A.3/4 OR A.3/5 OR A.3/7C OR A.3/9A OR A.3/10 OR A.3/11 OR A.3/13A THEN m ELSE n/a - - I-CSCF, S-CSCF, BGCF, AS acting as proxy, IBCF (THIG), additional routeing functionality, E-CSCF, ISC gateway function (THIG). |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/1 - - Additional for 100 (Trying) response

Table A.218BA: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 2 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 3 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 4 | Date | [26] 20.17 | c1 | c1 | [26] 20.17 | c2 | c2 |
| 5 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 6 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 7 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| c1: IF (A.162/9 AND A.162/5) OR A.162/4 THEN m ELSE n/a - - stateful proxy behaviour that inserts date, or stateless proxies. c2: IF A.162/4 THEN i ELSE m - - Stateless proxy passes on. |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response for all remaining status-codes

Table A.218C: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 0A | Allow | [26] 20.5 | m | m | [26] 20.5 | i | i |
| 1 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 2 | Call-Info | [26] 20.9 | m | m | [26] 20.9 | c3 | c3 |
| 2A | Cellular-Network-Info | 7.2.15 | n/a | c23 | 7.2.15 | n/a | c24 |
| 3 | Content-Disposition | [26] 20.11 | m | m | [26] 20.11 | i | i |
| 4 | Content-Encoding | [26] 20.12 | m | m | [26] 20.12 | i | i |
| 5 | Content-Language | [26] 20.13 | m | m | [26] 20.13 | i | i |
| 6 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 7 | Content-Type | [26] 20.15 | m | m | [26] 20.15 | i | i |
| 8 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 9 | Date | [26] 20.17 | m | m | [26] 20.17 | c1 | c1 |
| 9A | Expires | [26] 20.19 | m | m | [26] 20.19 | i | i |
| 10 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 10A | Geolocation-Error | [89] 4.3 | c17 | c17 | [89] 4.3 | c18 | c18 |
| 10B | History-Info | [66] 4.1 | c16 | c16 | [66] 4.1 | c16 | c16 |
| 11 | MIME-Version | [26] 20.24 | m | m | [26] 20.24 | i | i |
| 12 | Organization | [26] 20.25 | m | m | [26] 20.25 | c2 | c2 |
| 12A | P-Access-Network-Info | [52] 4.4, [52A] 4, [234] 2 | c13 | c13 | [52] 4.4, [52A] 4, [234] 2 | c14 | c14 |
| 12B | P-Asserted-Identity | [34] 9.1 | c5 | c5 | [34] 9.1 | c6 | c6 |
| 12C | P-Charging-Function-Addresses | [52] 4.5, [52A] 4 | c11 | c11 | [52] 4.5, [52A] 4 | c12 | c12 |
| 12D | P-Charging-Vector | [52] 4.6, [52A] 4 | c9 | c9 | [52] 4.6, [52A] 4 | c10 | c10 |
| 12F | P-Preferred-Identity | [34] 9.2 | x | x | [34] 9.2 | c4 | n/a |
| 12G | Privacy | [33] 4.2 | c7 | c7 | [33] 4.2 | c8 | c8 |
| 12H | Relayed-Charge | 7.2.12 | n/a | c22 | 7.2.12 | n/a | c22 |
| 12I | Reply-To | [26] 20.31 | m | m | [26] 20.31 | i | i |
| 12J | Require | [26] 20.32 | m | m | [26] 20.32 | c15 | c15 |
| 13 | Server | [26] 20.35 | m | m | [26] 20.35 | i | i |
| 13AA | Service-Interact-Info | Subclause 7.2.14 | n/a | c25 | Subclause 7.2.14 | n/a | c25 |
| 13A | Session-ID | [162] | c21 | c21 | [162] | c21 | c21 |
| 14 | Timestamp | [26] 20.38 | i | i | [26] 20.38 | i | i |
| 15 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 16 | User-Agent | [26] 20.41 | m | m | [26] 20.41 | i | i |
| 17 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| 18 | Warning | [26] 20.43 | m | m | [26] 20.43 | i | i |
| c1: IF A.162/9 THEN m ELSE i - - insertion of date in requests and responses. c2: IF A.162/19A OR A.162/19B THEN m ELSE i - - reading, adding or concatenating the Organization header. c3: IF A.162/19C OR A.162/19D THEN m ELSE i - - reading, adding or concatenating the Call-Info header. c4: IF A.162/30A THEN m ELSE n/a - - act as first entity within the trust domain for asserted identity. c5: IF A.162/30 THEN m ELSE n/a - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c6: IF A.162/30A or A.162/30B THEN m ELSE i - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks or subsequent entity within trust network that can route outside the trust network. c7: IF A.162/31 THEN m ELSE n/a - - a privacy mechanism for the Session Initiation Protocol (SIP). c8: IF A.162/31D OR A.162/31G THEN m ELSE IF A.162/31C THEN i ELSE n/a - - application of the privacy option "header" or application of the privacy option "id" or passing on of the Privacy header transparently. c9: IF A.162/45 THEN m ELSE n/a - - the P-Charging-Vector header extension. c10: IF A.162/46 THEN m ELSE IF A.162/45 THEN i ELSE n/a - - adding, deleting, reading or modifying the P-Charging-Vector header before proxying the request or response or the P-Charging-Vector header extension. c11: IF A.162/44 THEN m ELSE n/a - - the P-Charging-Function-Addresses header extension. c12: IF A.162/44A THEN m ELSE IF A.162/44 THEN i ELSE n/a - - adding, deleting or reading the P-Charging-Function-Addresses header before proxying the request or response, or the P-Charging-Function-Addresses header extension. c13: IF A.162/43 THEN x ELSE IF A.162/41 THEN m ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension. c14: IF A.162/43 THEN m ELSE IF A.162/41 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension. c15: IF A.162/11 OR A.162/13 THEN m ELSE i - - reading the contents of the Require header before proxying the request or response or adding or modifying the contents of the Require header before proxying the request or response for methods other than REGISTER. c16: IF A.162/57 THEN m ELSE n/a - - an extension to the session initiation protocol for request history information. c17: IF A.162/70 THEN m ELSE n/a - - SIP location conveyance. c18: IF A.162/70A THEN m ELSE IF A.162/70B THEN i ELSE n/a - - addition or modification of location in a SIP method, passes on locations in SIP method without modification. c21: IF A.162/101 THEN m ELSE n/a - - the Session-ID header. c22: IF A.162/121 THEN m ELSE n/a - - the Relayed-Charge header field extension. c23: IF A.162/43 THEN x ELSE IF A.162/123 THEN m ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the Cellular-Network-Info header extension. c24: IF A.162/43 THEN m ELSE IF A.162/123 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the Cellular-Network-Info header extension. c25: IF A.162/130 THEN m ELSE n/a - - Dynamic services interactions. |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/102 - - Additional for 2xx response

Table A.218D: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 0A | Accept-Resource-Priority | [116] 3.2 | c4 | c4 | [116] 3.2 | c4 | c4 |
| 1 | Allow-Events | [28] 8.2.2 | m | m | [28] 8.2.2 | c1 | c1 |
| 2 | Authentication-Info | [26] 20.6 | m | m | [26] 20.6 | i | i |
| 3 | Feature-Caps | [190] | c6 | c6 | [190] | c6 | c6 |
| 4 | P-Visited-Network-ID | [52B] 3 | o | o | [52B] 3 | o | o |
| 6 | Supported | [26] 20.37 | m | m | [26] 20.37 | i | i |
| c1: IF A.4/20 THEN m ELSE i - - SIP specific event notification extension. c4: IF A.162/80A THEN m ELSE n/a - - inclusion of MESSAGE, SUBSCRIBE, NOTIFY in communications resource priority for the session initiation protocol. c6: IF A.162/110 THEN m ELSE n/a - - indication of features supported by proxy. |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/103 OR A.164/104 OR A.164/105 OR A.164/106 - - Additional for 3xx – 6xx response

Table A.218DA: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Error-Info | [26] 20.18 | m | m | [26] 20.18 | i | i |
| 1A | Reason | [294] | o | c2 | [294] | o | c2 |
| 2 | Response-Source | 7.2.17 | n/a | c1 | 7.2.17 | n/a | c1 |
| c1: IF A.162/125 THEN o ELSE n/a - - use of the Response-Source header field in SIP error responses. c2: IF A.4/38C THEN o ELSE n/a - - use of the Reason header field in Session Initiation Protocol (SIP) responses (carrying STIR codes). |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/103 OR A.164/35 - - Additional for 3xx or 485 (Ambiguous) response

Table A.218E: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 2 | Contact | [26] 20.10 | m | m | [26] 20.10 | c1 | c1 |
| c1: IF A.162/19E THEN m ELSE i - - deleting Contact headers. |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/14 - - Additional for 401 (Unauthorized) response

Table A.218F: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 3 | Proxy-Authenticate | [26] 20.27 | m | m | [26] 20.27 | m | m |
| 6 | WWW-Authenticate | [26] 20.44 | m | m | [26] 20.44 | i | i |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/17 OR A.164/23 OR A.164/30 OR A.164/36 OR A.164/42 OR A.164/45 OR A.164/50 OR A.164/51 - - Additional for 404 (Not Found), 413 (Request Entity Too Large), 480(Temporarily not available), 486 (Busy Here), 500 (Internal Server Error), 503 (Service Unavailable), 600 (Busy Everywhere), 603 (Decline) response

Table A.218G: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 4 | Retry-After | [26] 20.33 | m | m | [26] 20.33 | i | i |

Table A.218H: Void

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/20 - - Additional for 407 (Proxy Authentication Required) response

Table A.218I: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 3 | Proxy-Authenticate | [26] 20.27 | m | m | [26] 20.27 | m | m |
| 6 | WWW-Authenticate | [26] 20.44 | m | m | [26] 20.44 | i | i |

Table 218IA: Void

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/25 - - Additional for 415 (Unsupported Media Type)

Table A.218J: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept | [26] 20.1 | m | m | [26] 20.1 | i | i |
| 2 | Accept-Encoding | [26] 20.2 | m | m | [26] 20.2 | i | i |
| 3 | Accept-Language | [26] 20.3 | m | m | [26] 20.3 | i | i |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/26A - - Additional for 417 (Unknown Resource-Priority) response

Table A.218JA: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept-Resource-Priority | [116] 3.2 | c1 | c1 | [116] 3.2 | c1 | c1 |
| c1: IF A.162/80A THEN m ELSE n/a - - inclusion of MESSAGE, SUBSCRIBE, NOTIFY in communications resource priority for the session initiation protocol. |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/27 - - Additional for 420 (Bad Extension) response

Table A.218K: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 5 | Unsupported | [26] 20.40 | m | m | [26] 20.40 | c3 | c3 |
| c3: IF A.162/18 THEN m ELSE i - - reading the contents of the Unsupported header before proxying the 420 response to a method other than REGISTER. |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/28 OR A.164/41A - - Additional for 421 (Extension Required), 494 (Security Agreement Required) response

Table A.218L: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 3 | Security-Server | [48] 2 | c1 | c1 | [48] 2 | n/a | n/a |
| c1: IF A.162/47 THEN m ELSE n/a - - security mechanism agreement for the session initiation protocol. |  |  |  |  |  |  |  |

Table A.218M: Void

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/29H - - Additional for 470 (Consent Needed) response

Table A.218MA: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Permission-Missing | [125] 5.9.3 | m | m | [125] 5.9.3 | m | m |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/46 - - Additional for 504 (Server Time-out) response

Table A.218MB: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Restoration-Info | subclause 7.2.11 | n/a | c1 | subclause 7.2.11 | n/a | n/a |
| c1: IF A.4/110 THEN o ELSE n/a - - HSS based P-CSCF restoration. |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response

Table A.218N: Supported message bodies within the MESSAGE response

| Item | Header | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 |  |  |  |  |  |  |  |

# *** End of Changes ***