# 3GPP TSG-CT WG1 Meeting #140	C1-231134

# Athens, Greece, 27 February – 3 March 2023

Title:	LS on indication of URSP provisioning support in EPS

Response to:	NA

Release:	Release 18

Work Item:	eUEPO

Source:	CT1

To:	SA2

Cc:	CT3

Contact Person:

Name:	ZHOU Xingyue (Joy)

E-mail Address:	zhou.xingyue@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	NA

# 1. Overall Description:

CT1 find that SA2#154AHE agreed S2-2301621(TS 23.501 CR3924) introduces following requirement to support URSP Provisioning in EPS:

"When the UE first registers in 5GS, the UE includes the Indication of URSP Provisioning Support in EPS in the UE Policy Container carried in Registration Request. Only when receiving this indication in the UE Policy Container, the PCF will provision the URSP to UE in EPS."

If fulfilling this requirement using existing mechanism in CT1, the indication of URSP provisioning support in EPS would be included in the UE STATE INDICATION message according to initial registration procedure specified in clause 5.5.1.2.2 of TS 24.501. However, sending the UE STATE INDICATION message needs to satisfy following condition:

"If the UE has one or more stored UE policy sections:

-	identified by a UPSI with the PLMN ID part indicating the HPLMN or the selected PLMN; or

-	identified by a UPSI with the PLMN ID part indicating the PLMN ID part of the SNPN identity of the selected SNPN and associated with the NID of the selected SNPN;

then the UE shall set the Payload container type IE to "UE policy container" and include the UE STATE INDICATION message (see annex D) in the Payload container IE of the REGISTRATION REQUEST message."

In another word, if there is no stored UE policy sections, it is not possible to convey this indication using UE policy container in the initial registration procedure. CT1 has not concluded a solution to work out this issue.

From CT1 perspective, how to use the protocol to convey this indication of URSP provisioning support in EPS is conveyed from UE to the PCF during the registration procedure belongs to stage 3 responsibility. It is not necessary to specify using what specific information element to convey the indication in stage 2.

Hence, CT1 asks SA2 to re-consider to describe UE providing the indication of URSP provisioning support in EPS to the network during the registration procedure without specifying using UE Policy Container.

# 2. Actions:

To SA WG2 group.

ACTION: 	CT1 kindly asks SA2 to take the above request into account.

# 3. Date of Next CT1 Meetings:

CT1#141e	17th - 21st April 2023 		E-Meeting

CT1#142	22nd – 26th May 2023 		Bratislava, Slovakia