3GPP TSG-CT WG1 Meeting #142	C1-233110

# Bratislava 22– 26 May 2023

Source:	MediaTek Inc.

Title:	Discussion on manual hosting network selection in TS 23.122

Agenda item:	18.2.5

Document for:	Discussion

1. Introduction

This paper discusses whether there is justification to include manual selection handling for access for localized services in SNPN in eNPN_Ph2 WID and in TS 23.122.

2. Background Facts that supports manual selection work in CT1

2.0 Inconsistency between latest SA2 WID and CT1 WID

In the approved SA2 WID (SP-230096), it specified

"…3. Support for providing access to Localized Services including the following based on conclusions in TR…

d. Functional description how to enable manual network selection and functional enhancements to automatic network selection, of hosting network giving access to Localized Services."

So the manual network selection work is in scope of SA2 work.

In a not-agreed CT1 WID (C1-230100), manual network selection was included too, but removed based on comments of some companies in the approved revision (C1-230184), our understanding is that the comments from the some companies is based on an mis-interpretation on an SA1 LS (S1-223540) which will be discussed in clause 2.2 in this discussion paper.

2.1 SA1 current TS 22.261 V18.9.0 (2023-03) requirements

…6.41	Providing Access to Local Services

6.41.1	Description

Providing access to local services refers to the capability to provide access to a hosting network and a set of services offered by the hosting network provider, and 3rd party service providers including other network operators and 3rd party application providers. The services can be localized (i.e. provided at specific/limited area) and can be bounded in time.

…

6.41.2.3	User Manual Selection of Localized Services via Hosting Network

The hosting network shall allow a UE to manually select temporary localized services which are provided via local breakout at the hosting network.

NOTE:	Localized services which are provided via local breakout at the hosting network can be based on interworking scenarios for hosting network owned/collaborative services as indicated in Annex H.

…

6.41.2.5	UE Discovery, Selection and Access

…

The 5G system shall support a mechanism to allow a user to manually select a specific local hosting network.

NOTE: 	Additional information can be presented to the user to facilitate the manual network selection.

…

2.1.1 Observations from SA1 TS 22.261

Observation-1: According to SA1 requirements, manual selection of SNPNs providing access for localized services is in the scope of eNPN_Ph2.

Proposal-1: Propose CT1 to agree to include hosting network manual selection work in eNPN_Ph2 WID.

Proposal-2: Propose CT1 to agree to work on hosting network manual selection in TS 23.122, taking SA1 requirements into account.

Observation-2: According to SA1 requirements, additional information can be presented to the user to facilitate the manual network selection.

Proposal-3: Propose CT1 to work on defining useful presented information from modem to user to facilitate the manual network selection for manually selecting SNPN providing access for localized services.

2.2 SA1 LS S1-223540 (Reply LS on Progress and open issues for NPN enhancements in Rel-18)

Whole contents of SA1 LS S1-223540 are copy-pasted below without any modification:

SA1 thanks SA2 for their LS on “Progress and open issues for NPN enhancements in Rel-18”. With regards to Question 1,

Question 1: A UE that intends to access a hosting network, to access localized services, using home network credentials/subscription, can select a hosting network either by automatic network selection or by manual network selection. Does the home network need to authorize the use of such network selection of a hosting network for both automatic and manual network selection or only for automatic network selection, and does it depend on whose credentials the UE uses to access the hosting network?

SA1 provides specific PALS related service requirements are presented below:

Automatic Network Selection Related Requirements (TS 22.261, Clause 6.41.2.5)

The 5G system shall enable the home network to allow a UE to automatically select a hosting network for accessing localized services when specified conditions (e.g., predefined time, location) are fulfilled.

The 5G system shall enable the home network to instruct a UE to select a hosting network with certain conditions (e.g., predefined time, location) based on the request from a service provider.

The 5G system shall enable the home network to allow a UE to select a hosting network or change to another hosting network, without any additional user intervention as long as the delivered services, both localized services and home routed services, are unchanged.

The 5G system shall be able to allow the home network to steer its UE(s) to a hosting network with the consideration of the location, times, coverage of the hosting network and services offered by the home network and hosting network.

Manual Network Selection Related Requirements (TS 22.261, Clause 6.41.2.5)

The 5G system shall support a mechanism to allow a user to manually select a specific local hosting network.

NOTE: 	Additional information can be presented to the user to facilitate the manual network selection.

Subject to operator’s policy and agreement between a 3rd party service provider and operator, the 5G system shall enable a UE to receive and use configuration provided by a 3rd party service provider to discover and access a hosting network and localized services, including the considerations of prior service agreement with a 3rd party service provider and no prior subscription to hosting network. If the UE is able to obtain services from two networks simultaneously, it may additionally select the hosting network. If the UE cannot maintain the connection to the home network while selecting the hosting network, the selection shall only be done on request by the user, i.e., using manual selection.

First, SA1 assumes that the use of “authorize” in the Question 1 “Does the home network need to authorize…” refers to home network control by configuration or policy (e.g., home network provides list of preferred hosting networks).

Therefore based on the service requirements presented above and assumption, SA1 believes automatic network selection by the UE is controlled by configuration or policy by the home network, while there is no SA1 requirement to request home network to control UE to manually select a local hosting network instead SA1 requirements leave manual hosting network selection to user’s decision.

Also, from SA1 perspective, after automatic network or manual selection, the UE may use home network, hosting network or 3rd party service provider credentials when it tries to access the localized services on the hosting network.

Lastly, SA1 would like to bring SA2’s attention to this PALS related requirement:

The 5G system may support means for a UE which may or may not have prior subscription to the hosting network to display human readable information on how to gain access to the hosting network and available 3rd party services.

2.2.1 Observations from SA1 LS S1-223540

Observation-3: According to SA1 LS, it leaves the manual selection to user’s decision and does not require the home network to control the selection candidates during manual selection, this is same as traditional network selection, e.g., UE/user can select a PLMN even it is in forbidden PLMN list, or UE/user can select an SNPN even if the SNPN is neither subscribed SNPN nor an SNPN(or GIN) in the SNPN selection parameters (SNPN selection parameters: user controlled prioritized list of preferred SNPNs, credentials holder controlled prioritized list of preferred SNPNs, and credentials holder controlled prioritized list of GINs). So the SA1 LS is not saying there is no requirements for hosting network manual selection! and there is no explicit restriction from SA1 on the information for UE to present to the user for manual selection for SNPN providing access for localized services.

2.3 SA2 current TS 23.501 V18.1.0 (2023-03) requirements

5.30.2.4.3	Manual network selection

For manual network selection UEs operating in SNPN access mode provide to the user the list of SNPNs (each is identified by a PLMN ID and NID) and related human-readable names (if available) of the available SNPNs the UE has respective SUPI and credentials for. If the UEs supports access to an SNPN using credentials from a Credentials Holder, the UE also presents available SNPNs which broadcast the "access using credentials from a Credentials Holder is supported" indication and the human-readable names related to the SNPNs (if available).

NOTE:	The details of manual SNPN selection are defined in TS 23.122 [17].

When a UE performs Initial Registration to an SNPN, the UE shall indicate the selected PLMN ID and NID as broadcast by the selected SNPN to NG-RAN. NG-RAN shall inform the AMF of the selected PLMN ID and NID.

2.3.1 Observations from SA2 current TS 23.501

In Rel-18, there are 5 different types of lists (designed for CH case use) for SNPN selection:

1)	a user controlled prioritized list of preferred SNPNs, where each entry contains an SNPN identity;

2)	a credentials holder controlled prioritized list of preferred SNPNs, where each entry contains an SNPN identity;

3)	a credentials holder controlled prioritized list of GINs;

…

4)	a "credentials holder controlled prioritized list of preferred SNPNs for access for localized services in SNPN", where each entry contains an SNPN identity and a validity information consisting of time validity information; and

5)	a "credentials holder controlled prioritized list of preferred GINs for access for localized services in SNPN", where each entry contains an GIN and a validity information consisting of time validity information;

The SNPNs providing access for localized services also supports access to an SNPN using credentials from a Credentials Holder, so based on SA2 “If the UEs supports access to an SNPN using credentials from a Credentials Holder, the UE also presents available SNPNs which broadcast the "access using credentials from a Credentials Holder is supported" indication” manual selection further considering 4) "credentials holder controlled prioritized list of preferred SNPNs for access for localized services in SNPN" and 5) "credentials holder controlled prioritized list of preferred GINs for access for localized services in SNPN" has SA2 requirements support.

Proposal-4: Propose CT1 to agree further consider "credentials holder controlled prioritized list of preferred SNPNs for access for localized services in SNPN" and "credentials holder controlled prioritized list of preferred GINs for access for localized services in SNPN" during manual SNPN selection.

Furthermore, in the SA2 NOTE:

NOTE:	The details of manual SNPN selection are defined in TS 23.122 [17].

It leaves the detail design of manual SNPN selection specified in TS 23.122, so further detail considerations is also supported by SA2 current texts, e.g., after user selects an SNPN how the modem determines an appropriate subscription.

Proposal-5: Propose CT1 to agree to further detail consideration during manual SNPN selection.

3. Observations and Proposals

Proposal-1: Propose CT1 to agree to include hosting network manual selection work in eNPN_Ph2 WID.

Proposal-2: Propose CT1 to agree to work on hosting network manual selection in TS 23.122, taking SA1 requirements into account.

Observation-2: According to SA1 requirements, additional information can be presented to the user to facilitate the manual network selection.

Proposal-3: Propose CT1 to work on defining useful presented information from modem to user to facilitate the manual network selection for manually selecting SNPN providing access for localized services.

Observation-3: According to SA1 LS, it leaves the manual selection to user’s decision and does not require the home network to control the selection candidates during manual selection, this is same as traditional network selection, e.g., UE/user can select a PLMN even it is in forbidden PLMN list, or UE/user can select an SNPN even if the SNPN is neither subscribed SNPN nor an SNPN(or GIN) in the SNPN selection parameters (SNPN selection parameters: user controlled prioritized list of preferred SNPNs, credentials holder controlled prioritized list of preferred SNPNs, and credentials holder controlled prioritized list of GINs). So the SA1 LS is not saying there is no requirements for hosting network manual selection! and there is no explicit restriction from SA1 on the information for UE to present to the user for manual selection for SNPN providing access for localized services.

Proposal-4: Propose CT1 to agree further consider "credentials holder controlled prioritized list of preferred SNPNs for access for localized services in SNPN" and "credentials holder controlled prioritized list of preferred GINs for access for localized services in SNPN" during manual SNPN selection.

Proposal-5: Propose CT1 to agree to further detail consideration during manual SNPN selection.

4. Conclusions

The C1-233111, taking all the aspects discussed above into account, is submitted, and propose CT1 to discuss and agree on it.