3GPP TSG-CT WG1 Meeting #146	C1-240085

# Online, 22 – 26 January 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.587 | CR | 0285 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_MBS4V2X |  |  |  |  |  | Date: |  |  | 2024-11-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 23.287 states: ------------------- Table 5.10.2-1: Information for MBS session announcement for V2X communication Information element; Description. TMGI; TMGI information. MBS Service Type; Indicates whether the MBS Session for the service is multicast or broadcast.. MBS service area information (NOTE 1); Identifies the service area for the MBS session.. Frequency and/or MBS Frequency Selection Area (FSA) ID(s) (NOTE 3); Identification of frequency if multi carrier support is provided. SDP information (NOTE 2); SDP with IP multicast address and port number used for V2X communication via MBS. Depending on the V2X application, the V2X message can be carried directly on top of UDP, without any streaming protocols.. NOTE 1: MBS service area information is optional for multicast services. NOTE 2: Typical V2X application does not require Codec information in the SDP information. NOTE 3: The Frequency and/or FSA ID(s) are optional and only apply to broadcast MBS sessions.. ------------------- and ------------------- Table 5.10.3-1: Information for MBS session announcement for V2X Application Server Discovery Information element; Description. TMGI; TMGI information. MBS Service Type; Indicates that the MBS Session for the service is broadcast.. MBS service area information; Identifies the service area for the MBS session.. Frequency and/or MBS Frequency Selection Area (FSA) ID(s); Identification of frequency if multi carrier support is provided. SDP information; SDP with IP multicast address and port number used for V2X Application Server discovery via MBS. The content of the message carries the local Service Information and should include following information: - Mapping of the V2X service types to V2X Application Server address information (consisting of IP address/FQDN and transport layer port#) for unicast, and MBS session announcement for V2X communication via MBS.. ------------------- This is not captured in 24.587 correctly. |
| Summary of change: |  | Parameters of MBS session for V2X communication over MBS are aligned with stage-2. Parameters of MBS session for V2X AS discover over MBS are aligned with stage-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-3 not aligned with stage-2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 5.2.4, 6.2.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: - 23.247 is not added in clause 2 due to overlap with another CR - superfluous occurences of "and" removed - in 2nd bullet list, adding bullet dedicated for NID was reverted - additional cosigners added - NID is moved to editor's note |  |  |  |  |  |  |  |  |

***** change *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[3]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[5]	3GPP TS 24.386 "User Equipment (UE) to V2X control function; protocol aspects; Stage 3".

[6]	3GPP TS 24.501: "Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[7]	3GPP TS 24.588: "Vehicle-to-Everything (V2X) services in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[8]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[9]	3GPP TS 38.304: "User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[10]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[11]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[12]	ETSI EN 302 636-3 v1.2.1: "Intelligent Transport Systems (ITS); Vehicular Communications; GeoNetworking; Part 3: Network Architecture".

[13]	IEEE 1609.3 2016: "IEEE Standard for Wireless Access in Vehicular Environments (WAVE) -- Networking Services".

[14]	IETF RFC 768: "User Datagram Protocol".

[15]	IETF RFC 4291: "IP Version 6 Addressing Architecture".

[16]	IETF RFC 4862: "Neighbor Discovery for IP version 6 (IPv6)".

[17]	ISO 29281-1:2018: "Intelligent transport systems -- Communication access for land mobiles (CALM) -- Non-IP networking -- Part 1: Fast networking & transport layer protocol (FNTP)".

[18]	ISO TS 17419 ITS-AID AssignedNumbers: http://standards.iso.org/iso/ts/17419/TS17419%20Assigned%20Numbers/TS17419_ITS-AID_AssignedNumbers.pdf

[19]	IETF RFC 1035: "DOMAIN NAMES - IMPLEMENTATION AND SPECIFICATION".

[20]	3GPP TS 33.536: "Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

[21]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[22]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[23]	ISO/IEC 10118-3:2018: "IT Security techniques – Hash-functions – Part 3: Dedicated hash-functions".

[24]	CCSA YD/T 3707-2020: "Technical requirements of network layer of LTE-based vehicular communication".

[25]	IETF RFC 793: "Transmission Control Protocol."

[26]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[27]	3GPP TS 24.554: "Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3".

[28]	3GPP TS 24.577: "Aircraft-to-Everything (A2X) services in 5G System (5GS) protocol aspects; Stage 3".

[29]	3GPP TS 24.514: "Ranging based services and sidelink positioning in 5G system(5GS); Stage 3".

[30]	3GPP TS 38.355:	"NR; Sidelink Positioning Protocol (SLPP); Protocol specification".

[31]	IETF RFC 4566: "SDP: Session Description Protocol".

[ts23501]	3GPP TS 23.501:	"System Architecture for the 5G System; Stage 2".

***** change *****

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

E-UTRA-PC5: PC5 reference point over E-UTRA. The term E-UTRA-PC5 used in the present document corresponds to the term LTE PC5 defined in 3GPP TS 23.287 [3].

NR-PC5: PC5 reference point over NR. The term NR-PC5 used in the present document corresponds to the term NR PC5 defined in 3GPP TS 23.287 [3].

PC5 QoS flow context: A context which includes a set of V2X service identifiers, a PQFI value and a set of PC5 QoS parameters.

PC5 QoS rule: A rule which includes a PC5 QoS rule identifier, a PQFI value, a precedence value and optionally a set of packet filters. The PC5 QoS rule is associated with a PC5 QoS flow context.

V2X service identifier: an identifier of a V2X service, e.g. PSID, ITS-AID, or AID of the V2X application. The term V2X service identifier used in the present document corresponds to the term V2X service type defined in 3GPP TS 23.287 [3].

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.287 [3] apply:

Application Identifier (AID)

Intelligent Transport Systems (ITS)

ITS Application Identifier (ITS-AID)

NR Tx Profile

Provider Service Identifier (PSID)

V2X communication

V2X message

V2X service

For the purposes of the present document, the following terms and definitions given in 3GPP TS 24.501 [6] apply:

5G-EA

5G-IA

For the purposes of the present document, the following terms and definitions given in 3GPP TS 24.501 [6] apply:

UE local configuration

For the purposes of the present document, the following terms and definitions given in 3GPP TS 38.331 [11] apply:

MBS Radio Bearer

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.247 [ts23247] apply:

MBS Frequency Selection Area (FSA) ID

MBS service area

***** change *****

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] , 3GPP TS 24.501 [6] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] and 3GPP TS 24.501 [6].

A2X	Aircraft-to-Everything

E-UTRA	Evolved Universal Terrestrial Radio Access

FQDN	Fully Qualified Domain Name

FSA	Frequency Selection Area

LSB	Least Significant 8 Bits

MBS	Multicast/Broadcast Services

MSB	Most Significant 8 Bits

NR	New Radio

NRPEK	NR PC5 Encryption Key

NRPIK	NR PC5 Integrity Key

V2X	Vehicle-to-Everything

V2XP	V2X Policy

PQFI	PC5 QoS Flow ID

PQI	PC5 5QI

ProSeP	5G ProSe Policy

RSLPP	Ranging and Sidelink Positioning Policy

***** change *****

### 5.2.4	Configuration parameters for V2X communication over Uu

The configuration parameters for V2X communication over Uu consist of:

a)	a validity timer for the validity of the configuration parameters for V2X communication over Uu to 5GCN;

b)	optionally, a list of V2X service identifier to PDU session parameters mapping rules. Each mapping rule contains one or more V2X service identifiers of a the V2X service and one or more parameters for establishment of a PDU session for V2X communication over Uu for the V2X services:

1)	one of the "IPv4", "IPv6", "IPv4v6" or "Unstructured" PDU session types;

2)	an SSC mode;

3)	a list of zero or more S-NSSAIs;

4)	a list of zero or more DNNs; and

5)	one of the UDP or TCP transport layer protocol if the PDU session type is "IPv4", "IPv6" or "IPv4v6"; and

c)	a list of PLMNs in which the UE is configured to use V2X communication over Uu. For each PLMN, the list contains:

1)	for transfer of a V2X message of a V2X service identified by a V2X service identifier:

i)	a list of V2X service identifier to V2X application server address mapping rules, applicable when the UE is registered to the PLMN. Each mapping rule contains:

A)	one or more V2X service identifiers;

B)	a V2X application server address for unicast consisting of:

-	an FQDN, or an IP address; and

-	a UDP port for uplink transport, a UDP port for downlink transport, a TCP port for bidirectional transport or any combination of them; and

C)	optionally a geographical area; and

D)	optionally, one or more V2X MBS configuration(s) for receiving V2X communication over Uu via MBS per V2X service identier consisting of:

-	a TMGI;

-	an MBS service type indicating multicast or broadcast MBS session;

-	an MBS service area. If the MBS service type indicates the multicast MBS session, MBS service area is optional. If the MBS service type indicates the broadcast MBS session, MBS service area is provided;

-	if the MBS service type indicates the broadcast MBS session, then optionally, a frequency, a list of one or more MBS FSA IDs, or both; and

-	an SDP body;

Editor’s note (WI: TEI18_MBS4V2X, CR: 0276):	The details of the SDP body encoding are FFS.

ii)	optionally, per type of data (IP and non-IP) and V2X message family (in case of non-IP) and optionally a geographical area, one or more default V2X application server addresses for the unicast V2X communication over Uu applicable when the UE is registered to the PLMN. Each V2X application server address consists of:

i)	an FQDN, or an IP address; and

ii)	a UDP port for uplink transport, a UDP port for downlink transport, a TCP port for bidirectional transport or any combination of them; and

2)	for transfer of a V2X message of a V2X service not identified by a V2X service identifier:

i)	a list of the V2X application servers per optional geographical area where usage of those V2X application servers applies, applicable when the UE is registered to the PLMN. Each entry of the list contains:

A)	a V2X application server address consisting of an FQDN, or an IP address; and

B)	optionally, a geographical area.

ii)	optionally, one or more V2X MBS configuration(s) for receiving V2X communication over Uu via MBS consisting of:

A)	a TMGI;

B)	an MBS service type indicating multicast or broadcast MBS session;

C)	an MBS service area. If the MBS service type indicates the multicast MBS session, MBS service area is optional. If the MBS service type indicates the broadcast MBS session, MBS service area is provided;

D)	if the MBS service type indicates the broadcast MBS session, then optionally, a frequency, a list of one or more MBS FSA IDs, or both; and

E)	an SDP body;

Editor’s note (WI: TEI18_MBS4V2X, CR: 0276):	The details of the SDP body encoding are FFS.

3)	optionally, a V2X MBS configuration for receiving V2X application server information via MBS consisting of:

i)	a TMGI;

ii)	an MBS service area;

iii)	a frequency, a list of one or more MBS FSA IDs, or both; and

iv)	an SDP body;

Editor’s note (WI: TEI18_MBS4V2X, CR: 0276):	The details of the SDP body encoding are FFS.

Editor’s note (WI: TEI18_MBS4V2X, CR: 0285):	It is FFS whether V2X communication over Uu using MBS can be used in an SNPN.

***** change *****

##### 6.2.6.2.2	Procedure for V2X application server discovery using MBS

The UE shall proceed as follows:

a)	the UE shall start listening to the MBS radio bearer of a broadcast MBS session corresponding to the TMGI, if the MBS session is provided within an SNPN, , MBS service area, frequency and list of MBS FSA IDs included in the V2X AS MBS configuration (see 3GPP TS 38.331 [11] clause 5.9) associated with the serving PLMN in the configuration parameters for V2X communication over Uu provisioned to the UE as specified in subclause 5.2.4

b)	the UE shall use the IP multicast address and port included in the V2X AS MBS configuration associated with the serving PLMN in the configuration parameters for V2X communication over Uu provisioned to the UE as specified in subclause 5.2.4 to receive the V2X local service information; and

Editor’s note (WI: TEI18_MBS4V2X, CR: 0275):	The details of the encoding of V2X local service information are FFS.

c)	If the V2X service identifier of the V2X service requesting V2X communication over Uu maps to a V2X application server FQDN in the V2X application server information obtained at step 2, the UE shall perform DNS lookup as specified in IETF RFC 1035 [19] to resolve the IP address(es) of the V2X application server.