3GPP TSG-CT WG1 Meeting #153	C1-250122

# Athens, Greece, 17-21 February 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 6707 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Determining the activation of optimized media routing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Nokia, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Description of P-CSCF determination of the activation of optimized media routing, based on Annex AE of TS 23.228, and CRs in S2-2500719-CR1562 and S2-2501092-CR1564 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Providing the procedure for P-CSCF to determine the activation of optimized media routing / UE-satellite-UE communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 requirements not fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AX.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 23.228 CR1562 and CR1564 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## AX.2.2	Procedures at the P-CSCF

### AX.2.2.1	Determining the activation of optimized media routing

If the P-CSCF supports optimized media routing, the P-CSCF determines that the UE is to be in its home country, the UE indicates in the SIP INVITE request message that P-Access-Network-Info header field value is set to indicate an IP-CAN associated with satellite NG-RAN, and the identifier of the satellite serving the UE is received by the P-CSCF, then the P-CSCF may initiate the procedure for optimized media routing after checking that the identifier of the satellite serving the UE supports optimized routing.

NOTE 1:	In order to determine the UE is in its home country, the P-CSCF checks the MCC fields received in the P-Access-Network-Info header field against the MCC code(s) stored in the P-CSCF.

NOTE 2:	The P-CSCF receives from the PCF, in the Access Network Information, the identifier of the satellite serving the UE. The PCF receives the information from the SMF.

NOTE 3:	For the purpose of optimized media routing, in this release of the specification, in the P-Access-Network-Info header field, the access-class field set to "3GPP-NR-SAT", "3GPP-NR(LEO)", "3GPP-NR(MEO)", or "3GPP-NR(OTHERSAT)" are considered associated with satellite NG-RAN.

NOTE 4:	In this release of the specification, how the P-CSCF uses the satellite identifier to derive whether optimized media routing is possible is not specified.

Upon receiving the SIP INVITE message, the P-CSCF shall allocate an IMS-AGW on ground and send the identifier of the satellite serving the UE received together with an SDP offer to the terminating side to trigger media negotiation. The terminating P-CSCF shall return the 18X response together with the identifier of the satellite serving the remote UE, if any, and the SDP answer. If the identifier of the satellite serving the remote UE is not present, the originating P-CSCF shall not activate optimized media routing.

If the terminating P-CSCF indicates its activation of the optimized media routing, the P-CSCF re-selects an IMS-AGW on satellite and releases the IMS-AGW on ground. The P-CSCF shall send the modified SDP offer containing the IMS-AGW transport address to the terminating P-CSCF. Upon receiving the SDP offer, the terminating P-CSCF shall update the IMS-AGW on satellite for the terminating UE with the transport address received in the SDP offer.