3GPP TSG-CT WG1 Meeting #153	C1-250191

# Athens, Greece, 17-21 February 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4212 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RAT utilization control IE corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | ECRATU |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Related to the RAT utilization control IE the following issues exist: - The RAT utilization control IE max length is 4 octets in the definition, and the IE can be used as either 2 octets or 4 octets, and the min/max length should be included in the definition - Several messages specify length to 4-n, but as the is defined as max 4 octets this should be corrected. - For Type of RAT utilization control = “01” it is specified that contents apply to “…PLMN(s) included in Equivalent PLMNs IE”, but this is not correct as the RAT utilization control IE can be included in messages not supporting the Equivalent PLMNs IE. It is therefore more correct to refer to the “equivalent PLMNs list” It is proposed to correct the above issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Min and max lengths are added in IE definition IE length is corrected in applicable messages The dependency to Equivalent PLMNs IE is changed to equivalent PLMNs list |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear and incorrect specification can result in incompatible implementations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1, 8.2.3.1, 8.2.16.1, 8.2.26.1, 8.2.28.1, 9.9.3.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.2.1.1	Message definition

This message is sent by the network to the UE to indicate that the corresponding attach request has been accepted. See table 8.2.1.1.

Message type:	ATTACH ACCEPT

Significance:	dual

Direction:	network to UE

Table 8.2.1.1: ATTACH ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  | Security header type | Security header type 9.3.1 | M | V | 1/2 |
|  | Attach accept message identity | Message type 9.8 | M | V | 1 |
|  | EPS attach result | EPS attach result 9.9.3.10 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.9.2.9 | M | V | 1/2 |
|  | T3412 value | GPRS timer 9.9.3.16 | M | V | 1 |
|  | TAI list | Tracking area identity list 9.9.3.33 | M | LV | 7-97 |
|  | ESM message container | ESM message container 9.9.3.15 | M | LV-E | 5-n |
| 50 | GUTI | EPS mobile identity 9.9.3.12 | O | TLV | 13 |
| 13 | Location area identification | Location area identification 9.9.2.2 | O | TV | 6 |
| 23 | MS identity | Mobile identity 9.9.2.3 | O | TLV | 7-10 |
| 53 | EMM cause | EMM cause 9.9.3.9 | O | TV | 2 |
| 17 | T3402 value | GPRS timer 9.9.3.16 | O | TV | 2 |
| 59 | T3423 value | GPRS timer 9.9.3.16 | O | TV | 2 |
| 4A | Equivalent PLMNs | PLMN list 9.9.2.8 | O | TLV | 5-47 |
| 34 | Emergency number list | Emergency number list 9.9.3.37 | O | TLV | 5-50 |
| 64 | EPS network feature support | EPS network feature support 9.9.3.12A | O | TLV | 3-5 |
| F- | Additional update result | Additional update result 9.9.3.0A | O | TV | 1 |
| 5E | T3412 extended value | GPRS timer 3 9.9.3.16B | O | TLV | 3 |
| 6A | T3324 value | GPRS timer 2 9.9.3.16A | O | TLV | 3 |
| 6E | Extended DRX parameters | Extended DRX parameters 9.9.3.46 | O | TLV | 3 |
| 65 | DCN-ID | DCN-ID 9.9.3.48 | O | TLV | 4 |
| E- | SMS services status | SMS services status 9.9.3.4B | O | TV | 1 |
| D- | Non-3GPP NW provided policies | Non-3GPP NW provided policies 9.9.3.49 | O | TV | 1 |
| 6B | T3448 value | GPRS timer 2 9.9.3.16A | O | TLV | 3 |
| C- | Network policy | Network policy 9.9.3.52 | O | TV | 1 |
| 6C | T3447 value | GPRS timer 3 9.9.3.16B | O | TLV | 3 |
| 7A | Extended emergency number list | Extended emergency number list 9.9.3.37A | O | TLV-E | 7-65538 |
| 7C | Ciphering key data | Ciphering key data 9.9.3.56 | O | TLV-E | 35-2291 |
| 66 | UE radio capability ID | UE radio capability ID 9.9.3.60 | O | TLV | 3-n |
| B- | UE radio capability ID deletion indication | UE radio capability ID deletion indication 9.9.3.61 | O | TV | 1 |
| 35 | Negotiated WUS assistance information | WUS assistance information 9.9.3.62 | O | TLV | 3-n |
| 36 | Negotiated DRX parameter in NB-S1 mode | NB-S1 DRX parameter 9.9.3.63 | O | TLV | 3 |
| 38 | Negotiated IMSI offset | IMSI offset 9.9.3.64 | O | TLV | 4 |
| 1D | Forbidden TAI(s) for the list of "forbidden tracking areas for roaming" | Tracking area identity list 9.9.3.33 | O | TLV | 8-98 |
| 1E | Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service" | Tracking area identity list 9.9.3.33 | O | TLV | 8-98 |
| 1F | Unavailability configuration | Unavailability configuration 9.9.3.70 | O | TLV | 3-9 |
| 20 | RAT utilization control | RAT utilization control 9.9.3.3A | O | TLV | 4 |

* * * Next Change * * * *

#### 8.2.3.1	Message definition

This message is sent by the network to the UE to indicate that the corresponding attach request has been rejected. See table 8.2.3.1.

Message type:	ATTACH REJECT

Significance:	dual

Direction:	network to UE

Table 8.2.3.1: ATTACH REJECT message content

| IEI |  | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  |  | Security header type | Security header type 9.3.1 | M | V | 1/2 |
|  |  | Attach reject message identity | Message type 9.8 | M | V | 1 |
|  |  | EMM cause | EMM cause 9.9.3.9 | M | V | 1 |
| 78 |  | ESM message container | ESM message container 9.9.3.15 | O | TLV-E | 6-n |
| 5F |  | T3346 value | GPRS timer 2 9.9.3.16A | O | TLV | 3 |
| 16 |  | T3402 value | GPRS timer 2 9.9.3.16A | O | TLV | 3 |
| A- |  | Extended EMM cause | Extended EMM cause 9.9.3.26A | O | TV | 1 |
| 1C |  | Lower bound timer value | GPRS timer 3 9.9.3.16B | O | TLV | 3 |
| 1D |  | Forbidden TAI(s) for the list of "forbidden tracking areas for roaming" | Tracking area identity list 9.9.3.33 | O | TLV | 8-98 |
| 1E |  | Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service" | Tracking area identity list 9.9.3.33 | O | TLV | 8-98 |
| 20 | RAT utilization control | RAT utilization control 9.9.3.3A | O | TLV | 4 |

* * * Next Change * * * *

#### 8.2.16.1	Message definition

This message is sent by the network to the UE to reallocate a GUTI and optionally to provide a new TAI list or a new DCN-ID or both. See table 8.2.16.1.

Message type:	GUTI REALLOCATION COMMAND

Significance:	dual

Direction:	network to UE

Table 8.2.16.1: GUTI REALLOCATION COMMAND message content

| IEI |  | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  |  | Security header type | Security header type 9.3.1 | M | V | 1/2 |
|  |  | GUTI reallocation command message identity | Message type 9.8 | M | V | 1 |
|  |  | GUTI | EPS mobile identity 9.9.3.12 | M | LV | 12 |
| 54 |  | TAI list | Tracking area identity list 9.9.3.33 | O | TLV | 8-98 |
| 65 |  | DCN-ID | DCN-ID 9.9.3.48 | O | TLV | 4 |
| 66 |  | UE radio capability ID | UE radio capability ID 9.9.3.60 | O | TLV | 3-n |
| B- |  | UE radio capability ID deletion indication | UE radio capability ID deletion indication 9.9.3.61 | O | TV | 1 |
| 20 | RAT utilization control | RAT utilization control 9.9.3.3A | O | TLV | 2-4 |

* * * Next Change * * * *

#### 8.2.26.1	Message definition

This message is sent by the network to the UE to provide the UE with EPS mobility management related data in response to a tracking area update request message. See table 8.2.26.1.

Message type:	TRACKING AREA UPDATE ACCEPT

Significance:	dual

Direction:	network to UE

Table 8.2.26.1: TRACKING AREA UPDATE ACCEPT message content

| IEI |  | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  |  | Security header type | Security header type 9.3.1 | M | V | 1/2 |
|  |  | Tracking area update accept message identity | Message type 9.8 | M | V | 1 |
|  |  | EPS update result | EPS update result 9.9.3.13 | M | V | 1/2 |
|  |  | Spare half octet | Spare half octet 9.9.2.9 | M | V | 1/2 |
| 5A |  | T3412 value | GPRS timer 9.9.3.16 | O | TV | 2 |
| 50 |  | GUTI | EPS mobile identity 9.9.3.12 | O | TLV | 13 |
| 54 |  | TAI list | Tracking area identity list 9.9.3.33 | O | TLV | 8-98 |
| 57 |  | EPS bearer context status | EPS bearer context status 9.9.2.1 | O | TLV | 4 |
| 13 |  | Location area identification | Location area identification 9.9.2.2 | O | TV | 6 |
| 23 |  | MS identity | Mobile identity 9.9.2.3 | O | TLV | 7-10 |
| 53 |  | EMM cause | EMM cause 9.9.3.9 | O | TV | 2 |
| 17 |  | T3402 value | GPRS timer 9.9.3.16 | O | TV | 2 |
| 59 |  | T3423 value | GPRS timer 9.9.3.16 | O | TV | 2 |
| 4A |  | Equivalent PLMNs | PLMN list 9.9.2.8 | O | TLV | 5-47 |
| 34 |  | Emergency number list | Emergency number list 9.9.3.37 | O | TLV | 5-50 |
| 64 |  | EPS network feature support | EPS network feature support 9.9.3.12A | O | TLV | 3-5 |
| F- |  | Additional update result | Additional update result 9.9.3.0A | O | TV | 1 |
| 5E |  | T3412 extended value | GPRS timer 3 9.9.3.16B | O | TLV | 3 |
| 6A |  | T3324 value | GPRS timer 2 9.9.3.16A | O | TLV | 3 |
| 6E |  | Extended DRX parameters | Extended DRX parameters 9.9.3.46 | O | TLV | 3 |
| 68 |  | Header compression configuration status | Header compression configuration status 9.9.4.27 | O | TLV | 4 |
| 65 |  | DCN-ID | DCN-ID 9.9.3.48 | O | TLV | 4 |
| E- |  | SMS services status | SMS services status 9.9.3.4B | O | TV | 1 |
| D- |  | Non-3GPP NW policies | Non-3GPP NW provided policies 9.9.3.49 | O | TV | 1 |
| 6B |  | T3448 value | GPRS timer 2 9.9.3.16A | O | TLV | 3 |
| C- |  | Network policy | Network policy 9.9.3.52 | O | TV | 1 |
| 6C |  | T3447 value | GPRS timer 3 9.9.3.16B | O | TLV | 3 |
| 7A |  | Extended emergency number list | Extended emergency number list 9.9.3.37A | O | TLV-E | 7-65538 |
| 7C |  | Ciphering key data | Ciphering key data 9.9.3.56 | O | TLV-E | 35-2291 |
| 66 |  | UE radio capability ID | UE radio capability ID 9.9.3.60 | O | TLV | 3-n |
| B- |  | UE radio capability ID deletion indication | UE radio capability ID deletion indication 9.9.3.61 | O | TV | 1 |
| 35 |  | Negotiated WUS assistance information | WUS assistance information 9.9.3.62 | O | TLV | 3-n |
| 36 |  | Negotiated DRX parameter in NB-S1 mode | NB-S1 DRX parameter 9.9.3.63 | O | TLV | 3 |
| 38 |  | Negotiated IMSI offset | IMSI offset 9.9.3.64 | O | TLV | 4 |
| 37 |  | EPS additional request result | EPS additional request result 9.9.3.67 | O | TLV | 3 |
| 1D |  | Forbidden TAI(s) for the list of "forbidden tracking areas for roaming" | Tracking area identity list 9.9.3.33 | O | TLV | 8-98 |
| 1E |  | Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service " | Tracking area identity list 9.9.3.33 | O | TLV | 8-98 |
| 39 |  | Maximum time offset | GPRS timer 3 9.9.3.16B | O | TLV | 3 |
| 1F |  | Unavailability configuration | Unavailability configuration 9.9.3.70 | O | TLV | 3-9 |
| 20 | RAT utilization control | RAT utilization control 9.9.3.3A | O | TLV | 4 |

* * * Next Change * * * *

#### 8.2.28.1	Message definition

This message is sent by the network to the UE in order to reject the tracking area updating procedure. See table 8.2.28.1.

Message type:	TRACKING AREA UPDATE REJECT

Significance:	dual

Direction:	network to UE

Table 8.2.28.1: TRACKING AREA UPDATE REJECT message content

| IEI |  | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  |  | Security header type | Security header type 9.3.1 | M | V | 1/2 |
|  |  | Tracking area update reject message identity | Message type 9.8 | M | V | 1 |
|  |  | EMM cause | EMM cause 9.9.3.9 | M | V | 1 |
| 5F |  | T3346 value | GPRS timer 2 9.9.3.16A | O | TLV | 3 |
| A- |  | Extended EMM cause | Extended EMM cause 9.9.3.26A | O | TV | 1 |
| 1C |  | Lower bound timer value | GPRS timer 3 9.9.3.16B | O | TLV | 3 |
| 1D |  | Forbidden TAI(s) for the list of "forbidden tracking areas for roaming" | Tracking area identity list 9.9.3.33 | O | TLV | 9-98 |
| 1E |  | Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service" | Tracking area identity list 9.9.3.33 | O | TLV | 9-98 |
| 20 | RAT utilization control | RAT utilization control 9.9.3.3A | O | TLV | 4 |

* * * Next Change * * * *

#### 9.9.3.3A	RAT utilization control

The purpose of the RAT utilization control information element is for the network to communicate the restricted access technologies information to the UE.

The RAT utilization control information element is coded as shown in figure 9.9.3.3A.1 and table 9.9.3.3A.1.

The RAT utilization control is a type 4 information element with a minimum length of 2 octets and a maximum length of 4 octets.

Editor's note:	The specification of encoding RAT utilization control for roaming partner PLMNs is FFS.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 | 3 |  |  |  | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RAT utilization control IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |
| Length of RAT utilization control contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |
| 0 spare | 0 spare |  | 0 Spare |  | 0 spare |  | 0 spare |  |  |  | 0 spare | Type of RAT utilization control |  |  |  | octet 3* |
| 0 spare | 0 spare |  | Sat-NG-RAN |  | Sat-E-UTRAN |  | NG-RAN |  |  | E-UTRAN |  | UTRAN |  | GERAN |  | octet 4* |

Figure 9.9.3.3A.1: RAT utilization control information element

Table 9.9.3.3A.1: RAT utilization control information element

| Type of RAT utilization control (octet 3, bit 1 to 2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 2 |  | 1 |  |  |
| 0 |  | 0 |  | current PLMN |
| 0 |  | 1 |  | current PLMN and its equivalent PLMN(s) |
| 1 |  | 0 |  | Unused, shall be interpreted as "current PLMN" if received by the UE |
| 1 |  | 1 |  | Unused, shall be interpreted as "current PLMN" if received by the UE |
| If Type of RAT utilization control = “00”, octet 4 applies to the current PLMN. If Type of RAT utilization control = “01”, octet 4 applies to the current PLMN and PLMN(s) included in the equivalent PLMNs list, if any. |  |  |  |  |
| GERAN (octet 4, bit 1) |  |  |  |  |
| Bit |  |  |  |  |
| 1 |  |  |  |  |
| 0 |  |  |  | GERAN is not restricted |
| 1 |  |  |  | GERAN is restricted |
| UTRAN (octet 4, bit 2) |  |  |  |  |
| Bit |  |  |  |  |
| 2 |  |  |  |  |
| 0 |  |  |  | UTRAN is not restricted |
| 1 |  |  |  | UTRAN is restricted |
| E-UTRAN (octet 4, bit 3) |  |  |  |  |
| Bit |  |  |  |  |
| 3 |  |  |  |  |
| 0 |  |  |  | E-UTRAN is not restricted |
| 1 |  |  |  | E-UTRAN is restricted |
| NG-RAN (octet 4, bit 4) |  |  |  |  |
| Bit 4 |  |  |  |  |
| 0 |  |  |  | NG-RAN is not restricted |
| 1 |  |  |  | NG-RAN is restricted |
| Sat-E-UTRAN (octet 4, bit 5) |  |  |  |  |
| Bit |  |  |  |  |
| 5 |  |  |  |  |
| 0 |  |  | satellite E-UTRAN is not restricted |  |
| 1 |  |  | satellite E-UTRAN is restricted |  |
| Sat-NG-RAN(octet 4, bit 6) |  |  |  |  |
| Bit 6 |  |  |  |  |
| 0 |  |  | satellite NG-RAN is not restricted |  |
| 1 |  |  | satellite NG-RAN is restricted |  |

* * * End of Changes * * * *