3GPP TSG-CT WG1 Meeting #153	C1-250193

# Athens, Greece, 17-21 February 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4213 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for network initiated detach with RAT utilization control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | ECRATU |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CR4138 (C1-247171) introduced requirements to trigger network initiated detach for the case when RAT restriction is negotiated and applied, but due to changes the RAT the UE currently camps on gets restricted. The requirement includes the condition that the UE indicated support in the ATTACH REQUEST message. However, to include message details from a different procedure that is completed implies in unnecessarily strict requirements for the MME. Additionally, it would be more reasonable to base the condition on the last procedure in which support is indicated (attach or normal TAU) which was executed by the current MME. To simplify wording it is however proposed to use terminology and style aligned to the condition when network initiates GUTI reallocation (and in 5GS case) to update RAT restriction information. The condition of UE support is the same for these cases, so it is reasonable to express in the same way. Additionally, the detailed requirements for UE support indication and the negotiation of feature use are specified in detail in the procedures where negotiation is done, so there is no need to repeat in detail which results in increased dependencies between requirements/procedures and higher specification maintenance effort. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The condition to trigger network initiated detach when current RAT gets restricted is generalized for the UE supporting RAT utilization control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Triggering of network initiated detach to restrict current RAT depends on a detailed condition that results in unnecessarily complex network implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 5.5.2.3	Network initiated detach procedure

##### 5.5.2.3.1	Network initiated detach procedure initiation

The network initiates the detach procedure by sending a DETACH REQUEST message to the UE (see example in figure 5.5.2.3.1).

NOTE:	If the MME performs a local detach, it will inform the UE with an EMM messages (e.g. SERVICE REJECT or TRACKING AREA UPDATE reject) with EMM cause #10 "implicitly detached" only when the UE initiates an EMM procedure.

The network may include an EMM cause IE to specify the reason for the detach request. The network shall start timer T3422. If the Detach type IE indicates "re-attach required" or "re-attach not required" and the EMM cause value is not #2 "IMSI unknown in HSS", or if the MME performs a local detach, the MME shall either store the current EPS security context if it is a native EPS security context, or the MME shall delete the current EPS security context if it is a mapped EPS security context. If the detach type IE indicates "re-attach required" or "re-attach not required" and the EMM cause value is not #2 "IMSI unknown in HSS", the network shall deactivate the EPS bearer context(s), if any, for the UE locally and enter state EMM-DEREGISTERED-INITIATED.

In NB-S1 mode or WB-S1 mode via satellite E-UTRAN access, if the network initiates the detach procedure because the network determines that the UE is in a location where the network is not allowed to operate (see 3GPP TS 23.401 [10]), the network shall set the EMM cause value in the DETACH REQUEST message to #78 "PLMN not allowed to operate at the present UE location".

If

the UE supports RAT utilization control and

the MME needs to update the RAT utilization control information, including restriction of the access technology of the E-UTRAN cell on which the UE is camping,

the MME shall send the DETACH REQUEST message with the 5GMM cause value set to #15 "No suitable cells in tracking area" and include the RAT utilization control IE. In the RAT utilization control IE, the MME shall indicate that the access technology of the E-UTRAN cell on which the DETACH REQUEST message is sent as restricted.

NOTE A:	Other restricted access technologies can be indicated in the RAT utilization control IE, if any.

If the network detach is triggered because the MME determines that, by UE subscription and operator's preferences, all of the TAIs received from the satellite E-UTRAN are forbidden for roaming or for regional provision of service, the MME shall include the TAI(s) in:

a)	the Forbidden TAI(s) for the list of "forbidden tracking areas for roaming" IE;

b)	the Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service" IE; or

c)	both,

in the DETACH REQUEST message.

Figure 5.5.2.3.1: Network initiated detach procedure

* * * End of Changes * * * *