3GPP TSG-CT WG1 Meeting #153	C1-250613

# Athens, Greece, 17-21 February 2025

Source:	NOKIA

Title:	Pseudo-CR on Solution for KI #7 – Providing access control in the VPLMN providing disaster roaming services in EPS

Spec:	3GPP TR 24.812

Agenda item:	20.1

Document for:	Decision

1. Introduction

The key issue #7: Prevention of signalling overload in the VPLMN providing disaster roaming services in EPS is under the scope of FS_MINT_Ph2 which has been approved in CT#105. This pCR proposes a solution to the defined key issue on how to provide access control by the EPS to prevent signaling overload by the UEs migrating to VPLMN providing disaster roaming services in EPS when disaster occurs in 5GS.

2. Reason for Change

This solution describes how to control the access from the migrating subscribers/UEs to the VPLMN providing disaster roaming services in EPS.

4. Proposal

It is proposed to agree the following solution and changes to 3GPP TS 24.812.

* * * First Change * * * *

## 6.x	Solution #x: Access control in the E-UTRAN providing disaster roaming services

### 6.x.1	Description

#### 6.x.1.1	Introduction

This solution is mapped to the Key Issue #7 – Prevention of signaling overload in the VPLMN providing disaster roaming services in EPS as shown in the following table:

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Solution #x |  |  |  |  |  |  | X |  |

#### 6.x.1.2	Detailed description

Once the disaster condition occurs in the VPLMN providing 5G-only national roaming, the UEs currently connected to it will try to attach to the VPLMN providing disaster roaming services in EPS (since it is the one that is currently available). In order to avoid congestion due to a potential large number of UEs requesting the establishment of a NAS-signalling connection at the same time or within a short period of time to the VPLMN providing disaster roaming services in EPS, it is necessary to control the access of the UEs to the EPS. Although the disaster wait range for EPS (as agreed solution using Access Identity 3 and uac-BarringFactorForAI3 in 5GS) might mitigate the issue of overloading condition up to some extent, it is not enough to solve it completely as the disaster wait range does not dynamically reflect the network and congestion situation and it is set (e.g., pre-configured) before the disaster condition actually happens. This is the reason UAC was modified in Release 17 MINT and an additional access identity (Access Identity 3) is used when an inbound disaster roamer wants to make an access attempt in the disaster service providing PLMN. However, in Release 19 MINT (MINT Ph2), access control in EPS is not defined as it is defined in 5GS (Access Identity 3 and uac-BarringFactorForAI3 in 5GS).

Therefore the following steps are proposed as the solution to provide access control in EPS in disaster scenario:

-	when the EMM sublayer of the UE needs to request the establishment of a NAS-signalling connection or to request the lower layers to resume a NAS signalling connectionsend while the UE is attached or is attaching for disaster roaming. The EMM sublayer in the UE sends, to the AS layer in the UE, a call type set to "disaster roaming";

-	the E-UTRAN of the VPLMN providing disaster roaming services in EPS broadcasts barring factor for disaster roaming;

-	upon receiving the call type "disaster roaming", the AS layer in the UE checks whether access to the cell is barred or not taking into account the broadcast barring factor for disaster roaming and a randomly drawn number. The AS layer then indicates the NAS layer if the access is barred.

### 6.x.2	Impacts on existing nodes and functionality

The following impacts can be identified:

E-UTRAN:

-	The E-UTRAN of the VPLMN providing disaster roaming services in EPS broadcasts barring factor for disaster roaming.

UE:

-	EMM sublayer in the UE sends, to the AS layer in the UE, a call type set to "disaster roaming" when the EMM sublayer of the UE needs to request the establishment of a NAS-signalling connection or to request the lower layers to resume a NAS signalling connectionsend while the UE is attached or is attaching for disaster roaming.

-	Upon receiving the call type "disaster roaming", the AS layer in the UE performs barring check using the broadcast barring factor for disaster roaming and a randomly drawn number. The AS layer then indicates the EMM sublayer if the access is barred or not.

* * * End of Changes * * * *