3GPP TS 29.522 V16.3.0 (2020-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System; Network Exposure Function Northbound APIs;

Stage 3

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Contents	3

Foreword	8

1	Scope	9

2	References	9

3	Definitions and abbreviations	10

3.1	Definitions	10

3.2	Abbreviations	10

4	NEF Northbound Interface	11

4.1	Overview	11

4.2	Reference model	12

4.3	Functional elements	12

4.3.1	NEF	12

4.3.2	AF	13

4.4	Procedures over NEF Northbound Interface	13

4.4.1	Introduction	13

4.4.2	Procedures for Monitoring	13

4.4.3	Procedures for Device Triggering	14

4.4.4	Procedures for resource management of Background Data Transfer	14

4.4.5	Procedures for CP Parameters Provisioning	15

4.4.6	Procedures for PFD Management	15

4.4.7	Procedures for Traffic Influence	15

4.4.7.1	General	15

4.4.7.2	AF request identified by UE address	16

4.4.7.3	AF request not identified by UE address	16

4.4.7.4	Handling of UP path management event notification	17

4.4.8	Procedures for changing the chargeable party at session set up or during the session	17

4.4.9	Procedures for setting up an AF session with required QoS	17

4.4.10	Procedures for MSISDN-less Mobile Originated SMS	18

4.4.11	Procedures for Network Configuration Parameters Provisioning	18

4.4.12	Procedures for Non-IP data delivery	18

4.4.12.1	General	18

4.4.12.2	NIDD configuration Triggered by the NEF	18

4.4.12.3	NIDD configuration triggered by the AF and NIDD delivery	18

4.4.13	Procedures for RACS Parameter Provisioning	19

4.4.14	Procedures for analytics information exposure	19

4.4.14.1	Subscription/unsubscription to notification of analytics information	19

4.4.14.2	Fetch analytics information	20

4.4.15	Procedures for 5G LAN Parameter Provisioning	21

4.4.15.1	General	21

4.4.15.2	Creation of a new subscription for 5G LAN parameter provisioning	21

4.4.15.3	Modification of an existing subscription for 5G LAN parameter provisioning	21

4.4.15.4	Deletion of an existing subscription for 5G LAN parameter provisioning	21

4.4.16	Procedures for applying BDT policy	21

4.4.17	Procedures for Enhanced Coverage Restriction Control	22

4.4.18	Procedures for IPTV Configuration	22

5	NEF Northbound APIs	23

5.1	Introduction	23

5.2	Information applicable to several APIs	23

5.3	Reused APIs	23

5.4	TrafficInfluence API	24

5.4.1	Resources	24

5.4.1.1	Overview	24

5.4.1.2	Resource: Traffic Influence Subscription	25

5.4.1.2.1	Introduction	25

5.4.1.2.2	Resource Definition	25

5.4.1.2.3	Resource Methods	25

5.4.1.2.3.1	General	25

5.4.1.2.3.2	GET	25

5.4.1.2.3.3	POST	26

5.4.1.3	Resource: Individual Traffic Influence Subscription	26

5.4.1.3.1	Introduction	26

5.4.1.3.2	Resource Definition	26

5.4.1.3.3	Resource Methods	27

5.4.1.3.3.1	General	27

5.4.1.3.3.2	GET	27

5.4.1.3.3.3	PUT	27

5.4.1.3.3.4	PATCH	28

5.4.1.3.3.5	DELETE	28

5.4.2	Notifications	29

5.4.2.1	Introduction	29

5.4.2.2	Event Notification	29

5.4.2.2.1	Description	29

5.4.2.2.2	Target URI	29

5.4.2.2.3	Operation Definition	29

5.4.2.2.3.1	Notification via HTTP POST	29

5.4.2.2.3.2	Notification via Websocket	30

5.4.2.3	Acknowledgement of event notification	30

5.4.2.3.1	Description	30

5.4.2.3.2	Target URI	30

5.4.2.3.3	Operation Definition	30

5.4.2.3.3.1	Notification via HTTP POST	30

5.4.3	Data Model	30

5.4.3.1	General	30

5.4.3.2	Reused data types	30

5.4.3.3	Structured data types	31

5.4.3.3.1	Introduction	31

5.4.3.3.2	Type: TrafficInfluSub	31

5.4.3.3.3	Type: TrafficInfluSubPatch	33

5.4.3.3.4	Type: EventNotification	35

5.4.3.3.5	Type: AfResultInfo	36

5.4.3.3.6	Type AfAckInfo	36

5.4.3.4	Simple data types and enumerations	36

5.4.3.4.1	Introduction	36

5.4.3.4.2	Simple data types	36

5.4.3.4.3	Enumeration: SubscribedEvent	36

5.4.3.4.4	Enumeration: AfResultStatus	36

5.4.4	Used Features	37

5.5	NiddConfigurationTrigger API	37

5.5.1	Resources	37

5.5.2	Notifications	37

5.5.2.1	Introduction	37

5.5.2.2	Event Notification	37

5.5.2.3	Operation Definition	38

5.5.2.3.1	Notification via HTTP POST	38

5.5.2.3.2	Notification via Websocket	38

5.5.3	Data Model	38

5.5.3.1	General	38

5.5.3.2	Reused data types	38

5.5.3.3	Structured data types	38

5.5.3.3.1	Introduction	38

5.5.3.3.2	Type: NiddConfigTrigger	38

5.5.3.3.3	Type: NiddConfigTriggerReply	39

5.5.3.4	Simple data types and enumerations	39

5.5.3.4.1	Introduction	39

5.5.3.4.2	Simple data types	39

5.5.4	Used Features	39

5.6	AnalyticsExposure API	40

5.6.1	Resources	40

5.6.1.1	Overview	40

5.6.1.2	Resource: Analytics Exposure Subscriptions	40

5.6.1.2.1	Introduction	40

5.6.1.2.2	Resource Definition	40

5.6.1.2.3	Resource Methods	41

5.6.1.2.3.1	General	41

5.6.1.2.3.2	GET	41

5.6.1.2.3.3	POST	41

5.6.1.3	Resource: Individual Analytics Exposure Subscription	42

5.6.1.3.1	Introduction	42

5.6.1.3.2	Resource Definition	42

5.6.1.3.3	Resource Methods	42

5.6.1.3.3.1	General	42

5.6.1.3.3.2	GET	42

5.6.1.3.3.3	PUT	43

5.6.1.3.3.4	DELETE	43

5.6.1a	Custom Operations without associated resources	44

5.6.1a.1	Overview	44

5.6.1a.2	Operation: fetch	44

5.6.1a.2.1	Description	44

5.6.1a.2.2	Operation Definition	44

5.6.2	Notifications	44

5.6.2.1	Introduction	44

5.6.2.2	Event Notification	45

5.6.2.3	Operation Definition	45

5.6.2.3.1	Notification via HTTP POST	45

5.6.2.3.2	Notification via Websocket	45

5.6.3	Data Model	45

5.6.3.1	General	45

5.6.3.2	Reused data types	45

5.6.3.3	Structured data types	46

5.6.3.3.1	Introduction	46

5.6.3.3.2	Type: AnalyticsExposureSubsc	46

5.6.3.3.3	Type: AnalyticsEventNotification	47

5.6.3.3.4	Type: AnalyticsEventNotif	47

5.6.3.3.5	Type: AnalyticsEventSubsc	47

5.6.3.3.6	Type: AnalyticsEventFilterSubsc	48

5.6.3.3.7	Type TargetUeId	48

5.6.3.3.8	Type UeMobilityExposure	48

5.6.3.3.9	Type UeTrajectoryInfo	49

5.6.3.3.10	Type UeLocationInfo	49

5.6.3.3.11	Type UeCommunicationExposure	49

5.6.3.3.12	Type: AnalyticsRequest	50

5.6.3.3.13	Type AnalyticsEventFilter	50

5.6.3.3.14	Type AnalyticsData	50

5.6.3.4	Simple data types and enumerations	50

5.6.3.4.1	Introduction	50

5.6.3.4.2	Simple data types	51

5.6.3.4.3	Enumeration: AnalyticsEvent	51

5.6.4	Used Features	51

5.7	5GLANParameterProvision API	51

5.7.1	Resources	51

5.7.1.1	Overview	51

5.7.1.2	Resource: 5GLAN Parameters Provision Subscriptions	52

5.7.1.2.1	Introduction	52

5.7.1.2.2	Resource Definition	52

5.7.1.2.3	Resource Methods	52

5.7.1.2.3.1	General	52

5.7.1.2.3.2	GET	53

5.7.1.2.3.3	POST	53

5.7.1.3	Resource: Individual 5GLAN Parameters Provision Subscription	54

5.7.1.3.1	Introduction	54

5.7.1.3.2	Resource Definition	54

5.7.1.3.3	Resource Methods	54

5.7.1.3.3.1	General	54

5.7.1.3.3.2	GET	54

5.7.1.3.3.3	PUT	55

5.7.1.3.3.4	DELETE	55

5.7.1.3.3.5	PATCH	56

5.7.2	Data Model	56

5.7.2.1	General	56

5.7.2.2	Reused data types	56

5.7.2.3	Structured data types	56

5.7.2.3.1	Introduction	56

5.7.2.3.2	Type: 5GLanParametersProvision	57

5.7.2.3.3	Type: 5GLanParameters	57

5.7.2.3.4	Type: AppDescriptor	57

5.7.2.3.5	Type: 5GLanParametersProvisionPatch	57

5.7.2.3.6	Type: 5GLanParametersPatch	58

5.7.2.4	Simple data types and enumerations	58

5.7.2.4.1	Introduction	58

5.7.2.4.2	Simple data types	58

5.7.3	Used Features	58

5.8	ApplyingBdtPolicy API	58

5.8.1	Resources	58

5.8.1.1	Overview	58

5.8.1.2	Resource: Applied BDT Policy Subscription	59

5.8.1.2.1	Introduction	59

5.8.1.2.2	Resource Definition	59

5.8.1.2.3	Resource Methods	59

5.8.1.2.3.1	General	59

5.8.1.2.3.2	GET	60

5.8.1.2.3.3	POST	60

5.8.1.3	Resource: Individual Applied BDT Policy Subscription	60

5.8.1.3.1	Introduction	60

5.8.1.3.2	Resource Definition	61

5.8.1.3.3	Resource Methods	61

5.8.1.3.3.1	General	61

5.8.1.3.3.2	GET	61

5.8.1.3.3.3	PATCH	61

5.8.1.3.3.4	DELETE	62

5.8.2	Notifications	62

5.8.3	Data Model	62

5.8.3.1	General	62

5.8.3.2	Reused data types	62

5.8.3.3	Structured data types	63

5.8.3.3.1	Introduction	63

5.8.3.3.2	Type: AppliedBdtPolicy	63

5.8.3.3.3	Type: AppliedBdtPolicyPatch	63

5.8.3.4	Simple data types and enumerations	63

5.8.3.4.1	Introduction	63

5.8.3.4.2	Simple data types	64

5.8.4	Used Features	64

5.9	IPTVConfiguration API	64

5.9.1	Resources	64

5.9.1.1	Overview	64

5.9.1.2	Resource: IPTV Configurations	65

5.9.1.2.1	Introduction	65

5.9.1.2.2	Resource Definition	65

5.9.1.2.3	Resource Methods	65

5.9.1.2.3.1	General	65

5.9.1.2.3.2	GET	65

5.9.1.2.3.3	POST	66

5.9.1.3	Resource: Individual IPTV Configuration	66

5.9.1.3.1	Introduction	66

5.9.1.3.2	Resource Definition	66

5.9.1.3.3	Resource Methods	67

5.9.1.3.3.1	General	67

5.9.1.3.3.2	GET	67

5.9.1.3.3.3	PUT	67

5.9.1.3.3.4	DELETE	68

5.9.1.3.3.5	PATCH	68

5.9.2	Data Model	69

5.9.2.1	General	69

5.9.2.2	Reused data types	69

5.9.2.3	Structured data types	69

5.9.2.3.1	Introduction	69

5.9.2.3.2	Type: IptvConfigData	70

5.9.2.3.3	Type: MulticastAccessControl	70

5.9.2.3.4	Type: IptvConfigDataPatch	70

5.9.2.4	Simple data types and enumerations	70

5.9.2.4.1	Introduction	70

5.9.2.4.2	Simple data types	71

5.9.2.4.3	Enumeration: AccessRightStatus	71

5.9.3	Used Features	71

6	Security	71

7	Using Common API Framework	71

7.1	General	71

7.2	Security	72

Annex A (normative):	OpenAPI representation for NEF Northbound APIs	73

A.1	General	73

A.2	TrafficInfluence API	73

A.3	NiddConfigurationTrigger API	80

A.4	AnalyticsExposure API	82

A.5	5GLANParameterProvision API	89

A.6	ApplyingBdtPolicy API	94

A.7	IPTVConfiguration API	97

Annex B (informative):	Change history	103

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present specification describes the protocol for the NEF Northbound interface between the NEF and the AF. The NEF Northbound interface and the related stage 2 functional requirements are defined in 3GPP TS 23.502 [2], 3GPP TS 23.316 [28] and 3GPP TS 23.288 [29].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.502: "Procedures for the 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G".

[4]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[5]	Open API Initiative, "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[6]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[7]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[8]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[9]	3GPP TS 29.521: "5G System; Binding Support Management Service; Stage 3".

[10]	Void.

[11]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[12]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[13]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[14]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[15]	Void.

[16]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[17]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[18]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[19]	3GPP TS 29.554: "5G System; Background Data Transfer Policy Control Service; Stage 3".

[20]	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

[21]	3GPP TR 21.900: "Technical Specification Group working methods".

[22]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[23]	3GPP TS 29.519: "5G System; Usage of the Unified Data Repository service for Policy Control Data, Application Data and Structured Data for Exposure; Stage 3".

[24]	3GPP TS 29.541: "5G System; Network Exposure FunctionServices for Non-IP Data Delivery (NIDD); Stage 3".

[25]	3GPP TS 29.542: "5G System, Session Management Services for Non-IP Data Delivery (NIDD); Stage 3".

[26]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[27]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[28]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G system (5GS)".

[29]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[30]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[31]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to Vehicle-to-Everything (V2X) services".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AF	Application Function

BDT	Background Data Transfer

CAPIF	Common API Framework

CP	Communication Pattern

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

GPSI	Generic Public Subscription Identifier

IPTV	Internet Protocol Television

NEF	Network Exposure Function

PCF	Policy Control Function

PCRF	Policy and Charging Rule Function

PFD	Packet Flow Description

PFDF	Packet Flow Description Function

REST	Representational State Transfer

SCEF	Service Capability Exposure Function

S-NSSAI	Single Network Slice Selection Assistance Information

UDR	Unified Data Repository

UP	User Plane

# 4	NEF Northbound Interface

## 4.1	Overview

The NEF Northbound interface is between the NEF and the AF. It specifies RESTful APIs that allow the AF to access the services and capabilities provided by 3GPP network entities and securely exposed by the NEF.

This document also specifies the procedures triggered at the NEF by API requests from the AF and by event notifications received from 3GPP network entities.

The stage 2 level requirements and signalling flows for the NEF Northbound interface are defined in 3GPP TS 23.502 [2].

The NEF Northbound interface supports the following procedures:

1)	Procedures for Monitoring

2)	Procedures for Device Triggering

3)	Procedures for resource management of Background Data Transfer

4)	Procedures for CP Parameters, Network Configuration Parameters Provisioning, 5G LAN Parameters Provisioning and Location Privacy Indication Parameters Provisioning

5)	Procedures for PFD Management

6)	Procedures for Traffic Influence

7)	Procedures for changing the chargeable party at session set up or during the session

8)	Procedures for setting up an AF session with required QoS

9)	Procedures for MSISDN-less Mobile Originated SMS

10)		Procedures for non-IP data delivery

11)	Procedures for analytics information exposure

12)	Procedure for applying BDT policy

13)	Procedures for Enhanced Coverage Restriction Control

14)	Procedures for IPTV Configuration

15)	Procedures for Service Parameter Provisioning

Which correspond to the following services respectively, supported by the NEF as defined in 3GPP TS 23.502 [2]:

1)	Nnef_EventExposure service and Nnef_APISupportCapability service

2)	Nnef_Trigger service

3)	Nnef_BDTPNegotiation service

4)	Nnef_ParameterProvision service

5)	Nnef_PFDManagement service

6)	Nnef_TrafficInfluence service

7)	Nnef_ChargeableParty service

8)	Nnef_AFsessionWithQoS service

9)	Nnef_MSISDN-less_MO_SMS service

10)		Nnef_NIDDConfiguration and Nnef_NIDD services

11)	Nnef_AnalyticsExposure service

12)	Nnef_ApplyPolicy service

13)	Nnef_ECRestriction service

14)	Nnef_IPTVConfiguration service

15)	Nnef_ServiceParameter service

NOTE 1:	For Nnef_PFDManagement service, only the Nnef_PFDManagement_Create/Update/Delete service operations are applicable for the NEF Northbound interface.

NOTE 2:	For Nnef_NIDD service, NF consumer other than the AF does not use the NEF Northbound interface.

NOTE 3:	For Nnef_NIDDConfiguration service, the Nnef_NIDDConfiguration_Trigger service operation is only applicable for the NEF Northbound interface.

NOTE 4:	The Nnef_APISupportCapability service is only applicable in the MonitoringEvent API when the monitoring type sets to "API_SUPPORT_CAPABILITY".

## 4.2	Reference model

The NEF Northbound interface resides between the NEF and the AF as depicted in figure 4.2.1. The overall NEF architecture is depicted in 3GPP TS 23.502 [2]. An AF can get services from multiple NEFs, and an NEF can provide service to multiple AFs.

NOTE:	The AF can be provided by the third party.

Figure 4.2-1: Reference Architecture for the Nnef Service; SBI representation

Figure 4.2-2: Reference Architecture for the Nnef Service; reference point representation

## 4.3	Functional elements

### 4.3.1	NEF

The Network Exposure Function (NEF) is a functional element that supports the following functionalities:

-	The NEF shall securely expose network capabilities and events provided by 3GPP NFs to AF.

-	The NEF shall provide a means for the AF to securely provide information to 3GPP network and may authenticate, authorize and assist in throttling the AF.

-	The NEF shall be able to translate the information received from the AF to the one sent to internal 3GPP NFs, and vice versa.

-	The NEF shall support to expose information (collected from other 3GPP NFs) to the AF.

-	The NEF may support a PFD Function which allows the AF to provision PFD(s) and may store and retrieve PFD(s) in the UDR. The NEF further provisions PFD(s) to the SMF.

A specific NEF instance may support one or more of the functionalities described above and consequently an individual NEF may support a subset of the APIs specified for capability exposure.

NOTE:	The NEF can access the UDR located in the same PLMN as the NEF.

### 4.3.2	AF

The Application Function (AF) may interact with the 3GPP Core Network via the NEF in order to access network capabilities.

## 4.4	Procedures over NEF Northbound Interface

### 4.4.1	Introduction

All procedures that operate across the NEF Northbound interface, as specified in 3GPP TS 23.502 [2], are specified in the following subclauses.

### 4.4.2	Procedures for Monitoring

The procedures for monitoring as described in subclause 4.4.2 of 3GPP TS 29.122 [4] shall be applicable in 5GS with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the HSS applies to the UDM, and the NEF shall interact with the UDM by using Nudm_EventExposure service as defined in 3GPP TS 29.503 [17];

-	description of the MME/SGSN applies to the AMF, and the NEF shall interact with the AMF by using Namf_EventExposure service as defined in 3GPP TS 29.518 [18];

-	description about the PCRF is not applicable;

-	description about the change of IMSI-IMEI(SV) association monitoring event applies to the change of SUPI-PEI association monitoring event;

-	description about the PDN connectivity status event applies to the PDU session status event;

-	when sending the UDM/AMF event report to the AF, the NEF may store the event data in the report in the UDR as part of the data for exposure as specified in 3GPP TS 29.519 [23] by using Nudr_DataRepository service as specified in 3GPP TS 29.504 [20].

-	If the "Downlink_data_delivery_status_5G" as defined in subclause 5.3.4 of 3GPP TS 29.122 [4] is supported, in order to support the downlink data delivery status notification,

-	the AF shall send an HTTP POST message to the NEF to the resource "Monitoring Event Subscriptions" as defined in 5.3.3.2 of 3GPP TS 29.122 [4] for creating an subscription or send an HTTP PUT message to the NEF to the resource "Individual Monitoring Event Subscription" as defined in 5.3.3.3 of 3GPP TS 29.122 [4] for updating the subscription with the following difference:

-	within the MonitoringEventSubscription data structure the AF may additionally include packet filter descriptor(s) within the "dddTraDescriptors" attribute and the list of monitoring downlink data delivery status event(s) within the "dddStati" attribute;

-	the NEF shall subscribe the events to the appropriate UDM(s) within the network by invoking the Nudm_EventExposure_Subscribe service operation as defined in subclause 5.5.2.2 of 3GPP TS 29.503 [17].

-	when the NEF receives the event notification as defined in subclause 4.4.2 of 3GPP TS 29.508 [26], the NEF shall send an HTTP POST message to the AF as defined in subclause 4.4.2.3 of 3GPP TS 29.122 [4] with the difference that within each MonitoringEventReport data structure, the NEF shall include:

-	the downlink data delivery status within the "dddStatus" attribute;

-	the downlink data descriptor impacted by the downlink data delivery status change within the "dddTraDescriptor" attribute;

-	the estimated buffering time within the "maxWaitTime" attribute if the downlink data delivery status is set to "BUFFERED";

-	If the "Availability_after_DDN_failure_notification_enhancement" feature as defined in subclause 5.3.4 of 3GPP TS 29.122 [4] is supported, the AF shall send an HTTP POST message to the NEF to the resource "Monitoring Event Subscriptions" as defined in 5.3.3.2 of 3GPP TS 29.122 [4] for creating an subscription or send an HTTP PUT message to the NEF to the resource "Individual Monitoring Event Subscription" as defined in 5.3.3.3 of 3GPP TS 29.122 [4] for updating the subscription with the difference that within the MonitoringEventSubscription data structure, the AF shall include packet filter descriptions within the "dddTraDescriptors" attribute.

4.4.3	Procedures for Device Triggering

The procedures for device triggering as described in subclause 4.4.6 of 3GPP TS 29.122 [4] shall be applicable in 5G with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the HSS applies to the UDM;

-	the NEF shall interact with the UDM by using the Nudm_SubscriberDataManagement service and the Nudm_UEContextManagement service as defined in 3GPP TS 29.503 [17]; and

-	the NEF acts as MTC-IWF.

### 4.4.4	Procedures for resource management of Background Data Transfer

The procedures for resource management of Background Data Transfer (BDT) in 5GS are described in subclause 4.4.3 of 3GPP TS 29.122 [4] with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	if the external group Id is sent from the AF to the NEF, the NEF shall interact with the UDM by using Nudm_SubscriberDataManagement service as defined in 3GPP TS 29.503 [17] to translate the external group identifier into the corresponding internal group identifier;

-	description of the PCRF applies to the PCF; and

-	the NEF shall interact with the PCF by using Npcf_BDTPolicyControl service as defined in 3GPP TS 29.554 [19].

-	If the "BdtNotification_5G" is supported, the AF may include a notification URI within the "notificationDestination" attribute in the BDT data type during the background data transfer policy negotiation. In addition, the AF may request to enable the BDT warning notification by setting the "warnNotifEnabled" attribute to true. When the NEF receives the BDT warning notification from the PCF as defined in clause 4.2.4.2 of 3GPP TS 29.554 [19] and the "warnNotifEnabled" attribute was set to true, the NEF shall send an HTTP POST message including the ExNotification data structure to the AF identified by the notification destination URI received during the background data transfer policy negotiation. The AF shall respond with an HTTP response to confirm the received notification. The AF may select one policy from the candidate of BDT policies if provided in the notification by using the HTTP PATCH message as described in subclause 5.4.3.3.3.3 of 3GPP TS 29.122 [4]. The AF may also request to disable/enable the BDT warning notification by including the "warnNotifEnabled" attribute in the HTTP PATCH message.

### 4.4.5	Procedures for CP Parameters Provisioning

The procedures for CP parameters provisioning as described in subclause 4.4.9 of 3GPP TS 29.122 [4] shall be applicable in 5G with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the HSS applies to the UDM;

-	the NEF shall interact with the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17].

### 4.4.6	Procedures for PFD Management

The procedures for PFD management as described in subclause 4.4.10 of 3GPP TS 29.122 [4] shall be applicable for 5GS with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF; and

-	the NEF (PFDF) shall interact with the UDR for PFD management by using Nudr_DataRepository service as defined in 3GPP TS 29.504 [20]. The PFDF is functionality within the NEF.

-	If the PFDs are provisioned to at least one of the subscribed SMFs (but not all) within the allowed delay, the NEF (PFDF) may notify the AF about the failed PFD provisioning with the HTTP POST message by including the PfdReport data structure in the body of the message. In addition, the NEF may include the location area(s) of the user plane(s) which are unable to enforce the provisioned PFD(s) within the "locationArea" attribute of the PFD report(s). If the PFDs are provisioned to none of the subscribed SMFs within the allowed delay, the NEF (PFDF) shall notify the AF about the failed PFD provisioning with the HTTP POST message using appropriate failure code as defined in Table 5.11.2.2.3-1 of 3GPP TS 29.122 [4].

NOTE 1:	Unsuccessful PFDs provisioning to the subscribed SMFs within the allowed delay means that the PFDs are not provisioned successfully to the UPFs served by the failed SMFs.

NOTE 2:	The NEF maps the 3GPP network area(s) to the geographic area(s), civic address(es) or DNAI(s) if the 3GPP network area(s) is not allowed to be exposed to the 3rd party according to the operator policy.

### 4.4.7	Procedures for Traffic Influence

#### 4.4.7.1	General

In order to create a resource for the Traffic Influence, the AF shall send an HTTP POST message to the NEF to the resource "Traffic Influence Subscription", the body of the HTTP POST message may include the AF Service Identifier, external Group Identifier, external Identifier, any UE Indication, the UE IP address, GPSI, DNN, S-NSSAI, Application Identifier or traffic filtering information, Subscribed Event, Notification destination address, a list of geographic zone identifier(s), AF Transaction Identifier, a list of DNAI(s), routing profile ID(s) or N6 traffic routing information, Indication of application relocation possibility, type of notifications, Temporal, spatial validity conditions, and if the URLLC feature is supported, Indication of AF acknowledgement to be expected and/or Indication of UE IP address preservation. The Notification destination address shall be included if the Subscribed Event is included in the HTTP request message.

In order to update an existing traffic influence subscription, the AF shall send an HTTP PUT or PATCH message to the resource "Individual Traffic Influence Subscription" requesting to change the traffic influence parameters.

In order to delete an existing traffic influence subscription, the AF shall send an HTTP DELETE message to the NEF to the resource "Individual Traffic Influence Subscription".

Upon receipt of the HTTP request from the AF, if the AF is authorized, the NEF shall perform the mapping as described in 3GPP TS 23.501 [3], and then perform as described in subclause 4.4.7.2 if the request is for an individual UE or perform as described in subclause 4.4.7.3 if the request is for multiple UEs.

#### 4.4.7.2	AF request identified by UE address

Upon receipt of the above AF request which is for an individual UE identified by IP or Ethernet address, the NEF may interact with the BSF to retrieve the related PCF information by invoking the Nbsf_Management_Discovery service operation as described in 3GPP TS 29.521 [9], if the NEF receives an error code from the BSF, the NEF shall not create, update or delete the resource and shall respond to the AF with a status code set to 500 Internal Server Error.

After receiving a successful response from the BSF, the NEF shall interact with the PCF by invoking the Npcf_PolicyAuthorization service as described in 3GPP TS 29.514 [7]. After receiving a successful response from the PCF, the NEF shall,

-	for the HTTP POST request, create a resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, addressed by a URI that contains the AF Identifier and an NEF-created subscription identifier, and shall respond to the AF with a 201 Created status code, including a Location header field containing the URI for the created resource. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this traffic influence subscription.

-	for the HTTP PUT or PATCH request, update a resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, and shall responds to the AF with a 200 OK status code.

-	for the HTTP DELETE request, remove all properties of the resource and delete the corresponding active resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, then shall responds to the AF with a 204 No Content status code.

If the NEF receives a response with an error code from the PCF, the NEF shall not create, update or delete the resource and shall respond to the AF with a status code set to 500 Internal Server Error.

#### 4.4.7.3	AF request not identified by UE address

For AF request not identified by UE address, it may target an individual UE, a group of UEs or any UE. For an individual UE identified by GPSI, or a group of UEs identified by External Group Identifier, the NEF shall interact with the UDM by invoking the Nudm_SubscriberDataManagement service as described in 3GPP TS 29.503 [17] to retrieve the SUPI or Internal Group Identifier.

The NEF shall interact with the UDR by invoking the Nudr_DataRepository service as described in 3GPP TS 29.504 [20], if the NEF receives an error code from the UDR, the NEF shall not create, update or delete the resource and shall respond to the AF with a status code set to 500 Internal Server Error.

After receiving a successful response from the UDR, the NEF shall,

-	for the HTTP POST request, create a resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, addressed by a URI that contains the AF Identifier and an NEF-created subscription identifier, and shall respond to the AF with a 201 Created status code, including a Location header field containing the URI for the created resource. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this traffic influence subscription.

-	for the HTTP PUT or PATCH request, update a resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, and shall responds to the AF with a 200 OK status code.

-	for the HTTP DELETE request, delete the corresponding active resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, and shall responds to the AF with a 204 No Content status code.

#### 4.4.7.4	Handling of UP path management event notification

If the NEF receives a UP path management event notification from the SMF indicating that the subscribed event has been detected, then the NEF shall provide a notification by sending an HTTP POST message that shall include the EventNotification data type at least with the subscribed event (e.g. UP Path has changed) to the AF identified by the notification destination received during creation of the Individual Traffic Influence Subscription. If a URI for AF acknowledgement within the "ackUri" attribute is provided by the SMF in the event notification as defined in 3GPP TS 29.508 [26], the NEF shall also provide a URI for AF acknowledgement within the "afAckUri" attribute in the EventNotification data.

Upon receipt of the event notification, the AF shall respond with a "204 No Content" status code to confirm the received event notification.

Afterwards, if a URI for AF acknowledgement within the "afAckUri" attribute is received during the UP path management event notification, the AF may determine that an application layer relocation is needed, and may then send an HTTP POST request as acknowledgement for the UP path management event notification to inform the NEF about the result of application layer relocation. If the application layer is ready and/or the application relocation is completed, within the payload of the HTTP POST request, the AF shall include the AfAckInfo data type with the "afStatus" attribute sets to "SUCCESS" and may provide the N6 traffic routing information associated to the target DNAI as a "trafficRoute" attribute within the AfResultInfo data; otherwise, the AF shall indicate the failure by including the AfAckInfo data type in the payload with the "afStatus" attribute sets to the corresponding failure cause. The NEF Northbound interface transaction identifier generated by the AF shall also be provided as the "afTransId" attribute within the AfAckInfo data if the AF has previously provided it.

Upon receipt of the AF acknowledgement, the NEF shall respond with a "204 No Content" status code to confirm the received acknowledgement.

### 4.4.8	Procedures for changing the chargeable party at session set up or during the session

The procedures for changing the chargeable party at session set up or during the session in 5GS are described in subclause 4.4.4 of 3GPP TS 29.122 [4] with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the PCRF applies to the PCF; and

-	the NEF may interact with BSF by using Nbsf_Management_Discovery service (as defined in 3GPP TS 29.521 [9]) to retrieve the PCF address; and

-	the NEF shall interact with the PCF by using Npcf_PolicyAuthorization service as defined in 3GPP TS 29.514 [7].

### 4.4.9	Procedures for setting up an AF session with required QoS

The procedures for setting up an AF session with required QoS in 5GS are described in subclause 4.4.13 of 3GPP TS 29.122 [4] with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the PCRF applies to the PCF; and

-	the NEF may interact with BSF by using Nbsf_Management_Discovery service as defined in 3GPP TS 29.521 [9] to retrieve the PCF address;

-	the NEF shall interact with the PCF by using Npcf_PolicyAuthorization service as defined in 3GPP TS 29.514 [7] ; and

-	if the "QoSMonitoring_5G" feature as defined in subclause 5.3.4 of 3GPP TS 29.122 [4] is supported, in order to support the QoS Monitoring, the AF shall include "qosMonInfo" attribute. Within the QosMonitoringInformation data structure, the AF shall include:

-	one or more requested QoS Monitoring Parameter(s) within the "reqQosMonParams"; and

-	one or more report frequency within the "repFreqs" attribute; and

-	when the "repFreqs" attribute includes the "PERIODIC" is set to the value "PERIODIC", the reporting period within the "repPeriod" attribute; and

-	when the "repFreqs" attribute set to the value "EVENT_TRIGGERED", the AF shall include:

-	the delay threshold for downlink with the "repThreshDl" attribute;

-	the delay threshold for uplink with the "repThreshUl" attribute; and/or

-	the delay threshold for round trip with the "repThreshRp" attribute; and

-		the minimum waiting time between subsequent reports within the "waitTime" attribute.

-	when the NEF receives the event notification as defined in subclause 4.4.2 of 3GPP TS 29.508 [26] or subclause 4.2.5.x of 3GPP TS 29.514 [7], the NEF shall include the QoS monitoring report within the "qosMonReport" attribute. Within the QosMonitoringReport data structure, the NEF shall include:

-	one or two uplink packet delays within the "ulDelays" attribute;

-	one or two downlink packet delays within the "dlDelays" attribute; and/or

-	one or two round trip packet delays within the "rtDelays" attribute.

-	if the "AlternativeQoS_5G" feature is supported, the AF may include an ordered list of QoS references within the "altQosReferences" attribute. The NEF shall transfer them to the PCF in the Npcf_PolicyAuthorization service. The NEF shall also subscribe to PCF event "QOS_NOTIF" in the Npcf_PolicyAuthorization service. When the NEF receives the notification of PCF event "QOS_NOTIF", it shall notify the AF with "QOS_GUARANTEED" event; or "QOS_NOT_GUARANTEED" event with the currently applied QoS reference.

NOTE:	Based on the operator configuration, the QoS reference identifiers received from the AF can be the same or different as the QoS reference identifiers known at the PCF. The NEF can perform a mapping for the QoS reference identifier.

### 4.4.10	Procedures for MSISDN-less Mobile Originated SMS

The procedures are used by the NEF to send the MSISDN-less MO-SMS to the AF in 5GS are described in subclause 4.4.14 of 3GPP TS 29.122 [4] with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF; and

-	the NEF shall interact with UDM by using Nudm_SubscriberDataManagement service (as defined in 3GPP TS 29.503 [17]) to retrieve the external identifier.

### 4.4.11	Procedures for Network Configuration Parameters Provisioning

The procedures for network configuration parameters provisioning as described in subclause 4.4.12 of 3GPP TS 29.122 [4] shall be applicable in 5GS with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the HSS applies to the UDM; and

-	the NEF shall interact with the UDM by using Nudm_ParameterProvision service as specified in 3GPP TS 29.503 [17].

### 4.4.12	Procedures for Non-IP data delivery

#### 4.4.12.1	General

The procedures are used by the NEF to send/receive the non-IP data to/from the AF. It comprises NIDD configuration and NIDD delivery.

The NIDD configuration may be triggered by the NEF or the AF. If it is triggered by the NEF, the NiddConfigurationTrigger API described in subclause 5.5 is used and the procedure is described in subclause 4.4.12.2.

#### 4.4.12.2	NIDD configuration Triggered by the NEF

If the NEF receives a NIDD connection establishment request from the SMF and if there is no NIDD configuration for the UE, the NEF may send a NIDD configuration trigger to the AF. The NEF determines the destination URI by local configuration. The NEF shall send to the determined destination URL an HTTP POST request that shall include a NiddConfiguarationTrigger data type with:

-	the NEF identifier,

-	the AF identifier, and

-	GPSI as UE identity.

The AF shall acknowledge the HTTP POST request with an HTTP 200 OK response. Then the AF may start NIDD configuration procedure as described in subclause 4.4.12.3.

#### 4.4.12.3	NIDD configuration triggered by the AF and NIDD delivery

The procedures for NIDD configuration triggered by the AF and NIDD delivery are described in subclause 4.4.5 of 3GPP TS 29.122 [4] with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the MME/SGSN applies to the SMF;

-	for the connection establishment, the interaction between the NEF and the SMF shall use Nnef_SMContext service as specified in 3GPP TS 29.541 [24];

-	for MO NIDD, the interaction between the SMF and the NEF shall use Nnef_SMContext service as specified in 3GPP TS 29.541 [24]; and

-	for MT NIDD, the interaction between the SMF and the NEF shall use Nsmf_NIDD service as specified in 3GPP TS 29.542 [25].

### 4.4.13	Procedures for RACS Parameter Provisioning

The procedures for RACS parameter provisioning as described in subclause 4.4.15 of 3GPP TS 29.122 [4] shall be applicable in 5G with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF.

### 4.4.14	Procedures for analytics information exposure

#### 4.4.14.1	Subscription/unsubscription to notification of analytics information

The procedures are used by the AF to subscribe/unsubscribe to retrieve analytics information via NEF, and are used by the NEF to notify the AF about the requested analytics information as described in 3GPP TS 23.288 [29].

In order to subscribe to retrieve analytics information, the AF shall send an HTTP POST message to the NEF to the resource "Analytics Exposure Subscriptions", the HTTP POST message shall include AnalyticsExposureSubsc data structure as request body.

The AnalyExposureSubsc data structure shall include:

-	an URI where to receive the requested notifications as "notifUri" attribute;

-	Notification Correlation Identifier assigned by the NF service consumer for the requested notifications as "notifId" attribute;

-	list of supported features by the service consumer as "supportedFeatures" attribute; and

-	a description of the subscribed events as "analyEventsSubs" attribute that for each event shall include

1)	 an event identifier as "analyEvent" attribute.

The AnalyExposureSubsc data structure may include:

-	event reporting requirement information as "analyRepInfo" attribute, which applies for all events in a subscription and may contain the following attributes:

1)		event notification method (periodic, one time, on event detection) as "notifMethod" attribute;

2)	maximum Number of Reports as "maxReportNbr" attribute;

3)	monitoring Duration as "monDur" attribute;

4)	repetition period for periodic reporting as "repPeriod" attribute;

5)	immediate reporting indication as "immRep" attribute;

6)	sampling ratio as "sampRatio" attribute;

7)	group reporting guard time as "grpRepTime" attribute.

8)	identification of time window to which the subscription applies via identification of date-time(s) as "startTs" and "endTs" attributes; and/or

9)	preferred level of accuracy of the analytics as "accuracy" attribute.

For each event included in the "analyEventsSubs" attribute, an event filter as "analyEventFilter" attribute may be provided which may include:

-	indication of the UEs to which the subscription applies via:

1)	identification of an individual UE via a "gpsi" attribute; or

2)	identification of a group of UE(s) via a "exterGroupId" attribute; or

3)	identification of any UE via the "anyUeInd" attribute.

In order to update an existing analytics exposure subscription, the AF shall send an HTTP PUT message to the NEF to the resource "Individual Analytics Exposure Subscription" requesting to change the subscription.

In order to delete an existing analytics exposure subscription, the AF shall send an HTTP DELETE message to the NEF to the resource "Individual Analytics Exposure Subscription".

Upon receipt of the HTTP request from the AF, if the AF is authorized, the NEF shall interact with the NWDAF to subscribe to, modify or cancel the subscription to the analytics information by using the Nnwdaf_EventsSubscription service as defined in 3GPP TS 29.520 [27]. If the NEF receives an error code from the NWDAF, the NEF shall not create, update or delete the resource and shall respond to the AF with a status code set to 500 Internal Server Error.

After receiving a successful response from the NWDAF, the NEF shall,

-	for the HTTP POST request, create a resource "Individual Analytics Exposure Subscription" which represents the analytics exposure subscription, addressed by a URI that contains the AF Identifier and an NEF-created subscription identifier, and shall respond to the AF with a 201 Created status code, including a Location header field containing the URI for the created resource. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this analytics exposure subscription.

-	for the HTTP PUT request, update a resource "Individual Analytics Exposure Subscription" which represents the analytics exposure subscription, and shall responds to the AF with a 200 OK or 204 No Content status code.

-	for the HTTP DELETE request, remove all properties of the resource and delete the corresponding active resource "Individual Analytics Exposure Subscription" which represents the analytics exposure subscription, then shall responds to the AF with a 204 No Content status code.

If the NEF receives an analytics information notification from the NWDAF indicating that the subscribed analytics event has been detected, the NEF shall provide a notification by sending HTTP POST message that include the AnalyEventNotification data structure at least with the detected analytics event to the AF identified by the notification URI together with the notification correlation identifier received during creation of the Individual Analytics Exposure Subscription. Upon receipt of the analytics event notification, the AF shall respond with a "204 No Content" status code to confirm the received notification.

#### 4.4.14.2	Fetch analytics information

The procedures are used by the AF to fetch analytics information via NEF.

In order to fetch analytics information, the AF shall send an HTTP POST message to the NEF to the customized operation URI "{apiRoot}/3gpp-analyticsexposure/v1/fetch", the HTTP POST message shall include AnalyticsRequest data structure as request body. The AnalyticsRequest data structure shall include:

-	identification of the analytics events as "analyEvents" attribute;

and may include:

-	description of the analytics reporting information as "analyRep" attribute;

-	an event filter as "evtFilter" attribute.

-	indication of the UEs to which the analytics request applies via:

a)	identification of an individual UE via a "gpsi" attribute; or

b)	identification of a group of UE(s) via a "exterGroupId" attribute; or

c)	identification of any UE via the "anyUeInd" attribute.

Upon the reception of an HTTP POST request, if the AF is authorized, the NEF shall interact with the UDM by using Nudm_SubscriberDataManagement service as defined in 3GPP TS 29.503 [17] to translate the GPSI or external group identifier into the corresponding SUPI or internal group identifier. After receiving a successful response from the UDM, the NEF shall interact with the NWDAF by using Nnwdaf_AnalyticsInfo service as defined in 3GPP TS 29.520 [27]. If the NEF receives an error code from the NWDAF, the NEF shall respond to the AF with a status code set to 500 Internal Server Error.If a successful response including analytics information is received from the NWDAF, the NEF shall translate the network internal information to external information (e.g. SUPI to GPSI, Internal Group ID to External Group ID) and send an HTTP POST response to the AF by including analytics information within the AnalyticsData data structure.

### 4.4.15	Procedures for 5G LAN Parameter Provisioning

#### 4.4.15.1	General

The procedures are used by the AF to provision 5G LAN type service related parameters to the NEF. The following procedures support:

-	Management of 5G Virtual Network group membership; and/or

-	Management of 5G Virtual Network group data

#### 4.4.15.2	Creation of a new subscription for 5G LAN parameter provisioning

In order to create a new subscription to provision 5G LAN related parameters, the AF shall initiate an HTTP POST request to the NEF for the "5GLAN Parameters Provision Subscriptions" resource. The body of the HTTP POST message shall include the 5G LAN service related parameters within the "5gLanParams" attribute.

Upon receipt of the corresponding HTTP POST message, if the AF is authorized by the NEF to provision the parameters, the NEF shall interact with the UDM to create a subscription at the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17]. If the request is accepted by the UDM and the UDM informs the NEF with a successful response, the NEF shall create a new subscription and assign a subscription identifier for the "Individual 5GLAN Parameters Provision Subscriptions" resource. Then the NEF shall send a HTTP "201 Created" response with 5gLanParametersProvision data structure as response body and a Location header field containing the URI of the created individual subscription resource.

#### 4.4.15.3	Modification of an existing subscription for 5G LAN parameter provisioning

To modify an existing subscription to provision 5G LAN parameters, the AF shall initiate an HTTP PUT/PATCH request to the NEF for the "Individual 5GLAN Parameters Provision Subscription" resource. The body of the HTTP PUT message shall include the 5GLanParametersProvision data type as defined in subclause 5.7.2.3.2. The External Group Identifier shall remain unchanged from previous values. The body of the HTTP PATCH message shall include the 5GLanParametersProvisionPatch data as defined in subclause 5.7.2.3.5.

Upon receipt of the corresponding HTTP PUT/PATCH message, if the AF is authorized by the NEF to provision the parameters, the NEF shall interact with the UDM to modify an existing subscription at the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17]. If the modification request is accepted by the UDM and the UDM informs the NEF with a successful response, the NEF shall update the existing subscription for the "Individual 5GLAN Parameters Provision Subscriptions" resource. Then the NEF shall send a HTTP response including "200 OK" status code with 5GLanParametersProvision data structure or "204 No Content" status code.

#### 4.4.15.4	Deletion of an existing subscription for 5G LAN parameter provisioning

To delete an existing subscription to 5GLAN provision parameters, the AF shall initiate an HTTP DELETE request to the NEF for the "Individual 5GLAN Parameters Provision Subscription" resource.

Upon receipt of the corresponding HTTP DELETE message, if the AF is authorized, the NEF shall interact with the UDM to delete an existing parameters provision subscription at the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17]. If the request is accepted by the UDM and informs the NEF with a successful response, the NEF shall delete the existing subscription for the "Individual 5GLAN Parameters Provision Subscriptions" resource. Then the NEF shall send a HTTP "204 No Content" response.

### 4.4.16	Procedures for applying BDT policy

In order to create a resource for the applying a previously negotiated Background Data Transfer Policy to a UE or a Group of UEs, the AF shall send an HTTP POST message to the NEF to the resource "Applied BDT Policy Subscriptions". The body of the HTTP POST message shall contain the external Group Identifier or external Identifier, and the Background Data Transfer Reference ID for a previously negotiated policy of a background data transfer.

Upon receipt of the HTTP POST request from the AF, if the AF is authorized, the NEF shall interact with the UDM by invoking the Nudm_SubscriberDataManagement service as described in 3GPP TS 29.503 [17] to retrieve the SUPI or Internal Group Identifier.

In order to update an existing applied BDT policy subscription, the AF shall send an HTTP PATCH message to the resource "Individual Applied BDT Policy Subscription" requesting to change the applied BDT policy. The AF shall include in the body of the HTTP PATCH request the new Background Data Transfer Reference ID.

In order to delete an existing applied BDT policy subscription, the AF shall send an HTTP DELETE message to the NEF to the resource "Individual Applied BDT Policy Subscription".

The NEF shall interact with the UDR by invoking the Nudr_DataRepository service as described in 3GPP TS 29.504 [20], if the NEF receives an error code from the UDR, the NEF shall not create, update or delete the resource and shall respond to the AF with a status code set to "500 Internal Server Error".

After receiving a successful response from the UDR, the NEF shall:

-	for the HTTP POST request, create a resource "Individual Applied BDT Policy Subscription" addressed by a URI that contains the AF Identifier and an NEF-created subscription identifier, and shall respond to the AF with a "201 Created" status code, including a Location header field containing the URI of the created resource. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this resource.

-	for the HTTP PATCH request, update a resource "Individual Applied BDT Policy Subscription" which represents the applied BDT policy subscription, and shall respond to the AF with a "200 OK" status code.

-	for the HTTP DELETE request, delete the corresponding active resource "Individual Applied BDT Policy Subscription", and shall respond to the AF with a "204 No Content" status code.

### 4.4.17	Procedures for Enhanced Coverage Restriction Control

The procedures for network configuration parameters provisioning as described in subclause 4.4.11 of 3GPP TS 29.122 [4] shall be applicable in 5GS with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the HSS applies to the UDM; and

-	the NEF shall interact with the UDM by using Nudm_SubscriberDataManagement service as specified in 3GPP TS 29.503 [17].

### 4.4.18	Procedures for IPTV Configuration

The procedures are used by the AF to authorize the request and forward the request for IPTV configuration information via NEF.

In order to configure IPTV information, the AF shall send an HTTP POST message to the NEF to the resource "IPTV Configurations", the HTTP POST message shall include IptvConfigData data structure as request body. The IptvConfigData data structure shall include:

-	indication of the UEs to which the subscription applies via:

a)	identification of an individual UE via a "gpsi" attribute; or

b)	identification of a group of UE(s) via a "exterGroupId" attribute;

-	an application identifier as "appId" attribute; and

-	a list of Multicast Access Control as "multiAccCtrls" attribute;

and may include:

-	an DNN as "dnn" attribute;

-	an S-NSSAI as "snssai" attribute.

In order to update an existing analytics exposure subscription, the AF shall send an HTTP PUT or HTTP PATCH message to the NEF to the resource "Individual IPTV Configuration" requesting to change the subscription.

In order to delete an existing analytics exposure subscription, the AF shall send an HTTP DELETE message to the NEF to the resource "Individual IPTV Configuration".

Upon receipt of the HTTP request from the AF, if the AF is authorized, the NEF shall interact with the UDM by invoking the Nudm_SubscriberDataManagement service as described in 3GPP TS 29.503 [17] to retrieve the SUPI or Internal Group Identifier. Then the NEF shall interact with the UDR to create, update or delete the IPTV configuration by using the Nudr_DataRepository service as defined in 3GPP TS 29.519 [23]. If the NEF receives an error code from the UDR, the NEF shall not create, update or delete the resource and shall respond to the AF with a status code set to 500 Internal Server Error.

After receiving a successful response from the UDR, the NEF shall,

-	for the HTTP POST request, create a resource "Individual IPTV Configuration" which represents the IPTV configuration request, addressed by a URI that contains the AF Identifier and an NEF-created configuration identifier, and shall respond to the AF with a 201 Created status code, including a Location header field containing the URI for the created resource. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this IPTV configuration.

-	for the HTTP PUT or HTTP PATCH request, update a resource "Individual IPTV Configuration" which represents the IPTV configuration, and shall responds to the AF with a 200 OK or 204 No Content status code.

-	for the HTTP DELETE request, remove all properties of the resource and delete the corresponding active resource "Individual IPTV Configuration", then shall responds to the AF with a 204 No Content status code.

- 4.4.19	Procedures for Location Privacy Indication Parameters Provisioning

The procedures are used by the AF to provision Location Privacy Indication parameters to the NEF. The procedures are applicable for an individual UE or a group of UEs.

In order to provision Location Privacy Indication parameters, the AF shall initiate an HTTP POST request to the NEF for the "LPI Parameters Provisionings" resource. The body of the HTTP POST message shall include the Location Privacy Indication related parameters within the LpiParametersProvision data structure.

Upon receipt of the corresponding HTTP POST message, if the AF is authorized by the NEF to provision the parameters, the NEF shall interact with the UDM to create a resource at the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17]. If the request is accepted by the UDM and the UDM informs the NEF with a successful response, the NEF shall create a new resource and assign an identifier for the "Individual LPI Parameters Provisioning" resource. Then the NEF shall send a HTTP "201 Created" response with LpiParameterProvision data structure as response body and a Location header field containing the URI of the created individual resource.

In order to update an existing individual LPI Parameters Provisioning, the AF may send an HTTP PUT message to the resource "individual LPI Parameters Provisioning" requesting the NEF to change all properties in the existing resource. The body of the HTTP PUT request message shall include LpiParametersProvision data type as defined in subclause 5.10.2.3.2. The External Group Identifier or GPSI shall remain unchanged from previous values.

Upon receipt of the corresponding HTTP PUT message, if the AF is authorized by the NEF to provision the parameters, the NEF shall interact with the UDM to modify an existing resource at the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17]. If the modification request is accepted by the UDM and the UDM informs the NEF with a successful response, the NEF shall update the existing resource for the "Individual LPI Parameters Provisioning" resource. Then the NEF shall send a HTTP response including "200 OK" status code with LpiParametersProvision data structure or "204 No Content" status code.

To delete an existing to LPI Parameters Provisioning, the AF shall initiate an HTTP DELETE request to the NEF for the "Individual LPI Parameters Provisioning" resource.

Upon receipt of the corresponding HTTP DELETE message, if the AF is authorized, the NEF shall interact with the UDM to delete an existing LPI Parameters Provisioning at the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17]. If the request is accepted by the UDM, the NEF shall delete the existing resource for the "Individual LPI Parameters Provisioning" resource. Then the NEF shall send a HTTP "204 No Content" response.

### 4.4.20	Procedures for service specific parameter provisioning

The procedures are used by the AF to provide service specific parameters to 5G system via NEF.

In order to provision the service specific parameter, the AF shall send an HTTP POST message to the NEF to the resource "Service Parameter Subscriptions", the HTTP POST message shall include ServiceParameterData data structure as request body. The ServiceParameterData data structure shall include:

-	service description via:

a)	an combination of DNN and S-NSSAI within the "dnn" attribute and the "snssai" attribute respectively;

b)	an AF servcie identifier within the "afServiceId" attribute; or

c)	an application identifier within the "appId" attribute;

-	indication of the UEs to which the subscription applies via:

a)	identification of an individual UE within the "gpsi" attribute;

b)	an IPv4 address of the UE within the "ueIpv4" attribute;

c)	an IPv6 address of the UE within the "ueIpv6" attribute;

d)	an MAC address of the UE within the "ueMac" attribute;

e)	an identification of a group of UE(s) within the "exterGroupId" attribute; or

f)	identification of any UE within the "anyUeInd" attribute.

-	configuration parameters for V2X communication over PC5 within the "paramOverPc5" attribute; and

-	configuration parameters for V2X communication over Uu within the "paramOverUu" attribute.

In order to update an existing analytics exposure subscription, the AF shall send an HTTP PUT or HTTP PATCH message to the NEF to the resource "Individual Service Parameter Subscription" requesting to change the subscription.

In order to delete an existing service paramter subscription, the AF shall send an HTTP DELETE message to the NEF to the resource "Individual Service Parameter Subscription".

Upon receipt of the HTTP request from the AF, if the AF is authorized, the NEF shall interact with the UDM by invoking the Nudm_SubscriberDataManagement service as described in 3GPP TS 29.503 [17] to retrieve the SUPI or Internal Group Identifier. Then the NEF shall interact with the UDR to create, update or delete the service parameter by using the Nudr_DataRepository service as defined in 3GPP TS 29.519 [23]. If the NEF receives an error code from the UDR, the NEF shall not create, update or delete the resource and shall respond to the AF with a status code set to 500 Internal Server Error.

After receiving a successful response from the UDR, the NEF shall,

-	for the HTTP POST request, create a resource "Individual Service Parameter Subscription" which represents the Service Parameter provisioning request, addressed by a URI that contains the AF Identifier and an NEF-created configuration identifier, and shall respond to the AF with a 201 Created status code, including a Location header field containing the URI for the created resource. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this Service Parameter Subscription.

-	for the HTTP PUT or HTTP PATCH request, update a resource "Individual Service Parameter Subscription" which represents the service parameter, and shall responds to the AF with a 200 OK or 204 No Content status code.

-	for the HTTP DELETE request, remove all properties of the resource and delete the corresponding active resource "Individual Service Parameter Subscription", then shall responds to the AF with a 204 No Content status code.

# 5	NEF Northbound APIs

## 5.1	Introduction

The NEF Northbound APIs are a set of APIs defining the related procedures and resources for the interaction between the NEF and the AF.

## 5.2	Information applicable to several APIs

The usage of HTTP, content type and URI structure definition, as specified in subclauses 5.2.2, 5.2.3 and 5.2.4 of 3GPP TS 29.122 [4] respectively, shall be applicable for NEF Northbound APIs.

The notification, error handling, feature negotiation, HTTP custom headers as specified in subclauses 5.2.5, 5.2.6, 5.2.7, 5.2.8 of 3GPP TS 29.122 [4] respectively, shall be applicable for NEF Northbound APIs except that the SCEF is replaced by the NEF and the SCS/AS is replaced by the AF.

The conventions for Open API specification files as specified in subclause 5.2.10 of 3GPP TS 29.122 [4] shall be applicable for NEF Northbound APIs.

## 5.3	Reused APIs

This subclause describes the northbound APIs which are applicable for both EPS and 5GS.

Table 5.3-1: Reused APIs applicable for both EPS and 5GS

| API Name | Differences |
| --- | --- |
| ResourceManagementOfBdt | - The "LocBdt_5G" feature as described in subclause 5.4.4 of 3GPP TS 29.122 [4] may only be supported in 5G. - The "Group_Id" feature as described in subclause 5.4.4 of 3GPP TS 29.122 [4] may be supported in 5G. - The "BdtNotification_5G" feature as described in subclause 5.4.4 of 3GPP TS 29.122 [4] may only be supported in 5G. |
| PfdManagement | The "FailureLocation_5G" feature as described in subclause 5.11.4 of 3GPP TS 29.122 [4] may only be supported in 5G. |
| MonitoringEvent | - The "Number_of_UEs_in_an_area_notification_5G" feature as described in subclause 5.3.4 of 3GPP TS 29.122 [4] may only be supported in 5G. - The "Downlink_data_delivery_status_5G" feature as described in subclause 5.3.4 of 3GPP TS 29.122 [4] may only be supported in 5G. - The "Availability_after_DDN_failure_notification_enhancement" feature as described in subclause 5.3.4 of 3GPP TS 29.122 [4] may only be supported in 5G. - For the "Pdn_connectivity_status" feature, APN is equivalent to DNN; the non-IP PDN type is equivalent to the unstructured PDU session type; and the enumeration InterfaceIndication value "PDN_GATEWAY" stands for PDU session anchored in UPF in 5G. |
| DeviceTriggering |  |
| CpProvisioning | - The "ExpectedUMT_5G" and "ExpectedUmtTime_5G" features as described in subclause 5.10.4 of 3GPP TS 29.122 [4] may only be supported in 5G. - The "ScheduledCommType_5G" feature as described in subclause 5.10.4 of 3GPP TS 29.122 [4] may only be supported in 5G. |
| ChargeableParty | - The "EthChgParty_5G" and "MacAddressRange_5G" features as described in subclause 5.5.4 of 3GPP TS 29.122 [4] may only be supported in 5G. - The events (i.e. LOSS_OF_BEARER, RECOVERY_OF_BEARER and RELEASE_OF_BEARER) do not apply for 5G. |
| AsSessionWithQoS | - The "EthAsSessionQoS_5G", "QoSMonitoring_5G", "MacAddressRange_5G" and "AlternativeQoS_5G" features as described in subclause 5.14.4 of 3GPP TS 29.122 [4] may only be supported in 5G. - The events (i.e. LOSS_OF_BEARER, RECOVERY_OF_BEARER and RELEASE_OF_BEARER) do not apply for 5G. |
| MsisdnLessMoSms |  |
| NpConfiguration | The "NpExpiry_5G” feature as described in subclause 5.13.4 of 3GPP TS 29.122 [4] may only be supported in 5G. |
| NIDD |  |
| RacsParameterProvisioning |  |
| ECRControl |  |

## 5.4	TrafficInfluence API

### 5.4.1	Resources

#### 5.4.1.1	Overview

This subclause describes the structure for the Resource URIs as shown in figure 5.4.1.1-1 and the resources and HTTP methods used for the TrafficInfluence API.

Figure 5.4.1.1-1: Resource URI structure of the TrafficInfluence API

Table 5.4.1.1-1 provides an overview of the resources and HTTP methods applicable for the TrafficInfluence API.

Table 5.4.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Traffic Influence Subscription | {apiRoot}/3gpp-traffic-Influence/v1/{afId}/subscriptions | GET | Read all subscriptions for a given AF |
|  |  | POST | Create a new subscription to traffic influence |
| Individual Traffic Influence Subscription | {apiRoot}/3gpp-traffic-Influence/v1/{afId}/subscriptions/{subscriptionId} | GET | Read the subscription to the traffic influence |
|  |  | PUT | Modify all of the properties of an existing subscription to an traffic influence |
|  |  | PATCH | Modify part of the properties of an existing subscription to an traffic influence |
|  |  | DELETE | Delete the subscription to the traffic influence |

#### 5.4.1.2	Resource: Traffic Influence Subscription

##### 5.4.1.2.1	Introduction

This resource allows a AF to read all active traffic influence subscribtions for the given AF.

##### 5.4.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-traffic-influence/v1/{afId}/subscriptions

This resource shall support the resource URI variables defined in table 5.4.1.2.2-1.

Table 5.4.1.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |

##### 5.4.1.2.3	Resource Methods

###### 5.4.1.2.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.4.1.2.2.

###### 5.4.1.2.3.2	GET

The GET method allows to read all active subscriptions for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.4.1.2.3.2-1.

Table 5.4.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.4.1.2.3.2-2 and the response data structures and response codes specified in table 5.4.1.2.3.2-3.

Table 5.4.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.4.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(TrafficInfluSub) | M | 0..N | 200 OK | The subscription information for the AF in the request URI are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.4.1.2.3.3	POST

The POST method creates a new subscription resource to traffic influence subscription for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.4.1.2.3.3-1 and the response data structures and response codes specified in table 5.4.1.2.3.3-2.

Table 5.4.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | Parameters to register a subscription to influencing traffic routing and/or notification about UP management events with the NEF. |

Table 5.4.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | 201 Created | The subscription was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

#### 5.4.1.3	Resource: Individual Traffic Influence Subscription

##### 5.4.1.3.1	Introduction

This resource allows a AF to register a subscription to influencing traffic routing and/or notification about UP management events with the NEF.

##### 5.4.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-traffic-influence/v1/{afId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.4.1.3.2-1.

Table 5.4.1.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |
| subscriptionId | Identifier of the subscription resource of type string. |

##### 5.4.1.3.3	Resource Methods

###### 5.4.1.3.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.4.1.3.2.

###### 5.4.1.3.3.2	GET

The GET method allows to read the active subscription for a given AF and subscription Id. The AF shall initiate the HTTP GET request message and theNEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.4.1.3.3.2-1.

Table 5.4.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.4.1.3.3.2-2 and the response data structures and response codes specified in table 5.4.1.3.3.2-3.

Table 5.4.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.4.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | 200 OK | The subscription information for the AF in the request URI are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.4.1.3.3.3	PUT

The PUT method modifies an existing subscription resource to update a subscription. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.4.1.3.3.3-1 and the response data structures and response codes specified in table 5.4.1.3.3.3-2.

Table 5.4.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | Modify an existing subscription to influencing traffic routing and/or notification about UP management events with the NEF. |

Table 5.4.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | 200 OK | The subscription was updated successfully. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.4.1.3.3.4	PATCH

The PATCH method allows to change some properties of an existing traffic influence subscription. The AF shall initiate the HTTP PATCH request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.4.1.3.3.4-1 and the response data structures and response codes specified in table 5.4.1.3.3.4-2.

Table 5.4.1.3.3.4-1: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrafficInfluSubPatch | M | 1 | Partial update of a subscription to influencing traffic routing and/or notifications about UP management events with the NEF. |

Table 5.4.1.3.3.4-2: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | 200 OK | The subscription was modified successfully. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.4.1.3.3.5	DELETE

The DELETE method deletes the traffic influence subscription for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.4.1.3.3.5-1.

Table 5.4.1.3.3.5-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.4.1.3.3.5-2 and the response data structures and response codes specified in table 5.4.1.3.3.5-3.

Table 5.4.1.3.3.5-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.4.1.3.3.5-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The subscription was terminated successfully. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

### 5.4.2	Notifications

#### 5.4.2.1	Introduction

Upon receipt of a UP management event notification from the SMF indicating the subscribed event (e.g. a DNAI has changed) is detected, the NEF shall send an HTTP POST message including the notified event to the AF.

Upon receipt of the event notification, the AF may send an HTTP POST request as acknowledgement for the UP path management event notification to inform the NEF about the result of application layer relocation.

The NEF and the AF shall support the notification mechanism as described in subclause 5.2.5 of 3GPP TS 29.122 [4].

#### 5.4.2.2	Event Notification

##### 5.4.2.2.1	Description

The Event Notification is used by the NEF to report the UP path management event notification from the SMF to the AF.

##### 5.4.2.2.2	Target URI

URI: {notificationDestination}

The operation shall support the URI variables defined in table 5.4.2.2.2-1.

Table 5.4.2.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| notificationDestination | Callback reference provided by the AF during creation of the subscription within the TrafficInfluSub data type as defined in Table 5.4.3.3.2-1. |

#### 5.4.2.2.3	Operation Definition

##### 5.4.2.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.4.2.2.3.1-1 and the response data structures and response codes specified in table 5.4.2.2.3.1-2.

Table 5.4.2.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EventNotification | M | 1 | The UP management event notification is provided by the NEF to the AF. |

Table 5.4.2.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The event notification is received successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

##### 5.4.2.2.3.2	Notification via Websocket

If supported by both AF and NEF and successfully negotiated, the EventNotification may alternatively be delivered through the Websocket mechanism as defined in subclause 5.2.5.4 of 3GPP TS 29.122 [4].

#### 5.4.2.3	Acknowledgement of event notification

##### 5.4.2.3.1	Description

The Acknowledgement of Event Notification is used by the AF to acknowledge the NEF about handling result of the event notification (e.g. UP path change).

##### 5.4.2.3.2	Target URI

URI: {afAckUri}

The operation shall support the URI variables defined in table 5.4.2.3.2-1.

Table 5.4.2.3.2-1: URI variables

| Name | Definition |
| --- | --- |
| afAckUri | Callback reference provided by the NEF during event notification within the EventNotification data type as defined in Table 5.4.3.3.4-1. |

##### 5.4.2.3.3	Operation Definition

###### 5.4.2.3.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.4.2.3.3.1-1 and the response data structures and response codes specified in table 5.4.2.3.3.1-2.

Table 5.4.2.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AfAckInfo | M | 1 | Acknowledgement information of event notification. |

Table 5.4.2.3.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The acknowledgement of event notification is received successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

### 5.4.3	Data Model

#### 5.4.3.1	General

This subclause specifies the application data model supported by the TrafficInfluence API.

#### 5.4.3.2	Reused data types

The data types reused by the TrafficInfluence API from other specifications are listed in table 5.4.3.2-1.

Table 5.4.3.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Dnai | 3GPP TS 29.571 [8] | Identifies a DNAI. |
| DnaiChangeType | 3GPP TS 29.571 [8] | Describes the types of DNAI change. |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |
| EthFlowDescription | 3GPP TS 29.514 [7] | Contains the Ethernet data flow information. (NOTE) |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| FlowInfo | 3GPP TS 29.122 [4] | Contains the IP data flow information. |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| Ipv4Addr | 3GPP TS 29.122 [4] | Identifies an IPv4 address. |
| Ipv6Addr | 3GPP TS 29.122 [4] | Identifies an IPv6 address. |
| Ipv6Prefix | 3GPP TS 29.571 [8] | Identifies an IPv6 Prefix. |
| Link | 3GPP TS 29.122 [4] | Identifies a referenced resource. |
| MacAddr48 | 3GPP TS 29.571 [8] | Identifies a MAC address. |
| Port | 3GPP TS 29.122 [4] | Identifies a port number. |
| RouteToLocation | 3GPP TS 29.571 [8] | Describes the traffic routes to the locations of the application. |
| Snssai | 3GPP TS 29.571 [8] | Identifies the S-NSSAI. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.4.4-1. |
| TemporalValidity | 3GPP TS 29.514 [7] | Indicates the time interval(s) during which the AF request is to be applied |
| WebsockNotifConfig | 3GPP TS 29.122 [4] | Contains the configuration parameters to set up notification delivery over Websocket protocol. |
| NOTE: In order to support a set of MAC addresses with a specific range in the traffic filter, feature MacAddressRange as specified in clause 5.4.4 shall be supported. |  |  |

#### 5.4.3.3	Structured data types

##### 5.4.3.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.4.3.3.2	Type: TrafficInfluSub

This type represents a traffic influence subscription. The same structure is used in the subscription request and subscription response.

Table 5.4.3.3.2-1: Definition of type TrafficInfluSub

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- | --- |
| afServiceId | string | O | 0..1 | Identifies a service on behalf of which the AF is issuing the request. |  |
| afAppId | string | O | 0..1 | Identifies an application. (NOTE 3) |  |
| afTransId | string | O | 0..1 | Identifies an NEF Northbound interface transaction, generated by the AF. |  |
| appReloInd | boolean | O | 0..1 | Identifies whether an application can be relocated once a location of the application has been selected. Set to "true" if it can be relocated; otherwise set to "false". Default value is "false" if omitted. |  |
| dnn | Dnn | O | 0..1 | Identifies a DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |
| snssai | Snssai | O | 0..1 | Identifies an S-NSSAI. |  |
| externalGroupId | ExternalGroupId | O | 0..1 | Identifies a group of users. (NOTE 2) |  |
| anyUeInd | boolean | O | 0..1 | Identifies whether the AF request applies to any UE. This attribute shall set to "true" if applicable for any UE, otherwise, set to "false". (NOTE 2) |  |
| subscribedEvents | array(SubscribedEvent) | O | 1..N | Identifies the requirement to be notified of the event(s). |  |
| gpsi | Gpsi | O | 0..1 | Identifies a user. (NOTE 2) |  |
| ipv4Addr | Ipv4Addr | O | 0..1 | Identifies the IPv4 address. (NOTE 2) |  |
| ipv6Addr | Ipv6Addr | O | 0..1 | Identifies the IPv6 address. (NOTE 2) |  |
| macAddr | MacAddr48 | O | 0..1 | Identifies the MAC address. |  |
| dnaiChgType | DnaiChangeType | O | 0..1 | Identifies a type of notification regarding UP path management event. |  |
| notificationDestination | Link | C | 0..1 | Contains the Callback URL to receive the notification from the NEF. It shall be present if the "subscribedEvents" is present. |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by the AF to request the NEF to send a test notification as defined in subclause 5.2.5.3 of 3GPP TS 29.122 [4]. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol. | Notification_websocket |
| self | Link | C | 0..1 | Link to the created resource. This parameter shall be supplied by the NEF in HTTP responses that include an object of TrafficInfluSub type |  |
| trafficFilters | array(FlowInfo) | O | 1..N | Identifies IP packet filters. (NOTE 3) |  |
| ethTrafficFilters | array(EthFlowDescription) | O | 1..N | Identifies Ethernet packet filters. (NOTE 3) |  |
| trafficRoutes | array(RouteToLocation) | O | 1..N | Identifies the N6 traffic routing requirement. |  |
| tfcCorrInd | boolean | O | 0..1 | Indication of traffic correlation. May only be included when "externalGroupId" attribute was included within the TrafficInfluSub data type previously. It is used to indicate that for the group of UEs, the targeted PDU sessions should be correlated by a common DNAI. Set to "true" if it should be correlated; otherwise set to "false". Default value is "false" if omitted. |  |
| tempValidities | array(TemporalValidity) | O | 0..N | Indicates the time interval(s) during which the AF request is to be applied. |  |
| validGeoZoneIds | array(string) | O | 1..N | Identifies a geographic zone that the AF request applies only to the traffic of UE(s) located in this specific zone. |  |
| afAckInd | boolean | O | 0..1 | Identifies whether the AF acknowledgement of UP path event notification is expected. Set to "true" if the AF acknowledge is expected; otherwise set to "false". Default value is "false" if omitted. | URLLC |
| addrPreserInd | boolean | O | 0..1 | Indicates whether UE IP address should be preserved. This attribute shall set to "true" if preserved, otherwise, set to "false". Defalult value is "false" if omitted. | URLLC |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the list of Supported features used as described in subclause 5.4.4. This attribute shall be provided in the POST request and in the response of successful resource creation. |  |
| NOTE 1: Properties marked with a feature as defined in subclause 5.4.4 are applicable as described in subclause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. NOTE 2: One of individual UE identifier (i.e. "gpsi", "ipv4Addr" or "ipv6Addr"), External Group Identifier (i.e. "externalGroupId") or any UE indication "anyUeInd" shall be included. NOTE 3: One of "afAppId", "trafficFilters" or "ethTrafficFilters" shall be included. |  |  |  |  |  |

##### 5.4.3.3.3	Type: TrafficInfluSubPatch

This type represents a subscription of traffic influence parameters provided by the AF to the NEF. The structure is used for HTTP PATCH request.

Table 5.4.3.3.3-1: Definition of type TrafficInfluSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appReloInd | boolean | O | 0..1 | Identifies whether an application can be relocated once a location of the application has been selected. (NOTE) |  |
| trafficFilters | array(FlowInfo) | O | 1..N | Identifies IP packet filters. |  |
| ethTrafficFilters | array(EthFlowDescription) | O | 1..N | Identifies Ethernet packet filters. |  |
| trafficRoutes | array(RouteToLocation) | O | 1..N | Identifies the N6 traffic routing requirement. (NOTE) |  |
| tfcCorrInd | boolean | O | 0..1 | Indication of traffic correlation. May only be included when "externalGroupId" attribute was included within the TrafficInfluSub data type previously. It is used to indicate that for the group of UEs, the targeted PDU sessions should be correlated by a common DNAI. |  |
| tempValidities | array(TemporalValidity) | O | 1..N | Indicates the time interval(s) during which the AF request is to be applied. (NOTE) |  |
| validGeoZoneIds | array(string) | O | 1..N | Identifies a geographic zone that the AF request applies only to the traffic of UE(s) located in this specific zone. (NOTE) |  |
| afAckInd | boolean | O | 0..1 | Identifies whether the AF acknowledgement of UP path event notification is expected. | URLLC |
| addrPreserInd | boolean | O | 0..1 | Indicates whether UE IP address should be preserved. (NOTE) | URLLC |
| NOTE: The value of the property shall be set to NULL for removal. |  |  |  |  |  |

##### 5.4.3.3.4	Type: EventNotification

Table 5.4.3.3.4-1: Definition of type EventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- | --- |
| afTransId | string | O | 0..1 | Identifies an NEF Northbound interface transaction, generated by the AF. |  |
| dnaiChgType | DnaiChangeType | M | 1 | Identifies the type of notification regarding UP path management event. |  |
| sourceTrafficRoute | RouteToLocation | O | 0..1 | Identifies the N6 traffic routing information associated to the source DNAI. May be present if the "subscribedEvent" sets to "UP_PATH_CHANGE". (NOTE 3) |  |
| subscribedEvent | SubscribedEvent | M | 1 | Identifies a UP path management event the AF requested to be notified of. |  |
| targetTrafficRoute | RouteToLocation | O | 0..1 | Identifies the N6 traffic routing information associated to the target DNAI. May be present if the "subscribedEvent" sets to "UP_PATH_CHANGE". (NOTE 3) |  |
| sourceDnai | Dnai | O | 0..1 | Source DN Access Identifier. Shall be included for event "UP_PATH_CHANGE" if the DNAI changed (NOTE 2, NOTE 3). |  |
| targetDnai | Dnai | O | 0..1 | Target DN Access Identifier. Shall be included for event "UP_PATH_CHANGE" if the DNAI changed (NOTE 2, NOTE 3). |  |
| gpsi | Gpsi | O | 0..1 | Identifies a user. |  |
| srcUeIpv4Addr | Ipv4Addr | O | 0..1 | The IPv4 Address of the served UE for the source DNAI. |  |
| srcUeIpv6Prefix | Ipv6Prefix | O | 0..1 | The Ipv6 Address Prefix of the served UE for the source DNAI. |  |
| tgtUeIpv4Addr | Ipv4Addr | O | 0..1 | The IPv4 Address of the served UE for the target DNAI. |  |
| tgtUeIpv6Prefix | Ipv6Prefix | O | 0..1 | The Ipv6 Address Prefix of the served UE for the target DNAI. |  |
| ueMac | MacAddr48 | O | 0..1 | UE MAC address of the served UE. |  |
| afAckUri | Link | O | 0..1 | The URI provided by the SMF for the AF acknowledgement. May only be included for event "UP_PATH_CHANGE". | URLLC |
| NOTE 1: Properties marked with a feature as defined in subclause 5.4.4 are applicable as described in subclause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. NOTE 2: If the DNAI is not changed while the N6 traffic routing information is changed, the "sourceDnai" attribute and "targetDnai" attribute shall not be provided. NOTE 3: The change from the UP path status where no DNAI applies to a status where a DNAI applies indicates the activation of the related AF request and therefore only the target DNAI and N6 traffic routing information is provided in the event notification; the change from the UP path status where a DNAI applies to a status where no DNAI applies indicates the de-activation of the related AF request and therefore only the source DNAI and N6 traffic routing information is provided in the event notification. |  |  |  |  |  |

##### 5.4.3.3.5	Type: AfResultInfo

Table 5.4.3.3.5-1: Definition of type AfResultInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| afStatus | AfResultStatus | M | 1 | Identifies the result of the application relocation. |
| trafficRoute | RouteToLocation | O | 0..1 | Identifies the N6 traffic routing information associated to the target DNAI. May only be present if the "afStatus" sets to "SUCCESS". |

##### 5.4.3.3.6	Type AfAckInfo

Table 5.4.3.3.6-1: Definition of type AfAckInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afTransId | string | C | 0..1 | Identifies an NEF Northbound interface transaction, generated by the AF. It shall be provided if the AF has previously provided it. |  |
| ackResult | AfResultInfo | M | 1 | Identifies the result of application layer handling. |  |
| gpsi | Gpsi | O | 0..1 | Identifies a GPSI. |  |

#### 5.4.3.4	Simple data types and enumerations

##### 5.4.3.4.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

##### 5.4.3.4.2	Simple data types

The simple data types defined in table 5.4.3.4.2-1 shall be supported.

Table 5.4.3.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 5.4.3.4.3	Enumeration: SubscribedEvent

The enumeration SubscribedEvent represents the type of UP patch management events of which the AF requests to be notified. It shall comply with the provisions defined in table 5.4.3.4.3-1.

Table 5.4.3.4.3-1: Enumeration SubscribedEvent

| Enumeration value | Description |
| --- | --- |
| UP_PATH_CHANGE | The AF requests to be notified when the UP path changes for the PDU session. |

##### 5.4.3.4.4	Enumeration: AfResultStatus

The enumeration AfResultStatus represents the status of application handling result. It shall comply with the provisions defined in table 5.4.3.4.4-1.

Table 5.4.3.4.4-1: Enumeration AfResultStatus

| Enumeration value | Description |
| --- | --- |
| SUCCESS | The application layer is ready or the relocation is completed. |
| TEMP_CONGESTION | The application relocation fails due to temporary congestion. |
| RELOC_NO_ALLOWED | The application relocation fails because application relocation is not allowed. |
| OTHER | The application relocation fails due to other reason. |

### 5.4.4	Used Features

The table below defines the features applicable to the TrafficInfluence API. Those features are negotiated as described in subclause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.4.4-1: Features used by TrafficInfluence API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_websocket | The delivery of notifications over Websocket is supported as described in 3GPP TS 29.122 [4]. This feature requires that the Notification_test_event feature is also supported. |
| 2 | Notification_test_event | The testing of notification connection is supported as described in 3GPP TS 29.122 [4]. |
| 3 | URLLC | This feature indicates support of Ultra Reliable Low Latency Communication (URLLC) requirements (i.e. AF application relocation acknowledgement and UE address(es) preservation). |
| 4 | MacAddressRange | Indicates the support of a set of MAC addresses with a specific range in the traffic filter. |
| Feature: A short name that can be used to refer to the bit and to the feature, e.g. "Notification". Description: A clear textual description of the feature. |  |  |

## 5.5	NiddConfigurationTrigger API

### 5.5.1	Resources

There is no resource defined for this API.

### 5.5.2	Notifications

#### 5.5.2.1	Introduction

Upon receipt of a NIDD connection establishment request from the SMF and there is no NIDD configuration for the UE, the NEF may send an HTTP POST message in order to trigger the AF to start the NIDD configuration procedure as described in subclause 5.6.3.2.3.4 of 3GPP TS 29.122 [4].

#### 5.5.2.2	Event Notification

URI: {notificationUrI}

The operation shall support the URI variables defined in table 5.5.2.2-1.

Table 5.5.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| notificationDestination | A URI indicating the notification destination where N33 notification requests shall be delivered to. This URI shall be preconfigured in the NEF. |

#### 5.5.2.3	Operation Definition

##### 5.5.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.5.2.3.1-1 and the response data structures and response codes specified in table 5.5.2.3.1-2.

Table 5.5.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NiddConfigTrigger | M | 1 | The NIDD Configuration Trigger is provided by the NEF to the AF. |

Table 5.5.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NiddConfigTriggerReply | M | 1 | 200 OK | The trigger is received successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

##### 5.5.2.3.2	Notification via Websocket

Not specified in the present specification.

### 5.5.3	Data Model

#### 5.5.3.1	General

This subclause specifies the application data model supported by the NiddConfigurationTrigger API.

#### 5.5.3.2	Reused data types

The data types reused by the NiddConfigurationTrigger API from other specifications are listed in table 5.5.3.2-1.

Table 5.5.3.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.5.4-1. |

#### 5.5.3.3	Structured data types

##### 5.5.3.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.5.3.3.2	Type: NiddConfigTrigger

This type represents a NIDD configuration trigger which is sent from the NEF to the AF.

Table 5.5.3.3.2-1: Definition of type NiddConfigTrigger

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Identifies the trigger receiving entity. |  |
| nefId | string | M | 1 | Identifies the trigger sending entity. |  |
| gpsi | Gpsi | M | 1 | Identifies a user. |  |
| suppFeat | SupportedFeatures | M | 1 | Indicates the list of Supported features used as described in subclause 5.5.4. |  |
| NOTE: Properties marked with a feature as defined in subclause 5.5.4 are applicable as described in subclause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. |  |  |  |  |  |

##### 5.5.3.3.3	Type: NiddConfigTriggerReply

This data type represents a reply to a NIDD configuration trigger and is sent from the AF to the NEF.

Table 5.15.2.1.3-1: Definition of type NiddConfigTriggerReply

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- | --- |
| suppFeat | SupportedFeatures | M | 1 | Indicates the list of Supported features used as described in subclause 5.5.4. |  |
| NOTE: Properties marked with a feature as defined in subclause 5.5.4 are applicable as described in subclause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. |  |  |  |  |  |

#### 5.5.3.4	Simple data types and enumerations

##### 5.5.3.4.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

##### 5.5.3.4.2	Simple data types

The simple data types defined in table 5.5.3.4.2-1 shall be supported.

Table 5.5.3.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.5.4	Used Features

The table below defines the features applicable to the NiddConfigurationTrigger API. Those features are negotiated as described in subclause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.5.4-1: Features used by NiddConfigurationTrigger API

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 5.6	AnalyticsExposure API

### 5.6.1	Resources

#### 5.6.1.1	Overview

This subclause describes the structure for the Resource URIs as shown in figure 5.6.1.1-1 and the resources and HTTP methods used for the AnalyticsExposure API.

Figure 5.6.1.1-1: Resource URI structure of the AnalyticsExposure API

Table 5.6.1.1-1 provides an overview of the resources and HTTP methods applicable for the AnalyticsExposure API.

Table 5.6.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Analytics Exposure Subscriptions | {apiRoot}/3gpp-analyticsexposure/v1/{afId}/subscriptions | GET | Read all subscriptions for a given AF |
|  |  | POST | Create a new subscription to analytics exposure |
| Individual Analytics Exposure Subscription | {apiRoot}/3gpp-analyticsexposure/v1/{afId}/subscriptions /{subscriptionId} | GET | Read the subscription to the analytics exposure |
|  |  | PUT | Modify all of the properties of an existing subscription to an analytics exposure |
|  |  | DELETE | Delete the subscription to the analytics exposure |

#### 5.6.1.2	Resource: Analytics Exposure Subscriptions

##### 5.6.1.2.1	Introduction

This resource allows a AF to read all active analytics exposure subscribtions for the given AF, or allows a AF to create a new subscription to retrieving analytics information.

##### 5.6.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-analyticsexposure/v1/{afId}/subscriptions

This resource shall support the resource URI variables defined in table 5.6.1.2.2-1.

Table 5.6.1.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |

##### 5.6.1.2.3	Resource Methods

###### 5.6.1.2.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.6.1.2.2.

###### 5.6.1.2.3.2	GET

The GET method allows to read all active subscriptions for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.6.1.2.3.2-1.

Table 5.6.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.6.1.2.3.2-2 and the response data structures and response codes specified in table 5.6.1.2.3.2-3.

Table 5.6.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.6.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(AnalyticsExposureSubsc) | M | 0..N | 200 OK | The subscription information for the AF in the request URI are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.6.1.2.3.3	POST

The POST method creates a new subscription resource to analytics exposure subscription for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.6.1.2.3.3-1 and the response data structures and response codes specified in table 5.6.1.2.3.3-2.

Table 5.6.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AnalyticsExposureSubsc | M | 1 | Parameters to request a subscription to retrieving analytics information with the NEF. |

Table 5.6.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AnalyticsExposureSubsc | M | 1 | 201 Created | The subscription was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

#### 5.6.1.3	Resource: Individual Analytics Exposure Subscription

##### 5.6.1.3.1	Introduction

This resource allows a AF to read/modify/cancel a subscription to retrieve analytics information with the NEF.

##### 5.6.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-analyticsexposure/v1/{afId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.6.1.3.2-1.

Table 5.6.1.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |
| subscriptionId | Identifier of the subscription resource of type string. |

##### 5.6.1.3.3	Resource Methods

###### 5.6.1.3.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.6.1.3.2.

###### 5.6.1.3.3.2	GET

The GET method allows to read the active subscription for a given AF and subscription Id. The AF shall initiate the HTTP GET request message and theNEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.6.1.3.3.2-1.

Table 5.6.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.6.1.3.3.2-2 and the response data structures and response codes specified in table 5.6.1.3.3.2-3.

Table 5.6.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.6.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AnalyticsExposureSubsc | M | 1 | 200 OK | The subscription information for the AF in the request URI are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.6.1.3.3.3	PUT

The PUT method modifies an existing subscription resource to update a subscription. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.6.1.3.3.3-1 and the response data structures and response codes specified in table 5.6.1.3.3.3-2.

Table 5.6.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AnalyticsExposureSubsc | M | 1 | Modify an existing subscription to retrieving analytics information with the NEF. |

Table 5.6.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AnalyticsExposureSubsc | M | 1 | 200 OK | The subscription was updated successfully. |
| N/A |  |  | 204 No Content | The subscription was updated successfully. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.6.1.3.3.4	DELETE

The DELETE method deletes the analytics exposure subscription for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.6.1.3.3.4-1.

Table 5.6.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.6.1.3.3.4-2 and the response data structures and response codes specified in table 5.6.1.3.3.4-3.

Table 5.6.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.6.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The subscription was terminated successfully. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

### 5.6.1a	Custom Operations without associated resources

#### 5.6.1a.1	Overview

Custom operations used for this API are summarized in table 5.6.1a.1-1. "apiRoot" is set as described in subclause 5.2.4 of 3GPP TS 29.122 [4].

Table 5.6.1a.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/3gpp-analyticsexposure/v1/{afId}/fetch | POST | Request to fetch analytics information |

#### 5.6.1a.2	Operation: fetch

##### 5.6.1a.2.1	Description

The custom operation allows a service consumer to fetch analytics information via the NEF.

##### 5.6.1a.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in tables 5.6.1a.2.2-1 and 5.6.1a.2.2-2.

Table 5.6.1a.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AnalyticsRequest | M | 1 | Parameters to request to fetch analytics information. |

Table 5.6.1a.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AnalyticsData | M | 1 | 200 OK | The requested analytics information was returned successfully. |
| n/a |  |  | 204 No Content | If the request Analytics data does not exist, the NEF shall respond with "204 No Content". |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 5.6.2	Notifications

#### 5.6.2.1	Introduction

Upon receipt of analytics information notification from the NWDAF indicating the subscribed analytics event is detected, the NEF shall send an HTTP POST message including the notified analytics event to the AF. The NEF and the AF shall support the notification mechanism as described in subclause 5.2.5 of 3GPP TS 29.122 [4].

#### 5.6.2.2	Event Notification

URI: {notifUri}

The operation shall support the URI variables defined in table 5.6.2.2-1.

Table 5.6.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| notifUri | Callback reference provided by the AF during creation of the subscription within the AnalyticsExposureSubsc data type as defined in Table 5.6.3.3.2-1. |

#### 5.6.2.3	Operation Definition

##### 5.6.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.6.2.3.1-1 and the response data structures and response codes specified in table 5.6.2.3.1-2.

Table 5.6.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AnalyticsEventNotification | M | 1 | The analytics event notification is provided by the NEF to the AF. |

Table 5.6.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The event notification is received successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

##### 5.6.2.3.2	Notification via Websocket

If supported by both AF and NEF and successfully negotiated, the AnalyticsEventNotification may alternatively be delivered through the Websocket mechanism as defined in subclause 5.2.5.4 of 3GPP TS 29.122 [4].

### 5.6.3	Data Model

#### 5.6.3.1	General

This subclause specifies the application data model supported by the AnalytcisExposure API.

#### 5.6.3.2	Reused data types

The data types reused by the AnalytcisExposure API from other specifications are listed in table 5.6.3.2-1.

Table 5.6.3.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| AdditionalMeasurement | 3GPP TS 29.520 [27] |  |
| AnalyticsReportingRequirement | 3GPP TS 29.520 [27] | Describes the analytics reporting requirement information. |
| BitRate | 3GPP TS 29.571 [8] |  |
| CongestionType | 3GPP TS 29.520 [27] |  |
| DateTime | 3GPP TS 29.122 [4] |  |
| Dnn | 3GPP TS 29.571 [8] |  |
| DurationSec | 3GPP TS 29.122 [4] | Seconds of duration. |
| EventReportingRequirement | 3GPP TS 29.520 [27] |  |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| ExceptionId | 3GPP TS 29.520 [27] |  |
| ExpectedAnalyticsType | 3GPP TS 29.520 [27] |  |
| ExpectedUeBehaviourData | 3GPP TS 29.503 [TS29503] |  |
| Float | 3GPP TS 29.571 [8] |  |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| NotificationMethod | 3GPP TS 29.508 [26] | Represents the Notification Method. |
| NetworkPerfRequirement | 3GPP TS 29.520 [27] |  |
| QosRequirement | 3GPP TS 29.520 [27] |  |
| RetainabilityThreshold | 3GPP TS 29.520 [27] |  |
| SamplingRatio | 3GPP TS 29.571 [8] | Indicates Sampling Ratio. |
| ScheduledCommunicationTime | 3GPP TS 29.122 [4] |  |
| Snssai | 3GPP TS 29.571 [8] |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.6.4-1. |
| TargetUeInformation | 3GPP TS 29.520 [27] |  |
| ThresholdLevel | 3GPP TS 29.520 [27] |  |
| TimeWindow | 3GPP TS 29.122 [4] |  |
| UeCommunication | 3GPP TS 29.520 [27] |  |
| Uinteger | 3GPP TS 29.571 [8] | Unsigned integer. |
| Uri | 3GPP TS 29.571 [8] | Identifies a referenced resource. |
| WebsockNotifConfig | 3GPP TS 29.122 [4] | Contains the configuration parameters to set up notification delivery over Websocket protocol. |
| LocationArea5G | 3GPP TS 29.122 [4] |  |

#### 5.6.3.3	Structured data types

##### 5.6.3.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.6.3.3.2	Type: AnalyticsExposureSubsc

This type represents an analytics exposure subscription. The same structure is used in the subscription request and subscription response.

Table 5.6.3.3.2-1: Definition of type AnalyticsExposureSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- | --- |
| analyEventsSubs | array(AnalyticsEventSubsc) | M | 1..N | Subscribed analytics events. |  |
| analyRepInfo | EventReportingRequirement | O | 0..1 | Reporting requirement information of the subscription. |  |
| notifUri | Uri | M | 1 | Notification URI for analytics event reporting. |  |
| notifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the list of Supported features used as described in subclause 5.6.4. This attribute shall be provided in the POST request and in the response of successful resource creation. |  |
| NOTE: Properties marked with a feature as defined in subclause 5.6.4 are applicable as described in subclause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. |  |  |  |  |  |

##### 5.6.3.3.3	Type: AnalyticsEventNotification

Table 5.6.3.3.3-1: Definition of type AnalyticsEventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| analyEventNotifs | array(AnalyticsEventNotif) | M | 1..N | Represents the analytics events to be reported according to the subscription corresponding to the Notification Correlation ID. |  |

##### 5.6.3.3.4	Type: AnalyticsEventNotif

Table 5.6.3.3.4-1: Definition of type AnalyticsEventNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyEvent | AnalyticsEvent | M | 1 | Detected analytics event. |  |
| timeStamp | DateTime | M | 1 | Time at which the event is observed. |  |
| ueMobilityInfos | array(UeMobilityExposure) | C | 1..N | Contains the UE mobility information. Shall be present if the "analyEvent" attribute sets to "UE_MOBILITY" | Ue_Mobility |
| ueCommInfos | array(UeCommunication) | C | 1..N | Contains the application communication information. Shall be present if the "analyEvent" attribute sets to "UE_COMM" | Ue_Communication |
| abnormalInfos | array(AbnormalExposure) | C | 1..N | Contains the user’s abnormal behavior information. Shall be present if the "analyEvent" attribute sets to "ABNORMAL_BEHAVIOR" | Abnormal_Behavior |
| congestInfos | array(CongestInfo) | C | 1..N | Contains the UE’s user data congestion information. Shall be present if the "analyEvent" attribute sets to "CONGESTION" | Congestion |
| nwPerfInfos | array(NetworkPerfExposure) | C | 1..N | The network performance information. Shall be present when the requested event is "NETWORK_PERFORMANCE". | Network_Performance |
| qosSustainInfos | array(QosSustainabilityExposure) | C | 1..N | Contains the QoS sustainability information. Shall be present if the "analyEvent" attribute is set to "QOS_SUSTAINABILITY" | QoS_Sustainability |

##### 5.6.3.3.5	Type: AnalyticsEventSubsc

Table 5.6.3.3.5-1: Definition of type AnalyticsEventSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyEvent | AnalyticsEvent | M | 1 | Requested analytics event. |  |
| analyEventFilter | AnalyticsEventFilterSubsc | O | 0..1 | Represents analytics event filter. | (NOTE) |
| tgtUe | TargetUeId | M | 1 | Identifies target UE information | (NOTE) |
| NOTE: Applicability is further described in the corresponding data type. |  |  |  |  |  |

##### 5.6.3.3.6	Type: AnalyticsEventFilterSubsc

Table 5.6.3.3.6-1: Definition of type AnalyticsEventFilterSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| locArea | LocationArea5G | O | 0..1 | Network area of interest (NOTE 1) | Abnormal_Behavior Congestion Ue_Communication Ue_Mobility QoS_Sustainability |
| appIds | array(ApplicationId) | O | 1..N | Each element identifies an application. | Abnormal_Behavior Ue_Communication |
| excepRequs | array(Exception) | O | 1..N | Represents a list of Exception Ids with associated thresholds. (NOTE 2, NOTE 3) | Abnormal_Behavior |
| exptAnaType | ExpectedAnalyticsType | O | 0..1 | Represents expected UE analytics type. (NOTE 3) | Abnormal_Behavior |
| exptUeBehav | ExpectedUeBehaviourData | O | 0..1 | Represents expected UE behaviour. | Abnormal_Behavior |
| reptThlds | array(ThresholdLevel) | O | 1..N | Represents the congestion levels to be reached in order to be notified by the NEF. (NOTE 4) | Congestion |
| nwPerfReqs | array(NetworkPerfRequirement) | C | 1..N | Represents the network performance requirements. This attribute shall be included when eventId is "NETWORK_PERFORMANCE". | Network_Performance |
| maxAnaEntry | Uinteger | O | 0..1 | Maximum number of analytics entries expected for an analytics report. | Ue_Mobility Ue_Communication Abnormal_Behavior QoS_Sustainability Congestion |
| snssai | Snssai | O | 0..1 | Identifies the network slice information | Ue_Communication QoS_Sustainability |
| qosReq | QosRequirement | C | 0..1 | Represents the QoS requirements. This attribute shall be included when eventId is "QOS_SUSTAINABILITY". | QoS_Sustainability |
| qosFlowRetThds | array(RetainabilityThreshold) | C | 1..N | Represents the QoS flow retainability thresholds, Shall be supplied for the 5QI of GBR resource type. (NOTE 5) | QoS_Sustainability |
| ranUeThrouThds | array(BitRate) | C | 1..N | Represents the RAN UE throughput thresholds. Shall be supplied for the 5QI of non-GBR resource type. (NOTE 5) | QoS_Sustainability |
| NOTE 1: The "locArea" attribute shall be provided if the "tgtUe" attribute sets to "anyUeInd". NOTE 2: Only "excepId" and "excepLevel" within the Exception data type apply to the "excepRequs" attribute. NOTE 3: Either "excepRequs" or "exptAnaType" shall be provided if the subscribed event is "ABNORMAL_BEHAVIOUR". NOTE 4: If the subscribed event is "CONGESTION", this attribute shall be provided if "notifMethod" within "analyRepInfo" sets to "ON_EVENT_DETECTION" or omitted. NOTE 5: For "QOS_SUSTAINABILITY", this property shall be provided if the "notifMethod" in "analyRepInfo" is set to "ON_EVENT_DETECTION" or omitted. |  |  |  |  |  |

5.6.3.3.7	Type TargetUeId

Table 5.6.3.3.7-1: Definition of type TargetUeId

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anyUeInd | boolean | O | 0..1 | Identifies whether the AF request applies to any UE. This attribute shall set to "true" if applicable for any UE, otherwise, set to "false". | Network_Performance QoS_Sustainability |
| gpsi | Gpsi | O | 0..1 | Identifies a GPSI for an UE. | Abnormal_Behavior Congestion Ue_Mobility Ue_Communication Network_Performance |
| exterGroupId | ExternalGroupId | O | 0..1 | Represents an external group identifier and identifies a group of UEs. | Abnormal_BehaviorUe_Mobility Ue_Communication Network_Performance |
| NOTE: For an applicable feature, only one attribute identifying the target UE shall be provided. |  |  |  |  |  |

##### 5.6.3.3.8	Void

Table 5.6.3.3.8-1: Definition of type UeMobilityExposure

##### 5.6.3.3.9	Type UeMobilityExposure

Table 5.6.3.3.9-1: Definition of type UeMobilityExposure

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ts | DateTime | O | 0..1 | This attribute identifies the timestamp when the UE arrives the location. (NOTE 1) |  |
| recurringTime | ScheduledCommunicationTime | O | 0..1 | Identifies time of the day and day of the week which are valid within the observation period when the UE moves. (NOTE 1, NOTE 2) |  |
| duration | DurationSec | M | 1 | This attribute identifies the time duration the UE stays in the location. If the analytics result applies for a group of UEs, it indicates the average duration for the group of UEs. |  |
| durationVariance | Float | C | 0..1 | This attribute indicates the variance of the analysed durations for the group of UEs. It shall be provided if the analytics result applies for a group of UEs. |  |
| locInfo | array(UeLocationInfo) | M | 1..N | This attribute includes a list of UE location information during the time duration. |  |
| NOTE 1: Either ts or recurringTime shall be provided. NOTE 2: If this attribute is present, it indicates the UE movement is periodic. This attribute is suitable to be present for a recurring mobility in a long observation time. |  |  |  |  |  |

##### 5.6.3.3.10	Type UeLocationInfo

Table 5.6.3.3.10-1: Definition of type UeLocationInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| loc | LocationArea5G | M | 1 | This attribute contains the detailed location. |  |
| ratio | SamplingRatio | C | 0..1 | If the analytics result applies for a group of UEs, this attribute contains the percentage of UEs in the group. The sum of all ratios on a given time slot is equal or less than 100%. |  |
| confidence | Uinteger | C | 0..1 | If the analytics result is a prediction, it indicates the confidence of the prediction. |  |

##### 5.6.3.3.11	Void

Table 5.6.3.3.11-1: Definition of type UeCommunicationExposure

##### 5.6.3.3.12	Type: AnalyticsRequest

Table 5.6.3.3.12-1: Definition of type AnalyticsRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyEvent | AnalyticsEvent | M | 1 | Identifies the analytics type. |  |
| analyEventFilter | AnalyticsEventFilter | C | 0..1 | Shall be included to identify the analytics when filter information is needed for the related event. |  |
| analyRep | AnalyticsReportingRequirement | O | 0..1 | Identifies the analytics reporting requirement information. |  |
| tgtUe | TargetUeInformation | O | 0..1 | Identifies the target UE information. |  |
| suppFeat | SupportedFeatures | M | 1 | Represents the features supported by the NF service consumer. |  |

##### 5.6.3.3.13	Type AnalyticsEventFilter

Table 5.6.3.3.13-1: Definition of type AnalyticsEventFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| locArea | LocationArea5G | C | 0..1 | This IE represents the network area where the NF service consumer wants to know the analytics result. Shall be present if the "analyEvent" attribute sets to "QOS_SUSTAINABILITY". | Ue_Mobility Ue_Communication Network_Performance QoS_Sustainability |
| maxAnaEntry | Uinteger | O | 0..1 | Maximum number of analytics entries expected for an analytics report. | Ue_Mobility Ue_Communication Network_Performance QoS_Sustainability Abnormal_Behavior Congestion |
| dnn | Dnn | O | 0..1 | Identifies the DNN. | Ue_Communication |
| nwPerfTypes | array(NetworkPerfType) | C | 1..N | Represents the network performance requirements. This attribute shall be included when eventId is "NETWORK_PERFORMANCE". | Network_Performance |
| appIds | array(ApplicationId) | O | 1..N | Each element identifies an application. The absence of appIds means all applications. | Ue_Communication Abnormal_Behavior |
| excepIds | array(ExceptionId) | O | 1..N | Represents a list of Exception Ids. (NOTE) | Abnormal_Behavior |
| exptAnaType | ExpectedAnalyticsType | O | 0..1 | Represents expected UE analytics type. (NOTE) | Abnormal_Behavior |
| exptUeBehav | ExpectedUeBehaviourData | O | 0..1 | Represents expected UE behaviour. | Abnormal_Behavior |
| snssai | Snssai | O | 0..1 | Identifies the network slice information | Ue_Communication QoS_Sustainability |
| qosReq | QosRequirement | C | 0..1 | Represents the QoS requirements. This attribute shall be included when analyEvent is "QOS_SUSTAINABILITY". | QoS_Sustainability |
| NOTE: Either "excepIds" or "exptAnaType" shall be provided if the subscribed event is "ABNORMAL_BEHAVIOUR". |  |  |  |  |  |

##### 5.6.3.3.14	Type AnalyticsData

Table 5.6.3.3.14-1: Definition of type AnalyticsData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueMobilityInfos | array(UeMobilityExposure) | C | 1..N | Contains the UE mobility information. Shall be present if the "analyEvent" attribute sets to "UE_MOBILITY" | Ue_Mobility Abnormal_Behavior |
| ueCommInfos | array(UeCommunication) | C | 1..N | Contains the application communication information. Shall be present if the "analyEvent" attribute sets to "UE_COMM" | Ue_Communication |
| nwPerfInfos | array(NetworkPerfExposure) | C | 1..N | The network performance information. Shall be present when the requested event is "NETWORK_PERFORMANCE". | Network_Performance |
| abnormalInfos | array(AbnormalExposure) | C | 1..N | Contains the user’s abnormal behavior information. Shall be present if the "analyEvent" attribute sets to "ABNORMAL_BEHAVIOR" | Abnormal_Behavior |
| congestInfos | array(CongestInfo) | C | 1..N | Contains the UE’s user data congestion information. Shall be present if the "analyEvent" attribute sets to "CONGESTION" | Congestion |
| qosSustainInfos | array(QosSustainabilityExposure) | C | 1..N | Contains the QoS sustainability information. Shall be present if the "analyEvent" attribute is set to "QOS_SUSTAINABILITY" | QoS_Sustainability |
| suppFeat | SupportedFeatures | M | 1 | Represents the features supported by the NF service consumer. |  |

##### 5.6.3.3.15	Type AbnormalExposure

Table 5.6.3.3.15-1: Definition of type AbnormalExposure

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsis | array(Gpsi) | C | 1..N | Each element identifies a UE which is affected with the Exception. Shall be present if the subscription request applies to more than one UE. |  |
| excep | Exception | M | 1 | Contains the exception information. |  |
| ratio | SamplingRatio | C | 0..1 | If the analytics result applies for a group of UEs or any UE, this attribute contains the percentage of UEs. The sum of all ratios on a given abnorBehavrs (which is of AbnormalBehaviour data type) is equal or less than 100%. |  |
| confidence | Uinteger | C | 0..1 | If the analytics result is a prediction, it indicates the confidence of the prediction. |  |
| addtMeasInfo | AdditionalMeasurement | O | 0..1 | Additional measurement. |  |

##### 5.6.3.3.16	Type CongestInfo

Table 5.6.3.3.16-1: Definition of type CongestInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| locArea | LocationArea5G | M | 1 | Network area of interest |  |
| cngAnas | array(CongestionAnalytics) | M | 1..N | Represents data congestion analytics for transfer over the user plane, control plane or both planes. |  |

##### 5.6.3.3.17	Type CongestionAnalytics

Table 5.6.3.3.17-1: Definition of type CongestionAnalytics

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| cngType | CongestionType | M | 1 | Represents congestion type. |  |
| tmWdw | TimeWindow | M | 1 | Represents a start time and a stop time observed for the congestion information. |  |
| nsi | ThresholdLevel | M | 1 | Represents network congestion level. |  |
| confidence | Uinteger | C | 0..1 | If the analytics result is a prediction, it indicates the confidence of the prediction. |  |

##### 5.6.3.3.18	Type QosSustainabilityExposure

Table 5.6.3.3.18-1: Definition of type QosSustainabilityExposure

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| locArea | LocationArea5G | M | 1 | Identification(s) of applicable location areas where the analytics result applies. |  |
| startTs | DateTime | M | 1 | Represents the start time of the applicable observing period. |  |
| endTs | DataTime | M | 1 | Represents the end time of the applicable observing period. |  |
| qosFlowRetThd | RetainabilityThreshold | O | 0..1 | The reporting QoS Flow Retainability Threshold that are met or crossed for 5QI of GBR resource type. (NOTE) |  |
| ranUeThrouThd | BitRate | O | 0..1 | The reporting RAN UE Throughput Threshold that are met or crossed for 5QI of non-GBR resource type. (NOTE) |  |
| confidence | Uinteger | C | 0..1 | If the analytics result is a prediction, it indicates the confidence of the prediction. |  |
| NOTE: Either qosFlowRetThd or ranUeThrouThd shall be provided. |  |  |  |  |  |

#### 5.6.3.4	Simple data types and enumerations

##### 5.6.3.4.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

##### 5.6.3.4.2	Simple data types

The simple data types defined in table 5.6.3.4.2-1 shall be supported.

Table 5.6.3.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 5.6.3.4.3	Enumeration: AnalyticsEvent

The enumeration represents the type of analytics events of which the AF requests to be notified. It shall comply with the provisions defined in table 5.6.3.4.3-1.

Table 5.6.3.4.3-1: Enumeration AnalyticsEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UE_MOBILITY | The AF requests to be notified about analytics information of UE mobility. | Ue_Mobility |
| UE_COMM | The AF requests to be notified about analytics information of UE communication. | Ue_Communication |
| ABNORMAL_BEHAVIOR | The AF requests to be notified about analytics information of UE’s abnormal behavior. |  |
| CONGESTION | The AF requests to be notified about analytics information of user data congestion information. |  |
| NETWORK_PERFORMANCE | The AF requests to be notified about analytics information of network performance information. |  |
| QOS_SUSTAINABILITY | The AF requests to be notified about analytics information of QoS sustainability. | QoS_Sustainability |

### 5.6.4	Used Features

The table below defines the features applicable to the AnalyticsExposure API. Those features are negotiated as described in subclause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.6.4-1: Features used by AnalyticsExposure API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Ue_Mobility | This feature indicates support for the analytics event related to UE mobility. |
| 2 | Ue_Communication | This feature indicates support for the analytics event related to UE communication information. |
| 3 | Abnormal_Behavior | This feature indicates support for the analytics event related to UE’s abnormal behaviour. |
| 4 | Congestion | This feature indicates support for the analytics event related to UE’s user data congestion information. |
| 5 | Network_Performance | This feature indicates support for the analytics event related to network performance. |
| 6 | QoS_Sustainability | This feature indicates support for the analytics event related to QoS sustainability. |

## 5.7	5GLANParameterProvision API

### 5.7.1	Resources

#### 5.7.1.1	Overview

This subclause describes the structure for the Resource URIs as shown in figure 5.7.1.1-1 and the resources and HTTP methods used for the 5GLANParameterProvision API.

Figure 5.7.1.1-1: Resource URI structure of the 5GLANParameterProvision API

Table 5.7.1.1-1 provides an overview of the resources and HTTP methods applicable for the 5GLANParameterProvision API.

Table 5.7.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| 5GLAN Parameters Provision Subscriptions | {apiRoot}/3gpp-5glan-pp/v1/{afId}/subscriptions | GET | Read all subscriptions for a given AF |
|  |  | POST | Create a new subscription to provision parameters |
| Individual 5GLAN Parameters Provision Subscription | {apiRoot}/3gpp-5glan-pp/v1/{afId}/subscriptions/{subscriptionId} | GET | Read an existing subscription identified by {subscriptionId} |
|  |  | PUT | Modify all of the properties of an existing subscription identified by {subscriptionId} |
|  |  | PATCH | Modify some properties of an existing subscription identified by {subscriptionId} |
|  |  | DELETE | Delete the subscription identified by {subscriptionId} |

#### 5.7.1.2	Resource: 5GLAN Parameters Provision Subscriptions

##### 5.7.1.2.1	Introduction

This resource allows a AF to read all active 5GLAN parameters provision subscribtions for the given AF, or create an new individual 5GLAN parameters provision subscription to provision parameters to the NEF.

##### 5.7.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-5glan-pp/v1/{afId}/subscriptions

This resource shall support the resource URI variables defined in table 5.7.1.2.2-1.

Table 5.7.1.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |

##### 5.7.1.2.3	Resource Methods

###### 5.7.1.2.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.7.1.2.2.

###### 5.7.1.2.3.2	GET

The GET method allows to read all active subscriptions for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.7.1.2.3.2-1.

Table 5.7.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.7.1.2.3.2-2 and the response data structures and response codes specified in table 5.7.1.2.3.2-3.

Table 5.7.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.7.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(5GLanParametersProvision) |  | 0..N | 200 OK | All the subscription information for the AF in the request URI are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.7.1.2.3.3	POST

The POST method creates a new resource to individual 5GLAN parameters provision subscription for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.7.1.2.3.3-1 and the response data structures and response codes specified in table 5.7.1.2.3.3-2.

Table 5.7.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| 5GLanParametersProvision | M | 1 | Parameters to create a subscription to provision parameters. |

Table 5.7.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| 5GLanParametersProvision | M | 1 | 201 Created | The subscription was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| n/a |  |  | 204 No Content | The subscription was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

#### 5.7.1.3	Resource: Individual 5GLAN Parameters Provision Subscription

##### 5.7.1.3.1	Introduction

This resource allows a AF to read, update or delete an existing subscription to provision 5GLAN parameters.

##### 5.7.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-5glan-pp/v1/{afId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.7.1.3.2-1.

Table 5.7.1.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |
| subscriptionId | Identifier of the subscription resource of type string. |

##### 5.7.1.3.3	Resource Methods

###### 5.7.1.3.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.7.1.3.2.

###### 5.7.1.3.3.2	GET

The GET method allows to read the active subscription for a given AF and subscription Id. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.7.1.3.3.2-1.

Table 5.7.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.7.1.3.3.2-2 and the response data structures and response codes specified in table 5.7.1.3.3.2-3.

Table 5.7.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.7.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| 5GLanParametersProvision | M | 1 | 200 OK | The information for the subscription in the request URI are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.7.1.3.3.3	PUT

The PUT method modifies an existing resource to update a subscription. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.7.1.3.3.3-1 and the response data structures and response codes specified in table 5.7.1.3.3.3-2.

Table 5.7.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| 5GLanParametersProvision | M | 1 | Modify an existing subscription to provision parameters. |

Table 5.7.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| 5GLanParametersProvision | M | 1 | 200 OK | The subscription was updated successfully. |
| n/a |  |  | 204 No Content | The subscription was updated successfully. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.7.1.3.3.4	DELETE

The DELETE method deletes an existing individual 5GLAN parameters provision subscription for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.7.1.3.3.4-1.

Table 5.7.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.7.1.3.3.4-2 and the response data structures and response codes specified in table 5.7.1.3.3.4-3.

Table 5.7.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.7.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The subscription was terminated successfully. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.7.1.3.3.5	PATCH

The PATCH method allows to change some properties of an existing resource to update a subscription. The AF shall initiate the HTTP PATCH request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.7.1.3.3.5-1 and the response data structures and response codes specified in table 5.7.1.3.3.5-2.

Table 5.7.1.3.3.5-1: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| 5GLanParametersProvisionPatch | M | 1 | Modify an existing subscription to provision parameters. |

Table 5.7.1.3.3.5-2: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| 5GLanParametersProvision | M | 1 | 200 OK | The subscription was updated successfully. |
| n/a |  |  | 204 No Content | The subscription was updated successfully. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

### 5.7.1A	Notifications

Notifications are not applicable to this API.

### 5.7.2	Data Model

#### 5.7.2.1	General

This subclause specifies the application data model supported by the 5GLANParameterProvision API.

#### 5.7.2.2	Reused data types

The data types reused by the 5GLANParameterProvision API from other specifications are listed in table 5.7.2.2-1.

Table 5.7.2.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| ApplicationId | 3GPP TS 29.571 [8] |  |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| Link | 3GPP TS 29.122 [4] | Identifies a referenced resource. |
| OsId | 3GPP TS 29.519 [23] | Operating System. |
| PduSessionType | 3GPP TS 29.571 [8] | PDU session type. |
| Snssai | 3GPP TS 29.571 [8] | Identifies the S-NSSAI. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.7.3-1. |

#### 5.7.2.3	Structured data types

##### 5.7.2.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.7.2.3.2	Type: 5GLanParametersProvision

Table 5.7.2.3.2-1: Definition of type 5GLanParametersProvision

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| self | Link | C | 0..1 | Identifies the individual parameters provision subscription resource. Shall be present in the HTTP GET response when reading all the subscriptions for an AF. |  |
| 5gLanParams | 5GLanParameters | M | 1 | Represents the 5G LAN service related parameters. |  |
| suppFeat | SupportedFeatures | M | 1 | Indicates the negotiated supported features. |  |

##### 5.7.2.3.3	Type: 5GLanParameters

This type represents the 5G LAN service related parameters need to be provisioned.

Table 5.7.2.3.3-1: Definition of type 5GLanParameters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| exterGroupId | ExternalGroupId | M | 1 | Identifies an 5G Virtual Network Group |  |
| gpsis | map(Gpsi) | M | 1..N | Represents the list of 5G VN Group members, each member is identified by GPSI |  |
| dnn | Dnn | M | 1 | DNN for the 5G VN group, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |
| snssai | Snssai | M | 1 | S-NSSAI for the 5G VN group |  |
| sessionType | PduSessionType | M | 1 | PDU Session Type allowed for 5G VN group. |  |
| appDesps | map(AppDescriptor) | M | 1..N | Describes the operation systems and the corresponding applications for each operation systems. The key of map is osId. |  |

##### 5.7.2.3.4	Type: AppDescriptor

Table 5.7.2.3.4-1: Definition of type AppDescriptor

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| osId | OsId | M | 1 | Identifies an operating system supported by the UE |  |
| appId | map(ApplicationId) | M | 1..N | Identifies applications that is running on the UE’s operating system. |  |

##### 5.7.2.3.5	Type: 5GLanParametersProvisionPatch

Table 5.7.2.3.5-1: Definition of type 5GLanParametersProvisionPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| 5gLanParamsPatch | 5GLanParametersPatch | O | 0..1 | Represents the 5G LAN servise related parameters. |  |

Editor’s Note:	Whether DNN and/or S-NSSAI can be changed are FFS.

##### 5.7.2.3.6	Type: 5GLanParametersPatch

Table 5.7.2.3.6-1: Definition of type 5GLanParametersPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsis | map(Gpsi) | O | 1..N | Represents the list of 5G VN Group members, each member is identified by GPSI |  |
| appDesps | map(AppDescriptor) | O | 1..N | Describes the operation systems and the corresponding applications for each operation systems. The key of map is osId. |  |

#### 5.7.2.4	Simple data types and enumerations

##### 5.7.2.4.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

##### 5.7.2.4.2	Simple data types

The simple data types defined in table 5.7.2.4.2-1 shall be supported.

Table 5.7.2.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.7.3	Used Features

The table below defines the features applicable to the 5GLANParameterProvision API. Those features are negotiated as described in subclause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.7.3-1: Features used by 5GLANParameterProvision API

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 5.8	ApplyingBdtPolicy API

### 5.8.1	Resources

#### 5.8.1.1	Overview

This subclause describes the structure for the Resource URIs as shown in figure 5.8.1.1-1 and the resources and HTTP methods used for the ApplyingBdtPolicy API.

Figure 5.8.1.1-1: Resource URI structure of the ApplyingBdtPolicy API

Table 5.8.1.1-1 provides an overview of the resources and HTTP methods applicable for the ApplyBdtPolicy API.

Table 5.8.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Applied BDT Policy Subscription | {apiRoot}/3gpp-applying-bdt-policy/v1/{afId}/subscriptions | GET | Read all applied BDT policy subscriptions for a given AF. |
|  |  | POST | Create a new applied policy subscription. |
| Individual Applied BDT Policy Subscription | {apiRoot}/3gpp-applying-bdt-policy /v1/{afId}/subscriptions/{subscriptionId} | GET | Read the applied BDT policy subscription. |
|  |  | PATCH | Modify BDT Reference ID of an existing subscription to a BDT policy. |
|  |  | DELETE | Delete the applied BDT policy subscription |

#### 5.8.1.2	Resource: Applied BDT Policy Subscription

##### 5.8.1.2.1	Introduction

This resource allows a AF to read all applied BDT policy subscription for the given AF.

##### 5.8.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-applying-bdt-policy/v1/{afId}/subscriptions

This resource shall support the resource URI variables defined in table 5.8.1.2.2-1.

Table 5.8.1.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |

##### 5.8.1.2.3	Resource Methods

###### 5.8.1.2.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.8.1.2.2.

###### 5.8.1.2.3.2	GET

The GET method allows to read all active applied BDT policy subscription for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.8.1.2.3.2-1.

Table 5.8.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.8.1.2.3.2-2 and the response data structures and response codes specified in table 5.8.1.2.3.2-3.

Table 5.8.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.8.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(AppliedBdtPolicy) |  | 0..N | 200 OK | The applied BDT Policy subscriptions for the AF in the request URI are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.8.1.2.3.3	POST

The POST method creates an applied BDT policy subscription for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.8.1.2.3.3-1 and the response data structures and response codes specified in table 5.8.1.2.3.3-2.

Table 5.8.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppliedBdtPolicy | M | 1 | Parameters to create an subscription of the applied BDT policy. |

Table 5.8.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppliedBdtPolicy | M | 1 | 201 Created | The subscription was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

#### 5.8.1.3	Resource: Individual Applied BDT Policy Subscription

##### 5.8.1.3.1	Introduction

This resource allows a AF to read or delete an active subscription of applied BDT policy.

##### 5.8.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-applying-bdt-policy/v1/{afId}/subscriptions/{SubscriptionId}

This resource shall support the resource URI variables defined in table 5.8.1.3.2-1.

Table 5.8.1.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |
| subscriptionId | Identifier of the subscription resource of type string. |

##### 5.8.1.3.3	Resource Methods

###### 5.8.1.3.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.8.1.3.2.

###### 5.8.1.3.3.2	GET

The GET method allows to read the active applied BDT policy for a given AF and subscription Id. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.8.1.3.3.2-1.

Table 5.8.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.8.1.3.3.2-2 and the response data structures and response codes specified in table 5.8.1.3.3.2-3.

Table 5.8.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.8.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppliedBdtPolicy | M | 1 | 200 OK | The subscription information for the AF in the request URI are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.8.1.3.3.3	PATCH

The PATCH method allows to change some properties of an existing applying BDT policy subscription. The AF shall initiate the HTTP PATCH request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.8.1.3.3.3-1 and the response data structures and response codes specified in table 5.8.1.3.3.3-2.

Table 5.8.1.3.3.3-1: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppliedBdtPolicyPatch | M | 1 | Partial update of a subscription to applying BDT policy subscritpion. |

Table 5.8.1.3.3.3-2: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppliedBdtPolicy | M | 1 | 200 OK | The subscription was modified successfully. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.8.1.3.3.4	DELETE

The DELETE method deletes the applying BDT policy subscription for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.8.1.3.3.4-1.

Table 5.8.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.8.1.3.3.4-2 and the response data structures and response codes specified in table 5.8.1.3.3.4-3.

Table 5.8.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.8.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The subscription was terminated successfully. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

### 5.8.2	Notifications

Notifications are not applicable to this API.

### 5.8.3	Data Model

#### 5.8.3.1	General

This subclause specifies the application data model supported by the ApplyingBdtPolicy API.

#### 5.8.3.2	Reused data types

The data types reused by the ApplyingBdtPolicy API from other specifications are listed in table 5.8.3.2-1.

Table 5.8.3.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| BdtReferenceId | 3GPP TS 29.122 [4] | Identifier of a selected BDT policy. |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.8.4-1. |

#### 5.8.3.3	Structured data types

##### 5.8.3.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.8.3.3.2	Type: AppliedBdtPolicy

This type represents an applied BDT policy which is sent from the AF to the NEF.

Table 5.8.3.3.2-1: Definition of type AppliedBdtPolicy

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- | --- |
| bdtRefId | BdtReferenceId | M | 1 | Identifies a selected policy of background data transfer. |  |
| gpsi | Gpsi | C | 0..1 | Identifies a user. |  |
| externalGroupId | ExternalGroupId | C | 0..1 | Identifies a user group. |  |
| suppFeat | SupportedFeatures | M | 1 | Indicates the list of Supported features used as described in subclause 5.8.4. This attribute shall be provided in the POST request and in the response of successful resource creation |  |
| NOTE: Only one of the properties "gpsi" or "externalGroupId" shall be included. |  |  |  |  |  |

##### 5.8.3.3.3	Type: AppliedBdtPolicyPatch

This type represents a subscription of applied BDT policy parameters provided by the AF to the NEF. The structure is used for HTTP PATCH request.

Table 5.8.3.3.2-1: Definition of type AppliedBdtPolicyPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| bdtRefId | BdtReferenceId | M | 1 | Identifies a selected policy of background data transfer. |  |

#### 5.8.3.4	Simple data types and enumerations

##### 5.8.3.4.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

##### 5.8.3.4.2	Simple data types

The simple data types defined in table 5.8.3.4.2-1 shall be supported.

Table 5.8.3.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.8.4	Used Features

The table below defines the features applicable to the ApplyingBdtPolicy API. Those features are negotiated as described in subclause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.8.4-1: Features used by ApplyingBdtPolicy API

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 5.9	IPTVConfiguration API

### 5.9.1	Resources

#### 5.9.1.1	Overview

This subclause describes the structure for the Resource URIs as shown in figure 5.9.1.1-1 and the resources and HTTP methods used for the IPTVConfiguration API.

Figure 5.9.1.1-1: Resource URI structure of the IPTVConfiguration API

Table 5.9.1.1-1 provides an overview of the resources and HTTP methods applicable for the IPTVConfiguration API.

Table 5.9.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| IPTV Configurations | {apiRoot}/3gpp-iptvconfiguration/v1/{afId}/configurations | GET | Read all configurations for a given AF |
|  |  | POST | Create a new IPTV configuration |
| Individual IPTV Configuration | {apiRoot}/3gpp-iptvconfiguration/v1/{afId}/configurations/{configurationId} | GET | Read an existing configuration identified by {configurationId} |
|  |  | PUT | Modify all of the properties of an existing configuration identified by {configurationId} |
|  |  | PATCH | Modify some of the properties of an existing configuration identified by {configurationId} |
|  |  | DELETE | Delete the configuration identified by {configurationId} |

#### 5.9.1.2	Resource: IPTV Configurations

##### 5.9.1.2.1	Introduction

This resource allows a AF to read all active IPTV configurations for the given AF, or create an new individual IPTV configuration in the NEF.

##### 5.9.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-iptvconfiguration/v1/{afId}/configurations

This resource shall support the resource URI variables defined in table 5.9.1.2.2-1.

Table 5.9.1.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |

##### 5.9.1.2.3	Resource Methods

###### 5.9.1.2.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.9.1.2.2.

###### 5.9.1.2.3.2	GET

The GET method allows to read all active configurations for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.9.1.2.3.2-1.

Table 5.9.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.9.1.2.3.2-2 and the response data structures and response codes specified in table 5.9.1.2.3.2-3.

Table 5.9.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.9.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(IptvConfigData) | M | 0..N | 200 OK | All the configuration information for the AF in the request URI are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.9.1.2.3.3	POST

The POST method creates a new resource to individual IPTV configuration for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.9.1.2.3.3-1 and the response data structures and response codes specified in table 5.9.1.2.3.3-2.

Table 5.9.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| IptvConfigData | M | 1 | Parameters to create an IPTV Configuration resource. |

Table 5.9.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| IptvConfigData | M | 1 | 201 Created | The configuration resource was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| n/a |  |  | 204 No Content | The configuration resource was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

#### 5.9.1.3	Resource: Individual IPTV Configuration

##### 5.9.1.3.1	Introduction

This resource allows a AF to read, update or delete an existing IPTV Configuration.

##### 5.9.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-iptvconfiguration/v1/{afId}/configurations/{configurationId}

This resource shall support the resource URI variables defined in table 5.9.1.3.2-1.

Table 5.9.1.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |
| configurationId | Identifier of the configuration resource of type string. |

##### 5.9.1.3.3	Resource Methods

###### 5.9.1.3.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.9.1.3.2.

###### 5.9.1.3.3.2	GET

The GET method allows to read the active configuration for a given AF and subscription Id. The AF shall initiate the HTTP GET request message and theNEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.9.1.3.3.2-1.

Table 5.9.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.9.1.3.3.2-2 and the response data structures and response codes specified in table 5.9.1.3.3.2-3.

Table 5.9.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.9.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| IptvConfigData | M | 1 | 200 OK | The information for the configuration in the request URI are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.9.1.3.3.3	PUT

The PUT method modifies an existing resource to update a configuration. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.9.1.3.3.3-1 and the response data structures and response codes specified in table 5.9.1.3.3.3-2.

Table 5.9.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| IptvConfigData | M | 1 | Modify an existing configuration. |

Table 5.9.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| IptvConfigData | M | 1 | 200 OK | The configuration resource was updated successfully. |
| n/a |  |  | 204 No Content | The configuration resource was updated successfully. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.9.1.3.3.4	DELETE

The DELETE method deletes an existing individual configuration for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.9.1.3.3.4-1.

Table 5.9.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.9.1.3.3.4-2 and the response data structures and response codes specified in table 5.9.1.3.3.4-3.

Table 5.9.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.9.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The configuration resource was terminated successfully. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.9.1.3.3.5	PATCH

The PATCH method allows to change some properties of an existing resource to update a configuration. The AF shall initiate the HTTP PATCH request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.9.1.3.3.5-1 and the response data structures and response codes specified in table 5.9.1.3.3.5-2.

Table 5.9.1.3.3.5-1: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| IptvConfigDataPatch | M | 1 | Partial update an existing configuration. |

Table 5.9.1.3.3.5-2: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| IptvConfigData | M | 1 | 200 OK | The configuration resource was updated successfully. |
| n/a |  |  | 204 No Content | The configuration resource was updated successfully. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

### 5.9.1A	Notifications

Notifications are not applicable to this API.

### 5.9.2	Data Model

#### 5.9.2.1	General

This subclause specifies the application data model supported by the IPTVConfiguration API.

#### 5.9.2.2	Reused data types

The data types reused by the IPTVConfiguration API from other specifications are listed in table 5.9.2.2-1.

Table 5.9.2.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| Link | 3GPP TS 29.122 [4] |  |
| Snssai | 3GPP TS 29.571 [8] | Identifies the S-NSSAI. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.9.3-1. |

#### 5.9.2.3	Structured data types

##### 5.9.2.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.9.2.3.2	Type: IptvConfigData

Table 5.9.2.3.2-1: Definition of type IptvConfigData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| self | Link | C | 0..1 | Identifies the individual IPTV configuration resource URI. Shall be present in the HTTP GET response when reading all the configurations for an AF. |  |
| gpsi | Gpsi | C | 0..1 | Identifies GPSI.(NOTE) |  |
| exterGroupId | ExternalGroupId | C | 0..1 | Represents a group of users. (NOTE) |  |
| afAppId | string | M | 1 | Identifies an application. |  |
| dnn | Dnn | O | 0..1 | Identifies a DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |
| snssai | Snssai | O | 0..1 | Identifies an S-NSSAI. |  |
| multiAccCtrls | map(MulticastAccessControl) | M | 1..N | Identifies a list of multicast address access control information. |  |
| suppFeat | SupportedFeatures | M | 1 | Indicates the negotiated supported features. |  |
| NOTE: Only one of the "gpsi" or "exterGroupId" attribute shall be provided. |  |  |  |  |  |

##### 5.9.2.3.3	Type: MulticastAccessControl

Table 5.9.2.3.3-1: Definition of type MulticastAccessControl

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| srcIpv4Addr | Ipv4Addr | O | 0..1 | Identifies the source IPv4 address of IPTV multicast channel. |  |
| srcIpv6Addr | Ipv6Addr | O | 0..1 | Identifies the source IPv6 address of IPTV multicast channel. |  |
| multicastV4Addr | Ipv4Addr | O | 0..1 | Identifies the multicast IPv4 address of IPTV multicast channel. (NOTE) |  |
| multicastV6Addr | Ipv6Addr | O | 0..1 | Identifies the multicast IPv6 address of IPTV multicast channel. (NOTE) |  |
| accStatus | AccessRightStatus | M | 1 | Represents access right status of the multicast channel. |  |
| NOTE: At least one of the "multicastV4Addr" or "multicastV6Addr" attribute shall be provided. |  |  |  |  |  |

##### 5.9.2.3.4	Type: IptvConfigDataPatch

Table 5.9.2.3.4-1: Definition of type IptvConfigDataPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| multiAccCtrls | map(MulticastAccessControl) | O | 1..N | Identifies a list of multicast address access control information. |  |

#### 5.9.2.4	Simple data types and enumerations

##### 5.9.2.4.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

##### 5.9.2.4.2	Simple data types

The simple data types defined in table 5.9.2.4.2-1 shall be supported.

Table 5.9.2.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 5.9.2.4.3	Enumeration: AccessRightStatus

The enumeration AccessRightStatus represents the parameters provision type of which the AF requests to provision. It shall comply with the provisions defined in table 5.9.2.4.3-1.

Table 5.9.2.4.3-1: Enumeration ProvisionType

| Enumeration value | Description |
| --- | --- |
| FULLY_ALLOWED | The User is fully allowed to access to the channel. |
| PREVIEW_ALLOWED | The User is preview allowed to access to the channel. |
| NO_ALLOWED | The User is not allowed to access to the channel. |

### 5.9.3	Used Features

The table below defines the features applicable to the IPTVConfiguration API. Those features are negotiated as described in subclause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.9.3-1: Features used by IPTVConfiguration API

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 5.10	LpiParameterProvision API

### 5.10.1	Resources

#### 5.10.1.1	Overview

This subclause describes the structure for the Resource URIs as shown in figure 5.10.1.1-1 and the resources and HTTP methods used for the LpiParameterProvision API.

Figure 5.10.1.1-1: Resource URI structure of the LpiParameterProvision API

Table 5.10.1.1-1 provides an overview of the resources and HTTP methods applicable for the LpiParameterProvision API.

Table 5.10.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| LPI Parameters Provisionings | {apiRoot}/3gpp-lpi-pp/v1/{afId}/provisionedLpis | GET | Read all LPI Parameters Provisioning resources for a given AF |
|  |  | POST | Create a new Individual LPI Parameters Provisioning resource |
| Individual LPI Parameters Provisioning | {apiRoot}/3gpp-lpi-pp/v1/{afId}/provisionedLpis/{provisionedLpiId} | GET | Read an existing Individual LPI Parameters Provisioning resource identified by {provisionedLpiId} |
|  |  | PUT | Modify all of the properties of an existing Individual LPI Parameters Provisioning resource identified by {provisionedLpiId} |
|  |  | DELETE | Delete the existing Individual LPI Parameters Provisioning resource identified by {provisionedLpiId} |

#### 5.10.1.2	Resource: LPI Parameters Provisionings

##### 5.10.1.2.1	Introduction

This resource allows a AF to read all active LPI Parameters Provisionings for the given AF, or create an new individual LPI Parameters Provisioning resource to provision parameters to the NEF.

##### 5.10.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-lpi-pp/v1/{afId}/provisionedLpis

This resource shall support the resource URI variables defined in table 5.10.1.2.2-1.

Table 5.10.1.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |

##### 5.10.1.2.3	Resource Methods

###### 5.10.1.2.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.10.1.2.3.

###### 5.10.1.2.3.2	GET

The GET method allows to read all active provisioned LPIs for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.10.1.2.3.2-1.

Table 5.10.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.10.1.2.3.2-2 and the response data structures and response codes specified in table 5.10.1.2.3.2-3.

Table 5.10.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.10.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(LpiParametersProvision) | M | 0..N | 200 OK | All the LPI Parameters Provisioning information for the AF in the request URI are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.10.1.2.3.3	POST

The POST method creates a new resource to LPI Parameters Provisionings for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.10.1.2.3.3-1 and the response data structures and response codes specified in table 5.10.1.2.3.3-2.

Table 5.10.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LpiParametersProvision | M | 1 | Parameters to create an LPI Parameters Provisionings resource to provision parameters. |

Table 5.10.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LpiParametersProvision | M | 1 | 201 Created | The resource was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

#### 5.10.1.3	Resource: Individual LPI Parameters Provisioning

##### 5.10.1.3.1	Introduction

This resource allows a AF to read, update or delete an existing provisioned LPI parameters resource.

##### 5.10.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-lpi-pp/v1/{afId}/provisionedLpis/{provisionedLpiId}

This resource shall support the resource URI variables defined in table 5.10.1.3.2-1.

Table 5.10.1.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |
| provisionedLpiId | Identifier of the provisioning resource of type string. |

##### 5.10.1.3.3	Resource Methods

###### 5.10.1.3.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.10.1.3.3.

###### 5.10.1.3.3.2	GET

The GET method allows to read an active providedLpis for a given AF and provisionedLpiId. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.10.1.3.3.2-1.

Table 5.10.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.10.1.3.3.2-2 and the response data structures and response codes specified in table 5.10.1.3.3.2-3.

Table 5.10.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.10.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LpiParametersProvision | M | 1 | 200 OK | The information for the source in the request URI are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.10.1.3.3.3	PUT

The PUT method modifies an existing resource to update an existing individual LPI Parameters Provisioning resource. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.10.1.3.3.3-1 and the response data structures and response codes specified in table 5.10.1.3.3.3-2.

Table 5.10.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LpiParametersProvision | M | 1 | Modify an existing individual LPI Parameters Provisioning resource to provision parameters. |

Table 5.10.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LpiParametersProvision | M | 1 | 200 OK | The resource was updated successfully. |
| n/a |  |  | 204 No Content | The resource was updated successfully and no additional content is sent in the response message. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.10.1.3.3.4	DELETE

The DELETE method deletes an existing individual LPI Parameters Provisioning resource for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.10.1.3.3.4-1.

Table 5.10.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.10.1.3.3.4-2 and the response data structures and response codes specified in table 5.10.1.3.3.4-3.

Table 5.10.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.10.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The resource was removed successfully. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

### 5.10.2	Data Model

#### 5.10.2.1	General

This subclause specifies the application data model supported by the LpiParameterProvision API.

#### 5.10.2.2	Reused data types

The data types reused by the LpiParameterProvision API from other specifications are listed in table 5.10.2.2-1.

Table 5.10.2.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| Link | 3GPP TS 29.122 [4] | Identifies a referenced resource. |
| Lpi | 3GPP TS 29.503 [17] | Identifies the Location Privacy Indication information. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.10.3-1. |

#### 5.10.2.3	Structured data types

##### 5.10.2.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.10.2.3.2	Type: LpiParametersProvision

Table 5.10.2.3.2-1: Definition of type LpiParametersProvision

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| self | Link | C | 0..1 | Identifies the individual parameters provisioning resource. This attribute shall be supplied by the NEF in HTTP responses that include an object of LpiParametersProvision type. |  |
| exterGroupId | ExternalGroupId | O | 0..1 | Identifies a group of UEs. (NOTE) |  |
| gpsi | Gpsi | O | 0..1 | Identifies an UE with GPSI. (NOTE) |  |
| lpi | Lpi | M | 1 | Location Privacy Indication parameters |  |
| suppFeat | SupportedFeatures | M | 1 | Indicates the negotiated supported features. |  |
| NOTE: Only one of the "gpsi" or "exterGroupId" attribute shall be provided. |  |  |  |  |  |

#### 5.10.2.4	Simple data types and enumerations

##### 5.10.2.4.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

##### 5.10.2.4.2	Simple data types

The simple data types defined in table 5.10.2.4.2-1 shall be supported.

Table 5.10.2.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.10.3	Used Features

The table below defines the features applicable to the LpiParameterProvision API. Those features are negotiated as described in subclause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.10.3-1: Features used by LpiParameterProvision API

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 5.11	ServiceParameter API

### 5.11.1	Resources

#### 5.11.1.1	Overview

This subclause describes the structure for the Resource URIs as shown in figure 5.11.1.1-1 and the resources and HTTP methods used for the ServiceParameter API.

Figure 5.9.1.1-1: Resource URI structure of the ServiceParameter API

Table 5.11.1.1-1 provides an overview of the resources and HTTP methods applicable for the ServiceParameter API.

Table 5.9.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Service Parameter Subscripions | {apiRoot}/3gpp-serviceparameter/v1/{afId}/subscriptions | GET | Read all subscriptions for a given AF. |
|  |  | POST | Create a new service parameter subscription. |
| Individual Service Parameter Subscripion | {apiRoot}/3gpp-serviceparameter/v1/{afId}/subscriptions/{subscriptionId} | GET | Read an existing subscription identified by {subscriptionId} |
|  |  | PUT | Modify all of the properties of an existing subscription. identified by {subscriptionId} |
|  |  | PATCH | Modify some of the properties of an existing subscription identified by {subscriptionId} |
|  |  | DELETE | Delete the subscription identified by {subscriptionId} |

#### 5.11.1.2	Resource: Service Parameter Subscriptions

##### 5.11.1.2.1	Introduction

This resource allows a AF to read all active Service Parameter Subscriptions for the given AF, or create an new individual service parameter subscription in the NEF.

##### 5.11.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-serviceparameter/v1/{afId}/subscriptions

This resource shall support the resource URI variables defined in table 5.11.1.2.2-1.

Table 5.11.1.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |

##### 5.11.1.2.3	Resource Methods

###### 5.11.1.2.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.11.1.2.3.

###### 5.11.1.2.3.2	GET

The GET method allows to read all active subscriptions for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.11.1.2.3.2-1.

Table 5.11.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.11.1.2.3.2-2 and the response data structures and response codes specified in table 5.11.1.2.3.2-3.

Table 5.11.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.11.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(ServiceParameterData) | M | 0..N | 200 OK | All the subscription information for the AF in the request URI are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.11.1.2.3.3	POST

The POST method creates a new resource to individual service parameter subscription for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.11.1.2.3.3-1 and the response data structures and response codes specified in table 5.11.1.2.3.3-2.

Table 5.11.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceParameterData | M | 1 | Parameters to create a service parameter subscription resource. |

Table 5.11.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceParameterData | M | 1 | 201 Created | The subscription resource was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| n/a |  |  | 204 No Content | The subscription resource was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

#### 5.11.1.3	Resource: Individual Service Parameter Subscription

##### 5.11.1.3.1	Introduction

This resource allows a AF to read, update or delete an existing service parameter subscription.

##### 5.11.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-serviceparameter/v1/{afId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.9.1.3.2-1.

Table 5.11.1.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |
| subscriptionId | Identifier of the subscription resource of type string. |

##### 5.11.1.3.3	Resource Methods

###### 5.11.1.3.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.11.1.3.3.

###### 5.11.1.3.3.2	GET

The GET method allows to read the active subscription for a given AF and subscription Id. The AF shall initiate the HTTP GET request message and theNEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.11.1.3.3.2-1.

Table 5.11.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.11.1.3.3.2-2 and the response data structures and response codes specified in table 5.11.1.3.3.2-3.

Table 5.11.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.11.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceParameterData | M | 1 | 200 OK | The information for the subscription in the request URI are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.11.1.3.3.3	PUT

The PUT method modifies an existing resource to update a configuration. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.9.1.3.3.3-1 and the response data structures and response codes specified in table 5.9.1.3.3.3-2.

Table 5.11.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceParameterData | M | 1 | Modify an existing subscription. |

Table 5.11.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceParameterData | M | 1 | 200 OK | The subscription resource was updated successfully. |
| n/a |  |  | 204 No Content | The subscription resource was updated successfully. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.11.1.3.3.4	DELETE

The DELETE method deletes an existing individual subscription for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.11.1.3.3.4-1.

Table 5.11.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.11.1.3.3.4-2 and the response data structures and response codes specified in table 5.11.1.3.3.4-3.

Table 5.11.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.11.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The subscription resource was terminated successfully. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.11.1.3.3.5	PATCH

The PATCH method allows to change some properties of an existing resource to update a subscription. The AF shall initiate the HTTP PATCH request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.11.1.3.3.5-1 and the response data structures and response codes specified in table 5.11.1.3.3.5-2.

Table 5.9.1.3.3.5-1: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceParameterDataPatch | M | 1 | Partial update an existing subscription. |

Table 5.9.1.3.3.5-2: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceParameterDataPatch | M | 1 | 200 OK | The subscription resource was updated successfully. |
| n/a |  |  | 204 No Content | The subscription resource was updated successfully. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

### 5.11.2	Data Model

#### 5.11.2.1	General

This subclause specifies the application data model supported by the ServiceParameter API.

#### 5.11.2.2	Reused data types

The data types reused by the ServiceParameter API from other specifications are listed in table 5.9.2.2-1.

Table 5.11.2.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| DataTime | 3GPP TS 29.571 [8] |  |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| IPv4Addr | 3GPP TS 29.571 [8] | Identifies an IPv4 address. |
| IPv6Addr | 3GPP TS 29.571 [8] | Identifies an IPv6 address. |
| Link | 3GPP TS 29.122 [4] |  |
| MacAddr48 | 3GPP TS 29.571 [8] | Identifies an MAC address. |
| Pc5QosFlowItem | 3GPP TS 29.571 [8] |  |
| Pc5QosPara | 3GPP TS 29.571 [8] |  |
| RatType | 3GPP TS 29.571 [8] |  |
| Snssai | 3GPP TS 29.571 [8] | Identifies the S-NSSAI. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.9.4-1. |
| Uinteger | 3GPP TS 29.571 [8] |  |

#### 5.11.2.3	Structured data types

##### 5.11.2.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.11.2.3.2	Type: ServiceParameterData

Table 5.11.2.3.2-1: Definition of type ServiceParameterData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| self | Link | C | 0..1 | Identifies the individual service parameter subscription resource URI. Shall be present by the NEF in HTTP responses that include an object of ServiceParameterData type. |  |
| dnn | Dnn | O | 0..1 | Identifies a DNN. |  |
| snssai | Snssai | O | 0..1 | Identifies an S-NSSAI. |  |
| afServiceId | string | O | 0..1 | Identifies a service on behalf of which the AF is issuing the request. |  |
| appId | string | O | 0..1 | Identifies an application identifier. |  |
| gpsi | Gpsi | O | 0..1 | Identifies GPSI. |  |
| ueIpv4 | Ipv4Addr | O | 0..1 | The IPv4 address of the served UE. |  |
| ueIpv6 | Ipv6Addr | O | 0..1 | The IPv6 address of the served UE. |  |
| ueMac | MacAddr48 | O | 0..1 | The MAC address of the served UE. |  |
| exterGroupId | ExternalGroupId | O | 0..1 | Represents a group of users. |  |
| anyUeInd | boolean | O | 0..1 | Identifies whether the service parameters applies to any UE. This attribute shall set to "true" if applicable for any UE, otherwise, set to "false". |  |
| paramOverPc5 | ParameterOverPc5 | O | 0..1 | Contains the service parameter used over PC5 |  |
| paramOverUu | ParameterOverUu | O | 0..1 | Contains the service parameter used over Uu |  |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the list of Supported features used as described in subclause 5.8. This parameter shall be supplied by the NF service consumer in the POST request that requested the creation of an individual SM policy resource. |  |
| NOTE 1: One of individual UE identifier (i.e. "gpsi", "ueIpv4", "ueIpv6" or "ueMac" attribute), External Group Identifier (i.e. "exterGroupId" attribute) or any UE indication (i.e. "anyUeInd" attribute) shall be included. NOTE 2: Either the "afServiceId" attribute, "appId" attribute or the combination of "snssai" and "dnn" attributes shall be provided. |  |  |  |  |  |

##### 5.11.2.3.3	Type: ParameterOverPc5

Table 5.11.2.3.3-1: Definition of type ParameterOverPc5

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| expiry | DateTime | O | 0..1 | When present, it indicates the expiration time for the validity of the configuration parameters for V2X communication over PC5. If not present, the configuration paremeters are always valid until the AF removes them. |  |
| plmmRatServed | array(PlmnRatServed) | O | 1..N | Contains a list of PLMNs and RATs in which the UE is authorized to use V2X communication over PC5 when the UE is served by E-UTRA or served by NR. |  |
| authNotServed | boolean | O | 0..1 | When present and set to TRUE, it indicates the UE is authorized to use V2X communication over PC5 when the UE is not served by E-UTRA and not served by NR. When not present or present and set to FALSE, it indicates the UE is not authorized to use V2X communication over PC5 when the UE is not served by E-UTRA and not served by NR. |  |
| radioParamsNotServed | array(RadioParameterNotServed) | O | 1..N | Contains a list of radio parameters for V2X communication over PC5 applicable when the UE is not served by E-UTRA, not served by NR and is located in the geographical area, with an indication of whether these radio parameters are "operator managed" or "non-operator managed" |  |
| serToTx | array(ServiceToTx) | O | 1..N | Contains a list of V2X service identifier to Tx profiles mapping rules. Each mapping rule contains one or more V2X service identifiers and a Tx profile |  |
| privacyParams | array(PrivacyParameter) | O | 1..N | Contains a list of V2X services requiring privacy. Each entry of the list contains one or more V2X service identifiers and one or more geographical areas where the privacy is required |  |
| configParaEutra | ConfigurationParametersEutra | O | 0..1 | Contains the configuration parameters for a V2X communication over PC5 in E-UTRA. |  |
| configParaNr | ConfigurationParametersNr | O | 0..1 | Contains the configuration parameters for a V2X communication over PC5 in NR. |  |

##### 5.11.2.3.4	Type: ParameterOverUu

Table 5.11.2.3.4-1: Definition of type ParameterOverUu

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| expiry | DateTime | O | 0..1 | When present, it indicates the expiration time for the validity of the configuration parameters for V2X communication over Uu to 5GCN. If not present, the configuration paremeters are always valid until the AF removes them. |  |
| serToPduSess | array(ServiceToPduSession) | O | 1..N | Contains a list of V2X service identifier to PDU session parameters mapping rules. Each mapping rule contains one or more V2X service identifiers of a the V2X service and one or more parameters for establishment of a PDU session for V2X communication over Uu for the V2X services |  |
| serToAppAddr | array(ServiceToApplicationServerAddress) | O | 1..N | Contains a list of V2X service identifier to V2X application server address mapping rules. |  |

##### 5.11.2.3.5	Type: PlmnRatServed

Table 5.11.2.3.5-1: Definition of type PlmmRatServed

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| plmn | Plmn | M | 1 | Contains a PLMN ID which the UE is authorized to use V2X communication over PC5 |  |
| rats | array(V2xRatType) | M | 1..N | Contains the RAT(s) in which the UE is authorized to use V2X communication over PC5. Only E-UTRAN and NR are applicable. |  |

##### 5.11.2.3.6	Type: RadioParameterNotServed

Table 5.11.2.3.6-1: Definition of type RadioParameterNotServed

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| radioParams | string | M | 1 | Contains the radio parameters for V2X communication over PC5 applicable when the UE is not served by E-UTRA, not served by NR |  |
| rat | V2xRatType | M | 1 | Contains the RAT which is not serving the UE. |  |
| geographicalArea | string | M | 1 | Refer to geographical Information. See 23.032 [30] subclause 7.3.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used. Allowed characters are 0-9 and A-F; |  |
| operManaged | OperatorManaged | M |  | Indicates whether these radio parameters are "operator managed" or "non-operator managed". |  |

Editor’s note:	The data type of "radioParams" attribute is FFS.

##### 5.11.2.3.7	Type: ServiceToTx

Table 5.11.2.3.7-1: Definition of type ServiceToTx

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serIds | array(string) | M | 1..N | Contains a list of V2X service identifier. |  |
| txProfile | string | M | 1 | Contains a Tx profile. |  |
| rat | V2xRatType | M | 1 | Contains the PC5 RAT used by the UE. |  |

Editor’s note:	The data type of "txProfile" attribute is FFS.

##### 5.11.2.3.8	Type: PrivacyParameter

Table 5.11.2.3.8-1: Definition of type PrivacyParameter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serIds | array(string) | M | 1..N | Contains a list of V2X service identifier. |  |
| geographicalArea | string | M | 1 | Refer to geographical Information. See 23.032 [30] subclause 7.3.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used. Allowed characters are 0-9 and A-F. |  |
| timer | DurationSec | M | 1 | Contains a privacy timer value. |  |

##### 5.11.2.3.9	Type: ConfigurationParametersEutra

Table 5.11.2.3.9-1: Definition of type ConfigurationParametersEutra

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serIdToLayer2Ids | array(ServiceIdToLayer2Id) | O | 1..N | Contains a list of V2X service identifier to destination layer-2 ID mapping rules. |  |
| ppppToPdbs | array(PpppToPdb) | O | 1..N | Contains a list of PPPP to PDB mapping rules |  |
| serIdToFrequs | array(ServiceIdToFrequency) | O | 1..N | Contains a list of V2X service identifier to V2X E-UTRA frequency mapping rules |  |
| serIdToPpprs | Array(ServiceIdToPppr) | O | 1..N | Contains a list of the V2X services authorized for ProSe Per-Packet Reliability (PPPR). |  |
| NOTE: At lease one of attributes shall be included. |  |  |  |  |  |

##### 5.11.2.3.10	Type: ConfigurationParametersNr

Table 5.11.2.3.10-1: Definition of type ConfigurationParametersNr

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serIdToBroLayer2Ids | array(ServiceIdToLayer2Id) | O | 1..N | Contains a list of V2X service identifier to destination layer-2 ID for broadcast mapping rules. |  |
| serIdToGroLayer2Ids | array(ServiceIdToLayer2Id) | O | 1..N | Contains a list of V2X service identifier to destination layer-2 ID for groupcast mapping rules. |  |
| serIdToDefLayer2Ids | array(ServiceIdToLayer2Id) | O | 1..N | Contains a list of V2X service identifier to default destination layer-2 ID for unicast initial signaling mapping rules. |  |
| serIdToFrequs | array(ServiceIdToFrequency) | O | 1..N | Contains a list of V2X service identifier to V2X E-UTRA frequency mapping rules |  |
| pc5QosMappings | array(Pc5QosMapping) | O | 1..N | Contains a list of PC5 QoS mapping rules. |  |
| slrbConfigs | array(SlrbConfigurations) | O | 1..N | Contains a list of SLRB mapping rules applicable when the UE is not served by E-UTRA and is not served by NR. |  |
| NOTE: At lease one of attributes shall be included. |  |  |  |  |  |

##### 5.11.2.3.11	Type: ServiceIdToLayer2Id

Table 5.11.2.3.11-1: Definition of type ServiceIdToLayer2Id

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serId | array(string) | M | 1..N | Contains a list of V2X service identifier |  |
| desLayer2Id | string | M | 1 | Contains a destination layer-2 ID. |  |

##### 5.11.2.3.12	Type: PpppToPdb

Table 5.11.2.3.12-1: Definition of type PpppToPdb

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pppp | Uinteger | M | 1 | Contains a ProSe Per-Packet Priority. |  |
| pdb | PacketDelBudget | M | 1 | Contains a Packet Delay Budget. |  |

##### 5.11.2.3.13	Type: ServiceIdToFrequency

Table 5.11.2.3.13-1: Definition of type ServiceIdToFrequency

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serId | array(string) | M | 1..N | Contains a list of V2X service identifier |  |
| frequency | number | M | 1 | Contains a frequency in the unit of Khz. |  |
| geographicalArea | string | M | 1 | Refer to geographical Information. See 23.032 [30] subclause 7.3.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used. Allowed characters are 0-9 and A-F. |  |

##### 5.11.2.3.14	Type: ServiceIdToPppr

Table 5.11.2.3.14-1: Definition of type ServiceIdToPppr

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serId | array(string) | M | 1..N | Contains a list of V2X service identifier |  |
| pppr | Uinteger | M | 1 | Contains a ProSe Per-Packet Reliability (PPPR) value. |  |

##### 5.11.2.3.15	Type: Pc5QosMapping

Table 5.11.2.3.15-1: Definition of type Pc5QosMapping

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serId | array(string) | M | 1..N | Contains a list of V2X service identifier |  |
| pc5QosFlowParam | Pc5QosFlowItem | M | 1 | Contains the authorized PC5 QoS parameter. |  |

##### 5.11.2.3.16	Type: SlrbConfigurations

Table 5.11.2.3.16-1: Definition of type SlrbConfigurations

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pc5QosPara | Pc5QosPara | M | 1 | Represents the PC5 QoS Parameters (see clause 5.4.2.2 of 3GPP TS 23.287 [31]). |  |
| slrb | string | M | 1 | Contains a SLRB. |  |

Editor’s note:	The data type of slrb is FFS.

##### 5.11.2.3.17	Type: ServiceToPduSession

Table 5.11.2.3.17-1: Definition of type ServiceToPduSession

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serIds | array(string) | M | 1..N | Contains a list of V2X service identifier |  |
| pduSessionType | PduSessionType | O | 0..1 | Indicates the type of a PDU session. |  |
| sscMode | SscMode | O | 0..1 | Indicates the SSC Mode. |  |
| sliceInfo | array(Snssai) | O | 1..N | Identifies ta list of S-NSSAIs. |  |
| dnns | array(Dnn) | O | 1..N | Identifies a list of DNNs. |  |

Editor’s note:	Whether the transport layer protocol shall be added is FFS.

##### 5.11.2.3.18	Type: ServiceToApplicationServerAddress

Table 5.11.2.3.18-1: Definition of type ServiceToApplicationServerAddress

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serId | array(string) | M | 1..N | Contains a list of V2X service identifier |  |
| fqdn | Fqdn | O | 0..1 | Indicates a FQDN of the V2X application server. |  |
| ipv4Addrs | array(Ipv4Addr) | O | 1..N | The IPv4 Address(es) of the V2X application server. |  |
| ipv6Addrs | array(Ipv6Addr) | O | 1..N | The IPv6 Address(es) of the V2X application server. |  |
| udpPortNumbers | array(Uinteger) | O | 1..N | UDP port number(s). |  |
| tcpPortNumbers | array(Uinteger) | O | 1..N | TCP port number(s). |  |
| plmns | array(Plmn) | M | 1..N | A list of PLMNs. |  |
| geographicalArea | string | M | 1 | Refer to geographical Information. See 23.032 [30] subclause 7.3.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used. Allowed characters are 0-9 and A-F. |  |
| NOTE: Either the FQDN (i.e. "fqdn" attribute) or IP endpoint(s) ( i.e. a combination of "ipv4Addrs" and "udpPortNumbers" or "tcpPortNumbers", or a combination of "ipv6Addrs" and "udpPortNumbers" or "tcpPortNumbers") shall be included. |  |  |  |  |  |

##### 5.11.2.3.19	Type: ServiceParameterDataPatch

Table 5.11.2.3.2-1: Definition of type ServiceParameterDataPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| paramOverPc5 | ParameterOverPc5Rm | O | 0..1 | Contains the service parameter used over PC5 |  |
| paramOverUu | ParameterOverUuRm | O | 0..1 | Contains the service parameter used over Uu |  |

Editor’s note:	The data type of ParameterOverPc5Rm and  ParameterOverUuRm are FFS.

#### 5.11.2.4	Simple data types and enumerations

##### 5.11.2.4.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

##### 5.11.2.4.2	Simple data types

The simple data types defined in table 5.11.2.4.2-1 shall be supported.

Table 5.11.2.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 5.11.2.4.3	Enumeration: OperatorManaged

The enumeration OperatorManaged represents whether these radio parameters are "operator managed" or "non-operator managed". It shall comply with the provisions defined in table 5.11.2.4.3-1.

Table 5.11.2.4.3-1: Enumeration OperatorManaged

| Enumeration value | Description |
| --- | --- |
| OPERATOR_MANAGED | The radio parameters are "operator managed". |
| NON_OPERATOR_MANAGED | The radio parameters are "non-operator managed". |

##### 5.11.2.4.4	Enumeration: V2xRatType

Table 5.11.2.4.4-1: Enumeration V2xRatType

| Enumeration value | Description |
| --- | --- |
| PC5_LTE | Indicates the LTE RAT type over PC5 interface. |
| PC5_NR | Indicates the NR RAT type over NR interface. |

### 5.11.3	Used Features

The table below defines the features applicable to the ServiceParameter API. Those features are negotiated as described in subclause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.9.3-1: Features used by ServiceParameter API

| Feature number | Feature Name | Description |
| --- | --- | --- |

# 6	Security

TLS (IETF RFC 5246 [16]) shall be used to support the security communication between the NEF and the AF over NEF Northbound interface as defined in subclause 12 of 3GPP TS 33.501 [6]. The access to the SCEF northbound APIs shall be authorized by means of OAuth2 protocol (see IETF RFC 6749 [13]), based on local configuration, using the "Client Credentials" authorization grant. If OAuth2 is used, a client, prior to consuming services offered by the NEF Northbound APIs, shall obtain a "token" from the authorization server.

# 7	Using Common API Framework

## 7.1	General

When CAPIF is used with an NEF that is used for external exposure, the NEF shall support the following as defined in 3GPP TS 29.222 [12]:

-	the API exposing function and related APIs over CAPIF-2/2e and CAPIF-3 reference points;

-	the API publishing function and related APIs over CAPIF-4 reference point;

-	the API management function and related APIs over CAPIF-5 reference point; and

-	at least one of the security methods for authentication and authorization, and related security mechanisms.

In a centralized deployment as defined in 3GPP TS 23.222 [11], where the CAPIF core function and API provider domain functions are co-located, the interactions between the CAPIF core function and API provider domain functions may be independent of CAPIF-3, CAPIF-4 and CAPIF-5 reference points.

## 7.2	Security

When CAPIF is used for external exposure, before invoking the API exposed by the NEF, the AF as API invoker shall negotiate the security method (PKI, TLS-PSK or OAUTH2) with CAPIF core function and ensure the NEF has enough credential to authenticate the AF (see 3GPP TS 29.222 [12], subclause 5.6.2.2 and subclause 6.2.2.2).

If PKI or TLS-PSK is used as the selected security method between the AF and the NEF, upon API invocation, the NEF shall retrieve the authorization information from the CAPIF core function as described in 3GPP TS 29.222 [12], subclause 5.6.2.4.

As indicated in 3GPP TS 33.122 [14], the access to the NEF northbound APIs may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [13]), using the "Client Credentials" authorization grant, where the CAPIF core function (see 3GPP TS 29.222 [12]) plays the role of the authorization server.

NOTE 1:	In this release, only "Client Credentials" authorization grant is supported.

If OAuth2 is used as the selected security method between the AF and the NEF, the AF, prior to consuming services offered by the NEF northbound APIs, shall obtain a "token" from the authorization server, by invoking the Obtain_Authorization service, as described in 3GPP TS 29.222 [12], subclause 5.6.2.3.2.

The NEF northbound APIs do not define any scopes for OAuth2 authorization. It is the NEF responsibility to check whether the AF is authorized to use an API based on the "token". Once the NEF verifies the "token", it shall check whether the NEF identifier in the "token" matches its own published identifier, and whether the API name in the "token" matches its own published API name. If those checks are passed, the AF has full authority to access any resource or operation for the invoked API.

NOTE 2:	For aforementioned security methods, the NEF needs to apply admission control according to access control policies after performing the authorization checks.

NOTE 3:	The security requirement in the current subclause does not apply for the NiddConfigurationTrigger and the MsisdnLessMoSms APIs since they are the NEF initiated interaction with the AF. How the security scheme works for the NiddConfigurationTrigger and MsisdnLessMoSms APIs is left to configuration.

Annex A (normative):
OpenAPI representation for NEF Northbound APIs

# A.1	General

This Annex is based on the OpenAPI 3.0.0 specification [5] and provides corresponding representations of all APIs defined in the present specification.

NOTE 1:	An OpenAPIs representation embeds JSON Schema representations of HTTP message bodies.

This Annex shall take precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 2:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on the public 3GPP file server in the following locations (see clause 5B of the 3GPP TR 21.900 [21] for further information):

-	https://www.3gpp.org/ftp/Specs/archive/OpenAPI/<Release>/, and

-	https://www.3gpp.org/ftp/Specs/<Plenary>/<Release>/OpenAPI/.

NOTE 3:	To fetch the OpenAPI specification files after CT#83 plenary meeting for Release 15 in the above links <Plenary> must be replaced with the date the CT Plenary occurs, in the form of year-month (yyyy-mm), e.g. for CT#83 meeting <Plenary> must be replaced with value "2019-03" and <Release> must be replaced with value "Rel-15".

# A.2	TrafficInfluence API

openapi: 3.0.0

info:

title: 3gpp-traffic-influence

version: 1.1.0.alpha-4

description: |

API for AF traffic influence

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.522 V16.3.0; 5G System; Network Exposure Function Northbound APIs.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-traffic-influence/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 5.2.4 of 3GPP TS 29.122.

paths:

/{afId}/subscriptions:

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

get:

summary: read all of the active subscriptions for the AF

tags:

- TrafficInfluence API SCS/AS level GET Operation

responses:

'200':

description: OK.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/TrafficInfluSub'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Creates a new subscription resource

tags:

- TrafficInfluence API Subscription level POST Operation

requestBody:

description: Request to create a new subscription resource

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EventNotification'

callbacks:

afAcknowledgement:

'{request.body#/afAckUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AfAckInfo'

responses:

'204':

description: No Content (successful acknowledgement)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'204':

description: No Content (successful notification)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Created (Successful creation of subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{afId}/subscriptions/{subscriptionId}:

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the subscription resource

required: true

schema:

type: string

get:

summary: read an active subscriptions for the SCS/AS and the subscription Id

tags:

- TrafficInfluence API Subscription level GET Operation

responses:

'200':

description: OK (Successful get the active subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Updates/replaces an existing subscription resource

tags:

- TrafficInfluence API subscription level PUT Operation

requestBody:

description: Parameters to update/replace the existing subscription

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

responses:

'200':

description: OK (Successful update of the subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Updates/replaces an existing subscription resource

tags:

- TrafficInfluence API subscription level PATCH Operation

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/TrafficInfluSubPatch'

responses:

'200':

description: OK. The subscription was modified successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an already existing subscription

tags:

- TrafficInfluence API Subscription level DELETE Operation

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

TrafficInfluSub:

type: object

properties:

afServiceId:

type: string

description: Identifies a service on behalf of which the AF is issuing the request.

afAppId:

type: string

description: Identifies an application.

afTransId:

type: string

description: Identifies an NEF Northbound interface transaction, generated by the AF.

appReloInd:

type: boolean

description: Identifies whether an application can be relocated once a location of the application has been selected.

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

externalGroupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

anyUeInd:

type: boolean

description: Identifies whether the AF request applies to any UE. This attribute shall set to "true" if applicable for any UE, otherwise, set to "false".

subscribedEvents:

type: array

items:

$ref: '#/components/schemas/SubscribedEvent'

minItems: 1

description: Identifies the requirement to be notified of the event(s).

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

ipv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

macAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

dnaiChgType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DnaiChangeType'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Link'

requestTestNotification:

type: boolean

description: Set to true by the SCS/AS to request the NEF to send a test notification as defined in subclause 5.2.5.3. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

self:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Link'

trafficFilters:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/FlowInfo'

minItems: 1

description: Identifies IP packet filters.

ethTrafficFilters:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/EthFlowDescription'

minItems: 1

description: Identifies Ethernet packet filters.

trafficRoutes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

minItems: 1

description: Identifies the N6 traffic routing requirement.

tfcCorrInd:

type: boolean

tempValidities:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/TemporalValidity'

validGeoZoneIds:

type: array

items:

type: string

minItems: 1

description: Identifies a geographic zone that the AF request applies only to the traffic of UE(s) located in this specific zone.

afAckInd:

type: boolean

addrPreserInd:

type: boolean

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

allOf:

- oneOf:

- required: [afAppId]

- required: [trafficFilters]

- required: [ethTrafficFilters]

- oneOf:

- required: [ipv4Addr]

- required: [ipv6Addr]

- required: [macAddr]

- required: [gpsi]

- required: [externalGroupId]

- required: [anyUeInd]

anyOf:

- not:

required: [subscribedEvents]

- required: [notificationDestination]

TrafficInfluSubPatch:

type: object

properties:

appReloInd:

type: boolean

description: Identifies whether an application can be relocated once a location of the application has been selected.

nullable: true

trafficFilters:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/FlowInfo'

minItems: 1

description: Identifies IP packet filters.

ethTrafficFilters:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/EthFlowDescription'

minItems: 1

description: Identifies Ethernet packet filters.

trafficRoutes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

minItems: 1

description: Identifies the N6 traffic routing requirement.

tfcCorrInd:

type: boolean

nullable: true

tempValidities:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/TemporalValidity'

minItems: 1

nullable: true

validGeoZoneIds:

type: array

items:

type: string

minItems: 1

description: Identifies a geographic zone that the AF request applies only to the traffic of UE(s) located in this specific zone.

nullable: true

afAckInd:

type: boolean

nullable: true

addrPreserInd:

type: boolean

nullable: true

EventNotification:

type: object

properties:

afTransId:

type: string

description: Identifies an NEF Northbound interface transaction, generated by the AF.

dnaiChgType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DnaiChangeType'

sourceTrafficRoute:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

subscribedEvent:

$ref: '#/components/schemas/SubscribedEvent'

targetTrafficRoute:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

sourceDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

targetDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

srcUeIpv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

srcUeIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

tgtUeIpv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

tgtUeIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

ueMac:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

afAckUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Link'

required:

- dnaiChgType

- subscribedEvent

AfResultInfo:

type: object

properties:

afStatus:

$ref: '#/components/schemas/AfResultStatus'

trafficRoute:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

required:

- afStatus

AfAckInfo:

type: object

properties:

afTransId:

type: string

ackResult:

$ref: '#/components/schemas/AfResultInfo'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

required:

- ackResult

SubscribedEvent:

anyOf:

- type: string

enum:

- UP_PATH_CHANGE

- type: string

description: >

Possible values are

- UP_PATH_CHANGE: The AF requests to be notified when the UP path changes for the PDU session.

AfResultStatus:

anyOf:

- type: string

enum:

- SUCCESS

- TEMPORARY_CONGESTION

- RELOC_NO_ALLOWED

- OTHER

- type: string

description: >

Possible values are

- SUCCESS: The application layer is ready or the relocation is completed.

- TEMPORARY_CONGESTION: The application relocation fails due to temporary congestion.

- RELOC_NO_ALLOWED: The application relocation fails because application relocation is not allowed.

- OTHER: The application relocation fails due to other reason.

# A.3	NiddConfigurationTrigger API

openapi: 3.0.0

info:

title: 3gpp-nidd-configuration-trigger

version: 1.0.0.alpha-1

description: |

API for NIDD Configuration Trigger.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.522 V16.3.0; 5G System; Network Exposure Function Northbound APIs.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 5.2.4 of 3GPP TS 29.122.

paths:

/:

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NiddConfigurationTrigger'

responses:

'200':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/NiddConfigurationTriggerReply'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

NiddConfigurationTrigger:

type: object

properties:

afId:

type: string

description: Identifies the trigger receiving entity.

nefId:

type: string

description: Identifies the trigger sending entity.

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

- nefId

- gpsi

- suppFeat

NiddConfigurationTriggerReply:

type: object

properties:

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- suppFeat

# A.4	AnalyticsExposure API

openapi: 3.0.0

info:

title: 3gpp-analyticsexposure

version: 1.0.0.alpha-2

description: |

API for Analytics Exposure.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.522 V16.3.0; 5G System; Network Exposure Function Northbound APIs.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-analyticsexposure/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 5.2.4 of 3GPP TS 29.122.

paths:

/{afId}/subscriptions:

get:

summary: read all of the active subscriptions for the AF

tags:

- AF level GET Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

responses:

'200':

description: OK (Successful get all of the active subscriptions for the AF)

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/AnalyticsExposureSubsc'

minItems: 0

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Creates a new subscription resource

tags:

- subscription level POST Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

requestBody:

description: new subscription creation

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AnalyticsExposureSubsc'

callbacks:

notification:

'{request.body#/notifUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AnalyticsEventNotification'

responses:

'204':

description: No Content (successful notification)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Created (Successful creation)

content:

application/json:

schema:

$ref: '#/components/schemas/AnalyticsExposureSubsc'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'204':

description: Successful case. The resource has been successfully created and no additional content is to be sent in the response message.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{afId}/subscriptions/{subscriptionId}:

get:

summary: read an active subscription for the AF and the subscription Id

tags:

- Subscription level GET Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the subscription resource

required: true

schema:

type: string

responses:

'200':

description: OK (Successful get the active subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/AnalyticsExposureSubsc'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Updates/replaces an existing subscription resource

tags:

- Subscription level PUT Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the subscription resource

required: true

schema:

type: string

requestBody:

description: Parameters to update/replace the existing subscription

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AnalyticsExposureSubsc'

responses:

'200':

description: OK (Successful deletion of the existing subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/AnalyticsExposureSubsc'

'204':

description: Successful case. The resource has been successfully updated and no additional content is to be sent in the response message.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an already existing subscription

tags:

- Subscription level DELETE Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the subscription resource

required: true

schema:

type: string

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{afId}/fetch:

post:

summary: Fetch analytics information

tags:

- custom operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AnalyticsRequest'

responses:

'200':

description: The requested information was returned successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/AnalyticsData'

'204':

description: No Content (The requested Analytics data does not exist)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

AnalyticsExposureSubsc:

type: object

properties:

analyEventsSubs:

type: array

items:

$ref: '#/components/schemas/AnalyticsEventSubsc'

minItems: 1

analyRepInfo:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/EventReportingRequirement'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- analyEventsSubs

- notifUri

- notifId

AnalyticsEventNotification:

type: object

properties:

notifId:

type: string

analyEventNotifs:

type: array

items:

$ref: '#/components/schemas/AnalyticsEventNotif'

minItems: 1

AnalyticsEventNotif:

type: object

properties:

analyEvent:

$ref: '#/components/schemas/AnalyticsEvent'

timeStamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

ueMobilityInfos:

type: array

items:

$ref: '#/components/schemas/UeMobilityExposure'

minItems: 1

ueCommInfos:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/UeCommunication'

minItems: 1

abnormalInfos:

type: array

items:

$ref: '#/components/schemas/AbnormalExposure'

minItems: 1

congestInfos:

type: array

items:

$ref: '#/components/schemas/CongestInfo'

minItems: 1

nwPerfInfos:

type: array

items:

$ref: '#/components/schemas/NetworkPerfExposure'

minItems: 1

qosSustainInfos:

type: array

items:

$ref: '#/components/schemas/QosSustainabilityExposure'

minItems: 1

required:

- analyEvent

- timeStamp

AnalyticsEventSubsc:

type: object

properties:

analyEvent:

$ref: '#/components/schemas/AnalyticsEvent'

analyEventFilter:

$ref: '#/components/schemas/AnalyticsEventFilterSubsc'

tgtUe:

$ref: '#/components/schemas/TargetUeId'

required:

- analyEvent

- tgtUe

AnalyticsEventFilterSubsc:

type: object

properties:

nwPerfReqs:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NetworkPerfRequirement'

minItems: 1

locArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

appIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

excepRequs:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/Exception'

minItems: 1

exptAnaType:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/ExpectedAnalyticsType'

exptUeBehav:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExpectedUeBehaviourData'

reptThlds:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/ThresholdLevel'

minItems: 1

maxAnaEntry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

qosReq:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/QosRequirement'

qosFlowRetThds:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/RetainabilityThreshold'

minItems: 1

ranUeThrouThds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

minItems: 1

TargetUeId:

type: object

properties:

anyUeInd:

type: boolean

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

exterGroupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

UeMobilityExposure:

type: object

properties:

ts:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

recurringTime:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

duration:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

durationVariance:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

locInfo:

type: array

items:

$ref: '#/components/schemas/UeLocationInfo'

minItems: 1

required:

- duration

- locInfo

UeLocationInfo:

type: object

properties:

loc:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

ratio:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SamplingRatio'

confidence:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- loc

AnalyticsRequest:

type: object

properties:

analyEvent:

$ref: '#/components/schemas/AnalyticsEvent'

analyEventFilter:

$ref: '#/components/schemas/AnalyticsEventFilter'

analyRep:

$ref: 'TS29520_Nnwdaf_AnalyticsInfo.yaml#/components/schemas/AnalyticsReportingRequirement'

tgtUe:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/TargetUeInformation'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- analyEvent

- suppFeat

AnalyticsEventFilter:

type: object

properties:

locArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

maxAnaEntry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

nwPerfTypes:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NetworkPerfType'

minItems: 1

appIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

excepIds:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/ExceptionId'

minItems: 1

exptAnaType:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/ExpectedAnalyticsType'

exptUeBehav:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExpectedUeBehaviourData'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

qosReq:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/QosRequirement'

AnalyticsData:

type: object

properties:

ueMobilityInfos:

type: array

items:

$ref: '#/components/schemas/UeMobilityExposure'

minItems: 1

ueCommInfos:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/UeCommunication'

minItems: 1

nwPerfInfos:

type: array

items:

$ref: '#/components/schemas/NetworkPerfExposure'

minItems: 1

abnormalInfos:

type: array

items:

$ref: '#/components/schemas/AbnormalExposure'

minItems: 1

congestInfos:

type: array

items:

$ref: '#/components/schemas/CongestInfo'

minItems: 1

qosSustainInfos:

type: array

items:

$ref: '#/components/schemas/QosSustainabilityExposure'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- suppFeat

NetworkPerfExposure:

type: object

properties:

locArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

nwPerfType:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NetworkPerfType'

relativeRatio:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SamplingRatio'

absoluteNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- locArea

- nwPerfType

AbnormalExposure:

type: object

properties:

gpsis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

excep:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/Exception'

ratio:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SamplingRatio'

confidence:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

addtMeasInfo:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/AdditionalMeasurement'

required:

- excep

CongestInfo:

type: object

properties:

locArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

cngAnas:

type: array

items:

$ref: '#/components/schemas/CongestionAnalytics'

required:

- locArea

- cngAnas

CongestionAnalytics:

type: object

properties:

cngType:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/CongestionType'

tmWdw:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

nsi:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/ThresholdLevel'

confidence:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- cngType

- tmWdw

- nsi

QosSustainabilityExposure:

type: object

properties:

locArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

startTs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

endTs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

qosFlowRetThd:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/RetainabilityThreshold'

ranUeThrouThd:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

confidence:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- locArea

- startTs

- endTs

AnalyticsEvent:

anyOf:

- type: string

enum:

- UE_MOBILITY

- UE_COMM

- ABNORMAL_BEHAVIOR

- CONGESTION

- NETWORK_PERFORMANCE

- QOS_SUSTAINABILITY

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- UE_MOBILITY: The AF requests to be notified about analytics information of UE mobility.

- UE_COMM: The AF requests to be notified about analytics information of UE communication.

- ABNORMAL_BEHAVIOR: The AF requests to be notified about analytics information of UE’s abnormal behavior.

- CONGESTION: The AF requests to be notified about analytics information of user data congestion information.

- NETWORK_PERFORMANCE: The AF requests to be notified about analytics information of network performance.

- QOS_SUSTAINABILITY: The AF requests to be notified about analytics information of QoS sustainability.

# A.5	5GLANParameterProvision API

openapi: 3.0.0

info:

title: 3gpp-5glan-pp

version: 1.0.0.alpha-2

description: |

API for 5G LAN Parameter Provision.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.522 V16.3.0; 5G System; Network Exposure Function Northbound APIs.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-5glan-pp/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 5.2.4 of 3GPP TS 29.122.

paths:

/{afId}/subscriptions:

get:

summary: read all of the active subscriptions for the AF

tags:

- AF level GET Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

responses:

'200':

description: OK (Successful get all of the active subscriptions for the AF)

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/5GLanParametersProvision'

minItems: 0

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Creates a new subscription resource

tags:

- subscription level POST Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

requestBody:

description: new subscription creation

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/5GLanParametersProvision'

responses:

'201':

description: Created (Successful creation)

content:

application/json:

schema:

$ref: '#/components/schemas/5GLanParametersProvision'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'204':

description: Successful case. The resource has been successfully created and no additional content is to be sent in the response message.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{afId}/subscriptions/{subscriptionId}:

get:

summary: read an active subscription for the AF and the subscription Id

tags:

- Subscription level GET Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the subscription resource

required: true

schema:

type: string

responses:

'200':

description: OK (Successful get the active subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/5GLanParametersProvision'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Updates/replaces an existing subscription resource

tags:

- Subscription level PUT Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the subscription resource

required: true

schema:

type: string

requestBody:

description: Parameters to update/replace the existing subscription

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/5GLanParametersProvision'

responses:

'200':

description: OK (Successful deletion of the existing subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/5GLanParametersProvision'

'204':

description: Successful case. The resource has been successfully updated and no additional content is to be sent in the response message.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Partial updates an existing subscription resource

tags:

- Subscription level PATCH Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the subscription resource

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/5GLanParametersProvisionPatch'

responses:

'200':

description: OK. The subscription was modified successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/5GLanParametersProvision'

'204':

description: Successful case. The resource has been successfully updated and no additional content is to be sent in the response message.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an already existing subscription

tags:

- Subscription level DELETE Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the subscription resource

required: true

schema:

type: string

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

5GLanParametersProvision:

type: object

properties:

self:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Link'

5gLanParams:

$ref: '#/components/schemas/5GLanParameters'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- 5gLanParams

- suppFeat

5GLanParametersProvisionPatch:

type: object

properties:

5gLanParamsPatch:

$ref: '#/components/schemas/5GLanParametersPatch'

5GLanParameters:

type: object

properties:

exterGroupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

gpsis:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minProperties: 1

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

sessionType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

appDesps:

type: object

additionalProperties:

$ref: '#/components/schemas/AppDescriptor'

minProperties: 1

required:

- exterGroupId

- gpsis

- dnn

- snssai

- sessionType

- appDesps

5GLanParametersPatch:

type: object

properties:

gpsis:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minProperties: 1

appDesps:

type: object

additionalProperties:

$ref: '#/components/schemas/AppDescriptor'

minProperties: 1

AppDescriptor:

type: object

properties:

osId:

$ref: 'TS29519_Policy_Data.yaml#/components/schemas/OsId'

appIds:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minProperties: 1

required:

- osId

- appIds

# A.6	ApplyingBdtPolicy API

openapi: 3.0.0

info:

title: 3gpp-applying-bdt-policy

version: 1.0.0.alpha-3

description: |

API for applicying BDT policy

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.522 V16.3.0; 5G System; Network Exposure Function Northbound APIs.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-appliying-bdt-policy/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 5.2.4 of 3GPP TS 29.122.

paths:

/{afId}/subscriptions:

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

get:

summary: read all of the active subscriptions for the AF

tags:

- ApplyingBdtPolicy API AF level GET Operation

responses:

'200':

description: OK.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/AppliedBdtPolicy'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Creates a new subscription resource

tags:

- ApplyingBdtPolicy API Subscription level POST Operation

requestBody:

description: Request to create a new subscription resource

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AppliedBdtPolicy'

responses:

'201':

description: Created (Successful creation of subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/AppliedBdtPolicy'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{afId}/subscriptions/{subscriptionId}:

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the subscription resource

required: true

schema:

type: string

get:

summary: read an active subscriptions for the SCS/AS and the subscription Id

tags:

- ApplyingBdtPolicy API Subscription level GET Operation

responses:

'200':

description: OK (Successful get the active subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/AppliedBdtPolicy'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Updates/replaces an existing subscription resource

tags:

- ApplyingBdtPolicy API subscription level PATCH Operation

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/AppliedBdtPolicyPatch'

responses:

'200':

description: OK. The subscription was modified successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/AppliedBdtPolicy'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an already existing subscription

tags:

- ApplyingBdtPolicy API Subscription level DELETE Operation

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

AppliedBdtPolicy:

type: object

properties:

externalGroupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

bdtRefId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/BdtReferenceId'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- bdtRefId

- suppFeat

oneOf:

- required: [gpsi]

- required: [externalGroupId]

AppliedBdtPolicyPatch:

type: object

properties:

bdtRefId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/BdtReferenceId'

required:

- bdtRefId

# A.7	IPTVConfiguration API

openapi: 3.0.0

info:

title: 3gpp-iptvconfiguration

version: 1.0.0.alpha-2

description: |

API for IPTV configuration.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.522 V16.3.0; 5G System; Network Exposure Function Northbound APIs.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-iptvconfiguration/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 5.2.4 of 3GPP TS 29.122.

paths:

/{afId}/configurations:

get:

summary: read all of the active configurations for the AF

tags:

- AF level GET Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

responses:

'200':

description: OK (Successful get all of the active configurations for the AF)

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/IptvConfigData'

minItems: 0

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Creates a new configuration resource

tags:

- Configuration level POST Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

requestBody:

description: new configuration creation

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/IptvConfigData'

responses:

'201':

description: Created (Successful creation of configuration)

content:

application/json:

schema:

$ref: '#/components/schemas/IptvConfigData'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'204':

description: Successful case. The resource has been successfully created and no additional content is to be sent in the response message.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{afId}/configurations/{configurationId}:

get:

summary: read an active configuration for the AF and the configuration Id

tags:

- Configuration level GET Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: configurationId

in: path

description: Identifier of the configuration resource

required: true

schema:

type: string

responses:

'200':

description: OK (Successful get the active configuration)

content:

application/json:

schema:

$ref: '#/components/schemas/IptvConfigData'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Updates/replaces an existing configuration resource

tags:

- Configuration level PUT Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: configurationId

in: path

description: Identifier of the configuration resource

required: true

schema:

type: string

requestBody:

description: Parameters to update/replace the existing configuration

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/IptvConfigData'

responses:

'200':

description: OK (Successful deletion of the existing configuration)

content:

application/json:

schema:

$ref: '#/components/schemas/IptvConfigData'

'204':

description: Successful case. The resource has been successfully updated and no additional content is to be sent in the response message.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Partial updates an existing configuration resource

tags:

- Configuration level PATCH Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: configurationId

in: path

description: Identifier of the configuration resource

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/IptvConfigDataPatch'

responses:

'200':

description: OK. The configuration was modified successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/IptvConfigData'

'204':

description: Successful case. The resource has been successfully updated and no additional content is to be sent in the response message.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an already existing configuration

tags:

- Configuration level DELETE Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: configurationId

in: path

description: Identifier of the configuration resource

required: true

schema:

type: string

responses:

'204':

description: No Content (Successful deletion of the existing configuration)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

IptvConfigData:

type: object

properties:

self:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Link'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

exterGroupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

afAppId:

type: string

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

multiAccCtrls:

type: object

additionalProperties:

$ref: '#/components/schemas/MulticastAccessControl'

minProperties: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afAppId

- multiAccCtrls

- suppFeat

IptvConfigDataPatch:

type: object

properties:

multiAccCtrls:

type: object

additionalProperties:

$ref: '#/components/schemas/MulticastAccessControl'

minProperties: 1

MulticastAccessControl:

type: object

properties:

srcIpv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

srcIpv6Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

multicastV4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

multicastV6Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

accStatus:

$ref: '#/components/schemas/AccessRightStatus'

required:

- accStatus

AccessRightStatus:

anyOf:

- type: string

enum:

- FULLY_ALLOWED

- PREVIEW_ALLOWED

- NO_ALLOWED

- type: string

description: >

Possible values are

- FULLY_ALLOWED: The User is fully allowed to access to the channel.

- PREVIEW_ALLOWED: The User is preview allowed to access to the channel.

- NO_ALLOWED: The User is not allowed to access to the channel.

# A.8	LpiParameterProvision API

openapi: 3.0.0

info:

title: 3gpp-lpi-pp

version: 1.0.0.alpha-1

description: |

API for Location Privacy Indication Parameters Provisioning.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.522 V16.3.0; 5G System; Network Exposure Function Northbound APIs.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-lpi-pp/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 5.2.4 of 3GPP TS 29.122.

paths:

/{afId}/provisionedLpis:

get:

summary: read all of the active LPI Parameters Provisioning resources for the AF

tags:

- Resource collection level GET Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

responses:

'200':

description: OK (Successful get all of the active resources for the AF)

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/LpiParametersProvision'

minItems: 1

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Creates a new LPI Parameters Provisioning resource

tags:

- Resource collection level POST Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

requestBody:

description: new resource creation

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LpiParametersProvision'

responses:

'201':

description: Created (Successful creation)

content:

application/json:

schema:

$ref: '#/components/schemas/LpiParametersProvision'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{afId}/provisionedLpis/{provisionedLpiId}:

get:

summary: read an active LPI Parameters Provisioning resource for the AF and the provisioned LPI Id

tags:

- Individual resource level GET Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: provisionedLpiId

in: path

description: Identifier of the provisioned LPI parameter resource

required: true

schema:

type: string

responses:

'200':

description: OK (Successful get the active resource)

content:

application/json:

schema:

$ref: '#/components/schemas/LpiParametersProvision'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Updates/replaces an existing LPI Parameters Provisioning resource

tags:

- Individual resource level PUT Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: provisionedLpiId

in: path

description: Identifier of the provisioned LPI parameter resource

required: true

schema:

type: string

requestBody:

description: Parameters to update/replace the existing resource

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LpiParametersProvision'

responses:

'200':

description: OK (Successful update of the existing resource)

content:

application/json:

schema:

$ref: '#/components/schemas/LpiParametersProvision'

'204':

description: Successful case. The resource has been successfully updated and no additional content is sent in the response message.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an already existing LPI Parameters Provisioning resource

tags:

- Individual resource level DELETE Operation

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: provisionedLpiId

in: path

description: Identifier of the provisioned LPI parameter resource

required: true

schema:

type: string

responses:

'204':

description: No Content (Successful deletion of the existing resource)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

LpiParametersProvision:

type: object

properties:

self:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Link'

exterGroupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

lpi:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/Lpi'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- lpi

- suppFeat

# A.9	ServiceParameter API

openapi: 3.0.0

info:

title: 3gpp-service-parameter

version: 1.0.0.alpha-1

description: |

API for AF service paramter

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.522 V16.3.0; 5G System; Network Exposure Function Northbound APIs.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-service-parameter/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 5.2.4 of 3GPP TS 29.122.

paths:

/{afId}/subscriptions:

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

get:

summary: read all of the active subscriptions for the AF

tags:

- ServiceParameter API SCS/AS level GET Operation

responses:

'200':

description: OK.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/ServiceParameterData'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Creates a new subscription resource

tags:

- ServiceParameter API Subscription level POST Operation

requestBody:

description: Request to create a new subscription resource

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

responses:

'201':

description: Created (Successful creation of subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{afId}/subscriptions/{subscriptionId}:

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the subscription resource

required: true

schema:

type: string

get:

summary: read an active subscriptions for the SCS/AS and the subscription Id

tags:

- ServiceParameter API Subscription level GET Operation

responses:

'200':

description: OK (Successful get the active subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Updates/replaces an existing subscription resource

tags:

- TrafficInfluence API subscription level PUT Operation

requestBody:

description: Parameters to update/replace the existing subscription

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

responses:

'200':

description: OK (Successful update of the subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Updates/replaces an existing subscription resource

tags:

- ServiceParameter API subscription level PATCH Operation

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/ServiceParameterDataPatch'

responses:

'200':

description: OK. The subscription was modified successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an already existing subscription

tags:

- ServiceParameter API Subscription level DELETE Operation

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ServiceParameterData:

type: object

properties:

afServiceId:

type: string

description: Identifies a service on behalf of which the AF is issuing the request.

appId:

type: string

description: Identifies an application.

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

externalGroupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

anyUeInd:

type: boolean

description: Identifies whether the AF request applies to any UE. This attribute shall set to "true" if applicable for any UE, otherwise, set to "false".

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

ueIpv4:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

ueIpv6:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

ueMac:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

self:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Link'

paramOverPc5:

$ref: '#/components/schemas/ParameterOverPc5'

paramOverUu:

$ref: '#/components/schemas/ParameterOverUu'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

ServiceParameterDataPatch:

type: object

properties:

paramOverPc5:

$ref: '#/components/schemas/ParameterOverPc5Rm'

ParamOverUu:

$ref: '#/components/schemas/ParameterOverUuRm'

ParameterOverPc5:

type: object

properties:

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

plmmRatServed:

type: array

items:

$ref: '#/components/schemas/PlmnRatServed'

minItems: 1

authNotServed:

type: boolean

radioParamsNotServed:

type: array

items:

$ref: '#/components/schemas/RadioParameterNotServed'

minItems: 1

serToTx:

type: array

items:

$ref: '#/components/schemas/ServiceToTx'

minItems: 1

privacyParams:

type: array

items:

$ref: '#/components/schemas/PrivacyParameter'

minItems: 1

configParaEutra:

$ref: '#/components/schemas/ConfigurationParametersEutra'

configParaNr:

$ref: '#/components/schemas/ConfigurationParametersNr'

ParameterOverUu:

type: object

properties:

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

serToPduSess:

type: array

items:

$ref: '#/components/schemas/ServiceToPduSession'

minItems: 1

serToAppAddr:

type: array

items:

$ref: '#/components/schemas/ServiceApplicationServerAddress'

minItems: 1

PlmnRatServed:

type: object

properties:

plmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

rats:

type: array

items:

$ref: '#/components/schemas/V2xRatType'

required:

- plmn

- rats

RadioParameterNotServed:

type: object

properties:

radioParams:

type: string

rat:

$ref: '#/components/schemas/V2xRatType'

geographicalArea:

type: string

operManaged:

$ref: '#/components/schemas/OperatorManaged'

required:

- radioParams

- geographicalArea

- operManaged

- rat

ServiceToTx:

type: object

properties:

serIds:

type: array

items:

type: string

minItems: 1

txProfile:

type: string

rat:

$ref: '#/components/schemas/V2xRatType'

required:

- serIds

- txProfile

- rat

PrivacyParameter:

type: object

properties:

serIds:

type: array

items:

type: string

minItems: 1

geographicalArea:

type: string

timer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

required:

- serIds

- geographicalArea

- timer

ConfigurationParametersEutra:

type: object

properties:

serToLayer2Ids:

type: array

items:

$ref: '#/components/schemas/ServiceIdToLayer2Id'

minItems: 1

ppppToPdbs:

type: array

items:

$ref: '#/components/schemas/PpppToPdb'

minItems: 1

serIdToFrequs:

type: array

items:

$ref: '#/components/schemas/ServiceIdToFrequency'

minItems: 1

serIdToPpprs:

type: array

items:

$ref: '#/components/schemas/ServiceIdToPppr'

minItems: 1

ConfigurationParametersNr:

type: object

properties:

serToBroLayer2Ids:

type: array

items:

$ref: '#/components/schemas/ServiceIdToLayer2Id'

minItems: 1

serToGroLayer2Ids:

type: array

items:

$ref: '#/components/schemas/ServiceIdToLayer2Id'

minItems: 1

serToDefLayer2Ids:

type: array

items:

$ref: '#/components/schemas/ServiceIdToLayer2Id'

minItems: 1

serIdToFrequs:

type: array

items:

$ref: '#/components/schemas/ServiceIdToFrequency'

minItems: 1

pc5QosMappings:

type: array

items:

$ref: '#/components/schemas/Pc5QosMapping'

minItems: 1

slrbConfigs:

type: array

items:

$ref: '#/components/schemas/SlrbConfigurations'

minItems: 1

ServiceIdToLayer2Id:

type: object

properties:

serIds:

type: array

items:

type: string

minItems: 1

desLayer2Id:

type: string

required:

- serIds

- desLayer2Id

PpppToPdb:

type: object

properties:

pppp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

pdb:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketDelBudget'

required:

- pppp

- pdb

ServiceIdToFrequency:

type: object

properties:

serIds:

type: array

items:

type: string

minItems: 1

frequency:

type: number

geographicalArea:

type: string

required:

- serIds

- frequency

- geopraphicalArea

ServiceIdToPppr:

type: object

properties:

serIds:

type: array

items:

type: string

minItems: 1

pppr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- serIds

- pppr

Pc5QosMapping:

type: object

properties:

serIds:

type: array

items:

type: string

minItems: 1

pc5QoSFlowParam:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pc5QosFlowItem'

required:

- serIds

- pc5QosFlowParam

SlrbConfigurations:

type: object

properties:

pc5QosPara:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pc5QoSPara'

slrb:

type: string

required:

- pc5QosPara

- slrb

ServiceToPduSession:

type: object

properties:

serIds:

type: array

items:

type: string

minItems: 1

pduSessionType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

sscMode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SscMode'

sliceInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

dnns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

required:

- serIds

ServiceApplicationServerAddress:

type: object

properties:

serIds:

type: array

items:

type: string

minItems: 1

fqdn:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/Fqdn'

Ipv4Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

Ipv6Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

udpPortNumbers:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minItems: 1

tcpPortNumbers:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minItems: 1

plmns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

geopraphicalArea:

type: string

required:

- serIds

- plmns

- geographicalArea

ParameterOverPc5Rm:

type: string

# Editor's note: Type is FFS. String only used to pass Syntax check.

ParameterOverUuRm:

type: string

# Editor's note: Type is FFS. String only used to pass Syntax check.

OperatorManaged:

anyOf:

- type: string

enum:

- OPERATOR_MANAGED

- NON_OPERATOR_MANAGED

- type: string

description: >

Possible values are

- OPERATOR_MANAGED: The radio parameters are "operator managed".

- NON_OPERATOR_MANAGED: The radio parameters are "non-operator managed".

V2xRatType:

anyOf:

- type: string

enum:

- PC5_LTE

- PC5_NR

- type: string

description: >

Possible values are

- PC5_LTE: Indicates the LTE RAT type over PC5 interface.

- PC5_NR: Indicates the NR RAT type over NR interface.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc. | CR | Rev | Cat | Subject/Comment | New |
| 2018-03 | CT3#95 |  |  |  |  | TS Skeleton | 0.0.0 |
| 2018-03 | CT3#95 |  |  |  |  | Inclusion of C3-181332 and TS skeleton of Network Exposure Function Northbound APIs in C3-181362. | 0.1.0 |
| 2018-04 | CT3#96 |  |  |  |  | Inclusion of C3-182407, C3-182408, C3-182504, C3-182418, C3-182505, C3-182443, C3-182421, C3-182422, C3-182501 and editorial changes from Rapporteur. | 0.2.0 |
| 2018-05 | CT3#97 |  |  |  |  | Inclusion of C3-183187, C3-183773, C3-183774, C3-183553, C3-183826, C3-183329, C3-183776, C3-183827, C3-183778, C3-183605 and editorial changes from Rapporteur. | 0.3.0 |
| 2018-06 | CT#80 |  |  |  |  | TS sent to plenary for approval | 1.0.0 |
| 2018-06 | CT#80 |  |  |  |  | TS approved by plenary | 15.0.0 |
| 2018-09 | CT#81 | CP-182015 | 0001 | 1 | F | DNAI change notification type | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0002 | - | F | Corrections on NEF Northbound interface | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0003 | 1 | F | TrafficInfluence API OpenAPI schema | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0004 | 1 | F | AF influence traffic routing cleanup | 15.1.0 |
| 2018-09 | CT#81 | CP-182031 | 0005 | 1 | F | Definition of Changing the Chargeable Party procedures and API | 15.1.0 |
| 2018-09 | CT#81 | CP-182031 | 0006 | 1 | F | Definition of setting up an AS session with required QoS procedure and API | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0007 | 2 | F | Resource structure update | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0008 | - | F | Procedures for monitoring – Reference | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0009 | - | F | Ethernet packet filter for AF traffic influence API | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0010 | 3 | F | Removable attribute definition for AF traffic influence | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0011 | - | F | Supported feature for AF traffic influence | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0012 | - | F | Version numbering change | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0013 | - | F | Removal of externaldocs field | 15.1.0 |
| 2018-09 | CT#81 | CP-182035 | 0014 | 1 | F | PFD Management Service Operation | 15.1.0 |
| 2018-12 | CT#82 | CP-183205 | 0015 | 2 | F | ExternalDocs field | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0019 | - | F | Default value for apiRoot | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0021 | 4 | F | Correct traffic route and Ethernet flow data type | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0022 | 1 | F | Event correction for AF influence traffic routing | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0024 | 1 | F | Supporting Ethernet UE in Chargeable Party and AF session with QoS | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0025 | 1 | F | Add AF application ID for traffic influence | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0026 | 1 | F | Add BSF interaction for Chargeable Party and Required QoS | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0028 | 2 | F | Security field | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0029 | 1 | F | Corrections on subscribed event | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0030 | 1 | F | Status code update for TrafficInfluence API | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0031 | 3 | F | UE information during notification | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0017 | 2 | F | Error status codes for HTTP response | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0016 | 3 | F | Support of 5G location requirement | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0023 | 2 | F | Correction to the AF influence traffic steering control | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0032 | - | F | Location header | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0033 | 1 | F | API Version Update | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0034 | 1 | F | Support of 5G SUPI-PEI association | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0035 | 1 | F | Clarification of default value for boolean data type | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0027 | 2 | F | Security adaptation for Nnef northbound APIs with CAPIF | 15.2.0 |
| 2019-03 | CT#83 | CP-190116 | 0037 | 2 | F | Event notification | 15.3.0 |
| 2019-03 | CT#83 | CP-190116 | 0038 | 1 | F | Correction on MacAddr48 and RouteToLocation data type reference in the OpenAPI file | 15.3.0 |
| 2019-03 | CT#83 | CP-190116 | 0040 | 1 | F | Correction on mandatory 5G features | 15.3.0 |
| 2019-03 | CT#83 | CP-190116 | 0041 | - | F | OpenAPI Version number update | 15.3.0 |
| 2019-06 | CT#84 | CP-191080 | 0042 | 4 | F | Resource structure and AF Identifier | 15.4.0 |
| 2019-06 | CT#84 | CP-191080 | 0048 | 2 | F | UDM interaction for AF influence traffic | 15.4.0 |
| 2019-06 | CT#84 | CP-191080 | 0049 | 2 | F | Correct condition for DNAI in UP path change | 15.4.0 |
| 2019-06 | CT#84 | CP-191080 | 0053 | 1 | F | Precedence of OpenAPI file | 15.4.0 |
| 2019-06 | CT#84 | CP-191080 | 0059 | 1 | F | Copyright Note in YAML file | 15.4.0 |
| 2019-06 | CT#84 | CP-191090 | 0047 | 1 | B | Support of external group Id | 16.0.0 |
| 2019-06 | CT#84 | CP-191070 | 0043 | 2 | B | Nnef_MSISDN-less_MO_SMS service | 16.0.0 |
| 2019-06 | CT#84 | CP-191070 | 0044 | 2 | B | Application function notification of downlink data delivery status | 16.0.0 |
| 2019-06 | CT#84 | CP-191070 | 0045 | 2 | B | Availability after DDN failure notification for multiple Afs | 16.0.0 |
| 2019-06 | CT#84 | CP-191070 | 0050 | 2 | B | Network parameter provisioning support | 16.0.0 |
| 2019-06 | CT#84 | CP-191070 | 0051 | 3 | B | NIDD configuration and delivery in 5G | 16.0.0 |
| 2019-06 | CT#84 | CP-191229 | 0054 | 5 | B | AF acknowledgement of UP path event notification | 16.0.0 |
| 2019-06 | CT#84 | CP-191071 | 0055 | 2 | B | UE IP address preservation indication | 16.0.0 |
| 2019-06 | CT#84 | CP-191104 | 0056 | 1 | B | PFD management notification | 16.0.0 |
| 2019-06 | CT#84 | CP-191100 | 0057 | 1 | B | NEF stored exposure data | 16.0.0 |
| 2019-06 | CT#84 | CP-191105 | 0058 | 1 | B | BDT Warning Notification Support | 16.0.0 |
| 2019-06 | CT#84 | CP-191101 | 0061 | 1 | F | API version update | 16.0.0 |
| 2019-09 | CT#85 | CP-192137 | 0063 | 1 | F | Resolving EN in NIDD | 16.1.0 |
| 2019-09 | CT#85 | CP-192156 | 0064 | 1 | B | Support a set of MAC addresses in traffic filter | 16.1.0 |
| 2019-09 | CT#85 | CP-192165 | 0066 | 1 | B | Support parameter provisioning in RACS | 16.1.0 |
| 2019-09 | CT#85 | CP-192157 | 0067 | 2 | B | Accurate UE moving trajectory definition | 16.1.0 |
| 2019-09 | CT#85 | CP-192157 | 0069 | 2 | B | Procedures for Nnef_AnalyticsExposure Service | 16.1.0 |
| 2019-09 | CT#85 | CP-192157 | 0070 | 2 | B | API definition for Nnef_AnalyticsExposure Service | 16.1.0 |
| 2019-09 | CT#85 | CP-192170 | 0071 | 1 | B | Procedures for 5G LAN type sevice over northbound interface | 16.1.0 |
| 2019-09 | CT#85 | CP-192170 | 0072 | 2 | B | API definition for 5G LAN type service over northbound interface | 16.1.0 |
| 2019-09 | CT#85 | CP-192169 | 0073 | 2 | B | PFD management partial failure | 16.1.0 |
| 2019-09 | CT#85 | CP-192157 | 0074 | 1 | B | Cancel the BDT warning notification | 16.1.0 |
| 2019-09 | CT#85 | CP-192219 | 0075 | 2 | B | Notification of downlink data delivery status | 16.1.0 |
| 2019-09 | CT#85 | CP-192179 | 0076 | 2 | B | Applying BDT policy | 16.1.0 |
| 2019-09 | CT#85 | CP-192152 | 0077 | 2 | B | API definition for Nnef_IPTVconfiguration service | 16.1.0 |
| 2019-09 | CT#85 | CP-192137 | 0079 | - | B | Nnef_ECRestriction service | 16.1.0 |
| 2019-09 | CT#85 | CP-192137 | 0080 | - | B | Differences betwwen EPC and 5GC | 16.1.0 |
| 2019-09 | CT#85 | CP-192158 | 0081 | 1 | F | Service consumer description Corrections | 16.1.0 |
| 2019-09 | CT#85 | CP-192138 | 0082 | 2 | B | AF acknowledgement of UP path event notification | 16.1.0 |
| 2019-09 | CT#85 | CP-192138 | 0083 | - | B | Successul AF acknowledgement without N6 traffic routing information | 16.1.0 |
| 2019-09 | CT#85 | CP-192173 | 0084 | - | F | OpenAPI version update for TS 29.522 Rel-16 | 16.1.0 |
| 2019-09 | CT#85 | CP-192251 | 0085 | 1 | B | Procedures for Nnef_IPTVconfiguration service | 16.1.0 |
| 2019-12 | CT#86 | CP-193179 | 0086 | 1 | B | Nnef_APISupportCapability Service | 16.2.0 |
| 2019-12 | CT#86 | CP-193181 | 0087 | - | B | OpenAPI file update to support AF acknowledgement | 16.2.0 |
| 2019-12 | CT#86 | CP-193179 | 0088 | 1 | B | Scheduled communication type | 16.2.0 |
| 2019-12 | CT#86 | CP-193181 | 0089 | 1 | F | Open issue for AddrPreservation feature | 16.2.0 |
| 2019-12 | CT#86 | CP-193222 | 0090 | 1 | B | Partial update for 5GLANParameterProvision API | 16.2.0 |
| 2019-12 | CT#86 | CP-193222 | 0091 | 2 | B | OpenAPI file for 5GLANParameterProvision API | 16.2.0 |
| 2019-12 | CT#86 | CP-193191 | 0092 | 3 | F | Clarify multicast access control | 16.2.0 |
| 2019-12 | CT#86 | CP-193222 | 0093 | 1 | F | Clarify the procedure for 5GLAN parameter provisioning | 16.2.0 |
| 2019-12 | CT#86 | CP-193223 | 0094 | - | F | Correct resource URI for xBDT | 16.2.0 |
| 2019-12 | CT#86 | CP-193220 | 0096 | 3 | B | PFD partial failure notification | 16.2.0 |
| 2019-12 | CT#86 | CP-193223 | 0097 | 1 | F | Correction to HTTP methods used to update BDT policy | 16.2.0 |
| 2019-12 | CT#86 | CP-193191 | 0099 | 1 | F | Partial update of IPTVConfiguration API | 16.2.0 |
| 2019-12 | CT#86 | CP-193191 | 0100 | 2 | B | OpenAPI file of IPTVConfiguration API | 16.2.0 |
| 2019-12 | CT#86 | CP-193198 | 0101 | 3 | B | AnalyticsEventNotif and AnalyticsExposureSubsc Data types | 16.2.0 |
| 2019-12 | CT#86 | CP-193198 | 0102 | - | B | Open issue for AnalyticsEvent data type | 16.2.0 |
| 2019-12 | CT#86 | CP-193198 | 0103 | 1 | B | Partial update of Nnef_AnalyticsExposure API | 16.2.0 |
| 2019-12 | CT#86 | CP-193198 | 0104 | 2 | B | Nnef_AnalyticsExposure_fetch service operation | 16.2.0 |
| 2019-12 | CT#86 | CP-193181 | 0105 | - | F | Correct the condition for AF relocation acknowledgement | 16.2.0 |
| 2019-12 | CT#86 | CP-193199 | 0106 | - | B | URI structure for N33 APIs | 16.2.0 |
| 2019-12 | CT#86 | CP-193198 | 0107 | - | B | OpenAPI file for AnalyticsExposure API | 16.2.0 |
| 2019-12 | CT#86 | CP-193222 | 0108 | 1 | D | Corrections on 5GLANParameterProvision API | 16.2.0 |
| 2019-12 | CT#86 | CP-193181 | 0109 | - | F | Definition of AfResultInfo in OpenAPI | 16.2.0 |
| 2019-12 | CT#86 | CP-193212 | 0110 | 1 | F | Update of API version and TS version in OpenAPI file | 16.2.0 |
| 2019-12 | CT#86 | CP-193188 | 0112 | 1 | A | make the storage of traffic influence request in the UDR mandatory | 16.2.0 |
| 2019-12 | CT#86 | CP-193223 | 0113 | 1 | F | missing required in ApplyingBdtPolicy API file | 16.2.0 |
| 2019-12 | CT#86 | CP-193188 | 0115 | - | A | Correct cardinality in traffic influence | 16.2.0 |
| 2019-12 | CT#86 | CP-193198 | 0116 | 1 | F | Feature name correction for BDT notification | 16.2.0 |
| 2020-03 | CT#87e | CP-200207 | 0118 | - | B | DNN Clarification | 16.3.0 |
| 2020-03 | CT#87e | CP-200198 | 0119 | 1 | B | Update of the Availability after DDN Failure event | 16.3.0 |
| 2020-03 | CT#87e | CP-200198 | 0120 | 1 | B | Update of the DDD status event | 16.3.0 |
| 2020-03 | CT#87e | CP-200212 | 0122 | 1 | B | Procedure of Nnef_ServiceParameter service | 16.3.0 |
| 2020-03 | CT#87e | CP-200212 | 0123 | 1 | B | Resources and data types of Nnef_ServiceParameter service | 16.3.0 |
| 2020-03 | CT#87e | CP-200266 | 0124 | 3 | B | OpenAPI file of Nnef_ServiceParameter service | 16.3.0 |
| 2020-03 | CT#87e | CP-200202 | 0125 | 1 | B | QoS Monitoring Report | 16.3.0 |
| 2020-03 | CT#87e | CP-200218 | 0126 | 1 | B | Indication of traffic correlation | 16.3.0 |
| 2020-03 | CT#87e | CP-200203 | 0127 | 1 | B | Clarification of IPTV configuration | 16.3.0 |
| 2020-03 | CT#87e | CP-200198 | 0128 | - | F | Correct TS number for NEF southbound NIDD service | 16.3.0 |
| 2020-03 | CT#87e | CP-200198 | 0129 | - | B | Support PDU session status | 16.3.0 |
| 2020-03 | CT#87e | CP-200137 | 0130 | 2 | F | Correct UE mobility and communication | 16.3.0 |
| 2020-03 | CT#87e | CP-200208 | 0131 | 1 | B | Support network performance analytics | 16.3.0 |
| 2020-03 | CT#87e | CP-200208 | 0132 | 1 | B | Support BDT policy candidates in notification | 16.3.0 |
| 2020-03 | CT#87e | CP-200212 | 0133 | 1 | B | Add alternative QoS requirements | 16.3.0 |
| 2020-03 | CT#87e | CP-200142 | 0134 | 2 | B | Support QoS sustainability analytics | 16.3.0 |
| 2020-03 | CT#87e | CP-200218 | 0135 | - | F | Definition of 5GLanParametersProvision | 16.3.0 |
| 2020-03 | CT#87e | CP-200203 | 0136 | - | F | Definition of IptvConfigData | 16.3.0 |
| 2020-03 | CT#87e | CP-200219 | 0137 | - | F | Usage of the "bdtRefId" property | 16.3.0 |
| 2020-03 | CT#87e | CP-200215 | 0138 | - | F | Miscellaneous errors | 16.3.0 |
| 2020-03 | CT#87e | CP-200259 | 0140 | 3 | B | UE Location Privacy Setting in NEF | 16.3.0 |
| 2020-03 | CT#87e | CP-200237 | 0142 | 2 | B | AnalyticsExposure API, Analytics Event Filter associated with all events | 16.3.0 |
| 2020-03 | CT#87e | CP-200208 | 0143 | 1 | B | AnalyticsExposure API, support of abnormal behaviour | 16.3.0 |
| 2020-03 | CT#87e | CP-200208 | 0144 | 1 | B | AnalyticsExposure API, support of data congestion | 16.3.0 |
| 2020-03 | CT#87e | CP-200216 | 0145 | - | F | Update of OpenAPI version and TS version in externalDocs field | 16.3.0 |