| 3GPP TS 29.549 V1.1.0 (2020-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage 3 (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	9

1	Scope	11

2	References	11

3	Definitions of terms and abbreviations	12

3.1	Terms	12

3.2	Abbreviations	12

4	Overview	12

5	Services offered by the SEAL servers	13

5.1	Introduction of SEAL services	13

5.2	Location management APIs	14

5.2.1	SS_Location_Reporting_Trigger API	14

5.2.1.1	Service Description	14

5.2.1.1.1	Overview	14

5.2.1.2	Service Operations	14

5.2.1.2.1	Introduction	14

5.2.1.2.2	Location_Reporting_Trigger	14

5.2.1.2.2.1	General	14

5.2.1.2.2.2	<Description> Location_Reporting_Trigger service operation	14

5.2.2	SS_Location_Info_Subscription API	14

5.2.2.1	Service Description	14

5.2.2.1.1	Overview	14

5.2.2.2	Service Operations	14

5.2.2.2.1	Introduction	14

5.2.2.2.2	Location_Info_Subscribe_Event	15

5.2.2.2.2.1	General	15

5.2.2.2.2.2	<Description> Location_Info_Subscribe_Event service operation	15

5.2.2.2.3	Location_Info_Notify_Event	15

5.2.2.2.3.1	General	15

5.2.2.2.3.2	<Description> Location_Info_Notify_Event service operation	15

5.2.3	SS_Obtain_Location_Info API	15

5.2.3.1	Service Description	15

5.2.3.1.1	Overview	15

5.2.3.2	Service Operations	15

5.2.3.2.1	Introduction	15

5.2.3.2.2	Obtain_Location_Info	15

5.2.3.2.2.1	General	15

5.2.3.2.2.2	<Description> Obtain_Location_Info service operation	15

5.3	Group management APIs	15

5.3.1	SS_GroupManagement API	15

5.3.1.1	Service Description	15

5.3.1.1.1	Overview	15

5.3.1.2	Service Operations	16

5.3.1.2.1	Introduction	16

5.3.1.2.2	Query_Group_Info	16

5.3.1.2.2.1	General	16

5.3.1.2.2.2	 VAL server fetching group membership and configuration information using Query_Group_Info service operation	16

5.3.1.2.3	Update_Group_Info	16

5.3.1.2.3.1	General	16

5.3.1.2.3.2	VAL server modifying group membership and configuration using Update_Group_Info service operation	16

5.3.1.2.4	Create_Group	17

5.3.1.2.4.1	General	17

5.3.1.2.4.2	VAL server creating new group using Create_Group service operation	17

5.3.1.2.5	Delete_Group	17

5.3.1.2.5.1	General	17

5.3.1.2.5.2	VAL server deleting VAL group using Delete_Group service operation	17

5.3.2	SS_GroupManagementEvent API	17

5.4	Configuration management APIs	18

5.4.1	SS_UserProfileRetrieval API	18

5.4.1.1	Service Description	18

5.4.1.1.1	Overview	18

5.4.1.2	Service Operations	18

5.4.1.2.1	Introduction	18

5.4.1.2.2	Obtain_User_Profile	18

5.4.1.2.2.1	General	18

5.4.1.2.2.2	 VAL server retrieving VAL user profile information using Obtain_User_Profile service operation	18

5.4.2	SS_ UserProfileEvent API	18

5.5	Network resource management APIs	19

5.5.1	SS_Network_Resource_Adaptation API	19

5.5.1.1	Service Description	19

5.5.1.1.1	Overview	19

5.5.1.2	Service Operations	19

5.5.1.2.1	Introduction	19

5.5.1.2.2	Network_Resource_Adapation	19

5.5.1.2.2.1	General	19

5.5.1.2.2.2	 <Description> Network_Resource_Adaptation service operation	19

5.6	Events APIs	19

5.6.1	SS_Events API	19

5.6.1.1	Service Description	19

5.6.1.1.1	Overview	19

5.6.1.2	Service Operations	19

5.6.1.2.1	Introduction	19

5.6.1.2.2	Subscribe_Event	20

5.6.1.2.2.1	General	20

5.6.1.2.2.1	Subscribing to SEAL events using Subscribe_Event service operation	20

5.6.1.2.3	Notify_Event	20

5.6.1.2.3.1	General	20

5.6.1.2.3.2	Notifying SEAL events using Notify_Event service operation	20

5.6.1.2.4	Unsubscribe_Event	20

5.6.1.2.4.1	General	20

5.6.1.2.4.2	Unsubscribing from SEAL events using Unsubscribe_Event service operation	21

5.7	Identity management APIs	21

5.8	Key management APIs	21

6	SEAL Design Aspects Common for All APIs	21

6.1	General	21

6.2	Data Types	21

6.2.1	General	21

6.2.2	Referenced structured data types	22

6.2.3	Referenced Simple data types and enumerations	22

6.3	Usage of HTTP	22

6.4	Content type	22

6.5	URI structure	22

6.6	Notifications	23

6.7	Error Handling	23

6.8	Feature negotiation	24

6.9	HTTP headers	24

6.10	Conventions for Open API specification files	24

7	SEAL API Definitions	24

7.1	Location management APIs	24

7.1.1	SS_Location_Reporting_Trigger API	24

7.1.1.1	API URI	24

7.1.1.2	Resources	24

7.1.1.2.1	Overview	24

7.1.1.2.2	Resource: <Resource name>	25

7.1.1.2.2.1	Description	25

7.1.1.2.2.2	Resource Definition	25

7.1.1.2.2.3	Resource Standard Methods	25

7.1.1.2.2.3.1	<Method Name>	25

7.1.1.2.2.4	 Resource Custom Operations	25

7.1.1.3	Notifications	25

7.1.1.3.1	General	25

7.1.1.3.2	<Notification name>	25

7.1.1.3.2.1	Description	25

7.1.1.3.2.2	Notification definition	25

7.1.1.4	Data Model	26

7.1.1.4.1	General	26

7.1.1.4.2	Structured data types	26

7.1.1.4.2.1	Introduction	26

7.1.1.4.2.2	Type: <Data type name>	26

7.1.1.4.3	Simple data types and enumerations	27

7.1.1.5	Error Handling	27

7.1.1.6	Feature negotiation	27

7.1.2	SS_Location_Info_Subscription API	27

7.1.2.1	API URI	27

7.1.2.2	Resources	27

7.1.2.2.1	Overview	27

7.1.2.2.2	Resource: <Resource name>	28

7.1.2.2.2.1	Description	28

7.1.2.2.2.2	Resource Definition	28

7.1.2.2.2.3	Resource Standard Methods	28

7.1.2.2.2.3.1	<Method Name>	28

7.1.2.2.2.4	 Resource Custom Operations	28

7.1.2.3	Notifications	28

7.1.2.3.1	General	28

7.1.2.3.2	<Notification name>	28

7.1.2.3.2.1	Description	28

7.1.2.3.2.2	Notification definition	28

7.1.2.4	Data Model	29

7.1.2.4.1	General	29

7.1.2.4.2	Structured data types	29

7.1.2.4.2.1	Introduction	29

7.1.2.4.2.2	Type: <Data type name>	29

7.1.2.4.3	Simple data types and enumerations	30

7.1.2.5	Error Handling	30

7.1.2.6	Feature negotiation	30

7.1.3	SS_Obtain_Location_Info API	30

7.1.3.1	API URI	30

7.1.3.2	Resources	30

7.1.3.2.1	Overview	30

7.1.3.2.2	Resource: <Resource name>	31

7.1.3.2.2.1	Description	31

7.1.3.2.2.2	Resource Definition	31

7.1.3.2.2.3	Resource Standard Methods	31

7.1.3.2.2.3.1	<Method Name>	31

7.1.3.2.2.4	 Resource Custom Operations	31

7.1.3.3	Notifications	31

7.1.3.3.1	General	31

7.1.3.3.2	<Notification name>	31

7.1.3.3.2.1	Description	31

7.1.3.3.2.2	Notification definition	31

7.1.3.4	Data Model	32

7.1.3.4.1	General	32

7.1.3.4.2	Structured data types	32

7.1.3.4.2.1	Introduction	32

7.1.3.4.2.2	Type: <Data type name>	32

7.1.3.4.3	Simple data types and enumerations	33

7.1.3.5	Error Handling	33

7.1.3.6	Feature negotiation	33

7.2	Group management APIs	33

7.2.1	SS_GroupManagement API	33

7.2.1.1	API URI	33

7.2.1.2	Resources	33

7.2.1.2.1	Overview	33

7.2.1.2.2	Resource: All VAL Group Documents	34

7.2.1.2.2.1	Description	34

7.2.1.2.2.2	Resource Definition	34

7.2.1.2.2.3	Resource Standard Methods	34

7.2.1.2.2.3.1	POST	34

7.2.1.2.2.4	 Resource Custom Operations	35

7.2.1.2.3	Resource: Individual VAL Group Document	35

7.2.1.2.3.1	Description	35

7.2.1.2.3.2	Resource Definition	35

7.2.1.2.3.3	Resource Standard Methods	35

7.2.1.2.3.3.1	GET	35

7.2.1.2.3.3.2	PUT	36

7.2.1.2.3.3.3	DELETE	36

7.2.1.2.3.4	Resource Custom Operations	37

7.2.1.3	Notifications	37

7.2.1.4	Data Model	37

7.2.1.4.1	General	37

7.2.1.4.2	Structured data types	38

7.2.1.4.2.1	Introduction	38

7.2.1.4.2.2	Type: VALGroupDocument	38

7.2.1.4.3	Simple data types and enumerations	38

7.2.1.5	Error Handling	38

7.2.1.6	Feature negotiation	38

7.3	Configuration management APIs	38

7.3.1	SS_ UserProfileRetrieval API	38

7.3.1.1	API URI	38

7.3.1.2	Resources	39

7.3.1.2.1	Overview	39

7.3.1.2.2	Resource: Individual VAL Service	39

7.3.1.2.2.1	Description	39

7.3.1.2.2.2	Resource Definition	39

7.3.1.2.2.3	Resource Standard Methods	40

7.3.1.2.2.3.1	GET	40

7.3.1.2.2.4	 Resource Custom Operations	40

7.3.1.3	Notifications	40

7.3.1.4	Data Model	40

7.3.1.4.1	General	40

7.3.1.4.2	Structured data types	41

7.3.1.4.2.1	Introduction	41

7.3.1.4.2.2	Type: ProfileDoc	41

7.3.1.4.3	Simple data types and enumerations	41

7.3.1.5	Error Handling	41

7.3.1.6	Feature negotiation	41

7.4	Network resource management APIs	42

7.4.1	SS_Network_Resource_Adaptation API	42

7.4.1.1	API URI	42

7.4.1.2	Resources	42

7.4.1.2.1	Overview	42

7.4.1.2.2	Resource: Multicast Subscriptions	42

7.4.1.2.2.1	Description	42

7.4.1.2.2.2	Resource Definition	42

7.4.1.2.2.3	Resource Standard Methods	43

7.4.1.2.2.3.1	POST	43

7.4.1.2.2.4	 Resource Custom Operations	43

7.4.1.2.3	Resource: Individual Multicast Subscription	43

7.4.1.2.3.1	Description	43

7.4.1.2.3.2	Resource Definition	43

7.4.1.2.3.3	Resource Standard Methods	43

7.4.1.2.3.3.1	GET	43

7.4.1.2.3.3.2	DELETE	44

7.4.1.2.3.4	 Resource Custom Operations	44

7.4.1.3	Notifications	44

7.4.1.3.1	General	44

7.4.1.3.2	User Plane notification	45

7.4.1.3.2.1	Description	45

7.4.1.3.2.2	Notification definition	45

7.4.1.4	Data Model	45

7.4.1.4.1	General	45

7.4.1.4.2	Structured data types	46

7.4.1.4.2.1	Introduction	46

7.4.1.4.2.2	Type:  MulticastSubscription	46

7.4.1.4.2.3	Type: UserPlaneNotification	47

7.4.1.4.2.4	Type: NrmEventNotification	47

7.4.1.4.3	Simple data types and enumerations	47

7.4.1.4.3.1	Enumeration: ServiceAnnoucementMode	47

7.4.1.4.3.2	Enumeration: DeliveryMode	47

7.4.1.4.3.3	Enumeration: NrmEvent	47

7.4.1.5	Error Handling	48

7.4.1.6	Feature negotiation	48

7.5	Event APIs	48

7.5.1	SS_Events API	48

7.5.1.1	API URI	48

7.5.1.2	Resources	48

7.5.1.2.1	Overview	48

7.5.1.2.2	Resource: SEAL Events Subscriptions	49

7.5.1.2.2.1	Description	49

7.5.1.2.2.2	Resource Definition	49

7.5.1.2.2.3	Resource Standard Methods	49

7.5.1.2.2.3.1	POST	49

7.5.1.2.2.4	Resource Custom Operations	50

7.5.1.2.3	Resource: Individual SEAL Events Subscription	50

7.5.1.2.3.1	Description	50

7.5.1.2.3.2	Resource Definition	50

7.5.1.2.3.3	Resource Standard Methods	50

7.5.1.2.3.3.1	DELETE	50

7.5.1.2.3.4	Resource Custom Operations	51

7.5.1.3	Notifications	51

7.5.1.3.1	General	51

7.5.1.3.2	SEAL Event Notification	51

7.5.1.3.2.1	Description	51

7.5.1.3.2.2	Notification definition	51

7.5.1.4	Data Model	52

7.5.1.4.1	General	52

7.5.1.4.2	Structured data types	53

7.5.1.4.2.1	Introduction	53

7.5.1.4.2.2	SEALEventSubscription	53

7.5.1.4.2.3	SEALEventNotification	54

7.5.1.4.2.4	EventSubscription	54

7.5.1.4.2.5	SEALEventDetail	54

7.5.1.4.2.6	VALGroupFilter	55

7.5.1.4.2.7	IdentityFilter	55

7.5.1.4.3	Simple data types and enumerations	55

7.5.1.4.3.1	Introduction	55

7.5.1.4.3.2	Simple data types	55

7.5.1.4.3.3	Enumeration: SEALEvent	55

7.5.1.5	Error Handling	55

7.5.1.6	Feature Negotiation	56

7.6	Identity management APIs	56

7.7	Key management APIs	56

8	Using Common API Framework	56

8.1	General	56

8.2	Security	56

Annex A (normative):	OpenAPI specification	58

A.1	General	58

A.2	SS_GroupManagement API	58

A.3	SS_UserProfileRetrieval API	61

A.4	SS_NetworkResourceAdaptation API	62

A.5	SS_Events API	66

Annex B (informative):	Change history	70

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present specification describes the APIs for the Service Enabler Architecture Layer for Verticals (SEAL). The SEAL and related stage 2 architecture, functional requirements and information flows are specified in 3GPP TS 23.434 [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[4]	IETF RFC 6455: "The Websocket Protocol".

[5]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[6]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[7]	IETF RFC 7232: "Hypertext Transfer Protocol (HTTP/1.1): Conditional Requests".

[8]	IETF RFC 7233: "Hypertext Transfer Protocol (HTTP/1.1): Range Requests".

[9]	IETF RFC 7234: "Hypertext Transfer Protocol (HTTP/1.1): Caching".

[10]	IETF RFC 7235: "Hypertext Transfer Protocol (HTTP/1.1): Authentication".

[11]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[12]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[13]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[14]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[15]	Open API Initiative, “OpenAPI 3.0.0 Specification”, https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[16]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3”.

[17]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2”.

[18]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[19]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[20]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[21]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[22]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

VAL service: A generic name for any service offered by the VAL service provider to their VAL users.

SEAL service: A generic name for a common service (e.g. group management, configuration management, location management) that can be utilized by multiple vertical applications.

SEAL provider: Provider of SEAL service(s).

VAL server: A generic name for the server application function of a specific VAL service.

SEAL server: An entity that provides the server side functionalities corresponding to the specific SEAL service.

VAL system: The collection of applications, services, and enabling capabilities required to support a VAL service.

VAL group: A defined set of VAL UEs or VAL users configured for specific purpose in a VAL service.

NOTE:	The set could be of either VAL UEs or VAL users depending on the specific VAL service.

VAL group home system: The VAL system where the VAL group is defined.

VAL group member: A VAL service user, whose VAL user ID is listed in a particular VAL group.

Vertical application: An application catering to a specific vertical.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GS	5G System

AEF	API Exposing Function

API	Application Programming Interface

JSON	JavaScript Object Notation

PLMN	Public Land Mobile Network

REST	Representational State Transfer

SCEF	Service Capability Exposure Function

SCS	Service Capability Server

SEAL	Service Enabler Architecture Layer for Verticals

TMGI	Temporary Mobile Group Identity

UE	User Equipment

VAL	Vertical Application Layer

# 4	Overview

3GPP has considered in 3GPP TS 23.434 [2] the development of Service enabler architecture layer for verticals (SEAL) over 3GPP networks to support vertical applications (e.g. V2X applications). It specifies the functional architecture for SEAL and the procedures, information flows and APIs for each service within SEAL in order to support vertical applications over the 3GPP systems. To ensure efficient use and deployment of vertical applications over 3GPP systems, SEAL services includes, group management, configuration management, location management, identity management, key management and network resource management.

3GPP TS 23.434 [2], clause 6 specifies the functional entities and domains of the functional model, reference points descriptions and SEAL APIs for SEAL services.

The present document specifies the APIs needed to support SEAL.

# 5	Services offered by the SEAL servers

## 5.1	Introduction of SEAL services

The table 5.1-1 lists the SEAL server APIs below the service name. A service description clause for each API gives a general description of the related API.

Table 5.1-1: List of SEAL Service APIs

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| SS_Location_Reporting_Trigger | Location_Reporting_Trigger | Request/ Response | VAL server |
| SS_Location_Info_Subscription | Location_Info_Subscribe_Event | Subscribe/Notify | VAL server |
|  | Location_Info_Notify_Event |  | VAL server |
| SS_Obtain_Location_Info | Obtain_Location_Info | Request/ Response | VAL server |
| SS_GroupManagement | Query_Group_Info | Request/ Response | VAL server |
|  | Update_Group_Info | Request/ Response | VAL server |
|  | Create_Group | Request/ Response | VAL server |
|  | Delete_Group | Request/Response | VAL server |
| SS_GroupManagementEvent | Subscribe_Group_Info_Modification | Subscribe/Notify | VAL server |
|  | Notify_Group_Info_Modification |  | VAL server |
|  | Notify_Group_Creation |  | VAL server |
| SS_UserProfileRetrieval | Obtain_User_Profile | Request/ Response | VAL server |
| SS_UserProfileEvent | Subscribe_User_Profile_Update | Subscribe/Notify | VAL server |
|  | Notify_User_Profile_Update |  | VAL server |
| SS_Network_Resource_Adaptation | Network_Resource_Adaptation | Request/Response | VAL server |
| SS_Events | Subscribe_Event | Subscribe/Notify | VAL server |
|  | Notify_Event |  | VAL server |
|  | Unsubscribe_Event |  | VAL server |
| NOTE: The service operations of SS_Events API are reused by the SS_Location_Info_Subscription, SS_GroupManagementEvent and SS_UserProfileEvent for events related services. |  |  |  |

## 5.2	Location management APIs

### 5.2.1	SS_Location_Reporting_Trigger API

#### 5.2.1.1	Service Description

##### 5.2.1.1.1	Overview

#### 5.2.1.2	Service Operations

##### 5.2.1.2.1	Introduction

The service operation defined for SS_Location_Reporting_Trigger API is shown in the table 5.2.1.2.1-1.

Table 5.2.1.2.1-1: Operations of the SS_Location_Reporting_Trigger API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Location_Reporting_Trigger |  |  |

##### 5.2.1.2.2	Location_Reporting_Trigger

###### 5.2.1.2.2.1	General

###### 5.2.1.2.2.2	<Description> Location_Reporting_Trigger service operation

### 5.2.2	SS_Location_Info_Subscription API

#### 5.2.2.1	Service Description

##### 5.2.2.1.1	Overview

#### 5.2.2.2	Service Operations

##### 5.2.2.2.1	Introduction

The service operations defined for SS_Location_Info_Subscription API are shown in the table 5.2.2.2.1-1.

Table 5.2.2.2.1-1: Operations of the SS_Location_Info_Subscription API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Location_Info_Subscribe_Event |  |  |
| Location_Info_Notify_Event |  |  |

##### 5.2.2.2.2	Location_Info_Subscribe_Event

###### 5.2.2.2.2.1	General

###### 5.2.2.2.2.2	<Description> Location_Info_Subscribe_Event service operation

##### 5.2.2.2.3	Location_Info_Notify_Event

###### 5.2.2.2.3.1	General

###### 5.2.2.2.3.2	<Description> Location_Info_Notify_Event service operation

### 5.2.3	SS_Obtain_Location_Info API

#### 5.2.3.1	Service Description

##### 5.2.3.1.1	Overview

#### 5.2.3.2	Service Operations

##### 5.2.3.2.1	Introduction

The service operation defined for SS_Obtain_Location_Info API is shown in the table 5.2.3.2.1-1.

Table 5.2.3.2.1-1: Operations of the SS_Obtain_Location_Info API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Obtain_Location_Info |  |  |

##### 5.2.3.2.2	Obtain_Location_Info

###### 5.2.3.2.2.1	General

###### 5.2.3.2.2.2	<Description> Obtain_Location_Info service operation

## 5.3	Group management APIs

### 5.3.1	SS_GroupManagement API

#### 5.3.1.1	Service Description

##### 5.3.1.1.1	Overview

The SS_GroupManagement API, as defined 3GPP TS 23.434 [2], allows VAL server via GM-S reference point to fetch and update group membership and configuration information.

#### 5.3.1.2	Service Operations

##### 5.3.1.2.1	Introduction

The service operation defined for SS_GroupManagement API is shown in the table 5.3.1.2.1-1.

Table 5.3.1.2.1-1: Operations of the SS_GroupManagement API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Query_Group_Info | This service operation is used by VAL server to query for group membership list and configuration information. | VAL Server |
| Update_Group_Info | This service operation is used by VAL server to modify group membership and configuration information. | VAL server |
| Create_Group | This service operation is used by VAL server to configure new VAL group. | VAL server |
| Delete_Group | This service operation is used by the VAL server to delete the VAL group. | VAL server |

##### 5.3.1.2.2	Query_Group_Info

###### 5.3.1.2.2.1	General

This service operation is used by a VAL server to obtain group membership and configuration information.

###### 5.3.1.2.2.2		VAL server fetching group membership and configuration information using Query_Group_Info service operation

To obtain membership, configuration information of a VAL group, the VAL server shall send a HTTP GET message to the group management server, on VAL group document’s resource representation URI as specified in clause 7.2.1.2.3.3.1. The GET message may include the following query parameters: membership list, group configuration.

Upon receiving the HTTP GET message as described above, the group management server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to fetch the VAL group information;

2.   if the VAL server is authorized to obtain the group information, the group management server shall;

a.	if the request includes query parameters, then, return in the response message with VAL group information which includes, group membership list information if the request includes membership list query, group configuration information if the request includes group configuration query and  VAL group identifier;

b.   if the request does not include query parameter, then, return the VAL group document resource in the response message.

##### 5.3.1.2.3	Update_Group_Info

###### 5.3.1.2.3.1	General

This service operation is used by a VAL server to modify group membership and configuration information.

###### 5.3.1.2.3.2	VAL server modifying group membership and configuration using Update_Group_Info service operation

To modify group information of a VAL group, the VAL server shall send HTTP PUT message to the group management server to the Resource URI identifying the VAL group document resource representation, as specified in the clause 7.2.1.2.3.3.2. This request shall not replace valGroupId property in the existing resource. Upon receiving the HTTP PUT message, the group management server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to modify VAL group information;

2.	verify that valGroupId in the request is same as valGroupId of the VAL group document resource;

3.	if the VAL server is authorized to modify the group information and the valGroupId matches, then the group management server shall;

a.	if the group configuration information in the request is valid, update the resource identified by the Resource URI of the group document with group members list and group configuration information received in the request;

b.   return the updated VAL group document in the response

##### 5.3.1.2.4	Create_Group

###### 5.3.1.2.4.1	General

This service operation is used by a VAL server to create VAL group.

###### 5.3.1.2.4.2	VAL server creating new group using Create_Group service operation

To create a VAL group, the VAL server shall send a HTTP POST message to the group management server. The body of the POST message shall include VAL group document information as specified in clause 7.2.1.2.2.3.1. Upon receiving HTTP POST message, the group management server shall

1.	verify the identity of the VAL server and check if the VAL server is authorized to create VAL group document;

2.	if the VAL server is authorized to create VAL group document, shall create a new resource as defined in 7.2.1.2.2.3.1 and return the VAL group document and its Resource URI in the response message.

##### 5.3.1.2.5	Delete_Group

###### 5.3.1.2.5.1	General

This service operation is used by a VAL server to delete a VAL group.

###### 5.3.1.2.5.2	VAL server deleting VAL group using Delete_Group service operation

To delete a VAL group, the VAL server shall send a HTTP DELETE message to the Group Management server to its resource representation in the Group Management server as specified in clause 7.2.1.2.3.3.3. Upon receiving HTTP DELETE message, the Group Management server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to delete the VAL group document;

2.	if the VAL server is authorized to delete the VAL group document, the Group Management server shall

a.	 delete the resource representation pointed by the group document resource identifier.

### 5.3.2	SS_GroupManagementEvent API

The SS_GroupManagementEvent API, as defined 3GPP TS 23.434 [2], allows a VAL server via GM-S reference point to subscribe for and receive notifications from Group Management server on modifications to VAL Group membership and configuration information. The SS_GroupManagementEvent API supports this via the SS_Events API as specified in clause 7.5.

## 5.4	Configuration management APIs

### 5.4.1	SS_UserProfileRetrieval API

#### 5.4.1.1	Service Description

##### 5.4.1.1.1	Overview

The SS_UserProfileRetrieval API, as defined in 3GPP TS 23.434 [2], allows VAL server via CM-S reference point to obtain user profile from the configuration management server.

#### 5.4.1.2	Service Operations

##### 5.4.1.2.1	Introduction

The service operation defined for SS_UserProfileRetrieval API is shown in the table 5.4.1.2.1-1.

Table 5.4.1.2.1-1: Operations of the SS_ UserProfileRetrieval API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Obtain_User_Profile | This service operation is used by VAL server to obtain user profile. | VAL server |

##### 5.4.1.2.2	Obtain_User_Profile

###### 5.4.1.2.2.1	General

This service operation is used by a VAL server to obtain VAL user profile information.

###### 5.4.1.2.2.2		VAL server retrieving VAL user profile information using Obtain_User_Profile service operation

To obtain a VAL user’s profile, the VAL server shall send HTTP GET request message to configuration management server, on VAL service’s resource representation URI, with query parameter VAL user ID or VAL UE ID, as specified in 7.3.1.2.2.3.1.

Upon receiving the HTTP GET message as described above, the configuration management server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to fetch the VAL user profile information;

2.	if the VAL server is authorized to obtain the requested VAL user profile information, the configuration management server shall;

a.	return in the response message with profile information corresponding to the VAL user ID or VAL UE ID that was sent in the request message.

### 5.4.2	SS_ UserProfileEvent API

The SS_UserProfileEvent API, as defined in 3GPP TS 23.434 [2], allows a VAL server via CM-S reference point to subscribe for and receive notifications from the Configuration Management server on profile updates to VAL User or VAL UE. The SS_UserProfileEvent API supports this via the SS_Events API as specified in clause 7.5.

## 5.5	Network resource management APIs

### 5.5.1	SS_Network_Resource_Adaptation API

#### 5.5.1.1	Service Description

##### 5.5.1.1.1	Overview

#### 5.5.1.2	Service Operations

##### 5.5.1.2.1	Introduction

The service operation defined for SS_Network_Resource_Adaptation API is shown in the table 5.5.1.2.1-1.

Table 5.5.1.2.1-1: Operations of the SS_Network_Resource_Adaptation API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Network_Resource_Adaptation |  |  |

##### 5.5.1.2.2	Network_Resource_Adapation

###### 5.5.1.2.2.1	General

###### 5.5.1.2.2.2		<Description> Network_Resource_Adaptation service operation

## 5.6	Events APIs

### 5.6.1	SS_Events API

#### 5.6.1.1	Service Description

##### 5.6.1.1.1	Overview

The SS_Events API, allows a VAL server via LM-S, GM-S, CM-S reference points to subscribe and unsubscribe from SEAL events and to receive notifications from the Location Management Server, Group Management Server and Configuration Management Server respectively.

#### 5.6.1.2	Service Operations

##### 5.6.1.2.1	Introduction

The service operations defined for the SS_Events API are shown in the table 5.6.1.2.1-1.

Table 5.6.1.2.1-1: Operations of the SS_Events API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Subscribe_Event | This service operation is used by VAL server to subscribe for events from SEAL servers. | VAL Server |
| Unsubscribe_Event | This service operation is used by VAL server to unsubscribe for events from SEAL servers. | VAL Server |
| Notify_Event | This service operation is used by SEAL servers to send the notifications to the VAL server. | SEAL servers (Location Management, Group Management, Configuration Management). |

##### 5.6.1.2.2	Subscribe_Event

###### 5.6.1.2.2.1	General

This service operation is used by a VAL server to subscribe to the SEAL events.

###### 5.6.1.2.2.1	Subscribing to SEAL events using Subscribe_Event service operation

To subscribe to SEAL events, the VAL server shall send an HTTP POST message to the SEAL server. The body of the HTTP POST message shall include VAL Server Identifier, Event Type, Event Filters, Reporting Requirements and a Notification Destination URI as specified in clause 7.5.1.2.2.3.1.

Upon receiving the above described HTTP POST message, the SEAL server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to subscribe to the SEAL events mentioned in the HTTP POST message;

2.	if the VAL server is authorized to subscribe to the SEAL events, the SEAL server shall:

a.	create a new resource as specified in clause 7.5.1.2.1; and

b.	return the SEAL Resource URI in the response message.

##### 5.6.1.2.3	Notify_Event

###### 5.6.1.2.3.1	General

This service operation is used by the SEAL servers to send notifications to the VAL server.

###### 5.6.1.2.3.2	Notifying SEAL events using Notify_Event service operation

To notify the SEAL events, the SEAL server shall send an HTTP POST message using the Notification Destination URI received in the subscription request. The body of the HTTP POST message shall include an Event Notification and SEAL Resource URI.

Upon receiving the HTTP POST message, the VAL server shall process the Event Notification.

##### 5.6.1.2.4	Unsubscribe_Event

###### 5.6.1.2.4.1	General

This service operation is used by a VAL server to un-subscribe from the SEAL events.

###### 5.6.1.2.4.2	Unsubscribing from SEAL events using Unsubscribe_Event service operation

To unsubscribe from SEAL events, the VAL server shall send an HTTP DELETE message to the resource representing the event in the SEAL server as specified in clause 7.5.1.2.3.3.1.

Upon receiving the HTTP DELETE message, the SEAL sever shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to Unsubscribe from the SEAL  event associated with the SEAL Resource URI; and

2.	if the VAL server is authorized to unsubscribe from the SEAL events, the SEAL server shall delete the resource pointed by the SEAL Resource URI

## 5.7	Identity management APIs

Editor's note: Definitions of Identity management APIs is FFS

## 5.8	Key management APIs

Editor's note: Definitions of Key management APIs is FFS

# 6	SEAL Design Aspects Common for All APIs

## 6.1	General

SEAL APIs allow secure access to the capabilities provided by SEAL.

This document specifies the procedures triggered at different functional entities as a result of API invocation requests and event notifications. The stage-2 level requirements and signalling flows are defined in 3GPP TS 23.434 [2].

Several design aspects, as mentioned in the following clauses, are specified in 3GPP TS 29.122 [3] and referenced by this specification.

## 6.2	Data Types

### 6.2.1	General

This clause defines structured data types, simple data types and enumerations that are applicable to several APIs defined in the present specification and can be referenced from data structures defined in the subsequent clauses.

In addition, data types that are defined in OpenAPI 3.0.0 Specification [15] can also be referenced from data structures defined in the subsequent clauses.

NOTE:	As a convention, data types names in the present specification are with an upper-case letter in the beginning. Parameters are with a lower-case letter in the beginning. As an exception, data types that are also defined in OpenAPI 3.0.0 Specification [15] can use a lower-case case letter in the beginning for consistency.

Table 6.2.1-1 specifies data types re-used by the SEAL from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the SEAL.

Table 6.2.1-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Uri | 3GPP TS 29.122 [3] |  |
| TestNotification | 3GPP TS 29.122 [3] | Following clarifications apply: - The SCEF is the SEAL server; and - The SCS/AS is the VAL server. |
| WebsockNotifConfig | 3GPP TS 29.122 [3] | Following clarifications apply: - The SCEF is the SEAL server; and - The SCS/AS is the VAL server. |

### 6.2.2	Referenced structured data types

Table 6.2.2-1 lists structured data types defined in this specification referenced by multiple services:

Table 6.2.2-1: Referenced Structured Data Types

| Data type | Reference | Description |
| --- | --- | --- |
| VALGroupDocument | Clause 7.2.1.4.2.2 | VAL Group document information. |
| ProfileDoc | Clause 7.3.1.4.2.2 | VAL User or VAL UE profile information. |

### 6.2.3	Referenced Simple data types and enumerations

Following simple data types defined in Table 6.2.3-1 are applicable to several APIs in this document:

Table 6.2.3-1: Simple data types applicable to several APIs

| Type name | Reference | Description |
| --- | --- | --- |

## 6.3	Usage of HTTP

For SEAL APIs, support of HTTP/1.1 (IETF RFC 7230 [5], IETF RFC 7231 [6], IETF RFC 7232 [7], IETF RFC 7233 [8], IETF RFC 7234 [9] and IETF RFC 7235 [10]) over TLS (IETF RFC 5246 [11]) is mandatory and support of HTTP/2 (IETF RFC 7540 [12]) over TLS (IETF RFC 5246 [11]) is recommended.

A functional entity desiring to use HTTP/2 shall use the HTTP upgrade mechanism to negotiate applicable HTTP version as described in IETF RFC 7540 [12].

Editor’s note: Usage of HTTP over TLS is based on security aspects defined by SA3.

## 6.4	Content type

The bodies of HTTP request and successful HTTP responses shall be encoded in JSON format (see IETF RFC 8259 [13]).

The MIME media type that shall be used within the related Content-Type header field is "application/json", as defined in IETF RFC 8259 [13].

NOTE:	This release only supports the content type JSON.

## 6.5	URI structure

- 6.5.1	Resource URI structure

All resource URIs of SEAL APIs should have the following root structure:

- {apiRoot}/{apiName}/{apiVersion}/

"apiRoot" is configured by means outside the scope of the present document. It includes the scheme ("https"), host and optional port, and an optional prefix string. "apiName" and "apiVersion" shall be set dependent on the API, as defined in the corresponding clauses below.

All resource URIs in the clauses below are defined relative to the above root URI.

NOTE 1:	The "apiVersion" will only be increased if the new API version contains backward incompatible changes. Otherwise, the supported feature mechanism defined in clause 6.8 can be used to negotiate extensions.

NOTE 2:	A different root structure can be used when the resource URI is preconfigured in the API invoking entity.

The root structure may be followed by "apiSpecificSuffixes" that are dependent on the API and are defined separately for each API where they apply:

- {apiRoot}/{apiName}/{apiVersion}/{apiSpecificSuffixes}

- 6.5.2	Custom operations URI structure

The custom operation definition is in Annex C of 3GPP TS 29.501 [14].

The URI of a custom operation which is associated with a resource shall have the following structure:

{apiRoot}/{apiName}/{apiVersion}/{apiSpecificResourceUriPart}/{custOpName}

Custom operations can also be associated with the service instead of a resource. The URI of a custom operation which is not associated with a resource shall have the following structure:

{apiRoot}/{apiName}/{apiVersion}/{custOpName}

In the above URI structures, "apiRoot", "apiName", "apiVersion" and "apiSpecificResourceUriPart" are as defined in clause 6.5.1 and "custOpName" represents the name of the custom operation as defined in clause 5.1.3.2 of 3GPP TS 29.501 [14]

## 6.6	Notifications

The functional entities

-	shall support the delivery of notifications using a separate HTTP connection towards an address;

-	may support testing delivery of notifications; and

-	may support the delivery of notification using WebSocket protocol (see IETF RFC 6455 [4]),

as described in 3GPP TS 29.122 [3], with the following clarifications:

-	the SCEF is the SEAL server; and

-	the SCS/AS is the Subscriber.

## 6.7	Error Handling

Response bodies for error handling, as described in 3GPP TS 29.122 [3], are applicable to all APIs in the present specification unless specified otherwise, with the following clarifications:

-	the SCEF is the SEAL server; and

-	the SCS/AS is the functional entity invoking an API.

## 6.8	Feature negotiation

The functional entity invoking an API (i.e. the VAL server) and the SEAL server use feature negotiation procedures defined in 3GPP TS 29.122 [3] to negotiate the supported features, with the following clarifications:

-	description of the SCEF applies to the SEAL server; and

-	description of the SCS/AS applies to the functional entity invoking an API.

## 6.9	HTTP headers

The HTTP headers described in 3GPP TS 29.122 [3] are applicable to all APIs in this document.

## 6.10	Conventions for Open API specification files

The conventions for Open API specification files as specified in clause 5.2.9 of 3GPP TS 29.122 [3] shall be applicable for all APIs in this document.

# 7	SEAL API Definitions

## 7.1	Location management APIs

### 7.1.1	SS_Location_Reporting_Trigger API

#### 7.1.1.1	API URI

#### 7.1.1.2	Resources

##### 7.1.1.2.1	Overview

Figure 7.1.1.2.1-1: Resource URI structure of the SS_Location_Reporting_trigger API

Table 7.1.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.1.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |

##### 7.1.1.2.2	Resource: <Resource name>

###### 7.1.1.2.2.1	Description

###### 7.1.1.2.2.2	Resource Definition

###### 7.1.1.2.2.3	Resource Standard Methods

7.1.1.2.2.3.1	<Method Name>

Table 7.1.1.2.2.3.1-1: URI query parameters supported by the <Method Name> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.1.1.2.2.3.1-2 and the response data structures and response codes specified in table 7.1.1.2.2.3.1-3.

Table 7.1.1.2.2.3.1-2: Data structures supported by the <Method Name> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |

Table 7.1.1.2.2.3.1-3: Data structures supported by the <Method Name> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |

###### 7.1.1.2.2.4		Resource Custom Operations

#### 7.1.1.3	Notifications

##### 7.1.1.3.1	General

Table 7.1.1.3.1-1: Notifications overview

| Notification | Resource URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |

##### 7.1.1.3.2	<Notification name>

###### 7.1.1.3.2.1	Description

###### 7.1.1.3.2.2	Notification definition

Resource URI: <Notification resource URI>

This method shall support the URI query parameters specified in table 7.1.1.3.2.2-1.

Table 7.1.1.3.2.2-1: URI query parameters supported by the <Method Name> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.1.3.2.2-2 and the response data structures and response codes specified in table 7.1.1.3.2.2-3.

Table 7.1.1.3.2.2-2: Data structures supported by the <Method Name> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |

Table 7.1.1.3.2.2-3: Data structures supported by the <Method Name> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |

#### 7.1.1.4	Data Model

##### 7.1.1.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause <6.X related to SEAL design aspects for all APIs> apply to this API

Table 7.1.1.4.1-1 specifies the data types defined specifically for the SS_Location_Reportng_Trigger API service.

Table 7.1.1.4.1-1: SS_Location_Reportng_Trigger API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |

Table 7.1.1.4.1-2 specifies data types re-used by the SS_Location_Reportng_Trigger API service.

Table 7.1.1.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |

##### 7.1.1.4.2	Structured data types

###### 7.1.1.4.2.1	Introduction

###### 7.1.1.4.2.2	Type: <Data type name>

Table 7.1.1.4.2.2-1: Definition of type <Data Type name>

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

##### 7.1.1.4.3	Simple data types and enumerations

#### 7.1.1.5	Error Handling

#### 7.1.1.6	Feature negotiation

Table 7.1.1.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 7.1.2	SS_Location_Info_Subscription API

#### 7.1.2.1	API URI

#### 7.1.2.2	Resources

##### 7.1.2.2.1	Overview

Figure 7.1.2.2.1-1: Resource URI structure of the SS_Location_Info_Subscription API

Table 7.1.2.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.1.2.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |

##### 7.1.2.2.2	Resource: <Resource name>

###### 7.1.2.2.2.1	Description

###### 7.1.2.2.2.2	Resource Definition

###### 7.1.2.2.2.3	Resource Standard Methods

7.1.2.2.2.3.1	<Method Name>

Table 7.1.2.2.2.3.1-1: URI query parameters supported by the <Method Name> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.1.2.2.2.3.1-2 and the response data structures and response codes specified in table 7.1.2.2.2.3.1-3.

Table 7.1.2.2.2.3.1-2: Data structures supported by the <Method Name> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |

Table 7.1.2.2.2.3.1-3: Data structures supported by the <Method Name> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |

###### 7.1.2.2.2.4		Resource Custom Operations

#### 7.1.2.3	Notifications

##### 7.1.2.3.1	General

Table 7.1.2.3.1-1: Notifications overview

| Notification | Resource URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |

##### 7.1.2.3.2	<Notification name>

###### 7.1.2.3.2.1	Description

###### 7.1.2.3.2.2	Notification definition

Resource URI: <Notification resource URI>

This method shall support the URI query parameters specified in table 7.1.2.3.2.2-1.

Table 7.1.2.3.2.2-1: URI query parameters supported by the <Method Name> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.2.3.2.2-2 and the response data structures and response codes specified in table 7.1.2.3.2.2-3.

Table 7.1.2.3.2.2-2: Data structures supported by the <Method Name> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |

Table 7.1.2.3.2.2-3: Data structures supported by the <Method Name> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |

#### 7.1.2.4	Data Model

##### 7.1.2.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause <6.X related to SEAL design aspects for all APIs> apply to this API

Table 7.1.2.4.1-1 specifies the data types defined specifically for the SS_Location_Info_Subscription API service.

Table 7.1.2.4.1-1: SS_Location_ Info_Subscription API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |

Table 7.1.2.4.1-2 specifies data types re-used by the SS_Location_Info_Subscription API service.

Table 7.1.2.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |

##### 7.1.2.4.2	Structured data types

###### 7.1.2.4.2.1	Introduction

###### 7.1.2.4.2.2	Type: <Data type name>

Table 7.1.2.4.2.2-1: Definition of type <Data Type name>

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

##### 7.1.2.4.3	Simple data types and enumerations

#### 7.1.2.5	Error Handling

#### 7.1.2.6	Feature negotiation

Table 7.1.2.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 7.1.3	SS_Obtain_Location_Info API

#### 7.1.3.1	API URI

#### 7.1.3.2	Resources

##### 7.1.3.2.1	Overview

Figure 7.1.3.2.1-1: Resource URI structure of the SS_Obtain_Location_Info API

Table 7.1.3.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.1.3.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |

##### 7.1.3.2.2	Resource: <Resource name>

###### 7.1.3.2.2.1	Description

###### 7.1.3.2.2.2	Resource Definition

###### 7.1.3.2.2.3	Resource Standard Methods

7.1.3.2.2.3.1	<Method Name>

Table 7.1.3.2.2.3.1-1: URI query parameters supported by the <Method Name> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.1.3.2.2.3.1-2 and the response data structures and response codes specified in table 7.1.3.2.2.3.1-3.

Table 7.1.3.2.2.3.1-2: Data structures supported by the <Method Name> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |

Table 7.1.3.2.2.3.1-3: Data structures supported by the <Method Name> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |

###### 7.1.3.2.2.4		Resource Custom Operations

#### 7.1.3.3	Notifications

##### 7.1.3.3.1	General

Table 7.1.3.3.1-1: Notifications overview

| Notification | Resource URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |

##### 7.1.3.3.2	<Notification name>

###### 7.1.3.3.2.1	Description

###### 7.1.3.3.2.2	Notification definition

Resource URI: <Notification resource URI>

This method shall support the URI query parameters specified in table 7.1.3.3.2.2-1.

Table 7.1.3.3.2.2-1: URI query parameters supported by the <Method Name> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.3.2.2-2 and the response data structures and response codes specified in table 7.1.3.3.2.2-3.

Table 7.1.3.3.2.2-2: Data structures supported by the <Method Name> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |

Table 7.1.3.3.2.2-3: Data structures supported by the <Method Name> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |

#### 7.1.3.4	Data Model

##### 7.1.3.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause <6.X related to SEAL design aspects for all APIs> apply to this API

Table 7.1.3.4.1-1 specifies the data types defined specifically for the SS_Obtain_Location_Info API service.

Table 7.1.3.4.1-1: SS_Obtain_Location_Info API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |

Table 7.1.3.4.1-2 specifies data types re-used by the SS_Obtain_Location_Info API service.

Table 7.1.3.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |

##### 7.1.3.4.2	Structured data types

###### 7.1.3.4.2.1	Introduction

###### 7.1.3.4.2.2	Type: <Data type name>

Table 7.1.3.4.2.2-1: Definition of type <Data Type name>

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

##### 7.1.3.4.3	Simple data types and enumerations

#### 7.1.3.5	Error Handling

#### 7.1.3.6	Feature negotiation

Table 7.1.3.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 7.2	Group management APIs

### 7.2.1	SS_GroupManagement API

#### 7.2.1.1	API URI

The request URI used in each HTTP request from the VAL server towards the Group management server shall have the structure as defined in clause 6.5 with the following clarifications:

-	The {apiName} shall be "group-management".

-	The {apiVersion} shall be "v1".

-	The {apiSpecificSuffixes} shall be set as described in clause 7.2.1.2

#### 7.2.1.2	Resources

##### 7.2.1.2.1	Overview

Figure 7.2.1.2.1-1: Resource URI structure of the SS_GroupManagement API

Table 7.2.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.2.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| All VAL Group Documents | {apiRoot} /group-management/{apiVersion} /group-documents | POST | Creates a new VAL group document. |
| Individual VAL Group Document | {apiRoot} /group-management/{apiVersion}/group-documents/ /{groupDocId} | GET | Retrieve an individual VAL group’s membership and configuration information according to query parameter on the resource identified by {groupDocId}. If there are no query parameters, fetch the whole VAL group document resource identified by {groupDocId}. |
|  |  | PUT | Updates an individual VAL group’s membership and configuration information identified by {groupDocId}. |

##### 7.2.1.2.2	Resource: All VAL Group Documents

###### 7.2.1.2.2.1	Description

The All VAL Group Documents resource represents all the VAL group documents that are created at a given group management server.

###### 7.2.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/group-management/{apiVersion}/group-documents

This resource shall support the resource URI variables defined in the table 7.2.1.2.2.2-1.

Table 7.2.1.2.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.5 |

###### 7.2.1.2.2.3	Resource Standard Methods

7.2.1.2.2.3.1	POST

This method shall support the URI query parameters specified in table 7.2.1.2.2.3.1-1.

Table 7.2.1.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.1.2.2.3.1-2 and the response data structures and response codes specified in table 7.2.1.2.2.3.1-3.

Table 7.2.1.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VALGroupDocument | M | 1 | Details of the VAL group that needs to be created, |

Table 7.2.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VALGroupDocument | M | 1 | 201 Created | VAL group created successfully. The URI of the created resource shall be returned in the “Location” HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

###### 7.2.1.2.2.4		Resource Custom Operations

None.

##### 7.2.1.2.3	Resource: Individual VAL Group Document

###### 7.2.1.2.3.1	Description

The Individual VAL Group Document resource represents an individual group document that is created at a given group management server.

###### 7.2.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/group-management/{apiVersion}/group-documents/{groupDocId}

This resource shall support the resource URI variables defined in the table 7.2.1.2.3.2-1.

Table 7.2.1.2.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.5 |
| groupDocId | String representing an individual group document resource. |

###### 7.2.1.2.3.3	Resource Standard Methods

7.2.1.2.3.3.1	GET

This operation retrieves VAL group information satisfying filter criteria. This method shall support the URI query parameters specified in table 7.2.1.2.3.3.1-1.

Table 7.2.1.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| group-members | boolean | O | 0..1 | When set to 'true', it indicates the group management server to send the members list information of the VAL group. Set to false or omitted otherwise. |
| group-configuration | boolean | O | 0..1 | When set to 'true', it indicates the group management server to send the configuration information of the VAL group. Set to false or omitted otherwise. |

This method shall support the request data structures specified in table 7.2.1.2.3.3.1-2 and the response data structures and response codes specified in table 7.2.1.2.3.3.1-3.

Table 7.2.1.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.2.1.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VALGroupDocument | M | 1 | 200 OK | The VAL group information based on the request from the VAL server. This response shall include VAL group members list if group-members flag is set to true in the request, VAL group configuration information if the group-configuration flag is set to true in the request, VAL group identifier, whole VAL group document resource if both group-members and group-configuration flags are omitted/set to false in the request. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

7.2.1.2.3.3.2	PUT

This operation updates the VAL group document. This method shall support the URI query parameters specified in table 7.2.1.2.3.3.2-1.

Table 7.2.1.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.1.2.3.3.2-2 and the response data structures and response codes specified in table 7.2.1.2.3.3.2-3.

Table 7.2.1.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VALGroupDocument | M | 1 | Updated details of the VAL group document. |

Table 7.2.1.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VALGroupDocument | M | 1 | 200 OK | The VAL group document updated successfully and the updated VAL group document returned in the response. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

7.2.1.2.3.3.3	DELETE

This operation deletes the VAL group document. This method shall support the URI query parameters specified in table 7.2.1.2.3.3.3-1.

Table 7.2.1.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.1.2.3.3.3-2 and the response data structures and response codes specified in table 7.2.1.2.3.3.3-3.

Table 7.2.1.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.2.1.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual VAL group document matching the groupDocId is deleted. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

###### 7.2.1.2.3.4	Resource Custom Operations

None.

#### 7.2.1.3	Notifications

None.

#### 7.2.1.4	Data Model

##### 7.2.1.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API

Table 7.2.1.4.1-1 specifies the data types defined specifically for the SS_GroupManagement API service.

Table 7.2.1.4.1-1: SS_GroupManagement API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| VALGroupDoument | 7.2.1.4.2.2 | VAL group document details. |  |

Table 7.2.1.4.1-2 specifies data types re-used by the SS_GroupManagement API service.

Table 7.2.1.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the applicability of optional features defined in table 7.2.1.6-1. |  |

##### 7.2.1.4.2	Structured data types

###### 7.2.1.4.2.1	Introduction

###### 7.2.1.4.2.2	Type: VALGroupDocument

Table 7.2.1.4.2.2-1: Definition of type VALGroupDocument

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valGroupId | string | M | 1 | This is VAL group identity (VAL group ID) as per TS 23.434 [2], which is a unique identifier within the VAL service that represents a VAL group, set of VAL users or VAL UEs according to the VAL service. |  |
| grpDesc | string | O | 0..1 | Text description of the VAL group. |  |
| membersList | array(string) | O | 1..N | List of VAL User IDs and VAL UE IDs, which are members of the VAL group. |  |
| valGrpConf | string | O | 0..1 | Configuration data for the VAL group. Shall be present in HTTP POST request message from VAL server to Group Management server. |  |
| valServiceIds | array(string) | O | 1..N | List of VAL services whose communications enabled on the group. |  |
| suppFeat | SupportedFeatures | O | 0..1 | Used to negotiate the supported optional features of the API as described in clause 6.8. This attribute shall be provided in the HTTP POST request and in the response of successful resource creation. |  |

##### 7.2.1.4.3	Simple data types and enumerations

None.

#### 7.2.1.5	Error Handling

General error responses are defined in clause 6.7.

#### 7.2.1.6	Feature negotiation

General feature negotiation procedures are defined in clause 6.8.

Table 7.2.1.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 7.3	Configuration management APIs

### 7.3.1	SS_ UserProfileRetrieval API

#### 7.3.1.1	API URI

The request URI used in each HTTP request from the VAL server towards the Configuration management server shall have the structure as defined in clause 6.5 with the following clarifications:

-	The {apiName} shall be "configuration-management".

-	The {apiVersion} shall be "v1".

-	The {apiSpecificSuffixes} shall be set as described in clause 7.3.1.2.

#### 7.3.1.2	Resources

##### 7.3.1.2.1	Overview

Figure 7.3.1.2.1-1: Resource URI structure of the SS_ UserProfileRetrieval API

Table 7.3.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.3.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Individual VAL service | {apiRoot} /configuration-management/{apiVersion}/ /{valServiceId} | GET | Retrieve a VAL User or VAL UE’s profile information belonging to a VAL service, according to query parameter (VAL User ID / VAL UE ID) on the resource identified by {valServiceId}. |

Editor’s note:	It is FFS, whether the current Resource URI fits into the requirement and how the VAL server is aware of valServiceId in advance.

##### 7.3.1.2.2	Resource: Individual VAL Service

###### 7.3.1.2.2.1	Description

The Individual VAL Service resource represents an individual VAL service that is created at a given configuration management server.

###### 7.3.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/configuration-management/{apiVersion}/{valServiceId}

This resource shall support the resource URI variables defined in the table 7.3.1.2.2.2-1.

Table 7.3.1.2.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.5 |
| valServiceId | String representing an individual VAL service resource. |

###### 7.3.1.2.2.3	Resource Standard Methods

7.3.1.2.2.3.1	GET

Table 7.3.1.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| valUserId | string | O | 0..1 | String identifying a VAL User. (NOTE) |
| valUEId | string | O | 0..1 | String identifying a VAL UE. (NOTE) |
| NOTE: Only one of the parameters "valUserId" or "valUEId" shall be included. |  |  |  |  |

This method shall support the request data structures specified in table 7.3.1.2.2.3.1-2 and the response data structures and response codes specified in table 7.3.1.2.2.3.1-3.

Table 7.3.1.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.3.1.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ProfileDoc | M | 1 | 200 OK | The response body contains the profile information of the VAL User /VAL UE corresponding to the identifier in the request. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

###### 7.3.1.2.2.4		Resource Custom Operations

None.

#### 7.3.1.3	Notifications

None.

#### 7.3.1.4	Data Model

##### 7.3.1.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API

Table 7.3.1.4.1-1 specifies the data types defined specifically for the SS_UserProfileRetrieval API service.

Table 7.3.1.4.1-1: SS_ UserProfileRetrieval API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ProfileDoc | 7.3.1.4.2.2 | Profile information associated with VAL user ID or VAL UE ID. |  |

Table 7.3.1.4.1-2 specifies data types re-used by the SS_UserProfileRetrieval API service.

Table 7.3.1.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

##### 7.3.1.4.2	Structured data types

###### 7.3.1.4.2.1	Introduction

###### 7.3.1.4.2.2	Type: ProfileDoc

Table 7.3.1.4.2.2-1: Definition of type ProfileDoc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| profileInformation | string | M | 1 | Profile information associated with valUserId or valUEId. |  |
| valUserId | string | O | 0..1 | Unique identifier of a VAL user. |  |
| valUEId | string | O | 0..1 | Unique identifier of a VAL UE. |  |
| NOTE: Either "valUserId" or "valUEId" shall be provided. |  |  |  |  |  |

##### 7.3.1.4.3	Simple data types and enumerations

None.

#### 7.3.1.5	Error Handling

General error responses are defined in clause 6.7.

#### 7.3.1.6	Feature negotiation

General feature negotiation procedures are defined in clause 6.8.

Table 7.3.1.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 7.4	Network resource management APIs

### 7.4.1	SS_Network_Resource_Adaptation API

#### 7.4.1.1	API URI

#### 7.4.1.2	Resources

##### 7.4.1.2.1	Overview

Figure 7.4.1.2.1-1: Resource URI structure of the SS_NetworkResourceAdaptation API

Table 7.4.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.4.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Multicast Subscriptions | {apiRoot}/ ss-nra/{apiVersion} /multicast-subscriptions | POST | Create a new Individual Multicast Subscription resource. |
| Individual Multicast Subscription | {apiRoot}/ ss-nra/{apiVersion} /multicast-subscriptions/{multiSubId} | GET | Read the Individual Multicast Subscription resource. |
|  |  | DELETE | Remove a new Individual Multicast Subscription resource. |

##### 7.4.1.2.2	Resource: Multicast Subscriptions

###### 7.4.1.2.2.1	Description

###### 7.4.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-nra/{apiVersion}/multicast-subscriptions

This resource shall support the resource URI variables defined in table 7.4.1.2.2.2-1.

Table 7.4.1.2.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 7.4.1.1 |
| apiVersion | See clause 7.4.1.1 |

###### 7.4.1.2.2.3	Resource Standard Methods

7.4.1.2.2.3.1	POST

Table 7.4.1.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.4.1.2.2.3.1-2 and the response data structures and response codes specified in table 7.4.1.2.2.3.1-3.

Table 7.4.1.2.2.3.1-2: Data structures supported by the POST  Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MulticastSubscription | M | 1 |  |

Table 7.4.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MulticastSubscription | M | 1 | 201 Created |  |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [22] shall also apply. |  |  |  |  |

###### 7.4.1.2.2.4		Resource Custom Operations

##### 7.4.1.2.3	Resource: Individual Multicast Subscription

###### 7.4.1.2.3.1	Description

###### 7.4.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-nra/{apiVersion}/multicast-subscriptions/{multiSubId}

This resource shall support the resource URI variables defined in table 7.4.1.2.3.2-1.

Table 7.4.1.2.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 7.4.1.1. |
| apiVersion | See clause 7.4.1.1. |
| multiSubId | The multicast subscription identifier. |

###### 7.4.1.2.3.3	Resource Standard Methods

7.4.1.2.3.3.1	GET

Table 7.4.1.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.2.3.3.1-2 and the response data structures and response codes specified in table 7.4.1.2.3.3.1-3.

Table 7.4.1.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.1.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MulticastSubscription | M | 1 | 200 OK |  |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [22] shall also apply. |  |  |  |  |

7.4.1.2.3.3.2	DELETE

Table 7.4.1.2.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.2.3.3.2-2 and the response data structures and response codes specified in table 7.4.1.2.3.3.2-3.

Table 7.4.1.2.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.1.2.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Individual Multicast Subscription resource was deleted. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [22] shall also apply. |  |  |  |  |

###### 7.4.1.2.3.4		Resource Custom Operations

None.

#### 7.4.1.3	Notifications

##### 7.4.1.3.1	General

Table 7.4.1.3.1-1: Notifications overview

| Notification | Resource URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| User Plane notification | {notificationURI} | POST | Report User Plane notification |

##### 7.4.1.3.2	User Plane notification

###### 7.4.1.3.2.1	Description

###### 7.4.1.3.2.2	Notification definition

Resource URI: {notificationURI}

This method shall support the URI query parameters specified in table 7.4.1.3.2.2-1.

Table 7.4.1.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.3.2.2-2 and the response data structures and response codes specified in table 7.4.1.3.2.2-3.

Table 7.4.1.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UserPlaneNotification | M | 1 |  |

Table 7.4.1.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [22] also apply. |  |  |  |  |

#### 7.4.1.4	Data Model

##### 7.4.1.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API

Table 7.4.1.4.1-1 specifies the data types defined specifically for the SS_Network_Resource_Adaptation API service.

Table 7.4.1.4.1-1: SS_Network_Resource_Adaptation API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| DeliveryMode | 7.4.1.4.3.2 |  |  |
| MulticastSubscription | 7.4.1.4.2.2 |  |  |
| NrmEvent | 7.4.1.4.3.3 |  |  |
| NrmEventNotification | 7.4.1.4.2.4 |  |  |
| ServiceAnnoucementMode | 7.4.1.4.3.1 |  |  |
| UserPlaneNotification | 7.4.1.4.2.3 |  |  |

Table 7.4.1.4.1-2 specifies data types re-used by the SS_Network_Resource_Adaptation API service.

Table 7.4.1.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.571 [21] |  |  |
| MbmsLocArea | 3GPP TS 29.122 [3] |  |  |
| SupportedFeatures | 3GPP TS 29.571 [21] |  |  |
| Uint32 | 3GPP TS 29.571 [21] |  |  |
| Uri | 3GPP TS 29.571 [21] |  |  |
| WebsockNotifConfig | 3GPP TS 29.122 [3] |  |  |

##### 7.4.1.4.2	Structured data types

###### 7.4.1.4.2.1	Introduction

###### 7.4.1.4.2.2	Type:  MulticastSubscription

Table 7.4.1.4.2.2-1: Definition of type MulticastSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valGroupId | string | M | 1 | The identity of the group that the MBMS bearer is requested for. |  |
| anncMode | ServiceAnnoucementMode | M | 1 | Indicates whether the service announcement is sent by NRM server or by the VAL server. |  |
| qosReq | QosRequirement | M | 1 | The QoS requirement for the multicast. |  |
| locArea | MbmsLocArea | O | 0..1 | Indicate the area where the MBMS bearer is requested for. |  |
| duration | DateTime | O | 0..1 | Identifies the absolute time at which the subscription resource is considered to expire. When omitted in the request, it indicates the resource is requested to be valid forever by the VAL server. When omitted in the response, it indicates the resource is set to valid forever by the VAL server. |  |
| tmgi | Uint32 | O | 0..1 | TMGI. Shall be provided by the NRM server if announcement mode is set to VAL. |  |
| notifUri | Uri | M | 1 | Identifies the notification URI where the NRM notification shall be sent to. |  |
| reqTestNotif | boolean | O | 0..1 | Set to true by the NF service consumer to request the VAE server to send a test notification as defined in clause 6.3.5.3. Set to false or omitted otherwise. | Notification_test_event |
| wsNotifCfg | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in clause 6.3.5.4. | Notification_websocket |
| suppFeat | SupportedFeatures | M | 1 | This parameter shall be supplied by VAL server in the POST request that request the creation of a Multicast Subscription resource and shall be supplied in the reply of corresponding request. |  |

Editor's note:	definition of QosRequirement is FFS.

Editor's note:	The service description and UP information provided in the response is FFS.

###### 7.4.1.4.2.3	Type: UserPlaneNotification

Table 7.4.1.4.2.3-1: Definition of type UserPlaneNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifId | Uri | M | 1 | The subscription resource Uri to which this notification is related. |  |
| eventNotifs | array(NrmEventNotification) | M | 1..N | Notifications about Individual Events |  |

###### 7.4.1.4.2.4	Type: NrmEventNotification

Table 7.4.1.4.2.4-1: Definition of type NrmEventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | NrmEvent | M | 1 | Event that is notified. |  |
| ts | DateTime | M | 1 | Time at which the event is observed. |  |
| deliveryMode | DeliveryMode | C | 0..1 | Indicates delivery of the user data to the UE(s) via unicast mode or multicast mode. Shall be present if event is UP_DELIVERY_MODE. |  |
| streamIds | array(string) | O | 1..N | Indicates the media streams (unicast or multicast) to be used. May be present if event is UP_DELIVERY_MODE and NRM already has the streams available. |  |

##### 7.4.1.4.3	Simple data types and enumerations

###### 7.4.1.4.3.1	Enumeration: ServiceAnnoucementMode

Table 7.4.1.4.3.1-1: Enumeration ServiceAnnoucementMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| NRM | NRM server performs the service announcement. |  |
| VAL | VAL server performs the service announcement. |  |

###### 7.4.1.4.3.2	Enumeration: DeliveryMode

Table 7.4.1.4.3.2-1: Enumeration DeliveryMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UNICAST | Unicast delivery |  |
| MULTICAST | Multicast delivery |  |

###### 7.4.1.4.3.3	Enumeration: NrmEvent

Table 7.4.1.4.3.3-1: Enumeration NrmEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UP_DELIVERY_MODE | User Plane delivery mode. |  |

#### 7.4.1.5	Error Handling

#### 7.4.1.6	Feature negotiation

Table 7.4.1.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | The testing of notification connection is supported according to clause 6.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 6.6. This feature requires that the Notification_test_event feature is also supported. |

## 7.5	Event APIs

### 7.5.1	SS_Events API

#### 7.5.1.1	API URI

The request URI use in each HTTP request from the VAL server towards the SEAL server shall use the structure as defined in clause 6.5 with the following clarifications:

-	The {apiName} shall be "ss-events".

-	The {apiVersion} shall be "v1".

-	The {apiSpecificSuffixes} shall be set as described in clause 7.5.1.2.

#### 7.5.1.2	Resources

##### 7.5.1.2.1	Overview

Figure 7.5.1.2.1-1: Resource URI structure of the SS_Events API

Table 7.5.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.5.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| SEAL Events Subscriptions | {apiRoot} /ss-events/{apiVersion} /subscriptions | POST | Creates a new individual SEAL Event Subscription. |
| Individual SEAL Events Subscription | {apiRoot} /ss-events/{apiVersion} /subscriptions/{subscriptionId} | DELETE | Deletes an individual SEAL Event Subscription identified by the subscriptionId. |

##### 7.5.1.2.2	Resource: SEAL Events Subscriptions

###### 7.5.1.2.2.1	Description

The SEAL Events Subscriptions represents all event subscriptions on the SEAL server.

###### 7.5.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-events/{apiVersion}/subscriptions

This resource shall support the resource URI variables defined in the table 7.5.1.2.2.2-1.

Table 7.5.1.2.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.5 |

###### 7.5.1.2.2.3	Resource Standard Methods

7.5.1.2.2.3.1	POST

This method shall support the URI query parameters specified in the table 7.5.1.2.2.3.1-1.

Table 7.5.1.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.5.1.2.2.3.1-2 and the response data structures and response codes specified in table 7.5.1.2.2.3.1-3.

Table 7.5.1.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SEALEventSubscription | M | 1 | Create a new individual SEAL Events Subscription resource. |

Table 7.5.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SEALEventSubscription | M | 1 | 201 Created | SEAL Events Subscription resource created successfully. The URI of the created resource shall be returned in the "Location" HTTP header |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

###### 7.5.1.2.2.4	Resource Custom Operations

None.

##### 7.5.1.2.3	Resource: Individual SEAL Events Subscription

###### 7.5.1.2.3.1	Description

The Individual SEAL Events Subscription resource represents an individual event subscription of a VAL server.

###### 7.5.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-events/{apiVersion}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in the table 7.5.1.2.3.2-1.

Table 7.5.1.2.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.5 |
| SubscriptionId | String identifying an Individual Events Subscription |

###### 7.5.1.2.3.3	Resource Standard Methods

7.5.1.2.3.3.1	DELETE

This method shall support the URI query parameters specified in table 7.5.1.2.3.3.1-1.

Table 7.5.1.2.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.5.1.2.3.3.1-2 and the response data structures and response codes specified in table 7.5.1.2.3.3.1-3.

Table 7.5.1.2.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.5.1.2.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual SEAL Events Subscription matching the subscriptionId is deleted. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

###### 7.5.1.2.3.4	Resource Custom Operations

None.

#### 7.5.1.3	Notifications

##### 7.5.1.3.1	General

The delivery of notifications shall conform to clause 6.6.

Table 7.5.1.3.1-1: Notifications overview

| Notification | Resource URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| SEAL Event Notification | {notificationDestination} | POST | Notifies subscriber of a SEAL Event |

##### 7.5.1.3.2	SEAL Event Notification

###### 7.5.1.3.2.1	Description

SEAL Event Notification is used by the SEAL server notify a VAL server of an Event. The VAL server shall be subscribed to such SEAL Event Notifications via the Individual SEAL Events Subscription Resource.

###### 7.5.1.3.2.2	Notification definition

The POST method shall be used for the event notification and the URI shall be the one provided by the VAL server during the subscription to the event.

Resource URI: {notificationDestination}

This method shall support the URI query parameters specified in table 7.5.1.3.2.2-1.

Table 7.5.1.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.5.1.3.2.2-2 and the response data structures and response codes specified in table 7.5.1.3.2.2-3.

Table 7.5.1.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SEALEventNotification | M | 1 | Notification information of a SEAL Event |

Table 7.5.1.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

#### 7.5.1.4	Data Model

##### 7.5.1.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.5.1.4.1-1 specifies the data types defined specifically for the SS_Events API service.

Table 7.5.1.4.1-1: SS_Events API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| SEALEventSubscription | 7.5.1.4.2.2 | Represents an individual SEAL Event Subscription resource |  |
| SEALEventNotification | 7.5.1.4.2.3 | Represents an individual SEAL Event Subscription Notification |  |
| EventSubscription | 7.5.1.4.2.4 | Represents the subscription to a single SEAL event. |  |
| SEALEventDetail | 7.5.1.4.2.5 | Represents the SEAL event detail |  |
| VALGroupFilter | 7.5.1.4.2.6 | Represents a filter of VAL group identifiers belonging to a VAL service. |  |
| IdentityFilter | 7.5.1.4.2.7 | Represents a filter of VAL User / UE identities belonging to a VAL service. |  |
| SEALEvent | 7.5.1.4.3.3 | Represents the type of SEAL events that can be subscribed. |  |

Table 7.5.1.4.1-2 specifies data types re-used by the SS_Events API service:

Table 7.5.1.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ReportingInformation | 3GPP TS 29.523 [20] | Used to indicate the reporting requirement, only the following information are applicable for SEAL: - immRep - notifMethod - maxReportNbr - monDur - repPeriod |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the applicability of optional features defined in table 7.5.1.6-1. |  |
| TestNotification | 3GPP TS 29.122 [3] | Following differences apply: - The SCEF is the SEAL server; and - The SCS/AS is the subscribing VAL server. |  |
| Uri | 3GPP TS 29.122 [3] |  |  |
| WebsockNotifConfig | 3GPP TS 29.122 [3] | Following differences apply: - The SCEF is the CAPIF core function; and - The SCS/AS is the Subscribing functional entity. |  |
| VALGroupDocument | Clause 7.2.1.4.2.2 | Used to send VAL group document as part of event detail in the event notification. |  |
| ProfileDoc | Clause 7.3.1.4.2.2 | Used to send VAL User or VAL UE profile information as part of event detail in the event notification. |  |

##### 7.5.1.4.2	Structured data types

###### 7.5.1.4.2.1	Introduction

###### 7.5.1.4.2.2	SEALEventSubscription

Table 7.5.1.4.2.2-1: Definition of type SEALEventSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscriberId | string | M | 1 | String identifying the subscriber of the event. |  |
| eventSubs | array(EventSubscription) | M | 1..N | Subscribed events. |  |
| eventReq | ReportingInformation | M | 1 | Represents the reporting requirements of the event subscription. |  |
| notificationDestination | Uri | M | 1 | URI where the notification should be delivered to. |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by Subscriber to request the SEAL server to send a test notification as defined in clause 6.6. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in clause 6.6. | Notification_websocket |
| suppFeat | SupportedFeatures | O | 0..1 | Used to negotiate the supported optional features of the API as described in clause 6.8. This attribute shall be provided in the HTTP POST request and in the response of successful resource creation. |  |

###### 7.5.1.4.2.3	SEALEventNotification

Table 7.5.1.4.2.3-1: Definition of type SEALEventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscriptionId | string | M | 1 | Identifier of the subscription resource to which the notification is related – SEAL resource identifier |  |
| eventDetails | array(SEALEventDetail) | M | 1..N | Detailed notifications of individual Events. |  |

###### 7.5.1.4.2.4	EventSubscription

Table 7.5.1.4.2.4-1: Definition of type EventSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventId | SEALEvent | M | 1 | Subscribed event |  |
| valGroups | array(VALGroupFilter) | C | 1..N | Each element of the array represents the VAL group identifier(s) of a VAL service that the subscriber wants to know in the interested event. This parameter shall be present only if the event subscribed is “GM_GROUP_INFO_CHANGE”. |  |
| Identities | array(IdentityFilter) | C | 1..N | Each element of the array represents the VAL User / UE IDs of a VAL service that the event subscriber wants to know in the interested event. This parameter shall be present only if the event subscribed is “CM_USER_PROFILE_CHANGE”. |  |

###### 7.5.1.4.2.5	SEALEventDetail

Table 7.5.1.4.2.5-1: Definition of type SEALEventDetail

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventId | SEALEvent | M | 1 | Event that is notified |  |
| valGroupDocuments | array(VALGroupDocument) | C | 1..N | The VAL groups documents with modified membership and configuration information. This parameter shall be present only if the event in event notification is “GM_GROUP_INFO_CHANGE”. |  |
| profileDocs | array(ProfileDoc) | C | 1..N | Updated profile information associated with VAL Users or VAL UEs. This parameter shall be present only if the event in event notification is “CM_USER_PROFILE_CHANGE”. |  |

###### 7.5.1.4.2.6	VALGroupFilter

Table 7.5.1.4.2.6-1: Definition of type VALGroupFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcId | string | O | 0..1 | Identity of the VAL Service that the subscriber is interested in. |  |
| valGrpIds | array(string) | M | 1..N | VAL Group identifiers that the event subscriber wants to know in the interested event. |  |

###### 7.5.1.4.2.7	IdentityFilter

Table 7.5.1.4.2.7-1: Definition of type IdentityFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcId | string | O | 0..1 | Identity of the VAL Service that the subscriber is interested in. |  |
| valUserIds | array(string) | C | 1..N | VAL User IDs that the event subscriber wants to know in the interested event. (NOTE) |  |
| valUEIds | array(string) | C | 1..N | VAL UE IDs that the event subscriber wants to know in the interested event. (NOTE) |  |
| NOTE: Only one of "valUserIds" or "valUEIds" shall be present, but not both. |  |  |  |  |  |

##### 7.5.1.4.3	Simple data types and enumerations

###### 7.5.1.4.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.5.1.4.3.2	Simple data types

None.

###### 7.5.1.4.3.3	Enumeration: SEALEvent

Table 7.5.1.4.3.3-1: Enumeration SEALEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| LM_LOCATION_INFO | Events related to the location information of VAL Users or VAL UEs from the Location Management Server. |  |
| GM_GROUP_INFO_CHANGE | Events related to the modification of VAL group membership and configuration information from the Group Management Server. |  |
| CM_USER_PROFILE_CHANGE | Events related to update of user profile information from the Configuration Management Server. |  |

#### 7.5.1.5	Error Handling

General error responses are defined in clause 6.7.

#### 7.5.1.6	Feature Negotiation

General feature negotiation procedures are defined in clause 6.8. Table 7.5.1.6-1 lists the supported features for SS_Events API.

Table 7.5.1.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to clause 6.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 6.6. This feature requires that the Notification_test_event feature is also supported. |

## 7.6	Identity management APIs

Editor's note: Definitions of Identity management APIs is FFS

## 7.7	Key management APIs

Editor's note: Definitions of Identity management APIs is FFS

# 8	Using Common API Framework

## 8.1	General

When CAPIF is used with a SEAL service, the SEAL server shall support the following as defined in 3GPP TS 29.222 [16]:

-	the API exposing function and related APIs over CAPIF-2/2e and CAPIF-3/3e reference points;

-	the API publishing function and related APIs over CAPIF-4/4e reference point;

-	the API management function and related APIs over CAPIF-5/5e reference point; and

-	at least one of the security methods for authentication and authorization, and related security mechanisms.

In a centralized deployment as defined in 3GPP TS 23.222 [17], where the CAPIF core function and API provider domain functions are co-located, the interactions between the CAPIF core function and API provider domain functions may be independent of CAPIF-3/3e, CAPIF-4/4e and CAPIF-5/5e reference points.

When CAPIF is used with a SEAL service, the SEAL server shall register all the features for northbound APIs in the CAPIF Core Function.

## 8.2	Security

When CAPIF is used for external exposure, before invoking the API exposed by the SEAL server, the VAL server as API invoker shall negotiate the security method (PKI, TLS-PSK or OAUTH2) with CAPIF core function and ensure the SEAL server has enough credential to authenticate the VAL server (see 3GPP TS 29.222 [16], clause 5.6.2.2 and clause 6.2.2.2).

If PKI or TLS-PSK is used as the selected security method between the VAL server and the SEAL server, upon API invocation, the SEAL server shall retrieve the authorization information from the CAPIF core function as described in 3GPP TS 29.222 [16], clause 5.6.2.4.

As indicated in 3GPP TS 33.122 [18], the access to the SEAL APIs may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [19]), using the "Client Credentials" authorization grant, where the CAPIF core function (see 3GPP TS 29.222 [16]) plays the role of the authorization server.

NOTE 1:	In this release, only "Client Credentials" authorization grant is supported.

If OAuth2 is used as the selected security method between the VAL server and the SEAL server, the VAL server, prior to consuming services offered by the SEAL APIs, shall obtain a "token" from the authorization server, by invoking the Obtain_Authorization service, as described in 3GPP TS 29.222 [16], clause 5.6.2.3.2.

The SEAL APIs do not define any scopes for OAuth2 authorization. It is the SEAL server responsibility to check whether the VAL server is authorized to use an API based on the "token". Once the SEAL server verifies the "token", it shall check whether the SEAL server identifier in the "token" matches its own published identifier, and whether the API name in the "token" matches its own published API name. If those checks are passed, the VAL server has full authority to access any resource or operation for the invoked API

NOTE 2:	For aforementioned security methods, the SEAL server needs to apply admission control according to access control policies after performing the authorization checks.

Annex A (normative):
OpenAPI specification

## A.1	General

This annex is based on the OpenAPI 3.0.0 specification [15] and provides corresponding representations of all APIs defined in the present specification in YAML format.

## A.2	SS_GroupManagement API

openapi: 3.0.0

info:

title: SS_GroupManagement

description: |

API for SEAL Group management.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0.alpha-1"

externalDocs:

description: 3GPP TS 29.549 V1.1.0 Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage 3.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/group-management/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/group-documents:

post:

description: Creates a new VAL group document.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VALGroupDocument'

responses:

'201':

description: VAL group created sucessfully.

content:

application/json:

schema:

$ref: '#/components/schemas/VALGroupDocument'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/group-management/v1/group-documents/{groupDocId}'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/group-documents/{groupDocId}:

get:

description: Retrieves VAL group information satisfying filter criteria

parameters:

- name: groupDocId

in: path

description: String identifying an individual VAL group document resource

required: true

schema:

type: string

- name: group-members

in: query

description: When set to true indicates the group management server to send the members list information of the VAL group.

schema:

type: boolean

- name: group-configuration

in: query

description: When set to true indicates the group management server to send the group configuration information of the VAL group.

schema:

type: boolean

responses:

'200':

description: The VAL group information based on the request from the VAL server. Includes VAL group members list if group-members flag is set to true in the request, VAL group configuration information if the group-configuration flag is set to true in the request, VAL group identifier, whole VAL group document resource if both group-members and group-configuration flags are omitted/set to false in the request.

content:

application/json:

schema:

$ref: '#/components/schemas/VALGroupDocument'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Updates an individual VAL group document.

parameters:

- name: groupDocId

in: path

description: String identifying an individual VAL group document resource

required: true

schema:

type: string

requestBody:

description: VAL group document to be updated in Group management server.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VALGroupDocument'

responses:

'200':

description: VAL group document updated successfully

content:

application/json:

schema:

$ref: '#/components/schemas/VALGroupDocument'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Deletes a VAL Group.

parameters:

- name: groupDocId

in: path

description: String identifying an individual VAL group document resource

required: true

schema:

type: string

responses:

'204':

description: The individual VAL group matching groupDocId was deleted.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

VALGroupDocument:

type: object

properties:

valGroupId:

type: string

description: The VAL group idenitity.

grpDesc:

type: string

description: The text description of the VAL group.

membersList:

type: array

description: The list of VAL User IDs and VAL UE IDs, which are members of the VAL group.

items:

type: string

minItems: 1

valGrpConf:

type: string

description: Configuration data for the VAL group.

valServiceIds:

type: array

description: The list of VAL services enabled on the group.

items:

type: string

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- valGroupId

## A.3	SS_UserProfileRetrieval API

openapi: 3.0.0

info:

title: SS_UserProfileRetrieval

description: |

API for SEAL Configuration management.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0.alpha-1"

externalDocs:

description: 3GPP TS 29.549 V1.1.0 Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage 3.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/configuration-management/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/{valServiceId}:

get:

description: Retrieve a VAL User or VAL UE's profile information belonging to a VAL service.

parameters:

- name: valServiceId

in: path

description: String identifying an individual VAL service resource

required: true

schema:

type: string

- name: valUserId

in: query

description: String identifying a VAL User. Shall be present if valUEId query parameter is not present.

schema:

type: string

- name: valUEId

in: query

description: String identifying a VAL UE. Shall be present if valUserId query parameter is not present.

schema:

type: string

responses:

'200':

description: The Profile information of the VAL User or VAL UE corresponding to the identifier in the request.

content:

application/json:

schema:

$ref: '#/components/schemas/ProfileDoc'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

ProfileDoc:

type: object

properties:

profileInformation:

type: string

description: Profile information associated with the valUserId or valUEId.

valUserId:

type: string

description: Unique identifier of a VAL user.

valUEId:

type: string

description: Unique identifuer of a VAL UE.

required:

- profileInformation

oneOf:

- required: [valUserId]

- required: [valUEId]

## A.4	SS_NetworkResourceAdaptation API

openapi: 3.0.0

info:

version: 1.0.0.alpha-1

title: SS_NetworkResourceAdaptation

description: |

SS Network Resource Adaptation Service.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.549 V1.1.0; Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage 3.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-nra/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

paths:

/multicast-subscriptions:

post:

summary: Creates a new Individual Multicast Subscription resource

operationId: CreateMulticastSubscription

tags:

- Multicast Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MulticastSubscription'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/MulticastSubscription'

headers:

Location:

description: 'Contains the URI of the created individual multicast subscription resource'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

UserPlaneNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UserPlaneNotification'

responses:

'204':

description: No Content, Notification was succesfull

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/multicast-subscriptions/{multiSubId}:

get:

summary: "Reads an existing Individual Multicast Subscription"

operationId: GetMulticastSubscription

tags:

- Individual Multicast Subscription (Document)

parameters:

- name: multiSubId

in: path

description: Multicast Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/MulticastSubscription'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: "Delete an existing Individual Multicast Subscription"

operationId: DeleteMulticastSubscription

tags:

- Individual Multicast Subscription (Document)

parameters:

- name: multiSubId

in: path

description: Multicast Subscription ID

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was succesfully deleted

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

MulticastSubscription:

type: object

properties:

valGroupId:

type: string

anncMode:

$ref: '#/components/schemas/ServiceAnnoucementMode'

qosReq:

$ref: '#/components/schemas/QosRequirement'

locArea:

$ref: 'TS29122_GMDviaMBMSbyMB2.yaml#/components/schemas/MbmsLocArea'

duration:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

tmgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

reqTestNotif:

type: boolean

wsNotifCfg:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- valGroupId

- anncMode

- qosReq

- notifUri

UserPlaneNotification:

type: object

properties:

notifId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

eventNotifs:

type: array

items:

$ref: '#/components/schemas/NrmEventNotification'

minItems: 1

required:

- notifId

- eventNotifs

NrmEventNotification:

type: object

properties:

event:

$ref: '#/components/schemas/NrmEvent'

ts:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

deliveryMode:

$ref: '#/components/schemas/DeliveryMode'

streamIds:

type: array

items:

type: string

minItems: 1

required:

- event

- ts

QosRequirement:

type: string

# QoS requirment definition is FFS

# Simple data types and Enumerations

ServiceAnnoucementMode:

anyOf:

- type: string

enum:

- NRM

- VAL

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- NRM: NRM server performs the service announcement.

- VAL: VAL server performs the service announcement.

DeliveryMode:

anyOf:

- type: string

enum:

- UNICAST

- MULTICAST

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- UNICAST: Unicast delivery.

- MULTICAST: Multicast delivery.

NrmEvent:

anyOf:

- type: string

enum:

- UP_DELIVERY_MODE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- UP_DELIVERY_MODE: User Plane delivery mode.

## A.5	SS_Events API

openapi: 3.0.0

info:

title: SS_Events

description: |

API for SEAL Events management.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0.alpha-1"

externalDocs:

description: 3GPP TS 29.549 V1.1.0 Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage 3.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/ss-events/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/subscriptions:

post:

description: Creates a new individual SEAL Event Subscription.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SEALEventSubscription'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SEALEventNotification'

responses:

'204':

description: No Content (successful notification)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: SEAL Events subscription resource created successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/SEALEventSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-events/v1/subscriptions/{subscriptionId}'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

delete:

description: Deletes an individual SEAL Event Subscription.

parameters:

- name: subscriptionId

in: path

description: Identifier of an individual Events Subscription

required: true

schema:

type: string

responses:

'204':

description: The individual SEAL Events Subscription matching the subscriptionId is deleted.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

SEALEventSubscription:

type: object

properties:

subscriberId:

type: string

description: String identifying the subscriber of the event.

eventSubs:

type: array

items:

$ref: '#/components/schemas/EventSubscription'

minItems: 1

description: Subscribed events.

eventReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: Set to true by Subscriber to request the SEAL server to send a test notification as defined in in clause 6.6. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- subscriberId

- eventSubs

- eventReq

- notificationDestination

SEALEventNotification:

type: object

properties:

subscriptionId:

type: string

description: Identifier of the subscription resource to which the notification is related – SEAL resource identifier

eventDetails:

type: array

items:

$ref: '#/components/schemas/SEALEventDetail'

minItems: 1

description: Detailed notifications of individual events.

required:

- subscriptionId

- eventDetails

EventSubscription:

type: object

properties:

eventId:

$ref: '#/components/schemas/SEALEvent'

valGroups:

type: array

items:

$ref: '#/components/schemas/VALGroupFilter'

minItems: 1

description: Each element of the array represents the VAL group identifier(s) of a VAL service that the subscriber wants to know in the interested event. This parameter shall be present only if the event subscribed is “GM_GROUP_INFO_CHANGE”.

identities:

type: array

items:

$ref: '#/components/schemas/IdentityFilter'

minItems: 1

description: Each element of the array represents the VAL User / UE IDs of a VAL service that the event subscriber wants to know in the interested event. This parameter shall be present only if the event subscribed is “CM_USER_PROFILE_CHANGE”.

required:

- eventId

SEALEventDetail:

type: object

properties:

eventId:

$ref: '#/components/schemas/SEALEvent'

valGroupDocuments:

type: array

items:

$ref: 'TS29549_SS_GroupManagement.yaml#/components/schemas/VALGroupDocument'

minItems: 1

description: The VAL groups documents with modified membership and configuration information.This parameter shall be present only if the event in event notification is “GM_GROUP_INFO_CHANGE”.

profileDocs:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ProfileDoc'

minItems: 1

description: Updated profile information associated with VAL Users or VAL UEs. This parameter shall be present only if the event in event notification is “CM_USER_PROFILE_CHANGE”.

required:

- eventId

VALGroupFilter:

type: object

properties:

valSvcId:

type: string

description: Identity of the VAL service

valGrpIds:

type: array

items:

type: string

minItems: 1

description: VAL group identifiers that event subscriber wants to know in the interested event.

required:

- valGrpIds

IdentityFilter:

type: object

properties:

valSvcId:

type: string

description: Identity of the VAL service

valUserIds:

type: array

items:

type: string

minItems: 1

description: VAL User IDs that the event subscriber wants to know in the interested event.

valUEIds:

type: array

items:

type: string

minItems: 1

description: VAL UE IDs that the event subscriber wants to know in the interested event.

oneOf:

- required: [valUserIds]

- required: [valUEIds]

SEALEvent:

anyOf:

- type: string

enum:

- LM_LOCATION_INFO

- GM_GROUP_INFO_CHANGE

- CM_USER_PROFILE_CHANGE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- LM_LOCATION_INFO: Events related to the location information of VAL Users or VAL UEs from the Location Management Server.

- GM_GROUP_INFO_CHANGE: Events related to the modification of VAL group membership and configuration information from the Group Management Server.

- CM_USER_PROFILE_CHANGE: Events related to update of user profile information from the Configuration Management Server.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-10 | CT3#106 |  |  |  |  | TS skeleton for Services Enabler Architecture Layer for Verticals Application Programming Interface specification. | 0.0.0 |
| 2019-10 | CT3#106 | C3-194418 |  |  |  | Inclusion of TS skeleton document with clauses reflecting SEAL service APIs, agreed in the meeting CT3#106: C3-194418 | 0.1.0 |
| 2019-10 | CT3#106 | C3-194314 |  |  |  | Inclusion of documents agreed in CT3#106: C3-194297, C3-194298, C3-194299, C3-194300 | 0.2.0 |
| 2019-11 | CT3#107 | C3-195307 |  |  |  | Inclusion of documents agreed in CT3#107: C3-195157, C3-195260, C3-195441, C3-195262, C3-195263, C3-195264, C3-195185 | 0.3.0 |
| 2019-12 | CT#86 | CP-193176 |  |  |  | Sent to plenary for Information | 1.0.0 |
| 2020-03 | CT3#108e |  |  |  |  | Inclusion of documents agreed in CT3#108-e meeting: C3-201346, C3-201347, C3-201348, C3-201349, C3-201350, C3-201456, C3-201457, C3-201351, C3-201352, C3-201271 | 1.1.0 |