3GPP TS 29.551 V16.3.0 (2020-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System; Packet Flow Description Management Service;

Stage 3

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Packet Flow Description Management Service	6

4.1	Service Description	6

4.1.1	Overview	6

4.1.2	Service Architecture	7

4.1.3	Network Functions	7

4.1.3.1	Packet Flow Description Function (PFDF)	7

4.1.3.2	NF Service Consumers	8

4.2	Service Operations	8

4.2.1	Introduction	8

4.2.2	Nnef_PFDmanagement_Fetch Service Operation	8

4.2.2.1	General	8

4.2.2.2	Retrieval of PFDs	8

4.2.3	Nnef_PFDmanagement_Subscribe Service Operation	9

4.2.3.1	General	9

4.2.3.2	Subscription for event notifications on PFDs change	9

4.2.3.3	Subscription update for event notifications on PFDs change	10

4.2.4	Nnef_PFDmanagement_Notify Service Operation	10

4.2.4.1	General	10

4.2.4.2	Notification of PFD change	11

4.2.5	Nnef_PFDmanagement_Unsubscribe Service Operation	11

4.2.5.1	General	11

4.2.5.2	Unsubscribe from event notifications on PFDs change	11

5	Nnef_PFDmanagement API	12

5.1	Introduction	12

5.2	Usage of HTTP	12

5.2.1	General	12

5.2.2	HTTP standard headers	12

5.2.2.1	General	12

5.2.2.2	Content type	12

5.2.3	HTTP custom headers	12

5.3	Resources	13

5.3.1	Resource Structure	13

5.3.2	Resource: PFD of applications	13

5.3.2.1	Description	13

5.3.2.2	Resource definition	14

5.3.2.3	Resource Standard Methods	14

5.3.2.3.1	GET	14

5.3.2.4	Resource Custom Operations	14

5.3.3	Resource: Individual application PFD	14

5.3.3.1	Description	14

5.3.3.2	Resource definition	14

5.3.3.3	Resource Standard Methods	15

5.3.3.3.1	GET	15

5.3.3.4	Resource Custom Operations	15

5.3.4	Resource: PFD subscriptions	15

5.3.4.1	Description	15

5.3.4.2	Resource definition	15

5.3.4.3	Resource Standard Methods	16

5.3.4.3.1	POST	16

5.3.4.4	Resource Custom Operations	16

5.3.5	Resource: Individual PFD subscription	16

5.3.5.1	Description	16

5.3.5.2	Resource definition	16

5.3.5.3	Resource Standard Methods	16

5.3.5.3.1	DELETE	16

5.3.5.3.2	PUT	17

5.3.5.4	Resource Custom Operations	17

5.4	Custom Operations without associated resources	17

5.5	Notifications	18

5.5.1	General	18

5.5.2	PFD Change Notification	18

5.5.2.1	Description	18

5.5.2.2	Target URI	18

5.5.2.3	Standard Methods	18

5.5.2.3.1	POST	18

5.6	Data Model	19

5.6.1	General	19

5.6.2	Structured data types	19

5.6.2.1	Introduction	19

5.6.2.2	Type: PfdDataForApp	20

5.6.2.3	Type: PfdSubscription	20

5.6.2.4	Type: PfdChangeNotification	20

5.6.2.5	Type: PfdContent	21

5.6.2.6	Type: PfdChangeReport	21

5.6.3	Simple data types and enumerations	21

5.6.3.1	Introduction	21

5.6.3.2	Simple data types	21

5.7	Error handling	22

5.7.1	General	22

5.7.2	Protocol Errors	22

5.7.3	Application Errors	22

5.8	Feature negotiation	22

5.9	Security	23

Annex A (normative):	OpenAPI specification	24

A.1	General	24

A.2	Nnef_PFDmanagement API	24

Annex B (informative):	Change history	30

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document provides the stage 3 specification of the PFD Management Service of 5G system.

The stage 2 definition and related procedures of the PFD Management Service are contained in 3GPP TS 23.502 [3] and 3GPP TS 23.503 [4]. The 5G System Architecture is defined in 3GPP TS 23.501 [2].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition of the 5G System are specified in 3GPP TS 29.500 [5] and 3GPP TS 29.501 [6].

The Packet Flow Description Function (PFDF) provides the PFD Management Service to the NF consumers (i.e. Session Management Function). The PFDF is functionality within the NEF.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[6]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[7]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[8]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[9]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[10]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[11]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[12]	3GPP TS 29.251: "Gw and Gwn reference points for sponsored data connectivity".

[13]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[14]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[15]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[16]	IETF RFC 7807: "Problem Details for HTTP APIs".

[17]	3GPP TR 21.900: "Technical Specification Group working methods".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

JSON	JavaScript Object Notation

NEF	Network Exposure Function

NRF	Network Repository Function

PFD	Packet Flow Description

PFDF	Packet Flow Description Function

SBI	Service Based Interface

SMF	Session Management Function

# 4	Packet Flow Description Management Service

## 4.1	Service Description

### 4.1.1	Overview

The PFD Management Service, as defined as defined in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.503 [4], is provided by the Packet Flow Description Function (PFDF).

The only known NF Service Consumer is the SMF.

This service:

-	allows an SMF to subscribe to and unsubscribe from PFD changes;

-	notifies an SMF about the changed PFDs; and

-	allows an SMF to retrieve PFDs.

### 4.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Policy and Charging related 5G architecture is also described in 3GPP TS 23.503 [4].

The PFD Management Service is provided by the PFDF to the SMF and shown in the SBI representation model in Figure 4.1.2-1. PFDF is functionality within NEF.

NEF

PFDF

Nnef

SMF

Nnef_PFDmanagement

Figure 4.1.2-1: Reference Architecture for the Nnef_PFDmanagement Service; SBI representation

NEF

PFDF

SMF

N29

Figure 4.1.2-2: Reference Architecture for the Nnef_PFDmanagement Service; reference point representation

### 4.1.3	Network Functions

#### 4.1.3.1	Packet Flow Description Function (PFDF)

The Packet Flow Description Function (PFDF):

-	provides PFDs associated with one or more Application Identifiers; and

-	allows NF consumers to subscribe to and unsubscribe from the notification of changes of PFDs for Application Identifier.

#### 4.1.3.2	NF Service Consumers

The SMF shall support:

-	requesting and receiving the PFD(s) for one or more Application Identifiers.

## 4.2	Service Operations

### 4.2.1	Introduction

Service operations defined for the Nnef_PFDmanagement Service are shown in table 4.2.1-1.

Table 4.2.1-1: Nnef_PFDmanagement Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Nnef_PFDmanagement_Fetch | Provides the PFDs for application identifier(s) to the NF service consumer. | SMF |
| Nnef_PFDmanagement_Subscribe | Allows NF service consumers to subscribe the notification of events when the PFDs for application identifier change. | SMF |
| Nnef_PFDmanagement_Notify | Notifies NF service consumer to update and/or delete the PFDs for application identifier(s). | PFDF |
| Nnef_PFDmanagement_Unsubscribe | Allows NF service consumers to unsubscribe the notification of events. | SMF |

### 4.2.2	Nnef_PFDmanagement_Fetch Service Operation

#### 4.2.2.1	General

The Nnef_PFDmanagement_Fetch service operation provides a means for the NF service consumer to retrieve the PFDs for one or more application identifier(s).

The following procedures using the Nnef_PFDmanagement_Fetch service operation are supported:

-	Retrieval of PFDs.

#### 4.2.2.2	Retrieval of PFDs

This procedure as shown in Figure 4.2.2.2-1 is used to retrieve PFDs for an application identifier from the PFDF.

Figure 4.2.2.2-1: Retrieval of PFDs

1.	The NF service consumer (i.e. SMF) shall send a GET request to the resource representing the PFD for application identifier(s) to be required:

-	for PFD of individual application identifier, the request URI shall include "{apiRoot}/nnef-pfdmanagement/v1/applications/{appId}"; and

-	for PFD of a collection of application identifiers, the request URI shall include "{apiRoot}/nnef-pfdmanagement/v1/applications/" with query parameters indicating the requested application identifier(s).

2.	On success, "200 OK" shall be returned; the payload body of GET response shall contain a representation of "Individual application PFD" resource or "PFD of applications" resource for the requested application identifier(s). If the resource of one or more requested application identifier(s) is not provided in the response, the SMF shall remove the PFD(s) of the requested application identifier(s) which is not included in the response.

On failure, one of the HTTP status code listed in table 5.3.2.3.1-3 or table 5.3.3.3.1-3 shall be returned. For "404 Not Found", the NF service consumer shall remove:

-	all the PFD(s) existing in the NF service consumer if the request is for PFD of all application identifiers;

-	the PFD(s) of the requested application identifier(s) in the NF service consumer if the request is for PFD of individual application identifier or a collection of application identifiers.

### 4.2.3	Nnef_PFDmanagement_Subscribe Service Operation

#### 4.2.3.1	General

The Nnef_PFDmanagement_Subscribe service operation enables NF service consumer to subscribe the notification of events when the PFDs for application identifier change.

#### 4.2.3.2	Subscription for event notifications on PFDs change

This procedure as shown in Figure 4.2.3.2-1 is used to subscribe the notification of events when the PFDs for application identifier change.

Figure 4.2.3.2-1: Creation of a subscription

1.	The NF service consumer (i.e. SMF) shall send a POST request to the request URI representing the PFD subscriptions resource "{apiRoot}/nnef-pfdmanagement/v1/subscriptions". The request payload body shall include:

-	subscribed application identifier(s) optionally; and;

-	an URI where to receive the requested notifications as "notifyUri" attribute.

2.	If the request is accepted, the PFDF shall:

-	create a new subscription;

-	assign an subscriptionId;

-	store the subscription; and

-	send the response with "201 Created". The payload body shall contain a representation of the created subscription, and the Location header shall contain the URI of the created subscription "{apiRoot}/nnef-pfdmanagement/v1/subscriptions/{subscriptionId}".

Otherwise, one of the HTTP status code listed in table 5.3.4.3.1-3 shall be returned.

#### 4.2.3.3	Subscription update for event notifications on PFDs change

This procedure as shown in Figure 4.2.3.3-1 is used to update an existing subscription for the notification of events when the PFDs for application identifier change.

Figure 4.2.3.3-1: Update of a subscription

1.	If the feature PfdChgSubsUpdate is supported, the NF service consumer (i.e. SMF) shall send a PUT request to the request URI representing the PFD subscriptions resource "{apiRoot}/nnef-pfdmanagement/v1/subscriptions/{subscriptionId}". The request payload body shall include:

-	subscribed application identifier(s) optionally; and;

-	an URI where to receive the requested notifications as "notifyUri" attribute.

2.	If the feature PfdChgSubsUpdate is supported and the request is accepted, the PFDF shall:

-	update the subscription; and

-	send the response with "200 OK". The payload body shall contain a representation of the updated subscription.

Otherwise, one of the HTTP status code listed in table 5.3.4.3.1-3 shall be returned.

### 4.2.4	Nnef_PFDmanagement_Notify Service Operation

#### 4.2.4.1	General

The Nnef_PFDmanagement_Notify service operation notifies the NF service consumer to update and/or delete the PFDs for application identifier(s).

The following procedures using the Nnef_PFDmanagement_Notify service operation are supported:

-	Management of PFDs.

#### 4.2.4.2	Notification of PFD change

Figure 4.2.4.2-1: Notification of PFD change

1.	The PFDF shall send a POST request to the NF service consumer (i.e. SMF) using the notifyUri provided during creation of the subscription resource as specified in subclause 5.6.2.3. The payload body of the POST request shall contain the PfdChangeNotification data structure.

2	If the notification is accepted, the NF service consumer shall reply with:

-	"204 No Content" indicating the successful provisioning of all PFDs; or

-	"200 OK" and the payload body of the response shall contain "PfdChangeReport" data structure with detailed information of failed application(s).

Otherwise, one of the HTTP status code listed in table 5.5.2.3.1-3 shall be returned.

### 4.2.5	Nnef_PFDmanagement_Unsubscribe Service Operation

#### 4.2.5.1	General

The Nnef_PFDmanagement_Unsubscribe service operation is used by the NF service consumer to unsubscribe the notification of events.

#### 4.2.5.2	Unsubscribe from event notifications on PFDs change

Figure 4.2.5.2-1: Unsubscribe from event notifications on PFDs change

1.	The NF Service Consumer (i.e. SMF) shall send a DELETE request to the resource URI representing the individual PFD subscription. The request body shall be empty.

2.	If the request is accepted, "204 No Content" shall be returned. The response body shall be empty.

Otherwise, one of the HTTP status code listed in table 5.3.5.3.1-3 shall be returned.

# 5	Nnef_PFDmanagement API

## 5.1	Introduction

The Packet Flow Description Management Service shall use the Nnef_PFDmanagement API.

The request URI used in HTTP request from the NF service consumer towards the PFDF shall have the structure defined in subclause 4.4.1 of 3GPP TS 29.501 [6], i.e.:

{apiRoot}/{apiName}/{apiVersion}/{apiSpecificResourceUriPart}

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [6].

-	The {apiName} shall be "nnef-pfdmanagement".

-	The {apiVersion} shall be "v1".

-	The {apiSpecificResourceUriPart} shall be set as described in subclause 5.3.

## 5.2	Usage of HTTP

### 5.2.1	General

HTTP/2, IETF RFC 7540 [7], shall be used as specified in subclause 5.2 of 3GPP TS 29.500 [5].

HTTP/2 shall be transported as specified in subclause 5.3 of 3GPP TS 29.500 [5].

The OpenAPI [9] specification of HTTP messages and content bodies for the Nnef_PFDmanagement service is contained in Annex A.

### 5.2.2	HTTP standard headers

#### 5.2.2.1	General

See subclause 5.2.2 of 3GPP TS 29.500 [5] for the usage of HTTP standard headers.

#### 5.2.2.2	Content type

JSON, IETF RFC 8259 [8], shall be used as content type of the HTTP bodies specified in the present specification, as specified in subclause 5.4 of 3GPP TS 29.500 [5]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [16].

### 5.2.3	HTTP custom headers

None

## 5.3	Resources

### 5.3.1	Resource Structure

Figure 5.3.1-1: Resource URI structure of the Nnef_PFDmanagement API

Table 5.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| PFD of applications | //{apiRoot}/ nnef-pfdmanagement/v1 /applications | GET | Nnef_PFDmanagement_Fetch. Retrieve PFDs for all applications or for one or multiple applications with query parameter. |
| Individual application PFD | //{apiRoot}/ nnef-pfdmanagement/v1 /applications/{appId} | GET | Nnef_PFDmanagement_Fetch. Retrieve the PFD for an application. |
| PFD subscriptions | //{apiRoot}/ nnef-pfdmanagement/v1 /subscriptions | POST | Nnef_PFDmanagement_Subscribe. Subscribe the notification of PFD changes. |
| Individual PFD subscription | {apiRoot}/ nnef-pfdmanagement/v1 /subscriptions/{subscriptionId} | PUT | Update a subscription of PFD change notification. |
| Individual PFD subscription | //{apiRoot}/ nnef-pfdmanagement/v1 /subscriptions/{subscriptionId} | DELETE | Nnef_PFDmanagement_Unsubscribe. Delete a subscription of PFD change notification. |

### 5.3.2	Resource: PFD of applications

#### 5.3.2.1	Description

This resource represents PFDs for all applications.

#### 5.3.2.2	Resource definition

Resource URI: {apiRoot}/nnef-pfdmanagement/v1/applications

This resource shall support the resource URI variables defined in table 5.3.2.2-1.

Table 5.3.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 5.1 |

#### 5.3.2.3	Resource Standard Methods

##### 5.3.2.3.1	GET

This method shall support the URI query parameters specified in table 5.3.2.3.1-1.

Table 5.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| application-ids | array(ApplicationId) | M | 1..N | The required application identifier(s) for the returned PFDs |
| supported-features | SupportedFeatures | O | 0..1 | To filter irrelevant responses related to unsupported features. |

This method shall support the request data structures specified in table 5.3.2.3.1-2 and the response data structures and response codes specified in table 5.3.2.3.1-3.

Table 5.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(PfdDataforApp) | M | 0..N | 200 OK | The PFDs for one or more application identifier(s) in the request URI are returned. |
| NOTE: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the GET method shall also apply. |  |  |  |  |

#### 5.3.2.4	Resource Custom Operations

None.

### 5.3.3	Resource: Individual application PFD

#### 5.3.3.1	Description

This resource represents the PFD identified by an application identifier.

#### 5.3.3.2	Resource definition

Resource URI: {apiRoot}/nnef-pfdmanagement/v1/applications/{appId}

This resource shall support the resource URI variables defined in table 5.3.3.2-1.

Table 5.3.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 5.1. |
| appId | String identifying a set of PFD for an application identifier. |

#### 5.3.3.3	Resource Standard Methods

##### 5.3.3.3.1	GET

This method shall support the URI query parameters specified in table 5.3.3.3.1-1.

Table 5.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | To filter irrelevant responses related to unsupported features. |

This method shall support the request data structures specified in table 5.3.3.3.1-2 and the response data structures and response codes specified in table 5.3.3.3.1-3.

Table 5.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PfdDataForApp | M | 1 | 200 OK | A representation of PFDs for an application in the request URI is returned. |
| NOTE: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the GET method shall also apply. |  |  |  |  |

#### 5.3.3.4	Resource Custom Operations

None.

### 5.3.4	Resource: PFD subscriptions

#### 5.3.4.1	Description

This resource represents a collection of subscriptions created by NF service consumers of Nnef_PFDmanagement service.

#### 5.3.4.2	Resource definition

Resource URI: {apiRoot}/nnef-pfdmanagement/v1/subscriptions

This resource shall support the resource URI variables defined in table 5.3.4.2-1.

Table 5.3.4.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 5.1 |

#### 5.3.4.3	Resource Standard Methods

##### 5.3.4.3.1	POST

This method shall support the URI query parameters specified in table 5.3.4.3.1-1.

Table 5.3.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.3.4.3.1-2 and the response data structures and response codes specified in table 5.3.4.3.1-3.

Table 5.3.4.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PfdSubscription | M | 1 | Create a PfdSubscription resource. |

Table 5.3.4.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PfdSubscription | M | 1 | 201 Created | The creation of a PfdSubscription resource is confirmed and a representation of that resource is returned. |
| NOTE: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the POST method shall also apply. |  |  |  |  |

#### 5.3.4.4	Resource Custom Operations

None.

### 5.3.5	Resource: Individual PFD subscription

#### 5.3.5.1	Description

This resource represents an individual PFD subscription created by NF service consumers of Nnef_PFDmanagement service.

#### 5.3.5.2	Resource definition

Resource URI: {apiRoot}/nnef-pfdmanagement/v1/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.3.5.2-1.

Table 5.3.5.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 5.1 |
| subscriptionId | String identifies an individual subscription to the PFD management service |

#### 5.3.5.3	Resource Standard Methods

##### 5.3.5.3.1	DELETE

This method shall support the URI query parameters specified in table 5.3.5.3.1-1.

Table 5.3.5.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.3.5.3.1-2 and the response data structures and response codes specified in table 5.3.5.3.1-3.

Table 5.3.5.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.3.5.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The PfdSubscription resource matching the subscriptionId was deleted successfully. |
| NOTE: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the DELETE method shall also apply. |  |  |  |  |

##### 5.3.5.3.2	PUT

This method shall support the URI query parameters specified in table 5.3.5.3.2-1.

Table 5.3.5.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.3.5.3.2-2 and the response data structures and response codes specified in table 5.3.5.3.2-3.

Table 5.3.5.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PfdSubscription | M | 1 | Update a PfdSubscription resource. |

Table 5.3.5.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PfdSubscription | M | 1 | 200 OK | The update of a PfdSubscription resource is confirmed and a representation of that resource is returned. |
| NOTE: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the PUT method shall also apply. |  |  |  |  |

#### 5.3.5.4	Resource Custom Operations

None.

## 5.4	Custom Operations without associated resources

None

## 5.5	Notifications

### 5.5.1	General

Notifications shall comply to subclause 6.2 of 3GPP TS 29.500 [5] and subclause 4.6.2.3 of 3GPP TS 29.501 [6].

Table 5.5.1-1: Notifications

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {notifyUri}/notify | POST | Notification of PFD change. |

### 5.5.2	PFD Change Notification

#### 5.5.2.1	Description

The PFD Change Notification is used by the PFDF to inform the NF service consumer, which has subscribed to this Notification via the PFD subscriptions resource.

#### 5.5.2.2	Target URI

The Notification URI "{notifyUri}/notify" shall be used with the URI variables defined in table 5.5.2.2-1.

Table 5.5.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| notifyUri | String formatted as URI with the Notification Uri as assigned within the PFD subscriptions resource and described within the PfdSubscription data type (see table 5.6.2.3-1). |

#### 5.5.2.3	Standard Methods

##### 5.5.2.3.1	POST

This method shall support the URI query parameters specified in table 5.5.2.3.1-1.

Table 5.5.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.5.2.3.1-2 and the response data structures and response codes specified in table 5.5.2.3.1-3.

Table 5.5.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PfdChangeNotification) | M | 1..N | Provides PFD change information. |

Table 5.5.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The PFD operation in the notification is performed successfully, i.e. all PFD changes are accepted by the SMF. |
| array(PfdChangeReport) | M | 1..N | 200 OK | The PFD operation in the notification is performed and the PfdChangeReport indicates failure reason for each failed application in the partial success. |
| ProblemDetails | M | 1 | 500 Internal Server Error | The server encountered an unexpected condition that prevented it from fulfilling the request. PFDs for all applications are not accepted by the SMF. (NOTE 2) |
| NOTE 1: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the POST method shall also apply. NOTE 2: Failure cases are described in subclause 5.7. |  |  |  |  |

## 5.6	Data Model

### 5.6.1	General

This subclause specifies the application data model supported by the API.

Table 5.6.1-1 specifies the data types defined for the Nnef_PFDmanagement service based interface protocol.

Table 5.6.1-1: Nnef_PFDmanagement specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| PfdDataForApp | 5.6.2.2 | Represents the PFDs for an application identifier. |  |
| PfdSubscription | 5.6.2.3 | Represents a PFD subscription. |  |
| PfdChangeNotification | 5.6.2.4 | Represents PFD change information. |  |
| PfdContent | 5.6.2.5 | Represents the content of a PFD for an application identifier. |  |
| PfdChangeReport | 5.6.2.6 | Represents error of PFD change. |  |

Table 5.6.1-2 specifies data types re-used by the Nnef_PFDmanagement service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnef_PFDmanagement service based interface.

Table 5.6.1-2: Nnef_PFDmanagement re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ApplicationId | 3GPP TS 29.571 [10] |  |  |
| DomainNameProtocol | 3GPP TS 29.122 [11] |  |  |
| DateTime | 3GPP TS 29.571 [10] |  |  |
| SupportedFeatures | 3GPP TS 29.571 [10] |  |  |
| Uri | 3GPP TS 29.571 [10] |  |  |

### 5.6.2	Structured data types

#### 5.6.2.1	Introduction

This subclause defines the structures to be used in resource representations.

Allowed structures are: array, object.

#### 5.6.2.2	Type: PfdDataForApp

Table 5.6.2.2-1: Definition of type PfdDataForApp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| applicationId | ApplicationId | M | 1 | Identifier of an application. |  |
| pfds | array(PfdContent) | M | 1..N | PFDs for the application identifier. |  |
| cachingTime | DateTime | O | 0..1 | Caching time for an application identifier. |  |

#### 5.6.2.3	Type: PfdSubscription

Table 5.6.2.3-1: Definition of type PfdSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| applicationIds | array(ApplicationId) | O | 1..N | Identifiers of applications with PFDs change. |  |
| notifyUri | Uri | M | 1 | Identifies the recipient of notifications sent by PFDF for this subscription. |  |
| supportedFeatures | SupportedFeatures | M | 1 | List of supported features used as described in subclause 5.8. This parameter shall be provided by the NF service consumer in the POST request that request the creation of a subscription, and shall be provided by the PFDF in the response of corresponding request. |  |

#### 5.6.2.4	Type: PfdChangeNotification

Table 5.6.2.4-1: Definition of type PfdChangeNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| applicationId | ApplicationId | M | 1 | Identifier of an application. |  |
| removalFlag | boolean | C | 0..1 | Indication of removal of PFDs for an existing application identifier. |  |
| partialFlag | boolean | C | 0..1 | Indication of partial update of PFDs for an existing application identifier if this operation is supported according to feature negotiation. | PartialUpdate |
| pfds | array(PfdContent) | C | 1..N | PFD creation/update for the application identifier as specified in subclause 6.3.3.5 of 3GPP TS 29.251 [12]. |  |

#### 5.6.2.5	Type: PfdContent

Table 5.6.2.5-1: Definition of type PfdContent

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pfdId | string | C | 0..1 | Identifies a PDF of an application identifier. If PartialUpdate feature is supported during PFD change event subscription, this attribute shall be provided by the PFDF. |  |
| flowDescriptions | array(string) | O | 1..N | Represents a 3-tuple with protocol, server ip and server port for UL/DL application traffic. (NOTE) |  |
| urls | array(string) | O | 1..N | Indicates a URL or a regular expression which is used to match the significant parts of the URL. (NOTE) |  |
| domainNames | array(string) | O | 1..N | Indicates an FQDN or a regular expression as a domain name matching criteria. (NOTE) |  |
| dnProtocol | DomainNameProtocol | C | 0..1 | Indicates the additional protocol and protocol field for domain names to be matched, it may only be provided when domainNames attribute is present. | DomainNameProtocol |
| NOTE: If a PFD contains multiple filter types, the PFD is only matched when every filter type contained in the PFD has a matching value. |  |  |  |  |  |

#### 5.6.2.6	Type: PfdChangeReport

Table 5.6.2.6-1: PfdChangeReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pfdError | ProblemDetails | M | 1 | More information on the error shall be provided in the "cause" attribute of the "ProblemDetails" structure. The "cause" attribute in the ProblemDetails shall be set to one of following application errors (see table 5.2.7.1-1 of 3GPP TS 29.500 [5]): - SYSTEM_FAILURE - INSUFFICIENT_RESOURCES - UNSPECIFIED_NF_FAILURE |  |
| applicationId | array(ApplicationId) | M | 1..N | Indicates the application identifier(s) which PFD(s) are failed to be added or modified. |  |

### 5.6.3	Simple data types and enumerations

#### 5.6.3.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

#### 5.6.3.2	Simple data types

The simple data types defined in table 5.6.3.2-1 shall be supported.

Table 5.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

## 5.7	Error handling

### 5.7.1	General

HTTP error handling shall be supported as specified in subclause 5.2.4 of 3GPP TS 29.500 [5].

For the Nnef_PFDmanagement API, HTTP error responses shall be supported as specified in subclause 4.8 of 3GPP TS 29.501 [6]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [5] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [5].

In addition, the requirements in the following subclauses shall apply.

### 5.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the Nnef_PFDmanagement API.

### 5.7.3	Application Errors

The application errors defined for the Nnef_PFDmanagement service are listed in table 5.7.3-1.

Table 5.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| SYSTEM_FAILURE | 500 Internal Server Error | Something functions wrongly in PFD provisioning or the PFD provisioning does not function at all. (NOTE) |
| INSUFFICIENT_RESOURCE | 500 Internal Server Error | There is limitation for resource storage. (NOTE) |
| UNSPECIFIED_NF_FAILURE | 500 Internal Server Error | Unspecified reason. (NOTE) |
| NOTE: This application error is included in the responses to the POST request of PFD change notification. |  |  |

## 5.8	Feature negotiation

The optional features in table 5.8-1 are defined for the Nnef_PFDmanagement API. They shall be negotiated using the extensibility mechanism defined in subclause 6.6 of 3GPP TS 29.500 [5].

Table 5.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | PartialUpdate | The PFDF can use this feature for partial update of PFDs. |
| 2 | DomainNameProtocol | This feature supports the additional protocol matching condition for the domain name in PFD data. |
| 3 | PfdChgSubsUpdate | The SMF can use this feature for updating the PFD change subscription. |

## 5.9	Security

As indicated in 3GPP TS 33.501 [14] and 3GPP TS 29.500 [5], the access to the Nnef_PFDmanagement API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [15]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [13]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nnef_PFDmanagement API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [13], subclause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nnef_PFDmanagement service.

The Nnef_PFDmanagement API defines a single scope "nnef-pfdmanagement" for OAuth2 authorization (as specified in 3GPP TS 33.501 [14]) for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

# A.1	General

The present Annex contains an OpenAPI [9] specification of HTTP messages and content bodies used by the Nnef_PFDmanagement API.

This Annex shall take precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API.

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification file contained in this 3GPP Technical Specification are available on the public 3GPP file server in the following locations (see clause 5B of the 3GPP TR 21.900 [17] for further information):

-	https://www.3gpp.org/ftp/Specs/archive/OpenAPI/<Release>/, and

-	https://www.3gpp.org/ftp/Specs/<Plenary>/<Release>/OpenAPI/.

NOTE 2:	To fetch the OpenAPI specification file after CT#83 plenary meeting for Release 15 in the above links <Plenary> must be replaced with the date the CT Plenary occurs, in the form of year-month (yyyy-mm), e.g. for CT#83 meeting <Plenary> must be replaced with value "2019-03" and <Release> must be replaced with value "Rel-15".

# A.2	Nnef_PFDmanagement API

openapi: 3.0.0

info:

title: Nnef_PFDmanagement Sevice API

version: 1.1.0.alpha-3

description: |

Packet Flow Description Management Service.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

#

externalDocs:

description: 3GPP TS 29.551 v16.3.0, 5G System; Packet Flow Description Management Service

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.551/'

#

servers:

- url: '{apiRoot}/nnef-pfdmanagement/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nnef-pfdmanagement

paths:

/applications:

get:

summary: Retrieve PFDs for all applications or for one or multiple applications with query parameter.

tags:

- PFD of applications

operationId: Nnef_PFDmanagement_Fetch

parameters:

- name: application-ids

description: The required application identifier(s) for the returned PFDs.

in: query

required: true

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

- name: supported-features

in: query

description: To filter irrelevant responses related to unsupported features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: The PFDs for one or more application identifier(s) in the request URI are returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PfdDataForApp'

minItems: 0

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

/applications/{appId}:

get:

summary: Retrieve the PFD for an application.

tags:

- Individual application PFD

operationId: Nnef_PFDmanagement_Fetch

parameters:

- name: appId

description: The required application identifier(s) for the returned PFDs.

in: path

required: true

schema:

type: string

- name: supported-features

in: query

description: To filter irrelevant responses related to unsupported features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: A representation of PFDs for an application in the request URI is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PfdDataForApp'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

/subscriptions:

post:

summary: Subscribe the notification of PFD changes.

tags:

- PFD subscriptions

operationId: Nnef_PFDmanagement_Subscribe

requestBody:

description: a PfdSubscription resource to be created.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PfdSubscription'

callbacks:

PfdChangeNotification:

'{request.body#/notifyUri}':

post:

summary: Notification of PFD change.

tags:

- PfdChangeNotification data

operationId: Nnef_PFDmanagement_Notify

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PfdChangeNotification'

minItems: 1

responses:

'200':

description: The PFD operation in the notification is performed and the PfdChangeReport indicates failure reason.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PfdChangeReport'

minItems: 1

'204':

description: The PFD operation in the notification is performed successfully.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

responses:

'201':

description: The creation of a PfdSubscription resource is confirmed and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PfdSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nnef-pfdmanagement/v1/subscriptions/{subscriptionId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

/subscriptions/{subscriptionId}:

put:

summary: Updates/replaces an existing subscription resource

tags:

- Individual PFD subscription

operationId: Nnef_PFDmanagement_Subscribe

parameters:

- name: subscriptionId

description: Identify the subscription.

in: path

required: true

schema:

type: string

requestBody:

description: Parameters to update/replace the existing subscription

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PfdSubscription'

responses:

'200':

description: OK (Successful update of the subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/PfdSubscription'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete a subscription of PFD change notification.

tags:

- Individual PFD subscription

operationId: Nnef_PFDmanagement_Unsubscribe

parameters:

- name: subscriptionId

description: Identify the subscription.

in: path

required: true

schema:

type: string

responses:

'204':

description: The PfdSubscription resource matching the subscriptionId was deleted successfully.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nnef-pfdmanagement: Access to the Nnef_PFDmanagement API

schemas:

#

# STRUCTURED DATA TYPES

#

PfdContent:

type: object

properties:

pfdId:

type: string

description: Identifies a PDF of an application identifier.

flowDescriptions:

type: array

items:

type: string

minItems: 1

description: Represents a 3-tuple with protocol, server ip and server port for UL/DL application traffic.

urls:

type: array

items:

type: string

minItems: 1

description: Indicates a URL or a regular expression which is used to match the significant parts of the URL.

domainNames:

type: array

items:

type: string

minItems: 1

description: Indicates an FQDN or a regular expression as a domain name matching criteria.

dnProtocol:

$ref: 'TS29122_PfdManagement.yaml#/components/schemas/DomainNameProtocol'

PfdDataForApp:

type: object

properties:

applicationId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

pfds:

type: array

items:

$ref: '#/components/schemas/PfdContent'

minItems: 1

cachingTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- applicationId

- pfds

PfdSubscription:

type: object

properties:

applicationIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

notifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifyUri

- supportedFeatures

PfdChangeNotification:

type: object

properties:

applicationId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

removalFlag:

type: boolean

default: false

partialFlag:

type: boolean

default: false

pfds:

type: array

items:

$ref: '#/components/schemas/PfdContent'

minItems: 1

required:

- applicationId

PfdChangeReport:

type: object

properties:

pfdError:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

applicationId:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

required:

- pfdError

- applicationId

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-01 |  |  |  |  |  | TS skeleton of PFD Management Service specification | 0.0.0 |
| 2018-01 | CT3#94 |  |  |  |  | Includes the following contribution agreed by CT3 at CT3#94: C3-180045, C3-180270, C3-180271. | 0.1.0 |
| 2018-03 | CT3#95 |  |  |  |  | Includes the following contribution agreed by CT3 at CT3#95: C3-181100, C3-181262, C3-181102, C3-181103, C3-181314. | 0.2.0 |
| 2018-04 | CT3#96 |  |  |  |  | Includes the following contribution agreed by CT3 at CT3#96: C3-182153, C3-182411, C3-182412, C3-182413, C3-182414, C3-182477, C3-182478. | 0.3.0 |
| 2018-05 | CT3#97 |  |  |  |  | Includes the following contribution agreed by CT3 at CT3#97: C3-183115, C3-183557, C3-183558, C3-183560. | 0.4.0 |
| 2018-06 | CT#80 | CP-181029 |  |  |  | TS sent to plenary for approval | 1.0.0 |
| 2018-06 | CT#80 | CP-181029 |  |  |  | TS approved by plenary | 15.0.0 |
| 2018-09 | CT#81 | CP-182015 | 0001 | 2 | F | Protocol error statement | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0002 | 1 | F | Description of Structured data types | 15.1.0 |
| 2018-12 | CT#82 | CP-183205 | 0003 |  | F | Default value for apiRoot | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0004 | 3 | F | Cardinality | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0005 | 1 | F | Correct Nnef_PFDmanagement API | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0006 |  | F | Location Header | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0007 | 1 | F | Alignment of openAPI | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0008 |  | F | API version update | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0009 |  | F | Security | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0010 |  | F | Content type | 15.2.0 |
| 2019-03 | CT#83 | CP-190116 | 0011 | 1 | F | Fetch PFD for all applications | 15.3.0 |
| 2019-03 | CT#83 | CP-190116 | 0014 | 1 | F | Correction of resource URIs | 15.3.0 |
| 2019-03 | CT#83 | CP-190130 | 0010 | 1 | B | PUT for PFD change subscription | 16.0.0 |
| 2019-03 | CT#83 | CP-190121 | 0012 | 3 | B | PFD extension | 16.0.0 |
| 2019-03 | CT#83 |  |  |  |  | Open API version update by MCC | 16.0.0 |
| 2019-06 | CT#84 | CP-191083 | 0016 | 2 | A | Precedence of OpenAPI file | 16.1.0 |
| 2019-06 | CT#84 | CP-191083 | 0019 |  | A | Correction to Notification of PFD change | 16.1.0 |
| 2019-06 | CT#84 | CP-191083 | 0021 |  | A | Copyright Note in YAML file | 16.1.0 |
| 2019-06 | CT#84 | CP-191101 | 0023 | 2 | F | API version Update | 16.1.0 |
| 2019-09 | CT#85 | CP-192149 | 0025 | 2 | A | Correct presence condition in PFD definition | 16.2.0 |
| 2020-03 | CT#87e | CP-200215 | 0026 |  | F | Reference of Error code | 16.3.0 |
| 2020-03 | CT#87e | CP-200216 | 0027 |  | F | Update of OpenAPI version and TS version in externalDocs field | 16.3.0 |