3GPP TSG-CT WG3 Meeting #108-e	C3-201380

- E-Meeting, 19th – 28th February 2020                                               	(revision of C3-201074)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.520 | CR | 0111 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the types of analytics events |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 4.2.2.2.1 missing some types of events for analytics subscription, i.e. it is not aligned with TS23.288 related clauses. Clause 4.2.2.2.1 and clause 4.3.1.1 missing some target UE information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added network performance information and expected behaviour information events for Analytics subscription. Added target UE information for the related analytics events. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage 2 requirement and incomplete description of the types of events for analytics subscription. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.2.1, 4.3.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not require a version update of the OpenAPI files. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- Additional discussion(if needed):

…

- Proposed changes:

- *** 1st Change ***

##### 4.2.2.2.1	General

The Nnwdaf_EventsSubscription_Subscribe service operation is used by an NF service consumer to subscribe or update subscription for event notifications. The following are the types of events for which a subscription may be made:

-	Load level of network slice instance;

-	NF load analytics information for a specific NF or a list of NFs, and optionally for a specific UE;

-	Service experience for an application or for a network slice with a given application or a set of applications or any application (i.e. all applications), for a specific UE or a group of UEs or any UE;

-	Network performance for any UE in an area of interest, or for a specific UE or a group of UEs;

-	QoS sustainability (for a certain area and time period, reports QoS change statistics or predicts the likelihood of a QoS change), for any UE;

-	UE mobility information, for a specific UE or a group of UEs;

-	UE communication information, for a specific UE or a group of UEs;

-	Expected behaviour information, for a specific UE or a group of UEs, by utilizing UE mobility and/or UE communication event(s);

-	Abnormal behaviour information, for a specific UE or a group of UEs or any UE; and

-	User data congestion for any UE in an area of interest, or for a specific UE.

- *** 2nd Change ***

#### 4.3.1.1	Overview

The Nnwdaf_AnalyticsInfo Service as defined in 3GPP TS 23.501 [2], 3GPP TS 23.288 [17] and 3GPP TS 23.503 [4], is provided by the Network Data Analytics Function (NWDAF).

This service:

-	allows NF consumers to request and get different type of analytic event information.

The types of observed events include:

-	Load level of network slice instance;

-	Service experience for an application or for a network slice with a given application or a set of applications or any application (i.e. all applications), for a specific UE or a group of UEs or any UE;

-	NF load analytics information for a specific NF or a list of NFs, and optionally for a specific UE;

-	Network performance for any UE in an area of interest, or for a specific UE or a group of UEs;

-	Expected behaviour information for a group of UEs or a specific UE, by utilizing UE mobility and/or UE communication event(s);

-	Abnormal behaviour information, for a group of UEs or a specific UE or any UE;

-	UE mobility information, for a group of UEs or a specific UE;

-	UE communication information, for a group of UEs or a specific UE;

-	User data congestion for any UE in an area of interest, or for a specific UE; and

-	QoS sustainability (for a certain area and time period, reports QoS change statistics or predicts the likelihood of a QoS change), for any UE.

- *** End of Changes ***