3GPP TSG-CT WG3 Meeting #108-e	C3-201164

- E-Meeting, 19th – 28th February 2020                                            (revision of C3-200abc)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.520 | CR | 0118 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections in TS29.520 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  |  | Date: |  | 2020-02-13 |
| Category: | D |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature) C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1) As UPF and URI have been mentioned the related abbreviations should be added in Abbreviations section. 2) Clarify the service name.’ Nnwdaf_AnalyticsSubscription Service’ is used in Stage 2 specification while the ‘Nnwdaf_EventsSubscription Service’ is used in Stage 3 specification. 3) Some other editoral changes in 4.2.1.3.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding UPF and URI in clause 3.2. - Clarify the Nnwdaf_EventsSubscription Service in stage 3 is mapping to the Nnwdaf_ AnalyticsSubscription Service in stage 2. - Correct some editorial mistakes in 4.2.1.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous descriptions lead to misunderstandings. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.2.1.1, 4.2.1.3.2, 4.3.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | There is no impact in OpenAPI. |  |  |  |  |  |  |  |  |

First change

- 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AF	Application Function

AMF	Access and Mobility Management Function

API	Application Programming Interface

JSON	JavaScript Object Notation

HTTP	Hypertext Transfer Protocol

NEF	Network Exposure Function

NF	Network Function

NRF	Network Repository Function

NWDAF	Network Data Analytics Function

NSSF	Network Slice Selection Function

OAM	Operation, Administration, and Maintenance

PCF	Policy Control Function

SMF	Session Management Function

UPF	User Plane Function

URI 	Uniform Resource Identifier

Next change

- 4.2.1.1	Overview

The Nnwdaf_EventsSubscription Service, corresponding to Nnwdaf_AnalyticsSubscription Service as defined in 3GPP TS 23.501 [2], 3GPP TS 23.288 [17] and 3GPP TS 23.503 [4], is provided by the Network Data Analytics Function (NWDAF).

This service:

-	allows NF consumers to subscribe to and unsubscribe from different analytic events; and

-	notifies NF consumers with a corresponding subscription about observed events.

The types of observed events include:

-	Load level of network slice instance;

-	Service experience for an application or for a network slice;

-	NF load analytics information for a specific NF or a list of NFs;

-	Network performance in an area of interest;

-	Expected behaviour information for a group of UEs or a specific UE;

-	Abnormal behaviour information for a group of UEs or a specific UE;

-	Mobility related information for a group of UEs or a specific UE;

-	Communication pattern for a group of UEs or a specific UE;

-	Congestion information of user data in a specific location; and

-	QoS sustainability (for a certain area and time period, reports QoS change statistics or predicts the likelihood of a QoS change).

Next change

- 4.2.1.3.2	NF Service Consumers

The Policy Control Function (PCF):

-	Supports taking input from NWDAF into consideration for policies on assignment of network resources and for traffic steering policies;

-	Supports (un)subscription to analytics information for background data transfer from NWDAF to determine whether the negotiated transfer policy is impacted;

-	Supports taking analytics information for QoS sustainability from NWDAF into consideration of QoS policies.

NOTE:	How this information is used by the PCF is not standardized in this release of the specification.

The Network Slice Selection Function (NSSF):

-	supports taking load level information from NWDAF into consideration for slice selection.

The Access and Mobility Management Function (AMF):

-	supports taking SMF load information from NWDAF into consideration for SMF selection.

The Session Management Function (SMF):

-	supports taking UPF load information from NWDAF into consideration for UPF selection.

The Network Exposure Function (NEF):

-	supports forwarding UE mobility information from NWDAF to the AF when it is untrusted;

-	supports forwarding UE communication information from NWDAF to the AF when it is untrusted;

-	supports forwarding user data congestion information from NWDAF to the AF when it is untrusted.

The Application Function (AF):

-	supports receiving UE mobility information from NWDAF or via the NEF;

-	supports receiving UE communication information from NWDAF or via the NEF;

-	supports receiving user data congestion information from NWDAF or via the NEF.

The Operation, Administration, and Maintenance (OAM):

-	supports receiving observed service experience from NWDAF.

Editor’s Note:	The functionalities of the consumers may be updated based on further stage 2 requirement.

Next change

##### 4.3.1.3.2	NF Service Consumers

The Policy Control Function (PCF):

-	Supports taking input from NWDAF into consideration for policies on assignment of network resources and for traffic steering policies.

-	Supports taking the analytics information for background data transfer from NWDAF into consideration to determine whether the negotiated transfer policy is impacted.

-	Supports taking analytics information for QoS sustainability from NWDAF into consideration of QoS policies.

NOTE:	How this information is used by the PCF is not standardized in this release of the specification.

The Network Slice Selection Function (NSSF):

-	supports taking load level information from NWDAF into consideration for slice selection.

The Access and Mobility Management Function (AMF):

-	supports taking SMF load information from NWDAF into consideration for SMF selection.

The Session Management Function (SMF):

-	supports taking UPF load information from NWDAF into consideration for UPF selection.

The Network Exposure Function (NEF):

-	supports forwarding UE mobility information from NWDAF to the AF when it is untrusted;

-	supports forwarding UE communication information from NWDAF to the AF when it is untrusted;

-	supports forwarding user data congestion information from NWDAF to the AF when it is untrusted.

The Application Function (AF):

-	supports receiving UE mobility information from NWDAF or via the NEF;

-	supports receiving UE communication information from NWDAF or via the NEF;

-	supports receiving user data congestion information from NWDAF or via the NEF.

The Operation, Administration, and Maintenance (OAM):

-	supports receiving observed service experience from NWDAF.

Editor’s Note:	The functionalities of the consumers maybe updated based on further stage 2 requirement.

End of change