3GPP TSG-CT WG3 Meeting #111e	C3-204xyz

- E-Meeting, 19th – 28th August 2020												(Revision of C3-204229)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.214 | CR | 1647 | rev | 1 | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RAN-NAS Release Cause feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2020-07-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | The RAN-NAS-Cause feature indicates the support for the detailed release cause code information from the EPS access network. The support for the 5GSM, 5GMM and NGAP cause codes is not covered by the RAN-NAS-Release-Cause AVP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of a new AVP, 5G-RAN-NAS-Release-Cause AVP, that defines the 5GSM, 5GMM and NGAP cause codes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interoperability problems that result in lack of support for RAN-NAS Cause feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 5.3.0, 5.3.x1 (new), 5.3.x2 (new), 5.3.x3 (new), 5.3.x4 (new), 5.3.x5 (new), 5.3.x6 (new), 5.4.1, 5.6.3, 5.6.6, E.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Cause value related AVPs type is changed to Unsigned32 |  |  |  |  |  |  |  |  |

- Additional discussion(if needed):

…

- Proposed changes:

- *** 1st Change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication and/or edition number or version number) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.203: "Policy and Charging Control architecture".

[3]	Void.

[4]	Void.

[5]	3GPP TS 29.209: "Policy control over Gq interface", latest Rel-6 version.

[6]	Void.

[7]	3GPP TS 29.211: "Rx Interface and Rx/Gx signalling flows", latest Rel-6 version.

[8]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[9]	3GPP TS 29.213: "Policy and Charging Control signalling flows and Quality of Service (QoS) parameter mapping".

[10]	Void.

[11]	IETF RFC 3556: "Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol (RTCP) Bandwidth".

[12]	IETF RFC 4005: "Diameter Network Access Server Application".

[13]	IETF RFC 4566: "SDP: Session Description Protocol".

[14]	IETF RFC 4006: "Diameter Credit Control Application".

[15]	ETSI TS 183 017 v3.2.1: "Telecommunications and Internet Converged Services and Protocols for Advanced Networking (TISPAN); Resource and Admission Control: DIAMETER protocol for session based policy set-up information exchange between the Application Function (AF) and the Service Policy Decision Function (SPDF); Protocol specification".

[16]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[17]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on SIP and SDP; Stage 3".

[18]	IETF RFC 3264: "An Offer/Answer Model with the Session Description Protocol (SDP)".

[19]	Void.

[20]	IETF RFC 3162: "Radius and IPv6".

[21]	IETF RFC 5031: "A Uniform Resource Name (URN) for Emergency and Other Well-Known Services".

[22]	Void.

[23]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[24]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging applications".

[25]	3GPP TS 29.229: "Cx and Dx interfaces based on the Diameter protocol; Protocol details"

[26]	3GPP TS 24.292: "IP Multimedia (IM) Core Network (CN) subsystem Centralized Services (ICS); Stage 3".

[27]	IETF RFC 3959 (December 2004): "The Early Session Disposition Type for the Session Initiation Protocol (SIP)".

[28]	3GPP TS 23.380: "IMS Restoration Procedures".

[29]	Void.

[30]	Void.

[31]	3GPP TS 22.153: "Multimedia Priority Service".

[32]	Void.

[33]	3GPP TS 29.274: "3GPP Evolved Packet System. Evolved GPRS Tunnelling Protocol for EPS (GTPv2)".

[34]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[35] 	IETF RFC 7683: "Diameter Overload Indication Conveyance".

[36]	3GPP TS 23.468: "Group Services and System Aspects; Group Communication System Enablers for LTE (GCSE LTE)".

[37]	3GPP TS 29.201: "Representational State Transfer (REST) reference point between Application Function (AF) and Protocol Converter (PC)".

[38]	3GPP TS 23.003: "Numbering, addressing and identification".

[39]	3GPP TS 29.273: "3GPP EPS AAA interfaces".

[40]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[41]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[42]	IETF RFC 5761: "Multiplexing RTP Data and Control Packets on a Single Port".

[43]	IETF RFC 7944: "Diameter Routing Message Priority".

[44]	Void.

[45]	IETF RFC 8101: "IANA Registration of New Session Initiation Protocol (SIP) Resource-Priority Namespace for Mission Critical Push To Talk service".

[46]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[47]	3GPP TS 29.154: "Service capability exposure functionality over Nt reference point".

[48]	IETF RFC 5009: "Private Header (P-Header) Extension to the Session Initiation Protocol (SIP) for Authorization of Early Media".

[49]	IETF RFC 3948: "UDP Encapsulation of IPsec ESP Packets".

[50]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; stage 3".

[51]	IETF RFC 8583: "Diameter Load Information Conveyance".

[52]	IETF RFC 6733: "Diameter Base Protocol".

[53]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[54]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[55]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[56]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[57]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[58]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[59]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[60]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[61]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[62]	3GPP TS 24.182: "IP Multimedia Subsystem (IMS) Customized Alerting Tones (CAT); Protocol specification".

[63]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[64]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[r1]	3GPP TS 24.501: "Non-Access-Stratum (NAS) Protocol for 5G System (5GS); Stage 3".

[r2]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

- *** 2nd Change ***

## 3.2	Abbreviations

For the purpose of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply:

5GMM	5G Mobility Management protocol

5GS	5G System

5GSM	5G Session Management protocol

ADC	Application Detection and Control

AF	Application Function

AS	Application Server

ASP	Application Service Provider

AVP	Attribute Value Pair

CRF	Charging Rules Function

DRMP	Diameter Routing Message Priority

DSCP	Differentiated Services Code Point

GCS	Group Communication Service

GCS AS	Group Communication Service Application Server

IP-CAN	IP Connectivity Access Network

MPS	Multimedia Priority Service

NGAP	NG Application Protocol

PCC	Policy and Charging Control

PCEF	Policy and Charging Enforcement Function

PCF	Policy Control Function

PCRF	Policy and Charging Rule Function

PDF	Policy Decision Function

P-CSCF	Proxy-Call Session Control Function

PSAP	Public Safety Answering Point

RCAF	RAN Congestion Awareness Function

QoS	Quality of Service

SCEF	Service Capability Exposure Function

SCS	Service Capability Server

SDF	Service Data Flow

SMF	Session Management Function

SPR	Subscriber Profile Repository

TDF	Traffic Detection Function

UDC	User Data Convergence

UE	User Equipment

UDR	User Data Repository

XML	Extensible Markup Language

- *** 3rd Change ***

### 5.3.0	General

Table 5.3.0.1 describes the Diameter AVPs defined for the Rx interface protocol, their AVP Code values, types, possible flag values, whether or not the AVP may be encrypted and which supported feature the AVP is applicable to. The Vendor-Id header of all AVPs defined in the present document shall be set to 3GPP (10415).

NOTE:	Most of these AVPs have already been defined in 3GPP TS 29.209 [5] for Rel-6. Their definition is based on the one used for Rel-6 with some possible modifications to be applied to the Rel-7 protocols.

- Table 5.3.0.1: Rx specific Diameter AVPs

|  |  |  |  | AVP Flag rules (Note 1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attribute Name | AVP Code | Clause defined | Value Type (Note 2) | Must | May | Should not | Must not | May Encr. | Applicability (Note 3) |
| 5GMM-Cause | Xxx2 | 5.3.x2 | Unsigned32 | V | P |  | M | Y | RAN-NAS-Cause |
| 5GS-RAN-NAS-Release-Cause | Xxx1 | 5.3.x1 | Group | V | P |  | M | Y | RAN-NAS-Cause |
| 5GSM-Cause | Xxx3 | 5.3.x3 | Unsigned32 | V | P |  | M | Y | RAN-NAS-Cause |
| Abort-Cause | 500 | 5.3.1 | Enumerated | M,V | P |  |  | Y |  |
| Access-Network-Charging-Address | 501 | 5.3.2 | Address | M,V | P |  |  | Y |  |
| Access-Network-Charging-Identifier | 502 | 5.3.3 | Grouped | M,V | P |  |  | Y |  |
| Access-Network-Charging-Identifier-Value | 503 | 5.3.4 | OctetString | M,V | P |  |  | Y |  |
| Acceptable-Service-Info | 526 | 5.3.24 | Grouped | M,V | P |  |  | Y |  |
| AF-Application-Identifier | 504 | 5.3.5 | OctetString | M,V | P |  |  | Y |  |
| AF-Charging-Identifier | 505 | 5.3.6 | OctetString | M,V | P |  |  | Y |  |
| AF-Requested-Data | 551 | 5.3.50 | Unsigned32 | V | P |  | M | Y |  |
| AF-Signalling-Protocol | 529 | 5.3.26 | Enumerated | V | P |  | M | Y | ProvAFsignalFlow |
| Application-Service-Provider-Identity | 532 | 5.3.29 | UTF8String | V | P |  | M | Y | SponsoredConnectivity |
| Codec-Data | 524 | 5.3.7 | OctetString | M,V | P |  |  | Y |  |
| Content-Version | 552 | 5.3.49 | Unsigned64 | V | P |  | M | Y | MediaComponentVersioning |
| Extended-Max-Requested-BW-DL | 554 | 5.3.52 | Unsigned32 | V | P |  | M | Y | Extended-Max-Requested-BW-NR |
| Extended-Max-Requested-BW-UL | 555 | 5.3.53 | Unsigned32 | V | P |  | M | Y | Extended-Max-Requested-BW-NR |
| Extended-Max-Supported-BW-DL | 556 | 5.3.54 | Unsigned32 | V | P |  | M | Y | Extended-BW-E2EQOSMTSI-NR |
| Extended-Max-Supported-BW-UL | 557 | 5.3.55 | Unsigned32 | V | P |  | M | Y | Extended-BW-E2EQOSMTSI-NR |
| Extended-Min-Desired-BW-DL | 558 | 5.3.56 | Unsigned32 | V | P |  | M | Y | Extended-BW-E2EQOSMTSI-NR |
| Extended-Min-Desired-BW-UL | 559 | 5.3.57 | Unsigned32 | V | P |  | M | Y | Extended-BW-E2EQOSMTSI-NR |
| Extended-Min-Requested-BW-DL | 560 | 5.3.58 | Unsigned32 | V | P |  | M | Y | Extended-Min-Requested-BW-NR |
| Extended-Min-Requested-BW-UL | 561 | 5.3.59 | Unsigned32 | V | P |  | M | Y | Extended-Min-Requested-BW-NR |
| Flow-Description | 507 | 5.3.8 | IPFilterRule | M,V | P |  |  | Y |  |
| Flow-Number | 509 | 5.3.9 | Unsigned32 | M,V | P |  |  | Y |  |
| Flows | 510 | 5.3.10 | Grouped | M,V | P |  |  | Y |  |
| Flow-Status | 511 | 5.3.11 | Enumerated | M,V | P |  |  | Y |  |
| Flow-Usage | 512 | 5.3.12 | Enumerated | M,V | P |  |  | Y |  |
| GCS-Identifier | 538 | 5.3.36 | OctetString | V | P |  | M | Y | GroupComService |
| IMS-Content-Identifier | 563 | 5.3.60 | OctetString | V | P |  |  | Y | VBC |
| IMS-Content-Type | 564 | 5.3.61 | Enumerated | V | P |  |  | Y | VBC |
| IP-Domain-Id | 537 | 5.3.35 | OctetString | V | P |  | M | Y |  |
| Max-Requested-Bandwidth-DL | 515 | 5.3.14 | Unsigned32 | M,V | P |  |  | Y |  |
| Max-Requested-Bandwidth-UL | 516 | 5.3.15 | Unsigned32 | M,V | P |  |  | Y |  |
| Max-Supported-Bandwidth-DL | 543 | 5.3.41 | Unsigned32 | V | P |  | M | Y | E2EQOSMTSI |
| Max-Supported-Bandwidth-UL | 544 | 5.3.42 | Unsigned32 | V | P |  | M | Y | E2EQOSMTSI |
| MCPTT-Identifier | 547 | 5.3.45 | OctetString | V | P |  | M | Y | MCPTT |
| MCVideo-Identifier | 562 | 5.3.45a | OctetString | V | P |  | M | Y | MCVideo |
| Media-Component-Description | 517 | 5.3.16 | Grouped | M,V | P |  |  | Y |  |
| Media-Component-Number | 518 | 5.3.17 | Unsigned32 | M,V | P |  |  | Y |  |
| Media-Component-Status | 549 | 5.3.48 | Unsigned32 | V | P |  | M | Y |  |
| Media-Sub-Component | 519 | 5.3.18 | Grouped | M,V | P |  |  | Y |  |
| Media-Type | 520 | 5.3.19 | Enumerated | M,V | P |  |  | Y |  |
| MPS-Identifier | 528 | 5.3.30 | OctetString | V | P |  | M | Y | Rel10 |
| Min-Desired-Bandwidth-DL | 545 | 5.3.43 | Unsigned32 | V | P |  | M |  | E2EQOSMTSI |
| Min-Desired-Bandwidth-UL | 546 | 5.3.44 | Unsigned32 | V | P |  | M |  | E2EQOSMTSI |
| Min-Requested-Bandwidth-DL | 534 | 5.3.32 | Unsigned32 | V | P |  | M | Y | Rel10 |
| Min-Requested-Bandwidth-UL | 535 | 5.3.33 | Unsigned32 | V | P |  | M | Y | Rel10 |
| NGAP-Cause | Xxx4 | 5.3.x4 | Grouped | V | P |  | M | Y | RAN-NAS-Cause |
| NGAP-Group | Xxx5 | 5.3.x5 | Unsigned32 | V | P |  | M | Y | RAN-NAS-Cause |
| NGAP-Value | Xxx6 | 5.3.x6 | Unsigned32 | V | P |  | M | Y | RAN-NAS-Cause |
| Priority-Sharing-Indicator | 550 | 5.3.47 | Enumerated | V | P |  | M | Y | PrioritySharing |
| Pre-emption-Control-Info | 553 | 5.3.51 | Unsigned32 | V | P |  | M | Y | MCPTT-Preemption |
| Required-Access-Info | 536 | 5.3.34 | Enumerated | V | P |  | M | Y | NetLoc |
| Retry-Interval | 541 | 5.3.39 | Unsigned32 | V | P |  | M | Y | DeferredService |
| Rx-Request-Type | 533 | 5.3.31 | Enumerated | V | P |  | M | Y |  |
| RR-Bandwidth | 521 | 5.3.20 | Unsigned32 | M,V | P |  |  | Y |  |
| RS-Bandwidth | 522 | 5.3.21 | Unsigned32 | M,V | P |  |  | Y |  |
| Service-Authorization-Info | 548 | 5.3.46 | Unsigned32 | V | P |  | M | Y |  |
| Service-URN | 525 | 5.3.23 | OctetString | M,V | P |  |  | Y |  |
| Service-Info-Status | 527 | 5.3.25 | Enumerated | M,V | P |  |  | Y |  |
| Sharing-Key-DL | 539 | 5.3.37 | Unsigned32 | V | P |  | M | Y | ResShare |
| Sharing-Key-UL | 540 | 5.3.38 | Unsigned32 | V | P |  | M | Y | ResShare |
| Specific-Action | 513 | 5.3.13 | Enumerated | M,V | P |  |  | Y |  |
| SIP-Forking-Indication | 523 | 5.3.22 | Enumerated | M,V | P |  |  | Y |  |
| Sponsor-Identity | 531 | 5.3.28 | UTF8String | V | P |  | M | Y | SponsoredConnectivity |
| Sponsored-Connectivity-Data (NOTE 4) | 530 | 5.3.27 | Grouped | V | P |  | M | Y | SponsoredConnectivity SCTimeBasedUM |
| Sponsoring-Action | 542 | 5.3.40 | Enumerated | V | P |  | M | Y | SponsorChange |
| NOTE 1: The AVP header bit denoted as ‘M’, indicates whether support of the AVP is required. The AVP header bit denoted as ‘V’, indicates whether the optional Vendor-ID field is present in the AVP header. For further details, see IETF RFC 6733 [52]. NOTE 2: The value types are defined in IETF RFC 6733 [52]. NOTE 3: AVPs marked with a supported feature (e.g. "ProvAFsignalFlow", "SponsoredConnectivity", "Rel10" or "NetLoc") are applicable as described in subclause 5.4.1 NOTE 4: Volume Usage monitoring control functionality is applicable for SponsoredConnectivity supported feature. Time Based Usage monitoring control is applicable for SCTimeBasedUM supported feature. |  |  |  |  |  |  |  |  |  |

- *** 4th Change ***

### 5.3.x1	5GS-RAN-NAS-Release-Cause AVP (3GPP-5GS and Non-3GPP-5GS access type)

The 5GS-RAN-NAS-Release-Cause AVP (AVP code xxx1) is of type Grouped and indicates the RAN or NAS release cause code information in 3GPP-5GS and non-3GPP-5GS access types.

AVP Format:

5GS-RAN-NAS-Release-Cause ::= < AVP Header: xxx1 >

[5GMM-Cause]

[5GSM-Cause]

[NGAP-Cause]

*[ AVP ]

- *** 5th Change ***

### 5.3.x2	5GMM-Cause AVP

The 5GMM-Cause AVP (AVP code xxx2) is of type Unsigned32 and indicates the 5GMM cause code information. The AVP shall be coded as per the 5GMM Cause in subclause 9.11.3.2 of 3GPP TS 24.501 [r1].

- *** 6th Change ***

### 5.3.x3	5GSM-Cause AVP

The 5GSM-Cause AVP (AVP code xxx3) is of type Unsigned32 and indicates the 5GSM cause code information. The AVP shall be coded as per the 5GSM Cause in subclause 9.11.4.2 of 3GPP TS 24.501 [r1].

- *** 7th Change ***

### 5.3.x4	NGAP-Cause AVP

The NGAP-Cause AVP (AVP code xxx4) is of type Grouped and indicates the NG Application Protocol cause value as specified in subclause 9.4.5 of 3GPP TS 38.413 [r2].

AVP Format:

NGAP-Cause ::= < AVP Header: xxx4 >

{NGAP-Group}

{NGAP-Value}

- *** 8th Change ***

### 5.3.x5	NGAP-Group AVP

The NGAP-Group AVP (AVP code xxx5) is of type Unsigned32, and it indicates the group of the NGAP cause. The value of this IE shall equal to the ASN.1 value of the specified NGAP cause group

NGAP supports cause groups defined as separate enumerations, as specified in subclause 9.4.5 of 3GPP TS 38.413 [r2]. The following values are defined in this specification:

- 0:

This value indicates the NGAP cause group is "radioNetwork".

- 1:

This value indicates the NGAP cause group is "transport".

2:

This value indicates the NGAP cause group is "nas".

3:

This value indicates the NGAP cause group is "protocol".

4:

This value indicates the NGAP cause group is "misc".

- *** 9th Change ***

### 5.3.x6	NGAP-Value AVP

The NGAP-Value AVP (AVP code xxx6) is of type Unsigned32 and indicates the NG AP cause value in specific cause group identified by the NGAP-Group AVP, as specified in subclause 9.4.5 of 3GPP TS 38.413 [r2].

- *** 10th Change ***

### 5.4.1	Use of the Supported-Features AVP on the Rx reference point

The Supported-Features AVP is used during session establishment to inform the destination host about the required and optional features that the origin host supports. The client shall, in the first request of a Diameter session indicate the set of supported features. The server shall, in the first answer within the Diameter session indicate the set of features that it has in common with the client and that the server shall support within the same Diameter session. Any further command messages shall always be compliant with the list of supported features indicated in the Supported-Features AVPs during session establishment. Features that are not advertised as supported shall not be used to construct the command messages for that Diameter session. Unless otherwise stated, the use of the Supported-Features AVP on the Rx reference point shall be compliant with the requirements for dynamic discovery of supported features and associated error handling on the Cx reference point as defined in clause 7.2.1 of 3GPP TS 29.229 [25].

The base functionality for the Rx reference point is the 3GPP Rel-7 standard and a feature is an extension to that functionality. If the origin host does not support any features beyond the base functionality, the Supported-Features AVP may be absent from the Rx commands. As defined in clause 7.1.1 of 3GPP TS 29.229 [25], when extending the application by adding new AVPs for a feature, the new AVPs shall have the M bit cleared and the AVP shall not be defined mandatory in the command ABNF.

As defined in 3GPP TS 29.229 [25], the Supported-Features AVP is of type grouped and contains the Vendor-Id, Feature-List-ID and Feature-List AVPs. On the Rx reference point, the Supported-Features AVP is used to identify features that have been defined by 3GPP and hence, for features defined in this document, the Vendor-Id AVP shall contain the vendor ID of 3GPP (10415). If there are multiple feature lists defined for the Rx reference point, the Feature-List-ID AVP shall differentiate those lists from one another.

On receiving an initial request application message, the destination host shall act as defined in clause 7.2.1 of 3GPP TS 29.229 [25]. The following exceptions apply to the initial and stateless AAR/AAA command pair:

-	If the AF supporting post-Rel-7 Rx functionality is able to interoperate with a PCRF supporting Rel-7, the AAR shall include the features supported by the AF within Supported-Features AVP(s) with the ‘M’ bit cleared. Otherwise, the AAR shall include the supported features within the Supported-Features AVP(s) with the M-bit set.

NOTE 1:	One instance of Supported-Features AVP is needed per Feature-List-ID.

-	If the AAR command does not contain any Supported-Features AVP(s) and the PCRF supports Rel-7 Rx functionality, the AAA command shall not include the Supported-Features AVP. In this case, both AF and PCRF shall behave as specified in the Rel-7 version of this document.

-	If the AAR command contains the Supported-Features AVP(s), the PCRF shall include the Supported-Features AVP(s) in the AAA command, with the ‘M’ bit cleared, indicating only the features that both the PCRF and AF support. In this case, the PCRF should not use the 'M' bit setting of the Supported-Features AVP(s) to determine if the AAR is accepted or rejected.

NOTE 2:	The client will always declare all features that are supported according to table 5.4.1.1. When more than one feature identifying a release is supported by both AF and PCRF, the AF will work according to the latest common supported release.

Once the PCRF and AF have negotiated the set of supported features during session establishment, the set of common features shall be used during the lifetime of the Diameter session.

The table below defines the features applicable to the Rx interfaces for the feature list with a Feature-List-ID of 1.

- Table 5.4.1.1: Features of Feature-List-ID 1 used in Rx

| Feature bit | Feature | M/O | Description |
| --- | --- | --- | --- |
| 0 | Rel8 | M | This feature indicates the support of the base 3GPP Rel-8 functionality, including the AVPs and corresponding procedures supported by the base 3GPP Rel-7 Rx standard, but excluding those features represented by separate feature bits. AVPs introduced with this feature are marked with "Rel8" in Table 5.4.0.1. |
| 1 | Rel9 | M | This feature indicates the support of the base 3GPP Rel-9 functionality, including the AVPs and corresponding procedures supported by the Rel8 feature bit, but excluding those features represented by separate feature bits. |
| 2 | ProvAFsignalFlow | O | This indicates support for the feature of provisioning of AF signalling flow information as described in clause 4.4.5a. If the PCRF supports this feature the AF may provision AF signalling flow information. NOTE: This feature is used by the IMS Restoration Procedures to provide to the PDN-Gateway the address of the P-CSCF selected by the UE, refer to TS 23.380 [28]. |
| 3 | SponsoredConnectivity | O | This feature indicates support for sponsored data connectivity feature. If the PCRF supports this feature, the AF may provide sponsored data connectivity to the subscriber. |
| 4 | Rel10 | M | This feature indicates the support of the base 3GPP Rel-10 functionality, including the AVPs and corresponding procedures supported by the Rel8 and Rel9 feature bit, but excluding those features represented by separate feature bits. AVPs introduced with this feature are marked with "Rel10" in table 5.3.0.1. |
| 5 | NetLoc | O | This feature indicates the support of the Access Network Information Reporting. |
| 6 | ExtendedFilter | O | This feature indicates the support for the local (i.e. UE) address and mask being present in filters signalled between network and UE. |
| 7 | SCTimeBasedUM | O | This feature indicates support for sponsored data connectivity feature with time-based usage monitoring control required. If the PCRF supports this feature, the AF may provide time threshold for the usage monitoring control. |
| 8 | Netloc-Trusted-WLAN | O | This feature indicates the support for the Trusted WLAN access.It requires that NetLoc feature is also supported. |
| 9 | RAN-NAS-Cause | O | This feature indicates the support for the release cause code information (NOTE 1) from the access network. |
| 10 | GroupComService | O | This feature indicates the support of Group Communication services as described in TS 23.468 [36] for unicast services. |
| 11 | ResShare | O | This feature indicates the support of resource sharing among several AF sessions. |
| 12 | DeferredService | O | This feature indicates the support of deferred transfer of service information from the AF. |
| 13 | DSCP | O | This feature indicates that the AF may provide a DSCP value when describing a service flow by supplying the ToS-Traffic-Class AVP. |
| 14 | SponsorChange | O | This feature indicates that the AF provides information on whether it wants to enable or disable/not enable sponsoring a service. It requires that SponsoredConnectivity is also supported. |
| 15 | E2EQOSMTSI | O | This feature indicates that the AF supports QoS End-to-end MTSI extensions as defined in 3GPP TS 26.114 [41] |
| 16 | NetLoc-Untrusted-WLAN | O | This feature indicates the support of the Untrusted WLAN access as described in 3GPP TS 23.203 [2]. It requires that NetLoc feature is also supported. |
| 17 | MCPTT | O | This feature indicates the support of Mission Critical Push To Talk services as described in 3GPP TS 23.179 [44] |
| 18 | PrioritySharing | O | This feature indicates that Priority Sharing is supported as described in 3GPP TS 23.203 [2], subclause 6.1.19. |
| 19 | PLMNInfo | O | This feature indicates that reporting on changes of PLMN info is supported. |
| 20 | MediaComponentVersioning | O | This feature indicates the support of media component versioning as defined in subclause 4.4.9. |
| 21 | MCPTT-Preemption | O | This feature indicates the support of service pre-emption based on the information provided by the AF. It requires that both PrioritySharing and MCPTT features are also supported. |
| 22 | MCVideo | O | This feature indicates the support of Mission Critical Video services as described in 3GPP TS 23.281 [61]. |
| Feature bit: The order number of the bit within the Feature-List AVP where the least significant bit is assigned number "0". Feature: A short name that can be used to refer to the bit and to the feature, e.g. "EPS". M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O"). Description: A clear textual description of the feature. NOTE 1: In this release, 5GS and EPS release cause code information is supported. The 3GPP-EPS and Non-3GPP EPS release cause code information from the access network is encoded in the RAN-NAS-Release-Cause AVP and can include RAN/NAS release cause(s), a TWAN release cause or an untrusted WLAN release cause. The 3GPP 5GS and Non-3GPP 5GS release cause code is encoded in the 5GS-RAN-NAS-Release-Cause AVP and is only applicable to Rx interworking with N7 as specified in Annex E. |  |  |  |

- Table 5.4.1.2: Features of Feature-List-ID 2 used in Rx

| Feature bit | Feature | M/O | Description |
| --- | --- | --- | --- |
| 0 | PCSCF-Restoration-Enhancement | O | This feature indicates support of P-CSCF Restoration Enhancement. It is used for the PCRF and the P-CSCF to indicate if they support P-CSCF Restoration Enhancement. |
| 1 | Extended-Max-Requested-BW-NR | O | This feature indicates the support of the extended Max-Requested-Bandwidth for NR. |
| 2 | Extended-Min-Requested-BW-NR | O | This feature indicates the support of the extended Min-Requested-Bandwidth for NR. It requires that Rel-10 feature is also supported. |
| 3 | Extended-BW-E2EQOSMTSI-NR | O | This feature indicates the support of the extended E2EQOSMTSI bandwidth values for NR. It requires that E2EQOSMTSI feature and the Extended-Max-Requested-BW-NR are also supported. |
| 4 | VBC | O | This feature indicates the support of Volume Based Charging of IMS services as defined in subclause A.16. |
| Feature bit: The order number of the bit within the Feature-List AVP where the least significant bit is assigned number "0". Feature: A short name that can be used to refer to the bit and to the feature, e.g. "EPS". M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O"). Description: A clear textual description of the feature. |  |  |  |

- *** 11th Change ***

### 5.6.3	Re-Auth-Request (RAR) command

The RAR command, indicated by the Command-Code field set to 258 and the ‘R’ bit set in the Command Flags field, is sent by the PCRF to the AF in order to indicate an Rx specific action.

Message Format:

<RA-Request> ::= < Diameter Header: 258, REQ, PXY >

< Session-Id >

[ DRMP ]

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

{ Destination-Host }

{ Auth-Application-Id }

*{ Specific-Action }

[ OC-Supported-Features ]

*[ Access-Network-Charging-Identifier ]

[ Access-Network-Charging-Address ]

0*2[ AN-GW-Address ]

[ AN-Trusted ]

*[ Flows ]

*[ Subscription-Id ]

[ Abort-Cause ]

[ IP-CAN-Type ]

[ NetLoc-Access-Support ]

[ RAT-Type ]

[ Sponsored-Connectivity-Data ]

[ 3GPP-User-Location-Info ]

[ User-Location-Info-Time ]

[ 3GPP-MS-TimeZone ]

*[ RAN-NAS-Release-Cause ]

*[ 5GS-RAN-NAS-Release-Cause ]

[ 3GPP-SGSN-MCC-MNC ]

[ TWAN-Identifier ]

[ TCP-Source-Port ]

[ UDP-Source-Port ]

[ UE-Local-IP-Address ]

[ Origin-State-Id ]

*[ Class ]

*[ Proxy-Info ]

*[ Route-Record ]

*[ AVP ]

- *** 12th Change ***

### 5.6.6	Session-Termination-Answer (STA) command

The STA command, indicated by the Command-Code field set to 275 and the ‘R’ bit cleared in the Command Flags field, is sent by the PCRF to the AF in response to the S3GPP TR command.

Message Format:

<ST-Answer>  ::= < Diameter Header: 275, PXY >

< Session-Id >

[ DRMP ]

{ Origin-Host }

{ Origin-Realm }

[ Result-Code ]

[ Error-Message ]

[ Error-Reporting-Host ]

[ OC-Supported-Features ]

[ OC-OLR ]

[ Failed-AVP ]

[ Sponsored-Connectivity-Data ]

[ Origin-State-Id ]

[ 3GPP-User-Location-Info ]

[ User-Location-Info-Time ]

[ 3GPP-MS-TimeZone ]

*[ RAN-NAS-Release-Cause ]

*[ 5GS-RAN-NAS-Release-Cause ]

[ 3GPP-SGSN-MCC-MNC ]

[ TWAN-Identifier ]

[ TCP-Source-Port ]

[ UDP-Source-Port ]

[ UE-Local-IP-Address ]

[ Netloc-Access-Support ]

*[ Class ]

*[ Redirect-Host ]

[ Redirect-Host-Usage ]

[ Redirect-Max-Cache-Time ]

*[ Proxy-Info ]

*[ Load ]

*[ AVP ]

- *** 13th Change ***

# E.1	General

In the 5GS the Rx reference point is defined between the the PCF and the AF. The relationship between the involved functional entities is depicted in figure E.1.1. The overall PCC architecture is depicted in clause 4 of 3GPP TS 29.513 [59].

- Figure E.1.1: Rx reference model in 5GS

5GS supports an interworking with AFs via the Rx interface. Interworking with AFs related to existing IMS services is described in 3GPP TS 23.228 [16]. Interworking with AFs related to existing Mission Critical services is described in TS 23.280 [60]. The PCF in 5GS supports the corresponding Rx procedures and requirements defined in 3GPP TS 23.203 [2]. The stage 2 requirements for 5GS supporting Rx interface are described in 3GPP TS 23.503 [56].

The PCF exchanges PCC rules with the SMF over the N7 interface as specified in 3GPP TS 29.512 [57].

Signalling flows related to the both Rx and N7 interfaces are specified in 3GPP TS 29.513 [59].

The generic mechanisms for interaction between the AF and the PCF are described in main subclauses in this specification and with the following clarifications:

-	description of the PCRF and the PCEF applies to the PCF and the SMF respectively;

-	description of the SPR applies to the UDR;

-	BBERF is not applicable in 5GS;

-	features related to Trusted WLAN are not applicable in 5GS; and

-	the 5G-RAN-NAS-Release-Cause AVP defined in subclause 5.3.x1 is applicable only to a PCF. When the RAN-NAS-Cause feature is supported, it encodes the 5GS RAN cause and/or NAS cause, when available.

- *** End of Changes ***