3GPP TSG-CT WG4 Meeting #103-e	C4-212092

# E-Meeting, 14th – 24th April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.500 | CR | 0243 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Error code alignment for SCP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2021-04-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the last meeting CT4_102e, Ref- C4-211728, it is decided to include the separate cause code table for the SCP/SEPP. In the new table, 404- ‘USER_NOT_FOUND’ error code is also applicable for SCP, which is not defined currently. In model D the SCP can invoke the UDR service for resolving HSS/UDM group ID (Nudr_GroupIDmap). If the user is not found in the UDR, the SCP should return 404 ‘USER_NOT_FOUND’ |  |  |  |  |  |  |  |  |
| Summary of change: |  | 404 ‘USER_NOT_FOUND’ cause code is added for the SCP/SEPP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SCP cannot return the 404 to the NF service consumer |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not introduce any OpenAPI change |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

#### 5.2.7.4	SCP/SEPP

The SCP or SEPP shall be able to forward the HTTP status codes defined in Table 5.2.7.2-1 from HTTP Server to HTTP client. In addition, it shall be able to generate HTTP status codes to indicate failures during indirect communication (e.g. see clauses 6.10.3.2 and 6.10.6), error handling (see clause 6.10.8) and SCP or SEPP overload control (see clause 6.4) as defined in Table 5.2.7.4-1.

If the received HTTP request contains payload body larger than the SCP or SEPP is able to process, the SCP or SEPP shall reject the HTTP request with the HTTP status code "413 Payload Too Large".

An HTTP status code "429 Too Many Requests (NF_CONGESTION_RISK)" is sent, when the SCP or SEPP detects that a given NF Service Consumer is sending excessive traffic which, if continued over time, may lead to (or may increase) an overload situation in the SCP or SEPP. If the SCP decides to redirect HTTP requests to another less loaded SCP, it may send the HTTP status code "307 Temporary Redirect" or "308 Permanent Redirect" with the cause attribute set to "SCP_REDIRECTION" (see clause 6.10.9).

The SCP or SEPP should map application errors to the most similar 3xx/4xx/5xx HTTP status code specified in table 5.2.7.4-1. If no such code is applicable, it should use "400 Bad Request" status code for errors caused by client side or "500 Server Internal Error" status code for errors caused on server side.

Table 5.2.7.4-1: Protocol and application errors generated by the SCP/SEPP

| Protocol or application Error | HTTP status code | Description |
| --- | --- | --- |
| INVALID_API | 400 Bad Request | The HTTP request contains an unsupported API name or API version in the URI. |
| INVALID_MSG_FORMAT | 400 Bad Request | The HTTP request has an invalid format. |
| INVALID_QUERY_PARAM | 400 Bad Request | The HTTP request contains an unsupported query parameter in the URI. (NOTE 1) |
| MANDATORY_QUERY_PARAM_INCORRECT | 400 Bad Request | A mandatory query parameter, or a conditional query parameter but mandatory required, for an HTTP method was received in the URI with semantically incorrect value. (NOTE 1) |
| OPTIONAL_QUERY_PARAM_INCORRECT | 400 Bad Request | An optional query parameter for an HTTP method was received in the URI with a semantically incorrect value that prevents successful processing of the service request. (NOTE 1) |
| MANDATORY_QUERY_PARAM_MISSING | 400 Bad Request | Query parameter which is defined as mandatory, or as conditional but mandatory required, for an HTTP method is not included in the URI of the request. (NOTE 1) |
| MANDATORY_IE_INCORRECT | 400 Bad Request | A mandatory IE (within a variable part of an "apiSpecificResourceUriPart" or within an HTTP header), or conditional IE but mandatory required, for an HTTP method was received with a semantically incorrect value. (NOTE 1) |
| OPTIONAL_IE_INCORRECT | 400 Bad Request | An optional IE (within an HTTP header) for an HTTP method was received with a semantically incorrect value that prevents successful processing of the service request. (NOTE 1) |
| MANDATORY_IE_MISSING | 400 Bad Request | A mandatory IE (within the variable part of an "apiSpecificResourceUriPart" or within an HTTP header), or conditional IE but mandatory required, for an HTTP method is not included in the request. (NOTE 1) |
| UNSPECIFIED_MSG_FAILURE | 400 Bad Request | The request is rejected due to unspecified client error. (NOTE 2) |
| NF_DISCOVERY_FAILURE | 400 Bad Request | The request is rejected by the SCP because no NF Service Producer can be found matching the NF service discovery factors (see clause 6.10.6). |
| INVALID_DISCOVERY_PARAM | 400 Bad Request | The request is rejected by the SCP because it contains an unsupported discovery parameter (i.e. unknown 3gpp-Sbi-Discovery-* header) (see clause 6.10.3.2). (NOTE 1) |
| MISSING_ACCESS_TOKEN_INFO | 400 Bad Request | The request is rejected due to missing information in the service request that prevents the SCP from requesting an access token to the Authorization Server. See clause 6.10.3.5. |
| ACCESS_TOKEN_DENIED | 403 Forbidden | The request is rejected due to the Authorization Server rejecting to grant an access token to the SCP. See clause 6.10.3.5. |
| INCORRECT_LENGTH | 411 Length Required | The request is rejected due to incorrect value of a Content-length header field. |
| NF_CONGESTION_RISK | 429 Too Many Requests | The request is rejected due to excessive traffic which, if continued over time, may lead to (or may increase) an overload situation. |
| INSUFFICIENT_RESOURCES | 500 Internal Server Error | The request is rejected due to insufficient resources. |
| UNSPECIFIED_NF_FAILURE | 500 Internal Server Error | The request is rejected due to unspecified reason at the SCP or SEPP. (NOTE 3) |
| SYSTEM_FAILURE | 500 Internal Server Error | The request is rejected due to generic error condition in the SCP or SEPP. |
| NF_FAILOVER | 500 Internal Server Error | The request is rejected by the SCP due to the unavailability of the NF, and the requester may trigger an immediate re-selection of an alternative NF based on this information. |
| NF_SERVICE_FAILOVER | 500 Internal Server Error | The request is rejected by the SCP due to the unavailability of the NF service, and the requester may trigger an immediate re-selection of an alternative NF service based on this information. |
| NF_CONGESTION | 503 Service Unavailable | The SCP or SEPP experiences congestion and performs overload control, which does not allow the request to be processed. (NOTE 4) |
| TIMED_OUT_REQUEST | 504 Gateway Timeout | The request is rejected due a request that has timed out at the HTTP client (see clause 6.11.2). |
| TARGET_NF_NOT_REACHABLE | 504 Gateway Timeout | The request is not served as the target NF is not reachable (see clause 6.10.8.2). |
| SCP_REDIRECTION | 307 Temporary Redirect 308 Permanent Redirect | The request is redirected to a different SCP (see clause 6.10.9). |
| USER_NOT_FOUND | 404 Not Found | The request is rejected by the SCP when the UDR is not able to resolve the group ID and returns the status code as 404 (not found) |
| NOTE 1: "invalidParams" attribute shall be included in the "ProblemDetails" data structure indicating unsupported, missing or incorrect IE(s) or 3gpp-Sbi-Discovery-* header(s). NOTE 2: This application error indicates error in the HTTP request and there is no other application error value that can be used instead. NOTE 3: This application error indicates error condition in the SCP/SEPP and there is no other application error value that can be used instead. NOTE 4: If the reason for rejection is a temporary overload, the SCP/SEPP may include in the response a Retry-After header field to indicate how long the service is expected to be unavailable. |  |  |

* * * End of Changes * * * *