3GPP TSG-CT WG4 Meeting #124	C4-243180

# Maastricht, Netherlands; 19th – 23rd August 2024

Source:	Nokia

Title:	Status of the Extended Reality and Media service Phase 2 (XRM_Ph2) stage 2 WID and plan for the related CT work

Agenda item:	5

Document for:	INFORMATION

Abstract of the contribution:

This document provides information on the status of the Extended Reality and Media service Phase 2 (XRM_ph2) WID in SA2 and the plan for the related CT work item in Rel-19.

# Introduction

A Rel-19 WID on Extended Reality and Media service Phase 2 (XRM_Ph2) has been approved at 3GPP SA#104 in SP-240990, which is an extension of the Rel-18 work on Extended Reality and Media service (XRM).

# Stage 2 status

## SA2 status

The Study on architecture enhancement for Extended Reality and Media service (XRM) Phase 2 is covered in 3GPP TR 23.700-70. The SA2 study is 90% complete.  As documented in clause 8 of 3GPP TR 23.700-70:

-	Conclusions have been agreed for the KI#3 (Leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network), KI#5 (QoS Handling when Traffic Characteristics Change Dynamically), KI#6 (L4S for non-3GPP access networks and intermediate 5GS nodes), KI#7 (Support for PDU Set in non-3GPP access) and KI#8 (Enhancement for UE with the tethered devices). For the KI#8, there will be no normative work.

-	Interim conclusions have been agreed for the KI#1 (Support of PDU set based QoS handling enhancement), KI#4 (Traffic detection and QoS flow mapping for multiplexed data flows) and KI#9 (Enhancement for XR related network information exposure)-.

-	The conclusion for KI#2 (Support PDU Set information identification for end-to-end encrypted XRM traffic) is pending.

The related stage 2 normative work will start in Q3.

Nokia is the SA2 Rapporteur of the FS_XRM Ph2 SID and the Primary Rapporteur of the XRM Ph2 normative WID (SP-240990).

# Stage 3 impacts

Based on the above stage 2 status in Rel-19, the following stage 3 impacts are currently foreseen for CT1, CT3 and CT4.

## CT1 impacts

### Key Issue #4: Traffic detection and QoS flow mapping for multiplexed data flows

Based on the current interim agreement in SA2:

- For the multiplexed media flows which cannot be differentiated by using the legacy IP Packet Filter, the AF may provide additional Packet Filter in the "AF session with required QoS" procedure to the NEF/PCF (E.g., the additional Packet Filter can be Synchronization Source (SSRC), Payload Type (PT) and RTP-M header fields):

CT1 potential impacts:

- In TS 24.501, TS 24.502 to enhance NAS-level QoS rule exchanges to include and manage additional packet filters for multiplexed data flows.

  - One possibility could be to reuse the already existing IE defined for protocol description along with the associated QoS rules for the PDU set handling by UE in 24.501 (9.11.4.39). The IE can be extended this to include additional packet filet attributes, if needed.

  - Exclusive capability (supporting additional Packet Filters) indication by the UE to the network (SMF) as part of the SM capability could be useful: This would 1) Optimize network resources (e.g., RAN resources shall be reserved only if UE has sent the additional capability indication), 2) May help in details of managing legacy UEs Vs. multiplexed flow handling capable UEs.

### Key Issue #6: L4S for non-3GPP access networks and intermediate 5GS nodes

Based on the agreement in SA2:

- Dedicated 5G QoS Flow(s) and non-3GPP access resources (e.g. IPsec Child SAs) are used for carrying L4S enabled IP traffic.

CT1 potential impacts::

- Details of L4S handling in non-3GPP access needs to be updated in the Stage-3 specifications:

  - Dedicated 5G QoS Flow(s) and non-3GPP access resources (e.g. IPsec Child SAs) are used for carrying L4S enabled IP traffic.

- Potential impact in 24.502:

  - N3IWF/TNGF in UL shall support and UE in DL can support the IP-in-IP tunnel behaviour of copying ECN bits between outer and inner headers as per IETF RFC 6040

- Potential impacts in TS 24.501

  - to extend description for L4S handling for non-3GPP access [4.13 of 24.501]

- Potential enhancements needed in TS 23.316 on clauses impacting 5G-RG:

  - 5G-RG may receive NAS message related to L4S QoS flows.

  - The 5G-RG maps the QoS rule (s) received in NAS to appropriate L4S-enabled W-UP level QoS resources (s).

  - Encap-decap capability in the 5G-RG should be ensured.

### Key Issue #7: Support for PDU Set in non-3GPP access

Based on the agreement in SA2:

- PDU sets handling are to be supported over N3GPP access.

CT1 Potential impacts :

- PDU set handling for non-3GPP access needs to be updated in the specifications:

  - Potential impacts in TS 23.316, TS 24.501, TS 24.502 to enhance handling of PDU Set QoS handling in non-3GPP access.

  - Potential enhancement of the protocol description IE to mention non-3GPP access

  - to remove “PDU set handling does not apply for non-3GPP access.” from 4.7.3 of TS 24.501

  - some text description should be added to explain PDU set handling for non-3GPP accesses

## CT3 Impacts:

### Key Issue #1: Support of PDU set based QoS handling enhancement

Based on the current interim agreement in SA2:

- QoS Notification without AQP: the fulfilment or not of UL and/or DL PSDB or UL and/or DL PSER in the QoS profile are used by the NG-RAN to determine QoS notification if the PDU Set QoS handling is applied in the corresponding direction.

  - QoS notification to AF from NEF or PCF

CT3 potential impacts:

- Both the Network Exposure Function (NEF) and the Policy Control Function (PCF) will receive Alternative Service requirements from the Application Function (AF) for the 5GC. These requirements may include both Uplink (UL) and Downlink (DL) Packet Data Unit (PDU) Set QoS Parameters, such as UL/DL PDU Set Delay Buffer (PSDB) and UL/DL PDU Set Error Rate (PSER). This information will be potentially specified in the following technical specifications:

  - NEF: TS 29.522 and TS 29.122

  - PCF: TS 29.514 and TS 29.512

### Key Issue #4: Traffic detection and QoS flow mapping for multiplexed data flows

Based on the current interim agreement in SA2:

- The AF may provide QoS requirements for media flows and additional packet filters as part of "AF session with required QoS". The detectable media flows that require different QoS treatment are mapped to distinct QoS Flows. If the AF does not provide any QoS requirement for the media flows or it provides the same QoS requirement, then the media flows can be mapped to the same QoS Flow. For the multiplexed media flows which cannot be differentiated by using the legacy IP Packet Filter, the AF may provide additional Packet Filter in the "AF session with required QoS" procedure to the NEF/PCF.

CT3 potential impacts:

- Both the NEF and the PCF will receive additional packet filtering information from the AF through the "AF session with required QoS" API. This information includes:

- Synchronization Source (SSRC): Identifies the source of the media stream.

- Payload Type (PT): Specifies the type of media data being transmitted.

- RTP-M header fields: Provides additional information about the Real-time Transport Protocol (RTP) media stream.

This information will be potentially specified in the following technical specifications:

- NEF: TS 29.522 and TS 29.122

- PCF: TS 29.514 and TS 29.512

### Key Issue #5: QoS Handling when Traffic Characteristics Change Dynamically

Based on the agreement in SA2:

- The AF may indicate to the NEF/PCF to mark the Burst size.

- Based on the PCC rule and/or local configuration, the SMF instructs the UPF to identify and mark the Burst size and to further notify NG-RAN on the Burst size.

CT3 potential impacts:

- Both the NEF and the PCF will be responsible for marking the burst size per flow.

- NEF: This functionality will be potentially specified in TS 29.522 and TS 29.122, and the burst size will be determined based on the input received from the AF.

- PCF: This functionality will be potentially specified in TS 29.514 and TS 29.512. The PCF will mark the burst size per flow.

### Key Issue #7: Support for PDU Set in non-3GPP access

Based on the agreement in SA2:

- PDU sets are supported over Non-3GPP access

CT3 potential impacts:

- The PCF as defined in TS 29.512, will potentially support the use of Protocol Data Units (PDUs) in policy decisions for Non-3GPP access networks.

### Key Issue #9: Enhancement for XR related network information exposure

Based on the current interim agreement in SA2:

- For GBR QoS Flow, exposure of available data rate:

  - Based on the AF requests, the PCF instructs SMF to enable the available data rate report and exposure for a GBR QoS Flow.

CT3 potential impacts:

- PCF (TS 29.514, TS 29.512): Monitoring and reporting of GBR QoS flow data rates will require adjustments to the PCF.

- NEF (TS 29.522, 29.122): The NEF will need to be enhanced to handle the provisioning, forwarding, and usage of GBR QoS flow data rate information provided by the AF.

- NEF (TS 29.517): The NEF will need to expose GBR QoS flow data through AF event exposure mechanisms.

## CT4 Impacts

### Key Issue #1: Support of PDU set based QoS handling enhancement

Based on the current interim agreement in SA2:

- According to the Protocol Description, the 5GC sends NG-RAN an indicator indicating the DL PDU Set Information Marking is supported. Based on the indicator, the NG RAN may indicate the 5GC to perform the PDU Set Information marking.

CT4 potential impacts:

-	 TS 29.502: new indicators may be defined over N16/N16a to enable:

-	the SMF to indicate to the 5G-RAN that DL PDU Set information marking is supported; and

-	the 5G-RAN to indicate to the SMF to perform PDU Set information marking.

- The AF may provide Alternative Service requirements to 5GC which may contain the UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).

CT4 potential impacts:

-	TS 29.502: the Alternative QoS Profile may be extended with PDU Set QoS parameters.

### Key Issue #3: Leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network

Based on the conclusion in SA2:

- For every QoS Flow, the SMF shall determine the transport level packet marking value (e.g. the DSCP in the outer IP header) based on the 5QI, the Priority Level (if explicitly signalled) and optionally, the ARP priority level and provide the transport level packet marking value to the UPF as described in clause 5.8.2.7 of TS 23.501 [2]. For a QoS Flow configured for PDU Set QoS handling, besides the above information, the SMF may further consider PDU Set Importance values to be used by the QoS Flow, to provide a list of DSCP marking values, each of the DSCP marking values corresponding to one or several PDU Set Importance values.

- The SMF sends the mapping list of the transport level marking for DL packets (N3/N9 interface) and related PDU set importance value(s) in the FAR to the UPF, and the UPF performs the DSCP value marking based additionally on the PDU Set Importance per PDU Set.

CT4 impacts:

-	TS 29.244: PFCP need be extended to enable the UPF to perform DSCP marking of DL traffic also considering the PDU set importance value(s).

### Key Issue #4: Traffic detection and QoS flow mapping for multiplexed data flows

Based on the current interim agreement in SA2:

- For the multiplexed media flows which cannot be differentiated by using the legacy IP Packet Filter, the AF may provide additional Packet Filter in the "AF session with required QoS" procedure to the NEF/PCF.

- In order to unique identify each media flow, the additional packet filter is used together with the legacy packet filter. The additional Packet Filter is used to differentiate the media flow among multiple media flows that share the same legacy packet filter both in uplink (in UE) and downlink (in UPF) in the following case:

- For the media flows that are transported in (S)RTP, (S)RTCP and other associated protocols that are multiplexed into a single UDP/IP traffic flow as specified in IETF RFCs 5761, 5764, 7983, 8872, 8843 and 9443. The additional Packet Filter SHALL be the Synchronization Source (SSRC), Payload Type (PT) and RTP-M header fields.

- The SMF may provide the received additional Packet Filter along with legacy packet filter to the UPF in the N4 rules. The PDI in PDR is extended to support the additional Packet Filter.

- The SMF provides QoS rules for the uplink traffic for the UE. QoS rules are enhanced to include additional Packet Filter when required (additional packet filter is not expected to be interpreted by a legacy UE; in case of legacy UE, it will use legacy packet filter). If supported by the UE, the UE uses both the legacy packet filter and the additional packet filter for QoS Flow mapping.

CT4 potential impacts:

-	TS 29.244: The PDI in PDR is extended to support the additional Packet Filter.

-	TS 29.502: potential updates, depending on the NAS extensions defined by CT1, to enable the SMF to provide the Additional Packet Filter to the UE (note that the Nsmf_PDUSession API defines QoSRules as "Bytes" encoding the QoS Rule defined in NAS specification, so extending the QoS rule with the Additional Packet Filter info may be transparent to the Nsmf_PDUSession API).

### Key Issue #5: QoS Handling when Traffic Characteristics Change Dynamically

Based on the conclusion in SA2:

- The AF may indicate to the NEF/PCF to mark the Burst size.

- Based on the PCC rule and/or local configuration, the SMF instructs the UPF to identify and mark the Burst size and to further notify NG-RAN on the Burst size.

- Based on SMF instruction, the UPF identifies and marks the Burst size, based on the input (i.e. Burst size) carried in N6 DL packet header. The UPF sends the Burst size to NG-RAN via DL GTP-U header.

CT4 impacts:

-	TS 29.244: PFCP needs to be extended to enable the SMF to instruct the UPF to identify and mark the Burst size in DL GTP-U header.

Note: it is assumed that RAN3 will define the GTP-U extension to carry the Burst Size information.

### Key Issue #6: L4S for non-3GPP access networks and intermediate 5GS nodes

Based on the conclusion in SA2:

- Dedicated 5G QoS Flow(s) and non-3GPP access resources (e.g. IPsec Child SAs) are used for carrying L4S enabled IP traffic.

CT4 potential impacts:

-	TS 29.502: potential impacts for N16/N16a to reflect that L4S related IE may apply to Non-3GPP access / 5G-AN (i.e. not only 5G-RAN).

### Key Issue #7: Support for PDU Set in non-3GPP access

Based on the conclusion in SA2:

- PDU sets are supported over Non-3GPP accesses

CT4 potential impacts:

-	TS 29.502: potential impacts for N16/N16a to reflect that PDU sets related IE may apply to Non-3GPP access / 5G-AN (i.e. not only 5G-RAN).

### Key Issue #9: Enhancement for XR related network information exposure

Based on the current interim agreement in SA2:

- For GBR QoS Flow, exposure of available data rate:

-	Based on the AF requests, the PCF instructs SMF to enable the available data rate report and exposure for a GBR QoS Flow.

-	Following the instruction of SMF:

-	The NG-RAN reports the available data rate to the UPF via the GTP-U header of UL packets and UPF further exposes, based on mechanisms defined in clause 5.8.2.17 of TS 23.501 [2], the available data rate to the AF directly or via the local NEF.

CT4 potential impacts:

-	TS 29.244: potential impacts for the SMF to instruct the UPF to report available data rate

-	TS 29.564: exposure of the available data rate

-	TS 29.502: potential updates for N16/N16a to support instructing the RAN to report the available data rate

# Conclusion

SA2 is expected to complete the study on XRM Phase 2 and starts the stage 2 normative work in Q3 2024.

CT WGs (CT1, CT3, and CT4) will be impacted based on the current SA2 status.

Nokia intends to submit a CT WID in Q4 2024 to cover the stage 3 work of XRM Phase 2, once SA2 finalizes its conclusions for all the key issues. Stage 3 normative work can start once normative stage 2 requirements are available.

Based on the current understanding of SA2 impacts, it is assumed that CT3 would be the leading working group for the work item.