3GPP TSG-CT WG4 Meeting #124	C4-243194

# Maastricht, Netherlands; 19th – 23rd August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.527 | CR | 0083 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Restoration procedures for split PDU sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN3 has extended NGAP to enhance the restoration procedures for split PDU sessions when a GTP-U Error Indication is received from the 5G-AN or when a N3 UP path failure occurs (see RAN3 Reply LS in R3-243968). The extensions done by RAN3 differ from what was specified originally by CT4 in TS 23.527 on the following aspects: - the 5G AN triggers a PDU Session Modify Indication procedure when it decides to release the tunnel for the associated QoS flows or to move the associated QoS flows to the other GTP-U tunnel. - The SMF shall include the DL F-TEID in the PDU Session Resource Modify Request message also in case of a UP path failure (and not only the DL N3 IP address). Stage 2 (TS 23.527) should be aligned on stage 3 (TS 38.413). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The restoration procedures for split PDU sessions when a GTP-U Error Indication is received from the 5G-AN or when a N3 UP path failure occurs are aligned on stage 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between stage 2 and stage 3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.1, 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

.

* * * First Change * * * *

### 5.3.2	Procedure for GTP-U Error Indication received from 5G-AN

#### 5.3.2.1	Principles

Figure 5.3.2.1-1: GTP-U Error Indication from 5G-AN

1.	The user plane connection of an existing PDU session is activated. Downlink G-PDUs are sent towards the 5G-AN.

2.	The 5G-AN returns a GTP-U Error Indication if it does not have a corresponding GTP-U context (see clause 5.2).

3.	Upon receipt of a GTP-U Error Indication, the UPF shall identify the related PFCP session and send an Error Indication Report to the SMF, as specified in clause 5.10 of 3GPP TS 29.244 [4].

4.	For a GTP-U Error Indication received from a 5G-AN, the SMF shall modify the PFCP session to instruct the UPF to buffer downlink packets. For a split PDU session, the SMF may only do so for the FAR provisioned with the 5G-AN F-TEID for which an Error Indication Report was received.

5.	If the user plane connection of the PDU session is seen as activated by the SMF, the SMF shall initiate an Namf_Communication_N1N2MessageTransfer service operation to request the 5G-AN to release the PDU session's resources, as specified in clause 4.3.7 of 3GPP TS 23.502 [5]. 

If the affected PDU session is a split PDU session, the SMF may instead initiate an Namf_Communication_N1N2MessageTransfer service operation including the "PDU Session Resource Modify Request Transfer" IE containing the User Plane Failure Indication IE indicating the N3 DL F-TEID and theindication that the UPF has received a GTP-U Error Indication for that N3 DL F-TEID.

6.	Upon receipt of an Namf_Communication_N1N2MessageTransfer request to transfer the PDU Session Resource Release Command, the AMF shall:

-	proceed with the request, as specified in clause 5.2.2.3.1 of 3GPP TS 29.518 [6], if the UE is in CM-CONNECTED state for the Access Network Type associated to the PDU session;

-	otherwise, reject the request with an error indicating that the UE is in CM-IDLE state for the Access Network Type associated to the PDU session.

6a.	The AMF shall send a PDU Session Resource Modify Request to the 5G-AN if it receives the "PDU Session Resource Modify Request Transfer" IE in step 5.

7.	If the AMF sent a PDU Session Resource Release Command to the 5G-AN, the PDU session's resource release is acknowledged to the SMF.

If the AMF sent a PDU Session Resource Modify Request to the 5G-AN, the 5G-AN may allocate a new N3 DL F-TEID, or decide to move the QoS flows conveyed by the failed GTP-U tunnel to the other GTP-U tunnel or to remove the tunnel for the associated QoS flows, and indicate so in the User Plane Failure Indication Report IE in the "PDU Session Resource Modify Response Transfer". In the last two cases, the 5G-AN then initiates a PDU Session Resource Modify Indication procedure towards the SMF to move the associated QoS flows to the other GTP-U tunnel or to remove the tunnel for the associated QoS flows, as specified in clause 8.2.3.2 of 3GPP TS 38.413 [11].

8.	If the PDU session resource is released in the 5G-AN, the SMF initiates the Network Triggered Service Request procedure specified in clause 4.2.3.3 of 3GPP TS 23.502 [5], to re-activate the user plane connection of the PDU session.

8a.	If the 5G-AN decided to move the QoS flows conveyed by the failed GTP-U tunnel to the other GTP-U tunnel or to remove the tunnel for the associated QoS flows, the 5G-AN sends a PDU Session Resource Modify Indication to the AMF which forwards the message to the SMF.

9.	If the 5G-AN allocated a new DL F-TEID in the PDU Session Resource Modify Response, the SMF modifies the PFCP Session to provide the UPF with the new N3 DL F-TEID for the DL FAR, and changes the Apply Action to "FORW", to resume the transfer of DL packets. 

If the 5G-AN moved the associated QoS flows to the other GTP-U tunnel, the SMF modifies the PFCP session to associate these QoS flows to the DL FAR used for forwarding the DL packets to the other GTP-U tunnel.

If the 5G-AN released the GTP-U tunnel, the SMF shall release the associated QoS flows.

10.	The SMF sends a PDU Session Resource Modify Confirm to the 5G-AN (via the AMF), if the 5G-AN sent a PDU Session Resource Modify Indication.

* * * Next Change * * * *

## 5.4	Restoration Procedures upon User Plane Path Failure

Upon detecting a GTP-U user plane path failure as specified in clause 5.2.2, the UPF shall report the user plane path failure to the SMF, by sending a PFCP Node Report Request (see 3GPP TS 29.244 [4]) including a User Plane Path Failure Report with the IP address of the remote GTP-U peer(s) towards which a failure has been detected. The UPF should also notify the GTP-U user plane path failure via the Operation and Maintenance system.

Upon detecting a failed GTP-U user plane path become recovered, the UPF shall report the user plane path recovery to the SMF, by sending a PFCP Node Report Request (see 3GPP TS 29.244 [4]) including a User Plane Path Recovery Report with the IP address of the remote GTP-U peer(s) associated with the recovered user plane path. The UPF should also notify the GTP-U user plane path recovery via the Operation and Maintenance system.

When the SMF receives the PFCP Node Report Request with a User Plane Path Failure Report, the SMF may:

-	delete the PDU session contexts associated with the path in failure; or

-	maintain the PDU session contexts associated with the path in failure during an operator configurable maximum path failure duration. The SMF shall delete the PDU session contexts associated with the path in failure if the path is still down when this duration expires; or

NOTE 1:	During transient path failures (e.g. path failures not exceeding few minutes at most), maintaining the PDU session contexts associated with the peer's IP address enables the delivery of end user services (when the path is re-established again) and this also avoids unnecessary signalling in the network for restoring those PDU sessions.

NOTE 2:	It is not intended to maintain PDU session contexts during long path failures (e.g. exceeding few minutes at most) as this would imply undesirable effects like undue charging.

-	maintain the PDU session contexts associated with the path in failure if the path in failure is towards a 5G-AN. When deciding to maintain the PDU session contexts, it shall send a PFCP Session Modification Request message with changing Apply-Action from FORW to BUFF and NOCP for each affected PFCP session. In addition, upon receipt of subsequent downlink data notifications from the UPF or the service request from affected UE, the SMF will try to reestablish the PDU session resources in the NG-RAN. As an implementation option, the SMF may try to re-establish the PDU session resources in the NG-RAN for those PDU sessions associated with the user plane path in failure prior to receiving downlink data notifications from the UPF or service request from the affected UE.

-	For each split PDU session affected by the user plane path failure, the SMF may use the procedure specified in clause 5.3.2.1 to report to the master 5G-AN node that a user plane path failure has been detected towards one 5G-AN (i.e. for one of the GTP-U tunnel of the split PDU session), in which case the SMF shall include, in the PDU Session Resource Modify Request Transfer" IE, the User Plane Failure Indication IE indicating that a UP Path failure has been detected and the N3 DL F-TEID of the GTP-U tunnel of  the PDU session affected by the path failure.

NOTE 3:	This approach (as above for user plane path failure towards a 5G-AN) allows to maintain the PDU session even for non-transient path failures. For a PDU session with I/V-SMF, the pause of charging (as specified in clause 4.4.4 of 3GPP TS 23.502 [5]) ensures that there is no undue charging.

When deciding to delete the PDU session contexts associated with the path in failure, the SMF shall modify or delete the affected PFCP sessions in the UPF.

NOTE 4:	The SMF need to take care to smoothen the signalling load towards the UPF if a large number of PFCP sessions are affected by the user plane path failure.

* * * End of Changes * * * *