3GPP TSG-CT WG4 Meeting #124	C4-243305

# Maastricht, Netherlands; 19th – 23rd August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 1112 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MDT parameters for M1, M8 and M9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2024-07-31 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Event Threshold for SINR is defined in clause 5.10.7 of 3GPP TS 32.422, and in clause 9.3.1.171 M1 Configuration of 3GPP TS 38.413. - Bluetooth Measurement Configuration Name List is defined in clause 9.3.1.177 of 3GPP TS 38.413 for MDT measurement configuration. WLAN Measurement Configuration Name List is defined in clause 9.3.1.178 of 3GPP TS 38.413 for MDT measurement configuration. For Immediate MDT on M8 and M9, as defined in clause 5.4.1.1 of 3GPP TS 37.320: ⁻ M8: RSSI measurement by UE (for WLAN/Bluetooth measurement) ⁻ M9: RTT Measurement by UE (for WLAN measurement) It is indicated M8 configuration shall indicate WLAN measurement or Bluetooth measurement, so the WLAN RSSI or BT RSSI indication is included in WLAN Measurement Configuration or Bluetooth Measurement Configuration in clauses 9.3.1.177 and 9.3.1.178 of 3GPP TS 38.413. It is proposed to update the ImmediateMdtConf data type to support Event Threshold for SINR, Bluetooth Measurement Configuration and WLAN Measurement Configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the definition of the immediate MDT parameters for M1, M8 and M9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage2. MDT measurements on Bluetooth and WLAN can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1, 6.1.6.2.58, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution introduces backward compatible corrections to the OpenAPI file of Namf_Communication API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Namf_Communication service based interface protocol.

Table 6.1.6.1-1: Namf_Communication specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SubscriptionData | 6.1.6.2.2 | Data within an AMF Status Change Subscription request and response. |
| AmfStatusChangeNotification | 6.1.6.2.3 | Data within an AMF Status Change Notification request. |
| AmfStatusInfo | 6.1.6.2.4 | AMF Status Information |
| AssignEbiData | 6.1.6.2.5 | Data within an EBI assignment request. |
| AssignedEbiData | 6.1.6.2.6 | Data within a successful response to an EBI assignment request. |
| AssignEbiFailed | 6.1.6.2.7 | Represents failed assignment of EBI(s) |
| UEContextRelease | 6.1.6.2.8 | Data within a Release UE Context request. |
| N2InformationTransferReqData | 6.1.6.2.9 | Data within a N2 Information Transfer request containing the N2 information requested to be transferred to 5G AN. |
| NonUeN2InfoSubscriptionCreateData | 6.1.6.2.10 | Data within a create subscription requestfor non-UE specific N2 information notification. |
| NonUeN2InfoSubscriptionCreatedData | 6.1.6.2.11 | Data for the created subscription for non-UE specific N2 information notification. |
| UeN1N2InfoSubscriptionCreateData | 6.1.6.2.12 | Data within a create subscription request for UE specific N1 and/or N2 information notification. |
| UeN1N2InfoSubscriptionCreatedData | 6.1.6.2.13 | Data for the created subscription for UE specific N1 and/or N2 information notification. |
| N2InformationNotification | 6.1.6.2.14 | Data within a N2 information notification request. |
| N2InfoContainer | 6.1.6.2.15 | N2 information container. |
| N1MessageNotification | 6.1.6.2.16 | Data within a N1 message notification request. |
| N1MessageContainer | 6.1.6.2.17 | N1 Message Container. |
| N1N2MessageTransferReqData | 6.1.6.2.18 | Data within a N1/N2 message transfer request. |
| N1N2MessageTransferRspData | 6.1.6.2.19 | Data within a N1/N2 message transfer response. |
| RegistrationContextContainer | 6.1.6.2.20 | Registration Context Container used to send the UE context information, N1 message from UE, AN address etc during Registration with AMF re-allocation procedure. |
| AreaOfValidity | 6.1.6.2.21 | Area of validity information for N2 information transfer |
| UeContextTransferReqData | 6.1.6.2.23 | Data within a UE Context Transfer Request to start transferring of an individual ueContext resource from old AMF to new AMF. |
| UeContextTransferRspData | 6.1.6.2.24 | Data within a successful response to the UE Context Transfer request. |
| UeContext | 6.1.6.2.25 | Represents an individual ueContext resource |
| N2SmInformation | 6.1.6.2.26 | Represents the session management SMF related N2 information data part. |
| N2InfoContent | 6.1.6.2.27 | Represents a transparent N2 information content to be relayed by AMF. |
| NrppaInformation | 6.1.6.2.28 | Represents a NRPPa related N2 information data part. |
| PwsInformation | 6.1.6.2.29 | Represents a PWS related information data part. |
| N1N2MsgTxfrFailureNotification | 6.1.6.2.30 | Data within a N1/N2 Message Transfer Failure Notification request |
| N1N2MessageTransferError | 6.1.6.2.31 | Data within a N1/N2 Message Transfer Error response. |
| N1N2MsgTxfrErrDetail | 6.1.6.2.32 | N1/N2 Message Transfer Error Details |
| N2InformationTransferRspData | 6.1.6.2.33 | Data within a successful response to the N2 Information Transfer request to transfer N2 Information to the AN. |
| MmContext | 6.1.6.2.34 | Represents a Mobility Management Context in UE Context |
| SeafData | 6.1.6.2.35 | Represents SEAF data derived from data received from AUSF |
| NasSecurityMode | 6.1.6.2.36 | Indicates the NAS Security Mode |
| PduSessionContext | 6.1.6.2.37 | Represents a PDU Session Context in UE Context |
| NssaiMapping | 6.1.6.2.38 | Represents the mapping between a S-NSSAI in serving PLMN to a S-NSSAI in home PLMN. |
| UeRegStatusUpdateReqData | 6.1.6.2.39 | Data within a UE registration status update request to indicate a completion of transferring at a target AMF. |
| AssignEbiError | 6.1.6.2.40 | Data within a failure response to the EBI assignment request. |
| UeContextCreateData | 6.1.6.2.41 | Data within a request to create an individual ueContext resource |
| UeContextCreatedData | 6.1.6.2.42 | Data within a successful response for creating an individual ueContext resource |
| UeContextCreateError | 6.1.6.2.43 | Data within a failure response for creating a UE context |
| NgRanTargetId | 6.1.6.2.44 | Indicates a NG RAN as target of the handover |
| N2InformationTransferError | 6.1.6.2.45 | Data within a failure response for a non-UE related N2 Information Transfer. |
| PWSResponseData | 6.1.6.2.46 | Data related PWS included in a N2 Information Transfer response. |
| PWSErrorData | 6.1.6.2.47 | Data related to PWS error included in a N2 Information Transfer failure response. |
| NgKsi | 6.1.6.2.49 | Represents the ngKSI (see 3GPP TS 33.501 [27]) |
| KeyAmf | 6.1.6.2.50 | Represents the Kamf or K'amf. (see 3GPP TS 33.501 [27]). |
| ExpectedUeBehavior | 6.1.6.2.51 | Represents the expected UE behavior (e.g. UE moving trajectory) and its validity period. |
| UeRegStatusUpdateRspData | 6.1.6.2.52 | Data within a UE registration status update response to provide the status of UE context transfer status update at a source AMF. |
| N2RanInformation | 6.1.6.2.53 | Represents the RAN related N2 information data part. |
| N2InfoNotificationRspData | 6.1.6.2.54 | Data within a N2 information notification response. |
| SmallDataRateStatusInfo | 6.1.6.2.55 | Represents the small data rate status |
| SmfChangeInfo | 6.1.6.2.56 | SMF change information for PDU session(s) |
| V2xContext | 6.1.6.2.57 | Represents the V2X services related parameters |
| ImmediateMdtConf | 6.1.6.2.58 | Immediate MDT Configuration |
| V2xInformation | 6.1.6.2.59 | V2X related N2 information |
| EpsNasSecurityMode | 6.1.6.2.60 | Indicates the EPS NAS Security Mode |
| UeContextRelocateData | 6.1.6.2.61 | Data within a Relocate UE Context request |
| UeContextRelocatedData | 6.1.6.2.62 | Data within a Relocate UE Context |
| EcRestrictionDataWb | 6.1.6.2.64 | Enhanced Coverage Restriction Data for WB-N1 mode. |
| ExtAmfEventSubscription | 6.1.6.2.65 | AMF event subscription extended with additional information received for the subscription |
| AmfEventSubscriptionAddInfo | 6.1.6.2.66 | Additional information received for an AMF event subscription, e.g. binding indications. |
| UeContextCancelRelocateData | 6.1.6.2.67 | Data structure used for cancellation of UE Context Relocation. |
| UeDifferentiationInfo | 6.1.6.2.68 | Represents the UE Differentiation Information and its validity time. |
| CeModeBInd | 6.1.6.2.69 | CE-mode-B Support Indicator |
| LteMInd | 6.1.6.2.70 | LTE-M Indication |
| NpnAccessInfo | 6.1.6.2.71 | NPN Access Information |
| ProseContext | 6.1.6.2.72 | Represents the ProSE services related parameters |
| AnalyticsSubscription | 6.1.6.2.73 | Analytics subscriptions created in the NWDAF. |
| NwdafSubscription | 6.1.6.2.74 | Individual NWDAF subscription identified by the subscription Id. |
| UpdpSubscriptionData | 6.1.6.2.75 | UE policy delivery related N1 message notification subscription data |
| ProSeInformation | 6.1.6.2.76 | 5G ProSe related N2 information. |
| ReleaseSessionInfo | 6.1.6.2.77 | PDU session Id(s) and the cause for triggering the release |
| AreaOfInterestEventState | 6.1.6.2.78 | Area Of Interest Event State in old AMF |
| TssInformation | 6.1.6.2.79 | TSS related N2 information |
| AmPolicyInfoContainer | 6.1.6.2.80 | AM Policy Information Container |
| RslpInformation | 6.1.6.2.81 | Ranging/SL positioning related N2 information |
| A2xContext | 6.1.6.2.82 | Represents the A2X services related parameters |
| A2xInformation | 6.1.6.2.83 | A2X related N2 information |
| LcsUpContext | 6.1.6.2.84 | Represents the LCS UP related parameters |
| DeregInactTimerInfo | 6.1.6.2.85 | Network Slice Deregistration Inactivity Timer Information |
| TssRspPerNgran | 6.1.6.2.86 | TSS Information from NG-RAN |
| SliceReplacementMapping | 6.1.6.2.87 | Represents the mapping between a replaced S-NSSAI in serving PLMN to its alternative S-NSSAI. |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Identifier |
| Ppi | 6.1.6.3.2 | Paging Policy Indicator |
| NasCount | 6.1.6.3.2 | Represents a NAS COUNT |
| 5GMmCapability | 6.1.6.3.2 | Represents a 5GMM capability |
| UeSecurityCapability | 6.1.6.3.2 | Represents a UE Security Capability |
| S1UeNetworkCapability | 6.1.6.3.2 | Represents a S1 UE Network Capability |
| DrxParameter | 6.1.6.3.2 | Indicates the UE DRX Parameters |
| OmcIdentifier | 6.1.6.3.2 | Represents the OMC Identifier |
| MSClassmark2 | 6.1.6.3.2 | Indicates the MS Classmark 2 of a 5G SRVCC UE |
| SupportedCodec | 6.1.6.3.2 | Indicates the supported codec of a 5G SRVCC UE |
| StatusChange | 6.1.6.3.3 | Enumeration for AMF status |
| N2InformationClass | 6.1.6.3.4 | Enumeration for N2 Information Class |
| N1MessageClass | 6.1.6.3.5 | Enumeration for N1 Message Class |
| N1N2MessageTransferCause | 6.1.6.3.6 | Enumeration for N1N2Message Transfer Cause |
| UeContextTransferStatus | 6.1.6.3.7 | Describes the status of an individual ueContext resource in UE Context Transfer procedures |
| N2InformationTransferResult | 6.1.6.3.8 | Describes the result of N2 information transfer by AMF to the AN. |
| CipheringAlgorithm | 6.1.6.3.9 | Indicates the supported Ciphering Algorithm |
| IntegrityAlgorithm | 6.1.6.3.10 | Indicates the supported Integrity Algorithm |
| SmsSupport | 6.1.6.3.11 | Indicates the supported SMS delivery of a UE. |
| ScType | 6.1.6.3.12 | Indicates the security context type. |
| KeyAmfType | 6.1.6.3.13 | Indicates the Kamf type. |
| TransferReason | 6.1.6.3.14 | Indicates UE Context Transfer Reason |
| PolicyReqTrigger | 6.1.6.3.15 | Policy Request Triggers |
| RatSelector | 6.1.6.3.16 | Indicates the RAT type for the transfer of N2 information |
| NgapIeType | 6.1.6.3.17 | Indicates the supported NGAP IE types |
| N2InfoNotifyReason | 6.1.6.3.18 | N2 Information Notify Reason |
| SmfChangeIndication | 6.1.6.3.19 | Indicates the I-SMF or V-SMF change or removal |
| SbiBindingLevel | 6.1.6.3.20 | SBI Binding Level |
| EpsNasCipheringAlgorithm | 6.1.6.3.21 | Indicates the supported EPS NAS Ciphering Algorithm |
| EpsNasIntegrityAlgorithm | 6.1.6.3.22 | Indicates the supported EPS NAS Integrity Algorithm |
| PeriodicCommunicationIndicator | 6.1.6.3.23 | Indicates the Periodic Communication Indicator |
| UuaaMmStatus | 6.1.6.3.24 | Indicates UUAA-MM status |
| ReleaseCause | 6.1.6.3.25 | The cause for triggering the release |
| NgranFailureInfo | 6.1.6.3.26 | Information related to a NG-RAN failure |
| XrDeviceWith2Rx | 6.1.6.3.27 | XR Device with 2Rx information |

Table 6.1.6.1-2 specifies data types re-used by the Namf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Namf_Communication service based interface.

Table 6.1.6.1-2: Namf_Communication re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Snssai | 3GPP TS 29.571 [6] |  |
| Arp | 3GPP TS 29.571 [6] |  |
| PduSesisonId | 3GPP TS 29.571 [6] |  |
| Guami | 3GPP TS 29.571 [6] | Globally Unique AMF Identifier |
| AmfName | 3GPP TS 29.571 [6] | The name of the AMF |
| Supi | 3GPP TS 29.571 [6] | Subscription Permanent Identifier |
| Cause | 3GPP TS 29.571 [6] | 5G-AN Cause |
| ProblemDetails | 3GPP TS 29.571 [6] | Detailed problems in failure case |
| supportedFeatures | 3GPP TS 29.571 [6] | Supported Features |
| TimeZone | 3GPP TS 29.571 [6] |  |
| UserLocation | 3GPP TS 29.571 [6] |  |
| AccessType | 3GPP TS 29.571 [6] |  |
| AllowedNssai | 3GPP TS 29.531 [18] |  |
| NfInstanceId | 3GPP TS 29.571 [6] |  |
| Uri | 3GPP TS 29.571 [6] |  |
| Ecgi | 3GPP TS 29.571 [6] | EUTRA Cell Identifier |
| Ncgi | 3GPP TS 29.571 [6] | NR Cell Identifier |
| Uint16 | 3GPP TS 29.571 [6] |  |
| 5Qi | 3GPP TS 29.571 [6] | 5G QoS Identifier |
| CorrelationID | 3GPP TS 29.572 [25] | LCS Correlation ID |
| Pei | 3GPP TS 29.571 [6] |  |
| Dnn | 3GPP TS 29.571 [6] |  |
| Gpsi | 3GPP TS 29.571 [6] |  |
| GroupId | 3GPP TS 29.571 [6] |  |
| PlmnId | 3GPP TS 29.571 [6] |  |
| RfspIndex | 3GPP TS 29.571 [6] |  |
| EbiArpMapping | 3GPP TS 29.502 [16] | EBI - ARP mapping |
| NsiId | 3GPP TS 29.531 [18] |  |
| TraceData | 3GPP TS 29.571 [6] | Trace control and configuration parameters |
| ConfiguredSnssai | 3GPP TS 29.531 [18] |  |
| NgApCause | 3GPP TS 29.571 [6] | Represents the NG AP cause IE |
| Area | 3GPP TS 29.571 [6] |  |
| ServiceAreaRestriction | 3GPP TS 29.571 [6] |  |
| CoreNetworkType | 3GPP TS 29.571 [6] |  |
| Ambr | 3GPP TS 29.571 [6] |  |
| SliceMbr | 3GPP TS 29.571 [6] |  |
| GlobalRanNodeId | 3GPP TS 29.571 [6] |  |
| NfGroupId | 3GPP TS 29.571 [6] | Network Function Group Id |
| DurationSec | 3GPP TS 29.571 [6] |  |
| StnSr | 3GPP TS 29.571 [6] | Session Transfer Number for SRVCC |
| CMsisdn | 3GPP TS 29.571 [6] | Correlation MSISDN |
| DateTime | 3GPP TS 29.571 [6] |  |
| SmallDataRateStatus | 3GPP TS 29.571 [6] |  |
| NfSetId | 3GPP TS 29.571 [6] | NF Set ID |
| NfServiceSetId | 3GPP TS 29.571 [6] | NF Service Set ID |
| LMFIdentification | 3GPP TS 29.572 [25] | LMF Identification |
| PlmnAssiUeRadioCapId | 3GPP TS 29.571 [6] |  |
| ManAssiUeRadioCapId | 3GPP TS 29.571 [6] |  |
| NrV2xAuth | 3GPP TS 29.571 [6] | NR V2X services authorized |
| LteV2xAuth | 3GPP TS 29.571 [6] | LTE V2X services authorized |
| BitRate | 3GPP TS 29.571 [6] | Bit Rate |
| Pc5QoSPara | 3GPP TS 29.571 [6] | PC5 QoS parameters |
| PduSessionInfo | 3GPP TS 29.571 [6] | The Slice and DNN combination of a PDU session. |
| PcfUeCallbackInfo | 3GPP TS 29.571 [6] | The callback information of the PCF for the UE to allow the PCF for the PDU session to send SM Policy Association Establishment and Termination events notification. |
| CnAssistedRanPara | 3GPP TS 29.502 [16] | SMF derived CN assisted RAN Parameters Tuning |
| MoExpDataCounter | 3GPP TS 29.571 [6] | MO Exception Data Counter |
| CagData | 3GPP TS 29.503 [35] | Closed Access Group Data |
| NssaaStatus | 3GPP TS 29.571 [6] | Subscribed S-NSSAI subject to NSSAA procedure and the status |
| JobType | 3GPP TS 29.571 [6] | Job Type in the trace |
| MeasurementLteForMdt | 3GPP TS 29.571 [6] | Measurements used for MDT in LTE in the trace |
| MeasurementNrForMdt | 3GPP TS 29.571 [6] | Measurements used for MDT in NR in the trace |
| ReportingTrigger | 3GPP TS 29.571 [6] | Reporting Triggers for MDT in the trace |
| ReportIntervalMdt | 3GPP TS 29.571 [6] | Report Interval for MDT in LTE in the trace |
| ReportAmountMdt | 3GPP TS 29.571 [6] | Report Amount for MDT in the trace |
| CollectionPeriodRmmLteMdt | 3GPP TS 29.571 [6] | Collection period for RRM measurements LTE for MDT in the trace |
| MeasurementPeriodLteMdt | 3GPP TS 29.571 [6] | Measurement period LTE for MDT in the trace in |
| AreaScope | 3GPP TS 29.571 [6] | Area Scope |
| PositioningMethodMdt | 3GPP TS 29.571 [6] | Positioning Method for MDT in the trace in LTE |
| ReportIntervalNrMdt | 3GPP TS 29.571 [6] | Report Interval for MDT in NR in the trace |
| CollectionPeriodRmmNrMdt | 3GPP TS 29.571 [6] | Collection period for RRM measurements NR for MDT in the trace |
| SensorMeasurement | 3GPP TS 29.571 [6] | Sensor information for MDT in the trace |
| ScheduledCommunicationTime | 3GPP TS 29.571 [6] | Scheduled Communication Time |
| StationaryIndication | 3GPP TS 29.571 [6] | Stationary Indication |
| TrafficProfile | 3GPP TS 29.571 [6] | Traffic Profile |
| BatteryIndication | 3GPP TS 29.571 [6] | Battery Indication |
| NFType | 3GPP TS 29.510 [29] | NF type |
| UeAuth | 3GPP TS 29.571 [6] | UE authorisation for PC5 service |
| PartitioningCriteria | 3GPP TS 29.571 [6] | Partitioning Criteria |
| RedirectResponse | 3GPP TS 29.571 [6] | Response body of the redirect response message. |
| CagId | 3GPP TS 29.571 [6] | CAG ID |
| NnwdafEventsSubscription | 3GPP TS 29.520 [52] | Represents an Individual NWDAF Event Subscription resource |
| PresenceInfo | 3GPP TS 29.571 [6] |  |
| UePositioningCapabilities | 3GPP TS 29.572 [25] | Indicates the positioning capabilities supported by the UE. |
| SmfSelectionData | 3GPP TS 29.507 [32] |  |
| EpsInterworkingInfo | 3GPP TS 29.503 [35] |  |
| IpAddress | 3GPP TS 29.503 [35] |  |
| Fqdn | 3GPP TS 29.571 [6] |  |
| PresenceState | 3GPP TS 29.571 [6] | Presence State |
| SatelliteBackhaulCategory | 3GPP TS 29.571 [6] | Satellite Backhaul Category |
| WirelineServiceAreaRestriction | 3GPP TS 29.571 [6] |  |
| PartiallyAllowedSnssai | 3GPP TS 29.571 [6] | S-NSSAI with TAIs information |
| VarRepPeriod | 3GPP TS 29.571 [6] | Variable Reporting Periodicity |
| AsTimeDistributionParam | 3GPP TS 29.507 [32] |  |
| PruInd | 3GPP TS 29.503 [35] | PRU Indicator |
| SliceUsageControlInfo | 3GPP TS 29.571 [6] | Slice Usage Control Information |
| MbsrOperationAllowed | 3GPP TS 29.503 [35] | MBSR Operation Information |
| UpConnectionStatus | 3GPP TS 29.572 [25] | User Plane Connection Status |
| LMFIdentification | 3GPP TS 29.572 [25] |  |
| TaiRange | 3GPP TS 29.510 [29] | TAI range |
| BluetoothMeasurement | 3GPP TS 29.571 [6] | Bluetooth Measurement Configuration |
| WlanMeasurement | 3GPP TS 29.571 [6] | WLAN Measurement Configuration |

* * * Next Change * * * *

6.1.6.2.58	Type: ImmediateMdtConf

Table 6.1.6.2.58-1: Definition of type ImmediateMdtConf

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| jobType | JobType | M | 1 | This IE shall indicate the Job type for MDT, see 3GPP TS 32.422 [30]. |
| measurementLteList | array(MeasurementLteForMdt) | C | 1..N | This IE shall be present if available. When present, this IE shall contain a list of the measurements that shall be collected for LTE. |
| measurementNrList | array(MeasurementNrForMdt) | C | 1..N | This IE shall be present if available, when present, this IE shall contain a list of the measurements that shall be collected for NR. |
| reportingTriggerList | array(ReportingTrigger) | C | 1..N | This IE shall be present if available. When present, this IE shall contain a list of the reporting triggers. |
| reportInterval | ReportIntervalMdt | C | 0..1 | This IE shall be present if available. When present, this IE shall indicate the interval between the periodical measurements to be taken when UE is in connected in LTE. |
| reportIntervalNr | ReportIntervalNrMdt | C | 0..1 | This IE shall be present if available. When present, this IE shall indicate the interval between the periodical measurements to be taken when UE is in connected in NR. |
| reportAmount | ReportAmountMdt | C | 0..1 | This IE shall be present if available. When present, this IE shall indicate the number of measurement reports that shall be taken for periodical reporting while UE is in connected. |
| eventThresholdRsrp | integer | C | 0..1 | This IE shall be present if available. When present, this IE shall indicate the Event Threshold for RSRP. Minimum = 0. Maximum = 97. |
| eventThresholdRsrq | integer | C | 0..1 | This IE shall be present if available. When present, this IE shall indicate the Event Threshold for RSRQ. Minimum = 0. Maximum = 34. |
| eventThresholdRsrpNr | integer | C | 0..1 | This IE shall be present if available. When present, this IE shall indicate the Event Threshold for RSRP in NR. Minimum = 0. Maximum = 127. |
| eventThresholdRsrqNr | integer | C | 0..1 | This IE shall be present if available. When present, this IE shall indicate the Event Threshold for RSRQ in NR. Minimum = 0. Maximum = 127. |
| collectionPeriodRmmLte | CollectionPeriodRmmLteMdt | C | 0..1 | This IE shall be present if available. When present, it shall contain the collection period that should be used to collect available measurement samples in case of RRM configured measurements. |
| collectionPeriodRmmNr | CollectionPeriodRmmNrMdt | C | 0..1 | This IE shall be present if available. When present, it shall contain the collection period that should be used to collect available measurement samples in case of RRM configured measurements when UE is in NR. |
| measurementPeriodLte | MeasurementPeriodLteMdt | C | 0..1 | This IE shall be present if available. When present, it shall contain the measurement period that should be used for the Data Volume and Scheduled IP Throughput measurements in LTE. |
| areaScope | AreaScope | O | 0..1 | When present, this IE shall contain the area in Cells or Tracking Areas where the MDT data collection shall take place, see 3GPP TS 32.422 [30]. |
| positioningMethod | PositioningMethodMdt | O | 0..1 | When present, it shall indicate the positioning method that shall be used for the MDT job in LTE. |
| addPositioningMethodList | array(PositioningMethodMdt) | O | 1..N | This IE may be present if positioningMethod IE is present. When present, it shall indicate a list of the additional positioning methods that shall be used for the MDT job |
| mdtAllowedPlmnIdList | array(PlmnId) | O | 1..16 | When present, this IE shall contain the PLMNs related to MDT. |
| sensorMeasurementList | array(SensorMeasurement) | C | 1..N | This IE shall be present if available. When present, this IE shall include a list the sensor measurements to be collected for UE in NR if they are available. |
| sensorMeasurementLteList | array(SensorMeasurement) | C | 1..N | This IE shall be present if available. When present, this IE shall include a list of the sensor measurements to be collected for UE in LTE if they are available. |
| eventThresholdSinrNr | integer | C | 0..1 | This IE shall be present if the report trigger parameter is configured for A2 event reporting or A2 event triggered periodic reporting and the job type parameter is configured for Immediate MDT or combined Immediate MDT and Trace in NR. When present, this IE shall indicate the Event Threshold for SINR, and the value shall be between 0-127. |
| bluetoothMeasurementNr | BluetoothMeasurement | O | 0..1 | When present, this IE shall include the bluetooth measurements to be collected for UE in NR if they are available. |
| bluetoothMeasurementLte | BluetoothMeasurement | O | 0..1 | When present, this IE shall include the bluetooth measurements to be collected for UE in LTE if they are available. |
| wlanMeasurementNr | WlanMeasurement | O | 0..1 | When present, this IE shall include the WLAN Measurements to be collected for UE in NR if they are available. |
| wlanMeasurementLte | WlanMeasurement | O | 0..1 | When present, this IE shall include the WLAN Measurements to be collected for UE in LTE if they are available. |

* * * Next Change * * * *

A.2	Namf_Communication API

openapi: 3.0.0

info:

version: 1.3.0

title: Namf_Communication

description: |

AMF Communication Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

ImmediateMdtConf:

description: Immediate MDT Configuration

type: object

properties:

jobType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/JobType'

measurementLteList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MeasurementLteForMdt'

minItems: 1

measurementNrList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MeasurementNrForMdt'

minItems: 1

reportingTriggerList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ReportingTrigger'

minItems: 1

reportInterval:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ReportIntervalMdt'

reportIntervalNr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ReportIntervalNrMdt'

reportAmount:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ReportAmountMdt'

eventThresholdRsrp:

type: integer

minimum: 0

maximum: 97

eventThresholdRsrq:

type: integer

minimum: 0

maximum: 34

eventThresholdRsrpNr:

type: integer

minimum: 0

maximum: 127

eventThresholdRsrqNr:

type: integer

minimum: 0

maximum: 127

collectionPeriodRmmLte:

$ref: 'TS29571_CommonData.yaml#/components/schemas/CollectionPeriodRmmLteMdt'

collectionPeriodRmmNr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/CollectionPeriodRmmNrMdt'

measurementPeriodLte:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MeasurementPeriodLteMdt'

areaScope:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AreaScope'

positioningMethod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PositioningMethodMdt'

addPositioningMethodList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PositioningMethodMdt'

minItems: 1

mdtAllowedPlmnIdList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

minItems: 1

maxItems: 16

sensorMeasurementList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SensorMeasurement'

minItems: 1

sensorMeasurementLteList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SensorMeasurement'

minItems: 1

eventThresholdSinrNr:

type: integer

minimum: 0

maximum: 127

bluetoothMeasurementNr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BluetoothMeasurement'

bluetoothMeasurementLte:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BluetoothMeasurement'

wlanMeasurementNr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/WlanMeasurement'

wlanMeasurementLte:

$ref: 'TS29571_CommonData.yaml#/components/schemas/WlanMeasurement'

required:

- jobType

[…]

* * * * End of Changes * * * *