Third Generation Partnership Project (3GPP™)

DRAFT Meeting Report v0.0.1
for
TSG CT WG4
meeting: CT4#81

Reno, US, 27/11/2017 to 01/12/2017

this draft:  Friday, (2017-12-11) 17:25  [date/time according to secretary's PC time zone setting]

Contents:

Contents

1	Opening of the meeting and approval of the agenda	5

1.1	IPR Call	5

1.2	Antitrust declarations	5

1.3	Reminder for delegates attending the meeting	5

2	Allocation of documents to agenda items	5

3	Meeting Reports	6

4	Input liaison statements: allocation to agenda items as appropriate	6

5	Work item management	7

6	Release 15	11

6.1	CT4 Led WIs	11

6.1.1	EPC enhancements to support 5G New Radio via Dual Connectivity, CT aspects [EDCE5-CT]	11

6.1.2	CT aspects of unlicensed spectrum offloading system enhancements [USOS-CT]	17

6.2	CT4 Supported WIs	19

6.2.1	CT aspects on 5G System - Phase 1 [5GS_Ph1-CT]	19

6.2.1.1	Contributions to TR 29.891	22

6.2.1.2	Impacted Specifications	30

6.2.1.3	Contributions to TS 29.500	41

6.2.1.4	Contributions to TS 29.501	47

6.2.1.5	Contributions to TS 29.502	54

6.2.1.6	Contributions to TS 29.503	60

6.2.1.7	Contributions to TS 29.509	62

6.2.1.8	Contributions to TS 29.510	64

6.2.1.9	Contributions to TS 29.511	66

6.2.1.10	Contributions to TS 29.518	68

6.2.1.11	Contributions to TS 23.527	73

6.2.1.12	Contributions to TS 29.531	73

6.2.1.13	Contributions to TS 29.540	73

6.2.1.14	Contributions to TS 29.544	75

6.2.1.15	Contributions to TS 29.571	75

6.2.2	IMS impact due to 5GS IP-CAN [IMSo5G]	78

6.2.3	CT aspects of Northbound APIs for SCEF – SCS/AS Interworking [NAPS-CT]	78

6.2.4	CT aspects of support of voice services over WLAN Access [VoWLAN-CT]	78

6.2.5	WLAN direct discovery technologies for ProSe direct discovery [ProSe_WLAN_DD_Stage3]	78

6.2.6	CT aspects of 3GPP PS data off function [PS_DATA_OFF2-CT]	79

6.3	Any Other Business for Rel-15	79

6.3.1	GTP and PMIP [TEI15]	80

6.3.2	Addressing and Subscriber Data handling (23.003, 23.008) [TEI15]	81

6.3.3	EPS AAA interfaces (29.273) [TEI15]	81

6.3.4	Diameter based Interfaces (29.272, 29.173) [TEI15]	81

6.3.5	IMS [TEI15]	81

6.3.6	MAP [TEI15]	81

6.3.7	H.248 Interface [TEI15]	81

6.3.8	Diameter 29.230 CRs [TEI15]	81

6.3.9	SIMTC [TEI15]	81

6.3.10	CIoT [TEI15]	81

6.3.11	MONTE [TEI15]	81

6.3.12	CIoT	83

6.3.13	AOB Rel-15	83

7	Rel-14	83

7.1	CT4 Led WIDs	83

7.1.1	CT aspects of MTSI Extension on Multi-stream Multiparty Conferencing Media Handling [MMCMH-CT]	83

7.1.2	SCC AS Restoration [SCCAS_RES]	83

7.1.3	Shared Subscription Data Update [eSDU]	83

7.1.4	EIR check for WLAN access to EPC [EWE-CT]	83

7.1.5	CT aspects of Control and User Plane Separation of EPC nodes [CUPS-CT]	83

7.1.6	Diameter Load control mechanisms [DLoCME]	90

7.1.7	Support of EAP Re-authentication Protocol for WLAN Interworking [ERP-CT]	90

7.1.8	Diameter Base Protocol Specification Update [DBPU]	90

7.2	CT4 Supported WIDs	90

7.2.1	CT aspects for Non-IP for Cellular Internet of Things for 2G/3G-GPRS [NonIP_GPRS-CT]	90

7.2.2	CT aspects of Support of Emergency services over WLAN – phase 2 [SEW2-CT]	90

7.2.3	CT aspects of V2X Services [V2X-CT]	92

7.2.4	Enhanced DÉCOR [eDECOR-CT]	92

7.2.5	Protocol enhancements for Mission Critical Push To Talk over LTE [MCPTTProtoc1]	92

7.2.6	CT Aspects of Improvements of Awareness of User Location Change [AULC-CT]	92

7.2.7	CT aspects of system architecture enhancements for TV service [AE_enTV-CT]	92

7.2.8	CT aspects of extended Architecture support for CIoT [CIoT-Ext-CT]	92

7.2.9	CT aspects of signalling reduction to enable light connection for LTE [LTE_LIGHT_CON-CT]	94

7.2.10	CT aspects of Enhancements for Mission Critical Push To Talk [MCImp-eMCPTT-CT]	94

7.2.11	CT aspects of Mission Critical Data [MCImp-MCDATA-CT]	94

7.2.12	CT aspects of Mission Critical Video [MCImp-MCVIDEO-CT]	94

7.2.13	7.2.13 IMS Signalling Activated Trace [ISAT]	94

7.3	Any other Business for Rel-14	94

7.3.1	GTP and PMIP [TEI14]	94

7.3.2	Addressing and Subscriber Data Handling (23.003 and 23.008) [TEI14]	94

7.3.3	EPS AAA interfaces (29.273) [TEI14]	94

7.3.4	Diameter based Interfaces (29.272, 29.173) [TEI14]	94

7.3.5	IMS [TEI14]	94

7.3.6	MAP [TEI14]	94

7.3.7	H.248 Interfaces [TEI14]	94

7.3.8	Diameter 29.230 CRs [TEI14]	94

7.3.9	DSCP Marking based on QCI and ARP [TEI14]	94

7.3.10	CIoT [TEI14]	94

7.3.11	MONTE-TEI14 [TEI14]	94

7.3.12	AOB	95

8	Rel-13	96

8.1	CT4 Led WIDs	96

8.1.1	Support of RTP Transport Multiplexing (signalling) in IMS [RTP-MUX]	96

8.1.2	CT aspects of voice over E-UTRAN Paging Policy Differentiation [voE-UTRAN_PPD-CT]	96

8.1.3	P-CSCF restoration enhancements with WLAN [PCSCF_RES_WLAN]	96

8.1.4	Diameter Load control mechanisms [DLoCME]	96

8.1.5	Study on EPC Signalling Improvement for Race Scenarios [FS_EPC_SIG_RACE]	96

8.1.6	Shared Data for Multiple Subscribers [SHARED_SubData_UPD]	96

8.1.7	CT aspects of EVS in 3G Circuit-Switched Networks [EVSoCS-CT]	96

8.1.8	H.248 Aspects of WebRTC Data Channel on IMS Access Gateway	96

8.1.9	Monitoring Enhancements CT aspects [MONTE-CT]	96

8.1.10	Mobile Equipment signalling over the WLAN access [MEI_WLAN]	98

8.1.11	Study on SCC AS Restoration [SCCAS_RES]	98

8.1.12	SDP Capability Negotiation for IMS Media Plane [SDPCN_IMS]	98

8.1.13	CT aspects of Optimizations to Support High Latency Communications [HLcom-CT]	98

8.1.14	CT aspects of Group based Enhancements [GROUPE-CT]	98

8.1.15	CT Aspects of Video Enhancements by Region-of-Interest Information Signalling [ROI-CT]	98

8.1.16	S6a/S6d Shared Data Update [FS_eSDU]	98

8.1.17	EPC Signalling Improvement for Race Scenarios [EPC_SIG_RACE]	98

8.1.18	Dedicated Core Networks [DECOR-CT]	98

8.1.19	Feasibility Study on the Diameter Base Protocol Update [FS_DBPU]	98

8.1.20	Diameter Message Priority [DiaPri]	98

8.1.21	Indoor Positioning Enhancements for UTRA and E-UTRA [UTRA_LTE_iPos_enh-CT]	98

8.2	CT4 Supported WIDs	98

8.2.1	CT aspects of User Plane Congestion Management for BB I (CT3) [UPCON-DOTCON-CT]	98

8.2.2	Enhanced P-CSCF discovery using signalling for access to EPC via WLAN (CT1) [ePCSCF_WLAN]	98

8.2.3	CT aspects of QoS End to End MTSI extensions (CT3) [QOSE2EMTSI-CT]	98

8.2.4	Warning Status Report in EPS (CT1) [WSR_EPS]	98

8.2.5	IMS Signalling Activated Trace (CT1) [ISAT]	98

8.2.6	IP Flow Mobility support for S2a and S2b Interfaces on stage 3 (CT1) [NBIFOM]	98

8.2.7	CT aspects of enhancements to Proximity-based Services extensions (CT1) [eProSe-Ext-CT]	98

8.2.8	CT aspects of Architecture Enhancements for Service Capability (CT3) [AESE-CT]	98

8.2.9	Support of Emergency services over WLAN Ã¢â‚¬â€œ phase 1 (CT1) [SEW1-CT]	99

8.2.10	Extended DRX cycle for power consumption optimization (CT1) [eDRX]	99

8.2.11	Mission Critical Push To Talk over LTE protocol aspects (CT1) [MCPTT-CT]	99

8.2.12	Enhancements to WEBRTC interoperability stage 3 (CT1)  [eWebRTCi_CT]	99

8.2.13	MBMS Enhancements (CT3)  [MBMS_enh-CT]	99

8.2.14	CIoT-CT (CT1)  [CIoT-CT]	99

8.3	Any other Business for Rel-13	100

8.3.1	GTP and PMIP [TEI13]	100

8.3.2	Diameter 29.230 CRs [TEI13]	100

8.3.3	EPS AAA interfaces [TEI13]	100

8.3.4	Restoration Procedures [TEI13]	100

8.3.5	Addressing and Subscriber Data Handling [TEI13]	100

8.3.6	Diameter based Interfaces [TEI13]	100

8.3.7	IMS [TEI13]	100

8.3.8	MAP [TEI13]	100

8.3.9	H.248 Interfaces	100

8.3.10	I-WLAN Discontinuation [TEI13]	101

9	Release 12 and Earlier	101

9.1	GTP and PMIP [TEI8, TEI9, TEI10, TEI11,TEI12]	101

9.2	Addressing and Subscriber Data Handling (23.003 and 23.008) [TEI8, TEI9, TEI10, TEI11,TEI12]	104

9.3	IMS [TEI8, TEI9, TEI10, TEI11,TEI12]	104

9.4	Diameter based Interfaces (29.272, 29.173) [TEI8, TEI9, TEI10, TEI11,TEI12]	105

9.5	EPS AAA interfaces (29.273) [TEI8, TEI9, TEI10, TEI11,TEI12]	105

9.6	MAP and MAP IWF [TEI8, TEI9, TEI10, TEI11,TEI12]	105

9.7	Diameter 29.230 CRs [TEI8, TEI9, TEI10, TEI11,TEI12]	105

9.8	Restoration Procedures (23.007) [TEI8, TEI9, TEI10, TEI11,TEI12]	105

9.9	H.248 Interfaces [TEI8, TEI9, TEI10, TEI11,TEI12]	105

9.10	Any Other Business for Rel-12 and Earlier [TEI8, TEI9, TEI10, TEI11,TEI12]	105

10	Update of the Work Plan	105

11	AoB	105

11.1	Updated list of rapporteurs	105

12	Future meetings	105

13	Check of approved output documents	105

14	Closing of the meeting	106

15	History table	106

Annex A: List of contribution documents	107

Annex B: List of change requests	119

Annex C: Lists of liaisons	128

C1: Incoming liaison statements	128

C2: Outgoing liaison statements	128

Annex D: List of agreed/approved new and revised Work Items	129

Annex E: List of draft Technical Specifications and Reports	130

Annex F: List of action items	131

Annex G: List of decisions	132

Annex H: List of participants	133

Annex I: List of future meetings	135

## 1	Opening of the meeting and approval of the agenda

The meeting was chaired by Mr Lionel Morand (Orange) The Chairman welcomed the delegates to Reno, US and gave some practical information.

### 1.1	IPR Call

The attention of the delegates to the meeting of this Technical Specification Group was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of.

The delegates were asked to take note that they were thereby invited:

to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP;

to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms.

### 1.2	Antitrust declarations

The attention of the delegates to the meeting was drawn to the fact that 3GPP activities were subject to all applicable antitrust and competition laws and that compliance with said laws was therefore required by any participant of the meeting, including the Chairman and Vice-Chairmen and they were invited to seek any clarification needed with their legal counsel. The leadership would conduct the present meeting with impartiality and in the interests of 3GPP. Delegates were reminded that timely submission of work items in advance of TSG/WG meetings was important to allow for full and fair consideration of such matters.

### 1.3	Reminder for delegates attending the meeting

The chairman reminded the delegates that they should sign the participation register on-line from this meeting onwards, and should print and wear their badges.

## 2	Allocation of documents to agenda items

C4-176000	Draft Agenda
					Source: CT4 Chairman

Decision: 		The document was noted.

C4-176001	Detailed agenda & time plan for CT4#81: status at document deadline
					Source: CT4 Chairman

Decision: 		The document was noted.

C4-176002	Detailed agenda & time plan for CT4#81: status on eve of meeting
					Source: CT4 Chairman

Discussion:

Small agenda modifications were made.

Decision: 		The document was agreed.

C4-176003	Proposed allocation of documents to agenda items for CT4#81: status at document deadline
					Source: CT4 Chairman

Decision: 		The document was noted.

C4-176004	Proposed allocation of documents to agenda items for CT4#81: status on eve of meeting
					Source: CT4 Chairman

Decision: 		The document was agreed.

## 3	Meeting Reports

C4-176005	Previous CT4 meeting report
					Source: MCC

Decision: 		The document was approved.

## 4	Input liaison statements: allocation to agenda items as appropriate

C4-176156	Reply LS on Early Data Transmission
					Source: CT1

Abstract:

3GPP TSG CT WG1 asks SA2 group to consider clarifying in their specs that the MMEC pools of the PLMNs sharing the network should not overlap.

Decision: 		The document was noted.

C4-176158	Reply LS on the number of bearers
					Source: CT1

Abstract:

3GPP TSG CT WG1 kindly asks SA2 to consider the information provided in this LS.

Decision: 		The document was noted.

C4-176377	LS reply on support of Trace and MDT in NG-RAN in rel-15
					Source: SA5

Discussion:

MDT will not be covered in Rel-15.

CT4 need to wait SA5 requirement on Trace.

Decision: 		The document was noted.

C4-176384	LS on extension of S6x interface for BEST
					Source: SA3

Abstract:

SA have approved 3GPP TS 33.163 Rel.15, a normative specification for the BEST (Battery Efficient Security for very low Throughput MTC Devices). This service provides end-to-end security and SA3 decided that the LTE authentication vectors (CK, IK, RAND, AUTN, XRES) can be re-used for the purpose of setting up a secure connection between the UE and a newly introduced element in the home network, namely the HSE. In order to make that work, the HSE would have to be able to fetch LTE authentication vectors from the HSS and SA3 has identified that the S6a interface as a potential interface.

In LS S3-170671 (attached) SA3 informed CT4 of its intention to specify the S6a interface and kindly requested CT4 to update their specification, TS 29.272 accordingly and/or inform SA3 of any trouble while doing so or if CT4 is of the opinion that a new interface is necessary.

Another point that SA3 would like to raise is that during the BEST work a new energy efficient stage-3 protocol was designed 'EMSDP'. This protocol is now contained in an Annex. SA3 therefore kindly asks CT4 to review the protocol and inform SA3 of whether or not CT4 would like to adopt the protocol by including the protocol in any of CT4’s specifications.

As release 15 is closing, SA3 would like to confirm that CT4 is happy that the S6a interface is used and that CT4 do not want to adopt the EMSDP protocol.

Decision: 		The document was noted.

C4-176388	Reply LS on extension of S6x interface for BEST
					Source: CT4

Decision: 		The document was approved.

C4-176391	Reply LS on the number of bearers
					Source: SA2

Decision: 		The document was noted.

## 5	Work item management

C4-176022	Revised WID on CT aspects of 3GPP PS data off function – Phase 2
					Source: Ericsson

Abstract:

Revised to include CT3 impacts.

Discussion:

Stage 2 requirements have been simplified.

Nokia commented that it might be the case that there are no more CT4 impacts.

Decision: 		The document was revised to C4-176232.

C4-176232	Revised WID on CT aspects of 3GPP PS data off function – Phase 2
					Source: Ericsson, Huawei

(Replaces C4-176022)

Discussion:

SA2 agreed some CRs. There are no CT4 impacts anymore.

Decision: 		The document was endorsed.

C4-176037	Revised WID on CT aspects on 5G System – Phase 1
					Source: Nokia, Nokia Shanghai Bell

Discussion:

CT WGs and possible SA2 inputs during this week need to be captured.

Decision: 		The document was revised to C4-176229.

C4-176229	Revised WID on CT aspects on 5G System – Phase 1
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176037)

Decision: 		The document was revised to C4-176399.

C4-176399	Revised WID on CT aspects on 5G System – Phase 1
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176229)

Decision: 		The document was endorsed.

C4-176078	Revised WID on CT aspects of Northbound APIs for SCEF–SCS/AS Interworking
					Source: Huawei

Decision: 		The document was endorsed.

C4-176079	New WID on CT aspects on enhanced VoLTE performance
					Source: Huawei

Decision: 		The document was revised to C4-176221.

C4-176221	New WID on CT aspects on enhanced VoLTE performance
					Source: Huawei

(Replaces C4-176079)

Discussion:

Some CT3 impacted changes were needed.

Decision: 		The document was revised to C4-176236.

C4-176236	New WID on CT aspects on enhanced VoLTE performance
					Source: Huawei

(Replaces C4-176221)

Decision: 		The document was endorsed.

C4-176127	update WI 5GS_Ph1-CT for NUDR
					Source: china mobile

Abstract:

update WI 5GS_Ph1-CT for NUDR

Discussion:

Merged into C4-176229.

Decision: 		The document was merged.

C4-176144	Inclusion of WLAN direct discovery technologies as an alternative for ProSe direct discovery; Stage 3
					Source: Intel

Decision: 		The document was revised to C4-176217.

C4-176172	Study on User-plane Protocol
					Source: SoftBank Corp.

(Replaces C4-175222)

Discussion:

Alternative solutions for User Plane has been taken account in this version.

Nokia still maintain their opinion that it's still too early to start this work since open issues. Also this may impact negatively to CT4 Rel-15 work. This is important work which should be targeted to reasonable dates (starting in July 2018 at earliest). This work should be considered in the right timeframe. Also the proposed scope is too wide and goes beyond CT4 remits. Scope should be revisited to make it in remits of CT4. E.g. CT4 is responsible of N9 not N3 which is under RAN3. Coordination with other working groups are needed.

Ericsson agree with Nokia comments. Ericsson commented that this is in early state in IEFT and whether there are any clear advantages are still unclear. There are lot of things which can be questioned and many open issues. Ericsson also believe June 2018 is reasonable time frame to start this proposed study.

CT4 discussed reasonable dates for this study and Nokia commented that in July 2018 CT4 could start to work on this topic. Most probably TR could be sent to CT Plenary for information in December 2018. Probably SA2 study will be finished in March 2019.

Proximus requested why study cannot start now since most probably it will not take too much meeting time in next few months. Nokia commented that we do not have Rel-16 requirements available and there is no reason to overload CT4 Rel-15 workload. Nokia commented that they do not have any problems related to proposed work but they have concerns for proposed timeframe.

CT Chairman commented that IETF have started to work in DMM. CT4 could provide input to IETF to ensure that our requirements would be covered by the solution. Also if there are issues which need early treatment CT4 should start related work already if possible - keeping in mind that the target date to finalise the study is in March 2019.

CT4 chairman commented that coordination is needed at least between IETF, RAN3, SA5 and SA2. Key question is when and how to coordinate this work.

CT4 chairman proposed to have offline discussion to decide when this work can be start (either in March 2018 or June 2018).

Decision: 		The document was revised to C4-176235.

C4-176235	Study on User-plane Protocol
					Source: SoftBank Corp.

(Replaces C4-176172)

Decision: 		The document was revised to C4-176366.

C4-176366	Study on User-plane Protocol
					Source: SoftBank Corp.

(Replaces C4-176235)

Discussion:

It was proposed that in first phase this SID should be focused on ongoing IETF work. This could be triggered by LS to IETF.

Decision: 		The document was revised to C4-176383.

C4-176383	Study on User-plane Protocol
					Source: SoftBank Corp.

(Replaces C4-176366)

Discussion:

CT4 agreed the new SID.

It was agreed that in next 6 months CT4 will give this Study on User-plane Protocol only very limited time since Rel-15 5G work is prioritised in CT4 and need to be finished in June 2018.

Decision: 		The document was revised to C4-176400.

C4-176400	Study on User-plane Protocol
					Source: SoftBank Corp.

(Replaces C4-176383)

Decision: 		The document was agreed.

C4-176217	Inclusion of WLAN direct discovery technologies as an alternative for ProSe direct discovery; Stage 3
					Source: Intel

(Replaces C4-176144)

Decision: 		The document was revised to C4-176352.

C4-176352	Inclusion of WLAN direct discovery technologies as an alternative for ProSe direct discovery; Stage 3
					Source: Intel

(Replaces C4-176217)

Decision: 		The document was endorsed.

C4-176195	Revised WID on EPC enhancements to support 5G New Radio via Dual Connectivity, CT aspects
					Source: Ericsson

Decision: 		The document was revised to C4-176233.

C4-176233	Revised WID on EPC enhancements to support 5G New Radio via Dual Connectivity, CT aspects
					Source: Ericsson

(Replaces C4-176195)

Discussion:

T-Mobile shall be added as a supporting company.

Decision: 		The document was revised to C4-176286.

C4-176286	Revised WID on EPC enhancements to support 5G New Radio via Dual Connectivity, CT aspects
					Source: Ericsson

(Replaces C4-176233)

Decision: 		The document was agreed.

## 6	Release 15

### 6.1	CT4 Led WIs

#### 6.1.1	EPC enhancements to support 5G New Radio via Dual Connectivity, CT aspects [EDCE5-CT]

C4-176163	Reply LS on algorithm selection in E-UTRA-NR Dual Connectivity
					Source: SA2

Abstract:

3GPP SA WG2 kindly asks CT1 and SA3 to take provided information into account.

Decision: 		The document was noted.

C4-176288	Reply LS on algorithm selection in E-UTRA-NR Dual Connectivity
					Source: RAN2

Abstract:

3GPP RAN WG2 kindly asks CT1 and SA3 to take note of the RAN2 agreements in their specification work and to inform RAN2 of any concerns.

Discussion:

A new IE should be introduced that indicates the 5G algorithms. This impacts CT4.

Decision: 		The document was noted.

C4-176302	LS on EDCE5 Algorithm Indication between UE and SgNB
					Source: SA3

Abstract:

SA3 has considered the algorithm selection for encryption and integrity protection of the communication between the UE and the SgNB for EDCE5, in detail.  In particular, the following were considered:

•	Support for existing MME interworking interfaces.

•	Bidding down protection for both current and future algorithms

•	The indication of new 5G algorithms in the future

•	The protection of the algorithm indication on registration and handovers.

SA3 have concluded that:

•	A new IE should be introduced that indicates the 5G algorithms. This also allows the LTE and 5G capabilities to be indicated separately. A legacy MME that does not understand 5G algorithms must drop the new IE, and not forward it.

•	The eNB maps the 5G code points for NEA0, NEA1, NEA2, NEA3, NIA1, NIA2 and NIA3 to EEA0, EEA1, EEA2, EEA3, EIA1, EIA2 and EIA3 when the UE NR security capabilities not present. Therefore, MME upgrade is not required in R15 for security reasons.

•	The current R14 MME hash function should protect the new IE.

Discussion:

This affect also CT4 TS 29.274. Qualcomm has drafted a CR in C4-176279.

Qualcomm clarified that CT1 is discussing to add this also to 24.008 which would mean that CT4 TS 29.060 needs to be updated. This decision is still open.

Decision: 		The document was noted.

C4-176279	Addition of UE additional security capabilities
					29.274	  CR-1877  Cat: B (Rel-15) v15.1.0
					Source: Qualcomm

Abstract:

SA3 has agreed in CR 0627 to TS 33.401 that:

“The UE NR security capabilities shall be indicated to the network independently of the LTE security capabilities using new IE so that the support of LTE and NR algorithms can evolve independently. The UE shall send the UE NR security capabilities to the MME in initial Attach and TAU Request (when possibly changing MME). To enable the usage of dual connectivity with an MME that does not understand the UE NR security capabilities, such MME will drop the UE security capabilities and never save in the UE context. An MeNB that does not receive the UE NR security capabilities shall use the LTE security capabilities algorithms as the supported UE NR security capabilities”.

This requires extending the GTP-C signalling in TS 29.274.

Discussion:

Clarification is needed on the use of the IE by the target node.

Needs to be checked if a GTPv1 CR is required.

Decision: 		The document was revised to C4-176328.

C4-176328	Addition of UE additional security capabilities
					29.274	  CR-1877  rev 1 Cat: B (Rel-15) v15.1.0
					Source: Qualcomm

(Replaces C4-176279)

Decision: 		The document was agreed.

C4-176008	Updates on the Usage Data Reporting for Dual Connectivity with NR
					29.274	  CR-1848  Cat: B (Rel-15) v15.1.0
					Source: Ericsson, Verizon

Abstract:

Updates according to the SA2 CR3325 on TS 23.401.

Adding IE “Secondary RAT Usage Data Report” to Release Access Bearers Request procedure.

Discussion:

Needs to add a dependency on the CT4 CR agreed in Kochi.

Decision: 		The document was revised to C4-176329.

C4-176329	Updates on the Usage Data Reporting for Dual Connectivity with NR
					29.274	  CR-1848  rev 1 Cat: B (Rel-15) v15.1.0
					Source: Ericsson, Verizon

(Replaces C4-176008)

Decision: 		The document was agreed.

C4-176013	Secondary RAT related data usage reporting
					29.274	  CR-1845  rev 2 Cat: B (Rel-15) v15.1.0
					Source: Ericsson, Verizon

(Replaces C4-175261)

Abstract:

CR was agreed in CT4#80. This CR enhance CR which was agreed in Kochi meeting.

Rev2:

Add a clarification that when the MME reports to the PGW the received Secondary RAT usage data from eNodeB in a Connection Suspend, or S1 release procedure, by using Change Notification Request message, and in this case the MME shall also set the IRSGW flag to "0" and the IRPGW flag to "1".

Discussion:

Stage 2 changes on Inter-RAT mobility procedure are not covered completely.

Decision: 		The document was revised to C4-176330.

C4-176330	Secondary RAT related data usage reporting
					29.274	  CR-1845  rev 3 Cat: B (Rel-15) v15.1.0
					Source: Ericsson, Verizon

(Replaces C4-176013)

Decision: 		The document was revised to C4-176393.

C4-176393	Secondary RAT related data usage reporting
					29.274	  CR-1845  rev 4 Cat: B (Rel-15) v15.1.0
					Source: Ericsson, Verizon

(Replaces C4-176330)

Decision: 		The document was agreed.

C4-176124	New network capability
					29.303	  CR-0105  Cat: B (Rel-15) v14.3.0
					Source: Huawei, NTT DoCoMo

Discussion:

Merged into C4-176134.

Decision: 		The document was merged.

C4-176134	SGW/PGW selection for NR
					29.303	  CR-0104  rev 3 Cat: B (Rel-15) v14.3.0
					Source: Huawei, NTT DOCOMO, Nokia, Nokia Shanghai Bell

(Replaces C4-175351)

Abstract:

CR3273 to TS 23.401 and CR0047 to TS 23.214 have 0been agreed in SA2 to select the S/PGW or SGW-U/PGW-U that support enhanced function of NSA based on the UE’s capability on support NR, e.g. higher bitrates SGW/PGW.

DNS procedure shall also be updated to support this function.

Rev3: the procedure agreed during CT4#80 to select an SGW or PGW supporting NR is generalized to enable possible additions in future to support the selection of an SGW or PGW supporting other network capabilities, e.g. PGW supporting collocated SMF, with minimal specification and implementation impacts.

1.	New clause to define the SGW and PGW selection to select specific SGWs and PGWs supporting a particular network capability, and more specifically for UE supporting dual connectivity with NR and permitted to use NR, e.g. due to requirements of higher bitrates, data usage reporting over the secondary RAT, etc.

2.	Update the SGW-U and PGW-U selection, to include the new service parameter to select specific SGWs and PGWs supporting a particular network capability.

Decision: 		The document was revised to C4-176331.

C4-176331	SGW/PGW selection for NR
					29.303	  CR-0104  rev 4 Cat: B (Rel-15) v14.3.0
					Source: Huawei, NTT DOCOMO, Nokia, Nokia Shanghai Bell

(Replaces C4-176134)

Decision: 		The document was agreed.

C4-176218	Access Restrictions to NR as Secondary RAT on MM Context
					29.274	  CR-1876  Cat: B (Rel-15) v15.1.0
					Source: NTT DOCOMO

Abstract:

Add the use of Access Restriction Data for NR as Secondary RAT on S3.

Discussion:

Only used for SGW selection. May not be useful for the SGW.

Additional impacts on Gn interface (29.060).

Access restriction needs also to be conveyed over S6d (29.272).

Decision: 		The document was revised to C4-176332.

C4-176332	Access Restrictions to NR as Secondary RAT on MM Context
					29.274	  CR-1876  rev 1 Cat: B (Rel-15) v15.1.0
					Source: NTT DOCOMO

(Replaces C4-176218)

Decision: 		The document was revised to C4-176385.

C4-176333	Access Restrictions to NR as Secondary RAT on MM Context
					29.060	  CR-1057  Cat: B (Rel-15) v15.0.0
					Source: NTT DOCOMO

Decision: 		The document was revised to C4-176386.

C4-176334	Access Restrictions to NR as Secondary RAT on MM Context
					29.272	  CR-0747  Cat: B (Rel-15) v15.1.0
					Source: NTT DOCOMO

Decision: 		The document was revised to C4-176387.

C4-176335	Access Restrictions to NR as Secondary RAT on MM Context
					23.008	  CR-0543  Cat: B (Rel-15) v15.1.0
					Source: NTT DOCOMO

Decision: 		The document was withdrawn.

C4-176385	Access Restrictions to NR as Secondary RAT on MM Context
					29.274	  CR-1876  rev 2 Cat: B (Rel-15) v15.1.0
					Source: NTT DOCOMO

(Replaces C4-176332)

Decision: 		The document was agreed.

C4-176386	Access Restrictions to NR as Secondary RAT on MM Context
					29.060	  CR-1057  rev 1 Cat: B (Rel-15) v15.0.0
					Source: NTT DOCOMO

(Replaces C4-176333)

Decision: 		The document was revised to C4-176408.

C4-176387	Access Restrictions to NR as Secondary RAT on MM Context
					29.272	  CR-0747  rev 1 Cat: B (Rel-15) v15.1.0
					Source: NTT DOCOMO

(Replaces C4-176334)

Decision: 		The document was agreed.

C4-176408	Access Restrictions to NR as Secondary RAT on MM Context
					29.060	  CR-1057  rev 2 Cat: B (Rel-15) v15.0.0
					Source: NTT DOCOMO

(Replaces C4-176386)

Decision: 		The document was agreed.

C4-176355	Addition of UE additional security capabilities
					29.060	  CR-1058  Cat: B (Rel-15) v..
					Source: Qualcomm

Decision: 		The document was withdrawn.

#### 6.1.2	CT aspects of unlicensed spectrum offloading system enhancements [USOS-CT]

C4-176190	Access restriction to unlicensed spectrum as secondary RAT
					29.272	  CR-0746  Cat: B (Rel-15) v15.1.0
					Source: Qualcomm Incorporated / Lena

Abstract:

- Addition of a new bit in the Access-Restriction-Data bitmask, to control whether the subscriber is allowed to access unlicensed spectrum in the form of either LAA or LWA/LWIP via dual connectivity.

- Addition of a supported feature, so the MME and HSS can indicate whether they supports the feature to get access to EPC via unlicensed spectrum in the form of either LAA or LWA/LWIP as a secondary RAT.

Decision: 		The document was revised to C4-176336.

C4-176336	Access restriction to unlicensed spectrum as secondary RAT
					29.272	  CR-0746  rev 1 Cat: B (Rel-15) v15.1.0
					Source: Qualcomm Incorporated / Lena

(Replaces C4-176190)

Decision: 		The document was agreed.

C4-176191	Access restriction to unlicensed spectrum as secondary RAT
					23.008	  CR-0541  Cat: B (Rel-15) v15.1.0
					Source: Qualcomm Incorporated / Lena

Abstract:

Addition of unlicensed spectrum in the form of either LAA or LWA/LWIP in the Access Restriction Data.

Decision: 		The document was revised to C4-176337.

C4-176337	Access restriction to unlicensed spectrum as secondary RAT
					23.008	  CR-0541  rev 1 Cat: B (Rel-15) v15.1.0
					Source: Qualcomm Incorporated / Lena

(Replaces C4-176191)

Decision: 		The document was agreed.

C4-176192	Access restriction to unlicensed spectrum as secondary RAT
					29.274	  CR-1871  Cat: B (Rel-15) v15.1.0
					Source: Qualcomm Incorporated / Lena

Abstract:

Allocate a bit in the extended ARD information octet of the MM Context to be set when LAA/LWA/LWIP as Secondary RAT is not allowed.

Decision: 		The document was agreed.

C4-176193	Access restriction to unlicensed spectrum as secondary RAT
					29.002	  CR-1244  Cat: B (Rel-15) v15.1.0
					Source: Qualcomm Incorporated / Lena

Abstract:

- Addition of a new feature bit in the SupportedFeatures IE for LAA/LWA/LWIP used as Secondary RAT

- Addition of a new bit in new IE ext-AccessRestrictionData for use of unlicensed spectrum in the form of LAA or LWA/LWIP as secondary RAT not allowed

Discussion:

This CR overlaps with a CR which was agreed in CT4#80. It was agreed to postpone this CR to make implementation easier.

Vodafone commented that supported extended feature need to be supported also in the text.

There wasn't technical comments against this CR.

Decision: 		The document was revised to C4-176338.

C4-176338	Access restriction to unlicensed spectrum as secondary RAT
					29.002	  CR-1244  rev 1 Cat: B (Rel-15) v15.1.0
					Source: Qualcomm Incorporated / Lena

(Replaces C4-176193)

Decision: 		The document was postponed.

C4-176194	Data usage reporting with unlicensed spectrum used as secondary RAT
					29.274	  CR-1872  Cat: B (Rel-15) v15.1.0
					Source: Qualcomm Incorporated / Lena

Abstract:

Adding a value for Secondary RAT Type “LAA/LWA/LWIP” in new IE “Secondary RAT Usage Data Report" created by CR 1845 (agreed in C4-175261 at CT4#80) in new subclause 8.x.

Discussion:

Same handling like with C4-176193.

Decision: 		The document was revised to C4-176339.

C4-176339	Data usage reporting with unlicensed spectrum used as secondary RAT
					29.274	  CR-1872  rev 1 Cat: B (Rel-15) v15.1.0
					Source: Qualcomm Incorporated / Lena

(Replaces C4-176194)

Decision: 		The document was withdrawn.

### 6.2	CT4 Supported WIs

#### 6.2.1	CT aspects on 5G System - Phase 1 [5GS_Ph1-CT]

C4-176152	Reply to Reply LSs on PLMN and RAT selection policies for roaming (S2-175286 and C1-173751)
					Source: SA3

(Replaces C4-175208)

Abstract:

3GPP TSG SA3 kindly asks CT4 to comment on the potential impact on the AUSF.

Discussion:

CT1 reply in C4-176154.

Decision: 		The document was noted.

C4-176154	Reply LS on PLMN and RAT selection policies for roaming
					Source: CT1

Abstract:

CT1 kindly asks CT3 and CT4 to provide inputs on information requested by SA3.

Discussion:

This will be discussed in CT1. In CT1 there are at least 2 different proposals.

Ericsson believe UDM solution is preferred way forward. Ericsson doesn't recognise any drawbacks on that solution.

Reply LS to CT1 in C4-176238

Decision: 		The document was noted.

C4-176238	Reply LS on PLMN and RAT selection policies for roaming
					Source: CT4

Decision: 		The document was revised to C4-176362.

C4-176362	Reply LS on PLMN and RAT selection policies for roaming
					Source: CT4

(Replaces C4-176238)

Decision: 		The document was approved.

C4-176157	LS on transparent relay between SMF in the VPLMN and SMF in the HPLMN
					Source: CT1

Abstract:

With respect to the stage 2 requirement on session management in case of home routed roaming, CT1 asks the several questions from SA2.

Decision: 		The document was noted.

C4-176161	LS on defining GTP extension header for 5GS Container
					Source: RAN3

Abstract:

RAN3 has decided to specify a 5GS container which includes the Radio Network Layer user plane information elements which need to be exchanged over NG-U e.g. QFI (QoS Flow ID).

Discussion:

There is ongoing discussion.

Not sure if 0, 1 or more extensions are required. Wait for RAN3 feedback.

Decision: 		The document was postponed.

C4-176162	Reply LS on supporting non-3GPP access in NGAP
					Source: SA2

Abstract:

SA2 kindly asks RAN3 to take the answers into account for their work on supporting non-3GPP access in NGAP.

Decision: 		The document was noted.

C4-176230	LS on interworking between networks with different 5G deployments options in a roaming scenario
					Source: GSMA NG SIGNAL

Abstract:

GSMA NG SIGNAL is discussing the impact of the different 5G deployment options and of a new signalling protocol for 5G in the context of roaming.

GSMA NG SIGNAL would like to ask clarification on how the roaming interworking is envisaged between 5G networks with NGC and 5G networks with EPC.

Discussion:

CT4 believes this should be first discussed in SA2, to decide what to do regarding the possible roaming user case scenarios.

Decision: 		The document was noted.

C4-176166	Reply LS on applicability of service based interface for legacy core network elements
					Source: SA2

Abstract:

SA2 kindly asks CT1 to take the answers into account for their work on PWS.

Decision: 		The document was noted.

C4-176023	Discussion paper on the HATEOAS REST tenet and its benefits
					29.501 v..
					Source: Orange

Abstract:

1. Introduction

The HATEOAS (Hypermedia As The Engine of Application State) tenet is a pillar of REST architecture. Deciding not to embrace it would jeopardize most of the REST benefits (such as cloud-friendliness, ease to deploy and openness) that were advocated in the conclusions of TR 29.871, leading to the adoption of HTTP/2 and the REST model as the protocol support for the 5G Service-Based Interfaces.

This paper aims at:

•	providing lecture references to CT3 and CT4 delegates

•	clarifying the HATEOAS REST tenet

•	providing examples of API and use cases for which HATEOAS should be applied

•	proposing a minimum level of HATEOAS adoption by all 3GPP defined API

•	a proposed set of actions

The proposed discussion provides a summary of the HATEOAS tenet and its benefits. The following references have been used to build this overview:

•	"REST in practice" by Jim Webber, Savas Parastatidis, and Ian Robinson – O’Reilly

•	Roy Fielding's dissertation on REST

•	"Restful Web Services" by Leonard Richardson, Sam Ruby, David Heinemeier Hansson --O'Reilly (2007)

•	"RESTful Web Services Cookbook_ Solutions for Improving Scalability and Simplicity" by Subbu Allamaraju -Yahoo Press (2010)

•	"REST API Design Rulebook" by Mark Masse -O'Reilly Media (2011)

Discussion:

C3 document number is C3-176017.

Decision: 		The document was noted.

C4-176289	Use of Subscriber Identity in HTTP URI
					Source: CT4

Decision: 		The document was revised to C4-176395.

C4-176395	Use of Subscriber Identity in HTTP URI 
					Source: CT4

(Replaces C4-176289)

Decision: 		The document was approved.

C4-176373	LS on C3-176034 on PLMN and RAT selection policies for roaming
					Source: CT3

Decision: 		The document was noted.

##### 6.2.1.1	Contributions to TR 29.891

C4-176007	Protocol Selection for Nudr
					29.891 v1.1.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was withdrawn.

C4-176020	Conclusion and solution selection UDR interface protocol stack for structured data storage
					29.891 v1.1.0
					Source: Orange

(Replaces C4-175008)

Abstract:

3GPP CT4 is assessing candidate protocols to be used over the interface between UDR and UDM, PCF and NEF. This PCR proposes to select the SBI protocol stack.

Discussion:

Merged into C4-176396

Decision: 		The document was merged.

C4-176128	Using SBI between FEs and UDR
					29.891 v1.1.0
					Source: China Mobile

Abstract:

As depicted in section 4.2.5 of TS23.501, the 5G system architecture allows the UDM FE, PCF FE and NEF FE to store data in the UDR, including subscription data and policy data by UDM and PCF, structured data for exposure and application data by the NEF.

Decision: 		The document was revised to C4-176244.

C4-176244	Using SBI between FEs and UDR
					29.891 v1.1.0
					Source: China Mobile, Orange, Deutsche Telekom, Huawei

(Replaces C4-176128)

Decision: 		The document was revised to C4-176381.

C4-176381	Using SBI between FEs and UDR
					29.891 v1.1.0
					Source: China Mobile, Orange, Deutsche Telekom, Huawei

(Replaces C4-176244)

Decision: 		The document was revised to C4-176396.

C4-176396	Using SBI between FEs and UDR
					29.891 v1.1.0
					Source: China Mobile, Orange, Deutsche Telekom, Huawei

(Replaces C4-176381)

Decision: 		The document was agreed.

C4-176019	Addition of new service based interfaces based on latest version of TS 23.501
					29.891 v1.1.0
					Source: Orange

Abstract:

Conclude on protocol selection for Nudr.

Discussion:

Merged into C4-176244.

Decision: 		The document was merged.

C4-176038	Work planning on 5G System - Phase 1 - CT4 Aspects
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

C4-176089	Skeleton for Nudr, Unified Data Repository for subscription data
					Source: Huawei

Discussion:

Merged into C4-176297.

Decision: 		The document was postponed.

C4-176039	TR clean up
					29.891 v1.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

TR 29.891 is expected to be sent to CT#78 for Approval. This pCR makes a general clean-up of the TR, e.g. removing guidance information, a few empty clauses and a few editor's notes which have either been solved now or which served as guidance information.

It also documents the conclusion earlier agreed on N4 protocol selection in the conclusion section.

Decision: 		The document was revised to C4-176239.

C4-176239	TR clean up
					29.891 v1.1.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176039)

Decision: 		The document was agreed.

C4-176240	NRF service subscription and notification
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was agreed.

C4-176040	Content-type of Multipart message
					29.891 v1.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

It has been agreed to support multipart messages over 5GC SBIs that require to transport large opaque binary payloads. However, it has not yet been concluded which content-type to use in the multipart messages.

Given the conclusion agreed at the last CT4 meeting to cross reference the binary body part from the JSON body part, it is proposed to use the multipart/related content-type.

Decision: 		The document was agreed.

C4-176041	New GTP-U extension headers
					29.891 v1.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Subclause 11.2 contains the following editor's note:

Editor's Note: It is FFS whether CT4 needs to specify new GTP-U extension header(s) in 3GPP TS 29.281 for the 5GS Container.

The resolution of this editor's note is pending on a decision from RAN3 that is expected to be taken during RAN3#98.

As TR 29.891 is to be sent to CT#78 for Approval, this pCR aims at capturing the conclusion from RAN3, assuming it will be communicated to CT4 during CT4#81. The pCR will be revised during CT4#81 with the RAN3 outcome.

Decision: 		The document was revised to C4-176241.

C4-176241	New GTP-U extension headers
					29.891 v1.1.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176041)

Decision: 		The document was withdrawn.

C4-176042	HTTP extensibility
					29.891 v1.1.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was withdrawn.

C4-176043	AMF discovery by 5G-AN
					29.891 v1.1.0
					Source: Nokia, Nokia Shanghai Bell, Verizon

Abstract:

The 5G-AN (NG-RAN and N3IWF) is responsible for initiating SCTP connectivity with the AMF. This requires the 5G-AN to first obtain the AMF's IP address.

In E-UTRAN, the association of MMEs to an eNB is quite static. Therefore, the IP addresses of MMEs are typically pre-configured to the eNB via OAM.

In 5GS however, the association of AMFs to 5G-AN nodes can be dynamic, e.g. in virtualized deployments where AMF instances may be added or removed for scalability or planned maintenance. In order to support dynamic associations between AMF and NG-RAN nodes, SA2 agreed the following requirement in subclause 5.21.2.1 (AMF Addition/Update) of 3GPP TS 23.501 [3]:

Information about new AMF should be published and available in the DNS system. It should allow 5G-AN to discover AMF and setup associations with the AMF required.

It allows the option for the 5G-AN to obtain the AMF IP endpoint(s) via DNS, as a possible alternative to OAM.

It is proposed to standardize in CT4 (TS 29.303 "Domain Name System Procedures; Stage 3") the stage 3 DNS procedure for AMF discovery by 5G-AN to ease 5G-AN implementations and interoperability (RAN and CN operators may be different when using network sharing), and ensure a clear and uniform AMF discovery procedure for all 5G-ANs (eNBs, gNBs, N3IWF connected to 5GC).

5G-ANs may discover available AMFs e.g. based on (pre-configured) AMF Regions or AMF Set IDs or based on the TAIs the 5G-ANs are serving. The exact procedure (and criteria for AMF discovery) will be determined during the stage 3 normative work.

NOTE:	The 5G-AN learns during the subsequent setup of the NGAP association the GUAMIs and S-NSSAIs an AMF is supporting (see subclause 16.3.4.2 of 3GPP TS 38.300).

Decision: 		The document was agreed.

C4-176077	N4 requirement to deactivate UP connection when UPF detects that the PDU Session has no data activity
					29.891 v1.1.0
					Source: NEC

Abstract:

Changes from C4-175078: The EPC has following requirement in TS 23.401. The N4 can reuse CUPS functionality for it. Therefore, this pCR concludes that the required functionality is supported by CUPS and there are no required modifications on CUPS.

TS 23.401 v15.1.0:

5.4.4.1 PDN GW initiated bearer deactivation

For an emergency PDN connection the PDN GW initiates the deactivation of all bearers of that emergency PDN connection when the PDN connection is inactive (i.e. not transferring any packets) for a configured period of time or when triggered by dynamic PCC.

Changes from C4-175225: Changing the word: "a configured period" -> "a specified Inactivity period provided by the SMF". This change is to align with the latest TS 23.502 v1.3.0.

NOTE:	This pCR depends on the CR 29.224 #0026 which introduces the required functionality for CUPS.

Discussion:

This pCR depends on the CR 29.244 #0026 (C4-176067) which introduces the required functionality for CUPS.

Decision: 		The document was agreed.

C4-176100	SMF service operations correction
					29.891 v1.1.0
					Source: Huawei

Abstract:

S2-177635 and S2-177637 agreed in SA2 #123 meeting identify the service operation for the following steps in Inter NG-RAN node N2 based handover,

Decision: 		The document was agreed.

C4-176101	AMF service operations correction
					29.891 v1.1.0
					Source: Huawei

Decision: 		The document was agreed.

C4-176102	Conclusion on Address info mapping
					29.891 v1.1.0
					Source: Huawei

Abstract:

For PGW/SMF address information mapping, it is proposed to use the enhanced solutions in subclause 8.2.3.3 for the mapping mechanism.

Decision: 		The document was revised to C4-176242.

C4-176242	Conclusion on Address info mapping
					29.891 v1.1.0
					Source: Huawei

(Replaces C4-176102)

Decision: 		The document was revised to C4-176369.

C4-176369	Conclusion on Address info mapping
					29.891 v1.1.0
					Source: Huawei

(Replaces C4-176242)

Decision: 		The document was agreed.

C4-176103	Stateless AMF without UDSF
					29.891 v1.1.0
					Source: Huawei

Decision: 		The document was revised to C4-176356.

C4-176356	Stateless AMF without UDSF
					29.891 v1.1.0
					Source: Huawei

(Replaces C4-176103)

Decision: 		The document was revised to C4-176394.

C4-176394	Stateless AMF without UDSF
					29.891 v1.1.0
					Source: Huawei

(Replaces C4-176356)

Decision: 		The document was revised to C4-176398.

C4-176398	Stateless AMF without UDSF
					29.891 v1.1.0
					Source: Huawei

(Replaces C4-176394)

Decision: 		The document was agreed.

C4-176125	Procedure between the AMF and PCF
					29.891 v1.1.0
					Source: Huawei

Abstract:

As described in subclause 5.2.2.3 of TS 23.502, the PCF can be a consumer of the Namf_EventExposure service. The procedure between the AMF and the PCF is needed from CT4 point of view.

Decision: 		The document was agreed.

C4-176126	User plane management procedures correction
					29.891 v1.1.0
					Source: Huawei

Decision: 		The document was agreed.

C4-176168	Stateless Network Functions
					29.891 v1.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR completes clause 10.4.2.3 on Stateless UDMs and concludes on solutions for Stateless AMFs.

Decision: 		The document was revised to C4-176245.

C4-176245	Stateless Network Functions
					29.891 v1.1.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176168)

Decision: 		The document was agreed.

C4-176196	HTTP Patch
					29.891 v1.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Different possibilities for the encoding of PATCH bodies exist, and it is desirable to select a suitable one.

Decision: 		The document was postponed.

C4-176145	draft skeleton for TS 29.XXX Unified Data Repository Services
					29.891 v1.1.0
					Source: china mobile

Decision: 		The document was revised to C4-176397.

C4-176397	draft skeleton for TS 29.XXX Unified Data Repository Services
					29.891 v1.1.0
					Source: china mobile

(Replaces C4-176145)

Discussion:

Wil be discussed and agreed offline before CT4#82.

Decision: 		The document was postponed.

C4-176146	scope and reference for TS 29.xxx Nudr
					29.891 v1.1.0
					Source: china mobile

Decision: 		The document was postponed.

C4-176147	Nudr UnifiedDataManagement Service Description
					29.891 v1.1.0
					Source: china mobile

Decision: 		The document was postponed.

C4-176148	Protocol selection for N18
					29.891 v1.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

To complete the protocol selection for N18.

Solution 1 (reuse of Ud) is the existing protocol solution for database access since Rel-9 and shall not be discontinued in 5G.

Decision: 		The document was revised to C4-176368.

C4-176368	Protocol selection for N18
					29.891 v1.1.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176148)

Decision: 		The document was agreed.

C4-176200	Supported Feature Negotiation
					29.891 v1.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Clarify pro/cons of the extensibility mechanism based on feature related information embedded in HTTP bodies.

Corresponding normative changes are proposed in C4-176201, C4-176202, C4-176211.

A similar CR to TR 29.890 is in C3-176037.

Discussion:

The top-level resource is misleading. It should be associated with the node for a given session.

A specific case of null resource is FFS.

A server supported feature can be discovered by NRF.

Decision: 		The document was revised to C4-176261.

C4-176261	Supported Feature Negotiation
					29.891 v1.1.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176200)

Decision: 		The document was agreed.

C4-176243	HTTP PATCH Encoding
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

C4-176432	3GPP TS 29.891 v1.2.0
					29.891 v1.2.0
					Source: Nokia

Discussion:

TR Will be sent for approval in CT#78.

Decision: 		The document was agreed.

##### 6.2.1.2	Impacted Specifications

C4-176165	LS answer on Standardised Slice Service Types
					Source: SA2

Abstract:

SA2 kindly asks CT4 to clarify the range for each of the standardised and the non-standardised SSTs.

Discussion:

See related CR in C4-176044.

Decision: 		The document was noted.

C4-176299	Reply LS answer on Standardised Slice Service Types
					Source: CT4

Decision: 		The document was approved.

C4-176044	Introducing the S-NSSAI definition
					23.003	  CR-0485  Cat: B (Rel-15) v15.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

The identification of a Network Slice in the 5GS requires an end to end label known as S-NSSAI (Single Network Slice Selection Assistance Information). This is composed of two fields: the SST (Slice/Service Type) and the SD (Slice Differentiator): see TS 23.501 for their definition. SA2 has already assumed (see S2-174051 agreed at SA2#121) that the SST field is 8 bits long and the SD field is 24 bits long, so the S-NSSAI is 32 bits long. This CR implements this decision in TS 23.003.

Since the S-NSSAI is an end to end identifier used in the UE, RAN, CN, TS 23.003 is the right place to document the format of this information.

Decision: 		The document was revised to C4-176298.

C4-176298	Introducing the S-NSSAI definition
					23.003	  CR-0485  rev 1 Cat: B (Rel-15) v15.1.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176044)

Decision: 		The document was agreed.

C4-176045	Extending the scope of the specification to cover the N26 interface
					29.274	  CR-1852  Cat: B (Rel-15) v15.1.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was revised to C4-176224.

C4-176224	Extending the scope of the specification to cover the N26 interface
					29.274	  CR-1852  rev 1 Cat: B (Rel-15) v15.1.0
					Source: Nokia, Nokia Shanghai Bell, Huawei

(Replaces C4-176045)

Abstract:

CT4 has decided to specify the N26 interface in TS 29.274. The scope of the TS needs to be extended accordingly.

Decision: 		The document was agreed.

C4-176046	GTP-C tunnel per UE over the N26 interface
					29.274	  CR-1853  Cat: B (Rel-15) v15.1.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was revised to C4-176225.

C4-176225	GTP-C tunnel per UE over the N26 interface
					29.274	  CR-1853  rev 1 Cat: B (Rel-15) v15.1.0
					Source: Nokia, Nokia Shanghai Bell, Huawei

(Replaces C4-176046)

Abstract:

CT4 has decided to specify the N26 interface in TS 29.274. The principles of how to set the GTP-C tunnel over the N26 interface need to be specified.

There shall be one GTP-C tunnel per UE over the N26 interface.

Decision: 		The document was agreed.

C4-176047	GTP-C messages over the N26 interface
					29.274	  CR-1854  Cat: B (Rel-15) v15.1.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was revised to C4-176226.

C4-176226	GTP-C messages over the N26 interface
					29.274	  CR-1854  rev 1 Cat: B (Rel-15) v15.1.0
					Source: Nokia, Nokia Shanghai Bell, Huawei

(Replaces C4-176047)

Abstract:

The following messages are supported over the N26 interface:

- Context Request / Response / Acknowledge

- Forward Relocation Request / Response

- Forward Relocation Complete Notification / Acknowledge

- Relocation Cancel Request / Response

Discussion:

Ericsson request to clarify whether the Identification RequestResponse apply to N26.

Nokia replied that currently this is not so clear in stage 2 and this should be clarified. CT4 can come back on this if needed after stage 2 clarification.

Decision: 		The document was agreed.

C4-176048	Forward Relocation Request over the N26 interface
					29.274	  CR-1855  Cat: B (Rel-15) v15.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

CT4 has decided to specify the N26 interface in TS 29.274. Inter-system handovers from EPS to 5GS and from 5GS to EPS use the Forward Relocation Request message. Corresponding descriptions need to be added.

Forward Relocation Request message is sent to the target MME or AMF during an inter-system handover between 5GS and EPS.

The content of the message during inter-system handovers is specified.

Discussion:

In this CR, it is proposed to set the SGW address to 0. Aligned with current stage 2.

In case of combo smf/pgw, the AMF could set SGW FTEID and PGW FTEID. This should be first confirmed at the stage 2 level.

Decision: 		The document was revised to C4-176303.

C4-176303	Forward Relocation Request over the N26 interface
					29.274	  CR-1855  rev 1 Cat: B (Rel-15) v15.1.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176048)

Decision: 		The document was agreed.

C4-176049	Forward Relocation Response over the N26 interface
					29.274	  CR-1856  Cat: B (Rel-15) v15.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Forward Relocation Response message is sent to the target MME or AMF during an inter-system handover between 5GS and EPS.

The content of the message during inter-system handovers is specified.

Decision: 		The document was revised to C4-176304.

C4-176304	Forward Relocation Response over the N26 interface
					29.274	  CR-1856  rev 1 Cat: B (Rel-15) v15.1.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176049)

Decision: 		The document was agreed.

C4-176106	Supports N26 interface
					29.274	  CR-1859  Cat: B (Rel-15) v15.1.0
					Source: Huawei

Discussion:

Merged into C4-176224, C4-176225 and C4-176226.

Decision: 		The document was merged.

C4-176206	Support of Idle Mode Mobility procedures for single-registration mode
					29.274	  CR-1873  Cat: B (Rel-15) v15.1.0
					Source: Huawei

Abstract:

SA2 has agreed Idle Mode Mobility procedures for single-registration mode.

The procedure makes use of Context Request/response/acknowledge message.

Context Request/response/acknowledge message is enhanced  to support Idle Mode Mobility procedures for single-registration mode.

Discussion:

Nokia commented that the CR seems to cover only mobility from 5GS to EPS. Both directions should be covered. All the IE descriptions need to be covered. What applies to MME should apply to AMF in the description of procedures and IE.

It needs to be checked that the CR is based on the correct baseline.

Huawei needs to complete the list of clauses affected.

Decision: 		The document was revised to C4-176305.

C4-176305	Support of Idle Mode Mobility procedures for single-registration mode
					29.274	  CR-1873  rev 1 Cat: B (Rel-15) v15.1.0
					Source: Huawei

(Replaces C4-176206)

Decision: 		The document was revised to C4-176443.

C4-176107	Node name and F-TEID on N26 interface
					29.274	  CR-1860  Cat: B (Rel-15) v15.1.0
					Source: Huawei

Abstract:

GTPv2 will be used for the N26 interface between AMF and the MME, in order to avoid the impact on the legacy MME, it is proposed that the AMF performs the MME functions during interworking with legacy MME and the N26 interface is same as the S10 interface between MMEs.

1.To avoid the impact to the MME, the following statement is introduced into the general part of the tunnel management messages:

The Requirements specified in this clause for MME also apply to AMF if the AMF supports GTP-C (v2) based N26 interface for roaming and inter access mobility between MMEs and AMFs.

2.During the handover/TAU from the AMF to the MME, the SM related information is mapped by the SMF.

There will be V-SMF and H-SMF in roaming with home routed scenario, it is defined that the H-SMF shall assign the PGW info and the V-SMF shall assign the SGW info. As interface type shall be set in the F-TEID, it is defines that the S1-U for GTPU and S11 for GTPC shall be used by the SMF.

Discussion:

After discussion, it was seen that this CR is not needed but changes can be covered in Nokia CR.

Merged into C4-176304.

Decision: 		The document was merged.

C4-176443	Support of Idle Mode Mobility procedures for single-registration mode
					29.274	  CR-1873  rev 2 Cat: B (Rel-15) v15.1.0
					Source: Huawei

(Replaces C4-176305)

Decision: 		The document was agreed.

C4-176208	Forward Relocation Complete Notification / Acknowledge over the N26 interface
					29.274	  CR-1874  Cat: B (Rel-15) v15.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Forward Relocation Complete Notification / Acknowledge message is sent to the target MME or AMF during an inter-system handover between 5GS and EPS.

The content of the message during inter-system handovers is specified.

Decision: 		The document was agreed.

C4-176209	Relocation Cancel Request / Response over the N26 interface
					29.274	  CR-1875  Cat: B (Rel-15) v15.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Relocation Cancel Request / Response message is sent to the target MME or AMF during an inter-system handover between 5GS and EPS, to cancel an on-going handover. The content of the message during inter-system handovers is specified.

Decision: 		The document was agreed.

C4-176082	Session establishment on N4
					29.244	  CR-0049  Cat: B (Rel-15) v14.1.0
					Source: Huawei

Abstract:

The content of messages for session establishment need to be clarified.

It is concluded to use GTP-U on N3 and N9.

Stage 2 requires the support of N4 Session Establishment Request message, e.g. if the SMF selects a new UPF to act as intermediate UPF for the PDU Session, an N4 Session Establishment Request message is sent to the new UPF, providing Packet detection, Data forwarding, enforcement and reporting rules to be installed on the intermediate UPF. The PDU Session anchor addressing information (on N9) for this PDU Session is also provided to the intermediate UPF.

Stage 2 requires the support of N4 Session Establishment Response message, e.g. (extract from Stage 2) The new intermediate UPF sends an N4 Session Establishment Response message to the SMF. In case the UPF allocates CN tunnel info, it provides CN DL tunnel info for the UPF acting as PDU Session Anchor and UL tunnel info (i.e. CN N3 tunnel info) to the SMF. If the Data forwarding indication is received, the new (intermediate) UPF acting as N3 terminating point also sends CN DL tunnel info for the old (intermediate) UPF to the SMF.

It is assumed that Charging requirements are in principle the same in 5G as in 4G.

Final confirmation fromSA5 is outstanding.

Interception is required, final details are outstanding on additional or changed requirements compared to 4G.

Content of PFCP Session Establishment Request/Response are modified regarding support of parameters for N4.

Editor's note that URR configurations needs final confirmation from SA5.

Editor's note that Interception needs final confirmation from SA3.

Discussion:

Ethernet case should be covered.

"NR" should be replaced by "5G-AN".

Editor's note needs to be added on Dropped DL Traffic Threshold.

QER correlation ID may not be used over N4. This is FFS.

Editor's note is needed to clarify the use of Downlink Data Notification Delay over N4.

Decision: 		The document was revised to C4-176314.

C4-176314	Session establishment on N4
					29.244	  CR-0049  rev 1 Cat: B (Rel-15) v14.1.0
					Source: Huawei

(Replaces C4-176082)

Decision: 		The document was revised to C4-176444.

C4-176444	Session establishment on N4
					29.244	  CR-0049  rev 2 Cat: B (Rel-15) v14.1.0
					Source: Huawei

(Replaces C4-176314)

Decision: 		The document was agreed.

C4-176083	Session Modification on N4
					29.244	  CR-0050  Cat: B (Rel-15) v14.1.0
					Source: Huawei

Abstract:

Content of PFCP Session Modification Request/Response are modified regarding support of parameters for N4.

Section which need final confirmation for charging and interception are marked with editor's notes.

Discussion:

QER correlation ID may not be used over N4. This is FFS.

Decision: 		The document was revised to C4-176315.

C4-176315	Session Modification on N4
					29.244	  CR-0050  rev 1 Cat: B (Rel-15) v14.1.0
					Source: Huawei

(Replaces C4-176083)

Decision: 		The document was revised to C4-176445.

C4-176445	Session Modification on N4
					29.244	  CR-0050  rev 2 Cat: B (Rel-15) v14.1.0
					Source: Huawei

(Replaces C4-176315)

Decision: 		The document was agreed.

C4-176084	Session deletion on N4
					29.244	  CR-0051  Cat: B (Rel-15) v14.1.0
					Source: Huawei

Abstract:

Stage 2 requires:

N4 Session Release Request (N4 Session ID) message to the UPF. The UPF shall drop any remaining packets of the PDU Session and release all tunnel resource and contexts associated with the N4 Session.

The UPF acknowledges the N4 Session Release Request by the transmission of an N4 Session Release Response (N4 Session ID) message to the SMF.

If the UP function may detect an error in the request, this should be possible to report.

The UP function may be requested to  do some measurements if so these measurements need to be reported to the SMF.

No new requirements for PFCP Session Deletion Request are identified.

Charging requirements are currently outstanding, it is assumed that they will be  similar as in 4G for CUPS. This is captured  by an editor's note.

PFCP Session Deletion Response is modified to reflect parameters to be sent on N4.

Decision: 		The document was revised to C4-176316.

C4-176316	Session deletion on N4
					29.244	  CR-0051  rev 1 Cat: B (Rel-15) v14.1.0
					Source: Huawei

(Replaces C4-176084)

Decision: 		The document was agreed.

C4-176085	Session Reporting on N4
					29.244	  CR-0052  Cat: B (Rel-15) v14.1.0
					Source: Huawei

Abstract:

Stage 2 requires that UP function may be instructed to send reports:

-on reception of downlink data

-to report an error on the userplane

-to report  measurements to generate charging tickets

Charging requirements are currently outstanding, it is assumed that they will be  based on charging requirements of 4G for CUPS.

This is captured  by an editor's note.

The CP function may reply to a report from UP function with change of instruction on buffering or may instruct the UP function to drop the packets received.

PFCP Session Report Request/Response messages are modified to reflect parameters to be sent on N4.

Decision: 		The document was revised to C4-176317.

C4-176317	Session Reporting on N4
					29.244	  CR-0052  rev 1 Cat: B (Rel-15) v14.1.0
					Source: Huawei

(Replaces C4-176085)

Discussion:

Typos corrected after agreement.

Decision: 		The document was revised to C4-176420.

C4-176420	Session Reporting on N4
					29.244	  CR-0052  rev 2 Cat: B (Rel-15) v14.1.0
					Source: Huawei

(Replaces C4-176317)

Decision: 		The document was agreed.

C4-176086	Load-overload control on N4
					29.244	  CR-0053  Cat: B (Rel-15) v14.1.0
					Source: Huawei

Abstract:

The SMF has to support Load and overload control and should take this into account when selecting an UPF.

It would be useful to reuse the load and overload  handling used on Sx interfaces for CUPS on N4.

Load and overload parameters are added to be supported on N4.

Decision: 		The document was revised to C4-176318.

C4-176318	Load-overload control on N4
					29.244	  CR-0053  rev 1 Cat: B (Rel-15) v14.1.0
					Source: Huawei

(Replaces C4-176086)

Discussion:

Typos corrected after agreement.

Decision: 		The document was revised to C4-176421.

C4-176421	Load-overload control on N4
					29.244	  CR-0053  rev 2 Cat: B (Rel-15) v14.1.0
					Source: Huawei

(Replaces C4-176318)

Decision: 		The document was agreed.

C4-176087	QOS enhancements on N4 for 5G
					29.244	  CR-0054  Cat: B (Rel-15) v14.1.0
					Source: Huawei

Abstract:

In 5G the QoS flow is the finest granularity of QoS differentiation in the PDU session. A QoS Flow ID (QFI) is used to identify a QoS flow in the 5G system. User Plane traffic with the same QFI within a PDU session receives the same traffic forwarding treatment (e.g. scheduling, admission threshold). The QFI is carried in an encapsulation header on N3 (and N9) i.e. without any changes to the e2e packet header. It can be applied to PDUs with different types of payload, i.e. IP packets, unstructured PDUs and Ethernet frames. The QFI shall be unique within a PDU session.

In TS 23.501 it is specified that each QOS rule shall contain the QFI of the QoS Flow.

In 5G it is possible to request reflective QoS for the UL.

Both information have to be added in the packet header on UP. SMF has to provide this information to the UPF. It is proposed to add RQI to the PDR IE. RQI can be changed at any time duing the lifetime of the flow/PDR.

QFI is added to create and update QER.

RQI is added to create and update PDR.

For the support of RQI a feature bit is introduced.

Discussion:

RQI should not be related to packet detection Information.

The RQIU flag is not required as it is not an option for the UPF to support it.

C4-176082: In table 7525-1: a lot of QOS related IEs description is specific to EPC. An Editor's note needs to be added in somewhere that update is needed.

Decision: 		The document was revised to C4-176322.

C4-176322	QOS enhancements on N4 for 5G
					29.244	  CR-0054  rev 1 Cat: B (Rel-15) v14.1.0
					Source: Huawei

(Replaces C4-176087)

Decision: 		The document was revised to C4-176446.

C4-176446	QOS enhancements on N4 for 5G
					29.244	  CR-0054  rev 2 Cat: B (Rel-15) v14.1.0
					Source: Huawei

(Replaces C4-176322)

Decision: 		The document was agreed.

C4-176088	Support of Ethernet frames on N4
					29.244	  CR-0055  Cat: B (Rel-15) v14.1.0
					Source: Huawei

Abstract:

Traffic detection type Ethernet is requested to be supported in 5GC.

An Ethernet Packet Filter Set as defined in clause 5.7.6.3.

For Ethernet PDU session type, the Packet Filter Set shall support packet filtering based on at least any combination of:

-	Source/destination MAC address

-	Ethertype as defined in IEEE 802.3 [yy]

-	Customer-VLAN tag (C-TAG) and/or Service-VLAN tag (S-TAG) VID fields as defined in IEEE 802.1Q

-	Customer-VLAN tag (C-TAG) and/or Service-VLAN tag (S-TAG) PCP/DEI fields as defined in IEEE 802.1Q

-	IP Packet Filter Set, in case Ethertype indicates IPv4/IPv6 payload

NOTE 1:	The MAC address may be specified as address ranges.

NOTE 2:	A value left unspecified in a filter matches any value of the corresponding information in a packet.

Per PDU session the list of allowed MAC addresses is limited to a maximum of 16 MAC addresses.

The SMF may provide to the UPF Ethernet Packet Filter Set and forwarding rule(s) based on the Ethernet frame structure and UE MAC address(es). The UPF detects and forwards Ethernet frames based on the Ethernet Packet Filter Set and forwarding rule(s) received from the SMF.

Ethernet Packet Filter Set and IP filterset are defined.

Ethernet Packet Filter Set is added to the Create PDR, Create FAR and update FAR.

Discussion:

More time is required to see how to design packet detection filter set, with CT3 coordination.. CT4 should have a full picture (requirements) before going further in enhancing PFCP for support of Ethernet traffic.

.

For MAC address, we need to see if source and destination needs to be supported.

Forwarding traffic capabilities need to be clarified for Ethernet traffic.

Decision: 		The document was postponed.

C4-176104	Supports N3, N9 and Xn userplane interface
					29.281	  CR-0084  Cat: B (Rel-15) v15.0.0
					Source: Huawei

Discussion:

In table 7.5.3.1-1 two IEs are missed over N4.

Setting in table 7.5.7.1 needs to be corrected.

Decision: 		The document was revised to C4-176370.

C4-176370	Supports N3, N9 and Xn userplane interface
					29.281	  CR-0084  rev 1 Cat: B (Rel-15) v15.0.0
					Source: Huawei

(Replaces C4-176104)

Decision: 		The document was agreed.

C4-176105	End Marker per QoS Flow
					29.281	  CR-0085  Cat: B (Rel-15) v15.0.0
					Source: Huawei

Decision: 		The document was postponed.

C4-176371	End Marker per QoS Flow
					29.281	  CR-0085  rev 1 Cat: B (Rel-15) v15.0.0
					Source: Huawei

(Replaces C4-176105)

Decision: 		The document was withdrawn.

##### 6.2.1.3	Contributions to TS 29.500

C4-176027	NF Service Advertisement URL
					29.500 v0.2.0
					Source: Ericsson

Abstract:

In Service Based Architecture (SBA) framework, HTTP is selected for service operation realization, which is based on client-server unidirectional communication. To enable bidirectional communication between NFs, e.g. Server-initiated communication, two client-server pairs are recommended (See 6.2.2.2.2.2 of 3GPP 29.891 v1.1.0).

In a subscribe/notify service operation, the NF service consumer as a HTTP client provides a notification URL to NF service producer as a HTTP server when invoking service operation request for creation of the subscription. When sending notification, the NF service producer as a HTTP client invokes HTTP methods towards the notification URL on NF service consumer as a HTTP server.

Besides subscribe/notify service operations, there are many other procedures also requires two-way communications between two NFs. E.g. between AMF and SMF. After a AMF selected a SMF and created a PDU session for certain UE using CreateSMContext service operation, the selected SMF shall use this AMF to send N1/N2 messages to the UE, using N1N2MessageTransfer service operation on this AMF. To do that, the SMF cannot "compose" the AMF service operation URL using the AMF instance address ({apiRoot}), because this AMF might only provide partial of services or might implement a different version of the services. The SMF shall use NRF discovery service to find out the AMF service end-point information. In case the SMF and AMF is using different NRFs, e.g. SMF is located in a specific network slice and the slice owns a slice specific NRF, the service discovery procedure might become low efficient or even unsuccessful.

Additionally, in a stateless solution, e.g. stateless AMF, the original AMF instance may become out of service and UE contexts is restored on another AMF instance. The SMF shall be able to find out the successor AMF and continue the service procedures.

This CR proposes a NF service consumer shall provide NF Service Advertisement URL(s) in service operation request to a NF service producer, and NF service producer may provide NF Service Advertisement URL(s) in service operation response to NF service consumer.

The NF Service Advertisement URL contains directly access to the NRF to which the NF has registered, thus the peer NF can easily and quickly discover the service available. e.g.:

{apiRoot}/Nnrf_disc/nfInstances?nfInstanceId={nfInstanceId}

The NF Service Advertisement URL (or via an alternative NF Service Advertisement URL) can also be used to discover the successor NF within the stateless NF group, in case the original NF become out of service. E.g.:

{apiRoot}/Nnrf_disc/nfInstances?nfInstanceId={nfInstanceId}&nfGroupId={nfGroupId}

Or

{apiRoot}/Nnrf_disc/nfInstances?nfInstanceId={alternativeNfInstanceId}

Decision: 		The document was revised to C4-176252.

C4-176252	NF Service Advertisement URL
					29.500 v0.2.0
					Source: Ericsson

(Replaces C4-176027)

Decision: 		The document was revised to C4-176401.

C4-176401	NF Service Advertisement URL
					29.500 v0.2.0
					Source: Ericsson

(Replaces C4-176252)

Decision: 		The document was revised to C4-176412.

C4-176412	NF Service Advertisement URL
					29.500 v0.2.0
					Source: Ericsson

(Replaces C4-176401)

Decision: 		The document was agreed.

C4-176028	HTTP Server Push
					29.500 v0.2.0
					Source: Ericsson

Decision: 		The document was revised to C4-176231.

C4-176231	HTTP Server Push
					29.500 v0.2.0
					Source: Ericsson

(Replaces C4-176028)

Abstract:

In Service Based Architecture (SBA) network, there are procedures requiring transferring multiple resources from one NF to another NF. E.g. Querying a collection of resources (see sub-clause 4.6.1.1.2.2 of TS 29.501 v0.2.0). When the number of resources is large, the solution of transferring these resources is yet to be studied.

The simplest way is to return representation of all resources in the response body. When the number of resources is large, the size of the response body may become large or even huge. Using a large/huge response body has drawbacks:

-	It is more vulnerable to transport layer problems. When transport layer fails, the whole body needs to be retransmitted.

-	Response latency may increase by buffering configuration mismatching among intermediaries.

-	HTTP/JSON stacks may hold the response in stack layer until the whole body is received and parsed, before passing to up layer application logic. This cause extra delay of data processing in receiving CN NFs.

-	The receiving NF doesn’t know how many resources and which resources will be received before parsing the whole body, if not informed in some other ways.

SBA has already selected HTTP/2 as the protocol and HTTP/2 supports Server-side initiated push (see sub-clause 8.2 of IETF RFC 7540), which allows the server to proactively push responses (after it sent the corresponding " PUSH_PROMISE " requests) to a client in association with a previous client initiated request.

HTTP/2 Server Push can be useful to transfer multiple resources between NFs. With HTTP/2 Server Push, the server first sending PUSH_PROMISE frames together with the response of the initial client request. Each PUSH_PROMISE contains the URI of one resource to be pushed as well as the reserved stream ID to be used for transferring the resource. Then server will send each resource in one Push Response via the reserved stream.

The benefits of using HTTP/2 Server Push are obvious:

-	Before receiving of any data representation of the resources, the receiving NF already got the information of the resources to be received from the service operation response, including number of the resources and all URIs of the resources. The receiving NF logic can already reserve physical machine resources (e.g. memory, worker, etc.) for processing the data.

-	Data representation of each resource are sent in separate Push Response, HTTP/JSON stack can work more efficiently and pass it to up layer application logic one by one.

-	Each resource is sent via separate stream. Single resource transport failure will not impact others. In case of certain resource is not received successfully, receiving NF can issue explicit request for this resource to fetch it, as the URI of this resource is already known.

Considering the drawbacks of using HTTP/2 Server Push, the main concern is the behavior of intermediaries. As stated in IETF RFC 7540, "An intermediary can receive pushes from the server and choose not to forward them on to the client. In other words, how to make use of the pushed information is up to that intermediary. Equally, the intermediary might choose to make additional pushes to the client, without any action taken by the server." This could be overcome, 3GPP standards could address this problem by putting corresponding requirements on intermediaries in their controlled environment. Such a requirement can be “an intermediary must relay to the server the SETTINGS_ENABLE_PUSH received from the client”.

Another consideration is the overhead by using HTTP/2 Server Push, as requires the server to generate and send a promised request for every pushes response. This can be handled by server NF logic dynamically. If the data representation of each resource is rather small, smaller than the Push Promise Frames, the server NF can choose not to use Server Push and return the data in the service operation body.

Conclusion: this CR recommend use HTTP/2 Server Push for transfer multiple resources between NFs.

Decision: 		The document was revised to C4-176255.

C4-176255	HTTP Server Push
					29.500 v0.2.0
					Source: Ericsson

(Replaces C4-176231)

Decision: 		The document was revised to C4-176402.

C4-176402	HTTP Server Push
					29.500 v0.2.0
					Source: Ericsson

(Replaces C4-176255)

Decision: 		The document was revised to C4-176413.

C4-176413	HTTP Server Push
					29.500 v0.2.0
					Source: Ericsson

(Replaces C4-176402)

Decision: 		The document was agreed.

C4-176108	HTTP Standard headers
					29.500 v0.2.0
					Source: Huawei

Abstract:

HTTP/2 is selected for the Service Based Interface, with the following standard headers will be used in 5GC:

	Accept header, e.g. for GET method, the "Accept" HTTP header of the HTTP/2 request signals the content formats that a client supports.

	Content-Length header, to provide the anticipated size, as a decimal number of octets, for a potential payload body.

	Content-Type header, Content-type field in HTTP/2 header performs as representation header fields and describes the representation data that would have been enclosed in the data frame, e.g. if content-type is application/json, resource representation in data frame is serialized in JSON format.

	Host header, to provide the host and port information from the target URI.

	Location header, e.g. the payload body of the POST response shall contain the representation of the created resource, and the "Location" header shall contain the URI of the created resource.

Whether Other headers, e.g. ETag will be used are FFS.

Discussion:

Support of other headers is still FFS.

The use of Host in HTTP/2 needs to be clarified.

Decision: 		The document was revised to C4-176257.

C4-176257	HTTP Standard headers
					29.500 v0.2.0
					Source: Huawei

(Replaces C4-176108)

Decision: 		The document was revised to C4-176409.

C4-176409	HTTP Standard headers
					29.500 v0.2.0
					Source: Huawei

(Replaces C4-176257)

Decision: 		The document was agreed.

C4-176109	HTTP Error Handling
					29.500 v0.2.0
					Source: Huawei

Abstract:

IETF RFC 7540 defines two classes of error in HTTP/2.

	An error condition that renders the entire connection unusable is connection error.

	An error in an individual stream is a stream error.

The GOAWAY frame or the RST_STREAM frame includes the error code will be sent to the peer node if the above errors happen.

Decision: 		The document was revised to C4-176258.

C4-176258	HTTP Error Handling
					29.500 v0.2.0
					Source: Huawei

(Replaces C4-176109)

Decision: 		The document was agreed.

C4-176110	Serialization protocol
					29.500 v0.2.0
					Source: Huawei

Abstract:

Application/json and the multipart messages are agreed to be used as serialization data format, this paper propose to introduce this agreement into TS 29.500.

Decision: 		The document was revised to C4-176259.

C4-176259	Serialization protocol
					29.500 v0.2.0
					Source: Huawei

(Replaces C4-176110)

Decision: 		The document was agreed.

C4-176111	Interface Definition Language
					29.500 v0.2.0
					Source: Huawei

Abstract:

OpenAPI 3.0.0 is agreed to be used as IDL, this paper propose to introduce this agreement into TS 29.500.

Decision: 		The document was agreed.

C4-176112	Server-initiated communication
					29.500 v0.2.0
					Source: Huawei

Abstract:

Subscribe-Notify service operations shall be supported with two client-server pairs, one per direction, this paper propose to introduce this agreement into TS 29.500.

Decision: 		The document was revised to C4-176260.

C4-176260	Server-initiated communication
					29.500 v0.2.0
					Source: Huawei

(Replaces C4-176112)

Decision: 		The document was revised to C4-176410.

C4-176410	Server-initiated communication
					29.500 v0.2.0
					Source: Huawei

(Replaces C4-176260)

Decision: 		The document was agreed.

C4-176199	HTTP PATCH
					29.500 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was postponed.

C4-176201	Supported Feature Negotiation
					29.500 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was revised to C4-176262.

C4-176262	Supported Feature Negotiation
					29.500 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176201)

Decision: 		The document was revised to C4-176411.

C4-176411	Supported Feature Negotiation
					29.500 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176262)

Decision: 		The document was agreed.

C4-176433	3GPP TS 29.500 v0.3.0
					29.500 v0.3.0
					Source: Huawei

Decision: 		The document was agreed.

##### 6.2.1.4	Contributions to TS 29.501

C4-176021	Discussion paper on resource archetype in REST
					29.501 v..
					Source: Orange

Abstract:

Resource archetypes help API designers to structure the resources. It also provides meaningful information to API consumers. The controller resource archetype is intended to be used for non-CRUD operations using the POST method. It can replace the custom operations or provide a REST compliant substitute.

Conclusion

Resource archetypes help API designers to structure the resources. It also provides meaningful information to API consumers. The controller resource archetype is intended to be used for non-CRUD operations using the POST method. It can replace the custom operations or provide a REST compliant substitute.

It is proposed to:

1.	Describe the different archetypes and their URI template in TS 29.501.

2.	Mandate that the archetype shall be provided when modelling a resource in an API specification;

3.	Replace the custom operation associated with a service by a controller resource archetype;

Corresponding CR have been proposed for TS 29.501.

•	C4-176132: Introducing resource archetypes in TS 29.501.

Discussion:

CT3 tdoc number is C3-176016.

Work on pCR to introduce the notion of archetypes in 29.501 is needed.

Decision: 		The document was noted.

C4-176132	Introducing resource archetypes in TS 29.501
					29.501 v0.2.0
					Source: Orange

Abstract:

This PCR aims at introducing resource archétypes defined in Mark Masse book.

"When modelling an API’s resources, we can start with the some basic resource archetypes.

Decision: 		The document was revised to C4-176253.

C4-176182	HTTP PATCH format evaluation
					29.501 v..
					Source: Ericsson

Decision: 		The document was revised to C4-176228.

C4-176253	Introducing resource archetypes in TS 29.501
					29.501 v0.2.0
					Source: Orange, Nokia, Nokia Shanghai Bell, Ericsson

(Replaces C4-176132)

Decision: 		The document was revised to C4-176414.

C4-176414	Introducing resource archetypes in TS 29.501
					29.501 v0.2.0
					Source: Orange, Nokia, Nokia Shanghai Bell, Ericsson

(Replaces C4-176253)

Decision: 		The document was agreed.

C4-176183	PATCH format
					29.501 v0.2.0
					Source: Ericsson

Decision: 		The document was postponed.

C4-176198	HTTP PATCH
					29.501 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was postponed.

C4-176202	Supported Feature Negotiation
					29.501 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Further related changes that this P-CR assumes are in C4-176201.

Decision: 		The document was revised to C4-176372.

C4-176372	Supported Feature Negotiation
					29.501 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176202)

Decision: 		The document was agreed.

C4-176130	Guidelines on hypermedia usage in 3GPP CT APIs
					29.501 v0.2.0
					Source: Orange

Abstract:

This PCR aims at providing guidelines on the usage of hypermedia in CT APIs

Decision: 		The document was revised to C4-176254.

C4-176254	Guidelines on hypermedia usage in 3GPP CT APIs
					29.501 v0.2.0
					Source: Orange

(Replaces C4-176130)

Decision: 		The document was revised to C4-176447.

C4-176447	Guidelines on hypermedia usage in 3GPP CT APIs
					29.501 v0.2.0
					Source: Orange

(Replaces C4-176254)

Decision: 		The document was agreed.

C4-176113	Resource Structure
					29.501 v0.2.0
					Source: Huawei

Decision: 		The document was revised to C4-176246.

C4-176246	Resource Structure
					29.501 v0.2.0
					Source: Huawei

(Replaces C4-176113)

Decision: 		The document was revised to C4-176415.

C4-176415	Resource Structure
					29.501 v0.2.0
					Source: Huawei

(Replaces C4-176246)

Decision: 		The document was agreed.

C4-176114	Resource and HTTP Methods
					29.501 v0.2.0
					Source: Huawei

Abstract:

This pCR provides guidelines on documentation of the APIs resource and their supported methods. As serialization protocol is JSON, an editor's note is added to discuss whether to add a column into the table of request and response body to specify the data structure name which would be the key value in JSON representation.

Decision: 		The document was revised to C4-176247.

C4-176247	Resource and HTTP Methods
					29.501 v0.2.0
					Source: Huawei

(Replaces C4-176114)

Decision: 		The document was revised to C4-176416.

C4-176416	Resource and HTTP Methods
					29.501 v0.2.0
					Source: Huawei

(Replaces C4-176247)

Decision: 		The document was agreed.

C4-176115	Representing RPC as Custom Operations on Resources
					29.501 v0.2.0
					Source: Huawei

Abstract:

This pCR provides guidelines on documentation of the API's remote procedure call operation. As serialization protocol is JSON, an editor's note is added to discuss whether to add a column into the table of request and response body to specify the data structure name which would be the key value in JSON representation.

Decision: 		The document was revised to C4-176248.

C4-176248	Representing RPC as Custom Operations on Resources
					29.501 v0.2.0
					Source: Huawei

(Replaces C4-176115)

Decision: 		The document was revised to C4-176417.

C4-176417	Representing RPC as Custom Operations on Resources
					29.501 v0.2.0
					Source: Huawei

(Replaces C4-176248)

Decision: 		The document was agreed.

C4-176116	Data Models
					29.501 v0.2.0
					Source: Huawei

Abstract:

This proposal provides guidelines on documentation of the API's data models.

Decision: 		The document was revised to C4-176212.

C4-176212	Data Models
					29.501 v0.2.0
					Source: Huawei Technologies

(Replaces C4-176116)

Abstract:

This proposal provides guidelines on documentation of the API's data models.

Discussion:

Clarify the use of Int for Enum type.

Should be consistently used across a spec (int or string but not both)

Decision: 		The document was revised to C4-176249.

C4-176249	Data Models
					29.501 v0.2.0
					Source: Huawei Technologies

(Replaces C4-176212)

Decision: 		The document was revised to C4-176418.

C4-176418	Data Models
					29.501 v0.2.0
					Source: Huawei Technologies

(Replaces C4-176249)

Decision: 		The document was agreed.

C4-176133	Replacing custom operations not associated with resource by controller resource archetype
					29.501 v0.2.0
					Source: Orange

Abstract:

It is proposed to replace custom operations associated with the service by a resource controller. The two concept are equivalent but it isn't needed to flag it explicitly as RPC. It is REST compliant to use URI to identify non CRUD operations when the URI

Decision: 		The document was withdrawn.

C4-176149	Creating a resource using PUT
					29.501 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was revised to C4-176227.

C4-176227	Creating a resource using PUT
					29.501 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176149)

Abstract:

Describe resource creation using the PUT method.

Discussion:

The location header is mandatory for consistency with POST.

Decision: 		The document was revised to C4-176250.

C4-176250	Creating a resource using PUT
					29.501 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176227)

Decision: 		The document was agreed.

C4-176228	HTTP PATCH format evaluation
					29.501 v..
					Source: Ericsson

(Replaces C4-176182)

Abstract:

As indicated in "JSON Patch and JSON Merge Patch": http://erosb.github.io/post/json-patch-vs-merge-patch/, the JSON Merge Patch format is a simpler format, but maybe it is a "naively simple format", with limited usability. It may be appropriate for very simple scenarios, but probably not so much for a complex system such as the 5GC. The clear functional limitations do not make up for the alleged simplicity, with particular emphasis in the limitation of not being able to manipulate individual array elements.

Decision: 		The document was noted.

C4-176197	POST response body
					29.501 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

When creating a resource using POST, a successful response may carry a representation of the created resource that differs from the representation as received in the request.

Decision: 		The document was revised to C4-176251.

C4-176251	POST response body
					29.501 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176197)

Decision: 		The document was revised to C4-176419.

C4-176419	POST response body
					29.501 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176251)

Decision: 		The document was agreed.

C4-176256	HTTP Server Push
					29.501 v0.2.0
					Source: Ericsson

Decision: 		The document was revised to C4-176389.

C4-176389	HTTP Server Push
					29.501 v0.2.0
					Source: Ericsson

(Replaces C4-176256)

Decision: 		The document was revised to C4-176404.

C4-176404	HTTP Server Push
					29.501 v0.2.0
					Source: Ericsson

(Replaces C4-176389)

Decision: 		The document was revised to C4-176422.

C4-176422	HTTP Server Push
					29.501 v0.2.0
					Source: Ericsson

(Replaces C4-176404)

Decision: 		The document was agreed.

C4-176434	3GPP TS 29.501 v0.3.0
					29.501 v0.3.0
					Source: Nokia

Decision: 		The document was agreed.

##### 6.2.1.5	Contributions to TS 29.502

C4-176006	3GPP TS 29.502 v0.2.0
					29.502 v0.2.0
					Source: Nokia

Decision: 		The document was withdrawn.

C4-176050	SMF APIs Design
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Conclusions for SMF API Design for service operations for use over N11:

*It is possible to design the Create/Update/Delete SM context and Notify SM context Status service operations as RESTful service operations.

*This is a good fit for the interactions to model.

*The RESTful modelling approach, summarized in slide 24, was agreed during CT4#80 for the service operations for use over N11.

*Stage 3 work in progress based on this modelling.

Conclusions for SMF API Design for service operations for use over N16

*It is possible to design the Create/Update/Delete/Notify Status service operations with a REST-like modelling.

**with individual pdu sessions resources managed by the H-SMF and with a vsmfPduSessionUri URI identifying the same pdu session representation in the V-SMF

**H-SMF initiated requests to modify or send status of a pdu session supported by sending standard http method or custom operation to the V-SMF on vsmfPduSessionUri provided by V-SMF during the PDU session establishment

*This model fits well the stage 2 requirements/procedures.

*This model is similar to the modelling agreed for Create/Update/Delete SM Context operations (N11)

*This model can be assimilated as RESTful from the perspective of H-SMF and V-SMF respectively, although the overall model is not RESTful (both SMFs play a client and server role for the pdu session resource, with application logic for data replication).

*QoS Flows can be modelized as an attribute (array of individual QoS Flows) of the V-SMF pdu session resource, with H-SMF using rpc/modify custom operation to create/update/delete QoS Flows(s).

*It is proposed to design the service operations over N16 as proposed above and summarized in slide 39.

Discussion:

CT4 agreed the principle of API design.

Decision: 		The document was noted.

C4-176051	Nsmf_PDUSession Service Description
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Two new service operations (Status Notify and SM Context Status Notify) have been specified in TS 23.502 to enable the H-SMF (in HR roaming scenarios) or the SMF to notify the V-SMF or AMF respectively about the status of a PDU session or SM context e.g. when a PDU session or SM Context is released in the H-SMF or SMF respectively.

The semantic of an SM context has also been clarified in TS 23.502. This represents an association between the NF Service Consumer (e.g. AMF) and the SMF for a PDU session.

The Release Service Operation may be invoked by the V-SMF.

See S2-178098, S2-178015, S2-178016 and S2-17878.

Decision: 		The document was agreed.

C4-176052	Resources and methods overview
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR implements the resources and methods overview presented in C4-176050 (SMF APIs Design).

Decision: 		The document was revised to C4-176263.

C4-176263	Resources and methods overview
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176052)

Decision: 		The document was agreed.

C4-176053	Create SM Context service operation
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR introduces text specifying the SM contexts collection resource and starting to specify the attributes of the SmContextCreateData representation in the HTTP POST request.

Discussion:

It was agreed to use different data types in request and response as they can be different. E.g. SmContextCreateData and SmContextCreatedData.

Decision: 		The document was revised to C4-176264.

C4-176264	Create SM Context service operation
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176053)

Decision: 		The document was agreed.

C4-176054	Update SM Context service operation
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR specifies the Update SM Context service operation according to the SMF API Design principles agreed by CT4 for the N11 service operations (see C4-176050).

Decision: 		The document was revised to C4-176265.

C4-176265	Update SM Context service operation
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176054)

Decision: 		The document was agreed.

C4-176055	Release SM Context service operation
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes updates to the description of the Release SM Context service operation:

˗	additional use cases for using this operation;

˗	updates of the URI to comply with the naming conventions agreed in TS 29.501;

˗	 renaming the data types in the Request and Response.

Discussion:

"smcontextid" should be "smcontextref".

Decision: 		The document was revised to C4-176266.

C4-176266	Release SM Context service operation
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176055)

Decision: 		The document was agreed.

C4-176056	Notify SM Context Status service operation
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

A new service operations (SM Context Status Notify) has been specified in TS 23.502 to enable the SMF to notify the AMF about the status of an SM context e.g. when an SM Context is released in the SMF.

This pCR proposes text to document this new service operation, according to the SMF API Design principles proposed in C4-176050.

Decision: 		The document was agreed.

C4-176057	Individual SM context resource
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR introduces text specifying the Individual SM context resource and starting to document the attributes sent in the SmContextUpdateData, SmContextUpdatedData, SmContextReleaseData and SmContextReleasedData.

Decision: 		The document was revised to C4-176268.

C4-176268	Individual SM context resource
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176057)

Decision: 		The document was agreed.

C4-176058	SM context status notification callback resource
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR introduces text specifying the SM context status notification callback resource and starting to document the attributes sent in the SmContextStatusNotification.

Decision: 		The document was revised to C4-176270.

C4-176270	SM context status notification callback resource
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176058)

Decision: 		The document was agreed.

C4-176059	Create service operation
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR introduces text specifying the Create service operation.

Decision: 		The document was revised to C4-176271.

C4-176271	Create service operation
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176059)

Decision: 		The document was agreed.

C4-176060	Update service operation
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR specifies the Update service operation according to the SMF API Design principles discussed in C4-176050.

Discussion:

Offline discussion is needed on PATCH.

Decision: 		The document was revised to C4-176273.

C4-176273	Update service operation
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176060)

Decision: 		The document was agreed.

C4-176061	Release service operation
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR specifies the Release service operation according to the SMF API Design principles discussed in C4-176050.

Decision: 		The document was agreed.

C4-176062	Notify Status service operation
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR specifies the Notify Status service operation according to the SMF API Design principles discussed in C4-176050.

Decision: 		The document was agreed.

C4-176274	Notify Status service operation
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176062)

Decision: 		The document was withdrawn.

C4-176063	Individual PDU session (H-SMF) resource
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR introduces text specifying the Individual PDU session resource and starting to document the data sent in the methods supported by this resource.

Decision: 		The document was revised to C4-176276.

C4-176276	Individual PDU session (H-SMF) resource
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176063)

Decision: 		The document was agreed.

C4-176064	Individual PDU session (V-SMF) resource
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was revised to C4-176277.

C4-176277	Individual PDU session (V-SMF) resource
					29.502 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176064)

Decision: 		The document was agreed.

C4-176435	3GPP TS 29.502 v0.3.0
					29.502 v0.3.0
					Source: Nokia

Decision: 		The document was agreed.

##### 6.2.1.6	Contributions to TS 29.503

C4-176150	Editorial Corrections
					29.503 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Some editorial corrections are proposed.

Decision: 		The document was agreed.

C4-176153	Nudm Overview
					29.503 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR provides input for clause 4 Overview.

Decision: 		The document was agreed.

C4-176155	Nudm_SDM service operations
					29.503 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

a) Remove implicit subscription to notification as a side effect from registration, as Nudm_SDM and Nudm_UECM are separate services.

b) Add subclauses in 5.2.2 for the (explicit) Subscribe and Unsubscribe service operations.

c) Update clause 6.1 to cover subscribe / unsubscribe service operations.

Discussion:

Implicit subscription to notification needs to be removed as a side effect from registration, since Nudm_SDM and Nudm_UECM are separate services.

Subclauses in 5.2.2 needs to be added for the (explicit) Subscribe and Unsubscribe service operations.

Clause 6.1 needs to be updated to cover subscribe / unsubscribe service operations.

Decision: 		The document was revised to C4-176300.

C4-176300	Nudm_SDM service operations
					29.503 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176155)

Decision: 		The document was revised to C4-176423.

C4-176423	Nudm_SDM service operations
					29.503 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176300)

Decision: 		The document was agreed.

C4-176159	Nudm_UEContextManagement
					29.503 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

To specify stage 3 details of the Nudm_UEContextManagement Service

Decision: 		The document was revised to C4-176365.

C4-176365	Nudm_UEContextManagement
					29.503 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176159)

Decision: 		The document was agreed.

C4-176160	Nudm_UEAuthentication Service
					29.503 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Provide stage 3 details for the Nudm_UEAuthentication Service

Decision: 		The document was revised to C4-176367.

C4-176367	Nudm_UEAuthentication Service
					29.503 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176160)

Decision: 		The document was revised to C4-176424.

C4-176424	Nudm_UEAuthentication Service
					29.503 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176367)

Decision: 		The document was agreed.

C4-176164	Nudm_EventExposure Service
					29.503 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Provide stage 3 details for the Nudm_EventExposure service

Discussion:

GPSI could be added to the resource tree.

One subscription per event.

Could we use one POST for multiple subscriptions? This can be added in Editor's note.

Could be used as a framework in 29.501 for subscription/notification

enumeration should be of type String.

Decision: 		The document was revised to C4-176374.

C4-176374	Nudm_EventExposure Service
					29.503 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176164)

Decision: 		The document was revised to C4-176425.

C4-176425	Nudm_EventExposure Service
					29.503 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176374)

Decision: 		The document was agreed.

C4-176436	3GPP TS 29.503 v0.3.0
					29.503 v0.3.0
					Source: Nokia

Decision: 		The document was agreed.

##### 6.2.1.7	Contributions to TS 29.509

C4-176169	Discussion paper on AUSF services
					29.509 v..
					Source: Orange

Abstract:

Disc paper to propose a REST-like approach for the AUSF service

Decision: 		The document was noted.

C4-176170	AUSF Services Overview
					29.509 v0.2.0
					Source: Orange

Abstract:

Overview of the AUSF service

Decision: 		The document was revised to C4-176267.

C4-176267	AUSF Services Overview
					29.509 v0.2.0
					Source: Orange

(Replaces C4-176170)

Decision: 		The document was agreed.

C4-176171	Introduction of the AUSF service
					29.509 v0.2.0
					Source: Orange

Abstract:

Introduction to the AUSF service

Decision: 		The document was revised to C4-176269.

C4-176269	Introduction of the AUSF service
					29.509 v0.2.0
					Source: Orange

(Replaces C4-176171)

Decision: 		The document was agreed.

C4-176173	AUSF Service operation: Authenticate
					29.509 v0.2.0
					Source: Orange

Decision: 		The document was revised to C4-176272.

C4-176272	AUSF Service operation: Authenticate
					29.509 v0.2.0
					Source: Orange

(Replaces C4-176173)

Decision: 		The document was revised to C4-176426.

C4-176426	AUSF Service operation: Authenticate
					29.509 v0.2.0
					Source: Orange

(Replaces C4-176272)

Decision: 		The document was agreed.

C4-176174	Resources Overview
					29.509 v0.2.0
					Source: Orange UK

Abstract:

Provides an overview of resources.

Decision: 		The document was postponed.

C4-176175	Resource: Authenticate
					29.509 v0.2.0
					Source: Orange

Decision: 		The document was revised to C4-176275.

C4-176275	Resource: Authenticate
					29.509 v0.2.0
					Source: Orange

(Replaces C4-176175)

Decision: 		The document was revised to C4-176427.

C4-176427	Resource: Authenticate
					29.509 v0.2.0
					Source: Orange

(Replaces C4-176275)

Decision: 		The document was agreed.

C4-176176	Resource created: confirm or eap-session
					29.509 v0.2.0
					Source: Orange

Decision: 		The document was postponed.

C4-176437	3GPP TS 29.509 v0.3.0
					29.509 v0.3.0
					Source: Orange

Decision: 		The document was agreed.

##### 6.2.1.8	Contributions to TS 29.510

C4-176099	Pseudo-CR on TS 29.510, Clarification on N27 interface
					29.510 v0.2.0
					Source: ZTE

Abstract:

This contribution clarifies the N27 interface is a reference representation of Nnrf service based interface.

Discussion:

Needs to be clarified that Nnrf is used over N27.

Decision: 		The document was revised to C4-176278.

C4-176278	Pseudo-CR on TS 29.510, Clarification on N27 interface
					29.510 v0.2.0
					Source: ZTE

(Replaces C4-176099)

Decision: 		The document was agreed.

C4-176184	NRF Skeleton update
					29.510 v0.2.0
					Source: Ericsson

Abstract:

Alignment with latest changes in SA2, where the NFStatus service has been merged into the NFManagement service.

Decision: 		The document was agreed.

C4-176185	NRF Services
					29.510 v0.2.0
					Source: Ericsson

Abstract:

Alignment with latest changes in SA2, where the NFStatus service has been merged into the NFManagement service.

Decision: 		The document was revised to C4-176280.

C4-176280	NRF Services
					29.510 v0.2.0
					Source: Ericsson

(Replaces C4-176185)

Decision: 		The document was agreed.

C4-176186	NFManagement Resources
					29.510 v0.2.0
					Source: Ericsson

Abstract:

Alignment with latest changes in SA2, where the NFStatus service has been merged into the NFManagement service.

Discussion:

Need to be checked if the convention for API name should be "nrf-nfm" or just "nfm".

Decision: 		The document was revised to C4-176281.

C4-176281	NFManagement Resources
					29.510 v0.2.0
					Source: Ericsson

(Replaces C4-176186)

Decision: 		The document was agreed.

C4-176187	NFDiscovery Resources
					29.510 v0.2.0
					Source: Ericsson

Abstract:

Initial resource structure for Nnrf_NFDiscovery service in NRF.

Discussion:

Need to be checked if the convention for API name should be "nrf-nfm" or just "nfm".

Decision: 		The document was revised to C4-176282.

C4-176282	NFDiscovery Resources
					29.510 v0.2.0
					Source: Ericsson

(Replaces C4-176187)

Decision: 		The document was agreed.

C4-176438	3GPP TS 29.510 v0.3.0
					29.510 v0.3.0
					Source: Ericsson

Decision: 		The document was agreed.

##### 6.2.1.9	Contributions to TS 29.511

C4-176122	Equipment Identity Check description
					29.511 v0.2.0
					Source: Huawei

Abstract:

GET operation for N5g-eir_EquipmentIdentityCheck service will be used to check the PEI in the 5G-EIR. 5G-EIR will responses the equipment status to the AMF to indicated WHITELISTED/ BLACKLISTED/ GREYLISTED of the UE.

equipmentStatus is proposed to be the resource of this operation, and the equipmentStatusId will be the PEI of the UE.

Discussion:

Needs to be checked what will be used as URI of the GET.

PEI is mandatory but SUPI is required as an input parameter.

Decision: 		The document was revised to C4-176283.

C4-176283	Equipment Identity Check description
					29.511 v0.2.0
					Source: Huawei

(Replaces C4-176122)

Decision: 		The document was revised to C4-176428.

C4-176428	Equipment Identity Check description
					29.511 v0.2.0
					Source: Huawei

(Replaces C4-176283)

Decision: 		The document was agreed.

C4-176123	Equipment Identity Check API
					29.511 v0.2.0
					Source: Huawei

Abstract:

GET operation for N5g-eir_EquipmentIdentityCheck service will be used to check the PEI in the 5G-EIR. 5G-EIR will responses the equipment status to the AMF to indicated WHITELISTED/ BLACKLISTED/ GREYLISTED of the UE.

equipmentStatus is proposed to be the resource of this operation, and the equipmentStatusId will be the PEI of the UE.

Discussion:

Alignment with 6283 on the URI is needed.

Decision: 		The document was revised to C4-176284.

C4-176284	Equipment Identity Check API
					29.511 v0.2.0
					Source: Huawei

(Replaces C4-176123)

Decision: 		The document was revised to C4-176429.

C4-176429	Equipment Identity Check API
					29.511 v0.2.0
					Source: Huawei

(Replaces C4-176284)

Decision: 		The document was agreed.

C4-176439	3GPP TS 29.511 v0.3.0
					29.511 v0.3.0
					Source: Deutsche Telekom

Decision: 		The document was agreed.

##### 6.2.1.10	Contributions to TS 29.518

C4-176029	N1N2MessageTransfer failure handling
					29.518 v0.2.0
					Source: Ericsson

Abstract:

The N1N2MessageTransfer service operation is used by a CN NF requesting to transfer downlink N1 and/or N2 information to the UE through the AMF. It is used by many procedures, the AMF acting as a relay transfers the received N1 and/or N2 information to the UEs and/or RAN respectively.

From procedure point of view, the N1N2MessageTransfer request may not be delivered successfully, e.g. when UE is not reachable.

In some conditions, AMF could immediately aware UE is unreachable and reject the service request; or otherwise, AMF may send trigger a paging request and UE doesn’t respond to the paging request.

These two types of failure cases which are other than protocol related error scenarios need to be addressed.

Decision: 		The document was revised to C4-176285.

C4-176285	N1N2MessageTransfer failure handling
					29.518 v0.2.0
					Source: Ericsson

(Replaces C4-176029)

Decision: 		The document was agreed.

C4-176030	N1N2MessageTransfer HTTP method
					29.518 v0.2.0
					Source: Ericsson

Abstract:

To further study the AMF Service Operation N1N2MessageTransfer HTTP command selection and failure handling.

Discussion:

Editor's note needs to be added on the handling of retramission

CT4 should consider the DRX period to see how to manage the request. This will be covered by Editor's note.

Decision: 		The document was revised to C4-176287.

C4-176287	N1N2MessageTransfer HTTP method
					29.518 v0.2.0
					Source: Ericsson

(Replaces C4-176030)

Decision: 		The document was revised to C4-176403.

C4-176403	N1N2MessageTransfer HTTP method
					29.518 v0.2.0
					Source: Ericsson

(Replaces C4-176287)

Decision: 		The document was agreed.

C4-176031	N1MessageNotify HTTP method
					29.518 v0.2.0
					Source: Ericsson

Abstract:

The N1MessageNotify service operation is used by a AMF to notify the N1 message received from the UE to a destination CN NF.

However, per 3GPP TS 23.502-v1.3.0, this service operation is only used in the procedure Registration with AMF re-allocation as specified in subclause 4.2.2.2.3.

The operation semantics of this service operation N1MessageNotify is Subscribe/Notify, i.e. a NF service consumer need either explicit subscribe or implicit subscribe to get N1 message notified, then the Service Producer will deliver the N1 message received from the UE subsequently. Therefore, in this procedure, the initial AMF shall be served as a server to notify the client, the target AMF, the received N1 message.

However, such semantics doesn't fit the purpose the procedure, where the initial AMF, upon receiving the N1 message, it will read the N1 message, and attempt to retrieve some subscription data, and then based on local policy and subscription information, to perform a Service Discovery procedure, to find a proper AMF that can serve the UE, i.e. to create the relevant UE Context in that selected AMF. Apparently, in this procedure, the initial AMF is served as a client.

So this pCR proposes that the initial AMF shall use HTTP POST to request the target AMF to serve the UE and create the corresponding UE Context.

Decision: 		The document was revised to C4-176290.

C4-176290	N1MessageNotify HTTP method
					29.518 v0.2.0
					Source: Ericsson

(Replaces C4-176031)

Decision: 		The document was agreed.

C4-176032	N2MessageNotify HTTP Method
					29.518 v0.2.0
					Source: Ericsson

Abstract:

The N2MessageNotify service operation is used by a AMF to notify a particular N2 message information towards the NFs that have subscribed (implicitly or explicitly) for the specific information.

Decision: 		The document was revised to C4-176291.

C4-176291	N2MessageNotify HTTP Method
					29.518 v0.2.0
					Source: Ericsson

(Replaces C4-176032)

Decision: 		The document was agreed.

C4-176033	Namf_Communication_EBIAssignment HTTP Method
					29.518 v0.2.0
					Source: Ericsson

Abstract:

The Namf_Communication_EBIAssignment service operation is used by a consumer NF, e.g, SMF, to request AMF to allocate a number of EPS bearer IDs for a PDU session, and subsequently to request the AMF that certain EBIs allocated before can be recycled, i.e. be free to be used later.

Discussion:

Needs to be clarified if PATCH is supposed to be atomic.

Format of JSON is FFS.

Decision: 		The document was revised to C4-176292.

C4-176292	Namf_Communication_EBIAssignment HTTP Method
					29.518 v0.2.0
					Source: Ericsson

(Replaces C4-176033)

Decision: 		The document was agreed.

C4-176034	Namf_Communication Resource and Methods  Overview
					29.518 v0.2.0
					Source: Ericsson

Abstract:

The pCR proposes some general description in chapter 5, and summarize the Namf_Communication resource and method in chapter 6.

Discussion:

It was agreed that a NOTE 2 is not needed.

It was commented that a callback URI is missing.

Decision: 		The document was revised to C4-176293.

C4-176293	Namf_Communication Resource and Methods  Overview
					29.518 v0.2.0
					Source: Ericsson

(Replaces C4-176034)

Decision: 		The document was agreed.

C4-176117	Create UE Context Description
					29.518 v0.2.0
					Source: Huawei

Abstract:

CreateUEContext service operation is used in the Inter NG-RAN node N2 based handover procedure to create the UE Context in the target AMF by the source AMF, which is agreed in S2-177635 in SA2 #123 meeting.

It is proposed the PUT method can be used to perform this service operation.

Discussion:

In step 1, in the body, we may include the notification URI.

Editor's note needs to be added to indicate that the use of POST is FFS.

Decision: 		The document was revised to C4-176375.

C4-176375	Create UE Context Description
					29.518 v0.2.0
					Source: Huawei

(Replaces C4-176117)

Decision: 		The document was agreed.

C4-176118	Create UE Context API
					29.518 v0.2.0
					Source: Huawei

Abstract:

CreateUEContext service operation is used in the Inter NG-RAN node N2 based handover procedure to create the UE Context in the target AMF by the source AMF, which is agreed in S2-177635 in SA2 #123 meeting.

It is proposed the PUT method can be used to perform this service operation. The resource is ueContext, and the corresponding URI is: {apiRoot}/namf-communication/v1/ue-contexts/{ueContextId}, ueContextId shall be the supi of the user.

Decision: 		The document was revised to C4-176376.

C4-176376	Create UE Context API
					29.518 v0.2.0
					Source: Huawei

(Replaces C4-176118)

Decision: 		The document was agreed.

C4-176119	Release UE Context Description
					29.518 v0.2.0
					Source: Huawei

Decision: 		The document was revised to C4-176379.

C4-176379	Release UE Context Description
					29.518 v0.2.0
					Source: Huawei

(Replaces C4-176119)

Decision: 		The document was agreed.

C4-176120	Release UE Context API
					29.518 v0.2.0
					Source: Huawei

Abstract:

ReleaseUEContext service operation is used in the Inter NG-RAN node N2 based handover, Cancel procedure to release the UE Context in the target AMF by the source AMF, which is agreed in S2-177635 and S2-177637 in SA2 #123 meeting.

As the PDU session IDs and the Relocation Cancel Indication shall be included in the request, for each PDU session indicated by S-RAN by the PDU session IDs, the target AMF invokes the Nsmf_PDUSession_UpdateSMContext request (SUPI, Relocation Cancel Indication) to SMF.

It is proposed the RPC method can be used to perform this service operation, and to include the PDU session ID and the Relocation Cancel Indication in the message body.

3. Conclusions

Discussion:

Editor's note is needed to cover that the data type is FFS.

Decision: 		The document was revised to C4-176378.

C4-176378	Release UE Context API
					29.518 v0.2.0
					Source: Huawei

(Replaces C4-176120)

Decision: 		The document was agreed.

C4-176121	Data Types of Namf-MT Service
					29.518 v0.2.0
					Source: Huawei

Abstract:

ueReachability in the ueReachInd resource is proposed to be enumeration type , with value 0 indicates the UE unreachable, and value 1 indicates the UE reachable.

The consumer shall set the value to 1 and sent to the producer to make the UE state changed to UE reachable.

Section 6.3.1 is updated to add the API URI for Namf_MT Service API, and some editorial changes are also captured.

Decision: 		The document was revised to C4-176380.

C4-176380	Data Types of Namf-MT Service
					29.518 v0.2.0
					Source: Huawei

(Replaces C4-176121)

Decision: 		The document was agreed.

C4-176440	3GPP TS 29.518 v0.3.0
					29.518 v0.3.0
					Source: Ericsson

Decision: 		The document was agreed.

##### 6.2.1.11	Contributions to TS 23.527

##### 6.2.1.12	Contributions to TS 29.531

##### 6.2.1.13	Contributions to TS 29.540

C4-176092	Pseudo-CR on TS 29.540 subclause 4 Overview
					29.540 v0.2.0
					Source: ZTE

Abstract:

This contribution proposes updates to subclause 4 of TS 29.540 v0.2.0.

- Necessary abbreviations are added.

- The reference of service operations provided by Nsmsf interface is added.

Decision: 		The document was agreed.

C4-176093	Pseudo-CR on TS 29.540 subclause 5 Services
					29.540 v0.2.0
					Source: ZTE

Abstract:

This contribution adds general description to subclause 5 services of TS 29.540 v0.2.0.

Decision: 		The document was revised to C4-176346.

C4-176346	Pseudo-CR on TS 29.540 subclause 5 Services
					29.540 v0.2.0
					Source: ZTE

(Replaces C4-176093)

Decision: 		The document was agreed.

C4-176094	Pseudo-CR on TS 29.540 subclause 5, Service Operation - Activate
					29.540 v0.2.0
					Source: ZTE

Abstract:

This contribution addresses the aspects of Activate service operation to subclause 5 services of TS 29.540 v0.2.0.

Decision: 		The document was revised to C4-176347.

C4-176347	Pseudo-CR on TS 29.540 subclause 5, Service Operation - Activate
					29.540 v0.2.0
					Source: ZTE

(Replaces C4-176094)

Decision: 		The document was agreed.

C4-176095	Pseudo-CR on TS 29.540 subclause 5, Service Operation - Deactivate
					29.540 v0.2.0
					Source: ZTE

Abstract:

This contribution addresses the aspects of Deactivate service operation to subclause 5 services of TS 29.540 v0.2.0.

Decision: 		The document was revised to C4-176349.

C4-176349	Pseudo-CR on TS 29.540 subclause 5, Service Operation - Deactivate
					29.540 v0.2.0
					Source: ZTE

(Replaces C4-176095)

Decision: 		The document was agreed.

C4-176096	Pseudo-CR on TS 29.540 subclause 5, Service Operation - UplinkSMS
					29.540 v0.2.0
					Source: ZTE

Abstract:

This contribution addresses the aspects of UplinkSMS service operation to subclause 5 services of TS 29.540 v0.2.0.

Decision: 		The document was revised to C4-176351.

C4-176351	Pseudo-CR on TS 29.540 subclause 5, Service Operation - UplinkSMS
					29.540 v0.2.0
					Source: ZTE

(Replaces C4-176096)

Decision: 		The document was agreed.

C4-176097	Pseudo-CR on TS 29.540 subclause 6.1.1 API URI
					29.540 v0.2.0
					Source: ZTE

Abstract:

This contribution defines the API URI of SMSF interface to TS 29.540 v0.2.0.

Decision: 		The document was agreed.

C4-176098	Pseudo-CR on TS 29.540 subclause 6.1.3 Resources
					29.540 v0.2.0
					Source: ZTE

Abstract:

This contribution proposes definition of the Resources structure to TS 29.540 v0.2.0.

Decision: 		The document was revised to C4-176353.

C4-176353	Pseudo-CR on TS 29.540 subclause 6.1.3 Resources
					29.540 v0.2.0
					Source: ZTE

(Replaces C4-176098)

Decision: 		The document was agreed.

C4-176441	3GPP TS 29.540 v0.3.0
					29.540 v0.3.0
					Source: ZTE

Decision: 		The document was agreed.

##### 6.2.1.14	Contributions to TS 29.544

##### 6.2.1.15	Contributions to TS 29.571

C4-176131	Introducing a chapter in TS 29.571 for relation type considerations
					29.571 v0.2.0
					Source: Orange

Abstract:

This PCR aims at intrWhen two resources are linked together, the relation sematic shall not be given by the linked resource URI but by a semantic markup instead. In the hypermedia format it is carried by a “rel” attribute. The client infers the relation from the “rel” attribute content. It is therefore needed to agree on supported relation type and make it available to everyone in our Technical Specification. It is proposed to use TS 29.571 as the repository of relation types supported by 3GPP APIs.

There are already registered relation type such as “self”, “collection” or “next”. When the relation between two resources corresponds to a standard relation type we shall use them. So it is needed to add a chapter where we would reference all the registered relation types supported in our APIs amongst those registered by IANA here:

Whenever a relation type used in a 3GPP API does not match one of the IANA registry then it is needed to define its semantic. So it is also needed to add a chapter to describe 3GPP specific relations. An example of the information needed for a link relation type definition can be found here http://www.w3.org/TR/html5/links.html#link-type-next.

Discussion:

It needs to be checked if this is covered in 29.571 or 29.501.

Decision: 		The document was revised to C4-176295.

C4-176295	Introducing a chapter in TS 29.571 for relation type considerations
					29.571 v0.2.0
					Source: Orange

(Replaces C4-176131)

Discussion:

Merged into C4-176254

Decision: 		The document was merged.

C4-176203	Common Data Types suggested by CT3
					29.571 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

CT3 already identified some candidates for common data types: API encoding proposals in TR 29.890 contain notes identifying such data types. T8 TS 29.122 contains a section defining data types common between multiple T8 APIs; some of these data types appear to be of quite generic nature and to be of interest also for 5G APIs.

In addition to data types identified in that manner, data types defined in the OpenAPI 3.0.0 specification are included. OpenAPI defines a format parameter that allows to refine the definition of basic data types. This parameter can take values defined in the OpenApi specification, but also values defined elsewhere. For combinations of basic data types and formats defined in the OpenApi specification, data type names are defined.

It is also suggested that own simple data types defined by 3GPP carry their name as format parameter within OpenAPI schema definitions.

This contribution is also presented for information to CT3 in C3-176045.

Decision: 		The document was revised to C4-176294.

C4-176294	Common Data Types suggested by CT3
					29.571 v0.2.0
					Source: Nokia, Nokia Shanghai Bell, Huawei

(Replaces C4-176203)

Decision: 		The document was revised to C4-176431.

C4-176431	Common Data Types suggested by CT3
					29.571 v0.2.0
					Source: Nokia, Nokia Shanghai Bell, Huawei

(Replaces C4-176294)

Decision: 		The document was agreed.

C4-176211	Supported Feature Negotiation
					29.571 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

An Extensibility mechanisms needs to be documented. Related reasoning is in contribution C4-176200.

Further related changes that this P-CR assumes are in C4-176201.

Decision: 		The document was revised to C4-176296.

C4-176296	Supported Feature Negotiation
					29.571 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176211)

Decision: 		The document was postponed.

C4-176215	Pseudo-CR on definition of GSPI
					29.571 v0.2.0
					Source: Huawei

Abstract:

Stage 2  has defined a  SUPI. SUPI is used on many SBI interface.

Discussion:

Merged into C4-176294.

Decision: 		The document was merged.

C4-176216	Pseudo-CR on definition of SUPI
					29.571 v0.2.0
					Source: Huawei

Discussion:

Merged into C4-176294.

Decision: 		The document was merged.

C4-176219	Defining an hypermedia format for 3GPP API
					29.571 v0.2.0
					Source: Orange

Abstract:

A hypermedia format shall be used to identify resource relation. 3GPP shall either select an existing standard hypermedia format or define a new one in its own application domain. The format shall have at least two attributes to identify the linked resource address and the relation type.

Discussion:

There is no need for Link header at this stage.

It needs to be checked if covered in 29.571 or 29.501.

Decision: 		The document was revised to C4-176297.

C4-176297	Defining an hypermedia format for 3GPP API
					29.571 v0.2.0
					Source: Orange

(Replaces C4-176219)

Discussion:

Merged into 6254

Decision: 		The document was merged.

C4-176442	3GPP TS 29.571 v0.3.0
					29.571 v0.3.0
					Source: Huawei

Decision: 		The document was agreed.

#### 6.2.2	IMS impact due to 5GS IP-CAN [IMSo5G]

#### 6.2.3	CT aspects of Northbound APIs for SCEF – SCS/AS Interworking [NAPS-CT]

#### 6.2.4	CT aspects of support of voice services over WLAN Access [VoWLAN-CT]

#### 6.2.5	WLAN direct discovery technologies for ProSe direct discovery [ProSe_WLAN_DD_Stage3]

C4-176143	Updates for ProSe WLAN Direct Discovery
					29.345	  CR-0053  Cat: B (Rel-15) v14.3.0
					Source: Intel

Decision: 		The document was revised to C4-176348.

C4-176348	Updates for ProSe WLAN Direct Discovery
					29.345	  CR-0053  rev 1 Cat: B (Rel-15) v14.3.0
					Source: Intel

(Replaces C4-176143)

Decision: 		The document was agreed.

C4-176205	Storage of PC5-tech for WLAN based ProSe Direct Discovery
					23.008	  CR-0542  Cat: B (Rel-15) v15.1.0
					Source: Intel

Decision: 		The document was revised to C4-176350.

C4-176350	Storage of PC5-tech for WLAN based ProSe Direct Discovery
					23.008	  CR-0542  rev 1 Cat: B (Rel-15) v15.1.0
					Source: Intel

(Replaces C4-176205)

Decision: 		The document was agreed.

C4-176354	Vivek Gupta
					29.230	  CR-0627  Cat: B (Rel-15) v15.0.0
					Source: Intel

Decision: 		The document was agreed.

#### 6.2.6	CT aspects of 3GPP PS data off function [PS_DATA_OFF2-CT]

### 6.3	Any Other Business for Rel-15

C4-176177	Updates for SRv6 Mobile User-Plane
					Source: Softbank

(Replaces C4-174204)

Abstract:

This document discusses the applicability of SRv6 (Segment Routing IPv6) to user-plane of mobile networks that SRv6 source routing capability with its programmability can fulfill the user-plane functions, such as access and anchor functions.

This contribution brings 3GPP CT4 updates of SRv6 mobile user-plane work after CT4#79 where it was first presented.  The proposal can be found as an Internet-Draft at following link:

https://tools.ietf.org/html/draft-matsushima-spring-dmm-srv6-mobile-uplane

The purpose of the contribution is to review the updates and discuss about it with 3GPP experts.

Discussion:

It was requested that the delegates read and comment this discussion paper and give related comments also to IETF.

Decision: 		The document was noted.

#### 6.3.1	GTP and PMIP [TEI15]

C4-176151	LS on a new requirement on “Unpredictable GTP TEID”.
					Source: SA3

(Replaces C4-175194)

Abstract:

SA3 would like to inform CT4 that a new requirement on GTP TEID assignment has been agreed for inclusion in the (Rel15) draft TS33.250:

The GTP TEID shall be unpredictable

Decision: 		The document was postponed.

C4-176012	Setting of DSCP value for priority services on a GTPv2-C Interface
					29.274	  CR-1837  rev 2 Cat: B (Rel-15) v15.1.0
					Source: Vencore Labs, OEC, AT&T, T-Mobile USA

(Replaces C4-174293)

Abstract:

This CR proposes additional text to allow setting of the DSCP on a GTPv2-C Interface in support of priority services.

Discussion:

All the requirements in 8.15 and 8.64 shall be captured in 10.X.

Decision: 		The document was revised to C4-176327.

C4-176327	Setting of DSCP value for priority services on a GTPv2-C Interface
					29.274	  CR-1837  rev 3 Cat: B (Rel-15) v15.1.0
					Source: Vencore Labs, OEC, AT&T, T-Mobile USA

(Replaces C4-176012)

Decision: 		The document was revised to C4-176382.

C4-176382	Setting of DSCP value for priority services on a GTPv2-C Interface
					29.274	  CR-1837  rev 4 Cat: B (Rel-15) v15.1.0
					Source: Vencore Labs, OEC, AT&T, T-Mobile USA

(Replaces C4-176327)

Decision: 		The document was agreed.

#### 6.3.2	Addressing and Subscriber Data handling (23.003, 23.008) [TEI15]

#### 6.3.3	EPS AAA interfaces (29.273) [TEI15]

#### 6.3.4	Diameter based Interfaces (29.272, 29.173) [TEI15]

#### 6.3.5	IMS [TEI15]

C4-176204	IMEI based SIP URI for P-Preferred-Identity
					23.003	  CR-0486  Cat: F (Rel-15) v15.1.0
					Source: Qualcomm Incorporated

Decision: 		The document was revised to C4-176220.

C4-176220	IMEI based SIP URI for P-Preferred-Identity
					23.003	  CR-0486  rev 1 Cat: F (Rel-15) v15.1.0
					Source: Qualcomm Incorporated

(Replaces C4-176204)

Decision: 		The document was revised to C4-176301.

C4-176301	IMEI based SIP URI for P-Preferred-Identity
					23.003	  CR-0486  rev 2 Cat: F (Rel-15) v15.1.0
					Source: Qualcomm Incorporated

(Replaces C4-176220)

Decision: 		The document was agreed.

#### 6.3.6	MAP [TEI15]

#### 6.3.7	H.248 Interface [TEI15]

#### 6.3.8	Diameter 29.230 CRs [TEI15]

#### 6.3.9	SIMTC [TEI15]

#### 6.3.10	CIoT [TEI15]

#### 6.3.11	MONTE [TEI15]

C4-176167	Reachability Type
					29.336	  CR-0100  rev 2 Cat: A (Rel-15) v15.0.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-174070)

Decision: 		The document was revised to C4-176326.

C4-176326	Reachability Type
					29.336	  CR-0100  rev 2 Cat: F (Rel-15) v15.0.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176167)

Decision: 		The document was agreed.

C4-176178	Corrections to S6t procedures
					29.336	  CR-0113  Cat: F (Rel-15) v15.0.0
					Source: Ericsson

Decision: 		The document was revised to C4-176392.

C4-176392	Corrections to S6t procedures
					29.336	  CR-0113  rev 1 Cat: F (Rel-15) v15.0.0
					Source: Ericsson

(Replaces C4-176178)

Decision: 		The document was agreed.

C4-176179	AVP code for PLMN-ID-Requested in S6t
					29.230	  CR-0626  Cat: F (Rel-15) v15.0.0
					Source: Ericsson

Decision: 		The document was agreed.

C4-176180	Extension of SCEF-Reference-ID
					Source: Ericsson

Decision: 		The document was noted.

C4-176181	PLMN-ID-Requested in Monitoring-Event-Configuration
					29.272	  CR-0745  Cat: F (Rel-15) v15.1.0
					Source: Ericsson

Decision: 		The document was withdrawn.

C4-176323	Reachability Type
						29.336	  CR-0114  Cat: F (Rel-13) v13.9.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was agreed.

C4-176325	Reachability Type
						29.336	  CR-0115  Cat: A (Rel-14) v14.3.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was agreed.

#### 6.3.12	CIoT

#### 6.3.13	AOB Rel-15

## 7	Rel-14

### 7.1	CT4 Led WIDs

#### 7.1.1	CT aspects of MTSI Extension on Multi-stream Multiparty Conferencing Media Handling [MMCMH-CT]

#### 7.1.2	SCC AS Restoration [SCCAS_RES]

#### 7.1.3	Shared Subscription Data Update [eSDU]

#### 7.1.4	EIR check for WLAN access to EPC [EWE-CT]

#### 7.1.5	CT aspects of Control and User Plane Separation of EPC nodes [CUPS-CT]

C4-176014	Clarification on Create PDR and Create FAR IEs in Sx Session Establishment Request
					29.244	  CR-0043  Cat: F (Rel-14) v14.1.0
					Source: Ericsson

Abstract:

It is ambiguous how to encode multiple PDRs or FARs, e.g.. whether it is possible to encode multiple PDRs in the same IE, or it requires multiple IE instance.

Add clarification to Create PDR and Create FAR IE, these IEs shall be present for at least one PDR/FAR to be associated to the Sx session. And several IEs within the same IE type may be present to represent multiple PDRs.

Decision: 		The document was revised to C4-176341.

C4-176341	Clarification on Create PDR and Create FAR IEs in Sx Session Establishment Request
					29.244	  CR-0043  rev 1 Cat: F (Rel-14) v14.1.0
					Source: Ericsson

(Replaces C4-176014)

Decision: 		The document was agreed.

C4-176035	Corrections to Sx session update procedures
					29.244	  CR-0044  Cat: F (Rel-14) v14.1.0
					Source: Cisco Systems

Decision: 		The document was revised to C4-176237.

C4-176237	Corrections to Sx session update procedures
					29.244	  CR-0044  rev 1 Cat: F (Rel-14) v14.1.0
					Source: Cisco Systems, Huawei

(Replaces C4-176035)

Abstract:

1.	There will be a logical discrepancy between the processing flow and the actual implementation which will make the system more complex.

2.	F-TEID and IP address information is duplicated in each PDI within a PDR, which substantially increases the memory requirement for the PDI and PDR

3.	When the F-TEID is allocated by the UP, the CP ends up having to update each and every PDR/PDI with the allocated F-TEID information.

4.	A flat table for all the PDR’s substantially increases the lookup cost for the flow.

5.	Correlation ID to correlate the Uplink and downlink PDR of the same bearer.

6.	For rule pre-configured at the UP function, the CP function binding service flow to the bearer  and provide the TFT to the UP function.

Discussion:

Changes are important and they do impact the processing of the messages.

Same benefits with limited changes in the message handling is proposed in C4-176210.

Decision: 		The document was withdrawn.

C4-176210	Packet Detection Information
					29.244	  CR-0057  Cat: F (Rel-14) v14.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

1/ A new EndPoint ID IE is defined in the PDI IE to identify an endpoint (i.e. combination of Local -FTEID, Network Instance, and/or UE IP address) for a given Sx session and source interface.

Once an endpoint has been provisioned for an Sx session, subsequent PDRs can be created by just provisioning the EndPoint ID, without providing again the Local-FTEID, Network Instance and UE IP address parameters.

This also removes the need to support a CHOOSE ID in the F-TEID IE. The CP function can create multiple PDRs for the same F-TEID by simply provisioning the same EndPoint ID in corresponding PDRs.

2/ A new Correlation EndPoint ID is added to the FAR to allow the UP function to correlate UL and DL PDRs using the same bearer.

Discussion:

Offline discussion after this meeting.

Companies agreed on the need to enhance the definition of the Packet Detection Information to avoid having to provision multiple times the same PDI parameters for different PDRs/SDFs of a same Sx session, to support bi-directional filters and means to correlate UL/DL traffic using a same bearer.

If companies can agree on a proposal prior to CT Plenary, companies can send a revised CR to CT Plenary for approval in December Plenary, as this is the very last chance to introduce such changes in Rel-14.

Decision: 		The document was postponed.

C4-176036	Maximum-Change-Conditon-Gz-Trigger support
					29.244	  CR-0045  Cat: F (Rel-14) v14.1.0
					Source: Cisco Systems

Abstract:

According to 32.251, PGWCDR reporting happens at Service-Data-Flow(SDF) level and triggers are applicable at both SDF as well as IP-CAN Bearer level.

The decision to generate a PGWCDR or not will be taken by the CP function, hence there is a need to let UP function know if the SDF level usage need to be deducted from Bearer-level URR.

•	Add the optional ‘Update URR IE’ field in ‘Sx Session Report Response’.

•	Add optional IE field ‘Adjust Voume Measurement’ inside ‘Update URR IE’ to indicate the number of octets to be adjusted(subtracted) from the URR accumulated usage counts.

•	This will be used by CP function to facilitate UP function to update the Bearer-level URR to reflect the updated usage counts if the SDF-level URR reporting resulted in PGWCDR generation.

•	The optional IE field inside ‘Update URR IE’ may be used by CP even in case of Sx-Session-Modification request.

•	In case of no CDR generation, no update is needed.

Discussion:

Other companies did not see this as a problem in Rel-14.

Decision: 		The document was withdrawn.

C4-176065	F-TEID allocation by UP function
					29.244	  CR-0025  rev 3 Cat: F (Rel-14) v14.1.0
					Source: Nokia, Nokia Shanghai Bell, Verizon, ZTE, Ericsson, Cisco, NEC

(Replaces C4-175314)

Abstract:

The network should be configured to use F-TEID allocation in the UP function, if F-TEID allocation in the UP function is supported by the UP function and by all the CP functions controlling the UP function.

Discussion:

Huawei have a discussion paper related on topic and why Huawei thinks it's not useful to do this. See C4-176340.

Huawei commented that the advantage and disadvantage of F-TEID allocation at the CP or UP function is dependent, it shall be completely determined by the network configuration after taking into account all the criteria presented in the actual network.

F-TEID allocation in UP function is a clear option, adding any kind of recommendation which are assuming a specific implementation/design should be avoided. These kind of recommendations which are vendor specific should be given by the vendor to his customer directly but not put into the standard.

After discussion it was agreed that Rel-14 version is not needed. Rel-15 CR shall add only an informative note to clarify current situation. Some companies still have problems to agree even Rel-15 CR with informative note. Offline discussion is needed.

Decision: 		The document was revised to C4-176342.

C4-176342	F-TEID allocation by UP function
					29.244	  CR-0025  rev 4 Cat: F (Rel-14) v14.1.0
					Source: Nokia, Nokia Shanghai Bell, Verizon, ZTE, Ericsson, Cisco, NEC

(Replaces C4-176065)

Decision: 		The document was rejected.

C4-176344	F-TEID allocation by UP function
					29.244	  CR-0025  rev 5 Cat: F (Rel-15) v14.1.0
					Source: Nokia, Nokia Shanghai Bell, Verizon, ZTE, Ericsson, Cisco, NEC, AT&T

(Replaces C4-176065)

Discussion:

China Mobile commented that advantage of CP allocation should also be added in proposed note.

Huawei, Deutsche Telekom, China Mobile do not see any need for informative note even in Rel-15. This is implementation issue by operator and should not put in standards.

After discussion the CR was rejected.

Decision: 		The document was rejected.

C4-176080	F-TEID-U allocation
					Source: Huawei, Deutsche Telekom

Decision: 		The document was revised to C4-176340.

C4-176340	F-TEID-U allocation
					Source: Huawei, Deutsche Telekom, China Mobile, China Telekom

(Replaces C4-176080)

Abstract:

During CT4#80 meeting, the CR C4-175314 was discussed and postponed to next meeting. The CR lists some advantages of F-TEID allocation by UP and propose “The network should be configured to use F-TEID allocation in the UP function, if F-TEID allocation in the UP function is supported by the UP function and by all the CP functions controlling the UP function.”.

This paper intends to provide further details and discuss the F-TEID allocation by CP or UP.

Decision: 		The document was revised to C4-176343.

C4-176343	F-TEID-U allocation
					Source: Huawei, Deutsche Telekom, China Mobile, China Telekom, China Unicom

(Replaces C4-176340)

Decision: 		The document was noted.

C4-176066	Detection and Reporting of User Plane Inactivity to the CP function
					Source: Nokia, Nokia Shanghai Bell, NEC, ZTE

Abstract:

Several stage 2 requirements require the CP function to be able to request the UP function to detect and report user plane inactivity.

Two alternative approaches have been discussed during CT4#80 to achieve the above functionalities:

a)	Define a new Reporting Trigger for user plane inactivity detection and reporting, with a new Inactivity Reporting Time IE defined in the URR indicating the user plane inactivity time after which a report needs to be generated (C4-175306, Nokia, postponed to CT4#81);

b)	Reuse the existing Reporting Trigger "Quota Holding Time" and the associated Quota Holding Time value in the URR.

The decision on the approach to retain was postponed to CT4#81.

Conclusion:

It is strongly encouraged to define a new reporting trigger value and timer to support the user plane inactivity detection and reporting requirements, with a clean, generic and future proof protocol design, as opposed to messing up the protocol and interoperability by overloading the semantic and use of existing IEs designed for completely different use cases.

This has minimal impacts on the protocol and CP and UP functions' implementations.

Discussion:

This has minimal impacts on the protocol and CP and UP functions' implementations.

Decision: 		The document was noted.

C4-176067	Reporting User Plane Inactivity
					29.244	  CR-0026  rev 2 Cat: F (Rel-14) v14.1.0
					Source: Nokia, Nokia Shanghai Bell, NEC, ZTE, Verizon

(Replaces C4-175306)

Abstract:

A new Reporting Trigger and Usage Report Trigger value is defined to request reporting & to report user plane inactivity to the CP function. A new Inactivity Reporting Time IE is defined in the URR indicating the user plane inactivity time after which a report needs to be generated.

Discussion:

All the URR has to be linked in one single URR

It could be simpler to use it at the session level. Deviate a little bit the use the reporting modelling.

Decision: 		The document was revised to C4-176345.

C4-176068	Bidirectional SDF Filters
					29.244	  CR-0046  Cat: F (Rel-14) v14.1.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was revised to C4-176222.

C4-176345	Reporting User Plane Inactivity
					29.244	  CR-0026  rev 3 Cat: F (Rel-14) v14.1.0
					Source: Nokia, Nokia Shanghai Bell, NEC, ZTE, Verizon, Cisco

(Replaces C4-176067)

Decision: 		The document was revised to C4-176390.

C4-176390	Reporting User Plane Inactivity
					29.244	  CR-0026  rev 4 Cat: F (Rel-14) v14.1.0
					Source: Nokia, Nokia Shanghai Bell, NEC, ZTE, Verizon, Cisco, Ericsson

(Replaces C4-176345)

Decision: 		The document was agreed.

C4-176069	Outer Header Creation
					29.244	  CR-0047  Cat: F (Rel-14) v14.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

1/ The CP function may receive, from peer GTP-U entities, F-TEIDs containing both an IPv4 and an IPv6 address. In this case, the CP function should be able to provide both addresses to the UP function. For instance, if the PGW-C receives SGW-U F-TEIDs with both IPv4 and IPv6 addresses, the PGW-C may provide both addresses to the PGW-U such as to allow the PGW-U to use any of these addresses e.g. dependent on the IP address type it supports.

2/ Table 8.2.56-1 refers to a NOTE 3 which is not existing.

Decision: 		The document was revised to C4-176361.

C4-176361	Outer Header Creation
					29.244	  CR-0047  rev 1 Cat: F (Rel-14) v14.1.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces C4-176069)

Decision: 		The document was agreed.

C4-176081	Clarification on presence of Activate Predefined Rule Name
					29.244	  CR-0048  Cat: F (Rel-14) v14.1.0
					Source: Huawei

Abstract:

A predefined Rule may contain a set of predefined rules, in addition there could  be the requirement to activate multiple predefined rule names.

From the text is not  clear how to handle this case.

The condition on handle the case on including multiple Predefined Rule name is clarified.

Decision: 		The document was revised to C4-176359.

C4-176359	Clarification on presence of Activate Predefined Rule Name
					29.244	  CR-0048  rev 1 Cat: F (Rel-14) v14.1.0
					Source: Huawei

(Replaces C4-176081)

Decision: 		The document was agreed.

C4-176207	Correction on Load Control Information IE Type value
					29.244	  CR-0056  Cat: F (Rel-14) v14.1.0
					Source: Huawei

Abstract:

Table 7.5.3.3-1 the type value is changed to 51'

Decision: 		The document was agreed.

C4-176214	Provisioning of Subsequent Time Quota and Subsequent Volume Quota
					29.244	  CR-0058  Cat: F (Rel-14) v14.1.0
					Source: Huawei

Abstract:

According to RFC 4006: The Diameter credit-control server SHOULD include both the Tariff-Time-Change and Tariff-Change-Usage AVPs in two quota allocations in the answer message (i.e., two instances of the Multiple-Services-Credit- Control AVP). One of the granted units is allocated to be used before the potential tariff change, while the second granted units are for use after a tariff change. Both granted unit quotas MUST contain the same Service-Identifier and/or Rating-Group. This dual quota mechanism ensures that the overall reported used units would never exceed the credit reservation. The Diameter credit-control client reports both the used units before and after the tariff change in a single instance of the Multiple-Services-Credit-Control AVP.

After Tariff change only time and volume Threshold is provided Time Quota and Volume Quota is not provided.

Discussion:

Section 5.2.2 needs to be updated.

Decision: 		The document was revised to C4-176360.

C4-176360	Provisioning of Subsequent Time Quota and Subsequent Volume Quota
					29.244	  CR-0058  rev 1 Cat: F (Rel-14) v14.1.0
					Source: Huawei

(Replaces C4-176214)

Decision: 		The document was revised to C4-176407.

C4-176407	Provisioning of Subsequent Time Quota and Subsequent Volume Quota
					29.244	  CR-0058  rev 2 Cat: F (Rel-14) v14.1.0
					Source: Huawei

(Replaces C4-176360)

Decision: 		The document was agreed.

C4-176222	Bidirectional SDF Filters
					29.244	  CR-0046  rev 1 Cat: F (Rel-14) v14.1.0
					Source: Nokia, Nokia Shanghai Bell, Huawei

(Replaces C4-176068)

Abstract:

The CP function may provision bidirectional SDF Filters in the UP function:

˗	when provisioning a bidirectional SDF Filter the first time for an Sx session, the CP function shall provision the SDF filter definition together with a SDF Filter ID uniquely identifying the SDF Filter among all the SDF Filters provisioned for a given Sx Session.

˗	the CP function may then provision additional PDR(s) for the same Sx session by just provisioning the SDF Filter ID in the PDR(s), without provisioning again the SDF filter definition.

Discussion:

Ericsson agreed CT4 have to have some mechanism to reduce message size. Anyway CT4 should have more generic way to do this how to efficiently transport this information.

Decision: 		The document was postponed.

#### 7.1.6	Diameter Load control mechanisms [DLoCME]

#### 7.1.7	Support of EAP Re-authentication Protocol for WLAN Interworking [ERP-CT]

#### 7.1.8	Diameter Base Protocol Specification Update [DBPU]

### 7.2	CT4 Supported WIDs

#### 7.2.1	CT aspects for Non-IP for Cellular Internet of Things for 2G/3G-GPRS [NonIP_GPRS-CT]

#### 7.2.2	CT aspects of Support of Emergency services over WLAN – phase 2 [SEW2-CT]

C4-176010	Align emergency number FQDN and Replacement field with procedures in TS 24.302
					23.003	  CR-0483  Cat: F (Rel-14) v14.5.0
					Source: BlackBerry UK Limited

Abstract:

1)		The Visited Country Emergency Numbers FQDN format unnecessarily suggests that the home country cannot use the same mechanism to provide emergency numbers. TS 22.101 states “The serving network may download emergency call numbers to the UE in order to ensure that local emergency call numbers are known to the UE.” Neither TS 22.101 nor TS 24.302 limits this capability to roaming UEs only.

2)		No procedures have been defined in TS 24.302 for handling receipt of emergency numbers without associated emergency types. Moreover, via 3GPP accesses it is impossible to provide (during registration (updating)) emergency numbers without associated emergency types. It is proposed to remove the “unspecified emergency type” response from the grammer of the replacement field.

3)		In some circumstances, the DNS provided emergency numbers may be used over CS (TS 24.008). However, 19.4.2.9A.7 only mentions TS 24.229.

Subclause 19.4.2.9.4 contains an editorial (additionally, the new text in 19.4.2.9A.6 was inspired by 19.4.2.9.4).

Proposed changes:

1)	Remove “visited-country” label from FQDN in 19.4.2.9A.6 and 19.4.2.9A.7.

2)	Remove option to receive emergency numbers without associated emergency type(s).

3)	Remove reference to TS 24.229.

Fixed the editorial in subclause 19.4.2.9.4.

Correct style and colour of paragraph mark in 19.4.2.9A.7:

1)		The style of the mark is not a 3GPP style (see TR 21.801, Annex H.1)

2)		The colour of the mark is not set to "automatic" (see TR 21.801, Annex H.7.1)

Discussion:

Nokia agreed with the purpose of this CR, but Nokia commented that proposed UE bahavior goes beyond CT4 remit.

Decision: 		The document was revised to C4-176363.

C4-176363	Align emergency number FQDN and Replacement field with procedures in TS 24.302
					23.003	  CR-0483  rev 1 Cat: F (Rel-14) v14.5.0
					Source: BlackBerry UK Limited

(Replaces C4-176010)

Decision: 		The document was revised to C4-176405.

C4-176405	Align emergency number FQDN and Replacement field with procedures in TS 24.302
					23.003	  CR-0483  rev 2 Cat: F (Rel-14) v14.5.0
					Source: BlackBerry UK Limited

(Replaces C4-176363)

Decision: 		The document was agreed.

C4-176011	Align emergency number FQDN and Replacement field with procedures in TS 24.302
					23.003	  CR-0484  Cat: A (Rel-15) v15.1.0
					Source: BlackBerry UK Limited

Decision: 		The document was revised to C4-176364.

C4-176364	Align emergency number FQDN and Replacement field with procedures in TS 24.302
					23.003	  CR-0484  rev 1 Cat: A (Rel-15) v15.1.0
					Source: BlackBerry UK Limited

(Replaces C4-176011)

Decision: 		The document was revised to C4-176406.

C4-176406	Align emergency number FQDN and Replacement field with procedures in TS 24.302
					23.003	  CR-0484  rev 2 Cat: A (Rel-15) v15.1.0
					Source: BlackBerry UK Limited

(Replaces C4-176364)

Decision: 		The document was agreed.

#### 7.2.3	CT aspects of V2X Services [V2X-CT]

#### 7.2.4	Enhanced DÉCOR [eDECOR-CT]

#### 7.2.5	Protocol enhancements for Mission Critical Push To Talk over LTE [MCPTTProtoc1]

#### 7.2.6	CT Aspects of Improvements of Awareness of User Location Change [AULC-CT]

#### 7.2.7	CT aspects of system architecture enhancements for TV service [AE_enTV-CT]

#### 7.2.8	CT aspects of extended Architecture support for CIoT [CIoT-Ext-CT]

C4-176090	S11-U interface separation from S1-U interface
					29.274	  CR-1857  Cat: F (Rel-14) v14.5.0
					Source: China Mobile, ZTE

Abstract:

SA2 has agreed  that the SGW can separate S11-u interface from S1-u interface (see S2-177065 CR#3345 for R14, S2-177669 CR#3346 for R15). Accordingly, TS 29.274 needs to be updated to support the changes.

Decision: 		The document was revised to C4-176223.

C4-176223	S11-U interface separation from S1-U interface
					29.274	  CR-1857  rev 1 Cat: C (Rel-14) v14.5.0
					Source: China Mobile, ZTE, Ericsson, Huawei

(Replaces C4-176090)

Abstract:

SA2 has agreed  that the SGW can separate S11-u interface from S1-u interface (see S2-177065 CR#3345 for R14, S2-177669 CR#3346 for R15). Accordingly, TS 29.274 needs to be updated to support the changes.

Decision: 		The document was revised to C4-176357.

C4-176357	S11-U interface separation from S1-U interface
					29.274	  CR-1857  rev 2 Cat: C (Rel-14) v14.5.0
					Source: China Mobile, ZTE, Ericsson, Huawei, Nokia, Nokia Shanghai Bell

(Replaces C4-176223)

Decision: 		The document was agreed.

C4-176091	S11-U interface separation from S1-U interface
					29.274	  CR-1858  Cat: A (Rel-15) v15.1.0
					Source: China Mobile, ZTE

Abstract:

SA2 has agreed  that the SGW can separate S11-u interface from S1-u interface (see S2-177065 CR#3345 for R14, S2-177669 CR#3346 for R15). Accordingly, TS 29.274 needs to be updated to support the changes.

Decision: 		The document was revised to C4-176358.

C4-176358	S11-U interface separation from S1-U interface
					29.274	  CR-1858  rev 1 Cat: A (Rel-15) v15.1.0
					Source: China Mobile, ZTE, Ericsson, Huawei, Nokia, Nokia Shanghai Bel

(Replaces C4-176091)

Decision: 		The document was agreed.

C4-176188	S11-U interface separation from S1-U interface
					29.274	  CR-1869  Cat: C (Rel-14) v14.5.0
					Source: Huawei

Discussion:

Merged into C4-176223.

Decision: 		The document was merged.

C4-176189	S11-U interface separation from S1-U interface
					29.274	  CR-1870  Cat: A (Rel-14) v14.5.0
					Source: Huawei

Discussion:

Merged into C4-176358.

Decision: 		The document was merged.

#### 7.2.9	CT aspects of signalling reduction to enable light connection for LTE [LTE_LIGHT_CON-CT]

#### 7.2.10	CT aspects of Enhancements for Mission Critical Push To Talk [MCImp-eMCPTT-CT]

#### 7.2.11	CT aspects of Mission Critical Data [MCImp-MCDATA-CT]

#### 7.2.12	CT aspects of Mission Critical Video [MCImp-MCVIDEO-CT]

#### 7.2.13	7.2.13 IMS Signalling Activated Trace [ISAT]

### 7.3	Any other Business for Rel-14

#### 7.3.1	GTP and PMIP [TEI14]

#### 7.3.2	Addressing and Subscriber Data Handling (23.003 and 23.008) [TEI14]

#### 7.3.3	EPS AAA interfaces (29.273) [TEI14]

#### 7.3.4	Diameter based Interfaces (29.272, 29.173) [TEI14]

#### 7.3.5	IMS [TEI14]

#### 7.3.6	MAP [TEI14]

#### 7.3.7	H.248 Interfaces [TEI14]

#### 7.3.8	Diameter 29.230 CRs [TEI14]

#### 7.3.9	DSCP Marking based on QCI and ARP [TEI14]

#### 7.3.10	CIoT [TEI14]

#### 7.3.11	MONTE-TEI14 [TEI14]

C4-176073	Group Member Reporting from the MME
					29.336	  CR-0109  Cat: F (Rel-14) v14.3.0
					Source: Convida Wireless LLC, InterDigital Inc

Abstract:

Fixing event reporting for group member UEs.

Discussion:

Per the explanations from HP and Nokia during the online discussion, Convida Wireless agree that the stage 3 specs will work for the case where the UE has one External ID.  However, Convida Wireless think that the current specifications will not work for the case where the UE has more than one external ID.

That being said, TS 23.682 has the following text:

A subscription used for MTC has one IMSI and may have one or several External Identifier(s) that are stored in the HSS.

NOTE 1:   If several External Identifiers are mapped to one IMSI, some functions might not work in this release of the specification.

So it seems like it might not be required to work for multiple external IDs and

Convida Wireless do not feel a strong need to add support for monitoring UE’s that have multiple external ID’s, so Convida Wireless will withdraw the CR’s.

Decision: 		The document was withdrawn.

C4-176074	Group Member Reporting from the MME
					29.336	  CR-0110  Cat: A (Rel-15) v15.0.0
					Source: Convida Wireless LLC, InterDigital Inc.

Abstract:

Fixing event reporting for group member UEs

Decision: 		The document was withdrawn.

C4-176075	Final Group Reports from the HSS
					29.336	  CR-0111  Cat: C (Rel-14) v14.3.0
					Source: Convida Wireless LLC, InterDigital Inc.

Abstract:

Updated the Group-Monitoring-Event-Report AVP so that the HSS can include the External Group ID in the report to indicate to the SCEF that the event has been detected for all group members

Decision: 		The document was withdrawn.

C4-176076	Final Group Reports from the HSS
					29.336	  CR-0112  Cat: A (Rel-15) v15.0.0
					Source: Convida Wireless LLC, InterDigital Inc.

Abstract:

Updated the Group-Monitoring-Event-Report AVP so that the HSS can include the External Group ID in the report to indicate to the SCEF that the event has been detected for all group members

Decision: 		The document was withdrawn.

#### 7.3.12	AOB

C4-176009	Clarification on SGmb path failure
					23.007	  CR-0355  Cat: F (Rel-14) v14.3.0
					Source: Ericsson

Abstract:

The requirement was added during GCSE_LTE-CT work item, that upon a SGi-mb path failure, if the MBMS session is not re-established and if it was activated by a GCS AS, the BM-SC shall notify the GCS AS that the MBMS session has been deactivated. (CR312)

This should be also applicable if BMSC detects SGmb a non-transient path failure and re-establishment of MBMS Session is not possible, e.g. there is no available MBMS-GW, in such case, the BMSC will delete MBMS session and it shall also notify the GCS AS that the MBMS session has been deactivated.

Decision: 		The document was agreed.

## 8	Rel-13

### 8.1	CT4 Led WIDs

#### 8.1.1	Support of RTP Transport Multiplexing (signalling) in IMS [RTP-MUX]

#### 8.1.2	CT aspects of voice over E-UTRAN Paging Policy Differentiation [voE-UTRAN_PPD-CT]

#### 8.1.3	P-CSCF restoration enhancements with WLAN [PCSCF_RES_WLAN]

#### 8.1.4	Diameter Load control mechanisms [DLoCME]

#### 8.1.5	Study on EPC Signalling Improvement for Race Scenarios [FS_EPC_SIG_RACE]

#### 8.1.6	Shared Data for Multiple Subscribers [SHARED_SubData_UPD]

#### 8.1.7	CT aspects of EVS in 3G Circuit-Switched Networks [EVSoCS-CT]

#### 8.1.8	H.248 Aspects of WebRTC Data Channel on IMS Access Gateway

#### 8.1.9	Monitoring Enhancements CT aspects [MONTE-CT]

C4-176070	Fixing the R Bit in T6a Requests
					29.128	  CR-0063  Cat: F (Rel-13) v13.4.0
					Source: Convida Wireless LLC, InterDigital Inc.

Abstract:

Fixed it so that the R bit is set in all Diameter Requests defined in 29.128 section 6.2

Decision: 		The document was revised to C4-176324.

C4-176324	Fixing the R Bit in T6a Requests
					29.128	  CR-0063  rev 1 Cat: F (Rel-13) v13.4.0
					Source: Convida Wireless LLC, InterDigital Inc.

(Replaces C4-176070)

Decision: 		The document was agreed.

C4-176071	Fixing the R Bit in T6a Requests
					29.128	  CR-0064  Cat: A (Rel-14) v14.4.0
					Source: Convida Wireless LLC, InterDigital Inc.

Abstract:

Fixed it so that the R bit is set in all Diameter Requests defined in 29.128 section 6.2

Decision: 		The document was revised to C4-176320.

C4-176320	Fixing the R Bit in T6a Requests
					29.128	  CR-0064  rev 1 Cat: A (Rel-14) v14.4.0
					Source: Convida Wireless LLC, InterDigital Inc.

(Replaces C4-176071)

Decision: 		The document was agreed.

C4-176321	Fixing the R Bit in T6a Requests
					29.128	  CR-0064  rev 1 Cat: A (Rel-14) v14.4.0
					Source: Convida Wireless LLC, InterDigital Inc.

(Replaces C4-176071)

Decision: 		The document was withdrawn.

C4-176072	Fixing the R Bit in T6a Requests
					29.128	  CR-0065  Cat: A (Rel-15) v15.0.0
					Source: Convida Wireless LLC, InterDigital Inc.

Abstract:

Fixed it so that the R bit is set in all Diameter Requests defined in 29.128 section 6.2

Decision: 		The document was revised to C4-176319.

C4-176319	Fixing the R Bit in T6a Requests
					29.128	  CR-0065  rev 1 Cat: A (Rel-15) v15.0.0
					Source: Convida Wireless LLC, InterDigital Inc.

(Replaces C4-176072)

Decision: 		The document was agreed.

#### 8.1.10	Mobile Equipment signalling over the WLAN access [MEI_WLAN]

#### 8.1.11	Study on SCC AS Restoration [SCCAS_RES]

#### 8.1.12	SDP Capability Negotiation for IMS Media Plane [SDPCN_IMS]

#### 8.1.13	CT aspects of Optimizations to Support High Latency Communications [HLcom-CT]

#### 8.1.14	CT aspects of Group based Enhancements [GROUPE-CT]

#### 8.1.15	CT Aspects of Video Enhancements by Region-of-Interest Information Signalling [ROI-CT]

#### 8.1.16	S6a/S6d Shared Data Update [FS_eSDU]

#### 8.1.17	EPC Signalling Improvement for Race Scenarios [EPC_SIG_RACE]

#### 8.1.18	Dedicated Core Networks [DECOR-CT]

#### 8.1.19	Feasibility Study on the Diameter Base Protocol Update [FS_DBPU]

#### 8.1.20	Diameter Message Priority [DiaPri]

#### 8.1.21	Indoor Positioning Enhancements for UTRA and E-UTRA [UTRA_LTE_iPos_enh-CT]

### 8.2	CT4 Supported WIDs

#### 8.2.1	CT aspects of User Plane Congestion Management for BB I (CT3) [UPCON-DOTCON-CT]

#### 8.2.2	Enhanced P-CSCF discovery using signalling for access to EPC via WLAN (CT1) [ePCSCF_WLAN]

#### 8.2.3	CT aspects of QoS End to End MTSI extensions (CT3) [QOSE2EMTSI-CT]

#### 8.2.4	Warning Status Report in EPS (CT1) [WSR_EPS]

#### 8.2.5	IMS Signalling Activated Trace (CT1) [ISAT]

#### 8.2.6	IP Flow Mobility support for S2a and S2b Interfaces on stage 3 (CT1) [NBIFOM]

#### 8.2.7	CT aspects of enhancements to Proximity-based Services extensions (CT1) [eProSe-Ext-CT]

#### 8.2.8	CT aspects of Architecture Enhancements for Service Capability (CT3) [AESE-CT]

#### 8.2.9	Support of Emergency services over WLAN Ã¢â‚¬â€œ phase 1 (CT1) [SEW1-CT]

#### 8.2.10	Extended DRX cycle for power consumption optimization (CT1) [eDRX]

#### 8.2.11	Mission Critical Push To Talk over LTE protocol aspects (CT1) [MCPTT-CT]

#### 8.2.12	Enhancements to WEBRTC interoperability stage 3 (CT1)  [eWebRTCi_CT]

#### 8.2.13	MBMS Enhancements (CT3)  [MBMS_enh-CT]

#### 8.2.14	CIoT-CT (CT1)  [CIoT-CT]

C4-176024	ePCO reference to TS 24.301
					29.274	  CR-1849  Cat: F (Rel-13) v13.11.0
					Source: Ericsson

Abstract:

The ePCO IE reference should be updated to 3GPP TS 24.301 [23], subclause  9.9.4.26, where ePCO definition is specified.

Updating ePCO IE reference to 3GPP TS 24.301[23], subclause 9.9.4.26.

Decision: 		The document was agreed.

C4-176025	ePCO reference to TS 24.301
					29.274	  CR-1850  Cat: A (Rel-14) v14.5.0
					Source: Ericsson

Decision: 		The document was agreed.

C4-176026	ePCO reference to TS 24.301
					29.274	  CR-1851  Cat: A (Rel-15) v15.1.0
					Source: Ericsson

Decision: 		The document was agreed.

### 8.3	Any other Business for Rel-13

#### 8.3.1	GTP and PMIP [TEI13]

#### 8.3.2	Diameter 29.230 CRs [TEI13]

#### 8.3.3	EPS AAA interfaces [TEI13]

#### 8.3.4	Restoration Procedures [TEI13]

#### 8.3.5	Addressing and Subscriber Data Handling [TEI13]

#### 8.3.6	Diameter based Interfaces [TEI13]

#### 8.3.7	IMS [TEI13]

#### 8.3.8	MAP [TEI13]

#### 8.3.9	H.248 Interfaces

C4-176015	Reference update: draft-ietf-mmusic-dtls-sdp
					23.333	  CR-0120  Cat: F (Rel-13) v13.5.0
					Source: Ericsson

Abstract:

Title and the version number of draft-ietf-mmusic-dtls-sdp is updated to reflect the latest draft version.

Decision: 		The document was agreed.

C4-176016	Reference update: draft-ietf-mmusic-dtls-sdp
					23.333	  CR-0121  Cat: A (Rel-14) v14.2.0
					Source: Ericsson

Decision: 		The document was agreed.

C4-176017	Reference update: draft-ietf-mmusic-dtls-sdp
					23.334	  CR-0143  Cat: F (Rel-13) v13.8.0
					Source: Ericsson

Abstract:

Title and the version number of draft-ietf-mmusic-dtls-sdp is updated to reflect the latest draft version.

Subclause 5.20.2.2: deleted bullet 3-c in case of reception of the SDP answer from the WebRTC access network.

Decision: 		The document was agreed.

C4-176018	Reference update: draft-ietf-mmusic-dtls-sdp
					23.334	  CR-0144  Cat: A (Rel-14) v14.3.0
					Source: Ericsson

Decision: 		The document was agreed.

#### 8.3.10	I-WLAN Discontinuation [TEI13]

## 9	Release 12 and Earlier

### 9.1	GTP and PMIP [TEI8, TEI9, TEI10, TEI11,TEI12]

C4-176213	Issues faced with SGW handling of TFT and proposal
					29.274 v..
					Source: NTT DOCOMO

Abstract:

We have experienced an issue in production, where IMS voice over LTE (VoLTE) was not available due to improper SGW behaviour. The issue was with the handling of TFT at SGW, however it is not explicitly specified how SGW is to behave, and proposes to correct SGW behaviour in order to avoid failure of service.

Proposal

It shall be corrected that contents of TFT shall not be modified by SGW.

Given the nature of impact, the change shall be applied to all SGWs, and thus proposes to change from Rel-8 onwards.

Proposed CRs are provided in C4-176135 to C4-176142.

Discussion:

Principle was agreed.

Decision: 		The document was noted.

C4-176135	SGW handling of TFT
					29.274	  CR-1861  Cat: F (Rel-8) v8.11.0
					Source: NTT DOCOMO

Abstract:

Specify TFT shall be transparently passed on by SGW from PGW to UE.

Discussion:

It needs to be clarified that SGW shall not modify the content of TFT which are provided on Create Bearer Request, and Update Bearer Request. Based on DP 6213.

Could be something like:

"SGW shall set the content of the IE on the S4/S11 interface as equivalent to the content of IE received from S5/S8 interface. The SGW shall not modify the content of this IE."

The consequence if not approved needs to be expanded.

Decision: 		The document was revised to C4-176306.

C4-176306	SGW handling of TFT
					29.274	  CR-1861  rev 1 Cat: F (Rel-8) v8.11.0
					Source: NTT DOCOMO

(Replaces C4-176135)

Decision: 		The document was agreed.

C4-176136	SGW handling of TFT
					29.274	  CR-1862  Cat: A (Rel-9) v9.13.0
					Source: NTT DOCOMO

Abstract:

Specify TFT shall be transparently passed on by SGW from PGW to UE.

Decision: 		The document was revised to C4-176307.

C4-176307	SGW handling of TFT
					29.274	  CR-1862  rev 1 Cat: A (Rel-9) v9.13.0
					Source: NTT DOCOMO

(Replaces C4-176136)

Decision: 		The document was agreed.

C4-176137	SGW handling of TFT
					29.274	  CR-1863  Cat: A (Rel-10) v10.14.0
					Source: NTT DOCOMO

Abstract:

Specify TFT shall be transparently passed on by SGW from PGW to UE.

Decision: 		The document was revised to C4-176308.

C4-176308	SGW handling of TFT
					29.274	  CR-1863  rev 1 Cat: A (Rel-10) v10.14.0
					Source: NTT DOCOMO

(Replaces C4-176137)

Decision: 		The document was agreed.

C4-176138	SGW handling of TFT
					29.274	  CR-1864  Cat: A (Rel-11) v11.13.0
					Source: NTT DOCOMO

Abstract:

Specify TFT shall be transparently passed on by SGW from PGW to UE.

Decision: 		The document was revised to C4-176309.

C4-176309	SGW handling of TFT
					29.274	  CR-1864  rev 1 Cat: A (Rel-11) v11.13.0
					Source: NTT DOCOMO

(Replaces C4-176138)

Decision: 		The document was agreed.

C4-176139	SGW handling of TFT
					29.274	  CR-1865  Cat: A (Rel-12) v12.13.0
					Source: NTT DOCOMO

Abstract:

Specify TFT shall be transparently passed on by SGW from PGW to UE.

Decision: 		The document was revised to C4-176310.

C4-176310	SGW handling of TFT
					29.274	  CR-1865  rev 1 Cat: A (Rel-12) v12.13.0
					Source: NTT DOCOMO

(Replaces C4-176139)

Decision: 		The document was agreed.

C4-176140	SGW handling of TFT
					29.274	  CR-1866  Cat: A (Rel-13) v13.11.0
					Source: NTT DOCOMO

Abstract:

Specify TFT shall be transparently passed on by SGW from PGW to UE.

Decision: 		The document was revised to C4-176311.

C4-176311	SGW handling of TFT
					29.274	  CR-1866  rev 1 Cat: A (Rel-13) v13.11.0
					Source: NTT DOCOMO

(Replaces C4-176140)

Decision: 		The document was agreed.

C4-176141	SGW handling of TFT
					29.274	  CR-1867  Cat: A (Rel-14) v14.5.0
					Source: NTT DOCOMO

Abstract:

Specify TFT shall be transparently passed on by SGW from PGW to UE.

Decision: 		The document was revised to C4-176312.

C4-176312	SGW handling of TFT
					29.274	  CR-1867  rev 1 Cat: A (Rel-14) v14.5.0
					Source: NTT DOCOMO

(Replaces C4-176141)

Decision: 		The document was agreed.

C4-176142	SGW handling of TFT
					29.274	  CR-1868  Cat: A (Rel-15) v15.1.0
					Source: NTT DOCOMO

Abstract:

Specify TFT shall be transparently passed on by SGW from PGW to UE.

Decision: 		The document was revised to C4-176313.

C4-176313	SGW handling of TFT
					29.274	  CR-1868  rev 1 Cat: A (Rel-15) v15.1.0
					Source: NTT DOCOMO

(Replaces C4-176142)

Decision: 		The document was agreed.

### 9.2	Addressing and Subscriber Data Handling (23.003 and 23.008) [TEI8, TEI9, TEI10, TEI11,TEI12]

### 9.3	IMS [TEI8, TEI9, TEI10, TEI11,TEI12]

C4-176234	Outstanding 3GPP WebRTC Internet Drafts
					Source: Blackberry

Abstract:

The core RTCweb WG internet drafts are completing in IETF with many of the drafts now in the RFC editor’s queue. In fact the short RTCweb WG session at the recent IETF#100 will probably be the last RTCweb WG session at an IETF (after nearly 8 years of IETF work on RTCweb). While it may still take some time for all the RFCs to be actually published and some dependencies in other IETF WGs still require a little more work, the RTCweb baseline is now pretty stable. However, 3GPP has some additional dependencies on top of this baseline for IMS_WebRTC and eWebRTCi work items and these drafts have not had much traction in recent years primarily due to the original authors no longer participating full time in standards work.

Conclusion:

Without resources being prioritized by companies interested in completing 3GPP WebRTC, work on progressing the above drafts that are dependencies required by 3GPP will not complete in IETF. The core IETF work on RTCweb is completing and once completed it can be expected that the core IETF experts whose input will also be needed to progress these drafts will become focused elsewhere and it will become even harder to complete the outstanding 3GPP dependencies.

If resources are not forthcoming to make good progress then 3GPP will need to consider removing the functionality that depends on these drafts so that the IMS WebRTC work can be completed.

It is also time again to make updates of references to IETF drafts in 3GPP specifications to take account of the progress of recent work in IETF on RTCweb drafts many of which are in the RFC editor’s queue or have completed WGLC.

Discussion:

It has been suggested that the three existing usage-data-channel drafts (draft-ietf-mmusic-msrp-usage-data-channel, draft-schwarz-mmusic-t140-usage-data-channel, draft-schwarz-mmusic-bfcp-usage-data-channel) be consolidated into a single draft to reduce the overhead of progressing three separate drafts. It seems a new author is required to consolidate the content of these three stalled drafts into a new draft and then drive this work to completion.

Decision: 		The document was noted.

### 9.4	Diameter based Interfaces (29.272, 29.173) [TEI8, TEI9, TEI10, TEI11,TEI12]

### 9.5	EPS AAA interfaces (29.273) [TEI8, TEI9, TEI10, TEI11,TEI12]

### 9.6	MAP and MAP IWF [TEI8, TEI9, TEI10, TEI11,TEI12]

### 9.7	Diameter 29.230 CRs [TEI8, TEI9, TEI10, TEI11,TEI12]

### 9.8	Restoration Procedures (23.007) [TEI8, TEI9, TEI10, TEI11,TEI12]

### 9.9	H.248 Interfaces [TEI8, TEI9, TEI10, TEI11,TEI12]

### 9.10	Any Other Business for Rel-12 and Earlier [TEI8, TEI9, TEI10, TEI11,TEI12]

## 10	Update of the Work Plan

C4-176430	Update of the WorkPlan
					Source: CT4 Chairman

Decision: 		The document was agreed.

## 11	AoB

C4-176129	Testing 3GU
					Source: ETSI

Decision: 		The document was withdrawn.

### 11.1	Updated list of rapporteurs

## 12	Future meetings

## 13	Check of approved output documents

C4-176448	Output Document
					Source: CT4 Chairman

Decision: 		The document was agreed.

## 14	Closing of the meeting

The Chairman expressed his satisfaction with the progress of the meeting, and thanked the delegates for their usual enthusiastic participation and hard work. This was probably the last meeting of Caixia QI, the Chairman and CT4 thanked Caixia of her excellent contributions.

The Chairman wished everyone a safe journey home. The meeting was closed on Friday at 16:20.

## 15	History table

| Date | Subject/Comment | Old | New |
| --- | --- | --- | --- |
| 2017-12-02 | Version 0.0.0 provided on the 3GPP server |  | 0.0.0 |
| 2017/12-11 | Comments: Nokia (B.L.) | 0.0.0 | 0,0,1 |

Report prepared by: Kimmo Kymalainen

## Annex A: List of contribution documents

| Document | Title | Source | Decision | Replaces | Replaced by |
| --- | --- | --- | --- | --- | --- |
| C4-176000 | Draft Agenda | CT4 Chairman | noted | - | - |
| C4-176001 | Detailed agenda & time plan for CT4#81: status at document deadline | CT4 Chairman | noted | - | - |
| C4-176002 | Detailed agenda & time plan for CT4#81: status on eve of meeting | CT4 Chairman | agreed | - | - |
| C4-176003 | Proposed allocation of documents to agenda items for CT4#81: status at document deadline | CT4 Chairman | noted | - | - |
| C4-176004 | Proposed allocation of documents to agenda items for CT4#81: status on eve of meeting | CT4 Chairman | agreed | - | - |
| C4-176005 | Previous CT4 meeting report | MCC | approved | - | - |
| C4-176006 | 3GPP TS 29.502 v0.2.0 | Nokia | withdrawn | - | - |
| C4-176007 | Protocol Selection for Nudr | Nokia, Nokia Shanghai Bell | withdrawn | - | - |
| C4-176008 | Updates on the Usage Data Reporting for Dual Connectivity with NR | Ericsson, Verizon | revised | - | C4-176329 |
| C4-176009 | Clarification on SGmb path failure | Ericsson | agreed | - | - |
| C4-176010 | Align emergency number FQDN and Replacement field with procedures in TS 24.302 | BlackBerry UK Limited | revised | - | C4-176363 |
| C4-176011 | Align emergency number FQDN and Replacement field with procedures in TS 24.302 | BlackBerry UK Limited | revised | - | C4-176364 |
| C4-176012 | Setting of DSCP value for priority services on a GTPv2-C Interface | Vencore Labs, OEC, AT&T, T-Mobile USA | revised | C4-174293 | C4-176327 |
| C4-176013 | Secondary RAT related data usage reporting | Ericsson, Verizon | revised | C4-175261 | C4-176330 |
| C4-176014 | Clarification on Create PDR and Create FAR IEs in Sx Session Establishment Request | Ericsson | revised | - | C4-176341 |
| C4-176015 | Reference update: draft-ietf-mmusic-dtls-sdp | Ericsson | agreed | - | - |
| C4-176016 | Reference update: draft-ietf-mmusic-dtls-sdp | Ericsson | agreed | - | - |
| C4-176017 | Reference update: draft-ietf-mmusic-dtls-sdp | Ericsson | agreed | - | - |
| C4-176018 | Reference update: draft-ietf-mmusic-dtls-sdp | Ericsson | agreed | - | - |
| C4-176019 | Addition of new service based interfaces based on latest version of TS 23.501 | Orange | merged | - | - |
| C4-176020 | Conclusion and solution selection UDR interface protocol stack for structured data storage | Orange | merged | C4-175008 | - |
| C4-176021 | Discussion paper on resource archetype in REST | Orange | noted | - | - |
| C4-176022 | Revised WID on CT aspects of 3GPP PS data off function – Phase 2 | Ericsson | revised | - | C4-176232 |
| C4-176023 | Discussion paper on the HATEOAS REST tenet and its benefits | Orange | noted | - | - |
| C4-176024 | ePCO reference to TS 24.301 | Ericsson | agreed | - | - |
| C4-176025 | ePCO reference to TS 24.301 | Ericsson | agreed | - | - |
| C4-176026 | ePCO reference to TS 24.301 | Ericsson | agreed | - | - |
| C4-176027 | NF Service Advertisement URL | Ericsson | revised | - | C4-176252 |
| C4-176028 | HTTP Server Push | Ericsson | revised | - | C4-176231 |
| C4-176029 | N1N2MessageTransfer failure handling | Ericsson | revised | - | C4-176285 |
| C4-176030 | N1N2MessageTransfer HTTP method | Ericsson | revised | - | C4-176287 |
| C4-176031 | N1MessageNotify HTTP method | Ericsson | revised | - | C4-176290 |
| C4-176032 | N2MessageNotify HTTP Method | Ericsson | revised | - | C4-176291 |
| C4-176033 | Namf_Communication_EBIAssignment HTTP Method | Ericsson | revised | - | C4-176292 |
| C4-176034 | Namf_Communication Resource and Methods Overview | Ericsson | revised | - | C4-176293 |
| C4-176035 | Corrections to Sx session update procedures | Cisco Systems | revised | - | C4-176237 |
| C4-176036 | Maximum-Change-Conditon-Gz-Trigger support | Cisco Systems | withdrawn | - | - |
| C4-176037 | Revised WID on CT aspects on 5G System – Phase 1 | Nokia, Nokia Shanghai Bell | revised | - | C4-176229 |
| C4-176038 | Work planning on 5G System - Phase 1 - CT4 Aspects | Nokia, Nokia Shanghai Bell | noted | - | - |
| C4-176039 | TR clean up | Nokia, Nokia Shanghai Bell | revised | - | C4-176239 |
| C4-176040 | Content-type of Multipart message | Nokia, Nokia Shanghai Bell | agreed | - | - |
| C4-176041 | New GTP-U extension headers | Nokia, Nokia Shanghai Bell | revised | - | C4-176241 |
| C4-176042 | HTTP extensibility | Nokia, Nokia Shanghai Bell | withdrawn | - | - |
| C4-176043 | AMF discovery by 5G-AN | Nokia, Nokia Shanghai Bell, Verizon | agreed | - | - |
| C4-176044 | Introducing the S-NSSAI definition | Nokia, Nokia Shanghai Bell | revised | - | C4-176298 |
| C4-176045 | Extending the scope of the specification to cover the N26 interface | Nokia, Nokia Shanghai Bell | revised | - | C4-176224 |
| C4-176046 | GTP-C tunnel per UE over the N26 interface | Nokia, Nokia Shanghai Bell | revised | - | C4-176225 |
| C4-176047 | GTP-C messages over the N26 interface | Nokia, Nokia Shanghai Bell | revised | - | C4-176226 |
| C4-176048 | Forward Relocation Request over the N26 interface | Nokia, Nokia Shanghai Bell | revised | - | C4-176303 |
| C4-176049 | Forward Relocation Response over the N26 interface | Nokia, Nokia Shanghai Bell | revised | - | C4-176304 |
| C4-176050 | SMF APIs Design | Nokia, Nokia Shanghai Bell | noted | - | - |
| C4-176051 | Nsmf_PDUSession Service Description | Nokia, Nokia Shanghai Bell | agreed | - | - |
| C4-176052 | Resources and methods overview | Nokia, Nokia Shanghai Bell | revised | - | C4-176263 |
| C4-176053 | Create SM Context service operation | Nokia, Nokia Shanghai Bell | revised | - | C4-176264 |
| C4-176054 | Update SM Context service operation | Nokia, Nokia Shanghai Bell | revised | - | C4-176265 |
| C4-176055 | Release SM Context service operation | Nokia, Nokia Shanghai Bell | revised | - | C4-176266 |
| C4-176056 | Notify SM Context Status service operation | Nokia, Nokia Shanghai Bell | agreed | - | - |
| C4-176057 | Individual SM context resource | Nokia, Nokia Shanghai Bell | revised | - | C4-176268 |
| C4-176058 | SM context status notification callback resource | Nokia, Nokia Shanghai Bell | revised | - | C4-176270 |
| C4-176059 | Create service operation | Nokia, Nokia Shanghai Bell | revised | - | C4-176271 |
| C4-176060 | Update service operation | Nokia, Nokia Shanghai Bell | revised | - | C4-176273 |
| C4-176061 | Release service operation | Nokia, Nokia Shanghai Bell | agreed | - | - |
| C4-176062 | Notify Status service operation | Nokia, Nokia Shanghai Bell | agreed | - | - |
| C4-176063 | Individual PDU session (H-SMF) resource | Nokia, Nokia Shanghai Bell | revised | - | C4-176276 |
| C4-176064 | Individual PDU session (V-SMF) resource | Nokia, Nokia Shanghai Bell | revised | - | C4-176277 |
| C4-176065 | F-TEID allocation by UP function | Nokia, Nokia Shanghai Bell, Verizon, ZTE, Ericsson, Cisco, NEC | revised | C4-175314 | C4-176342 |
| C4-176066 | Detection and Reporting of User Plane Inactivity to the CP function | Nokia, Nokia Shanghai Bell, NEC, ZTE | noted | - | - |
| C4-176067 | Reporting User Plane Inactivity | Nokia, Nokia Shanghai Bell, NEC, ZTE, Verizon | revised | C4-175306 | C4-176345 |
| C4-176068 | Bidirectional SDF Filters | Nokia, Nokia Shanghai Bell | revised | - | C4-176222 |
| C4-176069 | Outer Header Creation | Nokia, Nokia Shanghai Bell | revised | - | C4-176361 |
| C4-176070 | Fixing the R Bit in T6a Requests | Convida Wireless LLC, InterDigital Inc. | revised | - | C4-176324 |
| C4-176071 | Fixing the R Bit in T6a Requests | Convida Wireless LLC, InterDigital Inc. | revised | - | C4-176320 |
| C4-176072 | Fixing the R Bit in T6a Requests | Convida Wireless LLC, InterDigital Inc. | revised | - | C4-176319 |
| C4-176073 | Group Member Reporting from the MME | Convida Wireless LLC, InterDigital Inc | withdrawn | - | - |
| C4-176074 | Group Member Reporting from the MME | Convida Wireless LLC, InterDigital Inc. | withdrawn | - | - |
| C4-176075 | Final Group Reports from the HSS | Convida Wireless LLC, InterDigital Inc. | withdrawn | - | - |
| C4-176076 | Final Group Reports from the HSS | Convida Wireless LLC, InterDigital Inc. | withdrawn | - | - |
| C4-176077 | N4 requirement to deactivate UP connection when UPF detects that the PDU Session has no data activity | NEC /Naoaki | agreed | - | - |
| C4-176078 | Revised WID on CT aspects of Northbound APIs for SCEF–SCS/AS Interworking | Huawei | endorsed | - | - |
| C4-176079 | New WID on CT aspects on enhanced VoLTE performance | Huawei | revised | - | C4-176221 |
| C4-176080 | F-TEID-U allocation | Huawei, Deutsche Telekom | revised | - | C4-176340 |
| C4-176081 | Clarification on presence of Activate Predefined Rule Name | Huawei | revised | - | C4-176359 |
| C4-176082 | Session establishment on N4 | Huawei | revised | - | C4-176314 |
| C4-176083 | Session Modification on N4 | Huawei | revised | - | C4-176315 |
| C4-176084 | Session deletion on N4 | Huawei | revised | - | C4-176316 |
| C4-176085 | Session Reporting on N4 | Huawei | revised | - | C4-176317 |
| C4-176086 | Load-overload control on N4 | Huawei | revised | - | C4-176318 |
| C4-176087 | QOS enhancements on N4 for 5G | Huawei | revised | - | C4-176322 |
| C4-176088 | Support of Ethernet frames on N4 | Huawei | postponed | - | - |
| C4-176089 | Skeleton for Nudr, Unified Data Repository for subscription data | Huawei | postponed | - | - |
| C4-176090 | S11-U interface separation from S1-U interface | China Mobile, ZTE | revised | - | C4-176223 |
| C4-176091 | S11-U interface separation from S1-U interface | China Mobile, ZTE | revised | - | C4-176358 |
| C4-176092 | Pseudo-CR on TS 29.540 subclause 4 Overview | ZTE | agreed | - | - |
| C4-176093 | Pseudo-CR on TS 29.540 subclause 5 Services | ZTE | revised | - | C4-176346 |
| C4-176094 | Pseudo-CR on TS 29.540 subclause 5, Service Operation - Activate | ZTE | revised | - | C4-176347 |
| C4-176095 | Pseudo-CR on TS 29.540 subclause 5, Service Operation - Deactivate | ZTE | revised | - | C4-176349 |
| C4-176096 | Pseudo-CR on TS 29.540 subclause 5, Service Operation - UplinkSMS | ZTE | revised | - | C4-176351 |
| C4-176097 | Pseudo-CR on TS 29.540 subclause 6.1.1 API URI | ZTE | agreed | - | - |
| C4-176098 | Pseudo-CR on TS 29.540 subclause 6.1.3 Resources | ZTE | revised | - | C4-176353 |
| C4-176099 | Pseudo-CR on TS 29.510, Clarification on N27 interface | ZTE | revised | - | C4-176278 |
| C4-176100 | SMF service operations correction | Huawei | agreed | - | - |
| C4-176101 | AMF service operations correction | Huawei | agreed | - | - |
| C4-176102 | Conclusion on Address info mapping | Huawei | revised | - | C4-176242 |
| C4-176103 | Stateless AMF without UDSF | Huawei | revised | - | C4-176356 |
| C4-176104 | Supports N3, N9 and Xn userplane interface | Huawei | revised | - | C4-176370 |
| C4-176105 | End Marker per QoS Flow | Huawei | postponed | - | C4-176371 |
| C4-176106 | Supports N26 interface | Huawei | merged | - | - |
| C4-176107 | Node name and F-TEID on N26 interface | Huawei | merged | - | - |
| C4-176108 | HTTP Standard headers | Huawei | revised | - | C4-176257 |
| C4-176109 | HTTP Error Handling | Huawei | revised | - | C4-176258 |
| C4-176110 | Serialization protocol | Huawei | revised | - | C4-176259 |
| C4-176111 | Interface Definition Language | Huawei | agreed | - | - |
| C4-176112 | Server-initiated communication | Huawei | revised | - | C4-176260 |
| C4-176113 | Resource Structure | Huawei | revised | - | C4-176246 |
| C4-176114 | Resource and HTTP Methods | Huawei | revised | - | C4-176247 |
| C4-176115 | Representing RPC as Custom Operations on Resources | Huawei | revised | - | C4-176248 |
| C4-176116 | Data Models | Huawei | revised | - | C4-176212 |
| C4-176117 | Create UE Context Description | Huawei | revised | - | C4-176375 |
| C4-176118 | Create UE Context API | Huawei | revised | - | C4-176376 |
| C4-176119 | Release UE Context Description | Huawei | revised | - | C4-176379 |
| C4-176120 | Release UE Context API | Huawei | revised | - | C4-176378 |
| C4-176121 | Data Types of Namf-MT Service | Huawei | revised | - | C4-176380 |
| C4-176122 | Equipment Identity Check description | Huawei | revised | - | C4-176283 |
| C4-176123 | Equipment Identity Check API | Huawei | revised | - | C4-176284 |
| C4-176124 | New network capability | Huawei, NTT DoCoMo | merged | - | - |
| C4-176125 | Procedure between the AMF and PCF | Huawei | agreed | - | - |
| C4-176126 | User plane management procedures correction | Huawei | agreed | - | - |
| C4-176127 | update WI 5GS_Ph1-CT for NUDR | china mobile | merged | - | - |
| C4-176128 | Using SBI between FEs and UDR | China Mobile | revised | - | C4-176244 |
| C4-176129 | Testing 3GU | ETSI | withdrawn | - | - |
| C4-176130 | Guidelines on hypermedia usage in 3GPP CT APIs | Orange | revised | - | C4-176254 |
| C4-176131 | Introducing a chapter in TS 29.571 for relation type considerations | Orange | revised | - | C4-176295 |
| C4-176132 | Introducing resource archetypes in TS 29.501 | Orange | revised | - | C4-176253 |
| C4-176133 | Replacing custom operations not associated with resource by controller resource archetype | Orange | withdrawn | - | - |
| C4-176134 | SGW/PGW selection for NR | Huawei, NTT DOCOMO, Nokia, Nokia Shanghai Bell | revised | C4-175351 | C4-176331 |
| C4-176135 | SGW handling of TFT | NTT DOCOMO | revised | - | C4-176306 |
| C4-176136 | SGW handling of TFT | NTT DOCOMO | revised | - | C4-176307 |
| C4-176137 | SGW handling of TFT | NTT DOCOMO | revised | - | C4-176308 |
| C4-176138 | SGW handling of TFT | NTT DOCOMO | revised | - | C4-176309 |
| C4-176139 | SGW handling of TFT | NTT DOCOMO | revised | - | C4-176310 |
| C4-176140 | SGW handling of TFT | NTT DOCOMO | revised | - | C4-176311 |
| C4-176141 | SGW handling of TFT | NTT DOCOMO | revised | - | C4-176312 |
| C4-176142 | SGW handling of TFT | NTT DOCOMO | revised | - | C4-176313 |
| C4-176143 | Updates for ProSe WLAN Direct Discovery | Intel | revised | - | C4-176348 |
| C4-176144 | Inclusion of WLAN direct discovery technologies as an alternative for ProSe direct discovery; Stage 3 | Intel | revised | - | C4-176217 |
| C4-176145 | draft skeleton forTS 29.XXX Unified Data Repository Services | china mobile | revised | - | C4-176397 |
| C4-176146 | scope and reference for TS 29.xxx Nudr | china mobile | postponed | - | - |
| C4-176147 | Nudr UnifiedDataManagement Service Description | china mobile | postponed | - | - |
| C4-176148 | Protocol selection for N18 | Nokia, Nokia Shanghai Bell | revised | - | C4-176368 |
| C4-176149 | Creating a resource using PUT | Nokia, Nokia Shanghai Bell | revised | - | C4-176227 |
| C4-176150 | Editorial Corrections | Nokia, Nokia Shanghai Bell | agreed | - | - |
| C4-176151 | LS on a new requirement on “Unpredictable GTP TEID”. | SA3 | postponed | C4-175194 | - |
| C4-176152 | Reply to Reply LSs on PLMN and RAT selection policies for roaming (S2-175286 and C1-173751) | SA3 | noted | C4-175208 | - |
| C4-176153 | Nudm Overview | Nokia, Nokia Shanghai Bell | agreed | - | - |
| C4-176154 | Reply LS on PLMN and RAT selection policies for roaming | CT1 | noted | - | - |
| C4-176155 | Nudm_SDM service operations | Nokia, Nokia Shanghai Bell | revised | - | C4-176300 |
| C4-176156 | Reply LS on Early Data Transmission | CT1 | noted | - | - |
| C4-176157 | LS on transparent relay between SMF in the VPLMN and SMF in the HPLMN | CT1 | noted | - | - |
| C4-176158 | Reply LS on the number of bearers | CT1 | noted | - | - |
| C4-176159 | Nudm_UEContextManagement | Nokia, Nokia Shanghai Bell | revised | - | C4-176365 |
| C4-176160 | Nudm_UEAuthentication Service | Nokia, Nokia Shanghai Bell | revised | - | C4-176367 |
| C4-176161 | LS on defining GTP extension header for 5GS Container | RAN3 | postponed | - | - |
| C4-176162 | Reply LS on supporting non-3GPP access in NGAP | SA2 | noted | - | - |
| C4-176163 | Reply LS on algorithm selection in E-UTRA-NR Dual Connectivity | SA2 | noted | - | - |
| C4-176164 | Nudm_EventExposure Service | Nokia, Nokia Shanghai Bell | revised | - | C4-176374 |
| C4-176165 | LS answer on Standardised Slice Service Types | SA2 | noted | - | - |
| C4-176166 | Reply LS on applicability of service based interface for legacy core network elements | SA2 | noted | - | - |
| C4-176167 | Reachability Type | Nokia, Nokia Shanghai Bell | revised | C4-174070 | C4-176326 |
| C4-176168 | Stateless Network Functions | Nokia, Nokia Shanghai Bell | revised | - | C4-176245 |
| C4-176169 | Discussion paper on AUSF services | Orange | noted | - | - |
| C4-176170 | AUSF Services Overview | Orange | revised | - | C4-176267 |
| C4-176171 | Introduction of the AUSF service | Orange | revised | - | C4-176269 |
| C4-176172 | Study on User-plane Protocol | SoftBank Corp. | revised | C4-175222 | C4-176235 |
| C4-176173 | AUSF Service operation: Authenticate | Orange | revised | - | C4-176272 |
| C4-176174 | Resources Overview | Orange UK | postponed | - | - |
| C4-176175 | Ressource: Authenticate | Orange | revised | - | C4-176275 |
| C4-176176 | Resource created: confirm or eap-session | Orange | postponed | - | - |
| C4-176177 | Updates for SRv6 Mobile User-Plane | Softbank | noted | C4-174204 | - |
| C4-176178 | Corrections to S6t procedures | Ericsson | revised | - | C4-176392 |
| C4-176179 | AVP code for PLMN-ID-Requested in S6t | Ericsson | agreed | - | - |
| C4-176180 | Extension of SCEF-Reference-ID | Ericsson | noted | - | - |
| C4-176181 | PLMN-ID-Requested in Monitoring-Event-Configuration | Ericsson | withdrawn | - | - |
| C4-176182 | HTTP PATCH format evaluation | Ericsson | revised | - | C4-176228 |
| C4-176183 | PATCH format | Ericsson | postponed | - | - |
| C4-176184 | NRF Skeleton update | Ericsson | agreed | - | - |
| C4-176185 | NRF Services | Ericsson | revised | - | C4-176280 |
| C4-176186 | NFManagement Resources | Ericsson | revised | - | C4-176281 |
| C4-176187 | NFDiscovery Resources | Ericsson | revised | - | C4-176282 |
| C4-176188 | S11-U interface separation from S1-U interface | Huawei | merged | - | - |
| C4-176189 | S11-U interface separation from S1-U interface | Huawei | merged | - | - |
| C4-176190 | Access restriction to unlicensed spectrum as secondary RAT | Qualcomm Incorporated / Lena | revised | - | C4-176336 |
| C4-176191 | Access restriction to unlicensed spectrum as secondary RAT | Qualcomm Incorporated / Lena | revised | - | C4-176337 |
| C4-176192 | Access restriction to unlicensed spectrum as secondary RAT | Qualcomm Incorporated / Lena | agreed | - | - |
| C4-176193 | Access restriction to unlicensed spectrum as secondary RAT | Qualcomm Incorporated / Lena | revised | - | C4-176338 |
| C4-176194 | Data usage reporting with unlicensed spectrum used as secondary RAT | Qualcomm Incorporated / Lena | revised | - | C4-176339 |
| C4-176195 | Revised WID on EPC enhancements to support 5G New Radio via Dual Connectivity, CT aspects | Ericsson | revised | - | C4-176233 |
| C4-176196 | HTTP Patch | Nokia, Nokia Shanghai Bell | postponed | - | - |
| C4-176197 | POST response body | Nokia, Nokia Shanghai Bell | revised | - | C4-176251 |
| C4-176198 | HTTP PATCH | Nokia, Nokia Shanghai Bell | postponed | - | - |
| C4-176199 | HTTP PATCH | Nokia, Nokia Shanghai Bell | postponed | - | - |
| C4-176200 | Supported Feature Negotiation | Nokia, Nokia Shanghai Bell | revised | - | C4-176261 |
| C4-176201 | Supported Feature Negotiation | Nokia, Nokia Shanghai Bell | revised | - | C4-176262 |
| C4-176202 | Supported Feature Negotiation | Nokia, Nokia Shanghai Bell | revised | - | C4-176372 |
| C4-176203 | Common Data Types suggested by CT3 | Nokia, Nokia Shanghai Bell | revised | - | C4-176294 |
| C4-176204 | IMEI based SIP URI for P-Preferred-Identity | Qualcomm Incorporated | revised | - | C4-176220 |
| C4-176205 | Storage of PC5-tech for WLAN based ProSe Direct Discovery | Intel | revised | - | C4-176350 |
| C4-176206 | Support of Idle Mode Mobility procedures for single-registration mode | Huawei | revised | - | C4-176305 |
| C4-176207 | Correction on Load Control Information IE Type value | Huawei | agreed | - | - |
| C4-176208 | Forward Relocation Complete Notification / Acknowledge over the N26 interface | Nokia, Nokia Shanghai Bell | agreed | - | - |
| C4-176209 | Relocation Cancel Request / Response over the N26 interface | Nokia, Nokia Shanghai Bell | agreed | - | - |
| C4-176210 | Packet Detection Information | Nokia, Nokia Shanghai Bell | postponed | - | - |
| C4-176211 | Supported Feature Negotiation | Nokia, Nokia Shanghai Bell | revised | - | C4-176296 |
| C4-176212 | Data Models | Huawei Technologies | revised | C4-176116 | C4-176249 |
| C4-176213 | Issues faced with SGW handling of TFT and proposal | NTT DOCOMO | noted | - | - |
| C4-176214 | Provisioning of Subsequent Time Quota and Subsequent Volume Quota | Huawei | revised | - | C4-176360 |
| C4-176215 | Pseudo-CR on definition of GSPI | Huawei | merged | - | - |
| C4-176216 | Pseudo-CR on definition of SUPI | Huawei | merged | - | - |
| C4-176217 | Inclusion of WLAN direct discovery technologies as an alternative for ProSe direct discovery; Stage 3 | Intel | revised | C4-176144 | C4-176352 |
| C4-176218 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | revised | - | C4-176332 |
| C4-176219 | Defining an hypermedia format for 3GPP API | Orange | revised | - | C4-176297 |
| C4-176220 | IMEI based SIP URI for P-Preferred-Identity | Qualcomm Incorporated | revised | C4-176204 | C4-176301 |
| C4-176221 | New WID on CT aspects on enhanced VoLTE performance | Huawei | revised | C4-176079 | C4-176236 |
| C4-176222 | Bidirectional SDF Filters | Nokia, Nokia Shanghai Bell, Huawei | postponed | C4-176068 | - |
| C4-176223 | S11-U interface separation from S1-U interface | China Mobile, ZTE, Ericsson, Huawei | revised | C4-176090 | C4-176357 |
| C4-176224 | Extending the scope of the specification to cover the N26 interface | Nokia, Nokia Shanghai Bell, Huawei | agreed | C4-176045 | - |
| C4-176225 | GTP-C tunnel per UE over the N26 interface | Nokia, Nokia Shanghai Bell, Huawei | agreed | C4-176046 | - |
| C4-176226 | GTP-C messages over the N26 interface | Nokia, Nokia Shanghai Bell, Huawei | agreed | C4-176047 | - |
| C4-176227 | Creating a resource using PUT | Nokia, Nokia Shanghai Bell | revised | C4-176149 | C4-176250 |
| C4-176228 | HTTP PATCH format evaluation | Ericsson | noted | C4-176182 | - |
| C4-176229 | Revised WID on CT aspects on 5G System – Phase 1 | Nokia, Nokia Shanghai Bell | revised | C4-176037 | C4-176399 |
| C4-176230 | LS on interworking between networks with different 5G deployments options in a roaming scenario | GSMA NG SIGNAL | noted | - | - |
| C4-176231 | HTTP Server Push | Ericsson | revised | C4-176028 | C4-176255 |
| C4-176232 | Revised WID on CT aspects of 3GPP PS data off function – Phase 2 | Ericsson, Huawei | endorsed | C4-176022 | - |
| C4-176233 | Revised WID on EPC enhancements to support 5G New Radio via Dual Connectivity, CT aspects | Ericsson | revised | C4-176195 | C4-176286 |
| C4-176234 | Outstanding 3GPP WebRTC Internet Drafts | Blackberry | noted | - | - |
| C4-176235 | Study on User-plane Protocol | SoftBank Corp. | revised | C4-176172 | C4-176366 |
| C4-176236 | New WID on CT aspects on enhanced VoLTE performance | Huawei | endorsed | C4-176221 | - |
| C4-176237 | Corrections to Sx session update procedures | Cisco Systems, Huawei | withdrawn | C4-176035 | - |
| C4-176238 | Reply LS on PLMN and RAT selection policies for roaming | CT4 | revised | - | C4-176362 |
| C4-176239 | TR clean up | Nokia, Nokia Shanghai Bell | agreed | C4-176039 | - |
| C4-176240 | NRF service subscription and notification | Nokia, Nokia Shanghai Bell | agreed | - | - |
| C4-176241 | New GTP-U extension headers | Nokia, Nokia Shanghai Bell | withdrawn | C4-176041 | - |
| C4-176242 | Conclusion on Address info mapping | Huawei | revised | C4-176102 | C4-176369 |
| C4-176243 | HTTP PATCH Encoding | Nokia, Nokia Shanghai Bell | noted | - | - |
| C4-176244 | Using SBI between FEs and UDR | China Mobile, Orange, Deutche Telekom, Huawei | revised | C4-176128 | C4-176381 |
| C4-176245 | Stateless Network Functions | Nokia, Nokia Shanghai Bell | agreed | C4-176168 | - |
| C4-176246 | Resource Structure | Huawei | revised | C4-176113 | C4-176415 |
| C4-176247 | Resource and HTTP Methods | Huawei | revised | C4-176114 | C4-176416 |
| C4-176248 | Representing RPC as Custom Operations on Resources | Huawei | revised | C4-176115 | C4-176417 |
| C4-176249 | Data Models | Huawei Technologies | revised | C4-176212 | C4-176418 |
| C4-176250 | Creating a resource using PUT | Nokia, Nokia Shanghai Bell | agreed | C4-176227 | - |
| C4-176251 | POST response body | Nokia, Nokia Shanghai Bell | revised | C4-176197 | C4-176419 |
| C4-176252 | NF Service Advertisement URL | Ericsson | revised | C4-176027 | C4-176401 |
| C4-176253 | Introducing resource archetypes in TS 29.501 | Orange, Nokia, Nokia Shanghai Bell, Ericsson | revised | C4-176132 | C4-176414 |
| C4-176254 | Guidelines on hypermedia usage in 3GPP CT APIs | Orange | revised | C4-176130 | C4-176447 |
| C4-176255 | HTTP Server Push | Ericsson | revised | C4-176231 | C4-176402 |
| C4-176256 | HTTP Server Push | Ericsson | revised | - | C4-176389 |
| C4-176257 | HTTP Standard headers | Huawei | revised | C4-176108 | C4-176409 |
| C4-176258 | HTTP Error Handling | Huawei | agreed | C4-176109 | - |
| C4-176259 | Serialization protocol | Huawei | agreed | C4-176110 | - |
| C4-176260 | Server-initiated communication | Huawei | revised | C4-176112 | C4-176410 |
| C4-176261 | Supported Feature Negotiation | Nokia, Nokia Shanghai Bell | agreed | C4-176200 | - |
| C4-176262 | Supported Feature Negotiation | Nokia, Nokia Shanghai Bell | revised | C4-176201 | C4-176411 |
| C4-176263 | Resources and methods overview | Nokia, Nokia Shanghai Bell | agreed | C4-176052 | - |
| C4-176264 | Create SM Context service operation | Nokia, Nokia Shanghai Bell | agreed | C4-176053 | - |
| C4-176265 | Update SM Context service operation | Nokia, Nokia Shanghai Bell | agreed | C4-176054 | - |
| C4-176266 | Release SM Context service operation | Nokia, Nokia Shanghai Bell | agreed | C4-176055 | - |
| C4-176267 | AUSF Services Overview | Orange | agreed | C4-176170 | - |
| C4-176268 | Individual SM context resource | Nokia, Nokia Shanghai Bell | agreed | C4-176057 | - |
| C4-176269 | Introduction of the AUSF service | Orange | agreed | C4-176171 | - |
| C4-176270 | SM context status notification callback resource | Nokia, Nokia Shanghai Bell | agreed | C4-176058 | - |
| C4-176271 | Create service operation | Nokia, Nokia Shanghai Bell | agreed | C4-176059 | - |
| C4-176272 | AUSF Service operation: Authenticate | Orange | revised | C4-176173 | C4-176426 |
| C4-176273 | Update service operation | Nokia, Nokia Shanghai Bell | agreed | C4-176060 | - |
| C4-176274 | Notify Status service operation | Nokia, Nokia Shanghai Bell | withdrawn | C4-176062 | - |
| C4-176275 | Ressource: Authenticate | Orange | revised | C4-176175 | C4-176427 |
| C4-176276 | Individual PDU session (H-SMF) resource | Nokia, Nokia Shanghai Bell | agreed | C4-176063 | - |
| C4-176277 | Individual PDU session (V-SMF) resource | Nokia, Nokia Shanghai Bell | agreed | C4-176064 | - |
| C4-176278 | Pseudo-CR on TS 29.510, Clarification on N27 interface | ZTE | agreed | C4-176099 | - |
| C4-176279 | Addition of UE additional security capabilities | Qualcomm | revised | - | C4-176328 |
| C4-176280 | NRF Services | Ericsson | agreed | C4-176185 | - |
| C4-176281 | NFManagement Resources | Ericsson | agreed | C4-176186 | - |
| C4-176282 | NFDiscovery Resources | Ericsson | agreed | C4-176187 | - |
| C4-176283 | Equipment Identity Check description | Huawei | revised | C4-176122 | C4-176428 |
| C4-176284 | Equipment Identity Check API | Huawei | revised | C4-176123 | C4-176429 |
| C4-176285 | N1N2MessageTransfer failure handling | Ericsson | agreed | C4-176029 | - |
| C4-176286 | Revised WID on EPC enhancements to support 5G New Radio via Dual Connectivity, CT aspects | Ericsson | agreed | C4-176233 | - |
| C4-176287 | N1N2MessageTransfer HTTP method | Ericsson | revised | C4-176030 | C4-176403 |
| C4-176288 | Reply LS on algorithm selection in E-UTRA-NR Dual Connectivity | RAN2 | noted | - | - |
| C4-176289 | USE of User ID in URI | CT4 | revised | - | C4-176395 |
| C4-176290 | N1MessageNotify HTTP method | Ericsson | agreed | C4-176031 | - |
| C4-176291 | N2MessageNotify HTTP Method | Ericsson | agreed | C4-176032 | - |
| C4-176292 | Namf_Communication_EBIAssignment HTTP Method | Ericsson | agreed | C4-176033 | - |
| C4-176293 | Namf_Communication Resource and Methods Overview | Ericsson | agreed | C4-176034 | - |
| C4-176294 | Common Data Types suggested by CT3 | Nokia, Nokia Shanghai Bell, Huawei | revised | C4-176203 | C4-176431 |
| C4-176295 | Introducing a chapter in TS 29.571 for relation type considerations | Orange | merged | C4-176131 | - |
| C4-176296 | Supported Feature Negotiation | Nokia, Nokia Shanghai Bell | postponed | C4-176211 | - |
| C4-176297 | Defining an hypermedia format for 3GPP API | Orange | merged | C4-176219 | - |
| C4-176298 | Introducing the S-NSSAI definition | Nokia, Nokia Shanghai Bell | agreed | C4-176044 | - |
| C4-176299 | Reply LS answer on Standardised Slice Service Types | CT4 | approved | - | - |
| C4-176300 | Nudm_SDM service operations | Nokia, Nokia Shanghai Bell | revised | C4-176155 | C4-176423 |
| C4-176301 | IMEI based SIP URI for P-Preferred-Identity | Qualcomm Incorporated | agreed | C4-176220 | - |
| C4-176302 | LS on EDCE5 Algorithm Indication between UE and SgNB | SA3 | noted | - | - |
| C4-176303 | Forward Relocation Request over the N26 interface | Nokia, Nokia Shanghai Bell | agreed | C4-176048 | - |
| C4-176304 | Forward Relocation Response over the N26 interface | Nokia, Nokia Shanghai Bell | agreed | C4-176049 | - |
| C4-176305 | Support of Idle Mode Mobility procedures for single-registration mode | Huawei | revised | C4-176206 | C4-176443 |
| C4-176306 | SGW handling of TFT | NTT DOCOMO | agreed | C4-176135 | - |
| C4-176307 | SGW handling of TFT | NTT DOCOMO | agreed | C4-176136 | - |
| C4-176308 | SGW handling of TFT | NTT DOCOMO | agreed | C4-176137 | - |
| C4-176309 | SGW handling of TFT | NTT DOCOMO | agreed | C4-176138 | - |
| C4-176310 | SGW handling of TFT | NTT DOCOMO | agreed | C4-176139 | - |
| C4-176311 | SGW handling of TFT | NTT DOCOMO | agreed | C4-176140 | - |
| C4-176312 | SGW handling of TFT | NTT DOCOMO | agreed | C4-176141 | - |
| C4-176313 | SGW handling of TFT | NTT DOCOMO | agreed | C4-176142 | - |
| C4-176314 | Session establishment on N4 | Huawei | revised | C4-176082 | C4-176444 |
| C4-176315 | Session Modification on N4 | Huawei | revised | C4-176083 | C4-176445 |
| C4-176316 | Session deletion on N4 | Huawei | agreed | C4-176084 | - |
| C4-176317 | Session Reporting on N4 | Huawei | revised | C4-176085 | C4-176420 |
| C4-176318 | Load-overload control on N4 | Huawei | revised | C4-176086 | C4-176421 |
| C4-176319 | Fixing the R Bit in T6a Requests | Convida Wireless LLC, InterDigital Inc. | agreed | C4-176072 | - |
| C4-176320 | Fixing the R Bit in T6a Requests | Convida Wireless LLC, InterDigital Inc. | agreed | C4-176071 | - |
| C4-176321 | Fixing the R Bit in T6a Requests | Convida Wireless LLC, InterDigital Inc. | withdrawn | C4-176071 | - |
| C4-176322 | QOS enhancements on N4 for 5G | Huawei | revised | C4-176087 | C4-176446 |
| C4-176323 | Reachability Type | Nokia, Nokia Shanghai Bell | agreed | - | - |
| C4-176324 | Fixing the R Bit in T6a Requests | Convida Wireless LLC, InterDigital Inc. | agreed | C4-176070 | - |
| C4-176325 | Reachability Type | Nokia, Nokia Shanghai Bell | agreed | - | - |
| C4-176326 | Reachability Type | Nokia, Nokia Shanghai Bell | agreed | C4-176167 | - |
| C4-176327 | Setting of DSCP value for priority services on a GTPv2-C Interface | Vencore Labs, OEC, AT&T, T-Mobile USA | revised | C4-176012 | C4-176382 |
| C4-176328 | Addition of UE additional security capabilities | Qualcomm | agreed | C4-176279 | - |
| C4-176329 | Updates on the Usage Data Reporting for Dual Connectivity with NR | Ericsson, Verizon | agreed | C4-176008 | - |
| C4-176330 | Secondary RAT related data usage reporting | Ericsson, Verizon | revised | C4-176013 | C4-176393 |
| C4-176331 | SGW/PGW selection for NR | Huawei, NTT DOCOMO, Nokia, Nokia Shanghai Bell | agreed | C4-176134 | - |
| C4-176332 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | revised | C4-176218 | C4-176385 |
| C4-176333 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | revised | - | C4-176386 |
| C4-176334 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | revised | - | C4-176387 |
| C4-176335 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | withdrawn | - | - |
| C4-176336 | Access restriction to unlicensed spectrum as secondary RAT | Qualcomm Incorporated / Lena | agreed | C4-176190 | - |
| C4-176337 | Access restriction to unlicensed spectrum as secondary RAT | Qualcomm Incorporated / Lena | agreed | C4-176191 | - |
| C4-176338 | Access restriction to unlicensed spectrum as secondary RAT | Qualcomm Incorporated / Lena | postponed | C4-176193 | - |
| C4-176339 | Data usage reporting with unlicensed spectrum used as secondary RAT | Qualcomm Incorporated / Lena | withdrawn | C4-176194 | - |
| C4-176340 | F-TEID-U allocation | Huawei, Deutsche Telekom, China Mobile, China Telekom | revised | C4-176080 | C4-176343 |
| C4-176341 | Clarification on Create PDR and Create FAR IEs in Sx Session Establishment Request | Ericsson | agreed | C4-176014 | - |
| C4-176342 | F-TEID allocation by UP function | Nokia, Nokia Shanghai Bell, Verizon, ZTE, Ericsson, Cisco, NEC | rejected | C4-176065 | - |
| C4-176343 | F-TEID-U allocation | Huawei, Deutsche Telekom, China Mobile, China Telekom, China Unicom | noted | C4-176340 | - |
| C4-176344 | F-TEID allocation by UP function | Nokia, Nokia Shanghai Bell, Verizon, ZTE, Ericsson, Cisco, NEC, AT&T | rejected | C4-176065 | - |
| C4-176345 | Reporting User Plane Inactivity | Nokia, Nokia Shanghai Bell, NEC, ZTE, Verizon, Cisco | revised | C4-176067 | C4-176390 |
| C4-176346 | Pseudo-CR on TS 29.540 subclause 5 Services | ZTE | agreed | C4-176093 | - |
| C4-176347 | Pseudo-CR on TS 29.540 subclause 5, Service Operation - Activate | ZTE | agreed | C4-176094 | - |
| C4-176348 | Updates for ProSe WLAN Direct Discovery | Intel | agreed | C4-176143 | - |
| C4-176349 | Pseudo-CR on TS 29.540 subclause 5, Service Operation - Deactivate | ZTE | agreed | C4-176095 | - |
| C4-176350 | Storage of PC5-tech for WLAN based ProSe Direct Discovery | Intel | agreed | C4-176205 | - |
| C4-176351 | Pseudo-CR on TS 29.540 subclause 5, Service Operation - UplinkSMS | ZTE | agreed | C4-176096 | - |
| C4-176352 | Inclusion of WLAN direct discovery technologies as an alternative for ProSe direct discovery; Stage 3 | Intel | endorsed | C4-176217 | - |
| C4-176353 | Pseudo-CR on TS 29.540 subclause 6.1.3 Resources | ZTE | agreed | C4-176098 | - |
| C4-176354 | Vivek Gupta | Intel | agreed | - | - |
| C4-176355 | Addition of UE additional security capabilities | Qualcomm | withdrawn | - | - |
| C4-176356 | Stateless AMF without UDSF | Huawei | revised | C4-176103 | C4-176394 |
| C4-176357 | S11-U interface separation from S1-U interface | China Mobile, ZTE, Ericsson, Huawei, Nokia, Nokia Shanghai Bell | agreed | C4-176223 | - |
| C4-176358 | S11-U interface separation from S1-U interface | China Mobile, ZTE, Ericsson, Huawei, Nokia, Nokia Shanghai Bel | agreed | C4-176091 | - |
| C4-176359 | Clarification on presence of Activate Predefined Rule Name | Huawei | agreed | C4-176081 | - |
| C4-176360 | Provisioning of Subsequent Time Quota and Subsequent Volume Quota | Huawei | revised | C4-176214 | C4-176407 |
| C4-176361 | Outer Header Creation | Nokia, Nokia Shanghai Bell | agreed | C4-176069 | - |
| C4-176362 | Reply LS on PLMN and RAT selection policies for roaming | CT4 | approved | C4-176238 | - |
| C4-176363 | Align emergency number FQDN and Replacement field with procedures in TS 24.302 | BlackBerry UK Limited | revised | C4-176010 | C4-176405 |
| C4-176364 | Align emergency number FQDN and Replacement field with procedures in TS 24.302 | BlackBerry UK Limited | revised | C4-176011 | C4-176406 |
| C4-176365 | Nudm_UEContextManagement | Nokia, Nokia Shanghai Bell | agreed | C4-176159 | - |
| C4-176366 | Study on User-plane Protocol | SoftBank Corp. | revised | C4-176235 | C4-176383 |
| C4-176367 | Nudm_UEAuthentication Service | Nokia, Nokia Shanghai Bell | revised | C4-176160 | C4-176424 |
| C4-176368 | Protocol selection for N18 | Nokia, Nokia Shanghai Bell | agreed | C4-176148 | - |
| C4-176369 | Conclusion on Address info mapping | Huawei | agreed | C4-176242 | - |
| C4-176370 | Supports N3, N9 and Xn userplane interface | Huawei | agreed | C4-176104 | - |
| C4-176371 | End Marker per QoS Flow | Huawei | withdrawn | C4-176105 | - |
| C4-176372 | Supported Feature Negotiation | Nokia, Nokia Shanghai Bell | agreed | C4-176202 | - |
| C4-176373 | LS on C3-176034 on PLMN and RAT selection policies for roaming | CT3 | noted | - | - |
| C4-176374 | Nudm_EventExposure Service | Nokia, Nokia Shanghai Bell | revised | C4-176164 | C4-176425 |
| C4-176375 | Create UE Context Description | Huawei | agreed | C4-176117 | - |
| C4-176376 | Create UE Context API | Huawei | agreed | C4-176118 | - |
| C4-176377 | LS reply on support of Trace and MDT in NG-RAN in rel-15 | SA5 | noted | - | - |
| C4-176378 | Release UE Context API | Huawei | agreed | C4-176120 | - |
| C4-176379 | Release UE Context Description | Huawei | agreed | C4-176119 | - |
| C4-176380 | Data Types of Namf-MT Service | Huawei | agreed | C4-176121 | - |
| C4-176381 | Using SBI between FEs and UDR | China Mobile, Orange, Deutche Telekom, Huawei | revised | C4-176244 | C4-176396 |
| C4-176382 | Setting of DSCP value for priority services on a GTPv2-C Interface | Vencore Labs, OEC, AT&T, T-Mobile USA | agreed | C4-176327 | - |
| C4-176383 | Study on User-plane Protocol | SoftBank Corp. | revised | C4-176366 | C4-176400 |
| C4-176384 | LS on extension of S6x interface for BEST | SA3 | noted | - | - |
| C4-176385 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | agreed | C4-176332 | - |
| C4-176386 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | revised | C4-176333 | C4-176408 |
| C4-176387 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | agreed | C4-176334 | - |
| C4-176388 | Reply LS on extension of S6x interface for BEST | CT4 | approved | - | - |
| C4-176389 | HTTP Server Push | Ericsson | revised | C4-176256 | C4-176404 |
| C4-176390 | Reporting User Plane Inactivity | Nokia, Nokia Shanghai Bell, NEC, ZTE, Verizon, Cisco, Ericsson | agreed | C4-176345 | - |
| C4-176391 | Reply LS on the number of bearers | SA2 | noted | - | - |
| C4-176392 | Corrections to S6t procedures | Ericsson | agreed | C4-176178 | - |
| C4-176393 | Secondary RAT related data usage reporting | Ericsson, Verizon | agreed | C4-176330 | - |
| C4-176394 | Stateless AMF without UDSF | Huawei | revised | C4-176356 | C4-176398 |
| C4-176395 | USE of User ID in URI | CT4 | approved | C4-176289 | - |
| C4-176396 | Using SBI between FEs and UDR | China Mobile, Orange, Deutche Telekom, Huawei | agreed | C4-176381 | - |
| C4-176397 | draft skeleton forTS 29.XXX Unified Data Repository Services | china mobile | postponed | C4-176145 | - |
| C4-176398 | Stateless AMF without UDSF | Huawei | agreed | C4-176394 | - |
| C4-176399 | Revised WID on CT aspects on 5G System – Phase 1 | Nokia, Nokia Shanghai Bell | endorsed | C4-176229 | - |
| C4-176400 | Study on User-plane Protocol | SoftBank Corp. | agreed | C4-176383 | - |
| C4-176401 | NF Service Advertisement URL | Ericsson | revised | C4-176252 | C4-176412 |
| C4-176402 | HTTP Server Push | Ericsson | revised | C4-176255 | C4-176413 |
| C4-176403 | N1N2MessageTransfer HTTP method | Ericsson | agreed | C4-176287 | - |
| C4-176404 | HTTP Server Push | Ericsson | revised | C4-176389 | C4-176422 |
| C4-176405 | Align emergency number FQDN and Replacement field with procedures in TS 24.302 | BlackBerry UK Limited | agreed | C4-176363 | - |
| C4-176406 | Align emergency number FQDN and Replacement field with procedures in TS 24.302 | BlackBerry UK Limited | agreed | C4-176364 | - |
| C4-176407 | Provisioning of Subsequent Time Quota and Subsequent Volume Quota | Huawei | agreed | C4-176360 | - |
| C4-176408 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | agreed | C4-176386 | - |
| C4-176409 | HTTP Standard headers | Huawei | agreed | C4-176257 | - |
| C4-176410 | Server-initiated communication | Huawei | agreed | C4-176260 | - |
| C4-176411 | Supported Feature Negotiation | Nokia, Nokia Shanghai Bell | agreed | C4-176262 | - |
| C4-176412 | NF Service Advertisement URL | Ericsson | agreed | C4-176401 | - |
| C4-176413 | HTTP Server Push | Ericsson | agreed | C4-176402 | - |
| C4-176414 | Introducing resource archetypes in TS 29.501 | Orange, Nokia, Nokia Shanghai Bell, Ericsson | agreed | C4-176253 | - |
| C4-176415 | Resource Structure | Huawei | agreed | C4-176246 | - |
| C4-176416 | Resource and HTTP Methods | Huawei | agreed | C4-176247 | - |
| C4-176417 | Representing RPC as Custom Operations on Resources | Huawei | agreed | C4-176248 | - |
| C4-176418 | Data Models | Huawei Technologies | agreed | C4-176249 | - |
| C4-176419 | POST response body | Nokia, Nokia Shanghai Bell | agreed | C4-176251 | - |
| C4-176420 | Session Reporting on N4 | Huawei | agreed | C4-176317 | - |
| C4-176421 | Load-overload control on N4 | Huawei | agreed | C4-176318 | - |
| C4-176422 | HTTP Server Push | Ericsson | agreed | C4-176404 | - |
| C4-176423 | Nudm_SDM service operations | Nokia, Nokia Shanghai Bell | agreed | C4-176300 | - |
| C4-176424 | Nudm_UEAuthentication Service | Nokia, Nokia Shanghai Bell | agreed | C4-176367 | - |
| C4-176425 | Nudm_EventExposure Service | Nokia, Nokia Shanghai Bell | agreed | C4-176374 | - |
| C4-176426 | AUSF Service operation: Authenticate | Orange | agreed | C4-176272 | - |
| C4-176427 | Ressource: Authenticate | Orange | agreed | C4-176275 | - |
| C4-176428 | Equipment Identity Check description | Huawei | agreed | C4-176283 | - |
| C4-176429 | Equipment Identity Check API | Huawei | agreed | C4-176284 | - |
| C4-176430 | Update of the WorkPlan | CT4 Chairman | agreed | - | - |
| C4-176431 | Common Data Types suggested by CT3 | Nokia, Nokia Shanghai Bell, Huawei | agreed | C4-176294 | - |
| C4-176432 | 3GPP TS 29.891 v1.2.0 | Nokia | agreed | - | - |
| C4-176433 | 3GPP TS 29.500 v0.3.0 | Huawei | agreed | - | - |
| C4-176434 | 3GPP TS 29.501 v0.3.0 | Nokia | agreed | - | - |
| C4-176435 | 3GPP TS 29.502 v0.3.0 | Nokia | agreed | - | - |
| C4-176436 | 3GPP TS 29.503 v0.3.0 | Nokia | agreed | - | - |
| C4-176437 | 3GPP TS 29.509 v0.3.0 | Orange | agreed | - | - |
| C4-176438 | 3GPP TS 29.510 v0.3.0 | Ericsson | agreed | - | - |
| C4-176439 | 3GPP TS 29.511 v0.3.0 | Deutsche Telekom | agreed | - | - |
| C4-176440 | 3GPP TS 29.518 v0.3.0 | Ericsson | agreed | - | - |
| C4-176441 | 3GPP TS 29.540 v0.3.0 | ZTE | agreed | - | - |
| C4-176442 | 3GPP TS 29.571 v0.3.0 | Huawei | agreed | - | - |
| C4-176443 | Support of Idle Mode Mobility procedures for single-registration mode | Huawei | agreed | C4-176305 | - |
| C4-176444 | Session establishment on N4 | Huawei | agreed | C4-176314 | - |
| C4-176445 | Session Modification on N4 | Huawei | agreed | C4-176315 | - |
| C4-176446 | QOS enhancements on N4 for 5G | Huawei | agreed | C4-176322 | - |
| C4-176447 | Guidelines on hypermedia usage in 3GPP CT APIs | Orange | agreed | C4-176254 | - |
| C4-176448 | Output Document | CT4 Chairman | agreed | - | - |

## Annex B: List of change requests

| Document | Title | Source | Spec | CR | Rev | Rel | Cat | WI | Decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C4-176323 | Reachability Type | Nokia, Nokia Shanghai Bell | 29.336 | 0114 | - | Rel-13 | F | TEI13 | agreed |
| C4-176325 | Reachability Type | Nokia, Nokia Shanghai Bell | 29.336 | 0115 | - | Rel-14 | A | TEI13 | agreed |
| C4-176010 | Align emergency number FQDN and Replacement field with procedures in TS 24.302 | BlackBerry UK Limited | 23.003 | 0483 | - | Rel-14 | F | SEW2-CT | revised |
| C4-176363 | Align emergency number FQDN and Replacement field with procedures in TS 24.302 | BlackBerry UK Limited | 23.003 | 0483 | 1 | Rel-14 | F | SEW2-CT | revised |
| C4-176405 | Align emergency number FQDN and Replacement field with procedures in TS 24.302 | BlackBerry UK Limited | 23.003 | 0483 | 2 | Rel-14 | F | SEW2-CT | agreed |
| C4-176011 | Align emergency number FQDN and Replacement field with procedures in TS 24.302 | BlackBerry UK Limited | 23.003 | 0484 | - | Rel-15 | A | SEW2-CT | revised |
| C4-176364 | Align emergency number FQDN and Replacement field with procedures in TS 24.302 | BlackBerry UK Limited | 23.003 | 0484 | 1 | Rel-15 | A | SEW2-CT | revised |
| C4-176406 | Align emergency number FQDN and Replacement field with procedures in TS 24.302 | BlackBerry UK Limited | 23.003 | 0484 | 2 | Rel-15 | A | SEW2-CT | agreed |
| C4-176044 | Introducing the S-NSSAI definition | Nokia, Nokia Shanghai Bell | 23.003 | 0485 | - | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176298 | Introducing the S-NSSAI definition | Nokia, Nokia Shanghai Bell | 23.003 | 0485 | 1 | Rel-15 | B | 5GS_Ph1-CT | agreed |
| C4-176204 | IMEI based SIP URI for P-Preferred-Identity | Qualcomm Incorporated | 23.003 | 0486 | - | Rel-15 | F | TEI15 | revised |
| C4-176220 | IMEI based SIP URI for P-Preferred-Identity | Qualcomm Incorporated | 23.003 | 0486 | 1 | Rel-15 | F | TEI15 | revised |
| C4-176301 | IMEI based SIP URI for P-Preferred-Identity | Qualcomm Incorporated | 23.003 | 0486 | 2 | Rel-15 | F | TEI15 | agreed |
| C4-176009 | Clarification on SGmb path failure | Ericsson | 23.007 | 0355 | - | Rel-14 | F | TEI14 | agreed |
| C4-176191 | Access restriction to unlicensed spectrum as secondary RAT | Qualcomm Incorporated / Lena | 23.008 | 0541 | - | Rel-15 | B | USOS | revised |
| C4-176337 | Access restriction to unlicensed spectrum as secondary RAT | Qualcomm Incorporated / Lena | 23.008 | 0541 | 1 | Rel-15 | B | USOS | agreed |
| C4-176205 | Storage of PC5-tech for WLAN based ProSe Direct Discovery | Intel | 23.008 | 0542 | - | Rel-15 | B | ProSe_WLAN_DD_Stage3 | revised |
| C4-176350 | Storage of PC5-tech for WLAN based ProSe Direct Discovery | Intel | 23.008 | 0542 | 1 | Rel-15 | B | ProSe_WLAN_DD_Stage3 | agreed |
| C4-176335 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | 23.008 | 0543 | - | Rel-15 | B | EDCE5-CT | withdrawn |
| C4-176015 | Reference update: draft-ietf-mmusic-dtls-sdp | Ericsson | 23.333 | 0120 | - | Rel-13 | F | TEI13 | agreed |
| C4-176016 | Reference update: draft-ietf-mmusic-dtls-sdp | Ericsson | 23.333 | 0121 | - | Rel-14 | A | TEI13 | agreed |
| C4-176017 | Reference update: draft-ietf-mmusic-dtls-sdp | Ericsson | 23.334 | 0143 | - | Rel-13 | F | TEI13 | agreed |
| C4-176018 | Reference update: draft-ietf-mmusic-dtls-sdp | Ericsson | 23.334 | 0144 | - | Rel-14 | A | TEI13 | agreed |
| C4-176193 | Access restriction to unlicensed spectrum as secondary RAT | Qualcomm Incorporated / Lena | 29.002 | 1244 | - | Rel-15 | B | USOS | revised |
| C4-176338 | Access restriction to unlicensed spectrum as secondary RAT | Qualcomm Incorporated / Lena | 29.002 | 1244 | 1 | Rel-15 | B | USOS | postponed |
| C4-176333 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | 29.060 | 1057 | - | Rel-15 | B | EDCE5-CT | revised |
| C4-176386 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | 29.060 | 1057 | 1 | Rel-15 | B | EDCE5-CT | revised |
| C4-176408 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | 29.060 | 1057 | 2 | Rel-15 | B | EDCE5-CT | agreed |
| C4-176355 | Addition of UE additional security capabilities | Qualcomm | 29.060 | 1058 | - | Rel-15 | B | EDCE5-CT | withdrawn |
| C4-176070 | Fixing the R Bit in T6a Requests | Convida Wireless LLC, InterDigital Inc. | 29.128 | 0063 | - | Rel-13 | F | MONTE-CT | revised |
| C4-176324 | Fixing the R Bit in T6a Requests | Convida Wireless LLC, InterDigital Inc. | 29.128 | 0063 | 1 | Rel-13 | F | MONTE-CT | agreed |
| C4-176071 | Fixing the R Bit in T6a Requests | Convida Wireless LLC, InterDigital Inc. | 29.128 | 0064 | - | Rel-14 | A | MONTE-CT | revised |
| C4-176320 | Fixing the R Bit in T6a Requests | Convida Wireless LLC, InterDigital Inc. | 29.128 | 0064 | 1 | Rel-14 | A | MONTE-CT | agreed |
| C4-176321 | Fixing the R Bit in T6a Requests | Convida Wireless LLC, InterDigital Inc. | 29.128 | 0064 | 1 | Rel-14 | A | MONTE-CT | withdrawn |
| C4-176072 | Fixing the R Bit in T6a Requests | Convida Wireless LLC, InterDigital Inc. | 29.128 | 0065 | - | Rel-15 | A | MONTE-CT | revised |
| C4-176319 | Fixing the R Bit in T6a Requests | Convida Wireless LLC, InterDigital Inc. | 29.128 | 0065 | 1 | Rel-15 | A | MONTE-CT | agreed |
| C4-176179 | AVP code for PLMN-ID-Requested in S6t | Ericsson | 29.230 | 0626 | - | Rel-15 | F | MONTE-CT, TEI15 | agreed |
| C4-176354 | Vivek Gupta | Intel | 29.230 | 0627 | - | Rel-15 | B | ProSe_WLA | agreed |
| C4-176065 | F-TEID allocation by UP function | Nokia, Nokia Shanghai Bell, Verizon, ZTE, Ericsson, Cisco, NEC | 29.244 | 0025 | 3 | Rel-14 | F | CUPS-CT | revised |
| C4-176342 | F-TEID allocation by UP function | Nokia, Nokia Shanghai Bell, Verizon, ZTE, Ericsson, Cisco, NEC | 29.244 | 0025 | 4 | Rel-14 | F | CUPS-CT | rejected |
| C4-176344 | F-TEID allocation by UP function | Nokia, Nokia Shanghai Bell, Verizon, ZTE, Ericsson, Cisco, NEC, AT&T | 29.244 | 0025 | 5 | Rel-15 | F | TEI15, CUPS-CT | rejected |
| C4-176067 | Reporting User Plane Inactivity | Nokia, Nokia Shanghai Bell, NEC, ZTE, Verizon | 29.244 | 0026 | 2 | Rel-14 | F | CUPS-CT | revised |
| C4-176345 | Reporting User Plane Inactivity | Nokia, Nokia Shanghai Bell, NEC, ZTE, Verizon, Cisco | 29.244 | 0026 | 3 | Rel-14 | F | CUPS-CT | revised |
| C4-176390 | Reporting User Plane Inactivity | Nokia, Nokia Shanghai Bell, NEC, ZTE, Verizon, Cisco, Ericsson | 29.244 | 0026 | 4 | Rel-14 | F | CUPS-CT | agreed |
| C4-176014 | Clarification on Create PDR and Create FAR IEs in Sx Session Establishment Request | Ericsson | 29.244 | 0043 | - | Rel-14 | F | CUPS-CT | revised |
| C4-176341 | Clarification on Create PDR and Create FAR IEs in Sx Session Establishment Request | Ericsson | 29.244 | 0043 | 1 | Rel-14 | F | CUPS-CT | agreed |
| C4-176035 | Corrections to Sx session update procedures | Cisco Systems | 29.244 | 0044 | - | Rel-14 | F | CUPS | revised |
| C4-176237 | Corrections to Sx session update procedures | Cisco Systems, Huawei | 29.244 | 0044 | 1 | Rel-14 | F | CUPS-CT | withdrawn |
| C4-176036 | Maximum-Change-Conditon-Gz-Trigger support | Cisco Systems | 29.244 | 0045 | - | Rel-14 | F | CUPS-CT | withdrawn |
| C4-176068 | Bidirectional SDF Filters | Nokia, Nokia Shanghai Bell | 29.244 | 0046 | - | Rel-14 | F | CUPS-CT | revised |
| C4-176222 | Bidirectional SDF Filters | Nokia, Nokia Shanghai Bell, Huawei | 29.244 | 0046 | 1 | Rel-14 | F | CUPS-CT | postponed |
| C4-176069 | Outer Header Creation | Nokia, Nokia Shanghai Bell | 29.244 | 0047 | - | Rel-14 | F | CUPS-CT | revised |
| C4-176361 | Outer Header Creation | Nokia, Nokia Shanghai Bell | 29.244 | 0047 | 1 | Rel-14 | F | CUPS-CT | agreed |
| C4-176081 | Clarification on presence of Activate Predefined Rule Name | Huawei | 29.244 | 0048 | - | Rel-14 | F | CUPS-CT | revised |
| C4-176359 | Clarification on presence of Activate Predefined Rule Name | Huawei | 29.244 | 0048 | 1 | Rel-14 | F | CUPS-CT | agreed |
| C4-176082 | Session establishment on N4 | Huawei | 29.244 | 0049 | - | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176314 | Session establishment on N4 | Huawei | 29.244 | 0049 | 1 | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176444 | Session establishment on N4 | Huawei | 29.244 | 0049 | 2 | Rel-15 | B | 5GS_Ph1-CT | agreed |
| C4-176083 | Session Modification on N4 | Huawei | 29.244 | 0050 | - | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176315 | Session Modification on N4 | Huawei | 29.244 | 0050 | 1 | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176445 | Session Modification on N4 | Huawei | 29.244 | 0050 | 2 | Rel-15 | B | 5GS_Ph1-CT | agreed |
| C4-176084 | Session deletion on N4 | Huawei | 29.244 | 0051 | - | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176316 | Session deletion on N4 | Huawei | 29.244 | 0051 | 1 | Rel-15 | B | 5GS_Ph1-CT | agreed |
| C4-176085 | Session Reporting on N4 | Huawei | 29.244 | 0052 | - | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176317 | Session Reporting on N4 | Huawei | 29.244 | 0052 | 1 | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176420 | Session Reporting on N4 | Huawei | 29.244 | 0052 | 2 | Rel-15 | B | 5GS_Ph1-CT | agreed |
| C4-176086 | Load-overload control on N4 | Huawei | 29.244 | 0053 | - | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176318 | Load-overload control on N4 | Huawei | 29.244 | 0053 | 1 | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176421 | Load-overload control on N4 | Huawei | 29.244 | 0053 | 2 | Rel-15 | B | 5GS_Ph1-CT | agreed |
| C4-176087 | QOS enhancements on N4 for 5G | Huawei | 29.244 | 0054 | - | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176322 | QOS enhancements on N4 for 5G | Huawei | 29.244 | 0054 | 1 | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176446 | QOS enhancements on N4 for 5G | Huawei | 29.244 | 0054 | 2 | Rel-15 | B | 5GS_Ph1-CT | agreed |
| C4-176088 | Support of Ethernet frames on N4 | Huawei | 29.244 | 0055 | - | Rel-15 | B | 5GS_Ph1-CT | postponed |
| C4-176207 | Correction on Load Control Information IE Type value | Huawei | 29.244 | 0056 | - | Rel-14 | F | CUPS-CT | agreed |
| C4-176210 | Packet Detection Information | Nokia, Nokia Shanghai Bell | 29.244 | 0057 | - | Rel-14 | F | CUPS-CT | postponed |
| C4-176214 | Provisioning of Subsequent Time Quota and Subsequent Volume Quota | Huawei | 29.244 | 0058 | - | Rel-14 | F | CUPS-CT | revised |
| C4-176360 | Provisioning of Subsequent Time Quota and Subsequent Volume Quota | Huawei | 29.244 | 0058 | 1 | Rel-14 | F | CUPS-CT | revised |
| C4-176407 | Provisioning of Subsequent Time Quota and Subsequent Volume Quota | Huawei | 29.244 | 0058 | 2 | Rel-14 | F | CUPS-CT | agreed |
| C4-176181 | PLMN-ID-Requested in Monitoring-Event-Configuration | Ericsson | 29.272 | 0745 | - | Rel-15 | F | MONTE-CT, TEI15 | withdrawn |
| C4-176190 | Access restriction to unlicensed spectrum as secondary RAT | Qualcomm Incorporated / Lena | 29.272 | 0746 | - | Rel-15 | B | USOS | revised |
| C4-176336 | Access restriction to unlicensed spectrum as secondary RAT | Qualcomm Incorporated / Lena | 29.272 | 0746 | 1 | Rel-15 | B | USOS | agreed |
| C4-176334 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | 29.272 | 0747 | - | Rel-15 | B | EDCE5-CT | revised |
| C4-176387 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | 29.272 | 0747 | 1 | Rel-15 | B | EDCE5-CT | agreed |
| C4-176012 | Setting of DSCP value for priority services on a GTPv2-C Interface | Vencore Labs, OEC, AT&T, T-Mobile USA | 29.274 | 1837 | 2 | Rel-15 | B | TEI15 | revised |
| C4-176327 | Setting of DSCP value for priority services on a GTPv2-C Interface | Vencore Labs, OEC, AT&T, T-Mobile USA | 29.274 | 1837 | 3 | Rel-15 | B | TEI15 | revised |
| C4-176382 | Setting of DSCP value for priority services on a GTPv2-C Interface | Vencore Labs, OEC, AT&T, T-Mobile USA | 29.274 | 1837 | 4 | Rel-15 | B | TEI15 | agreed |
| C4-176013 | Secondary RAT related data usage reporting | Ericsson, Verizon | 29.274 | 1845 | 2 | Rel-15 | B | EDCE5-CT | revised |
| C4-176330 | Secondary RAT related data usage reporting | Ericsson, Verizon | 29.274 | 1845 | 3 | Rel-15 | B | EDCE5-CT | revised |
| C4-176393 | Secondary RAT related data usage reporting | Ericsson, Verizon | 29.274 | 1845 | 4 | Rel-15 | B | EDCE5-CT | agreed |
| C4-176008 | Updates on the Usage Data Reporting for Dual Connectivity with NR | Ericsson, Verizon | 29.274 | 1848 | - | Rel-15 | B | EDCE5-CT | revised |
| C4-176329 | Updates on the Usage Data Reporting for Dual Connectivity with NR | Ericsson, Verizon | 29.274 | 1848 | 1 | Rel-15 | B | EDCE5-CT | agreed |
| C4-176024 | ePCO reference to TS 24.301 | Ericsson | 29.274 | 1849 | - | Rel-13 | F | CIoT-CT | agreed |
| C4-176025 | ePCO reference to TS 24.301 | Ericsson | 29.274 | 1850 | - | Rel-14 | A | CIoT-CT | agreed |
| C4-176026 | ePCO reference to TS 24.301 | Ericsson | 29.274 | 1851 | - | Rel-15 | A | CIoT-CT | agreed |
| C4-176045 | Extending the scope of the specification to cover the N26 interface | Nokia, Nokia Shanghai Bell | 29.274 | 1852 | - | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176224 | Extending the scope of the specification to cover the N26 interface | Nokia, Nokia Shanghai Bell, Huawei | 29.274 | 1852 | 1 | Rel-15 | B | 5GS_Ph1-CT | agreed |
| C4-176046 | GTP-C tunnel per UE over the N26 interface | Nokia, Nokia Shanghai Bell | 29.274 | 1853 | - | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176225 | GTP-C tunnel per UE over the N26 interface | Nokia, Nokia Shanghai Bell, Huawei | 29.274 | 1853 | 1 | Rel-15 | B | 5GS_Ph1-CT | agreed |
| C4-176047 | GTP-C messages over the N26 interface | Nokia, Nokia Shanghai Bell | 29.274 | 1854 | - | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176226 | GTP-C messages over the N26 interface | Nokia, Nokia Shanghai Bell, Huawei | 29.274 | 1854 | 1 | Rel-15 | B | 5GS_Ph1-CT | agreed |
| C4-176048 | Forward Relocation Request over the N26 interface | Nokia, Nokia Shanghai Bell | 29.274 | 1855 | - | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176303 | Forward Relocation Request over the N26 interface | Nokia, Nokia Shanghai Bell | 29.274 | 1855 | 1 | Rel-15 | B | 5GS_Ph1-CT | agreed |
| C4-176049 | Forward Relocation Response over the N26 interface | Nokia, Nokia Shanghai Bell | 29.274 | 1856 | - | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176304 | Forward Relocation Response over the N26 interface | Nokia, Nokia Shanghai Bell | 29.274 | 1856 | 1 | Rel-15 | B | 5GS_Ph1-CT | agreed |
| C4-176090 | S11-U interface separation from S1-U interface | China Mobile, ZTE | 29.274 | 1857 | - | Rel-14 | F | CIoT_Ext-CT | revised |
| C4-176223 | S11-U interface separation from S1-U interface | China Mobile, ZTE, Ericsson, Huawei | 29.274 | 1857 | 1 | Rel-14 | C | CIoT_Ext-CT | revised |
| C4-176357 | S11-U interface separation from S1-U interface | China Mobile, ZTE, Ericsson, Huawei, Nokia, Nokia Shanghai Bell | 29.274 | 1857 | 2 | Rel-14 | C | CIoT_Ext-CT | agreed |
| C4-176091 | S11-U interface separation from S1-U interface | China Mobile, ZTE | 29.274 | 1858 | - | Rel-15 | A | CIoT_Ext-CT | revised |
| C4-176358 | S11-U interface separation from S1-U interface | China Mobile, ZTE, Ericsson, Huawei, Nokia, Nokia Shanghai Bel | 29.274 | 1858 | 1 | Rel-15 | A | CIoT_Ext-CT | agreed |
| C4-176106 | Supports N26 interface | Huawei | 29.274 | 1859 | - | Rel-15 | B | 5GS_Ph1-CT | merged |
| C4-176107 | Node name and F-TEID on N26 interface | Huawei | 29.274 | 1860 | - | Rel-15 | B | 5GS_Ph1-CT | merged |
| C4-176135 | SGW handling of TFT | NTT DOCOMO | 29.274 | 1861 | - | Rel-8 | F | TEI8 | revised |
| C4-176306 | SGW handling of TFT | NTT DOCOMO | 29.274 | 1861 | 1 | Rel-8 | F | TEI8 | agreed |
| C4-176136 | SGW handling of TFT | NTT DOCOMO | 29.274 | 1862 | - | Rel-9 | A | TEI8 | revised |
| C4-176307 | SGW handling of TFT | NTT DOCOMO | 29.274 | 1862 | 1 | Rel-9 | A | TEI8 | agreed |
| C4-176137 | SGW handling of TFT | NTT DOCOMO | 29.274 | 1863 | - | Rel-10 | A | TEI8 | revised |
| C4-176308 | SGW handling of TFT | NTT DOCOMO | 29.274 | 1863 | 1 | Rel-10 | A | TEI8 | agreed |
| C4-176138 | SGW handling of TFT | NTT DOCOMO | 29.274 | 1864 | - | Rel-11 | A | TEI8 | revised |
| C4-176309 | SGW handling of TFT | NTT DOCOMO | 29.274 | 1864 | 1 | Rel-11 | A | TEI8 | agreed |
| C4-176139 | SGW handling of TFT | NTT DOCOMO | 29.274 | 1865 | - | Rel-12 | A | TEI8 | revised |
| C4-176310 | SGW handling of TFT | NTT DOCOMO | 29.274 | 1865 | 1 | Rel-12 | A | TEI8 | agreed |
| C4-176140 | SGW handling of TFT | NTT DOCOMO | 29.274 | 1866 | - | Rel-13 | A | TEI8 | revised |
| C4-176311 | SGW handling of TFT | NTT DOCOMO | 29.274 | 1866 | 1 | Rel-13 | A | TEI8 | agreed |
| C4-176141 | SGW handling of TFT | NTT DOCOMO | 29.274 | 1867 | - | Rel-14 | A | TEI8 | revised |
| C4-176312 | SGW handling of TFT | NTT DOCOMO | 29.274 | 1867 | 1 | Rel-14 | A | TEI8 | agreed |
| C4-176142 | SGW handling of TFT | NTT DOCOMO | 29.274 | 1868 | - | Rel-15 | A | TEI8 | revised |
| C4-176313 | SGW handling of TFT | NTT DOCOMO | 29.274 | 1868 | 1 | Rel-15 | A | TEI8 | agreed |
| C4-176188 | S11-U interface separation from S1-U interface | Huawei | 29.274 | 1869 | - | Rel-14 | C | TEI14, CIoT_Ext-CT | merged |
| C4-176189 | S11-U interface separation from S1-U interface | Huawei | 29.274 | 1870 | - | Rel-14 | A | TEI14, CIoT_Ext-CT | merged |
| C4-176192 | Access restriction to unlicensed spectrum as secondary RAT | Qualcomm Incorporated / Lena | 29.274 | 1871 | - | Rel-15 | B | USOS | agreed |
| C4-176194 | Data usage reporting with unlicensed spectrum used as secondary RAT | Qualcomm Incorporated / Lena | 29.274 | 1872 | - | Rel-15 | B | USOS | revised |
| C4-176339 | Data usage reporting with unlicensed spectrum used as secondary RAT | Qualcomm Incorporated / Lena | 29.274 | 1872 | 1 | Rel-15 | B | USOS | withdrawn |
| C4-176206 | Support of Idle Mode Mobility procedures for single-registration mode | Huawei | 29.274 | 1873 | - | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176305 | Support of Idle Mode Mobility procedures for single-registration mode | Huawei | 29.274 | 1873 | 1 | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176443 | Support of Idle Mode Mobility procedures for single-registration mode | Huawei | 29.274 | 1873 | 2 | Rel-15 | B | 5GS_Ph1-CT | agreed |
| C4-176208 | Forward Relocation Complete Notification / Acknowledge over the N26 interface | Nokia, Nokia Shanghai Bell | 29.274 | 1874 | - | Rel-15 | B | 5GS_Ph1-CT | agreed |
| C4-176209 | Relocation Cancel Request / Response over the N26 interface | Nokia, Nokia Shanghai Bell | 29.274 | 1875 | - | Rel-15 | B | 5GS_Ph1-CT | agreed |
| C4-176218 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | 29.274 | 1876 | - | Rel-15 | B | EDCE5-CT | revised |
| C4-176332 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | 29.274 | 1876 | 1 | Rel-15 | B | EDCE5-CT | revised |
| C4-176385 | Access Restrictions to NR as Secondary RAT on MM Context | NTT DOCOMO | 29.274 | 1876 | 2 | Rel-15 | B | EDCE5-CT | agreed |
| C4-176279 | Addition of UE additional security capabilities | Qualcomm | 29.274 | 1877 | - | Rel-15 | B | EDCE5-CT | revised |
| C4-176328 | Addition of UE additional security capabilities | Qualcomm | 29.274 | 1877 | 1 | Rel-15 | B | EDCE5-CT | agreed |
| C4-176104 | Supports N3, N9 and Xn userplane interface | Huawei | 29.281 | 0084 | - | Rel-15 | B | 5GS_Ph1-CT | revised |
| C4-176370 | Supports N3, N9 and Xn userplane interface | Huawei | 29.281 | 0084 | 1 | Rel-15 | B | 5GS_Ph1-CT | agreed |
| C4-176105 | End Marker per QoS Flow | Huawei | 29.281 | 0085 | - | Rel-15 | B | 5GS_Ph1-CT | postponed |
| C4-176371 | End Marker per QoS Flow | Huawei | 29.281 | 0085 | 1 | Rel-15 | B | 5GS_Ph1-CT | withdrawn |
| C4-176134 | SGW/PGW selection for NR | Huawei, NTT DOCOMO, Nokia, Nokia Shanghai Bell | 29.303 | 0104 | 3 | Rel-15 | B | EDCE5-CT | revised |
| C4-176331 | SGW/PGW selection for NR | Huawei, NTT DOCOMO, Nokia, Nokia Shanghai Bell | 29.303 | 0104 | 4 | Rel-15 | B | EDCE5-CT | agreed |
| C4-176124 | New network capability | Huawei, NTT DoCoMo | 29.303 | 0105 | - | Rel-15 | B | EDCE5-CT | merged |
| C4-176167 | Reachability Type | Nokia, Nokia Shanghai Bell | 29.336 | 0100 | 2 | Rel-15 | A | TEI13 | revised |
| C4-176326 | Reachability Type | Nokia, Nokia Shanghai Bell | 29.336 | 0100 | 2 | Rel-15 | F | TEI15 | agreed |
| C4-176073 | Group Member Reporting from the MME | Convida Wireless LLC, InterDigital Inc | 29.336 | 0109 | - | Rel-14 | F | GENCEF | withdrawn |
| C4-176074 | Group Member Reporting from the MME | Convida Wireless LLC, InterDigital Inc. | 29.336 | 0110 | - | Rel-15 | A | GENCEF | withdrawn |
| C4-176075 | Final Group Reports from the HSS | Convida Wireless LLC, InterDigital Inc. | 29.336 | 0111 | - | Rel-14 | C | GENCEF | withdrawn |
| C4-176076 | Final Group Reports from the HSS | Convida Wireless LLC, InterDigital Inc. | 29.336 | 0112 | - | Rel-15 | A | GENCEF | withdrawn |
| C4-176178 | Corrections to S6t procedures | Ericsson | 29.336 | 0113 | - | Rel-15 | F | MONTE-CT, TEI15 | revised |
| C4-176392 | Corrections to S6t procedures | Ericsson | 29.336 | 0113 | 1 | Rel-15 | F | MONTE-CT, TEI15 | agreed |
| C4-176143 | Updates for ProSe WLAN Direct Discovery | Intel | 29.345 | 0053 | - | Rel-15 | B | ProSe_WLAN_DD_Stage3 | revised |
| C4-176348 | Updates for ProSe WLAN Direct Discovery | Intel | 29.345 | 0053 | 1 | Rel-15 | B | ProSe_WLAN_DD_Stage3 | agreed |

## Annex C: Lists of liaisons

### C1: Incoming liaison statements

| Document | Original | Title | From | Decision | Reply in |
| --- | --- | --- | --- | --- | --- |
| C4-176151 | S3-171536 | LS on a new requirement on “Unpredictable GTP TEID”. | SA3 | postponed |  |
| C4-176152 | S3-172581 | Reply to Reply LSs on PLMN and RAT selection policies for roaming (S2-175286 and C1-173751) | SA3 | noted |  |
| C4-176154 | C1-174589 | Reply LS on PLMN and RAT selection policies for roaming | CT1 | noted |  |
| C4-176156 | C1-174595 | Reply LS on Early Data Transmission | CT1 | noted |  |
| C4-176157 | C1-174596 | LS on transparent relay between SMF in the VPLMN and SMF in the HPLMN | CT1 | noted |  |
| C4-176158 | C1-174658 | Reply LS on the number of bearers | CT1 | noted |  |
| C4-176161 | R3-174196 | LS on defining GTP extension header for 5GS Container | RAN3 | postponed |  |
| C4-176162 | S2-177840 | Reply LS on supporting non-3GPP access in NGAP | SA2 | noted |  |
| C4-176163 | S2-178182 | Reply LS on algorithm selection in E-UTRA-NR Dual Connectivity | SA2 | noted |  |
| C4-176165 | S2-178186 | LS answer on Standardised Slice Service Types | SA2 | noted |  |
| C4-176166 | S2-178205 | Reply LS on applicability of service based interface for legacy core network elements | SA2 | noted |  |
| C4-176230 | SIGNAL99 Doc 104 | LS on interworking between networks with different 5G deployments options in a roaming scenario | GSMA NG SIGNAL | noted |  |
| C4-176288 | R2-1714124 | Reply LS on algorithm selection in E-UTRA-NR Dual Connectivity | RAN2 | noted |  |
| C4-176302 | S3-173444 | LS on EDCE5 Algorithm Indication between UE and SgNB | SA3 | noted |  |
| C4-176373 | C3-176332 | LS on C3-176034 on PLMN and RAT selection policies for roaming | CT3 | noted |  |
| C4-176377 | S5-176477 | LS reply on support of Trace and MDT in NG-RAN in rel-15 | SA5 | noted |  |
| C4-176384 | S3-173461 | LS on extension of S6x interface for BEST | SA3 | noted |  |
| C4-176391 | S2-179536 | Reply LS on the number of bearers | SA2 | noted |  |

### C2: Outgoing liaison statements

| Document | Title | To | Cc | reply to i/c LS |
| --- | --- | --- | --- | --- |
| C4-176299 | Reply LS answer on Standardised Slice Service Types | SA1 | CT3 | S1-173481/S2-176877 |
| C4-176362 | Reply LS on PLMN and RAT selection policies for roaming | CT1, SA3 | CT3, SA2 | - |
| C4-176388 | Reply LS on extension of S6x interface for BEST | SA3 | - | S3-173461 |
| C4-176395 | USE of User ID in URI | SA3 | CT3 | - |

## Annex D: List of agreed/approved new and revised Work Items

| Document | Title | Source | new/revised |
| --- | --- | --- | --- |
| C4-176400 | Study on User-plane Protocol | SoftBank Corp. | SID new |
| C4-176286 | Revised WID on EPC enhancements to support 5G New Radio via Dual Connectivity, CT aspects | Ericsson | WID revised |

## Annex E: List of draft Technical Specifications and Reports

| Document | Spec | vers | Doc title |
| --- | --- | --- | --- |
| C4-176432 | 29.891 | 1.2.0 | 3GPP TS 29.891 v1.2.0 |
| C4-176433 | 29.500 | 0.3.0 | 3GPP TS 29.500 v0.3.0 |
| C4-176434 | 29.501 | 0.3.0 | 3GPP TS 29.501 v0.3.0 |
| C4-176435 | 29.502 | 0.3.0 | 3GPP TS 29.502 v0.3.0 |
| C4-176436 | 29.503 | 0.3.0 | 3GPP TS 29.503 v0.3.0 |
| C4-176437 | 29.509 | 0.3.0 | 3GPP TS 29.509 v0.3.0 |
| C4-176438 | 29.510 | 0.3.0 | 3GPP TS 29.510 v0.3.0 |
| C4-176439 | 29.511 | 0.3.0 | 3GPP TS 29.511 v0.3.0 |
| C4-176440 | 29.518 | 0.3.0 | 3GPP TS 29.518 v0.3.0 |
| C4-176441 | 29.540 | 0.3.0 | 3GPP TS 29.540 v0.3.0 |
| C4-176442 | 29.571 | 0.3.0 | 3GPP TS 29.571 v0.3.0 |

## Annex F: List of participants

| Name | Representing | Status (OP) | E-mail |
| --- | --- | --- | --- |
| Aihua LI | China Mobile M2M Company Ltd. | 3GPPMEMBER (CCSA) | liaihua@chinamobile.com |
| Aki NODA | Fujitsu Limited | 3GPPMEMBER (ETSI) | aki.noda@jp.fujitsu.com |
| Alexej KULAKOV | Vodafone Italia SpA | 3GPPMEMBER (ETSI) | alexej.kulakov@vodafone.com |
| Amandeep VIRK | Qualcomm Korea | 3GPPMEMBER (TTA) | avirk@qti.qualcomm.com |
| Amer CATOVIC | Qualcomm Europe Inc.(France) | 3GPPMEMBER (ETSI) | amerc@qti.qualcomm.com |
| Ana Lucia PINHEIRO | Intel Mobile Denmark | 3GPPMEMBER (ETSI) | ana.lucia.a.pinheiro@intel.com |
| Anatoli LEVINE | Softil Ltd | 3GPPMEMBER (ETSI) | anatoli.levine@softil.com |
| Anders ASKERUP | Hewlett-Packard Enterprise | 3GPPMEMBER (ETSI) | Anders.Askerup@hpe.com |
| Andrew ALLEN | BlackBerry Japan Limited | 3GPPMEMBER (ARIB) | aallen@blackberry.com |
| Ban AL-BAKRI | NTT DOCOMO INC. | 3GPPMEMBER (ARIB) | ban.albakri@meadowcom.com |
| Baruh HASON | Motorola Solutions UK Ltd. | 3GPPMEMBER (ETSI) | Baruh.Hason@motorolasolutions.com |
| Boris ANTSEV | T-Mobile USA Inc. | 3GPPMEMBER (ATIS) | boris.antsev@t-mobile.com |
| Bruno LANDAIS | Nokia Belgium | 3GPPMEMBER (ETSI) | bruno.landais@nokia.com |
| Changhong SHAN | Intel Finland Oy | 3GPPMEMBER (ETSI) | chang.hong.shan@intel.com |
| Chen-ho CHIN | Intel Deutschland GmbH | 3GPPMEMBER (ETSI) | chen-hox.chin@intel.com |
| Christian HERRERO-VERON | Huawei Technologies Co. Ltd. | 3GPPMEMBER (ETSI) | christian.herrero@huawei.com |
| Cristina SEIBERT | NextNav | 3GPPMEMBER (ATIS) | cseibert@nextnav.com |
| Danny MOSES | Intel Corporation (UK) Ltd | 3GPPMEMBER (ETSI) | danny.moses@intel.com |
| David HUTTON | GSM Association | 3GPPMARK_REP (OTHER) | dhutton@gsma.com |
| Dawei ZHANG | Apple France | 3GPPMEMBER (ETSI) | dawei_zhang@apple.com |
| Di(Andy) LIU | Hytera Communications Corp. | 3GPPMEMBER (CCSA) | andy.liu@hytera.com |
| Dirk KOPPLIN | HUAWEI TECH. GmbH | 3GPPMEMBER (ETSI) | dirk.kopplin@huawei.com |
| Dominic LAZARA | Motorola Solutions Danmark A/S | 3GPPMEMBER (ETSI) | Dom.Lazara@motorolasolutions.com |
| Donald LUKACS | Vencore Labs | 3GPPMEMBER (ATIS) | dlukacs@appcomsci.com |
| Ellen C. LIAO | Intel Mobile Communications | 3GPPMEMBER (TSDSI) | ellen.c.liao@intel.com |
| Farni WEAVER | SPRINT Corporation | 3GPPMEMBER (ETSI) | Farni.Weaver@sprint.com |
| Fei LU | Nubia Technology Co.,Ltd | 3GPPMEMBER (CCSA) | lu.fei1@zte.com.cn |
| Florin BABOESCU | BROADCOM CORPORATION | 3GPPMEMBER (ETSI) | florin.baboescu@broadcom.com |
| Frank KORINEK | Motorola Solutions Germany | 3GPPMEMBER (ETSI) | Frank.Korinek@motorolasolutions.com |
| Georg MAYER | HuaWei Technologies Co., Ltd | 3GPPMEMBER (CCSA) | georg.mayer.huawei@gmx.com |
| Hao ZHANG | China Mobile International Ltd | 3GPPMEMBER (CCSA) | zhanghao@chinamobile.com |
| Heiko KRUSE | Morpho Cards GmbH | 3GPPMEMBER (ETSI) | heiko.kruse@safrangroup.com |
| Hiroshi ISHIKAWA | DOCOMO Communications Lab. | 3GPPMEMBER (ETSI) | hiroshi.ishikawa.ev@nttdocomo.com |
| Ingo HAGEDORN | Vodafone Romania S.A. | 3GPPMEMBER (ETSI) | ingo.hagedorn@vodafone.com |
| Ivo SEDLACEK | Ericsson France S.A.S | 3GPPMEMBER (ETSI) | ivo.sedlacek@ericsson.com |
| Jan VAN GEEL | Proximus Plc | 3GPPMEMBER (ETSI) | jan.van.geel@proximus.com |
| Jari ARKKO | Ericsson LM | 3GPPMEMBER (ETSI) | jari.arkko@ericsson.com |
| Jay LEE | Verizon UK Ltd | 3GPPMEMBER (ETSI) | jay.lee@verizon.com |
| Jennifer LIU | Nokia Shanghai Bell | 3GPPMEMBER (CCSA) | jennifer.liu@nokia.com |
| Jesus DE GREGORIO | Ericsson LM | 3GPPMEMBER (ETSI) | jesus.de.gregorio@ericsson.com |
| Jinglei LIU | China Mobile (Hangzhou) Inf. | 3GPPMEMBER (CCSA) | liujinglei@chinamobile.com |
| Jinguo ZHU | ZTE Wistron Telecom AB | 3GPPMEMBER (ETSI) | zhu.jinguo@zte.com.cn |
| Joey CHOU | Intel K.K. | 3GPPMEMBER (ARIB) | joey.chou@intel.com |
| John-Luc BAKKER | BlackBerry UK Limited | 3GPPMEMBER (ETSI) | jbakker@blackberry.com |
| Jörgen AXELL | Nanjing Ericsson Panda Com Ltd | 3GPPMEMBER (CCSA) | Jorgen.Axell@ericsson.com |
| Julien BOURNELLE | Orange UK | 3GPPMEMBER (ETSI) | julien.bournelle@orange.com |
| Khiem TRAN | Comtech Telecommunications Cor | 3GPPMEMBER (ATIS) | khitran@telecomsys.com |
| Kimmo KYMALAINEN | ETSI | 3GPPORG_REP (ETSI) | kimmo.kymalainen@etsi.org |
| Krisztian KISS | Apple GmbH | 3GPPMEMBER (ETSI) | kkiss@apple.com |
| Laurent DUBESSET | Orange Romania | 3GPPMEMBER (ETSI) | laurent.dubesset@orange.com |
| Lena CHAPONNIERE | Qualcomm Austria RFFE GmbH | 3GPPMEMBER (ETSI) | lguellec@qti.qualcomm.com |
| Li ZHENDONG | ZTE Corporation | 3GPPMEMBER (CCSA) | li.zhendong@zte.com.cn |
| Lionel MORAND | Orange | 3GPPMEMBER (ETSI) | lionel.morand@orange.com |
| Lu YUNJIE | Ericsson Japan K.K. | 3GPPMEMBER (ARIB) | jones.lu@ericsson.com |
| Marianne MOHALI | Orange Spain | 3GPPMEMBER (ETSI) | marianne.mohali@orange.com |
| Markus SOMMER | Vodafone España SA | 3GPPMEMBER (ETSI) | markus.sommer@vodafone.com |
| Matthew GRIME | NCSC | 3GPPMEMBER (ETSI) | matthew.grime@cesg.gsi.gov.uk |
| Michael KREIPL | Telekom Deutschland GmbH | 3GPPMEMBER (ETSI) | michael.kreipl@telekom.de |
| Michael STARSINIC | Convida Wireless LLC | 3GPPMEMBER (ETSI) | Starsinic.Michael@ConvidaWireless.com |
| Michele BERIONNE | Qualcomm Incorporated | 3GPPMEMBER (ATIS) | micheleb@qti.qualcomm.com |
| Nevenka BIONDIC | Ericsson GmbH, Eurolab | 3GPPMEMBER (ETSI) | nevenka.biondic@ericsson.com |
| Niranth AMOGH | Huawei Telecommunication India | 3GPPMEMBER (TSDSI) | namogh@huawei.com |
| Nobu UCHIDA | Qualcomm Europe Inc.(France) | 3GPPMEMBER (ETSI) | nuchida@qti.qualcomm.com |
| Osama LOTFALLAH | Qualcomm Wireless GmbH | 3GPPMEMBER (ETSI) | osamal@qti.qualcomm.com |
| Peter A. WILD | Vodafone GmbH | 3GPPMEMBER (ETSI) | peter.wild@vodafone.com |
| Peter DAWES | Vodafone Group Plc | 3GPPMEMBER (ETSI) | peter.dawes@vodafone.com |
| Peter LEIS | Nokia Germany | 3GPPMEMBER (ETSI) | peter.leis@nokia.com |
| Peter SANDERS | one2many B.V. | 3GPPMEMBER (ETSI) | peter.sanders@one2many.eu |
| Peter SCHMITT | Huawei Device Co., Ltd | 3GPPMEMBER (CCSA) | peter.schmitt@huawei.com |
| Puneet JAIN | Intel | 3GPPMEMBER (ATIS) | puneet.jain@intel.com |
| Reinhard LAUSTER | T-Mobile Polska S.A. | 3GPPMEMBER (ETSI) | reinhard.lauster@t-mobile.at |
| Robert FREDERICKS | Motorola Solutions Germany | 3GPPMEMBER (ETSI) | bobfredericks@motorolasolutions.com |
| Roberto QUERIO | Telecom Italia S.p.A. | 3GPPMEMBER (ETSI) | roberto.querio@telecomitalia.it |
| Ryosuke KUREBAYASHI | DOCOMO Communications Lab. | 3GPPMEMBER (ETSI) | ryousuke.kurebayashi.pz@nttdocomo.com |
| Saso STOJANOVSKI | Intel Mobile France | 3GPPMEMBER (ETSI) | saso.stojanovski@intel.com |
| Satoru MATSUSHIMA | SoftBank Corp. | 3GPPMEMBER (ARIB) | satoru.matsushima@g.softbank.co.jp |
| Scott BAILEY | BT plc | 3GPPMEMBER (ETSI) | scott.m.bailey@bt.com |
| Shahab LAVASANI | Huawei Technologies Sweden AB | 3GPPMEMBER (ETSI) | shahab.lavasani@huawei.com |
| Sung Hwan WON | Nokia Corporation | 3GPPMEMBER (ETSI) | sung.won@nokia.com |
| Takeda SHINYA | NTT DOCOMO INC. | 3GPPMEMBER (TTC) | takedashiny@nttdocomo.com |
| Tao SUN | CMDI | 3GPPMEMBER (CCSA) | suntao@chinamobile.com |
| Tarek ABOU-ASSALI | Oracle Corporation | 3GPPMEMBER (ETSI) | tarek.abou.assali@oracle.com |
| Terri BROOKS | T-Mobile USA Inc. | 3GPPMEMBER (ATIS) | terri.brooks49@t-mobile.com |
| Thomas BELLING | Nokia Japan | 3GPPMEMBER (ARIB) | Thomas.Belling@nokia.com |
| Tsuyoshi TAKAKURA | SHARP Corporation | 3GPPMEMBER (ARIB) | takakura.tsuyoshi@sharp.co.jp |
| Ulrich WIEHE | Nokia UK | 3GPPMEMBER (ETSI) | ulrich.wiehe@nokia.com |
| Upendra PRATURI | Qualcomm Finland RFFE Oy | 3GPPMEMBER (ETSI) | upraturi@qti.qualcomm.com |
| Uwe RAUSCHENBACH | Nokia Germany | 3GPPMEMBER (ETSI) | uwe.rauschenbach@nokia.com |
| Val OPRESCU | Motorola Solutions Germany | 3GPPMEMBER (ETSI) | voprescu@motorolasolutions.com |
| Vasil ALEKSIEV | T-Mobile Austria GmbH | 3GPPMEMBER (ETSI) | vasil.aleksiev@t-mobile.at |
| Viqar SHAIKH | Vencore Labs | 3GPPMEMBER (ATIS) | vshaikh@vencorelabs.com |
| Vivek GUPTA | Intel China Ltd. | 3GPPMEMBER (CCSA) | vivek.g.gupta@intel.com |
| Wei CHEN | China Mobile E-Commerce Co. | 3GPPMEMBER (CCSA) | chenweiyj@chinamobile.com |
| Xiaojian YAN | ZTE Trunking Technology Corp. | 3GPPMEMBER (CCSA) | yan.xiaojian@zte.com.cn |
| Xingyue ZHOU | ZTE Wistron Telecom AB | 3GPPMEMBER (ETSI) | zhou.xingyue@zte.com.cn |
| Yali YAN | Huawei Tech.(UK) Co., Ltd | 3GPPMEMBER (ETSI) | yanyali@huawei.com |
| Yang LU | Vodafone Ireland Plc | 3GPPMEMBER (ETSI) | yang.lu@vodafone.com |
| Yi JIANG | China Mobile Com. Corporation | 3GPPMEMBER (CCSA) | jiangyi@chinamobile.com |
| Yizhi YAO | Intel Korea, Ltd. | 3GPPMEMBER (TTA) | yizhi.yao@intel.com |
| Yong YANG | Ericsson Japan K.K. | 3GPPMEMBER (ARIB) | frank.yong.yang@ericsson.com |
| Yoshiki HIRAMOTO | Fujitsu Limited | 3GPPMEMBER (TTC) | y.hiramoto@jp.fujitsu.com |
| Yuchul KIM | Apple Computer Trading Co. Ltd | 3GPPMEMBER (CCSA) | yuchul_kim@apple.com |
| Yudai KAWASAKI | SHARP Corporation | 3GPPMEMBER (ARIB) | kawasaki.yudai@sharp.co.jp |
| Yue SONG | China Mobile Group Device Co. | 3GPPMEMBER (CCSA) | songyue@chinamobile.com |
| Yvette KOZA | Deutsche Telekom AG | 3GPPMEMBER (ETSI) | yvette.koza@t-mobile.at |
| Zhenhua XIE | Sanechips | 3GPPMEMBER (CCSA) | xie.zhenhua@zte.com.cn |
| Zhenning HUANG | China Mobile (Suzhou) Software | 3GPPMEMBER (CCSA) | huangzhenning@chinamobile.com |
| Zhijun LI | ZTE Corporation | 3GPPMEMBER (ETSI) | li.zhijun3@zte.com.cn |