3GPP TSG-CT WG4 Meeting #93	C4-193468

# Wroclaw, Poland; 26th – 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.128 | CR | 0073 | rev | 1 | Current version: | 14.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft-ietf-dime-load published as RFC 8583 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DLoCMe |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | the IETF draft on Diameter Load Information Conveyance has been published as RFC8583. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace the reference to the I-D "draft-ietf-dime-load" by a reference to the RFC 8583 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Essential Correction. Missing normative reference, which might lead to misimplementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.4.1, C.1, C.2, C.3, C.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications ".

[3]	Void.

[4]	3GPP TS 29.229: "Cx and Dx interfaces based on the Diameter protocol; protocol details ".

[5]	3GPP TS 29.336: "Home Subscriber Server (HSS) diameter interfaces for interworking with packet data networks and applications".

[6]	3GPP TS 29.228: "IP multimedia (IM) Subsystem Cx Interface; Signalling flows and Message Elements".

[7]	IETF RFC 4960: "Stream Control Transport Protocol".

[8]	IETF RFC 5234: "Augmented BNF for Syntax Specifications: ABNF".

[9]	IETF RFC 7683: "Diameter Overload Indication Conveyance".

[10]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[11]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[12]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[13]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[14]	3GPP TS 48.018: "General Packet Radio Service (GPRS); Base Station System (BSS) - Serving GPRS Support Node (SGSN); BSS GPRS protocol (BSSGP)".

[15]	IETF RFC 7944: "Diameter Routing Message Priority".

[16]	3GPP TS 29.272: "Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[17]	3GPP TS 29.329: "Sh Interface based on the Diameter protocol; Protocol details".

[18]	Void.

[19]	3GPP TS 23.007: "Restoration procedures".

[20]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging applications".

[21]	IETF RFC 5778: "Diameter Mobile IPv6: Support for Home Agent to Diameter Server Interaction".

[22]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging applications".

[23]	3GPP TS 32.253: "Telecommunication management; Charging management; Control Plane (CP) data transfer domain charging".

[24]	3GPP TS 23.003: "Numbering, addressing and identification".

[25]	3GPP TS 23.401: "GPRS enhancements for E-UTRAN access".

[26]	3GPP TS 29.172: "Location Services (LCS); Evolved Packet Core (EPC) LCS Protocol (ELP) between the Gateway Mobile Location Centre (GMLC) and the Mobile Management Entity (MME); SLg interface".

[27]	3GPP TS 29.338: "Diameter based protocols to support SMS capable MMEs".

[28]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[29]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[30]	3GPP TS 32.298: "Telecommunication Management; Charging Management; Charging Data Record (CDR) parameter description".

[31]	IETF RFC 8583: "Diameter Load Information Conveyance".

[32]	IETF RFC 6733: "Diameter Base Protocol".

* * * Next Change * * * *

### 6.4.1	General

The following table specifies the Diameter AVPs defined for the T6a/T6b interface protocol, their AVP Code values, types, possible flag values and whether or not the AVP may be encrypted. The Vendor-ID header of all AVPs defined in this specification shall be set to 3GPP (10415).

For all AVPs which contain bit masks and are of the type Unsigned32, bit 0 shall be the least significant bit. For example, to get the value of bit 0, a bit mask of 0x00000001 should be used.

Table 6.4.1-1: T6a/T6b specific Diameter AVPs

|  |  |  |  | AVP Flag rules |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attribute Name | AVP Code | Section defined | Value Type | Must | May | Should not | Must not | May Encr. |
| Communication-Failure-Information | 4300 | 6.4.4 | Grouped | M,V |  |  |  | No |
| Cause-Type | 4301 | 6.4.5 | Unsigned32 | M,V |  |  |  | No |
| S1AP-Cause | 4302 | 6.4.6 | Unsigned32 | M,V |  |  |  | No |
| RANAP-Cause | 4303 | 6.4.7 | Unsigned32 | M,V |  |  |  | No |
| BSSGP-Cause | 4309 | 6.4.8 | Unsigned32 | M,V |  |  |  | No |
| GMM-Cause | 4304 | 6.4.9 | Unsigned32 | M,V |  |  |  | No |
| SM-Cause | 4305 | 6.4.10 | Unsigned32 | M,V |  |  |  | No |
| Number-Of-UE-Per-Location-Configuration | 4306 | 6.4.11 | Grouped | M,V |  |  |  | No |
| Number-Of-UE-Per-Location-Report | 4307 | 6.4.12 | Grouped | M,V |  |  |  | No |
| UE-Count | 4308 | 6.4.13 | Unsigned32 | M,V |  |  |  | No |
| Connection-Action | 4314 | 6.4.18 | Unsigned32 | M,V |  |  |  | No |
| Non-IP-Data | 4315 | 6.4.19 | Octetstring | M,V |  |  |  | No |
| Serving-PLMN-Rate-Control | 4310 | 6.4.21 | Grouped | M,V |  |  |  | No |
| Uplink-Rate-Limit | 4311 | 6.4.23 | Unsigned32 | M,V |  |  |  | No |
| Downlink-Rate-Limit | 4312 | 6.4.22 | Unsigned32 | M,V |  |  |  | No |
| Extended-PCO | 4313 | 6.4.26 | OctetString | M,V |  |  |  | No |
| SCEF-Wait-Time | 4316 | 6.4.24 | Time | M,V |  |  |  | No |
| CMR-Flags | 4317 | 6.4.25 | Unsigned32 | M,V |  |  |  | No |
| RRC-Cause-Counter | 4318 | 6.4.27 | Grouped | M,V |  |  |  | No |
| Counter-Value | 4319 | 6.4.28 | Unsigned32 | M,V |  |  |  | No |
| RRC-Counter-Timestamp | 4320 | 6.4.29 | Time | M,V |  |  |  | No |
| TDA-Flags | 4321 | 6.4.31 | Unsigned32 | V |  |  | M | No |
| NOTE 1: The AVP header bit denoted as "M" indicates whether support of the AVP is required. The AVP header bit denoted as "V" indicates whether the optional Vendor-ID field is present in the AVP header. For further details, see IETF RFC 6733 [32]. NOTE 2: If the M-bit is set for an AVP and the receiver does not understand the AVP, it shall return a rejection. If the M-bit is not set for an AVP, the receiver shall not return a rejection, whether or not it understands the AVP. If the receiver understands the AVP but the M-bit value does not match with the definition in this table, the receiver shall ignore the M-bit. |  |  |  |  |  |  |  |  |

The following table specifies the Diameter AVPs re-used by the T6a/T6b interface protocol from existing Diameter Applications, including a reference to their respective specifications and when needed, a short description of their use within T6a/T6b.

Any other AVPs from existing Diameter Applications, except for the AVPs from Diameter Base Protocol, do not need to be supported. The AVPs from Diameter Base Protocol are not included in table 6.4.1-2, but they may be re-used for the T6a/T6b protocol.

Table 6.4.1-2: T6a/T6b re-used Diameter AVPs

| Attribute Name | Reference | Comments |
| --- | --- | --- |
| Monitoring-Event-Configuration | 3GPP TS 29.336 [5] | This AVP shall contain the monitoring event to be configured at the MME/SGSN or the IWK-SCEF. See 6.4.2. |
| Monitoring-Event-Report | 3GPP TS 29.336 [5] | This AVP shall contain the monitoring event reported by the MME/SGSN or the IWK-SCEF. See 6.4.3. |
| SCEF-Reference-ID | 3GPP TS 29.336 [5] |  |
| SCEF-ID | 3GPP TS 29.336 [5] |  |
| SCEF-Reference-ID-for-Deletion | 3GPP TS 29.336 [5] |  |
| Supported-Features | 3GPP TS 29.229 [4] |  |
| Feature-List-ID | 3GPP TS 29.229 [4] |  |
| Feature-List | 3GPP TS 29.229 [4] | See 6.4.14 |
| OC-Supported-Features | IETF RFC 7683 [9] |  |
| OC-OLR | IETF RFC 7683 [9] |  |
| Monitoring-Event-Config-Status | 3GPP TS 29.336 [5] | This AVP shall contain the status of configuration of each monitoring event identified by an SCEF-ID and SCEF-Reference-ID. |
| DRMP | IETF RFC 7944 [15] | see 6.4.15 |
| User-Identifier | 3GPP TS 29.336 [5] | See 6.4.16 |
| Bearer-Identity | 3GPP TS 29.212 [10] | See 6.4.17 |
| Monitoring-Type | 3GPP TS 29.336 [5] |  |
| Loss-Of-Connectivity-Reason | 3GPP TS 29.336 [5] |  |
| Maximum-Number-of-Reports | 3GPP TS 29.336 [5] |  |
| Monitoring-Duration | 3GPP TS 29.336 [5] |  |
| Charged-Party | 3GPP TS 32.299 [20] |  |
| UE-Reachability-Configuration | 3GPP TS 29.336 [5] |  |
| Location-Information-Configuration | 3GPP TS 29.336 [5] |  |
| Reachability-Information | 3GPP TS 29.336 [5] |  |
| EPS-Location-Information | 3GPP TS 29.272 [16] |  |
| Service-Selection | IETF RFC 5778 [21] | See 6.4.20 |
| PDN-Connection-Charging-Id | 3GPP TS 32.299 [22] |  |
| Maximum-Retransmission-Time | 3GPP TS 29.338 [27] |  |
| Requested-Retransmission-Time | 3GPP TS 29.338 [27] |  |
| Maximum-UE-Availability-Time | 3GPP TS 29.338 [27] |  |
| 3GPP-Charging-Characteristics | 3GPP TS 29.061 [29] |  |
| RAT-Type | 3GPP TS 29.212 [10] |  |
| Terminal-Information | 3GPP TS 29.272 [16] | See 6.4.30 |
| Visited-PLMN-Id | 3GPP TS 29.272 [16] |  |
| Load | IETF RFC 8583 [31] |  |

* * * Next Change * * * *

## C.1	General

The Diameter load control mechanism is an optional feature over the T6a/b and T7 interface, which may be applied to the traffic of request commands sent to the SCEF and/or to the traffic of request commands sent to the MME/SGSN.

It is recommended to make use of the IETF RFC 8583 [31] on the T6a/b and T7 interface where:

-	when applied to the traffic of request commands sent to the SCEF, the MME/SGSN, and as an alternative  the IWK-SCEF shall behave as a reacting node;

-	when applied to the traffic of request commands sent to the MME/SGSN, the SCEF, and as an alternative the IWK-SCEF, shall behave as a reacting node.

* * * Next Change * * * *

## C.2	SCEF behaviour

When performing next hop Diameter Agent selection for requests that are routed based on realm, the SCEF may take into account load values from Load AVPs of type PEER received from candidate next hop Diameter nodes, as per IETF RFC 8583 [31].

* * * Next Change * * * *

## C.3	MME/SGSN behaviour

When performing next hop Diameter Agent selection for requests that are routed based on realm, the MME/SGSN may take into account load values from Load AVPs of type PEER received from candidate next hop Diameter nodes, as per IETF RFC 8583 [31].

* * * Next Change * * * *

## C.4	IWK-SCEF behaviour

When performing next hop Diameter Agent selection for requests that are routed based on realm, the IWK-SCEF may take into account load values from Load AVPs of type PEER received from candidate next hop Diameter nodes, as per IETF RFC 8583 [31].

* * * End of Changes * * * *