3GPP TSG-CT WG4 Meeting #96e	C4-200484

# E-Meeting, 17th – 28th February 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.500 | CR | 0092 | rev | 1 | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Edirotial fixes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-01-30 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently clause 5.2.3.2.1 heading style is H4, while it shall be H5. Also, Table 5.2.3.2-1 number is wrong and shall be Table 5.2.3.2.1-1. There is an extra comma in the first paragraph of 6.4.1. Such editorial fixes may be overlooked in a technically challenging CRs. Therefore, a separate editorial CR looks appropriate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorials are fixed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong heading style and a wrong table number persist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.2.1, 6.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.2.3	HTTP custom headers

#### 5.2.3.1	General

The list of custom HTTP headers applicable to 3GPP Service Based NFs are specified below.

#### 5.2.3.2	Mandatory to support custom headers

#### 5.2.3.2.1	General

The 3GPP NF Services shall support the HTTP custom headers specified in Table 5.2.3.2-1 below. A description of each custom header and the normative requirements on when to include them are also provided in Table 5.2.3.2-1.

Table 5.2.3.2.1-1: Mandatory HTTP custom headers

| Name | Reference | Description |
| --- | --- | --- |
| 3gpp-Sbi-Message-Priority | Clause 5.2.3.2.2 | This header is used to specify the HTTP/2 message priority for 3GPP service based interfaces. This header shall be included in HTTP/2 messages when a priority for the message needs to be conveyed (e.g HTTP/2 messages related to Multimedia Priority Sessions). |
| 3gpp-Sbi-Callback | Clause 5.2.3.2.3 | This header is used to indicate if a HTTP/2 message is a callback (e.g notification). This header shall be included in HTTP POST messages for callbacks towards NF service consumer(s) in another PLMN via the SEPP (See 3GPP TS 29.573 [27]). |
| 3gpp-Sbi-Target-apiRoot | Clause 5.2.3.2.4 | This header is used by an HTTP client to indicate the apiRoot of the target URI when communicating indirectly with the HTTP server via an SCP using HTTPS. |
| 3gpp-Sbi-Server-Binding | Clause 5.2.3.2.5 | This header is used in a service request to signal binding information to direct the service request to an HTTP server which has the targeted NF Service Resource context (see clause 6.12). |
| 3gpp-Sbi-Client-Binding | Clause 5.2.3.2.6 | This header is used to signal binding information related to an NF Service Resource to a future consumer (HTTP client) of that resource (see clause 6.12). |
| 3gpp-Sbi-Discovery-* | Clause 5.2.3.2.7 | Headers beginning with the prefix 3gpp-Sbi-Discovery- are used in indirect communication mode for discovery and selection of a suitable producer by the SCP. Such headers may be included in any SBI message and include information allowing an SCP to find a suitable producer as per the consumer's included delegated discovery parameters. |
| 3gpp-Sbi-Producer-Id | Clause 5.2.3.2.8 | This header is used in a service response from the SCP to the NF Service Consumer, when using indirect communication with delegated discovery and the NF Service Producer does not return a binding indication in a service response creating a resource. See clause 6.10.3.4. |

* * * 2nd Change * * * *

## 6.4	Overload Control

### 6.4.1	General

Service Based Interfaces use HTTP/2 over TCP for communication between the NF Services. TCP provides transport level congestion control mechanisms as specified in IETF RFC 5681 [16], which may be used for congestion control between two TCP endpoints (i.e., hop by hop). HTTP/2 also provides flow control mechanisms and limitation of stream concurrency, as specified in IETF RFC 7540 [7] that may be configured for connection level congestion control.

In addition to TCP and HTTP/2 congestion control mechanisms, end to end overload control shall be supported per NF service / API according to the below defined principles.

An NF Service Producer may mitigate a potential overload status by sending the NF Service Consumer the following HTTP status codes as a response to requests received during, or close to reaching, an overload situation:

-	503 Service Unavailable;

-	429 Too Many Requests; or

-	307 Temporary Redirect

The first 2 status codes (503 and 429) are intended to inform the NF Service Consumer that the server cannot handle the current received traffic rate, so it shall abate the traffic sent to the NF Service Producer by throttling part of this traffic locally at the NF Service Consumer, or diverting it to an alternative destination (another NF Service Producer where an alternative resource exists) that is not overloaded. If possible, traffic diversion shall always be preferred to throttling; the result of the throttling is a permanent rejection of the transaction.

If the client needs to abate a certain part of the available traffic, it shall do it based on the determined priority of each message.

Depending on regional/national requirements and network operator policy, requests related to priority traffic and emergency shall be the last to be throttled by the client, and shall be exempted from throttling due to overload control up to the point where the required traffic reduction cannot be achieved without throttling the priority requests.

The last status code (307) is intended to inform the NF Service Consumer about the availability of other endpoints where the service offered by the NF Service Producer is available, so the NF Service Consumer does not need to discard traffic locally.

* * * End of Changes * * * *