3GPP TSG-CT WG4 Meeting #96e	C4-200794

# E-Meeting, 17h – 28th February 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.230 | CR | 0677 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of AVP code for IAB-Node-Information AVP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IABARC-CT |  |  |  |  |  | Date: |  |  | 2020-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In CR C4-200795 to TS 29.272, it is proposed to add new IAB-Operation-Permission AVP. A new AVP code needs to be allocated accordingly in TS 29.230. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new AVP code was allocated for AVP IAB-Node-Information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AVP code for new AVP IAB-Node-Information will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 29.272 CR 0813 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change ***

## 7.1	3GPP specific AVP codes

The 3GPP specific AVPs have the Vendor-Specific bit ('V' bit) set in the AVP header and they carry the 3GPP's vendor identifier in the Vendor-ID field of the AVP header. The 3GPP specific AVP codes are presented in the following table.

Table 7.1: 3GPP specific AVP codes

| AVP Code | Attribute Name | Data Type | Specified in the 3GPP TS |
| --- | --- | --- | --- |

(... table rows skipped for clarity ...)

| 1600 | MME-Location-Information |  | Grouped |  | 29.272 [21] |
| --- | --- | --- | --- | --- | --- |
| 1601 | SGSN-Location-Information |  | Grouped |  |  |
| 1602 | E-UTRAN-Cell-Global-Identity |  | OctetString |  |  |
| 1603 | Tracking-Area-Identity |  | OctetString |  |  |
| 1604 | Cell-Global-Identity |  | OctetString |  |  |
| 1605 | Routing-Area-Identity |  | OctetString |  |  |
| 1606 | Location-Area-Identity |  | OctetString |  |  |
| 1607 | Service-Area-Identity |  | OctetString |  |  |
| 1608 | Geographical-Information |  | OctetString |  |  |
| 1609 | Geodetic-Information |  | OctetString |  |  |
| 1610 | Current-Location-Retrieved |  | Enumerated |  |  |
| 1611 | Age-Of-Location-Information |  | Unsigned32 |  |  |
| 1612 | Active-APN |  | Grouped |  |  |
| 1613 | SIPTO-Permission |  | Enumerated |  |  |
| 1614 | Error-Diagnostic |  | Enumerated |  |  |
| 1615 | UE-SRVCC-Capability |  | Enumerated |  |  |
| 1616 | MPS-Priority |  | Unsigned32 |  |  |
| 1617 | VPLMN-LIPA-Allowed |  | Enumerated |  |  |
| 1618 | LIPA-Permission |  | Enumerated |  |  |
| 1619 | Subscribed-Periodic-RAU-TAU-Timer |  | Unsigned32 |  |  |
| 1620 | Ext-PDP-Type |  | OctetString |  |  |
| 1621 | Ext-PDP-Address |  | Address |  |  |
| 1622 | MDT-Configuration |  | Grouped |  |  |
| 1623 | Job-Type |  | Enumerated |  |  |
| 1624 | Area-Scope |  | Grouped |  |  |
| 1625 | List-Of-Measurements |  | Unsigned32 |  |  |
| 1626 | Reporting-Trigger |  | Unsigned32 |  |  |
| 1627 | Report-Interval |  | Enumerated |  |  |
| 1628 | Report-Amount |  | Enumerated |  |  |
| 1629 | Event-ThresholdRSRP |  | Unsigned32 |  |  |
| 1630 | Event-ThresholdRSRQ |  | Unsigned32 |  |  |
| 1631 | Logging-Interval |  | Enumerated |  |  |
| 1632 | Logging-Duration |  | Enumerated |  |  |
| 1633 | Relay-Node-Indicator |  | Enumerated |  |  |
| 1634 | MDT-User-Consent |  | Enumerated |  |  |
| 1635 | PUR-Flags |  | Unsigned32 |  |  |
| 1636 | Subscribed-VSRVCC |  | Enumerated |  |  |
| 1637 | Equivalent-PLMN-List |  | Grouped |  |  |
| 1638 | CLR-Flags |  | Unsigned32 |  |  |
| 1639 | UVR-Flags |  | Unsigned32 |  |  |
| 1640 | UVA-Flags |  | Unsigned32 |  |  |
| 1641 | VPLMN-CSG-Subscription-Data |  | Grouped |  |  |
| 1642 | Time-Zone |  | UTF8String |  |  |
| 1643 | A-MSISDN |  | OctetString |  |  |
| 1644 | Reserved |  | - |  |  |
| 1645 | MME-Number-for-MT-SMS |  | OctetString |  |  |
| 1646 | Reserved |  | - |  |  |
| 1647 | Reserved |  | - |  |  |
| 1648 | SMS-Register-Request |  | Enumerated |  |  |
| 1649 | Local-Time-Zone |  | Grouped |  |  |
| 1650 | Daylight-Saving-Time |  | Enumerated |  |  |
| 1651 | GMLC-Info |  | Grouped |  |  |
| 1652 | GMLC-Name |  | DiameterIdentity |  |  |
| 1653 | GMLC-Realm |  | DiameterIdentity |  |  |
| 1654 | Subscription-Data-Flags |  | Unsigned32 |  |  |
| 1655 | Measurement-Period-LTE |  | Enumerated |  |  |
| 1656 | Measurement-Period-UMTS |  | Enumerated |  |  |
| 1657 | Collection-Period-RRM-LTE |  | Enumerated |  |  |
| 1658 | Collection-Period-RRM-UMTS |  | Enumerated |  |  |
| 1659 | Positioning-Method |  | OctetString |  |  |
| 1660 | Measurement-Quantity |  | OctetString |  |  |
| 1661 | Event-Threshold-Event-1F |  | Integer32 |  |  |
| 1662 | Event-Threshold-Event-1I |  | Integer32 |  |  |
| 1663 | Restoration-Priority |  | Unsigned32 |  |  |
| 1664 | SGs-MME-Identity |  | UTF8String |  |  |
| 1665 | SIPTO-Local-Network-Permission |  | Unsigned32 |  |  |
| 1666 | Coupled-Node-Diameter-ID |  | DiameterIdentity |  |  |
| 1667 | WLAN-offloadability |  | Grouped |  |  |
| 1668 | WLAN-offloadability-EUTRAN |  | Unsigned32 |  |  |
| 1669 | WLAN-offloadability-UTRAN |  | Unsigned32 |  |  |
| 1670 | Reset-ID |  | OctetString |  |  |
| 1671 | MDT-Allowed-PLMN-Id |  | OctetString |  |  |
| 1672 | Adjacent-PLMNs |  | Grouped |  |  |
| 1673 | Adjacent-Access-Restriction-Data |  | Grouped |  |  |
| 1674 | DL-Buffering-Suggested-Packet-Count |  | Integer32 |  |  |
| 1675 |  | IMSI-Group-Id |  | Grouped |  |
| 1676 |  | Group-Service-Id |  | Unsigned32 |  |
| 1677 |  | Group-PLMN-Id |  | OctetString |  |
| 1678 |  | Local-Group-Id |  | OctetString |  |
| 1679 |  | AIR-Flags |  | Unsigned32 |  |
| 1680 |  | UE-Usage-Type |  | Unsigned32 |  |
| 1681 |  | Non-IP-PDN-Type-Indicator |  | Enumerated |  |
| 1682 |  | Non-IP-Data-Delivery-Mechanism |  | Unsigned32 |  |
| 1683 |  | Additional-Context-Identifier |  | Unsigned32 |  |
| 1684 |  | SCEF-Realm |  | DiameterIdentity |  |
| 1685 |  | Subscription-Data-Deletion |  | Grouped |  |
| 1686 |  | Preferred-Data-Mode |  | Unsigned32 |  |
| 1687 |  | Emergency-Info |  | Grouped |  |
| 1688 |  | V2X-Subscription-Data |  | Grouped |  |
| 1689 |  | V2X-Permission |  | Unsigned32 |  |
| 1690 |  | PDN-Connection-Continuity |  | Unsigned32 |  |
| 1691 |  | eDRX-Cycle-Length |  | Grouped |  |
| 1692 |  | eDRX-Cycle-Length-Value |  | OctetString |  |
| 1693 |  | UE-PC5-AMBR |  | Unsigned32 |  |
| 1694 |  | MBSFN-Area |  | Grouped |  |
| 1695 |  | MBSFN-Area-ID |  | Unsigned32 |  |
| 1696 |  | Carrier-Frequency |  | Unsigned32 |  |
| 1697 |  | RDS-Indicator |  | Enumerated |  |
| 1698 |  | Service-Gap-Time |  | Unsigned32 |  |
| 1699 |  | Aerial-UE-Subscription-Information |  | Unsigned32 |  |
| 1700 |  | Broadcast-Location-Assistance-Data-Types |  | Unsigned64 |  |
| 1701 |  | Paging-Time-Window |  | Grouped |  |
| 1702 |  | Operation-Mode |  | Unsigned32 |  |
| 1703 |  | Paging-Time-Window-Length |  | OctetString |  |
| 1704 |  | Core-Network-Restrictions |  | Unsigned32 |  |
| 1705 |  | eDRX-Related-RAT |  | Grouped |  |
| 1706 |  | Interworking-5GS-Indicator |  | Enumerated |  |
| 1707 |  | Ethernet-PDN-Type-Indicator |  | Enumerated |  |
| 1708 |  | Subscribed-ARPI |  | Unsigned32 |  |
| 17xx |  | IAB-Operation-Permission |  | Enumerated |  |
| Note: The AVP codes from 17xx to 1799 are reserved for TS 29.272. |  |  |  |  |  |

(... table rows skipped for clarity ...)

*** End of changes ***