Joint-Working-Group (Parlay, ETSI TISPAN Project OSA, 3GPP CT5)	C5-070558

Meeting CT5#40, 6 - 8 August 2007, Trondheim, NORWAY

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.198-15 | CR | 0012 |  rev | - |  Current version: | 6.5.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Clarify address and address range descriptions to prevent portability issues for OSA/Parlay clients |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | AePONA |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | C5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OSA3 |  |  |  |  |  |  | Date:  |  | 05/09/2007 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-6 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | Provide a clear explanation for address and address range descriptions in order to prevent portability issues for OSA/Parlay clients. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | The current Multi-media Messaging (MM) API specification contains a number of sections describing an address and address range where it is unclear whether source or destination address is being described. It is also unclear for createNotification and event criteria description what is meant by source or destination address, as this depends on whether the perspective is network or application centric. This CR proposes to add clarity to descriptions of address and address range in order to promote portability for Parlay/OSA applications using the Multi Media Messaging SCF. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Different intrepretation of address description may cause portability issues for Parlay/OSA applications. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 10, 11.2.2, 11.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  | R7 Mirror CR in C5-070530 |  |  |  |  |  |  |  |  |

1st Modified Section

# Multi-Media Messaging Service Properties

The following table lists properties relevant for the Multi-Media Messaging API.

| Property | Type | Description |
| --- | --- | --- |
| P_MESSAGE_DELIVERY_TYPE | STRING_SET | Specifies the message delivery capabilities which the SCS supports. Values are defined by TpMessageDeliveryType. |
| P_PUT_MESSAGE_FOLDER_TO_SEND | STRING_SET | Identifies the folder in the mailbox into which messages should be placed in order to be sent. |

The previous table lists properties related to capabilities of the SCS itself. The following table lists properties that are used in the context of the Service Level Agreement, e.g. to restrict the access of applications to the capabilities of the SCS.

| Property | Type | Description |
| --- | --- | --- |
| P_NOTIFICATION_ADDRESS_RANGES | XML_ADDRESS_RANGE_SET | Indicates for which addresses notifications may be set. This applies a restriction to addresses the SCS can request notification from. More than one range may be present. |
| P SOURCE_ADDRESS_RANGES | XML_ADDRESS_RANGE_SET | Indicates which addresses a client application on the SCS is allowed to use for its own identity, as used when sending a message to a destination or in criteria for notification of new message arrived. |

Next Modified Section

### TpMessagingEventCriteria

Defines the Tagged Choice of Data Elements that specify the criteria for an event notification to be generated.

|  | Tag Element Type |  |
| --- | --- | --- |
|  | TpMessagingEventName |  |

| Tag Element Value | Choice Element Type | Choice Element Name |
| --- | --- | --- |
| P_EVENT_MSG_NAME_UNDEFINED | NULL | Undefined |
| P_EVENT_MSG_NEW_MAILBOX_MESSAGE_ARRIVED | TpNewMailboxMessageArrivedCriteria | EventNewMailboxMessageArrived |
| P_EVENT_MSG_NEW_MESSAGE_ARRIVED | TpNewMessageArrivedCriteria | EventNewMessageArrived |
| P_EVENT_MSG_STATUS_REPORT_ARRIVED | TpAddressRange | EventNewMessageStatusReportArrived |

The address range defined for EventNewMessageStatusReportArrived refers to the addresses of devices in the network to which the client application on the SCS has sent or intends to send a message.

Next Modified Section

### TpNewMessageArrivedCriteria

Defines the Sequence of Data Elements that specify the criteria for a New Message Arrived event, for messages which are received outside the context of a mailbox.

| Sequence Element Name | Sequence Element Type | Description |
| --- | --- | --- |
| SourceAddress | TpAddressRange | Defines the source address for which the notification is requested. The source is the originator of the message, and is a device in the network. |
| DestinationAddress | TpAddressRange | Defines the destination address or address range for which the notification is requested. The destination is the address identity for the client application on the SCS. |
| CreateMultiMediaMessagingSession | TpBoolean | Used by the application to indicate if the SCF should create an instance of IpMultiMediaMessaging upon receipt of an event which matches these criteria. |

End of modifications