3GPP TSG-CT WG6 Meeting #117	C6-230683

# Chicago, US; 14th– 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.111 | CR | 0804 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Access Technology for Satellite E-UTRAN and Satellite NG-RAN definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-11-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During CT6#109-bis-e was agreed CR765 C6-220078 that introduced Satellite NG-RAN USAT. During CT6#111-e was agreed CR771 C6-220306 that introduced Satellite E-UTRAN USAT. The 3GPP TS 31.111 Release 17 references ETSI TS 102 223 V15.2.0. But none of those CRs specified Access Technologies neiher for Satellite NG-RAN, nor for Satellite E-UTRAN, and those Access Technologies are missing in referenced ETSI TS 102 223 V15.2.0. Satellite NG-RAN Access Technology was defined only since ETSI 102 223 V17.2.0 (claude 8.61). Satellite E-UTRAN Access Technology is undefined. Those Access Technologies have to be defined explicitly in 3GPP TS 31.111 since release 17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 8.62: - Rewording “3GPP NR” to most appropriate “3GPP NG-RAN” for ‘0A’ Access Technology, - definition of ‘0B’ Access Technology as “Satellite NG-RAN”, - definition of ‘0C’ Access Technology as “Satellite E-UTRAN”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unconsistency between TS 31.111 and ETSI ETSI 102 223 reference. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.62 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Was C6-230650 |  |  |  |  |  |  |  |  |

***** first change *****

## 8.62	Access Technology

ETSI TS 102 223 [32] clause 8.61 applies, with the following adjustment on the Access Technology coding:

-	'0A' = 3GPP NG-RAN;

-	'0B' = 3GPP Satellite NG-RAN;

-	'0C' = 3GPP Satellite E-UTRAN;

-	all other values are reserved for future use.

***** End of change *****