3GPP TSG-CT6 Meeting #98	C6-200029

# Sophia-Antipolis, France, 25th Feb 2020 - 28th Feb 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.121 | CR | 0312 | rev | - | Current version: | 15.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of incorrectly coded EF_ACC bytes in TC 5.4.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Comprion GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2020-01-23 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Table 5.4.9-1 and 5.4.9-2 show incorrect values for EF_ACC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of the EF_ACC values in Table 5.4.9-1 and 5.4.9-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EF_ACC coded in accordance to the table will cause incorrect barring behavior |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.4.9	Unified Access Control – Access Identities 12, 13 and 14 indicated by USIM

…

##### 5.4.9.4.3	Acceptance criteria

For the Table 5.4.9-1

-	After step a) the UE shall make a successful or not successful Registration to the network in accordance with the result indicated in the table.

-	After step b) the UE shall make a successful or not successful MO Data call in accordance with the result indicated in the table if the step is applicable.

For the Table 5.4.9-2

-	After step e) the UE shall make a successful or not successful RRC Resumption for RNA Update in accordance with the result indicated in the table.

Table 5.4.9-1

| TC Seq# | Access Category | USIM |  |  | SIB1 |  | Result |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | Access Identity | uac-BarringInfo | PLMN-Identity (MCC/MNC) | Registration successful? | MO Data call successful? |  |
| 1.1 | 7 | 0x00 00 00 00 | 00010 | 12 | Not Present | 246 / 081 | Yes | Yes |  |
| 1.2 | 3 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common( 3,0x0001000'B) | 246 / 081 | No | NA |  |
| 1.3 | 3 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_PerPLMN( 3,0x0001000'B) | 246 / 081 | No | NA |  |
| 1.4 | 3 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common( 3,0x0000000'B) | 246 / 082 | Yes | Yes |  |
| 1.5 | 3 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common( 3,0x0001000'B) | 246 / 082 | No | NA |  |
| 1.6 | 3 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common( 3,0x0000000'B) | 244 / 081 | No | NA |  |
| 1.7 | 3 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common( 3,0x0000100'B) | 246 / 081 | Yes | Yes |  |
| 1.8 | 3 | 0x00 00 00 00 | 01110 | 12,13,14 | UAC_BarringInfo_Common( 3,0x0000110'B) | 246 / 081 | Yes | Yes |  |
| 1.9 | 7 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common( 7,0x0001000'B) | 246 / 081 | Yes | No |  |
| 1.10 | 7 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common( 7,0x0000000'B) | 246 / 082 | Yes | No |  |
| 1.11 | 7 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common( 7,0x0000000'B) | 244 / 081 | Yes | No |  |
| 1.12 | 7 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common2 ( 3,0x0000100'B, 7,0x0001000'B ) | 246 / 081 | Yes | No |  |
| 1.13 | 3 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common( 3,0x0000100'B) | 246 / 081 | No | NA |  |
| 1.14 | 3 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_PerPLMN( 3,0x0000100'B) | 246 / 081 | No | NA |  |
| 1.15 | 3 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common( 3,0x0000000'B) | 246 / 082 | Yes | Yes |  |
| 1.16 | 3 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common( 3,0x0000100'B) | 246 / 082 | No | NA |  |
| 1.17 | 3 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common( 3,0x0000010'B) | 246 / 082 | Yes | Yes |  |
| 1.18 | 7 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common( 7,0x0000100'B) | 246 / 081 | Yes | No |  |
| 1.19 | 7 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common( 7,0x0000000'B) | 244 / 081 | Yes | No |  |
| 1.20 | 7 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common2( 3,0x0000010'B, 7,0x0000100'B) | 246 / 081 | Yes | No |  |
| 1.21 | 3 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common( 3,0x0000010'B) | 246 / 081 | No | NA |  |
| 1.22 | 3 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_PerPLMN( 3,0x0000010'B) | 246 / 081 | No | NA |  |
| 1.23 | 3 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common( 3,0x0000000'B) | 246 / 082 | Yes | Yes |  |
| 1.24 | 3 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common( 3,0x0000010'B) | 246 / 082 | No | NA |  |
| 1.25 | 3 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common( 3,0x0001100'B) | 246 / 082 | Yes | Yes |  |
| 1.26 | 7 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common( 7,0x0000010'B) | 246 / 081 | Yes | No |  |
| 1.27 | 7 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common( 7,0x0000000'B) | 244 / 081 | Yes | No |  |
| 1.28 | 7 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common2( 3,0x0000100'B, 7,0x0000010'B) | 246 / 081 | Yes | No |  |

Table 5.4.9-2

| TC Seq# | Access Category | USIM |  |  | SIB1 |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | Access Identity | uac-BarringInfo | PLMN-Identity (MCC/MNC) | RRCResumeRequest with resumeCause set to rna-Update successful? |
| 2.1 | 8 | 0x00 00 00 00 | 00010 | 12 | Not Present | 246 / 081 | Yes |
| 2.2 | 8 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common( 8,0x0001000'B) | 246 / 081 | No |
| 2.3 | 8 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_PerPLMN( 8,0x0000000'B) | 246 / 081 | Yes |
| 2.4 | 8 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common( 8,0x0010000'B) | 246 / 082 | Yes |
| 2.5 | 8 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common2 ( 7,0x0010000'B, 8,0x0000000'B) | 244 / 081 | No |
| 2.6 | 8 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common( 8,0x0000100'B) | 246 / 081 | No |
| 2.7 | 8 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_PerPLMN( 8,0x0000000'B) | 246 / 081 | Yes |
| 2.8 | 8 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common( 8,0x0000100'B) | 246 / 082 | No |
| 2.9 | 8 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common2 ( 7,0x0001000'B, 8,0x0000000'B) | 244 / 081 | No |
| 2.10 | 8 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common( 8,0x0000010'B) | 246 / 081 | No |
| 2.11 | 8 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_PerPLMN( 8,0x0000000'B) | 246 / 081 | Yes |
| 2.12 | 8 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common( 8,0x0000010'B) | 246 / 082 | No |
| 2.13 | 8 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common2 ( 7,0x0000100'B, 8,0x0000000'B) | 244 / 081 | No |