ETSI SMG SMG3 WPA GPRS Ad-hoc 	Tdoc N1-99414

Oslo, Norway

18-21 May 1999

| CHANGE REQUEST No : |  |  |  |  | A060 |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 04.64 |  | Version: |  |  | 6.3.0 |  |  |  |
| Submitted to SMG | # 29 |  | for approval |  | x |  | without presentation ("non-strategic") |  |  |  |  |  |
| list SMG plenary meeting no. here |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  | x |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Ericsson | Date: | 11 May 1999 |
| --- | --- | --- | --- |

| Subject: | Frame reject procedures |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | x |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  |  |  |

| Reason for change: | The LLC specification is derived from ITU-T Recommendation Q.921 (LAPD). LAPD allows the receipt but not the transmission of Frame Reject (FRMR) responses in order that interworking with equipment which uses the LAPB protocol is possible e.g. for support of a LAPB data link connection within the ISDN D channel. The LAPB protocol allows both transmission and reception of FRMR responses thus receipt of FRMR responses is necessary in LAPD, but transmission of the FRMR response is unnecessary. As a result of deriving the LLC specification from Q.921, the LAPB interworking procedures in Q.921 have been unnecessarily introduced into the LLC specification. As LLC has no requirement to interwork with LAPB, there is no requirement to support the FRMR response frame in the LLC protocol. This CR removes the LAPB interworking from the LLC specification by removing all reference to the FRMR response frame. |
| --- | --- |

| Clauses affected: | 3.2, 6.4, 6.4.1.5, 8.1, 8.7.1, 8.8.2, 8.8.3 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 3.2	Abbreviations

For the purposes of the present document the following abbreviations apply. Additional applicable abbreviations and symbols can be found in GSM 01.04 [1] and GSM 03.60.

ABM	Asynchronous Balanced Mode

ACK	ACKnowledgement

ADM	Asynchronous Disconnected Mode

CNF	Confirm

DISC	DISConnect

DM	Disconnected Mode

GMM	GPRS Mobility Management

GRR	GPRS Radio Resources service access point

I	Information

IOV	Input Offset Value

IND	Indication

LAPD	Link Access Procedure on the D-channel

LL	Logical Link

LLC	Logical Link Control

LLE	Logical Link Entity

LLGMM	LLC to GPRS Mobility Management service access point

LLM	Logical Link Management

LLME	Logical Link Management Entity

REQ	Request

RES	Response

RNR	Receive Not Ready

RR	Receive Ready

S	Supervisory

SABM	Set Asynchronous Balanced Mode

SACK	Selective ACKnowledgement

TIA	Telecommunications Industry Association

UA	Unnumbered Acknowledgement

UI	Unconfirmed Information

XID	eXchange IDentification

## 6.4	Commands and responses

The following commands and responses are used by the MS and the SGSN LLEs and are represented in Table 4. Each logical link connection shall support the appropriate set of commands and responses for the type of operation desired (see clause 8).

Those frame types not identified in Figure 8, Figure 9, or Table 4, shall be identified as having undefined command and/or response control fields, and shall be treated as defined in subclause 8.8.2.

Table 4: Commands and responses

|  |  |  |  |  |  | Encoding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Format |  | Commands |  | Responses |  | S1 | S2 | M4 |  | M3 |  |  | M2 |  | M1 |
|  | RR |  |  |  | RR | 0 | 0 |  | - |  | - | - |  | - |  |
| Information + | ACK |  |  |  | ACK | 0 | 1 |  | - |  | - | - |  | - |  |
| Supervisory | RNR |  |  |  | RNR | 1 | 0 |  | - |  | - | - |  | - |  |
|  | SACK |  |  |  | SACK | 1 | 1 |  | - |  | - | - |  | - |  |
|  | - |  |  |  | DM | - | - |  | 0 |  | 0 | 0 |  | 1 |  |
|  | DISC |  |  |  | - | - | - |  | 0 |  | 1 | 0 |  | 0 |  |
| Unnumbered | - |  |  |  | UA | - | - |  | 0 |  | 1 | 1 |  | 0 |  |
|  | SABM |  |  |  | - | - | - |  | 0 |  | 1 | 1 |  | 1 |  |
|  | XID |  |  |  | XID | - | - |  | 1 |  | 0 | 1 |  | 1 |  |

The commands and responses in Table 4 are defined in the following subclauses.

#### 6.4.1.5	Spare

and

not recoverable by retransmission of the identical frame below:

1)	receipt of a command or response control field that is undefined or not implemented (see subclause 6.4, second paragraph);

2)	receipt of a supervisory or unnumbered frame with incorrect length; or

3)	receipt of an I frame with an information field that exceeds the maximum established length.

An undefined control field is any of the control field encodings that are not identified in Figure 8, Figure 9, or Table 4.

### Spare