3GPP TSG CN WG1/ETSI SMG3 WPA	Tdoc N1-99418

Oslo, Norway

May 18-21, 1999

| CHANGE REQUEST No : |  |  |  |  |  | A673 | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 04.08 |  | Version |  | 6.3.0 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Ericsson | Date: | 15-04-1999 |
| --- | --- | --- | --- |

| Subject: | Change of network operation mode |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | X |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | This CR clarifies the behaviour of an MS in GPRS operation mode A or B when the RA is changed and the new RA has a different network mode of operation. This CR is based upon GSM 03.60 where the following is stated in section 5.4.5: A GPRS MS can operate in one of three modes of operation. The mode of operation depends on the services that the MS is attached to, i.e., only GPRS or both GPRS and other GSM services, and upon the MS's capabilities to operate GPRS and other GSM services simultaneously. - Class-A mode of operation: The MS is attached to both GPRS and other GSM services, and the MS supports simultaneous operation of GPRS and other GSM services. - Class-B mode of operation: The MS is attached to both GPRS and other GSM services, but the MS can only operate one set of services at a time. In network operation mode III (see subclause "Paging Co-ordination"), an MS that is capable of monitoring only one paging channel at a time cannot operate in class B mode of operation. In this case, such an MS shall revert to class-C mode of operation. - Class-C mode of operation: The MS is exclusively attached to GPRS services. And the following is stated in GSM 03.60, in section 6.9.1: In network mode of operation II and III, whenever an MS determines that it shall perform both an LA update and an RA update, the MS shall perform the LA update first. |
| --- | --- |

| Clauses affected: | 4.7.1.5 (new section), 4.7.3.1.5, 4.7.5.1.5 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

*** First modification ***

#### 4.7.1.5	Change of network mode of operation

Whenever an MS moves to a new RA, the procedures executed by the MS depend on the network mode of operation in the old and new routing area.

- In case the MS is in state GMM-REGISTERED or GMM-ROUTING-AREA-UPDATING-INITIATED and is in operation mode:

- A and B (capable of monitoring more than one paging channel), the MS shall execute:

| Network operation mode change | ## Procedure to execute |
| --- | --- |
| I ® II or I ® III or II ® III or III ® II | Normal Location Update, if a new LA is entered, followed by a Normal Routing Area Update |
| II ® I or III ® I | Combined Routing Area Update with IMSI attach |

- B (capable of monitoring only one paging channel), the MS shall execute:

| Network operation mode change | ## Procedure to execute |
| --- | --- |
| ## I ® II | Normal Location Update, if a new LA is entered, followed by a Normal Routing Area Update |
| I ® III or II ® III | Normal Routing Area Update, followed by a GPRS Detach with detach type IE indicating “IMSI detach” |
| II ® I or III ® I | Combined Routing Area Update procedure with IMSI attach |
| III ® II | IMSI attach if case explicit IMSI Attach is indicated in system information or otherwise a Normal Location Update, followed by a Normal Routing Area Update |

- In case the MS is in state GMM-REGISTERED-INITIATED and is in operation mode:

- A and B (capable of monitoring more than one paging channel), the MS shall execute:

| Network operation mode change | ## Procedure to execute |
| --- | --- |
| I ® II or I ® III or II ® III or III ® II | IMSI attach if indicated in system information, followed by a Normal GPRS Attach |
| II ® I or III ® I | Combined Attach |

- B (capable of monitoring only one paging channel), the MS shall execute:

| Network operation mode change | ## Procedure to execute |
| --- | --- |
| ## I ® II or III ® II | IMSI attach if indicated in system information, followed by a Normal GPRS Attach |
| I ® III or II ® III | Normal GPRS Attach |
| II ® I or III ® I | Combined Attach |

Further details are implementation issues.

*** Next modification ***

##### 4.7.3.1.5	Abnormal cases in the MS

The following abnormal cases can be identified:

a)	Access barred because of access class control

The GPRS attach procedure shall not be started. The MS stays in the current serving cell and applies normal cell reselection process. The GPRS attach procedure is started as soon as possible, i.e. when access is granted or because of a cell change.

b)	Lower layer failure before the ATTACH ACCEPT or ATTACH REJECT message is received

The procedure shall be aborted. The MS shall keep proceed as described below.

c)	T3310 time-out

On the first expiry of the timer, the MS reset and restart timer T3310 and shall retransmit the ATTACH REQUEST message. This retransmission is repeated four times, i.e. on the fifth expiry of timer T3310, the GPRS attach procedure shall be aborted and the MS shall proceed as described below.

ATTACH REJECT, other causes than those treated in section 4.7.3.1.4
The MS shall proceed as described below.

e)	Change of cell into a new routing area

If a cell change into a new routing area occurs before an ATTACH ACCEPT or ATTACH REJECT message has been received, the GPRS attach procedure shall be aborted and re-initiated immediately. 
If a routing area border is crossed when the ATTACH ACCEPT message is received but before an ATTACH COMPLETE message is sent, the GPRS attach procedure shall be aborted and the routing area updating procedure shall be initiated. If a P-TMSI was allocated during the GPRS attach procedure, this P-TMSI shall be used in the routing area updating procedure. If a P-TMSI signature was allocated together with the P-TMSI during the GPRS attach procedure, this P-TMSI signature shall be used in the routing area updating procedure. 
 If a routing area border is crossed before the ATTACH ACCEPT message is received, the MS shall proceed as specified in section 4.7.1.5.

*** Next modification ***

##### 4.7.5.1.5	Abnormal cases in the MS

The following abnormal cases can be identified:

a)	Access barred because of access class control

The routing area updating procedure shall not be started. The MS stays in the current serving cell and applies the normal cell reselection process. The procedure is started as soon as possible and if still necessary, i.e. when the barred state is removed or because of a cell change.

b)	Lower layer failure before the ROUTING AREA UPDATE ACCEPT or ROUTING AREA UPDATE REJECT message is received

The procedure shall be aborted. The MS shall proceed as described below.

c)	T3330 time-out

The procedure is restarted four times, i.e. on the fifth expiry of timer T3330, the MS shall abort the procedure. The MS shall proceed as described below.

d)	ROUTING AREA UPDATE REJECT, other causes than those treated in section 4.7.5.1.4

The MS shall proceed as described below.

e)	If a routing area border is crossed, when the MS is in state GMM-ROUTING-AREA-UPDATE-INITIATED, the routing area updating procedure shall be aborted and re-initiated immediatelythe MS shall proceed as specified in section 4.7.1.5.