ETSI SMG3 WPA GPRS Ad Hoc                                                                               Tdoc N1-99428

Oslo, Norway

May 18-21, 1999

From: SMG3-WPA

To:	 SMG2-WPA

Liaison Statement on Maximum frame relay payload length

SMG3 has detected that in the downlink direction (SGSN->MS), the worst case (all optional fields included) NS/BSSGP/LLC-Ack frame (1620 octets) does not fit into a Frame Relay SDU (max. 1600 octets).

Calculation of worst case NS-frame: 
  - NS-UNITDATA header size: 4 octets
  - BSSGP DL-UNITDATA header size (all fields): 56 octets 
  - LLC I-frame SACK format (full window): 37 octets
  - LLC SDU, i.e., N201-I (default: 1 520 octets),
  - LLC FCS: 3 octets
TOTAL: 4 + 56 + 37 + 1520 + 3 = 1620 (i.e., 20 octets too large)

SMG3 WPA kindly asks SMG2 WPA if an acceptable solution to this problem would be to extend the maximum frame relay payload length in specification GSM 08.16.

NOTES:

- This Liaison Statement comes directly from SMG3WPA GPRS Ad Hoc and has not been reviewed by SMG3WPA.

- A CR (Tdoc N1-99220) to remedy this problem by reducing the max. LLC SDU length was rejected by SMG3WPA. Reducing the max. LLC SDU length is undesirable as it forces the use of SNDCP layer segmentation for large N-PDU.