ETSI SMG3 WPA GPRS ad-hoc	Tdoc N1-99431

Oslo, Norway

May, 18-21, 1999

| CHANGE REQUEST No : |  |  |  |  | A681 |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 04.08 |  | Version: |  |  | 6.3.0 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X |  | without presentation ("non-strategic") |  |  |  | X |  |
| list SMG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Ericsson | Date: | 11.05.99 |
| --- | --- | --- | --- |

| Subject: | Improved SM messages error handling for non-semantically mandatory IE errors |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | X |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  |  |  |

| Reason for change: | For deactivation or request of a PDP context, the procedure should be completed if possible when the received message contains a mandatory IE with non-semantical error. Thus the same status on the MS and network side is obtained as quick as possible, saving resources and signalling. |
| --- | --- |

| Clauses affected: | 8.5 and new chapters 8.5.4 and 8.5.5 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 8.5	Non-semantical mandatory information element errors

When on receipt of a message,

-	an "imperative message part" error; or

-	a "missing mandatory IE" error

is diagnosed or when a message containing:

-	a syntactically incorrect mandatory IE; or

-	an IE unknown in the message, but encoded as "comprehension required" (see section 10.5); or

-	an out of sequence IE encoded as "comprehension required" (see section 10.5)

is received,

-	the mobile station shall proceed as follows:

If the message is not one of the messages listed in sections 8.5.1, 8.5.2, 8.5.3, 8.5.4 and 8.5.5, the mobile station shall ignore the message except for the fact that, if an RR connection exists, it shall return a status message (STATUS, RR STATUS or MM STATUS depending on the protocol discriminator) with cause # 96 "invalid mandatory information". If the message was a GMM message the GMM-STATUS message with cause #98 “Message type not compatible with protocol state” shall be returned. If the message was an SM message the SM-STATUS message with cause # 96 "invalid mandatory information" shall be returned.

-	the network shall proceed as follows:

When the message is not one of the messages listed in section 8.5.3 b), c), d) or e) and 8.5.5 a) or b), the network shall either

-	try to treat the message (the exact further actions are implementation dependent), or

-	ignore the message except that it should return a status message (STATUS, RR STATUS, MM STATUS (depending on the protocol discriminator), GMM STATUS, or SM STATUS) with cause # 96 "invalid mandatory information".

### 8.5.1	Radio resource management

For the mobile station the following procedures shall apply:

a)	If the message is a CHANNEL RELEASE message, the actions taken shall be the same as specified in 3.5 "RR connection release".

b)	If the message is a PARTIAL RELEASE message, the reactions of the MS are for further study.

### 8.5.2	Mobility management

No exceptional cases are described for mobility management messages.

### 8.5.3	Call control

a)	If the message is a SETUP message, a RELEASE COMPLETE message with cause # 96 "invalid mandatory information" shall be returned.

b)	If the message is a DISCONNECT message, a RELEASE message shall be returned with cause value # 96 "invalid mandatory information" and section 5.4. "call clearing" applies as normal.

c)	If the message is a RELEASE message, a RELEASE COMPLETE message shall be returned with cause value # 96 "invalid mandatory information".

d)	If the message is a RELEASE COMPLETE message, it shall be treated as a normal RELEASE COMPLETE message.

- If the message is a HOLD REJECT or RETRIEVE REJECT message, it shall be treated as a normal HOLD REJECT or RETRIEVE REJECT message.

- If the message is a STATUS message and received by the network, a RELEASE COMPLETE message may be returned with cause value # 96 "invalid mandatory information".

### GMM mobility management

No exceptional cases are described for mobility management messages.

### 8.5.5	Session management

a)	If the message is a DEACTIVATE PDP CONTEXT REQUEST, a DEACTIVATE PDP CONTEXT ACCEPT message shall be returned. All resources allocated for that context shall be released.

- If the message is a DEACTIVATE AA PDP CONTEXT REQUEST, a DEACTIVATE AA PDP CONTEXT ACCEPT message shall be returned. All resources allocated for that context shall be released.

- If the message is a REQUEST PDP CONTEXT ACTIVATION, a REQUEST PDP CONTEXT REJECT message with cause  # 96 "Invalid mandatory information" shall be returned