3GPP TSG CN WG1 / SMG3 WPA GPRS Ad Hoc	Tdoc N1-99439

Oslo, Norway																			revision of N1-99263

May 18-21 1999

| CHANGE REQUEST No : |  |  |  |  |  | A653r1 | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 04.08 |  | Version |  | 6.3.0 | +CR A581 |  |  |
| Submitted to SMG | 29 |  | For approval |  | x | Without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS (Phase 1) |
| --- | --- |

| Source: | Ericsson | Date: | 18-05-99 |
| --- | --- | --- | --- |

| Subject: | Reactivation of PDP contexts |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | x |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | The current wording in the description of the network initiated GPRS detach procedure with detach type IE indicating “re-attach required” is misleading and suggests that reactivation of PDP contexts at reattach is mandatory in the mobile. This cannot be the case as user interaction may be required before a PDP context can be activated again. This CR clarifies that it is not mandatory to reactivate a PDP context at network initiated GPRS detach, although it is highly desirable. This CR also corrects a similar problem in normal and periodic and combined RA updating not accepted by the network. |
| --- | --- |

| Clauses affected: | 4.7.4.2.1, 4.7.5.1.4, and 4.7.5.2.4 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: | This CR is dependent on the acceptance of 04.08 CR A581, as it modifies the text already modified in this CR. All text in CR A581 has been included in this CR, even if it is not relevant for this CR. (i.e section 10.5.5.14) |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

#### 4.7.4.2	Network initiated GPRS detach procedure

##### 4.7.4.2.1	Network initiated GPRS detach procedure initiation

The network initiates the GPRS detach procedure by sending a DETACH REQUEST message to the MS. The network shall start timer T3322, shall deactivate PDP contexts and the logical link(s), if any, and shall change to state GMM-DEREGISTERED-INITIATED. The DETACH REQUEST message shall include a detach type IE. In addition, the network may include a cause IE to specify the reason for the detach request.

If the detach type IE indicates "re-attach required", the MS shall perform a new attach procedure. The MS should also activate PDP context(s) to replace any previously active PDP contexts.

NOTE (Re-)activation of PDP contexts  (in the case where dynamic IP addresses  are used)  is normally triggered by the user, and so cannot be done automatically by the MS.

##### 4.7.4.2.2	Network initiated GPRS detach procedure completion by the MS

When receiving the DETACH REQUEST message and the detach type IE indicates “GPRS detach” or “combined GPRS/IMSI detach”, the MS shall deactivate all activated PDP contexts. The MS shall then send a DETACH ACCEPT message to the network and shall change state to GMM-DEREGISTERED. The MS shall, after the completion of the GPRS detach procedure, initiate a GPRS attach procedure if indicated by the network in the detach type IE..

When receiving the DETACH REQUEST message and the detach type IE indicates “IMSI detach”, the MS shall send a DETACH ACCEPT message to the network.

Depending on the received cause code, the MS shall act as follows:

# 2	(IMSI unknown in HLR)

The MS shall set the update status to U3 ROAMING NOT ALLOWED and shall delete any TMSI, LAI and ciphering key sequence number. The new MM state is MM IDLE. The SIM shall be considered as invalid for non-GPRS services until switching off or the SIM is removed.

# 3	(Illegal MS); or

# 6	(Illegal ME)

The MS shall set the GPRS update status to GU3 ROAMING NOT ALLOWED (and shall store it according to section 4.1.3.2) and shall delete any P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number. The new GMM state is GMM-DEREGISTERED. The SIM shall be considered as invalid for GPRS services until switching off or the SIM is removed.

If the MS is IMSI attached via MM procedures, the MS shall in addition set the update status to U3 ROAMING NOT ALLOWED, shall delete any TMSI, LAI and ciphering key sequence number. The new MM state is MM idle. The SIM shall be considered as invalid also for non-GPRS services until switching off or the SIM is removed.

# 7	(GPRS services not allowed)

The MS shall set the GPRS update status to GU3 ROAMING NOT ALLOWED (and shall store it according to section 4.1.3.2) and shall delete any P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number. The SIM shall be considered as invalid for GPRS services until switching off or the SIM is removed. The new state is GMM-DEREGISTERED.

# 8	(GPRS services and non-GPRS services not allowed)

The MS shall set the GPRS update status to GU3 ROAMING NOT ALLOWED and the update status to U3 ROAMING NOT ALLOWED (and shall store it according to section 4.1.3.2). Furthermore, it shall delete any P-TMSI, P-TMSI signature, TMSI, RAI, LAI, ciphering key sequence number and GPRS ciphering key sequence number and shall consider the SIM as invalid for GPRS and non-GPRS services until switching off or the SIM is removed.

# 11	(PLMN not allowed);

# 12	(Location area not allowed); or

# 13	(Roaming not allowed in this location area)

The MS shall delete any RAI or LAI, P-TMSI, P-TMSI signature and GPRS ciphering key sequence number, shall set the GPRS update status to GU3 ROAMING NOT ALLOWED (and shall store it according to section 4.1.3.2).

If the MS is IMSI attached via MM procedures, the MS shall in addition set the update status to U3 ROAMING NOT ALLOWED and shall delete any TMSI, LAI and ciphering key sequence number. The new MM state is MM IDLE.

The MS shall store the LAI or the PLMN identity in the appropriate forbidden list, i.e. in the “forbidden PLMN list” for cause #11, in the list of “forbidden location areas for regional provision of service” for cause #12 or in the list of “forbidden location areas for roaming” for cause #13. If #11or  #13 was received, the MS shall perform a PLMN selection instead of a cell selection.

Other cause values shall not impact the update status. Further actions of the MS are implementation dependent.

##### 4.7.5.1.4	Normal and periodic routing area updating procedure not accepted by the network

If the routing area updating cannot be accepted, the network sends a ROUTING AREA UPDATE REJECT message to the MS. An MS that receives a ROUTING AREA UPDATE REJECT message stops timer T3330 . The MS shall then take different actions depending on the received reject cause value:

# 3		(Illegal MS); or

# 6		(Illegal ME)

The MS shall set the GPRS update status to GU3 ROAMING NOT ALLOWED (and shall store it according to section 4.1.3.2) and enter the state GMM-DEREGISTERED. Furthermore, it shall delete any P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number and shall consider the SIM as invalid for GPRS services until switching off or the SIM is removed.

If the MS is IMSI attached via MM procedures, the MS shall in addition set the update status to U3 ROAMING NOT ALLOWED, shall delete any TMSI, LAI and ciphering key sequence number. The new MM state is MM idle. The SIM shall be considered as invalid also for non-GPRS services until switching off or the SIM is removed.

# 7		(GPRS services not allowed)

The MS shall set the GPRS update status to GU3 ROAMING NOT ALLOWED (and shall store it according to section 4.1.3.2.9) and shall delete any P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number. The SIM shall be considered as invalid for GPRS services until switching off or the SIM is removed. The new state is GMM-DEREGISTERED.

# 9	     (MS identity cannot be derived by the network), enter the state GMM-DEREGISTERED

The MS shall set the GPRS update status to GU2 NOT UPDATED (and shall store it according to section 4.1.3.2), and shall delete any P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number. Subsequently, the MS may automatically initiate the GPRS attach procedure.

# 10	(Implicitly detached)

The MS shall change to state GMM-DEREGISTERED.NORMAL-SERVICE. The MS shall then perform a new attach procedure  The MS should also activate PDP context(s) to replace any previously active PDP contexts.

NOTE (Re-)activation of PDP contexts  (in the case where dynamic IP addresses  are used)  is normally triggered by the user, and so cannot be done automatically by the MS.

# 11	(PLMN not allowed);

# 12	(Location area not allowed); or

# 13	(Roaming not allowed in this location area)

The MS shall delete any RAI or LAI, P-TMSI, P-TMSI signature and GPRS ciphering key sequence number, shall set the GPRS update status to GU3 ROAMING NOT ALLOWED (and shall store it according to section 4.1.3.2) and enter the state GMM-DEREGISTERED.

If the MS is IMSI attached via MM procedures, the MS shall in addition set the update status to U3 ROAMING NOT ALLOWED and shall delete any TMSI, LAI and ciphering key sequence number. The new MM state is MM IDLE.

The MS shall store the LAI or the PLMN identity in the appropriate forbidden list, i.e. in the “forbidden PLMN list” for cause #11, in the list of “forbidden location areas for regional provision of service” for cause #12 or in the list of “forbidden location areas for roaming” for cause #13. If #11or  #13 was received, the MS shall perform a PLMN selection instead of a cell selection.

Other values are considered as abnormal cases. The specification of the MS behaviour in those cases is given in described in section 4.7.5.1.5.

##### 4.7.5.2.4	Combined routing area updating not accepted by the network

If the combined routing area updating cannot be accepted, the network sends a ROUTING AREA UPDATE REJECT message to the MS. An MS that receives a ROUTING AREA UPDATE REJECT message stops timer T3330 and enters state MM IDLE. The MS shall then take different actions depending on the received reject cause:

# 3		(Illegal MS);

# 6		(Illegal ME); or

# 8		(GPRS services and non GPRS services not allowed)

The MS shall set the GPRS update status to GU3 ROAMING NOT ALLOWED and the update status to U3 ROAMING NOT ALLOWED (and shall store it according to section 4.1.3.2) and enter the state GMM-DEREGISTERED. Furthermore, it shall delete any P-TMSI, P-TMSI signature, TMSI, RAI, LAI, ciphering key sequence number and GPRS ciphering key sequence number and shall consider the SIM as invalid for GPRS and non GPRS services until switching off or the SIM is removed.

# 7	(GPRS services not allowed)

The MS shall set the GPRS update status to GU3 ROAMING NOT ALLOWED (and shall store it according to section 4.1.3.2.9) and shall delete any P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number. The SIM shall be considered as invalid for GPRS services until switching off or the SIM is removed. The new state is GMM-DEREGISTERED.

# 9		(MS identity cannot be derived by the network)

The MS shall set the GPRS update status to GU2 NOT UPDATED (and shall store it according to section 4.1.3.2), enter the state GMM-DEREGISTERED, and shall delete any P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number. Subsequently, the MS may automatically initiate the GPRS attach procedure.

# 10	(Implicitly detached)

The MS shall change to state GMM-DEREGISTERED.NORMAL-SERVICE. The MS shall then perform a new attach procedure The MS should also activate PDP context(s) to replace any previously active PDP contexts.

NOTE (Re-)activation of PDP contexts  (in the case where dynamic IP addresses  are used)  is normally triggered by the user, and so cannot be done automatically by the MS.

# 11	(PLMN not allowed);

# 12	(Location area not allowed); or

# 13	(Roaming not allowed in this location area)

The MS shall set the GPRS update status to GU3 ROAMING NOT ALLOWED and the update status to U3 ROAMING NOT ALLOWED (and shall store it according to section 4.1.3.2.9) and enter the state GMM-DEREGISTERED. Furthermore, it shall delete any P-TMSI, P-TMSI signature, TMSI, RAI, LAI, ciphering key sequence number GPRS ciphering key sequence number, reset the location update attempt counter, reset the routing area updating attempt counter and reset the GPRS attach attempt counter.

The MS shall store the LAI or the PLMN identity in the appropriate forbidden list, i.e. in the “forbidden PLMN list” for cause #11, in the list of “forbidden location areas for regional provision of service” for cause #12 or in the list of “forbidden location areas for roaming” for cause #13. If #11 or  #13 was received, the MS shall then perform a PLMN selection instead of a cell selection.

Other values are considered as abnormal cases. The specification of the MS behaviour in those cases is described in section 4.7.5.2.5.

#### 10.5.5.14	GMM cause

The purpose of the GMM cause information element is to indicate the reason why a GMM request from the mobile station is rejected by the network.

The GMM cause information element is coded as shown in figure 10.5.129/GSM 04.08 and table 10.5.147/GSM 04.08.

The GMM cause is a type 3 information element with 2 octets length.

|  | 8 7 6 5 4 3 2 1 |  |
| --- | --- | --- |
|  | GMM cause IEI | octet 1 |
|  | Cause value | octet 2 |

Figure 10.5.129/GSM 04.08: GMM cause information element

- Table 10.5.147/GSM 04.08: GMM cause information element

Cause value (octet 2) ª Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 1 0 IMSI unknown in HLR 0 0 0 0 0 0 1 1 Illegal MS 0 0 0 0 0 1 1 0 Illegal ME 0 0 0 0 0 1 1 1 GPRS services not allowed 0 0 0 0 1 0 0 0 GPRS services and non-GPRS services not allowed 0 0 0 0 1 0 0 1 MS identity cannot be derived by the network 0 0 0 0 1 0 1 0 Implicitly detached 0 0 0 0 1 0 1 1 PLMN not allowed 0 0 0 0 1 1 0 0 Location Area not allowed 0 0 0 0 1 1 0 1 Roaming not allowed in this location area 0 0 0 1 0 0 0 0 MSC temporarily not reachable 0 0 0 1 0 0 0 1 Network failure 0 0 0 1 0 1 1 0 Congestion 0 0 1 1 0 0 0 0 } to } retry upon entry into a new cell 0 0 1 1 1 1 1 1 } 0 1 0 1 1 1 1 1 Semantically incorrect message 0 1 1 0 0 0 0 0 Invalid mandatory information 0 1 1 0 0 0 0 1 Message type non-existent or not implemented 0 1 1 0 0 0 1 0 Message type not compatible with the protocol state 0 1 1 0 0 0 1 1 Information element non-existent or not implemented 0 1 1 0 0 1 0 0 Conditional IE error 0 1 1 0 0 1 0 1 Message not compatible with the protocol state 0 1 1 0 1 1 1 1 Protocol error, unspecified Any other value received by the mobile station shall be treated as 0110 1111, 'Protocol error,' unspecified'. Any other value received by the network shall be treated as 0110 1111, 'Protocol error, unspecified'. NOTE: The listed reject cause values are defined in Annex G.