ETSI SMG SMG3 WPA GPRS Ad Hoc	Tdoc N1-99441

Oslo, Norway

18-21 May 1999

| CHANGE REQUEST No : |  |  |  |  | A066 |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 04.64 |  | Version: |  |  | 6.3.0 |  |  |  |
| Submitted to SMG | # 29 |  | for approval |  | x |  | without presentation ("non-strategic") |  |  |  |  |  |
| list SMG plenary meeting no. here |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  | x |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Ericsson | Date: | 18 May 1999 |
| --- | --- | --- | --- |

| Subject: | Application of XID Reset parameter while LLC operation is suspended |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | x |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  |  |  |

| Reason for change: | The XID Reset parameter causes all LLEs in ABM state to revert to ADM state resulting in the loss of the values of the send and receive state variables, V(S) and V(R). Use of the XID Reset parameter during the Inter SGSN RAU procedure presents a problem. In this procedure the V(R) state variables must be exchanged between MS and SGSN to allow acknowledgement of correctly received I frames; if the value of the state variables is lost, the exchange of the state variables is worthless. However, LLC operation on SAPIs other than SAPI 1 is suspended during the Inter SGSN RAU. To overcome the problem of the V(R) values being lost this CR proposes that a Reset parameter while LLC operation is suspended shall not be applied to SAPIs other than SAPI 1 until LLC operaion is resumed. |
| --- | --- |

| Clauses affected: | 8.5.3.1 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

#### 8.5.3.1	Negotiation of parameter Reset

The Reset parameter shall be used, in the SGSN originating Reset and in the MS receiving Reset, to:

-	set all LLC layer parameters to the default values given in Table 9;

-	change any LLEs in ABM state to ADM state;

-	set the unconfirmed state variable V(U) to 0;

-	set the unconfirmed receive state variable V(UR) to 0; and

-	set the OCs for unacknowledged information transfer to 0.

The Reset parameter shall be treated before any additional XID parameters present in the same XID frame.

If LLC operation is suspended as a result of an LLGMM-SUSPEND-REQ the Reset shall not be applied to SAPIs other than SAPI 1 until LLC operation is resumed by an LLGMM-RESUME-REQ. The Reset shall be applied to SAPI 1 and LLC layer parameters that are common to all SAPIs immediately when Reset is received. When the LLGMM-RESUME-REQ is received the actions listed above shall be carried out on SAPIs other than SAPI 1 and the LLC layer parameters not common to all SAPIs.