ETSI SMG3 WPA GPRS ad-hoc	Tdoc N1-99464

Oslo													rev of tdoc 461

18, 23 May 99

| CHANGE REQUEST No : |  |  |  |  | A555r2 |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 4.08 |  | Version: |  |  | 6.3.0 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X |  | without presentation ("non-strategic") |  |  |  |  |  |
| list SMG plenary meeting no. here |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Alcatel | Date: | 17.03.1999 |
| --- | --- | --- | --- |

| Subject: | New DEREGISTERED ‘suspended’ sub-state |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | X |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  |  |  |

| Reason for change: | REGISTERED ‘suspended’ sub-state is used to specify the behaviour of the MS when a dedicated channel is established and the limitations of the MS make it unable to use RLC/MAC. The DEREGISTERED equivalent was missing and is added in this CR Suspended behaviour is corrected for REGISTERED state. |
| --- | --- |

| Clauses affected: | 4.1.3.1.2, 4.2.4.2 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

4.1.3.1.2	Substates of state GMM-DEREGISTERED

The GMM-DEREGISTERED state is subdivided into several substates as explained below. The substates pertain to the whole MS (ME alone if no SIM is inserted, or ME plus SIM). The selection of the appropriate substate depends on the GPRS update status, see section 4.1.3.2, and on the selected cell.

4.1.3.1.2.1	GMM-DEREGISTERED.NORMAL-SERVICE

Valid subscriber data is available, the GPRS update status is GU1 or GU2, a cell has been selected. In this state, a request for GPRS attach is performed using the stored temporary mobile subscriber identity for GPRS (P-TMSI), routing area identification (RAI) and GPRS ciphering key sequence number in case of GU1. If the GPRS update status is GU2, the IMSI shall be used to attach for GPRS services.

4.1.3.1.2.2	GMM-DEREGISTERED.LIMITED-SERVICE

Valid subscriber data is available, GPRS update status is GU3, and a cell is selected, which is known not to be able to provide normal service.

4.1.3.1.2.3	GMM-DEREGISTERED.ATTACH-NEEDED

Valid subscriber data is available and for some reason a GPRS attach must be performed as soon as possible. This state is usually of no duration, but can last, e.g. if the access class is blocked.

4.1.3.1.2.4	GMM-DEREGISTERED.ATTEMPTING-TO-ATTACH

The GPRS update status is GU2, a cell is selected, a previous GPRS attach was rejected. The execution of further attach procedures depends on the GPRS attach attempt counter. No GMM procedure except GPRS attach shall be initiated by the MS in this substate.

###### 4.1.3.1.2.5	GMM-DEREGISTERED.NO-IMSI

No valid subscriber data is available (no SIM, or the SIM is not considered valid by the ME) and a cell has been selected.

###### 4.1.3.1.2.6	GMM-DEREGISTERED.NO-CELL-AVAILABLE

No cell can be selected. This substate is entered after a first intensive search failed (substate PLMN SEARCH). Cells are searched for at a low rhythm. No services are offered.

###### 4.1.3.1.2.7	GMM-DEREGISTERED.PLMN-SEARCH

The mobile station is searching for PLMNs. This substate is left either when a cell has been selected (the new substate is NORMAL-SERVICE or LIMITED-SERVICE) or when it has been concluded that no cell is available at the moment (the new substate is NO-CELL-AVAILABLE).

###### 4.1.3.1.2.8	GMM-DEREGISTERED.SUSPENDED

The MS shall enter this substate when entering dedicated mode and the MS limitations make it unable to communicate on GPRS channels. The MS shall leave this substate when    leaving dedicated mode.

### 4.2.4	Behaviour in state GMM-DEREGISTERED

The state GMM-DEREGISTERED is entered when:

-	the MS is switched on;

-	the GPRS capability has been enabled in the MS;

-	a GPRS detach or combined GPRS detach procedure has been performed; or

-	a GMM procedure has failed (except routing area updating, see 4.7.5).

The selection of the appropriate substate of GMM-DEREGISTERED after switching on is described in section 4.2.4.1. The specific behaviour of the MS in state GMM-DEREGISTERED is described in section 4.2.4.2. The substate chosen when the GMM-DEREGISTERED state is returned to from another state except state GMM-NULL is described in section 4.2.4.3.

It should be noted that transitions between the various substates of GMM-DEREGISTERED are caused by (e.g.):

-	insertion or removal of the SIM;

-	cell selection/reselection (see also GSM 03.22 [14]);

-	PLMN search;

-	loss/regain of coverage; or

-	change of RA.

How various GMM procedures affect the GMM-DEREGISTERED substates and the GPRS update status is described in the detailed description of the GMM procedures in section 4.7.

#### 4.2.4.1	Primary substate selection

##### 4.2.4.1.1	Selection of the substate after power on or enabling the MS’s GPRS capability

When the MS is switched on, the substate shall be PLMN-SEARCH in case the SIM is inserted and valid. See GSM 03.22 [14] and 05.08 [34] for further details.

When the GPRS capability in an activated MS has been enabled, the selection of the GMM-DEREGISTERED substate depends on the MM state and the GPRS update status.

The substate chosen after PLMN-SEARCH, in case of power on or after enabling of the GPRS capability is:

-	if the cell is not supporting GPRS, the substate shall be NO-CELL-AVAILABLE;

-	if no SIM is present the substate shall be NO-IMSI;

-	if a cell supporting GPRS has been found and the PLMN or LA is not in the forbidden list, then the substate shall be NORMAL-SERVICE;

-	if the selected cell supporting GPRS is in a forbidden PLMN or a forbidden LA, then the MS shall enter the substate LIMITED-SERVICE;

-	if the MS is in manual network selection mode and no cell supporting GPRS of the selected PLMN has been found, the MS shall enter the substate NO-CELL-AVAILABLE.

##### 4.2.4.1.2	Other Cases

When the MM state is IDLE, the state PLMN-SEARCH shall also be entered in the following cases:

-	when a SIM is inserted in substate NO-IMSI;

-	when the user has asked for a PLMN selection in substate LIMITED-SERVICE;

-	when coverage is lost in substate NORMAL-SERVICE or substate LIMITED-SERVICE;

-	optionally, when the MS is in automatic network selection mode and the    maximum allowed number of subsequently unsuccessful attach attempts controlled by the GPRS attach attempt counter (section 4.7.3) have been performed.

-	when the MS is in automatic network selection mode and the    maximum allowed number of subsequently unsuccessful routing area update attempts controlled by the GPRS routing area update    attempt counter (section 4.7.5) have been performed.

#### 4.2.4.2	Detailed description of the MS behaviour in state GMM-DEREGISTERED

In state GMM-DEREGISTERED, the MS shall behave according to the substate. In the following sections, the behaviour is described for the non transient substates.

##### 4.2.4.2.1	Substate, NORMAL-SERVICE

The MS shall:

-	perform GPRS attach.

##### 4.2.4.2.2	Substate, ATTEMPTING-TO-ATTACH

The MS shall:

-	perform GPRS attach on the expiry of timers T3311 or T3302;

-	perform GPRS attach when the routing area of the serving cell has changed and the location area this cell is belonging to is not in the list of forbidden LAs;

-	if entry into this state was caused by b) or d) with cause “Retry upon entry into a new cell”,” of section 4.7.3.1.5 , GPRS attach shall be performed when a new cell is entered; and

-	if entry into this state was caused by c) or d) with cause different from ”Retry upon entry into a new cell” of section 4.7.3.1.5, GPRS attach shall not be performed when a new cell is entered.

##### 4.2.4.2.3	Substate, LIMITED-SERVICE

The MS shall:

-	perform GPRS attach when a cell is entered which may provide normal service (e.g. location area is not in one of the forbidden lists);

##### 4.2.4.2.4	Substate, NO-IMSI

The MS shall:

-	only perform default cell selection;

##### 4.2.4.2.5	Substate, NO-CELL

The MS shall:

-	perform cell selection according to GSM 03.22 [14] and shall choose an appropriate substate.

##### 4.2.4.2.6	Substate, PLMN-SEARCH

No specific action is required in this substate.

##### 4.2.4.2.7	Substate, ATTACH-NEEDED

The MS shall start a GPRS attach procedure if still needed as soon as the access class allows network contact in the selected cell.

##### 4.2.4.2.8	Substate, SUSPENDED

The MS :

-	shall not send any user data ; and

-	shall not send any signalling information.

###### NEXT MODIFICATIONS----

###### 4.1.3.1.3.2	GMM-REGISTERED.SUSPENDED

The MS shall enter this substate when entering dedicated mode and the MS limitations make it unable to communicate on GPRS channels. In this substate, no user data should be sent and no signalling information shall be sent. The MS shall leave this substate when leaving dedicated mode.

###### NEXT MODIFICATIONS----

##### 4.2.5.1.2	Substate, SUSPENDED

The MS:

-	shall not send any user data ; and

-	shall not send any signalling information.