ETSI SMG3 WPA GPRS ad-hoc																Tdoc N1-99488

Sophia Antipolis, France

June 1-4,	1999

Source:	Ericsson

Class B mode of operation mobiles in network mode of operation III

A GPRS mobile with a two radio receivers, operating in a mode III network, can properly monitor 100% both paging channels (packet and CS) in all circumstances.

A GPRS mobile with a single radio receiver, operating in a mode III network, cannot properly monitor 100% both paging channels (packet and CS) in all circumstances. It can 100% monitor the CS paging channel and partially monitor the packet paging channel, or vice versa, but some paging signals on the 2nd channel will be lost.

It maybe possible to restrict the allocation of CS paging channels and packet paging channels so that the single receiver GPRS mobile could properly monitor both, but such restrictions do not currently form part of the GSM standards. (Operators may object to such restrictions in any case).

Some technical details:

In packet idle mode, the mobile station shall monitor its paging blocks on both CCCH and PCCCH, when PCCCH is present in the cell. During the non-DRX mode, immediately following the transition from packet transfer to idle mode, it shall monitor all PCCCH blocks where paging may occur. Monitoring both CCCH and PCCCH may be possible, for instance, if the cell is configured with PCCCH on the adjacent timeslot to CCCH on the BCCH carrier. In a TDMA frame where both logical channels appears, the mobile station has then to receive bursts in both timeslots directly after each other.

On the other hand, if PCCCH is located on a different carrier than CCCH, possibly on a hopping carrier, the mobile station (only one receiver is assumed) has to switch between those carriers within the same TDMA frame. That is not possible if PCCCH is located at the same timeslot number, or the timeslot number immediately before or after CCCH (modulo 8). Even larger margin may be required, depending on the switching capabilities of the mobile station.

There is currently no particular restriction on the network where to put a PCCCH. It may use an hopping or non-hopping carrier and any timeslot number 0 to 7. It is possible to configure up to 16 PCCCHs within the cell, which means that it must be possible to put them on different carriers and it is highly desirable to use the full range of timeslot numbers. (There is a slight restriction, depending on which timeslot number the PBCCH is using, but that's a somewhat different issue.) The mobile stations select one PCCCH, depending on IMSI and certain cell parameters.

If a class-B mode of operation mobile station having only one receiver is to operate in a NMO-3 network, depending on the cell configuration, it may have to prioritize between CCCH and PCCCH. When a page block occurs on PCCCH, it might not be able to monitor CCCH, or vice versa. Since CCCH uses the 51-multiframe and PCCCH the 52-multiframe, page blocks will occur within the same TDMA frame repeatedly, regardless of cell parameters and mobile station DRX parameters.

Conclusion:

Unless the network choice of paging channels is deliberately configured to allow for this case (which surely would have to be standardised as different MSs would have different requirements), then, with the exception of class B mode of operation mobiles with 2 radio receivers, class B mode of operation mobiles will miss some paging messages in a mode III network.

For this reason, Ericsson recommends that all mobiles with a single radio receiver should revert to class C mode of operation (or CS only operation) in a mode III network. (This hence should be considered as the 'normal' behaviour for class B mode of operation mobiles). If it is desired the standards could be changed to this effect, but this paper merely serves to clarify the situation.