3GPP TSG-CT Meeting #106	CP-243250

# Madrid, Spain, 9th Dec 2024 - 10th Dec 2024										(revised to C4-245534)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0307 | rev | 5 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LMF Enhancements for LMF based AI/ML Positioning – Stage 3 aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Vivo, Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson, Vivo, Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Stage 2 (ref: CR#574 to 23.273; Tdoc: S2-2410978) has introduced new LMF services for data collection for NWDAF aide AI/ML positioning ML model training. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the new LMF services for data exposure that includes Notification, subscriptioon and unsubsription operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with Stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2,5.1, 5.x(new),5.x.1(new),5.x.2(new),6.x (new), 6.x.1(new),6.x.2(new),6.x.3(new),6.x.4(new),6.x.5(new),6.x.6(new),6.x.7(new),6.x.8(new),6.x.9(new), A.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.273 CR 0574 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces new backward compatible OpenAPI features to Nlmf_DataExposure API: - TS29572_Nlmf_DataExposure |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - Rev 1: - Addition of Co-signers - C4-245163,C4-245244 are merged with C4-245238 to form C4-245353 - Rev 2, 3: - Corrections. - Rev 4,5: OpenAPI correction |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[7]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)".

[8]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[10]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[11]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[12]	IETF RFC 9113: "HTTP/2".

[13]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[14]	OpenAPI Initiative, "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[15]	IETF RFC 9457: "Problem Details for HTTP APIs".

[16]	3GPP TR 21.900: "Technical Specification Group working methods".

[17]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[18]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[19]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[20]	3GPP TS 24.080: "Mobile radio interface layer 3 Supplementary services specification; Formats and coding".

[21]	3GPP TS 37.355: " LTE Positioning Protocol (LPP)".

[22]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[23]	3GPP TS 29.518: "Access and Mobility Management Services".

[24]	3GPP TS 29.171: "Location Services (LCS); LCS Application Protocol (LCS-AP) between the Mobile Management Entity (MME) and Evolved Serving Mobile Location Centre (E-SMLC); SLs interface".

[25]	IETF RFC 4119: "A Presence-based GEOPRIV Location Object Format".

[26]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[27]	3GPP TS 29.515: "5G System; Gateway Mobile Location Services Stage 3".

[28]	3GPP TS 29.515: "5G System; Gateway Mobile Location Services Stage 3".

[29]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[30]	3GPP TS 38.355: "NR; Sidelink Positioning Protocol (SLPP); Protocol Specification".

[xx]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

# * * * * Next change (new) * * * *

## 5.1	Introduction

The LMF offers to other NFs the following services:

-	Nlmf_Location

-	Nlmf_Broadcast

-	Nlmf_DataExposure

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Nlmf_Location | 6.1 | LMF Location Service | TS29572_Nlmf_Location.yaml | nlmf-loc | A.2 |
| Nlmf_Broadcast | 6.2 | LMF Broadcast Service | TS29572_Nlmf_Broadcast.yaml | nlmf-broadcast | A.3 |
| Nlmf_DataExposure | 6.x | LMF Data Exposure Service | TS29572_Nlmf_ DataExposure.yaml | nlmf-dataexposure | A.x |

# * * * * Next change (new) * * * *

## 5.x	Nlmf_DataExposure Service

### 5.x.1	Service Description

The Nlmf_DataExposure service enables an NF to subscribe to input data for requested location measurement data for ML training-based positioning.

### 5.x.2	Service Operation

#### 5.x.2.1	Introduction

The service operations defined for Nlmf_DataExposure service are defined as follows:

-	Lmf Data Exposure Subscription: It allows the consumer NF to subscribe to the input data used for ML model training or performance for LMF based AI/ML positioning.

-	Lmf Data Exposure Modification: It allows the NF consumer to modify the subscription used for ML model training or performance monitoring for LMF based AI/ML positioning.

-	Lmf Data Exposure Notification: It allows the consumer NF to get notification about the input data used for ML model training or performance monitoring for LMF based AI/ML positioning.

-	Lmf Data Exposure Unsubscription: It allows the consumer NF to cancel subscirption to input data used for ML model training or performance monitoring for LMF based AI/ML positioning.

#### 5.x.2.2	LMF Data Exposure Subscription

This procedure allows NF Service Consumer to subscribe by using HTTP POST method with the URI of individual subscription resource.

Figure 5.x.2.2-1: Subscription of NF service consumer to LMF data exposure

1.	The NF Service Consumer shall send an HTTP POST request to the resource URI associated with the subscription collection. The input parameters for the request shall include notification target address and may include notification correlation ID and AoI.

2a.	On success, "201 Created" shall be returned. The response body shall include a HTTP Location header including the subscription ID  together with the status code 201 indicating the requested resource is created in the response message.

2b.	On failure or redirection, one of the HTTP status codes listed in Table 6.X.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body should contain a ProblemDetails structure indicating appropriate additional error information.

#### 5.x.2.3	LMF Data Exposure Modification

This procedure allows NF Service Consumer to modify an existing subscription by using HTTP PATCH method with the URI of individual subscription resource.

Figure 5.x.2.3-1: Modification of subscription for LMF data exposure

1.	The NF Service Consumer shall send a PATCH request to modify a subscription resource in the LMF.

2a.	On success, "204 No Content" shall be returned.

2b.	On failure or redirection, one of the HTTP status codes listed in Table 6.x.3.3.3.1-3 shall be returned. For a 4xx/5xx response, the message body contains a ProblemDetails indicating appropriate additional error information.

#### 5.x.2.4	LMF Data Exposure Notification

This procedure allows a consumer NF to modify the notification about the input data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Figure 5.x.2.4-1: Notification for LMF data exposure

1.	LMF shall send a POST request to the NF Service Consumer with the information on Notification target address.The request body shall include sampling data and notification correlation ID, if received.

2a.	On success, "204 No content" shall be returned by the NF Service Consumer.

2b.	On failure or redirection, one of the appropriate HTTP status codes listed in Table 6.x.5.2.3-2 shall be returned. For a 4xx/5xx response, the message body should contain a ProblemDetails structure indicating appropriate additional error information.

#### 5.x.2.5	LMF Data Exposure Unsubscription

This procedure allows a consumer NF to delete the notification about the input data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Figure 5.x.2.5-1: Unsubscribe ServiceOperation for LMF data exposure

1.	The NF consumer shall send a Delete request to LMF to cancel the subscription to inpt data used for ML model training or model performance monitoring for LMF based AI/ML positioning.

2a.	 On success, "204 No content" shall be returned by the NF Service Consumer.

2b.	On failure or redirection, one of the appropriate HTTP status codes listed in Table 6.x.6.3.3.3.2-3 shall be returned. For a 4xx/5xx response, the message body should contain a ProblemDetails structure indicating appropriate additional error information.

# * * * * Next change (new) * * * *

## 6.x	Nlmf_DataExposure Service API

### 6.x.1	API URI

The Nlmf_DataExposure service shall use the Nlmf_DataExposure API.

The API URI of the Nlmf_DataExposure API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URI used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nlmf-dataexposure".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.x.3.

### 6.x.2	Usage of HTTP

#### 6.x.2.1	General

HTTP/2, as defined in IETF RFC 9113 [12], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

HTTP messages and bodies for the Nlmf_ DataExposure service shall comply with the OpenAPI [14] specification contained in Annex A.

#### 6.x.2.2	HTTP Standard Headers

##### 6.x.2.2.1	General

##### 6.x.2.2.2	Content type

The following content types shall be supported:

-	JSON, as defined in IETF RFC 8259 [13], shall be used as content type of the HTTP bodies specified in the present specification as indicated in clause 5.4 of 3GPP TS 29.500 [4].

-	The Problem Details JSON Object (IETF RFC 9457 [15]). The use of the Problem Details JSON object in a HTTP response body shall be signalled by the content type "application/problem+json".

#### 6.x.2.3	HTTP custom headers

##### 6.x.2.3.1	General

This API does not define any new HTTP custom headers.

#### 6.x.2.4	HTTP multipart messages

HTTP multipart messages shall include one JSON body part and one or more binary body parts.

The JSON body part shall be the "root" body part of the multipart message. It shall be encoded as the first body part of the multipart message. The "Start" parameter does not need to be included.

The multipart message shall include a "type" parameter (see IETF RFC 2387 [9]) specifying the media type of the root body part, i.e. "application/json". Please refer to clause 6.x.6.4 for more details.

# * * * * Next change (new) * * * *

### 6.x.3	Resources

#### 6.x.3.1	Overview

The structure of the Resource URIs of the Nlmf_DataExposure service is shown in figure 6.x.3.1-1

Figure 6.x.3.1-1: Resource URI structure of the Nlmf_DataExposure API

Table 6.x.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.x.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| subscriptions (Collection) | /subscriptions | POST | Mapped to the service operation subscribe, when to create a subscription |
| subscriptionid (Document) | /subscriptions/{subscriptionId} | PATCH | Modify the subscription identified by {subscriptionId} |
|  |  | DELETE | Mapped to the service operation Unsubscribe |

#### 6.x.3.2	Resource: subscriptions (Collection)

##### 6.x.3.2.1	Description

This resource represents a collection of subscriptions created by NF service consumers of Nlmf_DataExposure service.

This resource is modelled as the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [5]).

##### 6.x.3.2.2	Resource Definition

Resource URI: {apiRoot}/nlmf-dataexposure/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.x.3.2.2-1.

Table 6.2.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.x.1 |

##### 6.x.3.2.3	Resource Standard Methods

###### 6.x.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.x.3.2.3.1-1.

Table 6.x.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.x.3.2.3.1-2 and the response data structures and response codes specified in table 6.X.3.2.3.1-3.

Table 6.X.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LmfDataExposureSubscription | M | 1 | LMF Data Exposure Subscription to be created |

Table 6.X.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LmfDataExposureSubscription | M | 1 | 201 Created | Represents successful creation of an LMF Subscription |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| NOTE: The mandatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.x.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nlmf-dataexposure/<apiVersion>/subscriptions/{subscriptionId} |

Table 6.x.3.2.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same LMF or LMF (service) set. For the case when a request is redirected to the same target resource via a different SCP or SEPP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.x.3.2.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same LMF or LMF (service) set. For the case when a request is redirected to the same target resource via a different SCP or SEPP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

#### 6.x.3.3	Resource: subscriptionId (Document)

##### 6.x.3.3.1	Description

This resource represents a collection of subscriptions created by NF service consumers of Nlmf_DataExposure service.

This resource is modelled as the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [5]).

##### 6.x.3.3.2	Resource Definition

Resource URI: {apiRoot}/nlmf-dataexposure/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.x.3.3.2-1.

Table 6.x.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.x.1 |
| apiVersion | string | See clause 6.x.1. |
| subscriptionId | string | Represents the identity of a specific subscription |

##### 6.x.3.3.3	Resource Standard Methods

###### 6.x.3.3.3.1	PATCH

This method shall support the URI query parameters specified in table 6.x.3.3.3.1-1.

Table 6.x.3.3.3.1-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.x.3.3.3.1-2 and the response data structure and response codes specified in table 6.x.3.3.3.1-3.

Table 6.x.3.3.3.1-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | It contains the list of changes to be made to the subscription, according to the JSON PATCH format specified in IETF RFC 6902 [xx]. |

Table 6.x.3.3.3.1-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Represents a successful update on LMF Data Exposure Subscription |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP or SEPP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.x.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same LMF or LMF (service) set. For the case when a request is redirected to the same target resource via a different SCP or SEPP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.x.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same LMF or LMF (service) set. For the case when a request is redirected to the same target resource via a different SCP or SEPP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

###### 6.x.3.3.3.2	DELETE

This method shall support the URI query parameters specified in table 6.x.3.3.3.2-1.

Table 6.x.6.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.x.3.3.3.2-2 and the response data structures and response codes specified in table 6.x.3.3.3.2-3.

Table 6.x.6.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.x.6.3.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content |  |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP or SEPP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.x.6.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same LMF or LMF (service) set. For the case when a request is redirected to the same target resource via a different SCP or SEPP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.x.6.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same LMF or LMF (service) set. For the case when a request is redirected to the same target resource via a different SCP or SEPP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

### 6.x.4	Custom Operations without associated resources

There are no custom operations without associated resources supported on Nlmf_DataExposure Service.

### 6.x.5	Notifications

#### 6.x.5.1	Description

This clause specifies the notifications provided by the Nlmf_DataExposure service.

Table 6.x.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| LMF Data Exposure Notification | {NotificationURI} | POST |  |

#### 6.x.5.2	LMF Data Exposure Notification

A NF Service Consumer may be subscribed to obtain notification about the input data used for ML model training or ML model performance monitoring for LMF-based AI/ML positioning. In this case, the LMF shall notify the NF Service Consumer by using the HTTP POST method and deliver the notification to the call-back URI, following Subscribe/Notify mechanism defined in 3GPP TS 29.501 [5].

##### 6.x.5.2.1	Notification Definition

Call-back URI: {notificationUri}

Call-back URI is provided by NF Service Consumer during creation of the subscription. The description of the Callback URI of NF consumer is obtained by LMF is provided in clause 5.x.2.3.

##### 6.x.5.2.3	Notification Standard Methods

###### 6.x.5.2.3.1	POST

This method shall support the request data structures specified in table 6.x.5.2.3.1-1 and the response data structures and response codes specified in table 6.x.5.2.3.1-2.

Table 6.x.5.2.3.1-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LmfDataExposureNotification | M | 1 | Represents the notification to be delivered |

Table 6.x.5.2.3.1-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content |  |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP or SEPP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.x.5.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of the NF service consumer to which the notification should be sent. For the case when a request is redirected to the same target resource via a different SCP or SEPP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. For the case when a request is redirected to the same target resource via a different SCP or SEPP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |

Table 6.x.5.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of the NF service consumer to which the notification should be sent |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

# * * * * Next change (new) * * * *

### 6.x.6	Data Model

#### 6.x.6.1	General

This clause specifies the application data model supported by the API.

Table 6.x.6.1-1 specifies the data types defined for the Nlmf_DataExposure service-based interface protocol.

Table 6.x.6.1-1: Nlmf_DataExposure specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| LmfDataExposureSubscription | 6.x.6.2.2 | Represents an individual data collection subscription resource on LMF |
| LmfDataExposureNotification | 6.x.6.2.4 | Notify within an LMF exposure for data collection notification request. |
| LmfDataExposureReport | 6.x.6.2.5 | Represents the data collection exposure report. |

Table 6.x.6.1-2 specifies data types re-used by the Nlmf_DataExposure service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nlmf_DataExposure service based interface.

Table 6.x.6.1-2: Nlmf_DataExposure re-used Data Types

| Data type | Reference | Description |
| --- | --- | --- |
| PresenceInfo | 3GPP TS 29.571 [8] | Area of Interest |
| Uri | 3GPP TS 29.571 [8] | URI |
| RedirectResponse | 3GPP TS 29.571 [8] | Redirect Response |
| DateTime | 3GPP TS 29.571 [8] | Date time |
| PatchItem | 3GPP TS 29.571 [8] | Patch item |
| TimeWindow | 3GPP TS 29.122 [29] | Time window |

#### 6.x.6.2	Structured data types

##### 6.x.6.2.1	Introduction

Structured data types used in Nlmf_DataExposure service are specified in this clause.

##### 6.x.6.2.2	Type: LmfDataExposureSubscription

Table 6.x.6.2.2-1: Definition of type LmfDataExposureSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notificationUri | Uri | M | 1 | This IE shall be included to identify the recipients of notifications sent by LMF for data exposure services. |  |
| notifyCorrelationId | CorrelationID | M | 1 | This IE shall be included to identify the notification correlation ID. The LMF shall include this ID in the notifications. The value of this IE shall be unique per subscription for a given NF service consumer. |  |
| aoi | PresenceInfo | M | 1 | Area of Interest |  |
| timeWindow | TimeWindow | O | 0..1 | Data collection window. |  |
| noSamples | Uinteger | O | 0..1 | Number of samples. |  |

##### 6.x.6.2.3	Type: LmfDataExposureNotification

Table 6.x.6.2.4-1: Definition of type LmfDataExposureNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifyCorrelationId | CorrelationID | M | 1 | When present, this IE shall indicate the notification correlation ID during notify service operation. |  |
| subscriptionCorrelationId | Uri | C | 0..1 | When present, this IE shall indicate the notification for the assignment of a new Subscription Correlation ID by the LMF. |  |
| reports | array(LmfDataExposureReport) | M | 1..N | The report for the data collection. |  |

##### 6.x.6.2.4	Type: LmfDataExposureReport

Table 6.x.6.2.4-1: Definition of type LmfDataExposureReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| timeStamp | DateTime | M | 1 | This IE shall contain the UTC time. |  |
| samplingData | FFS | FFS | FFS | FFS |  |

Editor's note: The data samples used for LMF-based AI/ML Positioning will be decided by RAN WGs.

#### 6.x.6.3	Simple data types

##### 6.x.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.x.6.3.2	Simple data types

The simple data types defined in table 6.x.6.3.2-1 shall be supported.

Table 6.x.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |

#### 6.x.6.4	Binary data types

##### 6.x.6.4.1	Introduction

This clause defines the binary data that shall be supported in a binary body part in an HTTP multipart message.

##### 6.x.6.4.2	Binary data types

Table 6.x.6.4.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

Editor's note: The data samples used for LMF-based AI/ML Positioning will be decided by RAN WGs.

# * * * * Next change (new) * * * *

### 6.x.7	Error Handling

#### 6.x.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

#### 6.x.7.2	Protocol Errors

Protocol Error Handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [4].

# * * * * Next change (new) * * * *

### 6.x.8	Security

As indicated in 3GPP TS 33.501 [27], the access to the Nlmf_DataExposure API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [28]), using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [29]) plays the role of the authorization server.

If Oauth2 authorization is used, an NF Service Consumer, prior to consuming services offered by the Nlmf_ Nlmf_DataExposure API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [29], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nlmf_DataExposure service.

The Nlmf_DataExposure API defines scopes for OAuth2 authorization as specified in 3GPP TS 33.501 [27];

Table 6.x.8-1: OAuth2 scopes defined in Nlmf_DataExposure API

| Scope | Description |
| --- | --- |
| "nlmf-dataexposure" | Access to the Nlmf_DataExposure API. |

# * * * * Next change (new) * * * *

### 6.x.9	HTTP redirection

An HTTP request may be redirected to a different LMF service instance, within the same LMF or a different LMF of an LMF set, e.g. when an LMF service instance is part of an LMF (service) set or when using indirect communications (see 3GPP TS 29.500 [4]). See the ES3XX feature in clause 6.2.8.

An SCP that reselects a different LMF producer instance will return the NF Instance ID of the new LMF producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an LMF within an LMF set redirects a service request to a different LMF of the set using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new LMF towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

# * * * * Next change (new) * * * *

# A.X	Nlmf_DataExposure API

openapi: 3.0.0

info:

version: 1.0.0-alpha.1

title: Nlmf_DataExposure

description: |

LMF Data Exposure Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

security:

- {}

- oAuth2ClientCredentials:

- nlmf-dataexposure

externalDocs:

description: 3GPP TS 29.572 V19.1.0; 5G System; Location Management Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.572/'

servers:

- url: '{apiRoot}/nlmf-dataexposure/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

paths:

/subscriptions:

post:

summary: Nlmf_Data Exposure Subscribe service Operation

tags:

- Subscriptions(Collection)

operationId: CreateSubscription

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/LmfDataExposureSubscription'

required: true

responses:

'201':

description: Subsription Created

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/nlmf-dataexposure/<apiVersion>/subscriptions/{subscriptionId}

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/LmfDataExposureSubscription'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

onLmfDataExposureSubscriptionNotification:

'{$request.body#/subscription/notificationUri}':

post:

summary: LMF Data Exposure Notification

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/LmfDataExposureNotification'

required: true

responses:

'204':

description: Successful acknowledgement

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

patch:

summary: Nlmf_DataExposure Subscribe Modify service Operation

tags:

- Individual subscription (Document)

operationId: ModifySubscription

parameters:

- name: subscriptionId

in: path

required: true

description: Unique ID of the subscription to be modified

schema:

type: string

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'204':

description: Expected response to a successful subscription modification

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

delete:

summary: Nlmf_DataExposure Unsubscribe service Operation

tags:

- Individual subscription (Document)

operationId: DeleteSubscription

parameters:

- name: subscriptionId

in: path

required: true

description: Deletion of Subscription

schema:

type: string

responses:

'204':

description: Subsription deleted successfully

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nlmf-dataexposure: Access to the Nlmf_DataExposure API

schemas:

#

# STRUCTURED TYPES

#

LmfDataExposureSubscription:

description: Describes subscription operation of LMF data exposure

type: object

properties:

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifyCorrelationId:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CorrelationID'

aoi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

timeWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

required:

- notificationUri

- notifyCorrelationId

- aoi

LmfDataExposureNotification:

description: Describes Notification for LMF data collection

properties:

notifyCorrelationId:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CorrelationID'

subscriptionCorrelationId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

reports:

type: array

items:

$ref: '#/components/schemas/LmfDataExposureReport'

minItems: 1

required:

- notifyCorrelationId

- reports

LmfDataExposureReport:

description: Describes an LMF data collection exposure report

properties:

timestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- timestamp

# * * * * End of changes * * * *