3GPP TSG-CT3 Meeting #139	C3-250467

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025							revision of C3-250243

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0388 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement the API invoker onboarding with onboard criteria |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the stage-2 the requirement is to enhance the API invoker onboarding procedure to incorporate onboarding criteria provided by the API invoker. This enhancement will ensure that the CCF verifies the security methods specified by the API invoker before proceeding with the onboarding process |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to enhance the API invoker onboadring procedure and the respective data models and API definition with onboard criteria information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency between stage-2 and stage-3 requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.4.1, 8.4.4.2.5, 8.4.4.2.9(new), 8.4.4.2.10(new), 8.4.6, A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR provides backward compatible feature updates to the Open API TS29222_CAPIF_API_Invoker_Management_API.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st Change * * * *

#### 8.4.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 also apply to this API.

Table 8.4.4.1-1 specifies the data types defined specifically for the CAPIF_API_Invoker_Management_API service.

Table 8.4.4.1-1: CAPIF_API_Invoker_Management_API specific Data Types

| Data type | Section defined | Description | Applicability |  |
| --- | --- | --- | --- | --- |
| APIInvokerEnrolmentDetails | Clause 8.4.4.2.2 | Represents the onboarding information of the API Invoker. |  |  |
| APIInvokerEnrolmentDetailsPatch | Clause 8.4.4.2.8 | Represents the requested modifications to an On-boarded API Invoker data. | PatchUpdate |  |
| APIList | Clause 8.4.4.2.4 | Represents the list of service APIs that the API Invoker is allowed to invoke. |  |  |
| OnboardingCriteria | Clause 8.4.4.2.9 | Represents the onboarding criteria information. | CAPIF_Ext1 |
| OnboardingInformation | Clause 8.4.4.2.5 | Represents on-boarding information of the API Invoker. |  |  |
| OnboardingNotification | Clause 8.4.4.2.7 | Represents the notification of the on-boarding creation or update result. |  |  |
| RelatedCriteria | Clause 8.4.4.2.10 | Represents onboarding related criteria. | CAPIF_Ext1 |

Table 8.4.4.1-2 specifies data types re-used by the CAPIF_API_Invoker_Management_API service.

Table 8.4.4.1-2: CAPIF_API_Invoker_Management_API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.122 [14] | Used to indicate a date and a time. |  |
| DateTimeRm | 3GPP TS 29.122 [14] | Used to indicate the same as the DateTime data structure but with the OpenAPI "nullable: true" property. |  |
| SecurityMethod | Clause 8.2.4.3.6 | Represents the security method. |  |
| SupportedFeatures | 3GPP TS 29.571 [19] | Used to negotiate the applicability of optional features defined in table 8.4.6-1. |  |

# * * * * 2th Change * * * *

##### 8.4.4.2.5	Type: OnboardingInformation

Table 8.4.4.2.5-1: Definition of type OnboardingInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiInvokerPublicKey | string | M | 1 | Contains the Public Key of the API Invoker. |  |
| apiInvokerCertificate | string | O | 0..1 | Contains the API Invoker's generic client certificate. The subject field in the certificate shall be encoded with the API Invoker ID as Common Name as specified in IETF RFC 5280 [29]. |  |
| onboardingSecret | string | O | 0..1 | Contains the API Invoker's onboarding secret, provided by the CCF. |  |
| onboardingCriteria | array(OnboardingCriteria) | O | 1..N | Contains the onboarding criteria information necessary for the onboarding of the API Invoker. | CAPIF_Ext1 |

# * * * * 3rd Change * * * *

##### 8.4.4.2.9	Type: OnboardingCriteria

Table 8.4.4.2.9-1: Definition of type OnboardingCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| secMethods | array(SecurityMethod) | M | 1..N | Contains the security methods requested to be supported. |  |
| relatedCrieria | RelatedCriteria | O | 0..1 | Contains the conditions under which the security methods provided within the "secMethods" attribute shall apply. |  |

# * * * * 4th Change * * * *

##### 8.4.4.2.10	Type: RelatedCriteria

Table 8.4.4.2.10-1: Definition of type RelatedCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aefIds | array(string) | C | 1..N | Contains the list of AEF identifier(s). (NOTE) |  |
| apis | FFS | C | 0..1 | Contains the API information (e.g, apiName, apiVersion). (NOTE) |  |
| aefTypes | FFS | C | 0..1 | Contains the AEF type(s). (NOTE) |  |
| serviceAPICategories | array(string) | C | 0..1 | Contains the service API category(ies). (NOTE) |  |
| NOTE: At least one of these attributes shall be present. The attributes "apis" attribute and "serviceAPICategories" shall not be present at the same time. |  |  |  |  |  |

Editor’s NOTE: The requirement for defining the "apis" and the "aefTypes" attribute is FFS. It is FFS the full content of this data type and which combinations of the attributes make sense.

# * * * * 5th Change * * * *

### 8.4.6	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.4.6-1 lists the supported features for CAPIF_API_Invoker_Management_API.

Table 8.4.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Indicates the support of the testing of notification connection i according to clause 7.6. |
| 2 | Notification_websocket | Indicates the support of the delivery of notifications over Websocket according to clause 7.6. This feature requires the support of the "Notification_test_event" feature. |
| 3 | PatchUpdate | Indicates the support of the PATCH method for updating an "Individual On-boarded API Invoker" resource. |
| 4 | ExpirationTime | Indicates the support of expiration time for the API invoker onboarding functionality as part of the support of network slice capability exposure application layer framework. This feature enables the following functionalities: - provisioning/updating/deleting the expiration time of an onboarding. |
| 5 | CAPIF_Ext1 | Indicates the support of onboard criteria information for the API invoker onboarding functionality. This feature enables the following functionalities: - provisioning/updating the onboarding criteria information of an onboarding. |

# * * * * 6th Change * * * *

# A.5	CAPIF_API_Invoker_Management_API

openapi: 3.0.0

info:

title: CAPIF_API_Invoker_Management_API

version: 1.4.0-alpha.2

description: |

API for API invoker management.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.222 V19.1.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/api-invoker-management/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222

paths:

/onboardedInvokers:

post:

summary: Request the Creation of a new On-boarded API Invoker.

operationId: CreateOnboardedAPIInvoker

tags:

- On-boarded API Invokers (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

responses:

'201':

description: >

Created. The API Invoker is successfully on-boarded.

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

headers:

Location:

description: >

Contains the URI of the newly created resource.

required: true

schema:

type: string

'202':

description: >

Accepted. The CCF accepted the Onboarding request and is processing it.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

notificationDestination:

'{$request.body#/notificationDestination}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/OnboardingNotification'

responses:

'204':

description: >

No Content. The onboarding notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/onboardedInvokers/{onboardingId}:

parameters:

- name: onboardingId

in: path

required: true

schema:

type: string

delete:

description: Deletes an existing Individual On-boarded API Invoker.

summary: Delete an existing Individual On-boarded API Invoker resource.

operationId: DeleteIndOnboardedAPIInvoker

tags:

- Individual On-boarded API Invoker (Document)

responses:

'204':

description: >

No Content. The Individual On-boarded API Invoker resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual On-boarded API Invoker resource.

operationId: UpdateIndOnboardedAPIInvoker

tags:

- Individual On-boarded API Invoker (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

responses:

'200':

description: >

OK. The Individual On-boarded API Invoker resource is successfully updated and the

representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

'202':

description: >

Accepted. The request is accepted and under processing.

'204':

description: >

No Content. The Individual On-boarded API Invoker resource is successfully updated

and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

operationId: ModifyIndApiInvokeEnrolment

tags:

- Individual On-boarded API Invoker (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetailsPatch'

responses:

'200':

description: >

OK. The Individual On-boarded API Invoker resource is successfully modified and the

representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

'202':

description: >

Accepted. The request is accepted and under processing.

'204':

description: >

No Content. The Individual On-boarded API Invoker resource is successfully modified

and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

OnboardingInformation:

type: object

description: Represents the on-boarding information of the API Invoker.

properties:

apiInvokerPublicKey:

type: string

description: The API Invoker's public key

apiInvokerCertificate:

type: string

description: >

The API Invoker's generic client certificate, provided by the CAPIF core function.

onboardingSecret:

type: string

description: >

The API Invoker's onboarding secret, provided by the CAPIF core function.

onboardingCriteria:

type: array

items:

$ref: '#/components/schemas/OnboardingCriteria'

minItems: 1

required:

- apiInvokerPublicKey

APIList:

type: object

description: Represents a list of APIs with the corresponding information.

properties:

serviceAPIDescriptions:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceAPIDescription'

minItems: 1

description: >

Represents the list of service APIs that the API Invoker is allowed to invoke.

APIInvokerEnrolmentDetails:

description: Represents the onboarding information of the API Invoker.

type: object

properties:

apiInvokerId:

type: string

description: >

API invoker ID assigned by the CAPIF core function to the API invoker while

on-boarding the API invoker. Shall not be present in the HTTP POST request

from the API invoker to the CAPIF core function, to on-board itself. Shall be

present in all other HTTP requests and responses.

readOnly: true

onboardingInformation:

$ref: '#/components/schemas/OnboardingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: >

Set to true to request the CCF to send a

test notification as defined in in clause 7.6.

Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

apiList:

$ref: '#/components/schemas/APIList'

apiInvokerInformation:

type: string

description: >

Generic information related to the API invoker such as details of

the device or the application.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- onboardingInformation

- notificationDestination

OnboardingNotification:

type: object

description: Represents a notification of on-boarding creation or update result.

properties:

result:

type: boolean

description: Set to "true" to indicate successful on-boarding. Otherwise set to "false".

resourceLocation:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

apiInvokerEnrolmentDetails:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

apiList:

$ref: '#/components/schemas/APIList'

required:

- result

APIInvokerEnrolmentDetailsPatch:

type: object

description: Represents an API Invoker's enrolment details to be updated.

properties:

onboardingInformation:

$ref: '#/components/schemas/OnboardingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

apiList:

$ref: '#/components/schemas/APIList'

apiInvokerInformation:

type: string

description: >

Generic information related to the API invoker such as details of

the device or the application.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTimeRm'

OnboardingCriteria:

type: object

description: Represents the onboarding criteria information.

properties:

secMethods:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/SecurityMethod'

minItems: 1

relatedCrieria:

$ref: '#/components/schemas/RelatedCrieria'

required:

- secMethods

RelatedCriteria:

type: object

description: Represents onboarding related criteria.

properties:

aefIds:

type: string

serviceAPICategories:

type: array

items:

type: string

minItems: 1

anyOf:

- required: [aefIds]

- required: [serviceAPICategories]

# * * * * End of Change * * * *