3GPP TSG-CT3 Meeting #139	C3-250468

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025								revision of C3-250244

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0389 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement the CAPIF event with onboard criteria |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the stage-2 the requirement is to update the CAPIF events to support new event from CCF to API invoker when the onboarding criteria from API invoker is no longer met. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to update with a new CAPIF event to notify API invoker by CCF when onboarding criteria is not met. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency between stage-2 and stage-3 requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.2.2, 5.4.2.4.2, 8.3.4.2.3, 8.3.4.2.5, 8.3.4.4.4, 8.3.6, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR provides backward compatible feature updates to the Open API TS29222_CAPIF_Events_API.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st Change * * * *

##### 5.4.2.2.2	Subscribing to CAPIF events using Subscribe_Event service operation

To subscribe to CAPIF events, the Subscriber shall send an HTTP POST message to the CAPIF core function. The body of the HTTP POST message shall include Subscriber's Identifier, Event Type and a Notification Destination URI as specified in clause 8.3.2.2.3.1.

For all events included in the HTTP POST message, if the Enhanced_event_report feature is supported, the Subscriber may include an event report requirement in the "eventReq" attribute including:

-	event notification method (periodic, one time, on event detection) in the "notifMethod" attribute;

-	maximum Number of Reports in the "maxReportNbr" attribute;

-	monitoring duration in the "monDur" attribute;

-	repetition period for periodic reporting in the "repPeriod" attribute; and/or

-	immediate reporting indication in the "immRep" attribute.

If the Enhanced_event_report feature is supported, the Subscriber may also include an event filter in the "eventFilters" attribute. The "eventFilters" attribute shall include:

-	if the event is SERVICE_API_AVAILABLE, SERVICE_API_UNAVAILABLE or SERVICE_API_UPDATE, the API IDs in the "apiIds" attribute;

-	if the event is API_INVOKER_ONBOARDED or API_INVOKER_OFFBOARDED or API_INVOKER_UPDATED, the API invoker IDs in the "apiInvokerIds" attribute;

-	if the event is ACCESS_CONTROL_POLICY_UPDATE, the API invoker IDs in the "apiInvokerIds" attribute and/or API identifications in the "apiIds" attribute;

-	if the event is SERVICE_API_INVOCATION_SUCCESS or SERVICE_API_INVOCATION_FAILURE, the API invoker IDs in the "apiInvokerIds" attribute, AEF identifiers in the "aefIds" attribute and/or API IDs in the "apiIds" attribute; and/or

-	if the "CAPIF_Ext1" feature is supported and the event is API_INVOKER_ONBOARDING_CRI_FAILED, the API invoker ID(s) within the "apiInvokerIds" attribute, the AEF identifier(s) within the "aefIds" attribute and/or the API ID(s) within the "apiIds" attribute.

Upon receiving the above described HTTP POST message, the CAPIF core function shall:

1.	verify the identity of the Subscriber and check if the Subscriber is authorized to subscribe to the CAPIF events mentioned in the HTTP POST message;

2.	if the Subscriber is authorized to subscribe to the CAPIF events, the CAPIF core function shall:

a.	create a new resource as specified in clause 8.3.2.1; and

b.	return the CAPIF Resource URI in the response message;

and

3.	if errors occur when processing the request, the CAPIF core function shall respond to the Subscriber with an appropriate error status code as defined in clause 8.3.5.

# * * * * 2nd Change * * * *

##### 5.4.2.4.2	Notifying CAPIF events using Notify_Event service operation

To notify CAPIF events, the CAPIF core function shall send an HTTP POST message using the Notification Destination URI received in the subscription request. The body of the HTTP POST message shall include an Event Notification and CAPIF Resource URI.

If the Enhanced_event_report feature is supported, the CAPIF core function may include an event detail in the "eventDetail" attribute. The "eventDetail" attribute shall include:

-	if the event is SERVICE_API_AVAILABLE or SERVICE_API_UNAVAILABLE, the API IDs in the "apiIds" attribute and, if the "ApiStatusMonitoring" feature is supported, additionally, the service API information in the "serviceAPIDescriptions" attribute;

-	if the event is SERVICE_API_UPDATE, the API information in the "serviceAPIDescriptions" attribute;

-	if the event is API_INVOKER_ONBOARDED or API_INVOKER_OFFBOARDED or API_INVOKER_UPDATED, the API invoker IDs in the "apiInvokerIds" attribute;

-	if the event is ACCESS_CONTROL_POLICY_UPDATE, the access control policy information in the "accCtrlPolList" attribute;

-	if the event is SERVICE_API_INVOCATION_SUCCESS or SERVICE_API_INVOCATION_FAILURE, the API invocation logs in the "invocationLogs" attribute;

-	if the event is API_TOPOLOGY_HIDING_CREATED or API_TOPOLOGY_HIDING_REVOKED, the API topology hiding information in the "apiTopoHide" attribute; or

-	if the event is API_INVOKER_ONBOARDING_CRI_FAILED, the onboard criteria information within the "onboardingCriteria" attribute.

Upon receiving the HTTP POST message, the Subscriber shall process the Event Notification.

# * * * * 3rd Change * * * *

##### 8.3.4.2.3	Type: EventNotification

Table 8.3.4.2.3-1: Definition of type EventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscriptionId | string | M | 1 | Identifier of the subscription resource to which the notification is related – CAPIF resource identifier |  |
| Events | CAPIFEvent | M | 1 | Notifications of individual events |  |
| eventDetail | CAPIFEventDetail | C | 0..1 | Detailed information for the event. (NOTE) | Enhanced_event_report |
| NOTE: Within the CAPIFEventDetail data type, the "serviceAPIDescriptions" attribute shall be provided if the event is SERVICE_API_UPDATE, the "apiIds" attribute shall be provided if the event is SERVICE_API_AVAILABLE, SERVICE_API_UNAVAILABLE, the "apiInvokerIds" attribute shall be provided only if the event is attribute shall be provided if the event is API_INVOKER_ONBOARDED or API_INVOKER_OFFBOARDED, or API_INVOKER_UPDATED, the "accCtrlPolList" attribute shall be provided if the event is ACCESS_CONTROL_POLICY_UPDATE, the "invocationLogs" attribute shall be provided if the event is SERVICE_API_INVOCATION_SUCCESS or SERVICE_API_INVOCATION_FAILURE, the "apiTopoHide" attribute shall be provided if the event is API_TOPOLOGY_HIDING_CREATED or API_TOPOLOGY_HIDING_REVOKED, the "onboardingCriteria" attribute shall be provided if the event is API_INVOKER_ONBOARDING_CRI_FAILED. |  |  |  |  |  |

# * * * * 4th Change * * * *

##### 8.3.4.2.5	Type: CAPIFEventDetail

Table 8.3.4.2.5-1: Definition of type CAPIFEventDetail

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serviceAPIDescriptions | array(ServiceAPIDescription) | O | 1..N | Description of the service API as published by the APF. |  |
| apiIds | array(string) | O | 1..N | API identifiers. |  |
| apiInvokerIds | array(string) | O | 1..N | API invokers that are onboarded/offboarded. |  |
| accCtrlPolList | AccessControlPolicyListExt | O | 0..1 | Access control policy updated list. |  |
| invocationLogs | array(InvocationLog) | O | 1..N | Invocation logs |  |
| apiTopoHide | TopologyHiding | O | 0..1 | Topology hiding information for a service API |  |
| onboardingCriteria | array(OnboardingCriteria) | O | 1..N | Contains the onboarding criteria failed to be met during API Invokers onboarding. | CAPIF_Ext1 |

# * * * * 5th Change * * * *

##### 8.3.4.3.3	Enumeration: CAPIFEvent

Table 8.3.4.3.3-1: Enumeration CAPIFEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SERVICE_API_AVAILABLE | Events related to the availability of service APIs after the service APIs are published. |  |
| SERVICE_API_UNAVAILABLE | Events related to the unavailability of service APIs after the service APIs are unpublished. |  |
| SERVICE_API_UPDATE | Events related to change in service API information |  |
| API_INVOKER_ONBOARDED | Events related to API invoker onboarded to CAPIF |  |
| API_INVOKER_OFFBOARDED | Events related to API invoker offboarded from CAPIF |  |
| SERVICE_API_INVOCATION_SUCCESS | Events related to the successful invocation of service APIs |  |
| SERVICE_API_INVOCATION_FAILURE | Events related to the failed invocation of service APIs |  |
| ACCESS_CONTROL_POLICY_UPDATE | Events related to the update for the access control policy related to the service APIs |  |
| ACCESS_CONTROL_POLICY_UNAVAILABLE | Events related to the unavailability of the access control policy related to the service APIs (NOTE) |  |
| API_INVOKER_AUTHORIZATION_REVOKED | Events related to the revocation of the authorization of API invokers to access the service APIs. (NOTE) |  |
| API_INVOKER_UPDATED | Events related to API invoker profile updated to CAPIF. |  |
| API_TOPOLOGY_HIDING_CREATED | Events related to the creation or update of the API topology hiding information of the service API after the service APIs are published |  |
| API_TOPOLOGY_HIDING_REVOKED | Events related to the revocation of the API topology information of the service API after the service APIs are unpublished |  |
| API_INVOKER_ONBOARDING_CRI_FAILED | Events related to API Invoker onboarding criteria failed to be met. | CAPIF_Ext1 |
| NOTE: The present release does not specify further details (e.g. event filters) for this event. |  |  |

# * * * * 6th Change * * * *

### 8.3.6	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.3.6-1 lists the supported features for CAPIF_Events_API.

Table 8.3.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to clause 7.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 7.6. This feature requires that the Notification_test_event feature is also supported. |
| 3 | Enhanced_event_report | This feature supports the enhanced event report including event reporting requirement and event reporting details as defined in clause 5.4.2.2.2. |
| 4 | ApiStatusMonitoring | Indicates the support of the API status monitoring in CAPIF layer as a part of enhancement of SEAL framework. This feature enables the following functionality: - enhancement of the CAPIF event notification. |
| 5 | CAPIF_Ext1 | Indicates the support of onboarding criteria information for the API Invoker onboarding functionality. This feature enables the following functionalities: - indicates the support of the CAPIF event report due to failed onboarding criteria during API Invokers onboarding. |

# * * * * 7th Change * * * *

# A.4	CAPIF_Events_API

openapi: 3.0.0

info:

title: CAPIF_Events_API

description: |

API for event subscription management.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.4.0-alpha.1"

externalDocs:

description: 3GPP TS 29.222 V19.1.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/capif-events/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222

paths:

/{subscriberId}/subscriptions:

post:

description: Creates a new individual CAPIF Event Subscription.

parameters:

- name: subscriberId

in: path

description: Identifier of the Subscriber

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EventSubscription'

callbacks:

notificationDestination:

'{$request.body#/notificationDestination}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EventNotification'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Created (Successful creation of subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/EventSubscription'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/capif-events/v1/{subscriberId}/subscriptions/{subscriptionId}

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{subscriberId}/subscriptions/{subscriptionId}:

delete:

description: Deletes an individual CAPIF Event Subscription.

parameters:

- name: subscriberId

in: path

description: Identifier of the Subscriber

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of an individual Events Subscription

required: true

schema:

type: string

responses:

'204':

description: >

The individual CAPIF Events Subscription matching the subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Update of an existing individual CAPIF Event Subscription.

parameters:

- name: subscriberId

in: path

description: Identifier of the Subscriber

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the individual Subscriber

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EventSubscription'

responses:

'200':

description: OK (Successful update of the subscription).

content:

application/json:

schema:

$ref: '#/components/schemas/EventSubscription'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: Modification of an existing individual CAPIF Event Subscription.

parameters:

- name: subscriberId

in: path

description: Identifier of the Subscriber

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the individual Subscriber

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/EventSubscriptionPatch'

responses:

'200':

description: OK (Successful update of the subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/EventSubscription'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

EventSubscription:

type: object

description: Represents an individual CAPIF Event Subscription resource.

properties:

events:

type: array

items:

$ref: '#/components/schemas/CAPIFEvent'

minItems: 1

description: Subscribed events

eventFilters:

type: array

items:

$ref: '#/components/schemas/CAPIFEventFilter'

minItems: 1

description: Subscribed event filters

eventReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: >

Set to true by Subscriber to request the CAPIF core function to send a

test notification as defined in in clause 7.6. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- events

- notificationDestination

EventNotification:

type: object

description: Represents an individual CAPIF Event notification.

properties:

subscriptionId:

type: string

description: >

Identifier of the subscription resource to which the notification

is related – CAPIF resource identifier

events:

$ref: '#/components/schemas/CAPIFEvent'

eventDetail:

$ref: '#/components/schemas/CAPIFEventDetail'

required:

- subscriptionId

- events

CAPIFEventFilter:

type: object

description: Represents a CAPIF event filter.

properties:

apiIds:

type: array

items:

type: string

minItems: 1

description: Identifier of the service API

apiInvokerIds:

type: array

items:

type: string

minItems: 1

description: Identity of the API invoker

aefIds:

type: array

items:

type: string

minItems: 1

description: Identifier of the API exposing function

CAPIFEventDetail:

type: object

description: Represents a CAPIF event details.

properties:

serviceAPIDescriptions:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceAPIDescription'

minItems: 1

description: Description of the service API as published by the APF.

apiIds:

type: array

items:

type: string

minItems: 1

description: Identifier of the service API

apiInvokerIds:

type: array

items:

type: string

minItems: 1

description: Identity of the API invoker

accCtrlPolList:

$ref: '#/components/schemas/AccessControlPolicyListExt'

invocationLogs:

type: array

items:

$ref: 'TS29222_CAPIF_Logging_API_Invocation_API.yaml#/components/schemas/InvocationLog'

minItems: 1

description: Invocation logs.

apiTopoHide:

$ref: '#/components/schemas/TopologyHiding'

onboardingCriteria:

type: array

items:

$ref: 'TS29222_CAPIF_API_Invoker_Management_API.yaml#/components/schemas/OnboardingCriteria'

minItems: 1

AccessControlPolicyListExt:

description: Represents the extension for access control policies.

allOf:

- $ref: 'TS29222_CAPIF_Access_Control_Policy_API.yaml#/components/schemas/AccessControlPolicyList'

- type: object

properties:

apiId:

type: string

required:

- apiId

TopologyHiding:

type: object

description: Represents the routing rules information of a service API.

properties:

apiId:

type: string

routingRules:

type: array

items:

$ref: 'TS29222_CAPIF_Routing_Info_API.yaml#/components/schemas/RoutingRule'

minItems: 1

required:

- apiId

- routingRules

EventSubscriptionPatch:

type: object

description: >

Represents the parameters to request the updated of an individual CAPIF Event

Subscription resource.

properties:

events:

type: array

items:

$ref: '#/components/schemas/CAPIFEvent'

minItems: 1

description: Subscribed events

eventFilters:

type: array

items:

$ref: '#/components/schemas/CAPIFEventFilter'

minItems: 1

description: Subscribed event filters

eventReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

CAPIFEvent:

anyOf:

- type: string

enum:

- SERVICE_API_AVAILABLE

- SERVICE_API_UNAVAILABLE

- SERVICE_API_UPDATE

- API_INVOKER_ONBOARDED

- API_INVOKER_OFFBOARDED

- SERVICE_API_INVOCATION_SUCCESS

- SERVICE_API_INVOCATION_FAILURE

- ACCESS_CONTROL_POLICY_UPDATE

- ACCESS_CONTROL_POLICY_UNAVAILABLE

- API_INVOKER_AUTHORIZATION_REVOKED

- API_INVOKER_UPDATED

- API_TOPOLOGY_HIDING_CREATED

- API_TOPOLOGY_HIDING_REVOKED

- API_INVOKER_ONBOARDING_CRITERIA_FAILED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Describes the CAPIF event.

Possible values are:

- SERVICE_API_AVAILABLE:

Events related to the availability of service APIs after the service APIs are

published.

- SERVICE_API_UNAVAILABLE:

Events related to the unavailability of service APIs after the service APIs are

unpublished.

- SERVICE_API_UPDATE: Events related to change in service API information.

- API_INVOKER_ONBOARDED: Events related to API invoker onboarded to CAPIF.

- API_INVOKER_OFFBOARDED: Events related to API invoker offboarded from CAPIF.

- SERVICE_API_INVOCATION_SUCCESS:

Events related to the successful invocation of service APIs.

- SERVICE_API_INVOCATION_FAILURE: Events related to the failed invocation of service APIs.

- ACCESS_CONTROL_POLICY_UPDATE:

Events related to the update for the access control policy related to the service APIs.

- ACCESS_CONTROL_POLICY_UNAVAILABLE:

Events related to the unavailability of the access control policy related to

the service APIs.

- API_INVOKER_AUTHORIZATION_REVOKED: Events related to the revocation of the authorization

of API invokers to access the service APIs.

- API_INVOKER_UPDATED: Events related to API invoker profile updated to CAPIF.

- API_TOPOLOGY_HIDING_CREATED:

Events related to the creation or update of the API topology hiding

information of the service APIs after the service APIs are published.

- API_TOPOLOGY_HIDING_REVOKED:

Events related to the revocation of the API topology hiding information of

the service APIs after the service APIs are unpublished.

- API_INVOKER_ONBOARDING_CRITERIA_FAILED: Events related to API Invoker onboarding criteria

failed to be met.

# * * * * End of Change * * * *