# 3GPP TSG CT WG3 Meeting #139	C3-250144

# Athens, Greece, 17 – 23 February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.565 | CR | 0165 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the RFC reference for the Yang Module |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | DetNet |  |  |  |  |  | Date: |  |  | 2025-1-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The agreed CR C3-246498 of CT3 #138 meeting has corrected the reference of draft-ietf-detnet-yang to IETF RFC 9633. But the implementation of the CR was not correct in the release 18 version. In addtion, we further found the description of the reference still refer to the draft-ietf-detnet-yang-17, which is not precise. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the reference of draft-ietf to the IETF RFC 9633. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with IETF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.2.2.2.1, C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR does not impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### B.2.2.2.1	General

Table B.2.2.2.1-1 specifies the data types defined for the _3gpp-5gs-detnet-node Module.

Table B.2.2.2.1-1: _3gpp-5gs-detnet-node Module specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| _3gpp-5gs-node-configuration-outcome | B.2.2.2.2.3 | It is a container that represents the additional outcome the 5GS may provide to a configuration request. |  |
| _3gpp-5gs-node-configuration-status | B.2.2.2.3.3 | It is an enumeration that represents the 3GPP specific configuration status that may be reported by the 5GS node. |  |
| _3gpp-5gs-node-identity | B.2.2.2.2.4 | It is a container that represents the user-plane node Id handling the traffic of the reported DetNet flows/PDU session. |  |
| _3gpp-5gs-node-requirements | B.2.2.2.2.2 | It is a container that represents the maximum delay and/or the maximum loss the 5GS needs to satisfy for the traffic of the DetNet flows indicated by the forwarding sublayer. |  |

Table B.2.2.2.1-2 specifies data types re-used by the _3gpp-5gs-detnet-node Module from other YANG modules, including a reference to their respective specifications and when needed, a short description of their use.

Table BA.2.2.2.1-2: _3gpp-5gs-detnet-node Module re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| forwarding-sub-layer-ref | IETF RFC 9633 [28] | Contains a reference to the forwarding sublayer as specified in IETF RFC 9633 YANG module. |  |

# *** Next Change ***

# C.2	YANG module _3gpp-5gs-detnet-node

<CODE BEGINS> file "_3gpp-5gs-detnet-node.yang"

module _3gpp-5gs-detnet-node {

yang-version 1.1;

namespace "urn:3gpp:node:detnet:_3gpp-5gs-detnet-node";

prefix _5gs3gppdnet;

import ietf-detnet {

prefix dnet;

reference

"IETF-RFC-9633";

}

organization "3GPP CT3 Working Group";

contact

"CT3 WG Web:  <https://www.3gpp.org/3gpp-groups/core-network-terminals-ct/ct-wg3>";

description

"_3gpp-5gs-detnet-node module contains an extension of IETF RFC 9633 DetNet YANG module with

additional parameters defined for interworking with 3GPP 5GS.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

This version of this YANG module is specified in:

3GPP TS 29.565 V18.5.0; 5G System;

Time Sensitive Communication and Time Synchronization Function Services.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.565/";

reference

"Additional information to this YANG module is specified in:

3GPP TS 23.501, System architecture for the 5G System (5GS);

url: https://www.3gpp.org/ftp/Specs/archive/23_series/23.501/

3GPP TS 23.503, Policy and charging control framework (5GS)

url: https://www.3gpp.org/ftp/Specs/archive/23_series/23.503/

3GPP TS 29.513, Policy and Charging Control signalling flows and QoS parameter mapping

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.513/";

revision 2024-12-10 {

description "version: v1.0.0";

}

typedef _3gpp-5gs-node-configuration-status {

type enumeration {

enum _3gpp-5gs-node-resource-allocation-success {

description

"Successful configuration request.

Successful resource allocation within 5GS for the requested configuration";

}

enum _3gpp-5gs-node-resource-allocation-failure {

description

"Unsuccessful configuration request.

Failed resource allocation within 5GS for the requested configuration";

}

enum _3gpp-5gs-node-port-release {

description

"Port release due to the termination of PDU session with the requested

5GS configuration";

}

description

"_3gpp-5gs-node-configuration-status type identifies the 3GPP specific configuration

status that may be reported by the 3GPP 5GS node.";

}

container _3gpp-5gs-node-requirements {

description

"This container defines the maximum delay and/or the maximum loss the 5GS needs to satisfy";

leaf forwarding-sub-layer {

type dnet:forwarding-sub-layer-ref;

description

"Reference to the forwarding sub-layer that the maximum delay and/or the maximum loss

applies to";

}

leaf _3gpp-5gs-node-max-latency {

type uint32;

units "nanoseconds";

description

"Maximum latency from 5GS node ingress to 5GS node egress(es) for a single packet of the

DetNet flow. It is specified as an integer number of nanoseconds";

}

leaf _3gpp-5gs-node-max-loss {

type uint32;

description

"Maximum Packet Loss Ration (PLR) parameter for the DetNet service between the 5GS node

ingress and 5GS node egress(es)";

}

}

container _3gpp-5gs-node-configuration-outcome {

description

"This container defines the additional outcome the 5GS may provide to a configuration

request";

leaf forwarding-sub-layer {

type dnet:forwarding-sub-layer-ref;

description

"Reference to the forwarding sub-layer the outcome to a configuration request applies to";

}

leaf _3gpp-5gs-node-configuration-status {

type _3gpp-5gs-node-configuration-status;

description "Changes on configuration status reported by the 3GPP 5GS node";

}

}

container _3gpp-5gs-node-identity {

description

"This container defines the 5GS Node identity the 5GS provides to the DetNet controller";

leaf forwarding-sub-layer {

type dnet:forwarding-sub-layer-ref;

description

"Reference to the forwarding sub-layer that the 5GS node Id applies to";

}

leaf _3gpp-5gs-node-id {

type string;

description

"5GS node identity. It is the user-plane node Id handling the traffic of the indicated

DetNet flows";

}

}

}

<CODE ENDS>

# *** End of Changes ***