3GPP TSG-CT WG1 Meeting #153	C1-250855

# Athens, Greece, 17-21 February 2025													was C1-250276

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.281 | CR | 0275 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCVideo control plane user plane separation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2025-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA6#64 CR 0238 (tdoc S6-245364) was agreed that allows to use media distribution function and the transmission control server use different IP addresses than the MCVideo server. This CR covers the associated stage 3 changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 6.3.3.1.1: - changing controlling MCVideo function to media distribution function for the media stream - changing controlling MCVideo function to transmission control server for the media transmission control entity 6.3.3.2.1: - changing controlling MCVideo function to media distribution function for the media stream - changing controlling MCVideo function to transmission control server for the media transmission control entity Backward compatibility analysis: The changes are backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 not aligned with stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.1.1, 6.3.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.3.3.1.1	SDP offer generation

The SDP offer is generated based on the received SDP offer. The SDP offer generated by the controlling MCVideo function:

1)	when initiating a new MCVideo session the SDP offer;

a)	shall contain only one SDP media-level section for MCVideo video media stream as contained in the received SDP offer; and

b)	shall contain an SDP media-level section for one media-transmission control entity, if present in the received SDP offer; and

2)	when adding a new MCVideo user to an existing MCVideo Session, the SDP offer shall contain the media stream currently used in the MCVideo session.

When composing the SDP offer according to 3GPP TS 24.229 [11], the controlling MCVideo function:

1)	shall replace the IP address and port number for the offered media streams in the received SDP offer with the IP address and port number of the media distribution function of the controlling MCVideo function;

2)	for the MCVideo video media streams, shall include all media-level attributes from the received SDP offer;

3)	shall replace the IP address and port number for the offered media transmission control entity, if any, in the received SDP offer with the IP address and port number of the transmission control server of the controlling MCVideo function;

4)	shall include an mc_transmission_ssrc 'fmtp' attribute as specified in 3GPP TS 24.581 [5] clause 14 for the selected media plane control channel with the RTCP SSRC that the controlling MCVideo function has selected as specified in 3GPP TS 24.581 [5] clause 4.3 and shall be used by the answerer in the transmission control messages sent to the controlling MCVideo function for this session; and

NOTE:	The controlling MCVideo function will receive in the SDP answer the RTP SSRC it will have to use in the transmission control message it will send to the answerer in this session.

5)	for the offered media transmission control entity, shall include any other necessary 'fmtp' attribute as specified in 3GPP TS 24.581 [5] clause 14.

* * * Next Change * * * *

##### 6.3.3.2.1	SDP answer generation

When composing the SDP answer according to 3GPP TS 24.229 [11], the controlling MCVideo function:

1)	for the accepted media streams in the received SDP offer:

a)	shall replace the IP address and port number in the received SDP offer with the IP address and port number of the media distribution function of the controlling MCVideo function; and

2)	for the accepted media-transmission control entity, if present in the received SDP offer:

a)	shall replace the IP address and port number in the received SDP offer with the IP address and port number of the transmission control server of the controlling MCVideo function;

b)	shall include an mc_transmission_ssrc 'fmtp' attribute as specified in 3GPP TS 24.581 [5] clause 14 for the selected media plane control channel with the RTCP SSRC that the controlling MCVideo function has selected as specified in 3GPP TS 24.581 [5] clause 4.3 and shall be used by the offerer in the transmission control messages sent to the controlling MCVideo function for this session; and

NOTE:	The controlling MCVideo function has received in the SDP offer the RTP SSRC it will have to use in the transmission control message it will send to the offerer in this session.

c)	shall include any other necessary 'fmtp' attributes as specified in 3GPP TS 24.581 clause 14.

* * * End of Changes * * * *