3GPP TSG-CT Meeting #107	CP-250192

# Incheon, Korea; 12th – 13th March 2025	revision of C1-251026

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0705 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G ProSe multi-hop L3 U2U relay discovery for Ethernet and unstructured data unit type with model B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | ZTE |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 5G ProSe multi-hop layer-3 UE-to-UE relay discovery for non-IP communication procedure with model B needs to be specified according to 6.3.2.6.3.2 procedure for Multi-hop 5G ProSe UE-to-UE Relay Discovery of non-IP PDU type with Model A of TS 23.304. Comparing with multi-hop layer-3 UE-to-UE relay discovery for IP communication with model B, the main differences are： - the "List of multi-hop UE-to-UE relay paths" needs to be included in the solicitation; - the target end UE is discovered in this procedure; - other related parameters specified in clause 5.8.6.3 Identifiers for 5G ProSe Multi-hop UE-to-UE Relay discovery of non-IP PDU type of TS 23.304: "The 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) is sent by the source 5G ProSe End UE and includes the following parameters for multi-hop relaying in addition to as described in clause 5.3.2.4.2: - Hop count: indicates the number of hops that the message is already relayed. It is set to 0 initially by the Discoverer End UE and is increased by 1 per hop. - (Optional) Hop-Limit: value that indicates the limit of the number of hops of the message. It is set, by the 5G ProSe Remote UE, to a value smaller than or equal to the (pre)configured maximum number of hops. The 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) is sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC and includes the following parameters for multi-hop relaying in addition to as described in clause 5.3.2.4.2: - Hop count: indicates the number of hops that the message is already relayed. It is increased by 1 per hop. - (Optional) Hop-Limit: value that indicates the limit of the number of hops of the message. It is set, by the 5G ProSe Remote UE, to a value smaller than or equal to the (pre)configured maximum number of hops. - Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer UE. The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC includes the following parameters for multi-hop relaying in addition to as described in clause 5.3.2.4.2: - Hop count: indicates the number of hops that the message is already relayed. - Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer . The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the target 5G ProSe End UE matching the RSC includes the following the following parameters for multi-hop relaying in addition to as described in clause 5.3.2.4.2: - Hop count: indicates the number of hops that the message is already relayed. - Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer Prose End UE. " |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the procedures for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery for non-IP communication with model B including: - Discoverer end UE procedure for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery; - Discoverer relay UE procedure for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery; and - Discoveree end UE procedure for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 requirement is not implemented in stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8c.2.1.3.3 (new), 8c.2.1.3.3.1 (new), 8c.2.1.3.3.2 (new), 8c.2.1.3.3.2.1 (new), 8c.2.1.3.3.2.2 (new), 8c.2.1.3.3.2.3 (new), 8c.2.1.3.3.3 (new), 8c.2.1.3.3.4 (new), 8c.2.1.3.3.4.1 (new), 8c.2.1.3.3.4.2 (new), 8c.2.1.3.3.4.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The encoding of the List of multi-hop UE-to-UE relay paths IE is defined in TS 24.554 CR 0704. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#2 is submitted to CT#107 to fix the incorrect tdoc number in the cover sheet of C1-251026 which was agreed in CT1#153. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8c.2.1.3.3	5G ProSe multi-hop layer-3 UE-to-UE relay discovery for Ethernet and unstructured data unit type with model B

8c.2.1.3.3.1	General

In the following procedures:

a)	the source 5G ProSe multi-hop layer-3 end UE sending the PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery solicitation are called the "discoverer end UE";

b)	the 5G ProSe multi-hop layer-3 UE-to-UE relay UE sending the PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery solicitation based on the received PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery solicitation from other 5G ProSe multi-hop layer-3 UE-to-UE relay UE or 5G ProSe multi-hop layer-3 end UE are called the "discoverer relay UE"; and

c)	the target 5G ProSe multi-hop layer-3 end UE sending the corresponding PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery response is called the "discoveree end UE".

8c.2.1.3.3.2	Discoverer end UE procedure for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery

8c.2.1.3.3.2.1	General

For Ethernet and unstructured data unit type, the purpose described in clause 8c.2.1.2.3.2.1 applies with following addition:

a)	to enable a ProSe-enabled UE to discover the path between the source 5G ProSe multi-hop layer-3 end UE and the target 5G ProSe multi-hop layer-3 end UE.

8c.2.1.3.3.2.2	Discoverer end UE procedure for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery initiation

For Ethernet and unstructured data unit type, the procedure specified in clause 8c.2.1.2.3.2.2 applies with following differences and clarifications:

a)	the UE is configured with the relay service code parameter with the non-IP traffic type of the relayed service provided by a 5G ProSe multi-hop layer-3 UE-to-UE relay to be solicited as specified in clause 5.2.9;

b)	in the PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery solicitation, the UE shall include the List of multi-hop UE-to-UE relay paths IE as defined in clause 11.2.xx. In the List of multi-hop UE-to-UE relay paths IE:

1)	only "Path info #1" is included;

2)	the Hop count field in the "Path info #1" is set to 0; and

3)	the list of protected user infos field in the "Path info #1" only includes the protected user info of the source 5G ProSe multi-hop layer-3 end UE;

4)	the value of the length of the path of relays field in the "Path info #1" is set to 0; and

c)	if the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery response is the same as the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery solicitation, then the UE shall consider that the target 5G ProSe multi-hop layer-3 end UE the UE seeks has been discovered.

8c.2.1.3.3.2.3	Discoverer end UE procedure for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery completion

For Ethernet and unstructured data unit type, the procedure specified in clause 8c.2.1.2.3.2.3 applies.

8c.2.1.3.3.3	Discoverer relay UE procedure for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery

For Ethernet and unstructured data unit type, the procedure specified in clause 8c.2.1.2.3.3 applies with following differences and clarification:

a)	if the relay service code parameter of the received PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery solicitation is the same as the relay service code parameter configured as specified in clause 5.2.9 for the connectivity service, the discoverer relay UE determins to send the PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery solicitation upon receipt of a PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery solicitation from the source 5G ProSe multi-hop layer-3 end UE or other 5G ProSe multi-hop layer-3 UE-to-UE relay UE.

For the recevived List of multi-hop UE-to-UE relay paths IE, if:

1)	the value of the hop count field is less than the hop limit as described in clause 6.3.2.6.3.2 of 3GPP TS 23.304 [2]; and

2)	the user info ID of the discoverer relay UE is not listed in the "Path of relays" field, if any,

the UE shall generate a PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery solicitation and set the PROSE PC5 DISCOVERY message as specified in clause 8c.2.1.3.3.2.2 with following differences:

i)	shall update received List of multi-hop UE-to-UE relay paths IE as follows:

-	the Hop count field is increased by 1;

-	add the user info ID of the discoverer relay UE to the "Path of relays" as described in table 11.2.xx.2; and

-	update the value of the length of the path of relays field accordingly; and

i)	shall include the updated List of multi-hop UE-to-UE relay paths IE in the PROSE PC5 DISCOVERY message.

otherwise, UE shall drop the received PROSE PC5 DISCOVERY message;

b)	if the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery response is the same as the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery solicitation, and the user info ID of the discoverer relay UE is listed in the "Path of relays" field, the UE shall forward PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery response as specified in 6.3.2.6.3.2 of 3GPP TS 23.304 [2].

Editor's note (5G_ProSe_Ph3, CR0705):	The details of setting the PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery response are FFS.

8c.2.1.3.3.4	Discoveree end UE procedure for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery

8c.2.1.3.3.4.1	General

The purpose of the discoveree end UE procedure for multi-hop layer-3 UE-to-UE relay discovery is to enable the target 5G ProSe multi-hop layer-3 end UE to respond to solicitation from other ProSe-enabled UEs on proximity of a connectivity service via a 5G ProSe UE-to-UE relay UE and discover the path between the source 5G ProSe multi-hop layer-3 end UE and the target 5G ProSe multi-hop layer-3 end UE, upon a request from upper layers.

8c.2.1.3.3.4.2	Discoveree end UE procedure for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery initiation

The UE is authorised to perform the discoveree end UE procedure for multi-hop layer-3 UE-to-UE relay discovery if:

a)	the UE is authorised to act as a 5G ProSe multi-hop layer-3 end UE in the PLMN indicated by the serving cell, and

1)	the UE is served by NG-RAN; or

2)	the UE is not served by NG-RAN and intends to use the provisioned radio resources for multi-hop layer-3 5G ProSe UE-to-UE relay discovery; and

b)	the UE is configured with:

1)	the relay service code parameter identifying the non-IP connectivity service to be responded to as specified in clause 5.2.9 and the indicated security procedure for the relay service code is supported by the UE;

otherwise, the UE is not authorised to perform the discoveree end UE procedure for multi-hop UE-to-UE relay discovery.

Figure 8a.2.1.3.3.4.2.1 illustrates the interaction of the UEs in the discoveree end UE procedure for multi-hop layer-3 UE-to-UE relay discovery.

Figure 8a.2.1.3.3.4.2.1: Discoveree end UE procedure for multi-hop layer-3 UE-to-UE relay discovery

When the UE is triggered by the upper layers to start responding to solicitation on proximity of a connectivity service provided by a 5G ProSe multi-hop layer-3 UE-to-UE relay UE and if the UE is authorised to perform the discoveree end UE procedure for multi-hop layer-3 UE-to-UE relay discovery, then the UE:

a)	if the UE is served by NG-RAN and the UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages as specified in 3GPP TS 38.331 [13], shall perform a service request procedure as specified in 3GPP TS 24.501 [11]; and

b)	shall instruct the lower layers to start monitoring for PROSE PC5 DISCOVERY messages.

Editor's note [5G_ProSe_Ph3, CR#0705]:	Handling of PROSE PC5 DISCOVERY message using the security materials received from 5G DDNMF or 5G PKMF is FFS and depends on SA3 output.

Editor's note [5G_ProSe_Ph3, CR#0705]:	.How to handle the case when the discoveree UE receives multiple solicitation messages from the same source 5G ProSe multi-hop end UE is FFS.

Then, if:

a)	the relay service code parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation is the same as the relay service code parameter configured as specified in clause 5.2.9 for the connectivity service; and

b)	the Discoveree user info IE containing the application layer ID of discoveree end UE in the received PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery solicitation is the same as the application layer ID of the UE provided by upper layers ;

then the UE:

a)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time;

b)	shall generate a PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery response. In the PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery response, the UE:

1)	shall include the List of multi-hop UE-to-UE relay paths IE in the received PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery solicitation;

Editor's note (5G_ProSe_Ph3, CR#0705): The detailed description on updating the list of stored protected user info of 5G ProSe multi-hop layer-3 end UE(s), the path of relays, the hop count and the hop limit Hop limit and Hop count in List of multi-hop UE-to-UE relay paths IE is FFS.

2)	shall set the relay service code parameter to the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery solicitation;

3)	shall include the MIC field computed as described in 3GPP TS 33.503 [34];

4)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter; and

5)	shall set the ProSe direct discovery PC5 message type IE and ProSe direct discovery PC5 message content type extensions IE as specified in table 11.2.1.1 and table 11.2.1a.1;

c)	shall set the destination layer-2 ID to the source layer-2 ID from the 5G ProSe multi-hop layer-3 UE-to-UE relay UE used in the transportation of the PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery solicitation and self-assign a source layer-2 ID for sending the multi-hop layer-3 UE-to-UE relay discovery response message; and

NOTE 2:	The UE implementation ensures that the value of the self-assigned source layer-2 ID is different from any other self-assigned source layer-2 ID(s) in use for 5G ProSe direct communication as specified in clause 7.2 and is different from any other provisioned destination layer-2 ID(s) as specified in clause 5.2.

d)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery response along with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission over the PC5 interface.

8c.2.1.3.3.4.3	Discoveree end UE procedure for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery completion

When the UE is triggered by the upper layers to stop responding to solicitation on proximity of a connectivity service via a 5G ProSe multi-hop layer-3 UE-to-UE relay UE, or when the UE stops being authorised to perform the discoveree end UE procedure for multi-hop layer-3 UE-to-UE relay discovery, the UE shall instruct the lower layers to stop monitoring.

When the UE stops monitoring, if the UE is in 5GMM-CONNECTED mode, the UE shall trigger the corresponding procedure in lower layers as specified in 3GPP TS 38.331 [13].

* * * End of Changes * * * *