3GPP TSG-CT Meeting #93-e	CP-212112

# E-Meeting, 13th – 15th September 2021													Was C4-214858

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.505 | CR | 0386 | rev | 2 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SharedDataId Resource |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Hewlett Packard Enterprise, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Hewlett Packard Enterprise, Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc16 |  |  |  |  |  | Date: |  |  | 2021-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CR #703 to 29.503 defines the SharedDataId as a resource with a GET operation. Since the same resource also exist in the Nudr, the sharedDataId resource in 29.505 should also support the same capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The ShareDataId resource is defined with a GET operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between 29.503 and 29.505 wrt the SharedDataId resource. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1, 5.2.X (new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible corrections with impacts on the following OpenAPI specifications: - TS29504_Nudr_DR.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: -Removed supported-features query parameter -corrected the resource URI in table 5.2.1-1 Rev2: -Removal of extra default response in yaml |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

### 5.2.1	Overview

# Figure 5.2.1-1: Resource URI sub-level structure for subscription data

# Figure 5.2.1-2: Resource URI sub-level structure for subscription data (cont.)

# Figure 5.2.1-3: Resource URI sub-level structure for subscription data (cont.)

Table 5.2.1-1 provides an overview of the resources and applicable HTTP methods.

# Table 5.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| AuthenticationSubscription | /subscription-data/{ueId}/authentication-data/authentication-subscription | GET | Retrieve a UE's authentication subscription data |
|  |  | PATCH | Update a UE's authentication subscription data Updates shall be limited to the sequenceNumber attribute. Attempts to patch any other attribute shall be rejected by the UDR. |
| AuthenticationSoR | /subscription-data/{ueId}/ue-update-confirmation-data/sor-data | PUT | Store UE's SOR acknowledgement information (SoR-XMAC-IUE) |
|  |  | GET | Retrieve UE's SoR acknowledgement information |
| AuthenticationUPU | /subscription-data/{ueId}/ue-update-confirmation-data/upu-data | PUT | Store UE Parameter Update acknowledgement information (UPU-XMAC-IUE) |
|  |  | GET | Retrieve UE Parameter Update acknowledgement information |
| SubscribedSNSSAIs | /subscription-data/{ueId}/ue-update-confirmation-data/subscribed-snssais | PUT | Store UE-acknowledgement info for change of subscribed S-NSSAIs |
|  |  | GET | Retrieve UE-acknowledgement info for change of subscribed S-NSSAIs |
| SubscribedCAG | /subscription-data/{ueId}/ue-update-confirmation-data/subscribed-cag | PUT | Store UE-acknowledgement info for change of subscribed CAG |
|  |  | GET | Retrieve UE-acknowledgement info for change of subscribed CAG |
| AuthenticationStatus | /subscription-data/{ueId}/authentication-data/authentication-status | PUT | Store a UE's authentication status |
|  |  | GET | Retrieve a UE's authentication status |
|  |  | DELETE | Delete a UE's authentication status |
| IndividualAuthenticationStatus | /subscription-data/{ueId}/authentication-data/authentication-status/{servingNetworkName} | PUT | When the feature "PerUePerSnAuthStatus" is supported, store a UE's Individual authentication status |
|  |  | GET | When the feature "PerUePerSnAuthStatus" is supported, retrieve a UE's Individual authentication status |
|  |  | DELETE | When the feature "PerUePerSnAuthStatus" is supported, delete a UE's Individual authentication status |
| ProvisionedData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data | GET | Retrieve the UE's subscribed Provisioned Data |
| AccessAndMobilitySubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/am-data | GET | Retrieve the UE's subscribed Access and Mobility Data |
| SmfSelectionSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/smf-selection-subscription-data | GET | Retrieve the UE's subscribed SMF Selection Data |
| SessionManagementSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sm-data | GET | Retrieve the UE's subscribed SM Subscription Data |
| ContextData | /subscription-data/{ueId}/context-data | GET | Retrieve the UE's context Data |
| Amf3GppAccessRegistration | /subscription-data/{ueId}/context-data/amf-3gpp-access | PUT | Create and Update the AMF registration for 3GPP access |
|  |  | PATCH | Modify the AMF registration for 3GPP access |
|  |  | GET | Retrieve the AMF registration information for 3GPP access |
| AmfNon3GppAccessRegistration | /subscription-data/{ueId}/context-data/amf-non-3gpp-access | PUT | Update the AMF registration for non 3GPP access |
|  |  | PATCH | Modify the AMF registration for non 3GPP access |
|  |  | GET | Retrieve the AMF registration information for non 3GPP access |
| SmfRegistrations | /subscription-data/{ueId}/context-data/smf-registrations | GET | Retrieve the list of the SMF registrations |
| IndividualSmfRegistration | /subscription-data/{ueId}/context-data /smf-registrations/{pduSessionId} | PUT | Store an individual SMF registration identified by PDU Session Id |
|  |  | DELETE | Delete an individual SMF registration |
|  |  | GET | Retrieve individual SMF registration |
| OperatorSpecificData | /subscription-data/{ueId}/operator-specific-data | GET | retrieve the operator specific subscription data of a UE |
|  |  | PATCH | modify the operator specific subscription data of a UE |
| OperatorDeterminedBarringData | /subscription-data/{ueId}/operator-determined-barring-data | GET | Retrieve the UE's Operator Determined Barring |
| SMSManagementSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-mng-data | GET | Retrieve the UE's subscribed SMS management subscription data. |
| Smsf3GppAccessRegistration | /subscription-data/{ueId}/context-data /smsf-3gpp-access | PUT | Create or Update the SMSF registration |
|  |  | DELETE | Delete the SMSF registration for 3GPP access |
|  |  | GET | Retrieve the SMSF registration information |
| SmsfNon3GppAccessRegistration | /subscription-data/{ueId}/context-data /smsf-non-3gpp-access | PUT | Create or Update the SMSF registration for non 3GPP access |
|  |  | DELETE | Delete the SMSF registration for non 3GPP access |
|  |  | GET | Retrieve the SMSF registration information for non 3GPP access |
| IpSmGwRegistration | /subscription-data/{ueId}/context-data/ip-sm-gw | PUT | Create or Update the IP-SM-GW registration |
|  |  | DELETE | Delete the IP-SM-GW registration |
|  |  | PATCH | Modify the IP-SM-GW registration |
|  |  | GET | Retrieve the IP-SM-GW registration information |
| MessageWaitingData | /subscription-data/{ueId}/context-data/mwd | PUT | Create or Update the SMS Message Waiting Data |
|  |  | DELETE | Delete the SMS Message Waiting Data |
|  |  | PATCH | Modify the SMS Message Waiting Data |
|  |  | GET | Retrieve the SMS Message Waiting Data |
| SdmSubscriptions | /subscription-data/{ueId}/context-data/sdm-subscriptions | GET | Retrieve SDM subscriptions |
|  |  | POST | Create an individual SDM subscription |
| IndividualSdmSubscription | /subscription-data/{ueId}/context-data/sdm-subscriptions/{subsId} | PUT | Update an individual SDM subscription |
|  |  | DELETE | Delete an individual SDM subscription |
|  |  | PATCH | Update an individual SDM Subscription |
|  |  | GET | Retrieve an individual SDM subscription |
| EeSubscriptions | /subscription-data/{ueId}/context-data/ee-subscriptions | GET | Retrieve EE subscriptions |
|  |  | POST | Create an EE subscription |
| IndividualEeSubscription | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId} | PUT | Update an individual EE subscription |
|  |  | DELETE | Delete an individual EE subscription |
|  |  | PATCH | Update an individual EE subscription |
|  |  | GET | Retrieve an individual EE subscription |
| AmfSubscriptionInfo | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/amf-subscriptions | PUT | Store information related to the Amf-EE-Subscriptions |
|  |  | DELETE | Delete the Amf-EE-subscriptions |
|  |  | GET | Retrieve AMF-subscriptions |
|  |  | PATCH | Update AMF-subscriptions |
| SmfSubscriptionInfo | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/smf-subscriptions | PUT | Store information related to the Smf-EE-Subscriptions |
|  |  | DELETE | Delete the Smf-EE-subscriptions |
|  |  | GET | Retrieve SMF-subscriptions |
|  |  | PATCH | Update SMF-subscriptions |
| HssSubscriptionInfo | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/hss-subscriptions | PUT | Store information related to the Hss-EE-Subscriptions |
|  |  | DELETE | Delete the Hss-EE-subscriptions |
|  |  | GET | Retrieve Hss-EE-subscriptions |
|  |  | PATCH | Update Hss-EE-subscriptions |
| EeProfileData | /subscription-data/{ueId}/ee-profile-data | GET | Retrieve the UE's subscribed EE profile data. |
| ProvisionedParamenterData | /subscription-data/{ueId}/pp-data | PATCH | Update of provisioned parameters. |
|  |  | GET | Retrieves the UE's provisioned parameters. |
| PpProfileData | /subscription-data/{ueId}/pp-profile-data | GET | Retrieve the UE's subscribed PP profile data. |
| SMSSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-data | GET | Retrieve the UE's subscribed SMS subscription data. |
| SubscriptionDataSubscriptions | /subscription-data/subs-to-notify | GET | Retrieve existing subscriptions |
|  |  | POST | Create a subscription, i.e. subscribe a node to receive notification for change of data |
|  |  | DELETE | Deletes multiple subscriptions for a given UE. |
| IndividualSubscriptionDataSubscription | /subscription-data/subs-to-notify/{subsId} | DELETE | Delete the subscription identified by {subsId}, i.e. unsubscribe a node to receive notification for change of data |
|  |  | PATCH | Update an individual Subscription to notification |
|  |  | GET | Retrieve an individual Subscription to notification |
| EeGroupSubscriptions | /subscription-data/group-data/{ueGroupId}/ee-subscriptions | GET | Retrieve EE subscriptions for groups of UEs |
|  |  | POST | Create an EE subscription for groups of UEs |
| IndividualEeGroupSubscription | /subscription-data/group-data/{ueGroupId}/ee-subscriptions/{subsId} | PUT | Update an individual EE subscription for a group of UEs |
|  |  | DELETE | Delete an individual EE subscription for a group of UEs |
|  |  | PATCH | Update an individual EE subscription for a group of UEs |
|  |  | GET | Retrieve an individual EE subscription for a group of UEs |
| EeGroupProfileData | /subscription-data/group-data/{ueGroupId}/ee-profile-data | GET | Retrieve the subscribed EE profile data for a group of UEs. |
| TraceData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/trace-data | GET | Retrieve the UE's trace configuration data |
| IdentityData | /subscription-data/{ueId}/identity-data | GET | Retrieve identity data that corresponds to the provided ueId |
| SharedData (Collection) | /subscription-data/shared-data | GET | Retrieve shared data |
| IndividualSharedData (Document) | /subscription-data/shared-data/{sharedDataId} | GET | Retrieve the individual Shared Data |
| GroupIdentifiers | /subscription-data/group-data/group-identifiers | GET | Retrieve group identifiers and the UE identifiers belong to the group identifiers. |
| 5GVnGroups | /subscription-data/group-data/5g-vn-groups | GET | Retrieve 5G VN Groups |
| Individual5GVnGroup | /subscription-data/group-data/5g-vn-groups/{externalGroupId} | PUT | Create a 5G VN Group |
|  |  | PATCH | Update a 5G VN Group |
|  |  | DELETE | Delete a 5G VN Group |
|  |  | GET | Retrieve a 5G VN Group |
| 5GVnGroupsInternal | /subscription-data/group-data/5g-vn-groups/internal | GET | Retrieve 5G VN Group Data based on Internal Group Identifier(s) |
| Pp5gVnGroupProfileData | /subscription-data/group-data/5g-vn-groups/pp-profile-data | GET | Retrieve the UE's subscribed PP profile data for accessing 5G VN Groups service operations. |
| LcsPrivacySubscriptionData | /subscription-data/{ueId}/lcs-privacy-data | GET | Retrieve the UE's subscribed LCS privacy Subscription Data |
| LcsMobileOriginatedSubscriptionData | /subscription-data/{ueId}/lcs-mo-data | GET | Retrieve the UE's subscribed LCS Mobile Originated Subscription Data |
| NiddAuthorizationData | /subscription-data/{ueId}/nidd-authorization-data | GET | Retrieve the UE's NIDD Authorization Data |
| CoverageRestrictionData | /subscription-data/{ueId}/coverage-restriction-data | GET | Retrieve the UE's subscribed enhanced Coverage Restriction Data |
| Location | /subscription-data/{ueId}/context-data/location | GET | Retrieve the UE's Location Information |
| V2xSubscriptionData | /subscription-data/{ueId}/v2x-data | GET | Retrieve the UE's subscribed V2X Data |
| LcsBroadcastAssistanceSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/lcs-bca-data | GET | Retrieve the UE's subscribed LCS Broadcast Assistance subscription data |

* * * Next Change * * * *

### 5.2.X	Resource: IndividualSharedData

#### 5.2.X.1	Description

This resource represents the individual shared data.

#### 5.2.X.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/shared-data/{sharedDataId}

This resource shall support the resource URI variables defined in table 5.2.X.2-1.

# Table 5.2.X.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| sharedDataId | Contains the individual Shared Data Identifier. |

#### 5.2.X.3	Resource Standard Methods

##### 5.2.X.3.1	GET

This method shall support the URI query parameters specified in table 5.2.X.3.1-1.

# Table 5.2.X.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.X.3.1-2 and the response data structures and response codes specified in table 5.2.X.3.1-3.

# Table 5.2.X.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.X.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type |  | P |  | Cardinality |  | Response codes | Description |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SharedData |  | M |  | 1 |  | 200 OK | Upon success, a response body containing SharedData shall be returned. |  |  |
| ProblemDetails |  |  | O |  | 0..1 |  | 404 Not Found |  |  | The "cause" attribute may be used to indicate one of the following application errors: - DATA_NOT_FOUND |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

## A.2	Nudr_DataRepository API for Subscription Data

For the purpose of referencing entities in the Open API file defined in this Annex, it shall be assumed that this Open API file is contained in a physical file named "TS29505_Subscription_Data.yaml".

openapi: 3.0.0

info:

version: '-'

title: 'Unified Data Repository Service API file for subscription data'

description: |

Unified Data Repository Service (subscription data).

The API version is defined in 3GPP TS 29.504.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.505 V16.7.0; 5G System; Usage of the Unified Data Repository Service for subscription data; Stage 3

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.505/'

*************text not shown for clarity****************

/subscription-data/shared-data:

get:

summary: retrieve shared data

operationId: GetSharedData

tags:

- Retrieval of shared data

parameters:

- name: shared-data-ids

in: query

description: List of shared data ids

required: true

style: form

explode: false

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SharedDataIds'

- name: supportedFeatures

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: array

items:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SharedData'

minItems: 1

'400':

# $ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

# $ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

# $ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

# $ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/shared-data/{sharedDataId}:

get:

summary: retrieve individual shared data

operationId: GetIndividualSharedData

tags:

- Retrieval of individual shared data

parameters:

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SharedData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

'400':

# $ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

# $ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

# $ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

# $ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/subs-to-notify:

post:

summary: Subscription data subscriptions

operationId: SubscriptionDataSubscriptions

tags:

- Subs To Notify (Collection)

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/SubscriptionDataSubscriptions'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SubscriptionDataSubscriptions'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/subs-to-notify/{subsId}'

required: true

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

onDataChange:

'{request.body#/callbackReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DataChangeNotify'

responses:

'204':

description: Expected response to a valid request

get:

summary: Retrieves the list of subscriptions

operationId: QuerySubsToNotify

tags:

- Subs To Notify (Collection)

parameters:

- name: ue-id

in: query

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/SubscriptionDataSubscriptions'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes subscriptions identified by a given ue-id parameter

operationId: RemoveMultipleSubscriptionDataSubscriptions

tags:

- Subs To Notify (Collection)

parameters:

- name: ue-id

in: query

required: true

description: UE ID

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: nf-instance-id

in: query

required: false

description: NF Instance ID

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

- name: delete-all-nfs

in: query

required: false

description: Flag to delete subscriptions from all NFs

schema:

type: boolean

- name: implicit-unsubscribe-indication

in: query

required: false

description: Implicit Unsubscribe Indication

schema:

type: boolean

responses:

'204':

description: Expected response to a successful subscription removal

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

*************text not shown for clarity****************

* * * End of Changes * * * *